TEK VX433X/VX437X Master/Slave Scanner Modules

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for Tektronics

VX4332/VX4334 Scanner Master and VX4372/VX4374 Scanner Slave Modules.

It contains functions for opening, configuring, taking measurements from,

and closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

For VXI instruments drivers:

-  the instrument is installed in the VXI mainframe and you are

using one of the following controller options:

Embedded controller

AT-MXI

MC-MXI

GPIB-VXI

-  the logical address supplied to the initialize function must

match the logical address of the instrument.

-  if you are using GPIB-VXI, there must be at least one unused

device name in the handler.

________________________________________________________________________________

LabWindows Libraries:

In order to use this instrument driver outside of the LabWindows

environment, it must be linked with the following LabWindows libraries:

For VXI instruments drivers:

-  The appropriate VXI Library:

VXI386.LIB for embedded controllers

VXIAT.LIB for the AT-MXI

VXIMC.LIB for the MC-MXI

GPIB_C.LIB for GPIB-VXI (C users)

-  Format and I/O Library

-  LabWindows System Library

National Instruments strongly recommends that you use the LWMake utility to

build any standalone executable program based on this intrument driver.

________________________________________________________________________________

Error Codes:

Error codes are returned as the return value of each instrument driver

function. A program should examine this value after each call to an

instrument driver function to determine if any error occurred. Possible

error codes and their meanings are listed with the corresponding instrument

driver function.

________________________________________________________________________________

How To Use This Document:

This document is intended to be used as a programming reference manual.

It describes each function in the

TEK VX433X/VX437X Master/Slave Scanner Module

instrument driver. The functions are listed in alphabetical order with

a description of the function, C syntax of the function, a description

of each parameter, and possible error codes.

NOTE:

Throughout this manual and instrument driver, the Master Scanner Modules

are referred to as tkvx33X, references to slaves refer to the Slave Scanner

Modules.

________________________________________________________________________________

Function Tree Layout:

TK_VX433X Scanner Module            Function Name:

Initialize                        tkvx433X_init

Configure

Join Module                    tkvx433X_joinmodule

Switch Relays                  tkvx433X_switchrelays

Change Delay Times             tkvx433X_changedelaytimes

Set Interrupts                 tkvx433X_set_interrupts

Set Triggers                   tkvx433X_set_triggers

Program Routines

Input Program                  tkvx433X_input_program

End Program                    tkvx433X_end_program

Run Program                    tkvx433X_run_program

Pause Program                  tkvx433X_pause_program

Continue/step Program          tkvx433X_step_program

Low Level Routines

Set Time Out                   tkvx433X_set_timeout

Reset                          tkvx433X_reset

Wait                           tkvx433X_wait

List Error                     tkvx433X_list_error

Close                             tkvx433X_close

________________________________________________________________________________

Instrument Name: TK_VX433X Master Scanner Module

Description:     This instrument module provides programming support for

the TK_VX433X.  The module is divided into the following

functions:

Functions/Classes:

(1) Initialize

Initialize the instrument and set to default

configuration.

(2) Configure

This routines allow the control of the

scanner system by allowing to set different

delays for the different modules, triggers,

interrupt generation, close relay commands,

etc.

(2a) Join Module

Joins or disjoins the A and B sections of

any of the modules making up the system.

(2b) Switch Relays

Opens or closes any relay in any module.

(2c) Change Delay Times

Sets the time it takes for the instrument

to process another command after a close

or open relay command.

(2d) Set Interrupts

Sets the instrument to interrupt its VXI

commander with a Request True interrupt

for a variety of conditions.

(2e) Set Triggers

Disables or defines a specific VXIbus TTL

trigger line for trigger in and trigger

out commands and conditions.

(3) Program Routines

This routines allow programming the instrument

module.  The program that the module can

store is a command sequence using any of the

commands and routines in this instrument

driver.  This program can then be executed

using the continue/step function.

(3a) Input Program

Sets the instrument in Program Mode.

(3b) End Program

Terminates the Program Input Mode.

(3c) Run Program

Starts execution of the program in the

instrument's memory.

(3d) Pause Program

Pauses Program Mode.

(3e) Continue/step Program

Causes the instrument to continue after

it is pauses or to execute a desired

number of steps.

(4) Low Level Routines

This routines perform low level functions

such as reseting the instrument to its

power-up conditions, setting a wait time

within the instrument, and listing errors

from the instrument.

(4a) Set Time Out

Sets the time out for the instrument card

being used (GPIB or VXI).

(4b) Reset

Resets the instrument to its power-up

conditions.

(4c) Wait

Causes the instrument to wait a certain

number of milliseconds before processing

the next command.

(4d) List Errors

Queries the instrument to see if any

errors have ocurred.

(5) Close

Take the instrument offline.

________________________________________________________________________________

Following functions are in alphabetical order.

________________________________________________________________________________

Function Name      : tkvx433X_changedelaytimes

C Syntax           : int tkvx433X_changedelaytimes(Instrument_ID, Function,

Module_Number, Delay_millisec);

int Instrument_ID;

int Function;

int Module_Number;

long Delay_millisec;

Purpose            : This routine is able to perform any of the following

actions:

-  Set the time a specific module

will delay accepting another

command after receiving a

command to close or open a

relay.

-  Set the time that all modules

will delay accepting another

command after receiving a

command to close or open a

relay.

If another command is send to the

instrument before the set delay

time has expired, the platform

being used (GPIB or VXI) will have

to have a sufficiently long time

out set to be able to keep this

next command until the instrument is

able to accept it.  The Set Time

Out Routine can be used to adjust

this time out.

Parameter List     : Instrument ID

Description        : Accepts the Instrument ID returned by the Initialize

function to select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Function

0 - opentime (Default)

1 - opentime all

2 - closetime

3 - closetimeall

Description:   Specifies one of the following

actions:

-  Change the open or close delay

times of a specific module.

-  Change the open or close delay

times of all the modules to

the same time.

Variable Type: Integer

Valid Range:   0 - change the open delay time for

relays of a specific module.

1 - change the open delay times

for the relays in all modules.

2 - change the close delay time

for relays of a specific

module.

3 - change the close delay times

for the relays in all modules.

Module Number

Description:   Specifies the module whose delay

time is to be changed. This

parameter is ignored when the

functions "opentime all" or

"closetime all" are selected.

Variable Type: Integer

Valid Range:   The instrument is module #0. The

slave immediately to the right is

module #1, the next slave is

module #2, and so on.

Note:          The instrument can control up to

11 slave modules.  Each module that

has a slave to its right must have

its Local Bus Out Enable switches

enabled.

Delay (millisec)

Description:   Specifies the new value to which

the delay time is to be changed.

Variable Type: Integer

Valid Range:   Minimum value = 1000 millisec

Maximum value = 32767  millisec

= 32.767 sec

Error Codes:

0   Success

-1   Instrument ID out of range

-2   Function number out of range

-3   Module number out of range

-4   Delay time out of range

230   Error writing to instrument

232   Instrument not initialized

________________________________________________________________________________

Function Name      : tkvx433X_close

C Syntax           : int tkvx433X_close(Instrument_ID);

int Instrument_ID;

Purpose            : The close routine performs the following operations:

-  removes the entry for the

instrument from the Instrument

Table.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned by the Initialize

function to select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Error Codes:

0   Success

-1   Instrument ID out of range

221   Unable to close device

232   Device not initialized

________________________________________________________________________________

Function Name      : tkvx433X_end_program

C Syntax           : int tkvx433X_end_program(Instrument_ID);

int Instrument_ID;

Purpose            : This routine terminates the Input Program command.

Parameter List     : Instrument ID

Description        : Accepts the Instrument ID returned by the Initialize

function to select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Error Codes:

0   Success

-1   Instrument ID out of range

230   Error writing to instrument

232   Device not initialized

________________________________________________________________________________

Function Name      : tkvx433X_init

C Syntax           : int tkvx433X_init(Logical_Address, Interface,

Instrument_ID);

int Logical_Address;

int Interface;

int *Instrument_ID;

Purpose            : This routine performs the following initialization

actions:

-  Opens the instrument by

storing information about the

instrument in the Instrument

Table.

-  Initializes the VXI interface

(NI-VXI or NI-GPIB)

accordingly.

-  Performs an identification

query on the Instrument.

-  Sends initialization commands

to the instrument.

-  Returns an Instrument ID which

is used to differentiate

between instruments of the same

model type.  This value will be

used to identify the instrument

in subsequent calls.

-  Counts the number of modules

and the number of relays per

module present in the system.

Parameter List     : Logical Address

Description        : Specifies the Logical Address of the VXI instrument.

This value must corrrespond to the current internal

setting of the instrument.

Variable Type:  Integer

Valid Range:    0 - 255

Interface

1 - NI-VXI (Default)

2 - NI-GPIB

Description:   Specifies which protocol is used

to communicate with the VXI

mainframe.

NI-VXI should be selected for

MC-MXI, AT-MXI, or VXIpc-386.

NI-GPIB should be selected for

the GPIB-VXI.

Variable Type: Integer

Valid Range:   1 - NI-VXI

2 - NI-GPIB

Instrument ID

Description:   Returns an Instrument ID that is

used in all subsequent function

calls to select the instrument at

the address specified.

If more than one instrument of the

same model type is used, this ID

will be used to differentiate

between them. The first instrument

of this model type initialized

will be assigned an Instrument ID

of 1.

Variable Type: Integer

Scanner Master Type

Description:   Specifies whether the Master

Scanner Module to be initialized

is a tkvx4332 or tkvx4334 so the

correct identification query can

be issued.

Variable Type: Integer

Valid Range:   0 -  tkvx4332

1 -  tkvx4334

Error Codes:

0   Success

-1   Logical Address out of range

-2   Interface out of range

220   Unable to open instrument

221   Unable to close instrument

222   Unable to initialize VXI Library

223   Instrument identification query failed

230   Error writing to instrument

231   Error reading from instrument

________________________________________________________________________________

Function Name      : tkvx433X_input_program

C Syntax           : int tkvx433X_input_program(Instrument_ID);

int Instrument_ID;

Purpose            : This routine causes the instrument to store all

commands received after the Input Program command into

its memory for execution later.

Note:          An Input Program command is not valid

as part of a program definition (should

not appear between an Input Program and

an End Program command).

Parameter List     : Instrument ID

Description        : Accepts the Instrument ID returned by the Initialize

function to select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Error Codes:

0   Success

-1   Instrument ID out of range

230   Error writing to instrument

232   Device not initialized

________________________________________________________________________________

Function Name      : tkvx433X_joinmodule

C Syntax           : int tkvx433X_joinmodule(Instrument_ID, Module_Number,

Function);

int Instrument_ID;

int Module_Number;

int Function;

Purpose            : This routine is able to join or disjoin the sections A

and B of any of the modules present.

Note:          Performing a Join or Disjoin command on

any of the modules in the scanning

system will open all closed relays in

all modules, not only in the module

being joined or disjoined. When a

module is joined, closing a relay in

its A section will automatically open

any closed relay in either section A or

B.

Parameter List     : Instrument ID

Description        : Accepts the Instrument ID returned by the Initialize

function to select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Module Number

Description:   Specifies the module whose

sections A and B should be

joined or disjoined.

Variable Type: Integer

Valid Range:   The instrument is module #0. The

slave immediately to the right is

module #1, the next slave is module

#2, and so on.

Note:          The instrument can control up to

11 slave modules. Each module that

has a slave to its right must have

its Local Bus Out Enable switches

enabled.

Function

1 - join

0 - disjoin (Default)

Description:   Specifies whether the module will

be joined or disjoined.

Variable Type: Integer

Valid Range:   0 - disjoin module

1 - join module

Error Codes:

0   Success

-1   Instrument ID out of range

-2   Module number out of range

-3   Function number out of range

230   Error writing to instrument

232   Device not initialized

________________________________________________________________________________

Function Name      : tkvx433X_list_error

C Syntax           : int tkvx433X_list_error(Instrument_ID, Error_Message);

int Instrument_ID;

char *Error_Message;

Purpose            : This routine queries the error queue of the instrument and

it will return an error message if an error has

occurred since the last time this function was called.

The error message queue has a limit of 10 messages

therefore, if an overflow occurs, new messages will be

lost but an overflow error will be added to the queue.

One error message is returned per call to this routine.

Note:           A List Errors command is not valid

as part of a program definition

(should not appear between an

Input Program and End Program

command).

Parameter List     : Instrument ID

Description        : Accepts the Instrument ID returned by the Initialize

function to select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Error Message      : Returns an error message per time it is called. Error

messages are thrown away as they are read. If the

returned value is blank, no errors have occurred.  See

the appendixes of the Scanner Module Operating

Manual for a list of the possible error messages.

Variable Type: String

Length:        50

Error Codes:

0   Success

-1   Instrument ID out of range

230   Error writing to instrument

231   Error writing from instrument

232   Device not initialized

________________________________________________________________________________

Function Name      : tkvx433X_pause_program

C Syntax           : int tkvx433X_pause_program(Instrument_ID);

int Instrument_ID;

Purpose            : This routine causes an executing program to be paused.

Using the Continue/step Program command will continue

execution of the program.

Note:          A Pause Program command is only

valid as part of a program

definition (is only valid when

included between an Input Program

and an End Program command).

Parameter List     : Instrument ID

Description        :  Accepts the Instrument ID returned by the Initialize

function to select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Error Codes:

0   Success

-1   Instrument ID out of range

230   Error writing to instrument

232   Device not initialized

________________________________________________________________________________

Function Name      : tkvx433X_reset

C Syntax           : int tkvx433X_reset(Instrument_ID);

int Instrument_ID;

Purpose            : This routine resets the instrument to its power-up

condition:

-  All relays are opened including

the join relays.

-  All relay delay values are set

to their default values.

-  All program information is

lost (ie. any program stored in

the instrument's memory using

the Program Routines will be

erased).

Note:           A Reset command is not valid

as part of a program definition

(should not appear between an

Input Program and End Program

command).

Parameter List     : Instrument ID

Description        : Accepts the Instrument ID returned by the Initialize

function to select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Error Codes:

0   Success

-1   Instrument ID out of range

230   Error writing to instrument

232   Device not initialized

________________________________________________________________________________

Function Name      : tkvx433X_run_program

C Syntax           : int tkvx433X_run_program(Instrument_ID, Repetitions);

int Instrument_ID;

int Repetitions;

Purpose            : This routine causes the instrument to execute the

sequence of commands stored in it with the Input

Program command.

Note1:         If repetitions is set to 0, the

program will be repeated until

stopped.  The program will be

stopped and any closed relays will

be opened by one of the following

events:

- a VXIbus system reset.

- a VXI Abort Normal Operation.

- a VXI End Normal Operation.

- any hardware or sytax error.

Note2:         A Run Program command is not

valid as part of a program

definition (should not appear

between Input Program and End

Program command).

Parameter List     : Instrument ID

Description        : Accepts the Instrument ID returned by the Initialize

function to select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Repetitions

Description:   Specifies the number of times that

a program will be executed.

Variable Type: Integer

Valid Range:   0 to 32767

Note1:         If repetitions is set to 0, the

program will be repeated until

stopped.  The program will be

stopped and any closed relays will

be opened

Note2:         If another command is sent to the

instrument, the program is

terminated at the end of the

currently executing step and the

new command is executed.  The

program is then in a pause mode

and may be continued with the

"continue/step program" command.

Error Codes:

0   Success

-1   Instrument ID out of range

-2   Repetitions out of range

230   Error writing to instrument

232   Device not initialized

________________________________________________________________________________

Function Name      : tkvx433X_set_interrupts

C Syntax           : int tkvx433X_set_interrupts(Instrument_ID, Type,

Interrupt);

int Instrument_ID;

int Type;

int Interrupt;

Purpose            : This routine is able to perform several types of

interrupts commands which enable or disable generating

a Request True interrupt on the VXIbus on the

ocurrence of various events.

Note:          If this set of commands is used,

the user should take care of

acknowledging the interrupts.

Parameter List     : Instrument ID

Description        :   Accepts the Instrument ID returned by the Initialize

function to select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Type

Description:   Specifies one of the following

types of interrupts:

-  User1: To cause a Request True

Interrupt to occur in the

VXIbus when the instrument

receives this command.

-  User2: To cause a Request True

Interrupt to occur in the

VXIbus when the instrument

receives this command.

-  Relay Closed: To enable or

disable a Request True

Interrupt after a relay has

been closed and the relay close

delay time has expired.

-  Program Done: To enable or

disable a Request True

Interrupt at the end of a

program.

-  Error: To enable or disable a

Request True Interrupt for any

of three error conditions:

-  a relay failed to open after

the relay open delay.

-  a relay failed to close

after the relay close delay.

-  a syntax error was detected.

-  Trigger_in: To enable or

disable Request True Interrupt

when a trigger is detected.

Variable Type: Integer

Valid Range:   0 - User1 interrupt  (Default)

1 - User2 interrupt

2 - Relay Closed interrupt

3 - Program Done interrupt

4 - Error interrupt

5 - Trigger_in interrupt

Interrupt

Description:   Specifies whether the interrupt is

to be enabled or disabled.  This

parameter is ignored if User1 or

User2 interrupts are selected.

Variable Type: Integer

Valid Range:   0 - Interrupt disable

1 - Interrupt enable (Default)

Error Codes:

0   Success

-1   Instrument ID out of range

-2   Interrupt Type out of range

-3   Interrupt enable/disable function out

of range

230   Error writing to instrument

232   Device not initialized

________________________________________________________________________________

Function Name      : tkvx433X_set_timeout

C Syntax           : int tkvx433X_set_timeout(Instrument_ID, Time_Out_Code);

int Instrument_ID;

int Time_Out_Code;

Purpose            : This routine sets the time out for the platform being

used (GPIB or VXI). See note below.

Note:          The way the instrument handles

relay switching with timing is

the following:

Say relay 1 in module 0 has been

closed,  then, if relay 2 is to be

closed, relay 1 must be opened

first.  Therefore the time it will

take for relay 2 to be closed is

the opentime of relay 1.

Also, after closing relay 2, a

time equal to the closetime of

relay 2 must pass before the

instrument can accept another

command for execution.

In addition, when using the Wait

Routine,the platform's time out

should be greater than the wait

time so that commands are not

lost while the wait time is in

effect.

So if the time out of the

platform being used (GPIB or VXI)

is less than any of these times,

some of the commands will not be

executed since the platform will

suffer a time out and commands may

be lost.O

Parameter List     : Instrument ID

Description        : Accepts the Instrument ID returned by the Initialize

function to select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Time Out Code

Description:   Specifies the Time Out of the

interface being used. The value

entered represents a time out

which depends on the interface

used.  Refer to the following

table of codes to decide which

Time Out Code to enter.

_____TIME OUT____

Time Out Code        GPIB          VXI

0           0 (disabled)       0

1                10 us         1 ms

2                30 us         1 ms

3               100 us         1 ms

4               300 us         1 ms

5                 1 ms         1 ms

6                 3 ms         3 ms

7                10 ms        10 ms

8                30 ms        30 ms

9               100 ms       100 ms

10               300 ms       300 ms

11                 1 s          1 s

12                 3 s          3 s

13                10 s         10 s

14                30 s         30 s

15               100 s        100 s

16               300 s        300 s

17              1000 s       1000 s

Note:          The 1 sec time outs for the VXI

case (Time Out Codes 1 through 5)

are approximate.

Variable Type: Integer

Valid Range:   0 to 17.

Error Codes:

0   Success

-1   Instrument ID out of range

-2   Time Out Code out of range

239   Error in configuring timeout

________________________________________________________________________________

Function Name      : tkvx433X_set_triggers

C Syntax           : int tkvx433X_set_triggers(Instrument_ID, Trigger_Type,

Trigger_Line_Number);

int Instrument_ID;

int Trigger_Type;

int Trigger_Line_Number;

Purpose            : This routine is able to disable or define a specific

VXIbus TTL trigger line for trigger in and trigger out

commands and conditions.

Parameter List     : Instrument ID

Description        : Accepts the Instrument ID returned by the Initialize

function to select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Trigger Type

Description:   Specifies one of the following

trigger-related events to occur:

-  T_in : selects the VXIbus

trigger line specified in

T_Line Number to be monitored

by the instrument when executing

a T_Wait trigger command.

-  T_in disabled: disables the

T_in trigger command.

-  T_out: selects the VXIbus TTL

trigger line specified in

T_Line Number to be triggered

when a relay switch is closed

or a Trigger1 or Trigger2

command is received.

-  T_out disable: disables the

T_out trigger command.

-  Trigger1: triggers the VXIbus

TTL trigger line specified in

T_Line Number.

-  Trigger2: triggers the VXIbus

TTL trigger line selected by

the T_out command. In this

case, the next parameter,

T_Line Number, is not used.

-  T_Wait: causes the instrument to

wait for a VXIbus TTL trigger

line pulse on the line selected

by the T_in command before

processing any additional

commands.

Variable Type: Integer

Valid Range:   0 - Trigger_in (Default)

1 - Trigger_in disabled

2 - Trigger_out

3 - Trigger_out disabled

4 - Trigger1

5 - Trigger2

6 - Trigger_wait

Trigger Line Number

Description:   Specifies the VXIbus trigger line

that will be included with the

trigger command specified under

the second parameter (Trigger Type).

Variable Type: Integer

Valid Range:   0 to 7

Minimum Value : 0

Maximum Value : 7

Increment     : 1

Default Value : 0

Error Codes:

0   Success

-1   Instrument ID out of range

-2   Trigger Type out of range

-3   T_Line Number out of range

230   Error writing to instrument

232   Device not initialized

________________________________________________________________________________

Function Name      : tkvx433X_step_program

C Syntax           : int tkvx433X_step_program(Instrument_ID, Steps);

int Instrument_ID;

int Steps;

Purpose            : This routine causes the instrument to execute the

program stored in its memory one or more steps at a

time or to continue a paused program.

Note:          A Continue/step Program command is

not valid as part of a program

definition(should not appear

between Input Program and End

Program command).

Parameter List     : Instrument ID

Description        : Accepts the Instrument ID returned by the Initialize

function to select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Steps

Description:   Specifies the number of steps to

be executed within a program

starting from the beginning of the

program or from where the program

is currently paused.  Setting

steps to 0 sets the number of

steps to infinity.  See Note1

below.

Variable Type: Integer

Valid Range:   0 to 32767

Note1:         Setting Steps to 0 can result in

one of the following actions to be

taken:

-  If the program is in pause

mode, it will continue to

execute from the point it was

paused. Execution will continue

until the original loop count

is completed, another "pause

program" command is

encountered,or another command

is sent to the instrument.

Note3:         Setting Steps to a value greater

than the number of steps left to

execute will not extend the

program.

Error Codes:

0   Success

-1   Instrument ID out of range

-2   Steps out of range

230   Error writing to instrument

232   Device not initialized

________________________________________________________________________________

Function Name      : tkvx433X_switchrelays

C Syntax           : int tkvx433X_switchrelays(Instrument_ID, Module_Number,

Function, Relay_Number);

int Instrument_ID;

int Module_Number;

int Function;

int Relay_Number;

Purpose            : This routine is able to perform any of the following

actions:

-  Open or close any relay

on any of the modules.

-  Open relays in all modules

except the relays joining the A

and B sections of each module,

which are not affected.

Parameter List     : Instrument ID

Description        : Accepts the Instrument ID returned by the Initialize

function to select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Module Number

Description:   Specifies the module where the

relay to be switched is to be

found.  This parameter is ignored

if function "open all" is selected.

Variable Type: Integer

Valid Range:   The instrument is module #0. The

slave immediately to the right is

module #1, the next slave is

module #2 and so on.

Note:          The instrument can control up to 11

slave modules.  Each module that

has a slave to its right must have

its Local Bus Out Enable switches

enabled.

Function

Description:   Specifies one of the following

actions:

-  Close or open a specific relay.

-  Open the relays in all modules.

Variable Type: Integer

Valid Range:   0 - open a specific relay (Default)

1 - open all relays in all modules

but join relays are not

affected

2 - close a specific relay

Note:          The Open All command does not

affect the relays that join the A

and B sections together.

sections together

Relay Number

Description:   Specifies the relay to be opened

or closed. This parameter is

ignored if function "open all" is

selected.

Variable Type: Integer

Valid Range:   The number of relays in a module

will depend on its serial number.

Read the manual of each module to

find how many relays it contains.

The valid relay numbers are then

0 to (number of relays -1).

The tkvx332 module for example, is

made up of two sections (A and B),

each having 20 channels therefore

the valid range of relay numbers

is 0 to 39.

Error Codes:

0   Success

-1   Instrument ID out of range

-2   Module number out of range

-3   Function number out of range

-4   Relay number out of range

230   Error writing to instrument

232   Device not initialized

________________________________________________________________________________

Function Name      : tkvx433X_wait

C Syntax           : int tkvx433X_wait(Instrument_ID, Wait_Time_millisec);

int Instrument_ID;

int Wait_Time_millisec;

Purpose            : This routine causes the instrument to wait a specified

amount of time before accepting another command.

Note:          To prevent the interface being

used (GPIB or VXI) from suffering

a time out, the interface's time

out should be adjusted using the

Set Time Out function.

Parameter List     : Instrument ID

Description        : Accepts the Instrument ID returned by the Initialize

function to select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Wait Time (millisec)

Description:   Specifies the time the instrument

will wait before accepting the next

command.

Variable Type: Integer

Valid Range:   Minimum value = 0 millisec

Maximum value = 32767 millisec

= 32.767 sec

Error Codes:

0   Success

-1   Instrument ID out of range

-2   Wait time out of range

230   Error writing to instrument

232   Device not initialized