D-1010-N

430-450 MHz Amplifier

The MIRAGE D-1010-N represents the latest in 430-450 MHz amplifiers.  It incorporates features that make it the most versatile and useful amplifier available today.  The D-1010-N will amplify FM, SSB, and CW signals.  It has variable SSB delay, remote Keying, over-temperature protection, and remote control operation when using the optional RC-1, Remote.

SPECIFICATIONS:

FREQUENCY RANGE	430-450 MHz

POWER	INPUT:  3-15 watts

OUTPUT:	Min of 100 watts with 10W in

MODES	FM, SSB, CW, ATV with modifications

DC POWER	13.6 VDC @ 20 amps nominal

FUSE	35 AMP (internally mounted)

IMPENDANCE	50 Ohm input and output

DUTY CYCLE	Intermittent-internal overtemp

SIZE	12" x 3" x 5-1/2"

WEIGHT	5 LBS

INSTALLATION:

The D-1010-N may be mounted using the brackets supplied.  The D-1010-N must have adequate ventilation for the finned heat-sink.  Use #8 or larger wire to connect the D-1010-N to the battery is recommended.  RG-8U, or the equivalent, should be used between the D-1010-N and the antenna.  The antenna should be matched to better than 1.5:1 for best performance.  SWR greater than 3:1 may cause damage to the amplifier voiding and warranty.

FRONT PANEL FUNCTIONS:

POWER ON/OFF	This switch turns the power amplifier ON and OFF.

SSB/FM	Selects the relay time-delay for the mode of operation.  In either the SSB or FM position, the amplifier is still biased for linear operation.

LED (Power/On)	This LED will go out if the amplifier overheats.  It will come back on when the amplifier cools.

LED (Tx)	This LED will light during transmitting periods.

REAR PANEL FUNCTIONS:

"RADIO"	This connects to the transmitter or transceiver.

"ANT"	This connects to the antenna.

RCA Jack	This connector can be used to ground key the transmit relay if the input signal is too weak to activate the RF sensing circuit which normally handles the function.

6-Pin Remote Connector	The RC-1 Remote Control Head connects here.

precautions:

OUTPUT POWER	The D-1010-N puts out enough power to cause heating of the antenna coax.  RG-8 or equivalent is recommended between the amplifier and antenna.

HEAT SINK TEMPERATURE	Along with high power output, comes the possibility of high heat sink temperatures.  The D-1010-N must be mounted where air can circulate over the heat sink.  The D-1010-N has a built-in thermostat that will turn it "OFF" at 170 degrees F.  The amplifier will not come back on until the temperature drops to 140 degrees F.

INPUT POWER	Input power should not exceed 15 watts.  Higher power than this may cause failure of the input transistor.  This will VOID ANY WARRANTY.

SSB OPERATION	The D-1010-N can be overdriven causing the SSB signal to be distorted.  When using the D-1010-N on SSB, the drive level or the mic gain from the exciter should be adjusted for best "on the air" performance.

INTERNAL ADJUSTMENTS:

SSB DELAY	This allows the XMT relay delay time to be adjusted to various lengths.  Access to this adjustment is through the hole in the left side of the cover, behind the front panel. A small screwdriver can be used to make the adjustment.

RF ADJUSTMENTS	There are no internal adjustments dealing with the RF amplifier board.  Tuning of the RF amplifier is preset at the factory by our technicians.

FUSE	A 35-amp fuse is located under the cover on the PC board.  Should the fuse "blow," determine the cause and use the exact type in the unit as a replacement.

"ATV MOD"	Remove .1 mfd cap and 10 ohm resistor which crosses over both final power transistors and connects each base to its collect lead.  These cap/resistor networks are designed to eliminate low frequency oscillations and will distort ATV signals.

IN CASE OF DIFFICULTY:

1.	Check for loose antenna or B+ connections.

2	Check SWR of antenna.

3.	Look for bad or lossy coax.

Technical Assistance:

If you have any problem with this unit first check the appropriate section of this manual. If the manual does not reference your problem or your problem is not solved by reading the manual you may call MIRAGE at 601-323-8287.  You will be best helped if you have your unit, manual and all information on your station handy so you can answer any questions the technicians may ask.

You can also send questions by mail to MIRAGE, 921 HWY 25 South, Starkville, MS 39759 or by Fax to 601-323-6551. Send a complete description of your problem, an explanation of exactly how you are using your unit, and a complete description of your station.