# Q-MAC Fixed Broadband Dipole Antenna – Multi-Wire (QM7002)

## Description:

The QM7002 Fixed Broadband Dipole Multi-wire Antenna offers improved efficiency in a fixed base station application.

The QM7002 is an antenna designed and built for long life.  It is constructed with multi-strand stainless steel radiation elements that guarantee high resistance to corrosion.  This means that the QM7002 antenna will significantly outlast similar antennas.

The antenna can be mounted horizontally between two supports, or as an inverted V with one high central support and two lower end supports.

Note that the QM7002 antenna is supplied complete with 30m of RG-58 coaxial cable.  No mounting structures are included.

Specifications:

| ### ELECTRICAL Frequency Range: 2 – 30MHz Impedance: 50Ω Power Input: 130W (data) 250W PEP (voice) Input Connector: UHF Socket | ### MECHANICAL Shipping weight: 4.5kg Recommended mast height: 10m (minimum) Mast spacing: 30m (horizontal configuration) 18m (V configuration) Antenna Length: 27m Radiating elements: Stainless steel (multi-strand) *Wind rating: 210km/hr *In accordance with AS1170.2 – 1989 loading code – wind loads. |
| --- | --- |

| Illustration above shows one possible option for mounting the QM7002. This being the horizontal configuration. | Graph above shows an SWR plot for a QM7002 antenna from 2 – 30MHz |
| --- | --- |