# Q-MAC Fold-up Solar Panel Charger (QM8021)

## Description:

Suits Rechargeable Battery (7Ah) - 10.8 Watt rated power, complete with extension cable and appropriate connectors. Comprises five solar panels which fold into a compact canvas pouch.

## Features:

High efficiency single crystal solar cells provide high light to energy conversion efficiency.  Cells are textured and have an anti-reflection coating.  Circuits are laminated between layers of ethylene vinyl acetate (EVA) for moisture resistance, UV stability and electrical isolation.

A blocking diode is built into each circuit.

Durable, anti-reflective, polymer front sheet is used for environmental protection, resistance to abrasion, tears and punctures.

Rigid anodised aluminium backplate with a black anti-reflective coating.

Interconnect wiring is protected by special stress-relieved internal clips that help preserve laminate integrity.

Stiffener rods screw together and insert into the panel for self-supported operation.

Weather-resistant canvas case.

One year limited warranty on power output.

| ## Specifications: |  |  |  |
| --- | --- | --- | --- |
| (NB. May also be configured for 24V) At 12 Volts: |  |  |  |
| Rated Power | 10.8 Watts |  |  |
| Current (typical at load) | 0.640 Amps |  |  |
| Voltage (typical at load) | 16.8 Volts |  |  |
| Short Circuit Current (typical) | 0.680 Amps |  |  |
| Open Circuit (typical) | 20.8 Volts |  |  |
| Weight | 1.25 kg (2.75 lb) |  |  |
| Dimensions - |  |  |  |
| Deployed: |  | Folded Panel: |  |
| Length | 850 mm (33.5”) | Length | 241 mm (9.5”) |
| Width | 225 mm (9”) | Width | 127 mm (5”) |
| Depth | 7 mm (0.25”) | Depth | 70 mm (2.75”) |

Power specifications are at standard test conditions of 1000W/m2 solar irradiance.  25oC cell temperature and solar spectral irradiance per ASTM E892.

Specifications are subject to change without notice.