Burster Resistomat Type 2304+2305

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for Burster Resistomat Type 2304+2305.

It contains functions for opening, configuring, taking measurements from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

For GPIB instrument drivers:

-  the instrument is connected to the GPIB.

-  the GPIB address supplied to the initialize function must match the

GPIB address of the instrument.

For VXI instrument drivers:

-  the instrument is installed in the VXI mainframe and you are

using one of the following controller options:

Embedded controller

MXI

MXI2

GPIB-VXI

-  the logical address supplied to the initialize function must

match the logical address of the instrument.

For RS-232 instrument drivers:

-  the instrument is connected to the RS-232 interface.

-  the COM port, baud rate, parity, and timeout supplied to the

initialize function must match the settings of the instrument.

________________________________________________________________________________

Error and Status Information:

Each function in this instrument driver returns a status code that

either indicates success or describes an error or warning condition.

Your program should examine the status code from each call to an

instrument driver function to determine if an error occurred.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

The description of each instrument driver function lists possible error

codes and their meanings

________________________________________________________________________________

How To Use This Document:

Use this document as a programming reference manual.

It describes each function in the

Burster Resistomat Type 2304+2305

instrument.  The functions appear in alphabetical order, with a description

of the function and its Visual Basic syntax, a description of each parameter,

and a list of possible error codes.

________________________________________________________________________________

Function Tree Layout:

Class/Panel Name:                    Function Name:

Initialize

Initialize GPIB                bp2304_init_gpib

Initialize Serial              bp2304_init_serial

Local (GPIB only)              bp2304_local_gpib

Configure Measurement

Measurement Sequence           bp2304_config_meas_sequence

Measurement Mode               bp2304_config_meas_mode

Measurement Current            bp2304_config_meas_current

Measurement Grounding          bp2304_config_meas_grounding

Measure

Measure Start                  bp2304_meas_start

Measure Read                   bp2304_meas_read

Measure Stop                   bp2304_meas_stop

Correction

Set Temperature Coeff.         bp2304_corr_set_tk

Correction State               bp2304_corr_state

Get Current Temperature        bp2304_get_temp

Comparator

Comparator Reset               bp2304_comp_reset

Comparator Limit               bp2304_comp_limit

Comparator Data                bp2304_get_comp_data

Classification

Class. Comparator Reset     bp2304_class_comp_reset

Class. Comparator Limit     bp2304_class_comp_limit

Class. Comparator Data      bp2304_get_class_comp_data

Cooling curve

C-Measurement State            bp2304_cool_state

C-Measurement Initialize       bp2304_cool_meas_init

C-Manual Cold Resistance       bp2304_cool_man_Rk

C-Measure Rcool Start          bp2304_cool_meas_Rk_start

C-Measure Read Rcool           bp2304_cool_read_RK

C-Measure Rcool Stop           bp2304_cool_meas_Rk_stop

C-Measure Temperature 1        bp2304_cool_meas_t1

C-Measure Read Temp. 1         bp2304_cool_read_t1

C-Measure Temperature 2        bp2304_cool_meas_t2

C-Measure Read Temp. 2         bp2304_cool_read_t2

C-Removal of Load              bp2304_cool_removal_load

C-Measure Start                bp2304_cool_meas_start

C-Measure Read                 bp2304_cool_read_meas

C-Measure Stop                 bp2304_cool_meas_stop

Display

Display Mode                   bp2304_disp_mode

Display Enable                 bp2304_disp_enable

Display Contrast               bp2304_disp_contrast

System

Set Output                     bp2304_system_output

System Error                   bp2304_system_error

System Keyboard                bp2304_system_keyboard

System Buzzer                  bp2304_system_buzzer

SCPI

Send a SCPI-Message            bp2304_send_scpi

Receive a SCPI-Message         bp2304_receive_scpi

Close                             bp2304_close

________________________________________________________________________________

Burster Resistomat Type 2304+2305

Instrument Name:Burster Resistomat Typ 2304+2305

Description:   This instrument module provides

RS232 support for  Resistomat

Typ 2304+2305 from Burster.

The module contains the following

functions:

(1) Initialize Functions

Initialize the GPIB

Initialize the RS232-Port

(2) Configure  the measurement - Sequence

- Mode

- Current

- Grounding

(3) Measurement Functions

Start Measurement

Readt Measurement value

Stop Measurement

(4) Correction Functions

Set the temperature Coeffitient

Correction state

Get the current temperature

(5) Comparartor Functions

Comparator statistic data reset

Comparator Limits

Get Comparator statistic data

Class. Comparator statistic data reset

Class. Comparator Limits

Class. Get Comparator statistic data

(6) Cooling curve Functions

(7) Display Functions

Display Mode

Display Enable

Display Contrast

(8) System Functions

Set output pin 3+21 from I/O jack

Read the system error

Control the keyboard

Control the buzzer

(9) Close the instrument

*********

________________________________________________________________________________

The following functions are in alphabetical order.

________________________________________________________________________________

bp2304_class_comp_limit

Function bp2304_class_comp_limit& (ByVal instrumentID&, ByVal comparatorState&, ByVal barDisplay&, limit18#, ByVal nominal#, ByVal unit&)

Purpose

Panel Name:    Class. Comparator Limit

Descrpition:   Set the individual classes of the

classify function

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal comparatorState&

Control Name:  Comparator State

Description:   This parameter activate or

deactivate the sort function.

Variable Type: Integer

Valid Range:   0 = OFF

1 = ON

ByVal barDisplay&

Control Name:  Display Enable

Description:   This parameter activate or

deactivate the LCD-display.

(during the remote operation)

Variable Type: Integer

Valid Range:   0 = OFF  => LCD display is OFF

1 = ON   => LCD display is ON

limit18#

Control Name:  Limit 1-8

Description:   This parameter select the eight

limiting  value of the individual

classes of the classify function.

Variable Type: Double Array of 8 elements (0-7)

ByVal nominal#

Control Name:  Nominal Value

Description:   This parameter is specified value

of the comparator function.

This pararmeter can be send with

percent as unit or the unit is

the selected display unit.

Variable Type: Double

ByVal unit&

Control Name:  Unit

Description:   This parameter select the unit

of the comparator function.

The unit can be percent or the

the selected display unit.

Variable Type: Integer

Valid Range:   0 = Display unit

1 = Percent

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

-2     Invalid Comparator State

-3     Invalid Bar Display

-4     Invalid Unit

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

System Device Errors :

10   Cable rupture on measuring resistor

20   Amplifier overdriven

30   Overrange

40   Temperature of device too high

50   Error occurred during measurement

60   Current cannot be controled

-100  Command  Error

(An invalid command was send)

-105  Get Not Allowed

(A Get command was sent within a

command)

-110  Command  Header Error

( Command with invalid command

heading)

-200  Execuation Error

(Due to a certain state of the device,

a command could not be executed)

-220  Parameter Error

(A command with in incorrect parameter

was sent)

-350  Error Buffer Overflow

-400  Query Error

(Device was polled but not data was

present)

-410  Query Interrupted

(Device was interrupted with out

having send a complete response)

-420  Query Unterminated

(Device was polled without having

received a complete command)

________________________________________________________________________________

bp2304_class_comp_reset

Function bp2304_class_comp_reset& (ByVal instrumentID&)

Purpose

Panel Name:    Class. Comparator Reset

Descrpition:   Reset the classification statistic

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_close

Function bp2304_close& (ByVal instrumentID&)

Purpose

Panel Name :  Close

Descrpition:  Close the instrument

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_comp_limit

Function bp2304_comp_limit& (ByVal instrumentID&, ByVal comparatorState&, ByVal barDisplay&, ByVal lowerLimit#, ByVal upperLimit#, ByVal nominal#, ByVal unit&)

Purpose

Panel Name:    Comparator Limit

Descrpition:   Set the comparator high and low

limit

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal comparatorState&

Control Name:  Comparator State

Description:   This parameter activate or

deactivate the sort function.

Variable Type: Integer

Valid Range:   0 = OFF

1 = ON

ByVal barDisplay&

Control Name:  Display Enable

Description:   This parameter activate or

deactivate the LCD-display.

(during the remote operation)

Variable Type: Integer

Valid Range:   0 = OFF  => LCD display is OFF

1 = ON   => LCD display is ON

ByVal lowerLimit#

Control Name:  Lower Limit

Description:   This parameter select the lower

limit value for the comparator

function.

This pararmeter can be send with

percent as unit or the unit is

the selected display unit.

Variable Type: Double

ByVal upperLimit#

Control Name:  Upper Limit

Description:   This parameter select the upper

limit value for the comparator

function.

This pararmeter can be send with

percent as unit or the unit is

the selected display unit.

Variable Type: Double

ByVal nominal#

Control Name:  Nominal Value

Description:   This parameter is specified value

of the comparator function.

This pararmeter can be send with

percent as unit or the unit is

the selected display unit.

Variable Type: Double

ByVal unit&

Control Name:  Unit

Description:   This parameter select the unit

of the comparator function.

The unit can be percent or the

the selected display unit.

Variable Type: Integer

Valid Range:   0 = Display unit

1 = Percent

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

-2     Invalid Comparator State

-3     Invalid Display Bar

-4     Invalid Unit

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

System Device Errors :

10   Cable rupture on measuring resistor

20   Amplifier overdriven

30   Overrange

40   Temperature of device too high

50   Error occurred during measurement

60   Current cannot be controled

-100  Command  Error

(An invalid command was send)

-105  Get Not Allowed

(A Get command was sent within a

command)

-110  Command  Header Error

( Command with invalid command

heading)

-200  Execuation Error

(Due to a certain state of the device,

a command could not be executed)

-220  Parameter Error

(A command with in incorrect parameter

was sent)

-350  Error Buffer Overflow

-400  Query Error

(Device was polled but not data was

present)

-410  Query Interrupted

(Device was interrupted with out

having send a complete response)

-420  Query Unterminated

(Device was polled without having

received a complete command)

________________________________________________________________________________

bp2304_comp_reset

Function bp2304_comp_reset& (ByVal instrumentID&)

Purpose

Panel Name:    Comparator Reset

Descrpition:   Reset the comparator data's

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_config_meas_current

Function bp2304_config_meas_current& (ByVal instrumentID&, ByVal currentShape&, ByVal currentLevel&, ByVal loading&)

Purpose

Panel Name:    Initialize the measurement current

Descrpition:   Initializes the follow parameters:

- Current Level

- Current Shape

- Loading

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal currentShape&

Control Name:  Current Shape

Description: This parameter switch over between

unipolar and bipolar measurements.

Variable Type: Integer

Valid Range:   0 = Bipolar

1 = Unipolar

ByVal currentLevel&

Control Name: Current Level

Description: This parameter sets the measuring

current.

Variable Type: Integer

Valid Range:   0 = Small measuring current

1 = Medium measuring current

2 = Large measunring current

ByVal loading&

Control Name: Loading

Description: This parameter selects the load to

measured (mainly ohmic or mainly

indictive resistance).

Variable Type: Integer

Valid Range:   0 = mainly ohmic

1 = mainly inductive

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

-2     Invalid Current Shape

-3     Invalid Current Level

-4     Invalid Loading

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_config_meas_grounding

Function bp2304_config_meas_grounding& (ByVal instrumentID&, ByVal measurementGround&)

Purpose

Panel Name:    Initialize the measurement

Grounding

Descrpition:   Selected the potential fixing of

the measurement part

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal measurementGround&

Control Name:  User Adress

Description:   Selects the potential fixing of

the measurtment part.

Variable Type: Integer

Valid Range:   0  =  Internaly

1  =  Float

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

-2     Invalid Grounding Parameter

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_config_meas_mode

Function bp2304_config_meas_mode& (ByVal instrumentID&, ByVal measurementMode&, ByVal range&, ByVal resolution&)

Purpose

Panel Name:    Initialize the measurement mode

Descrpition:   Initializes the follow parameters:

- the measurement mode

- the measurement range

- the measurement resolution

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal measurementMode&

Control Name:  Measurement Mode

Description:   This parameter is meant for

changing between single and

repetition measurements.

Variable Type: Integer

Valid Range:   0 = Single

1 = Repetition

ByVal range&

Control Name:  Range

Description:   This parameter activates or de-

activates automatic measuremnet

range selection and selects the

desired measurement range.

Variable Type: Integer

Valid Range:   0 = Auto

1 = 200 uOhm (only used by 2304)

2 = 2   mOhm

3 = 20  mOhm

4 = 200 mOhm

5 = 2    Ohm

6 = 20   Ohm

7 = 200  Ohm

8 = 2   KOhm

9 = 20  KOhm

ByVal resolution&

Control Name:  Resolutionange

Description:   This parameter selects the

resolution of the measured value

display

Variable Type: Integer

Valid Range:   0 = low resolution

1 = medium resolution

2 = high resolution

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

-2     Invalid Meas. Mode

-3     Invalid Range

-4     Invalid Average

-5     Invalid Pause

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_config_meas_sequence

Function bp2304_config_meas_sequence& (ByVal instrumentID&, ByVal frequency&, ByVal time_base&, ByVal averaging&, ByVal measurementPause&)

Purpose

Panel Name:    Initialize the measurement

sequence

Descrpition:   Initializes the follow parameters:

- the nominal line frequency

- the measurement timebase

- the number of values for average

- the measurement pause

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal frequency&

Control Name:  Frequency

Description: This parameter syschronizes the

inte gation time of the  A/D-

converter with the nominal line

frequency at which the device is

operated.

Variable Type: Integer

Valid Range:   0 = 16.66 Hz

1 = 50    Hz

2 = 60    Hz

ByVal time_base&

Control Name: Timebase

Description: This parameter selects the measure-

ment time base,ie. the numberof line

periods over which the measurement

is carried out.

Variable Type: Integer

Valid Range:   1 - 9

ByVal averaging&

Control Name: Averaging

Description: This parameter selects the number of

measured values whose average is to

be determined.

Variable Type: Integer

Valid Range:   1 - 255

ByVal measurementPause&

Control Name: Measurement Pause

Description: This parameter selects the

measurement pause. This is the

waiting periode which allows the

circuit to settle after switchover

processes.

Variable Type: Integer

Valid Range:   1 - 255

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

-2     Invalid Frequenzy

-3     Invalid Timebase

-4     Invalid Average

-5     Invalid Pause

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_cool_man_Rk

Function bp2304_cool_man_Rk& (ByVal instrumentID&, ByVal manualRcoolOhm&)

Purpose

Panel Name : Manual Cold Resistance

Descrpition: Manual entry of the cold

resistance of the test unit.

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal manualRcoolOhm&

Control Name:  Manual Rcool

Description:   This parameter entry manual the

cold resistance.

Variable Type: Double

Valid Range:   0.0 - 999999 Ohm

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_cool_meas_init

Function bp2304_cool_meas_init& (ByVal instrumentID&, ByVal coolingMemoryClear&, ByVal delaysec#)

Purpose

Panel Name : Measurement Initialize

Descrpition: Initialize cooling curve measurement

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal coolingMemoryClear&

Control Name:  Cooling Memory Clear

Description:   This parameter is cleared the

cooling memory

Variable Type: Integer

Valid Range:   0 = Clear OFF

1 = Clear ON

ByVal delaysec#

Control Name:  Delay

Description:   This parameter is the interval

time between the cooling-curve

measurements.

Variable Type: Double

Valid Range:   0.0 - 99999.99 sec

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

-2     Invalid Memory Clear State

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_cool_meas_Rk_start

Function bp2304_cool_meas_Rk_start& (ByVal instrumentID&)

Purpose

Panel Name : Measurement Rcool Start

Descrpition: Start the measurement of the

cold resistance

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_cool_meas_Rk_stop

Function bp2304_cool_meas_Rk_stop& (ByVal instrumentID&)

Purpose

Panel Name : Measurement Rcool Stop

Descrpition: Stop the measurement of the

cold resistance

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_cool_meas_start

Function bp2304_cool_meas_start& (ByVal instrumentID&)

Purpose

Panel Name : Measure Start

Descrpition: Start the measurement of the

cooling curve.

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_cool_meas_stop

Function bp2304_cool_meas_stop& (ByVal instrumentID&)

Purpose

Panel Name : Measure Stop

Descrpition: Stop the measurement of the

cooling curve.

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_cool_meas_t1

Function bp2304_cool_meas_t1& (ByVal instrumentID&, ByVal measureT1&, ByVal manualT1Cel&)

Purpose

Panel Name : Measurement Temperature 1

Descrpition: Measure or entry the ambient

temperature of the test unit

before loading.

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal measureT1&

Control Name:  Measure T1

Description:   This parameter selected the mode

to enter ambient temperature

before loading.

Variable Type: Integer

Valid Range:   0 = Manual (Please enter the

Manual T1)

1 = Auto   ( Read the PT100)

ByVal manualT1Cel&

Control Name:  Manual T1

Description:   This parameter entry manual the

ambient temperature before

loading.

Variable Type: Double

Valid Range:   -99.9 - 999.9 Cel

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

-2     Invalid Measure T1 State

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_cool_meas_t2

Function bp2304_cool_meas_t2& (ByVal instrumentID&, ByVal measureT2&, ByVal manualT2Cel&)

Purpose

Panel Name : Measurement Temperature 2

Descrpition: Measure or entry the ambient

temperature of the test unit

after loading.

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal measureT2&

Control Name:  Measure T2

Description:   This parameter selected the mode

to enter ambient temperature

after loading.

Variable Type: Integer

Valid Range:   0 = Manual (Please enter the

Manual T2)

1 = Auto   ( Read the PT100)

ByVal manualT2Cel&

Control Name:  Manual T2

Description:   This parameter entry manual the

ambient temperature after

loading.

Variable Type: Double

Valid Range:   -99.9 - 999.9 Cel

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

-2     Invalid Measure T2 State

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_cool_read_meas

Function bp2304_cool_read_meas& (ByVal instrumentID&, timesec#, value#, ByVal unit$, numbers_ofValues&)

Purpose

Panel Name : Measure Read

Descrpition: Read the measurement values of

the cooling curve.

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

timesec#

Control Name:  Time [sec]

Description:  Return the time of the cooling

curve measurement

Variable Type: Double Array of 256 elements

value#

Control Name:  Value

Description:  Return the value of the cooling

curve measurement

Variable Type: Double Array of 256 elements

ByVal unit$

Control Name:  Unit

Description:  Return the unit of the cooling

curve measurement

Variable Type: String (Length = 8Byte)

numbers_ofValues&

Control Name:  Unit

Description:  Return the unit of the cooling

curve measurement

Variable Type: String (Length = 8Byte)

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

303   There are no Data to transfer in buffer

________________________________________________________________________________

bp2304_cool_read_RK

Function bp2304_cool_read_RK& (ByVal instrumentID&, value#, ByVal unit$)

Purpose

Panel Name : Measure Read

Descrpition: Read the measurement values of

the cooling curve.

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

value#

Control Name:  Value

Description:  Return the value of the cooling

curve measurement

Variable Type: Double

ByVal unit$

Control Name:  Unit

Description:  Return the unit of the cooling

curve measurement

Variable Type: String (Length = 8Byte)

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_cool_read_t1

Function bp2304_cool_read_t1& (ByVal instrumentID&, valueCel#)

Purpose

Panel Name : Measure Read

Descrpition: Read the measurement values of

the cooling curve.

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

valueCel#

Control Name:  Value

Description:  Return the Temperatur 1 of the

cooling curve measurement

Variable Type: Double

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_cool_read_t2

Function bp2304_cool_read_t2& (ByVal instrumentID&, valueCel#)

Purpose

Panel Name : Measure Read

Descrpition: Read the measurement values of

the cooling curve.

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

valueCel#

Control Name:  Value

Description:  Return the Temperatur 2 of the

cooling curve measurement

Variable Type: Double

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_cool_removal_load

Function bp2304_cool_removal_load& (ByVal instrumentID&)

Purpose

Panel Name : Removal of Load

Descrpition: Signalled the device the removal

of the load on the test unit.

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_cool_state

Function bp2304_cool_state& (ByVal instrumentID&, ByVal coolingCurveState&)

Purpose

Panel Name : C-Measurement State

Descrpition: Cooling curve state ON/OFF

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal coolingCurveState&

Control Name:  Cooling-Curve-State

Description:   This parameter is activates the

cooling curve menu

Variable Type: Integer

Valid Range:   0 = OFF

1 = ON

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

-2     Invalid State

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_corr_set_tk

Function bp2304_corr_set_tk& (ByVal instrumentID&, ByVal number_ofCoeff&, ByVal coeffValueppmk&)

Purpose

Panel Name:    Set temperatur coefficients

Descrpition:   Set the one of the ten temperature

coefficents.

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal number_ofCoeff&

Control Name:  Number of coefficient

Description:   This parameter selects one of ten

possible temperature coefficients

Variable Type: Integer

Valid Range:   1 to 10

ByVal coeffValueppmk&

Control Name:  Value of coefficient

Description:   This parameter specifices the

value of the desired temperature

coefficients.The unit is ppm/k.

Variable Type: Integer

Valid Range:   0-9999

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

-2     Invalid Number of TC

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_corr_state

Function bp2304_corr_state& (ByVal instrumentID&, ByVal correctionState&, ByVal correctionType&, ByVal manualTemperature#)

Purpose

Panel Name:    Correction state

Descrpition:   Set the follow parameters:

- toggle the state ON/OFF

- Input the manual temperature

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal correctionState&

Control Name:  Correction State

Description:   This parameter specifies whether a

temperature compensation si to be

carried out during the resistance

measurement.

Variable Type: Integer

Valid Range:   0 = OFF

1 = ON

ByVal correctionType&

Control Name:  Correction Type

Description:   This parameter specifies whether a

temperature value in the case of

temperature compensation should be

registered automatically or

entered manually.

Variable Type: Integer

Valid Range:   0 = Manualtemperature compensation

1 = Auto temperature compensation

ByVal manualTemperature#

Control Name:  Manual Temperature

Description:   This parameter specifies the

temperature value in the case of

temperature compensation should be

registered manually.

Variable Type: Integer

Valid Range:   -70 to 999.99

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

-2     Invalid State

-3     Invalid Correction Type

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_disp_contrast

Function bp2304_disp_contrast& (ByVal instrumentID&, ByVal displayContrast#)

Purpose

Panel Name:    Display Contrast

Descrpition:   Set the display contrast

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal displayContrast#

Control Name:  Display Contrast

Description:   This parameter is meant for

adjusting the display contrast.

Variable Type: Double

Valid Range:   0.0 - 1.0

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

-2     Invalid Contrast Value

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_disp_enable

Function bp2304_disp_enable& (ByVal instrumentID&, ByVal display&)

Purpose

Panel Name:    Display Enable

Descrpition:   Activate or deactivate LCD display

(during remote operation)

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal display&

Control Name:  Display Enable

Description:   This parameter activate or

deactivate the LCD-display.

(during the remote operation)

Variable Type: Integer

Valid Range:   0 = OFF  => LCD display is OFF

1 = ON   => LCD display is ON

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

-2     Invalid Display Parameter

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_disp_mode

Function bp2304_disp_mode& (ByVal instrumentID&, ByVal calculateState&, ByVal displayMode&, ByVal nominalOhm#, ByVal lenghtmft#, ByVal crossSectionmm2#, ByVal densitygcm3#, ByVal massg#, ByVal measLenghtm#)

Purpose

Panel Name:    Display Mode

Descrpition:   Selects the desiered display mode

of device

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal calculateState&

Control Name:  Calculate State

Description:   This parameter activate or

deactivate the calculate function.

Variable Type: Integer

Valid Range:   0 = OFF  => Ohm display mode

1 = ON   => desired display mode

ByVal displayMode&

Control Name:  Display Mode

Description:   This parameter selects the desired

display mode of the device.

Variable Type: Integer

Valid Range:   0 = R/Length0 [m]

1 = R/Length0 [km]

2 = R/Length0 [10ft]

3 = R/Length0 [kft]

4 = Delta %

5 = Rho1

6 = Rho2

7 = Gamma1

8 = Gamma2

ByVal nominalOhm#

Control Name:  Nominal Value

Description:   Entry of the specifed value in

ohms.

It's only used by display mode

- "Delta%"

Variable Type: Double

Valid Range:   1e-7 - 999999e3

ByVal lenghtmft#

Control Name:  Length

Description:   Entry of the length in meters or feet.

It's used by the follow display

mode :

- "R/Length[m]"

- "R/Length[Km]"

- "R/Length[10ft]"

- "R/Length[kft]"

- "Rho1"

- "Rho2"

- "Gamma1"

- "Gamma2"

Variable Type: Double

Valid Range by display mode:

- "R/Length[m]"    => 0.001-999999

- "R/Length[Km]"   => 0.001-999999

- "R/Length[10ft]" => 0.001-999999

- "R/Length[kft]"  => 0.001-999999

- "Rho1"           => 0.01-99.99

- "Rho2"           => 0.01-99.99

- "Gamma1"         => 0.01-99.99

- "Gamma2"         => 0.01-99.99

ByVal crossSectionmm2#

Control Name:  Cross-Section

Description:   Entry of the cross-section in mm^2

It's used by the follow display

mode :

- "Rho1"

- "Gamma1"

Variable Type: Double

Valid Range   : 1e-5 - 9999

ByVal densitygcm3#

Control Name:  density

Description:   Entry of the density in g/mm^3.

It's used by the follow display

mode :

- "Rho2"

- "Gamma2"

Variable Type: Double

Valid Range  : 0.01-99.0

ByVal massg#

Control Name:  Mass

Description:   Entry of the mass in g.

It's used by the follow display

mode :

- "Rho2"

- "Gamma2"

Variable Type: Double

Valid Range  : 1e-5 - 99999.9

ByVal measLenghtm#

Control Name:  Meas-Length

Description:   Entry of the measlength in meters.

It's used by the follow display

mode :

- "Rho2"

- "Gamma2"

Variable Type: Double

Valid Range by display mode:

- "Rho2"        => 0.01-99.999

- "Gamma2"      => 0.01-99.999

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

-2     Invalid Calculate State

-3     Invalid Display Mode

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_get_class_comp_data

Function bp2304_get_class_comp_data& (ByVal instrumentID&, data%)

Purpose

Panel Name:    Class Comparator Data

Descrpition:   Get the classification statistics

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

data%

The control return the classification statistics.

Variable Type : Interger Array with 10 elements

data[0] - [8]

- number of measured values

withinthe individual

classification limits.

data[9]

- total number of recorded

measurements

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_get_comp_data

Function bp2304_get_comp_data& (ByVal instrumentID&, data%)

Purpose

Panel Name:    Get Comparator Data

Descrpition:   Get the comparator data's

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

data%

The control return the comparator statistics.

Variable Type : Interger Array with 4 elements

data[0] = number of measured

values smaller than

lower limit  (<)

data[1] = number of measured

values larger or equal

than the lower limit

and smaller or equal

than the upper limit(=)

data[2] = number of measured

values larger than

upper limit (>)

data[3] = total number of

recorded measurements

(sum)

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_get_temp

Function bp2304_get_temp& (ByVal instrumentID&, currentTemperature#)

Purpose

Panel Name:    Get the current temperature

(PT100)

Descrpition:   Read the current PT100 temperature

when the state is ON.

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

currentTemperature#

The control return the current temperature value

from the external PT100-Sensor.The PT100 must

plugged in. The state of temperature compersation

must be ON.

The unit is degree.

Variable Type : double

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_init_gpib

Function bp2304_init_gpib& (ByVal address&, ByVal timeout&, ByVal reset&, ByVal instrumentIDString$, instrumentID&)

Purpose

Panel Name:    Initialize Gpib

Description:   Initializes the instrument by:

1) finding an unused device in the device table

using the supplied GPIB address

2) device clear and RESET  (if desired)

3) returns the ID-String of the instrument

4) returns an instrument ID

This is a multi-instrument driver, allowing

multiple Resistomat 2304+2305 instruments to be

controlled.

During the initialization, the returned ID is

based on the GPIB address.  This ID is used in

all of the other functions to distinguish

between multiple Resistomat 2304+2305

instruments.

Parameter List

ByVal address&

Control Name:  Address

Description:   Specifies the address on the GPIB

to be used to communicate with the instrument.

This value must correspond to the current

internal setting of the instrument.

Variable Type: Integer

Valid Range:   0 to 30

ByVal timeout&

ByVal reset&

Control Name:  Reset

Description:   If reset is on, a device clear and

power on reset is done during initialization.

Variable Type: Integer

Valid Range:   0 to 1

0  off

1  on

ByVal instrumentIDString$

Control Name:  Instrument ID String

Description:   Returns the ID-String of the

instrument.

Variable Type: String

instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid GPIB Address

-2     Invalid Reset

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

________________________________________________________________________________

bp2304_init_serial

Function bp2304_init_serial& (ByVal COMPort&, ByVal baudRate&, ByVal timeout_seconds#, ByVal blockCheck&, ByVal groupAddress&, ByVal userAddress&, ByVal instrumentIDString$, ByVal reset&, instrumentID&)

Purpose

Panel Name:    Initialize Serial

Description:   Initializes the instrument by:

1) finding an unused device in the device table

using the supplied serial group and user

address

2) device clear and RESET  (if desired)

3) returns the ID-String of the instrument

4) returns an instrument ID

This is a multi-instrument driver, allowing

multiple Resistomat 2304-2305 instruments to be

controlled.

During the initialization, the returned ID is

based on the serial group and user address.

This ID is used in all of the other functions

to distinguish between multiple Resistomat 2304+

2305 instruments.

Parameter List

ByVal COMPort&

Control Name:   COM-Port

Description:    Specifies the COM-Port to be

used to communicate with the

instrument.

Variable Type:  Integer

Valid Values:   1 - COM1   (=> Adress 0x3f8 IRQ4)

2 - COM2   (=> Adress 0x2f8 IRQ3)

3 - COM3   (=> Adress 0x3e8 IRQ4)

4 - COM4   (=> Adress 0x2e8 IRQ3)

Default:        1

ByVal baudRate&

Control Name:   Baud Rate

Description:    Specifies the baud rate of the

RS232-Port to communicate with

the instrument. This value must

correspond to the current

internal setting of the

instrument.

Variable Type:  Integer

Valid Values:   1200,2400,4800,9600 or 19200 baud

Default:        9600

ByVal timeout_seconds#

Control Name:   Time-out (seconds)

Description:    Specifies  the time-out limit for

I/O operations.

A  value  of  0.0  will  disable

time-out altogether.

Variable Type:  Double

Valid Range:    0.060 to 100.0 seconds

Default:        1.0

ByVal blockCheck&

Control Name:   Bcc-Check

Description:    This parameter switches the block

check function of the driver ON

or OFF.

This value must correspond to the

current internal setting of the

instrument.

Variable Type:  Integer

Valid Values:   0 = OFF, 1 = ON

Default:        0 = OFF

ByVal groupAddress&

Control Name:  Group Address

Description:   Specifies the group address on the

RS232 (RS485) to be used to

communicate with the instrument.

This value must correspond to the

current internal group address

from the instrument.

Variable Type: Integer

Valid Range:   0 to 15

Default:       0

ByVal userAddress&

Control Name:  User Adress

Description:   Specifies the user address on the

RS232 (RS485) to be used to

communicate with the instrument.

This value must correspond to the

current internal user address

from the instrument.

Variable Type: Integer

Valid Range:   0 to 15

Default:       0

ByVal instrumentIDString$

Control Name:  Instrument ID String

Description:   Returns the ID-String of the

instrument.

Variable Type: String

ByVal reset&

Control Name:  Reset

Description:   If reset is on, a device clear and

power on reset is done during initialization.

Variable Type: Integer

Valid Range:   0 to 1

0 = off

1 = on

instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   COM-Port is out of range

-2   Baud Rate is out of range

-3   Time-out is out of range

-4   Block Check parameter is out of range

-5   Group Address is out of range

-6   User Address is out of range

-7   Reset Device is out of range

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_local_gpib

Function bp2304_local_gpib& (ByVal instrumentID&)

Purpose

Panel Name:    Local (GPIB only)

Description:   Sets the  temporarily in

local mode.

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

________________________________________________________________________________

bp2304_meas_read

Function bp2304_meas_read& (ByVal instrumentID&, measuredValue#, ByVal unit$)

Purpose

Panel Name:    Measure Read

Descrpition:   Revieves measured values while

a measurement is done

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

measuredValue#

The control return the measurement value

Variable Type : double

ByVal unit$

The control return the measurement unit

Variable Type : String (Length=8Byte)

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

302   There ist's no measurement started

303   Timeout for wait of RQS Byte

304   GPIB Error for read the RQS Byte

System Device Errors : (=>SystemError + 300)

----------------------------------------------

310   Cable rupture on measuring resistor

320   Amplifier overdriven

330   Overrange

340   Temperature of device too high

350   Error occurred during measurement

360   Current cannot be controled

________________________________________________________________________________

bp2304_meas_start

Function bp2304_meas_start& (ByVal instrumentID&)

Purpose

Panel Name:    Measure Start

Descrpition:   Start the measurement

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_meas_stop

Function bp2304_meas_stop& (ByVal instrumentID&)

Purpose

Panel Name:    Measure Stop

Descrpition:   Stop the measurement

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_receive_scpi

Function bp2304_receive_scpi& (ByVal instrumentID&, ByVal sendSCPIMessage$, ByVal answer_fromSlave$)

Purpose

Panel Name:   Receive a SCPI-Message

Descrpition:  Send "Query form" from SCPI-

Message and receive a answers

=> Polling

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal sendSCPIMessage$

Control Name:  Send SCPI-Message

Description:   This control is a string parameter

You must input Query-Form from

the SCPI-message without the

LineFeed termination.

Example      : ":SENS:CORR:STATE?"

Variable Type: String

ByVal answer_fromSlave$

Control Name:  Answer from Slave

Description:   This control return a string from

the slave.

Variable Type: String (Lenght => see the 2304

Manual SCPI-Commands)

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_send_scpi

Function bp2304_send_scpi& (ByVal instrumentID&, ByVal sendSCPIMessage$)

Purpose

Panel Name:   Send a SCPI-Message

Descrpition:  Send every SCPI-Message to a

slave and receive a ACK

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal sendSCPIMessage$

Control Name:  Send SCPI-Message

Description:   This control is a string parameter

You must input the SCPI-message

without the LineFeed termination.

Example      : ":SENS:CORR:STATE ON"

Variable Type: String

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_system_buzzer

Function bp2304_system_buzzer& (ByVal instrumentID&, ByVal buzzer&)

Purpose

Panel Name:   System Buzzer

Descrpition:  Control the buzzer of the device

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal buzzer&

Control Name:  Buzzer

Description:   This parameter controls the buzzer

of the device.

When the buzzer is active,warning

tone occours in the case of

measurement on inductiveloads;

this tone persists from

measruement interruption until the

test unit has discharge.

Variable Type: Integer

Valid Range:   0  = OFF

1  = ON

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

-2     Invalid Buzzer State

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_system_error

Function bp2304_system_error& (ByVal instrumentID&)

Purpose

Panel Name:   System Error

Descrpition:  Read the system error of the device

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

System Device Errors :

10   Cable rupture on measuring resistor

20   Amplifier overdriven

30   Overrange

40   Temperature of device too high

50   Error occurred during measurement

60   Current cannot be controled

-100  Command  Error

(An invalid command was send)

-105  Get Not Allowed

(A Get command was sent within a

command)

-110  Command  Header Error

( Command with invalid command

heading)

-200  Execuation Error

(Due to a certain state of the device,

a command could not be executed)

-220  Parameter Error

(A command with in incorrect parameter

was sent)

-350  Error Buffer Overflow

-400  Query Error

(Device was polled but not data was

present)

-410  Query Interrupted

(Device was interrupted with out

having send a complete response)

-420  Query Unterminated

(Device was polled without having

received a complete command)

________________________________________________________________________________

bp2304_system_keyboard

Function bp2304_system_keyboard& (ByVal instrumentID&, ByVal keyboardState&, ByVal keyboardAccess&)

Purpose

Panel Name:   System Keybroad

Descrpition:  Control the keybroad of the device

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal keyboardState&

Control Name:  Keyboard

Description:   This parameter locks the keyboard

on the front pannel of the device.

Variable Type: Integer

Valid Range:   0  = Unlocked

1  = Locked

ByVal keyboardAccess&

Control Name:  Keyboard Access

Description:   This parameter selects the access

authorization for the keyboardon

the front panel.

Variable Type: Integer

Valid Range:   0  = Full access

1  = Measurement and range

selection

2  = Only measure selection

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

-2     Invalid Keyboard State

-3     Invalid Keyboard Access

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp2304_system_output

Function bp2304_system_output& (ByVal instrumentID&, ByVal pin3&, ByVal pin21&)

Purpose

Panel Name:   Set Output

Descrpition:  Set/Reset the output Pin 3+21

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal pin3&

Control Name:  Pin 3

Description:   With this parameter can be set or

reset the pin 3 of I/O jack.

Variable Type: Integer

Valid Range:   0  = OFF

1  = ON

ByVal pin21&

Control Name:  Pin 21

Description:   With this parameter can be set or

reset the pin 21 of I/O jack.

Variable Type: Integer

Valid Range:   0  = OFF

1  = ON

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

-2     Invalid Pin 3  Value

-3     Invalid Pin 21  Value

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT