Burster Digistant Type 4422

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for Burster Digistant Type 4422.

It contains functions for opening, configuring, taking measurements from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

For GPIB instrument drivers:

-  the instrument is connected to the GPIB.

-  the GPIB address supplied to the initialize function must match the

GPIB address of the instrument.

For VXI instrument drivers:

-  the instrument is installed in the VXI mainframe and you are

using one of the following controller options:

Embedded controller

MXI

MXI2

GPIB-VXI

-  the logical address supplied to the initialize function must

match the logical address of the instrument.

For RS-232 instrument drivers:

-  the instrument is connected to the RS-232 interface.

-  the COM port, baud rate, parity, and timeout supplied to the

initialize function must match the settings of the instrument.

________________________________________________________________________________

Error and Status Information:

Each function in this instrument driver returns a status code that

either indicates success or describes an error or warning condition.

Your program should examine the status code from each call to an

instrument driver function to determine if an error occurred.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

The description of each instrument driver function lists possible error

codes and their meanings

________________________________________________________________________________

How To Use This Document:

Use this document as a programming reference manual.

It describes each function in the

Burster Digistant Type 4422

instrument.  The functions appear in alphabetical order, with a description

of the function and its C syntax, a description of each parameter, and a list

of possible error codes.

________________________________________________________________________________

Function Tree Layout:

Class/Panel Name:                       Function Name:

Initialize                           bp4422_init

Calculate

Calculate State                   bp4422_calc_state

Calculate Reset                   bp4422_calc_reset

Configure

Config Device                     bp4422_config

Config Pressure Module            bp4422_config_p_module

Get Config Pressure Module        bp4422_get_config_p_module

Config Grounding                  bp4422_config_grounding

Config Output TC-RJ               bp4422_config_output_tc_rj

Config Input TC-RJ                bp4422_config_input_tc_rj

Sets a Value                      bp4422_set_value

Measuring

Start Measure                     bp4422_start_measure

Read Measure String               bp4422_read_measure_string

Read Measure Value                bp4422_read_measure

Read Pressure Measure String      bp4422_read_p_measure_string

Read Pressure Measure Value       bp4422_read_p_measure

Stop Measure                      bp4422_stop_measure

Presure Functions

Select Pressure Module by No.     bp4422_select_p_module_no

Select Pressure Module by Name    bp4422_select_p_module_name

Tare Pressure State               bp4422_tare_p_state

Tare Pressure Initiate            bp4422_tare_p_init

Get Tare Pressure Value           bp4422_get_tare_p_value

Get Pressure Module Name          bp4422_get_p_module_name

Set Pressure Module Name          bp4422_store_p_module_name

Check Pressure Module Batterie    bp4422_check_p_module_batterie

Datalogger

Datalogger Clear All              bp4422_clear_all_datalogger

Datalogger Clear Block            bp4422_clear_datalogger_block

Datalogger Mode                   bp4422_set_datalogger_mode

Datalogger Start/Stop             bp4422_datalogger_start_stop

Datalogger Step                   bp4422_datalogger_step

Sets Datalogger Time              bp4422_set_datalogger_time

Read Datalogger Aver+Dev          bp4422_datalogger_aver_devi

Read Datalogger Min/Max           bp4422_datalogger_min_max

Read Datalogger (String)          bp4422_read_datalogger

Read Datalogger Value             bp4422_read_datalogger_value

Ramp

Ramp Mode                         bp4422_set_ramp_mode

Ramp Start/Stop                   bp4422_ramp_start_stop

Ramp Step                         bp4422_ramp_step

Init. Ramp1                       bp4422_init_ramp1

Init. Ramp2                       bp4422_init_ramp2

Init. Ramp2 List                  bp4422_init_ramp2_list

Display

Display Backlight                 bp4422_set_display_backlight

Set Display Contrast              bp4422_set_display_contrast

System

System Error                      bp4422_system_error

Get Status                        bp4422_get_status

System Keyboard                   bp4422_system_keyboard

Send a SCPI-Message               bp4422_send_scpi

Receive a SCPI-Message            bp4422_receive_scpi

Close                                bp4422_close

________________________________________________________________________________

Burster Digistant Type 4422

Instrument Name: burster DIGISTANT Type 4422

Description:   This instrument module provides

RS232 support for DIGISTANT

Type 4422 from burster.

Functions and Classes:

(1) Initialize :

Initialize the instrument and sets to

default configuration

(2) Calculate:

Set the Calculate settings of the instrument

(3) Configure:

Configures the instrument by setting the

Voltage,Current,TC,RJ,RTD amd Output a Value

(4) Measuring :

Measuring functions of the DIGISTANT

(5) Datalogger :

Datalogger functions of the DIGISTANT

(6) Ramp :

Ramp functions of the DIGISTANT

(7) System:

Reads the system error

Reads the instrument ID-String

Controls the keyboard

Sends a SCPI-Message

Receives a SCPI-Message

(8) Close :

Takes the instrument off-line.

*********

________________________________________________________________________________

The following functions are in alphabetical order.

________________________________________________________________________________

bp4422_calc_reset

int bp4422_calc_reset (int instrumentID);

Purpose

Panel Name:    Calculate Reset

Description:   Reset Min/Max Value's of the

instrument

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Range

-4   Invalid Sense

-3   Invalid Current Limit

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp4422_calc_state

int bp4422_calc_state (int instrumentID, int state);

Purpose

Panel Name:    Calculate State

Description:   Switch the calculate state of the

instrument ON/OFF

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

state

Variable Type       int

Control Name:   State

Description:    This parameter switches the

Calculate State of he device

ON or OFF.

Variable Type:  Integer

Valid Values:   0 = OFF, 1 = ON

Default:        0 = OFF

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Range

-4   Invalid Sense

-3   Invalid Current Limit

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp4422_check_p_module_batterie

int bp4422_check_p_module_batterie (int instrumentID, char value[]);

Purpose

Panel Name:    Check Pressure Module Batterie

Description:   Check the pressure module batterie state

If the batterie voltage higher than 100mV,

the battiere is ok.

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

value

Variable Type       char []

Control Name:   Batterie Value

Description:    Return the batterie value

Variable Type:  String

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

319   The module batterie is lower than 100mV

________________________________________________________________________________

bp4422_clear_all_datalogger

int bp4422_clear_all_datalogger (int instrumentID);

Purpose

Panel Name:    Datalogger Clear All

Description:   Clear all datalogger block's in

the instrument

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

320   Datalogger is empty

________________________________________________________________________________

bp4422_clear_datalogger_block

int bp4422_clear_datalogger_block (int instrumentID, int blockNo);

Purpose

Panel Name:    Datalogger Clear Block

Description:   Clear one datalogger block No. in

the instrument

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

blockNo

Variable Type       int

Control Name:   Block No.

Description:    Specifies the datalogger Block No

Variable Type:  integer

Valid Range:    1 to 256

Default:        1

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

-2   Invalid Block No.

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

320   Datalogger is empty

________________________________________________________________________________

bp4422_close

int bp4422_close (int instrumentID);

Purpose

Panel Name:    Close

Description:   Takes the instrument off-line.

Note: The  instrument must  be reinitialized

to use it again.

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

221   Unable to close instrument

232   Instrument not initialized

________________________________________________________________________________

bp4422_config

int bp4422_config (int instrumentID, int mode, int RTDType, int RTDUnit,

int TCType, int TCUnit);

Purpose

Panel Name:    Config Device

Description:   Configure the instrument in

following ways:

- Selects the device mode of the

instrument

- Selects the TC and RTD Type

- Selects the TC and RTD Unit

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

mode

Variable Type       int

Control Name:   Mode

Description:    This parameter switches the

mode of the device

Variable Type:  Integer

Valid Values:   0 = Input/Output

1 = Output

2 = Input

Default:        0 = Input/Output

RTDType

Variable Type       int

Control Name:   RTD-Type

Description:    This parameter switches the

RTD-Type of the device

Variable Type:  Integer

Valid Values:   0 = PT100

1 = NI100

2 = PT200

3 = PT500

4 = PT1000

Default:        0 = Input/Output

RTDUnit

Variable Type       int

Control Name:   RTD-Unit

Description:    This parameter switches the

RTD-Unit of the device

Variable Type:  Integer

Valid Values:   0 = Ohm

1 = �C

2 = �F

3 = K

Default Value:  0 = Ohm

TCType

Variable Type       int

Control Name:   TC-Type

Description:    This parameter switches the

TC-Type of the device

Variable Type:  Integer

Valid Values:   0 = R

1 = S

2 = B

3 = J

4 = T

5 = E

6 = K

7 = U

8 = L

9 = N

10 = C

11 = D

12 = G2

13 = M

Default Value:  0 = R

TCUnit

Variable Type       int

Control Name:   TC-Unit

Description:    This parameter switches the

TC-Unit of the device

Variable Type:  Integer

Valid Values:   0 = �C

1 = �F

2 = K

Default Value:  0 = �C

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Mode

-3   Invalid RTD-Type

-4   Invalid RTD-Unit

-5   Invalid TC-Type

-6   Invalid TC-Unit

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp4422_config_grounding

int bp4422_config_grounding (int instrumentID, int mode);

Purpose

Panel Name:    Config Grounding

Description:   Configure the instrument in

following ways:

- Selects grounding function

of the instrument

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

mode

Variable Type       int

Control Name:   Mode

Description:    This parameter switches the

grounding mode of the device

Variable Type:  Integer

Valid Values:   0 = Intern

1 = Extern

2 = Auto

Default:        0 = Intern

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Mode

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp4422_config_input_tc_rj

int bp4422_config_input_tc_rj (int instrumentID, int TCRefJunction,

double TManTMin, double TMax);

Purpose

Panel Name:    Config Intput TC-RJ

Description:   Configure the instrument in

following ways:

- Selects the intput Reference

junction

- Input the manuel value for

the reference junction

- Input the min and max. value

for reference junction

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

TCRefJunction

Variable Type       int

Control Name:   TC-Ref-Junction

Description:    This parameter switches the

intput TC-Reference Junction

of the device

Variable Type:  Integer

Valid Values:   0 = RJ-Int

1 = RJ-Man

2 = RJ-Ext

3 = RJ 4-20mA

4 = RJ 0-20mA

TManTMin

Variable Type       double

Control Name:   T-Man/T-Min

Description:    Specifies  the T-Man for

RJ-MAN or T-Min for

RJ 4-20mA or RJ+ 0-20mA function

Variable Type:  Double

Default:        0.0

TMax

Variable Type       double

Control Name:   T-Max

Description:    Specifies  the T-Max for

RJ 4-20mA or RJ 0-20mA function

Variable Type:  Double

Default:        0.0

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid TC-RJ Junction

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp4422_config_output_tc_rj

int bp4422_config_output_tc_rj (int instrumentID, int TCRefJunction,

double TManTMin, double TMax);

Purpose

Panel Name:    Config Output TC-RJ

Description:   Configure the instrument in

following ways:

- Selects the output Reference

junction

- Input the manuel value for

the reference junction

- Input the min and max. value

for reference junction

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

TCRefJunction

Variable Type       int

Control Name:   TC-Ref-Junction

Description:    This parameter switches the

output TC-Reference Junction

of the device

Variable Type:  Integer

Valid Values:   0 = TC

1 = TC-RJ-Man

2 = TC-RJ Auto

3 = TC+ 4-20mA

4 = TC+ 0-20mA

Default:        0 = TC

TManTMin

Variable Type       double

Control Name:   T-Man/T-Min

Description:    Specifies  the T-Man for

TC-RJ-MAN or T-Min for

TC+ 4-20mA or TC+ 0-20mA function

Variable Type:  Double

Default:        0.0

TMax

Variable Type       double

Control Name:   T-Max

Description:    Specifies  the T-Max for

TC+ 4-20mA or TC+ 0-20mA function

Variable Type:  Double

Default:        0.0

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid TC-RJ-Junction

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp4422_config_p_module

int bp4422_config_p_module (int instrumentID, int moduleNo,

char serialNo[], char calDate[],

char unit[], double zero_point,

double zero_pressure, double finalValue,

double finalPValue, double maxValue);

Purpose

Panel Name:    Config Pressure Mode

Description:   Configure the Pressure Mode

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

moduleNo

Variable Type       int

Control Name:   Module No

Description:    This parameter select the module no.

Variable Type:  Integer

Valid Values:   0-9

Default:        0

serialNo

Variable Type       char []

Control Name:   Serial-No.

Description:    This parameter input the serial no.

Variable Type:  String

calDate

Variable Type       char []

Control Name:   Cal-Date

Description:    This parameter input the cal-date of the

module

Variable Type:  String

unit

Variable Type       char []

Control Name:   Unit

Description:    This parameter input the module unit

Variable Type:  String

zero_point

Variable Type       double

Control Name:   Zero Point

Description:    This parameter input the voltage zero point

value

Variable Type:  double

zero_pressure

Variable Type       double

Control Name:   Zero pressure

Description:    This parameter input the zero pressure value

Variable Type:  double

finalValue

Variable Type       double

Control Name:   Zero value

Description:    This parameter input the final voltage value

Variable Type:  double

finalPValue

Variable Type       double

Control Name:   Final P-Value

Description:    This parameter input the final pressure value

Variable Type:  double

maxValue

Variable Type       double

Control Name:   Max.Value

Description:    This parameter input the max. allowed module

value

Variable Type:  double

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Module No

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp4422_datalogger_aver_devi

int bp4422_datalogger_aver_devi (int instrumentID, int blockNo,

double *average, double *deviation,

char unit[]);

Purpose

Panel Name:    Read Datalogger Aver+Dev

Description:   Read the Average and the deviation

from the selected Block No in the

Datalogger

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

blockNo

Variable Type       int

Control Name:   Block No.

Description:    Specifies the datalogger Block No

Variable Type:  integer

Valid Range:    1 to 256

Default:        1

average

Variable Type       double (passed by reference)

Control Name:  Average

Description:   Returns the average of the

selected Block No.

Variable Type: double

deviation

Variable Type       double (passed by reference)

Control Name:  Deviation

Description:   Returns the deviation of the

selected Block No.

Variable Type: double

unit

Variable Type       char []

Control Name:  unit

Description:   Returns the unit from average and

and deviation of the selected

Block No.

Variable Type: String

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

-2   Invalid Block No.

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

320   Datalogger is empty

________________________________________________________________________________

bp4422_datalogger_min_max

int bp4422_datalogger_min_max (int instrumentID, int blockNo,

double *minValue, double *maxValue,

char unit[]);

Purpose

Panel Name:    Read Datalogger Max+Min

Description:   Read the Minimum and Maximum Value

from the selected Block No in the

Datalogger

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

blockNo

Variable Type       int

Control Name:   Block No.

Description:    Specifies the datalogger Block No

Variable Type:  integer

Valid Range:    1 to 256

Default:        1

minValue

Variable Type       double (passed by reference)

Control Name:  Min.Value

Description:   Returns the Min.Value of the

selected Block No.

Variable Type: double

maxValue

Variable Type       double (passed by reference)

Control Name:  Max.Value

Description:   Returns the Max.Value of the

selected Block No.

Variable Type: double

unit

Variable Type       char []

Control Name:  unit

Description:   Returns the unit from average and

and deviation of the selected

Block No.

Variable Type: String

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

-2   Invalid Block No.

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

320   Datalogger is empty

________________________________________________________________________________

bp4422_datalogger_start_stop

int bp4422_datalogger_start_stop (int instrumentID, int startStop);

Purpose

Panel Name:    Datalogger Start/Stop

Description:   Switch between datalogger Start or

Stop

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

startStop

Variable Type       int

Control Name:   Datalogger Start/Stop

Description:    This parameter switches the

Datalogger Start/Stop of the

device ON or OFF.

Variable Type:  Integer

Valid Values:   0 = Stop, 1 = Start

Default:        0 = Stop

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

-2   Invalid Start/Stop

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

320   Datalogger is empty

________________________________________________________________________________

bp4422_datalogger_step

int bp4422_datalogger_step (int instrumentID);

Purpose

Panel Name:    Datalogger Step

Description:   Recording one Value in Datalogger

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

320   Datalogger is empty

________________________________________________________________________________

bp4422_get_config_p_module

int bp4422_get_config_p_module (int instrumentID, int moduleNo,

char name[], char serialNo[],

char calDate[], char unit[],

double *zero_point,

double *zero_pressure,

double *finalValue, double *finalPValue,

double *maxValue);

Purpose

Panel Name:    Get Config Pressure Module

Description:   Get Configure pressure module

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

moduleNo

Variable Type       int

Control Name:   Module No

Description:    This parameter select the module no.

Variable Type:  Integer

Valid Values:   0-9

Default:        0

name

Variable Type       char []

Control Name:   Name

Description:    This parameter input the module name

Variable Type:  String

serialNo

Variable Type       char []

Control Name:   Serial-No.

Description:    This parameter input the serial no.

Variable Type:  String

calDate

Variable Type       char []

Control Name:   Cal-Date

Description:    This parameter input the cal-date of the

module

Variable Type:  String

unit

Variable Type       char []

Control Name:   Unit

Description:    This parameter input the module unit

Variable Type:  String

zero_point

Variable Type       double (passed by reference)

Control Name:   Zero Point

Description:    This parameter input the voltage zero point

value

Variable Type:  double

zero_pressure

Variable Type       double (passed by reference)

Control Name:   Zero pressure

Description:    This parameter input the zero pressure value

Variable Type:  double

finalValue

Variable Type       double (passed by reference)

Control Name:   Zero value

Description:    This parameter input the final voltage value

Variable Type:  double

finalPValue

Variable Type       double (passed by reference)

Control Name:   Final P-Value

Description:    This parameter input the final pressure value

Variable Type:  double

maxValue

Variable Type       double (passed by reference)

Control Name:   Max.Value

Description:    This parameter input the max. allowed module

value

Variable Type:  double

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Module No

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp4422_get_p_module_name

int bp4422_get_p_module_name (int instrumentID, int moduleNo,

char name[]);

Purpose

Panel Name:    Get Pressure Module Name

Description:   Get pressure module name

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

moduleNo

Variable Type       int

Control Name:   Module No

Description:    This parameter select the module no.

Variable Type:  Integer

Valid Values:   0-9

Default:        0

name

Variable Type       char []

Control Name:   Name

Description:    This parameter input the module name

Variable Type:  String

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Module No

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp4422_get_status

int bp4422_get_status (int instrumentID);

Purpose

Panel Name:   Get Status

Description:  Returns the device status

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

310   Device Voltage Error

311   Device Current Error

312   Device Temperature Error

313   Device Calibration Error

314   Device In Over Error

315   Device Out Over Error

316   Device Lowbat Error

317   Device Input and Output Over Error

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp4422_get_tare_p_value

int bp4422_get_tare_p_value (int instrumentID, char tareValue[]);

Purpose

Panel Name:    Get Tare Pressure Value

Description:   Get the tare value

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

tareValue

Variable Type       char []

Control Name:  Presure Value

Description:   Returns the presure value-String of the

instrument.

Variable Type: String

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

319   The module batterie is lower than 100mV

________________________________________________________________________________

bp4422_init

int bp4422_init (int COMPort, int baudRate, double timeout_seconds,

int blockCheck, int groupAddress, int userAddress,

char instrumentIDString[], int resetDevice,

int *instrumentID);

Purpose

Panel Name:    Initialize Serial

Description:   Initializes the instrument by:

1) finding an unused device in the device table

using the supplied serial group and user

address

2) device clear and RESET  (if desired)

3) returns the ID-String of the instrument

4) returns an instrument ID

Parameter List

COMPort

Variable Type       int

Control Name:   COM-Port

Description:    Specifies the COM-Port to be

used to communicate with the

instrument.

Variable Type:  Integer

Valid Values:   1 - COM1   (=> Adress 0x3f8 IRQ4)

2 - COM2   (=> Adress 0x2f8 IRQ3)

3 - COM3   (=> Adress 0x3e8 IRQ4)

4 - COM4   (=> Adress 0x2e8 IRQ3)

Default:        1

baudRate

Variable Type       int

Control Name:   Baud Rate

Description:    Specifies the baud rate of the

RS232-Port to communicate with

the instrument. This value must

correspond to the current

internal setting of the

instrument.

Variable Type:  Integer

Valid Values:   1200,2400,4800,9600 or 19200 baud

Default:        9600

timeout_seconds

Variable Type       double

Control Name:   Time-out (seconds)

Description:    Specifies  the time-out limit for

I/O operations.

A  value  of  0.0  will  disable

time-out altogether.

Variable Type:  Double

Valid Range:    0.060 to 100.0 seconds

Default:        1.0

blockCheck

Variable Type       int

Control Name:   Bcc-Check

Description:    This parameter switches the block

check function of the driver ON

or OFF.

This value must correspond to the

current internal setting of the

instrument.

Variable Type:  Integer

Valid Values:   0 = OFF, 1 = ON

Default:        0 = OFF

groupAddress

Variable Type       int

Control Name:  Group Address

Description:   Specifies the group address on the

RS232 (RS485) to be used to

communicate with the instrument.

This value must correspond to the

current internal group address

from the instrument.

Variable Type: Integer

Valid Range:   0 to 15

Default:       0

userAddress

Variable Type       int

Control Name:  User Adress

Description:   Specifies the user address on the

RS232 (RS485) to be used to

communicate with the instrument.

This value must correspond to the

current internal user address

from the instrument.

Variable Type: Integer

Valid Range:   0 to 15

Default:       0

instrumentIDString

Variable Type       char []

Control Name:  Instrument ID String

Description:   Returns the ID-String of the

instrument.

Variable Type: String

resetDevice

Variable Type       int

Control Name:  Reset

Description:   If reset is on, a device clear and

power on reset is done during initialization.

Variable Type: Integer

Valid Range:   0 to 1

0 = off

1 = on

instrumentID

Variable Type       int (passed by reference)

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   COM-Port is out of range

-2   Baud Rate is out of range

-3   Time-out is out of range

-4   Block Check parameter is out of range

-5   Group Address is out of range

-6   User Address is out of range

-7   Reset Device is out of range

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp4422_init_ramp1

int bp4422_init_ramp1 (int instrumentID, int waveform,

int number_ofCounts, double stepTimesec,

int type_andUnit, double startValue,

double endValue, double stepValue);

Purpose

Panel Name:    Init Ramp1

Description:   Initiated the ramp function 1

of the instrument in following

ways :

- Select the waveform

- Input the number of counts

- Input the step time

- Select the output type and unit

- Input the start value

- Input the end value

- Input the step value

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

waveform

Variable Type       int

Control Name:   Wavefrom

Description:    This parameter selected the

Waveform  of ramp1

Variable Type:  Integer

Valid Values:   0 = Triangle

1 = Sawtooth

Default:        0 = Triangle

number_ofCounts

Variable Type       int

Control Name:   Number of Counts

Description:    Specifies the number of counts

of ramp1

Variable Type:  Integer

Valid Range:    1 to 99

0 = Countiue

Default:        10

stepTimesec

Variable Type       double

Control Name:   Step Time (seconds)

Description:    Specifies  the step time

Variable Type:  Double

Valid Range:    0.1 to 99999.9 seconds

Default:        1.0

type_andUnit

Variable Type       int

Control Name:   Type and Unit

Description:    Specifies the output type and the

unit of start, stop and step

value

Variable Type:  Integer

Valid Range:    0 = Voltage [V]

1 = Voltage [mV]

2 = Current [mA]

3 = TC      [�C]

4 = TC      [�F]

5 = TC      [K]

6 = RTD     [Ohm]

7 = RTD     [�C]

8 = RTD     [�F]

9 = RTD     [K]

Default:        0 = Voltage [V]

startValue

Variable Type       double

Control Name:   Start Value

Description:    Specifies the start value of

ramp1

Variable Type:  Double

Valid Range:    by Voltage -11 to 11V

by Current -22 to 22mA

by TC      0   to 3000

by RTD     0   to 3000

Default:        0.0

endValue

Variable Type       double

Control Name:   End Value

Description:    Specifies the End value of

ramp1

Variable Type:  Double

Valid Range:    by Voltage -11 to 11V

by Current -22 to 22mA

by TC      0   to 3000

by RTD     0   to 3000

Default:        0.0

stepValue

Variable Type       double

Control Name:   Step Value

Description:    Specifies the step value of

ramp1

Variable Type:  Double

Valid Range:    by Voltage -11 to 11V

by Current -22 to 22mA

by TC      0   to 3000

by RTD     0   to 3000

Default:        0.0

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

-2   Invalid Waveform

-3   Invalid Number of Counts

-4   Invalid Step Time

-5   Invalid Type

-6   Invalid Start Value

-7   Invalid End Value

-8   Invalid Step Value

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

320   Datalogger is empty

________________________________________________________________________________

bp4422_init_ramp2

int bp4422_init_ramp2 (int instrumentID, int waveform,

int number_ofCounts, int number_ofPoints);

Purpose

Panel Name:    Init Ramp2

Description:   Initiated the ramp function 2

of the instrument in following

ways :

- Select the waveform

- Input the number of counts

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

waveform

Variable Type       int

Control Name:   Wavefrom

Description:    This parameter selected the

Waveform  of ramp2

Variable Type:  Integer

Valid Values:   0 = Triangle

1 = Sawtooth

Default:        0 = Triangle

number_ofCounts

Variable Type       int

Control Name:   Number of Counts

Description:    Specifies the number of counts

of ramp2

Variable Type:  Integer

Valid Range:    1 to 99

0 = Countiue

Default:        10

number_ofPoints

Variable Type       int

Control Name:   Number of Points

Description:    Specifies the number of points

of ramp2

Variable Type:  Integer

Valid Range:    2 to 30

Default:        2

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

-2   Invalid Waveform

-3   Invalid Number of Counts

-4   Invalid Number of points

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

320   Datalogger is empty

________________________________________________________________________________

bp4422_init_ramp2_list

int bp4422_init_ramp2_list (int instrumentID, int listNo, double value,

double stepTimesec, int type_andUnit);

Purpose

Panel Name:    Init Ramp2 List

Description:   Initiated the ramp function 2

of the instrument in following

ways :

- Select the output type and unit

- Input the List No.

- Input the List value

- Input the List step time

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

listNo

Variable Type       int

Control Name:   List No.

Description:    Specifies the List No. of the

input values

Variable Type:  Integer

Valid Range:    1 to 30

Default:        1

value

Variable Type       double

Control Name:   Value

Description:    Specifies the value of the

selected List No.

Variable Type:  Double

Valid Range:    by Voltage -11 to 11V

by Current -22 to 22mA

by TC      0   to 3000

by RTD     0   to 3000

Default:        0.0

stepTimesec

Variable Type       double

Control Name:   Step Time (seconds)

Description:    Specifies the step time of the

selected List No.

Variable Type:  Double

Valid Range:    0.1 to 99999.9 seconds

Default:        1.0

type_andUnit

Variable Type       int

Control Name:   Type and Unit

Description:    Specifies the output type and the

unit of start, stop and step

value

Variable Type:  Integer

Valid Range:    0 = Voltage [V]

1 = Voltage [mV]

2 = Current [mA]

3 = TC      [�C]

4 = TC      [�F]

5 = TC      [K]

6 = RTD     [Ohm]

7 = RTD     [�C]

8 = RTD     [�F]

9 = RTD     [K]

Default:        0 = Voltage [V]

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

-2   Invalid Type

-3   Invalid List No.

-4   Invalid Value

-5   Invalid Step Time

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

320   Datalogger is empty

________________________________________________________________________________

bp4422_ramp_start_stop

int bp4422_ramp_start_stop (int instrumentID, int startStop);

Purpose

Panel Name:    Ramp Start/Stop

Description:   Switch between ramp Start or

Stop

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

startStop

Variable Type       int

Control Name:   Ramp Start/Stop

Description:    This parameter switches the

Ramp Start/Stop of the

device ON or OFF.

Variable Type:  Integer

Valid Values:   0 = Stop, 1 = Start

Default:        0 = Stop

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

-2   Invalid Start/Stop

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

320   Datalogger is empty

________________________________________________________________________________

bp4422_ramp_step

int bp4422_ramp_step (int instrumentID, int mode);

Purpose

Panel Name:    Ramp Step

Description:   Toggle to the next ramp step

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

mode

Variable Type       int

Control Name:   Ramp Step Mode

Description:    This parameter switches the

Ramp Step of the device

ON or OFF.

Variable Type:  Integer

Valid Values:   0 = Up, 1 = Down

Default:        0 = Up

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

-2   Invalid Ramp Step

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

320   Datalogger is empty

________________________________________________________________________________

bp4422_read_datalogger

int bp4422_read_datalogger (int instrumentID, int memoryNo,

int *blockNo, char date[], char time[],

char value[]);

Purpose

Panel Name:    Read Datalogger Value as String

Description:   Read the Measure Value,Date and

Time from the selected Memory No

in the Datalogger

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

memoryNo

Variable Type       int

Control Name:   Memory No.

Description:    Specifies the datalogger MemoryNo

Variable Type:  integer

Valid Range:    1 to 256

Default:        1

blockNo

Variable Type       int (passed by reference)

Control Name:  Block No.

Description:   Returns the Block No. of the

selected Memory No.

Variable Type: String

date

Variable Type       char []

Control Name:  Date

Description:   Returns the Date of the

selected Memory No.

Variable Type: String

time

Variable Type       char []

Control Name:  Time

Description:   Returns the Time of the

selected Memory No.

Variable Type: String

value

Variable Type       char []

Control Name:  Value

Description:   Returns the value and unit of the

selected Memory No.

Variable Type: String

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

-2   Invalid Memory No.

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

320   Datalogger is empty

________________________________________________________________________________

bp4422_read_datalogger_value

int bp4422_read_datalogger_value (int instrumentID, int memoryNo,

int *blockNo, char date[],

char time[], double *value,

char unit[]);

Purpose

Panel Name:    Read Datalogger Value

Description:   Read the Measure Value,Date and

Time from the selected Memory No

in the Datalogger

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

memoryNo

Variable Type       int

Control Name:   Memory No.

Description:    Specifies the datalogger MemoryNo

Variable Type:  integer

Valid Range:    1 to 256

Default:        1

blockNo

Variable Type       int (passed by reference)

Control Name:  Block No.

Description:   Returns the Block No. of the

selected Memory No.

Variable Type: String

date

Variable Type       char []

Control Name:  Date

Description:   Returns the Date of the

selected Memory No.

Variable Type: String

time

Variable Type       char []

Control Name:  Time

Description:   Returns the Time of the

selected Memory No.

Variable Type: String

value

Variable Type       double (passed by reference)

Control Name:  Value

Description:   Returns the Value of the

selected Memory No.

Variable Type: double

unit

Variable Type       char []

Control Name:  Unit

Description:   Returns the Unit of the

selected Memory No.

Variable Type: String

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

-2   Invalid Memory No.

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

320   Datalogger is empty

________________________________________________________________________________

bp4422_read_measure

int bp4422_read_measure (int instrumentID, double *value, char unit[]);

Purpose

Panel Name:    Read Measure Value

Description:   Read the measurment value as

double and the unit

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

value

Variable Type       double (passed by reference)

Control Name:  Value

Description:   Returns the Value of the

instrument.

Variable Type: double

unit

Variable Type       char []

Control Name:  Unit

Description:   Returns the Unit of the

instrument.

Variable Type: String

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

310   Device Voltage Error

311   Device Current Error

312   Device Temperature Error

313   Device Calibration Error

314   Device In Over Error

315   Device Out Over Error

316   Device Lowbat Error

350   Measure is not started

351   Timeout => no measure values

________________________________________________________________________________

bp4422_read_measure_string

int bp4422_read_measure_string (int instrumentID, char value[]);

Purpose

Panel Name:    Read Measure String

Description:   Read the measurment value as

string

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

value

Variable Type       char []

Control Name:  Value

Description:   Returns the Value-String of the

instrument.

Variable Type: String

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

310   Device Voltage Error

311   Device Current Error

312   Device Temperature Error

313   Device Calibration Error

314   Device In Over Error

315   Device Out Over Error

316   Device Lowbat Error

317   Device Input and Output Over Error

350   Measure is not started

351   Timeout => no measure values

________________________________________________________________________________

bp4422_read_p_measure

int bp4422_read_p_measure (int instrumentID, double *pressureValue,

char pressureUnit[]);

Purpose

Panel Name:    Read Pressure Measure Value

Description:   Read the pressure measurment value as

double and the unit

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

pressureValue

Variable Type       double (passed by reference)

Control Name:  Presure Value

Description:   Returns the presure value of the

instrument.

Variable Type: double

pressureUnit

Variable Type       char []

Control Name:  Presure Unit

Description:   Returns the presure unit of the

instrument.

Variable Type: String

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

310   Device Voltage Error

311   Device Current Error

312   Device Temperature Error

313   Device Calibration Error

314   Device In Over Error

315   Device Out Over Error

316   Device Lowbat Error

317   Device Input and Output Over Error

318   Device Presure Over Error

350   Measure is not started

351   Timeout => no measure values

________________________________________________________________________________

bp4422_read_p_measure_string

int bp4422_read_p_measure_string (int instrumentID,

char pressureValue[]);

Purpose

Panel Name:    Read Pressure Measure String

Description:   Read the pressure measurement value as

string

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

pressureValue

Variable Type       char []

Control Name:  Presure Value

Description:   Returns the presure value-String of the

instrument.

Variable Type: String

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

310   Device Voltage Error

311   Device Current Error

312   Device Temperature Error

313   Device Calibration Error

314   Device In Over Error

315   Device Out Over Error

316   Device Lowbat Error

317   Device Input and Output Over Error

318   Device Presure Over Error

350   Measure is not started

351   Timeout => no measure values

________________________________________________________________________________

bp4422_receive_scpi

int bp4422_receive_scpi (int instrumentID, char sendSCPIMessage[],

char instrumentAnswers[]);

Purpose

Panel Name:   Receive a SCPI-Message

Description:  Sends "Query form" from SCPI-

Message and receive a answers

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

sendSCPIMessage

Variable Type       char []

Control Name:  Send SCPI-Message

Description:   This control is a string parameter

You must input Query-Form from

the SCPI-message without the

Line Feed termination.

Example      : ":SENS:CORR:STATE?"

Variable Type: String

instrumentAnswers

Variable Type       char []

Control Name:  Answer from Slave

Description:   This control return a string from

the slave.

Variable Type: String (Length => see the 6706

Manual SCPI-Commands)

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp4422_select_p_module_name

int bp4422_select_p_module_name (int instrumentID, char moduleName[]);

Purpose

Panel Name:    Select Pressure Module by Name.

Description:   Select the active pressure module by name.

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

moduleName

Variable Type       char []

Control Name:   Module Name

Description:    This parameter select the module name.

Variable Type:  String

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Mode

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp4422_select_p_module_no

int bp4422_select_p_module_no (int instrumentID, int moduleNo);

Purpose

Panel Name:    Select Pressure Module by No.

Description:   Select the active pressure module by no.

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

moduleNo

Variable Type       int

Control Name:   Module No

Description:    This parameter select the module no.

Variable Type:  Integer

Valid Values:   0-9

Default:        0

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Mode

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp4422_send_scpi

int bp4422_send_scpi (int instrumentID, char sendSCPIMessage[]);

Purpose

Panel Name:   Send a SCPI-Message

Description:  Sends a SCPI-Message to a

slave

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

sendSCPIMessage

Variable Type       char []

Control Name:  Send SCPI-Message

Description:   This control is a string parameter

You must input the SCPI-message

without the LineFeed termination.

Example      : ":SENS:CORR:STATE ON"

Variable Type: String

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp4422_set_datalogger_mode

int bp4422_set_datalogger_mode (int instrumentID, int mode);

Purpose

Panel Name:    Datalogger Mode

Description:   Switch the datalogger mode of the

instrument ON/OFF

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

mode

Variable Type       int

Control Name:   Datalogger mode

Description:    This parameter switches the

Datalogger Mode of the device

ON or OFF.

Variable Type:  Integer

Valid Values:   0 = OFF, 1 = ON

Default:        0 = OFF

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

-2   Invalid Mode

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

320   Datalogger is empty

________________________________________________________________________________

bp4422_set_datalogger_time

int bp4422_set_datalogger_time (int instrumentID, double time);

Purpose

Panel Name:    Sets Datalogger Time

Description:   Sets the Timer Value in the

Datalogger

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

time

Variable Type       double

Control Name:   Datalogger Time (seconds)

Description:    Specifies  the datalogger timer

Variable Type:  Double

Valid Range:    0.0 to 3600.0 seconds

Default:        1.0

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

-2   Invalid Time

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

320   Datalogger is empty

________________________________________________________________________________

bp4422_set_display_backlight

int bp4422_set_display_backlight (int instrumentID, int state,

double time);

Purpose

Panel Name:    Display Backlight

Description:   Switch the Display Backlight ON

or Off and selected the time of

when the Backlight is on

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

state

Variable Type       int

Control Name:   Backlight State

Description:    This parameter switches the

Backlight of the device display

ON or OFF.

Variable Type:  Integer

Valid Values:   0 = OFF, 1 = ON

Default:        0 = OFF

time

Variable Type       double

Control Name:   Backlight Time

Description:    This parameter inputs the time

when the display backlight is

on.

Variable Type:  Double

Valid Values:   1.0 to 60.0 Seconds.

Default:        1.0

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

-2   Invalid State

-3   Invalid Time

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

320   Datalogger is empty

________________________________________________________________________________

bp4422_set_display_contrast

int bp4422_set_display_contrast (int instrumentID, double contrast);

Purpose

Panel Name:    Display Contrast

Description:   Adjust the display contrast of the

instrument

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

contrast

Variable Type       double

Control Name:   Display Contrast

Description:    Specifies the dispaly contrast

Variable Type:  Double

Valid Range:    0.0 to 1.0

Default:        0.5

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

-2   Invalid Contrast

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

320   Datalogger is empty

________________________________________________________________________________

bp4422_set_ramp_mode

int bp4422_set_ramp_mode (int instrumentID, int mode);

Purpose

Panel Name:    Ramp Mode

Description:   Switch the ramp mode of the

instrument

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

mode

Variable Type       int

Control Name:   Ramp Mode

Description:    This parameter switches the

Ramp Mode of the

Variable Type:  Integer

Valid Values:   0 = OFF

1 = Ramp1

2 = Ramp2

Default:        0 = OFF

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

-2   Invalid Ramp mode

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

320   Datalogger is empty

________________________________________________________________________________

bp4422_set_value

int bp4422_set_value (int instrumentID, int outputType, int range,

double value);

Purpose

Panel Name:    Sets a Value

Description:   Sets a Output value of the

instrument in following ways:

- Selects the output type

- Selects the output range

- Input the output value

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

outputType

Variable Type       int

Control Name:  Output Type

Description:   Specifices the Output type of

the instrument .

Variable Type: Integer

Valid Range:   0 = Voltage

1 = Current

2 = Resistance

3 = TCouple

Default Range: 0 = Voltage

range

Variable Type       int

Control Name:  Range

Description:   Specifices the Output Range of

the instrument .

Variable Type : Integer

Valid Range : 0 - Autorange

1 -  10mV, 2mA,200�

2 - 100mV,20mA,2K�

3 -    1V,----,----

4 -   101V,----,----

value

Variable Type       double

Control Name:  Voltage Value

Description:   Specifices the voltage output

value of the instrument .

Variable Type: double

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

-2   Invalid Output Type

-3   Invalid Output Range

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp4422_start_measure

int bp4422_start_measure (int instrumentID, int type);

Purpose

Panel Name:    Start Measuring

Description:   Start the measuring of instrument

in following ways:

- Selects the input type of the

instrument and start the

measuring

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

type

Variable Type       int

Control Name:  Type

Description:   Selects the measuring type of

the instrument .

Variable Type: Integer

Valid Range:   0 = Voltage

1 = Current

2 = Resistance

3 = TCouple

4 = Presure

5 = Presure and Current

6 = Presure and Voltage

Default Range: 0 = Voltage

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

-2   Invalid Type

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

310   Device Voltage Error

311   Device Current Error

312   Device Temperature Error

313   Device Calibration Error

314   Device In Over Error

315   Device Out Over Error

316   Device Lowbat Error

317   Device Input and Output Over Error

________________________________________________________________________________

bp4422_stop_measure

int bp4422_stop_measure (int instrumentID);

Purpose

Panel Name:    Stop Measuring

Description:   Abort the measure

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

310   Device Voltage Error

311   Device Current Error

312   Device Temperature Error

313   Device Calibration Error

314   Device In Over Error

315   Device Out Over Error

316   Device Lowbat Error

________________________________________________________________________________

bp4422_store_p_module_name

int bp4422_store_p_module_name (int instrumentID, int moduleNo,

char name[]);

Purpose

Panel Name:    Set Pressure Module Name

Description:   Set pressure module name

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

moduleNo

Variable Type       int

Control Name:   Module No

Description:    This parameter select the module no.

Variable Type:  Integer

Valid Values:   0-9

Default:        0

name

Variable Type       char []

Control Name:   Name

Description:    This parameter input the module name

Variable Type:  String

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Module No

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp4422_system_error

int bp4422_system_error (int instrumentID, char systemError[]);

Purpose

Panel Name:   System Error

Description:  Reads the system error of the

device

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

systemError

Variable Type       char []

Control Name:  System Error

Description:   Displays the SCPI system error

Variable Type: String

(see manual Resistomat Type 6706)

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1   Invalid Instrument ID

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp4422_system_keyboard

int bp4422_system_keyboard (int instrumentID, int keyboard);

Purpose

Panel Name:   System Keybroad

Description:  Controls the keybroad of the device

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

keyboard

Variable Type       int

Control Name:  Keyboard

Description:   This parameter locks the keyboard

on the front panel of the

instrument

Variable Type: Integer

Valid Range:   0  = Unlocked

1  = Locked

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Value

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp4422_tare_p_init

int bp4422_tare_p_init (int instrumentID);

Purpose

Panel Name:    Tare Pressure Initate

Description:   This function initate the tare measurement

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

319   The module batterie is lower than 100mV

________________________________________________________________________________

bp4422_tare_p_state

int bp4422_tare_p_state (int instrumentID, int state);

Purpose

Panel Name:    Tare Pressure State

Description:   Sets the tare state ON or OFF

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

state

Variable Type       int

Control Name:   State

Description:    This parameter switches the

tare presure state of the device

ON or OFF.

Variable Type:  Integer

Valid Values:   0 = OFF, 1 = ON

Default:        0 = OFF

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid State

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT