Burster Digistant Type 6706

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for Burster Digistant Type 6706.

It contains functions for opening, configuring, taking measurements from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

For GPIB instrument drivers:

-  the instrument is connected to the GPIB.

-  the GPIB address supplied to the initialize function must match the

GPIB address of the instrument.

For VXI instrument drivers:

-  the instrument is installed in the VXI mainframe and you are

using one of the following controller options:

Embedded controller

MXI

MXI2

GPIB-VXI

-  the logical address supplied to the initialize function must

match the logical address of the instrument.

For RS-232 instrument drivers:

-  the instrument is connected to the RS-232 interface.

-  the COM port, baud rate, parity, and timeout supplied to the

initialize function must match the settings of the instrument.

________________________________________________________________________________

Error and Status Information:

Each function in this instrument driver returns a status code that

either indicates success or describes an error or warning condition.

Your program should examine the status code from each call to an

instrument driver function to determine if an error occurred.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

The description of each instrument driver function lists possible error

codes and their meanings

________________________________________________________________________________

How To Use This Document:

Use this document as a programming reference manual.

It describes each function in the

Burster Digistant Type 6706

instrument.  The functions appear in alphabetical order, with a description

of the function and its Visual Basic syntax, a description of each parameter,

and a list of possible error codes.

________________________________________________________________________________

Function Tree Layout:

Class/Panel Name:                    Function Name:

Initialize

Initialize GPIB                bp6706_init_gpib

Initialize Serial              bp6706_init_serial

Local (GPIB only)              bp6706_local_gpib

Configure

Sets Voltage Configure         bp6706_set_volt_config

Gets Voltage Configure         bp6706_get_volt_config

Sets Current Configure         bp6706_set_curr_config

Gets Current Configure         bp6706_get_curr_config

Output Values

Sets Voltage Value             bp6706_set_volt_value

Gets Voltage Value             bp6706_get_volt_value

Sets Current Value             bp6706_set_curr_value

Gets Current Value             bp6706_get_curr_value

System

System Error                   bp6706_system_error

System Keyboard                bp6706_system_keyboard

Send a SCPI-Message            bp6706_send_scpi

Receive a SCPI-Message         bp6706_receive_scpi

Close                             bp6706_close

________________________________________________________________________________

Burster Digistant Type 6706

Instrument Name: burster DIGISTANT Type 6706

Description:   This instrument module provides

GPIB,RS232 and RS485(full duplex)

support for DIGISTANT Type 6706

from burster.

Functions and Classes:

(1) Initialize :

Initialize the instrument and sets to

default configuration

(2) Configure:

Configures the instrument by setting the

range and protection mode

(3) Set Voltage :

Sets the voltage value of the DIGISTANT 6706

(4) Get Voltage :

Gets the voltage value of the DIGISTANT 6706

(5) Set Current :

Sets the current value of the DIGISTANT 6706

(6) Get Current :

Gets the current value of the DIGISTANT 6706

(7) System:

Reads the system error

Reads the instrument ID-String

Controls the keyboard

Sends a SCPI-Message

Receives a SCPI-Message

(8) Clear Device :

Clear the device.

(9) Close :

Takes the instrument off-line.

*********

________________________________________________________________________________

The following functions are in alphabetical order.

________________________________________________________________________________

bp6706_close

Function bp6706_close& (ByVal instrumentID&)

Purpose

Panel Name:    Close

Description:   Takes the instrument off-line.

Note: The  instrument must  be reinitialized

to use it again.

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

221   Unable to close instrument

232   Instrument not initialized

________________________________________________________________________________

bp6706_get_curr_config

Function bp6706_get_curr_config& (ByVal instrumentID&, currentRange&, voltageLimitV#)

Purpose

Panel Name:    Gets Current Configure

Description:   Configure the instrument in

following ways:

- Gets the current range of the

instrument

- Gets the voltage protection

level

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

currentRange&

Control Name:  Current Range

Description:   Displays the current range of

the instrument .

Variable Type: Integer

0 = Auto Range

1 = 2   mA

2 = 20  mA

3 = 200 mA

voltageLimitV#

Control Name:  Voltage Limit

Description:   Displays the voltage protection

limit of all current ranges of the

instrument .

Variable Type: double

Value Unit:    V

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp6706_get_curr_value

Function bp6706_get_curr_value& (ByVal instrumentID&, currentValuemA#)

Purpose

Panel Name:    Gets Current Configure

Description:   Configure the instrument in

following ways:

- Gets the current range of the

instrument

- Gets the voltage protection

level

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

currentValuemA#

Control Name:  Current Value

Description:   Displays the current output value.

Variable Type: double

Value Unit:    mA

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp6706_get_volt_config

Function bp6706_get_volt_config& (ByVal instrumentID&, voltageRange&, currentLimitmA#, voltageSense&)

Purpose

Panel Name:    Gets Voltage Configure

Description:   Configure the instrument in

following ways:

- Gets the voltage range of the

instrument

- Gets the sense function

Internal/External

- Gets the current protection

level

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

voltageRange&

Control Name:  Voltage Range

Description:   Displays the voltage range.

Variable Type: Integer

0 = Auto Range

1 = 20  mV

2 = 200 mV

3 = 2   V

4 = 20  V

5 = 180 V

currentLimitmA#

Control Name:  Current Limit

Description:   Displays the current protection

limit of all voltage ranges of the

instrument .

Variable Type: double

Value Unit:    mA

voltageSense&

Control Name:  Voltage Sense

Description:   Displays the adjustment of the

sense by output voltage.

Variable Type: Integer

0 = Internal Sense

1 = External Sense

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp6706_get_volt_value

Function bp6706_get_volt_value& (ByVal instrumentID&, voltageValueV#)

Purpose

Panel Name:    Gets Voltage Configure

Description:   Configure the instrument in

following ways:

- Gets the voltage range of the

instrument

- Gets the sense function

Internal/External

- Gets the current protection

level

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

voltageValueV#

Control Name:  Voltage Value

Description:   Displays the voltage output value.

Variable Type: double

Value Unit:    V

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp6706_init_gpib

Function bp6706_init_gpib& (ByVal address&, ByVal reset&, ByVal instrumentIDString$, instrumentID&)

Purpose

Panel Name:    Initialize Gpib

Description:   Initializes the instrument by:

1) finding an unused device in the device table

using the supplied GPIB address

2) device clear and RESET  (if desired)

3) returns the ID-String of the instrument

4) returns an instrument ID

This is a multi-instrument driver, allowing

multiple Digistant 6706 instruments to be

controlled.

During the initialization, the returned ID is

based on the GPIB address.  This ID is used in

all of the other functions to distinguish

between multiple Digistant 6706 instruments.

Parameter List

ByVal address&

Control Name:  Address

Description:   Specifies the address on the GPIB

to be used to communicate with the instrument.

This value must correspond to the current

internal setting of the instrument.

Variable Type: Integer

Valid Range:   0 to 30

ByVal reset&

Control Name:  Reset

Description:   If reset is on, a device clear and

power on reset is done during initialization.

Variable Type: Integer

Valid Range:   0 to 1

0  off

1  on

ByVal instrumentIDString$

Control Name:  Instrument ID String

Description:   Returns the ID-String of the

instrument.

Variable Type: String

instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid GPIB Address

-2     Invalid Reset

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

________________________________________________________________________________

bp6706_init_serial

Function bp6706_init_serial& (ByVal COMPort&, ByVal baudRate&, ByVal timeout_seconds#, ByVal blockCheck&, ByVal groupAddress&, ByVal userAddress&, ByVal instrumentIDString$, ByVal reset&, instrumentID&)

Purpose

Panel Name:    Initialize Serial

Description:   Initializes the instrument by:

1) finding an unused device in the device table

using the supplied serial group and user

address

2) device clear and RESET  (if desired)

3) returns the ID-String of the instrument

4) returns an instrument ID

This is a multi-instrument driver, allowing

multiple Digistant 6706 instruments to be

controlled.

During the initialization, the returned ID is

based on the serial group and user address.

This ID is used in all of the other functions

to distinguish between multiple Digistant

6706 instruments.

Parameter List

ByVal COMPort&

Control Name:   COM-Port

Description:    Specifies the COM-Port to be

used to communicate with the

instrument.

Variable Type:  Integer

Valid Values:   1 - COM1   (=> Adress 0x3f8 IRQ4)

2 - COM2   (=> Adress 0x2f8 IRQ3)

3 - COM3   (=> Adress 0x3e8 IRQ4)

4 - COM4   (=> Adress 0x2e8 IRQ3)

Default:        1

ByVal baudRate&

Control Name:   Baud Rate

Description:    Specifies the baud rate of the

RS232-Port to communicate with

the instrument. This value must

correspond to the current

internal setting of the

instrument.

Variable Type:  Integer

Valid Values:   1200,2400,4800,9600 or 19200 baud

Default:        9600

ByVal timeout_seconds#

Control Name:   Time-out (seconds)

Description:    Specifies  the time-out limit for

I/O operations.

A  value  of  0.0  will  disable

time-out altogether.

Variable Type:  Double

Valid Range:    0.060 to 100.0 seconds

Default:        1.0

ByVal blockCheck&

Control Name:   Bcc-Check

Description:    This parameter switches the block

check function of the driver ON

or OFF.

This value must correspond to the

current internal setting of the

instrument.

Variable Type:  Integer

Valid Values:   0 = OFF, 1 = ON

Default:        0 = OFF

ByVal groupAddress&

Control Name:  Group Address

Description:   Specifies the group address on the

RS232 (RS485) to be used to

communicate with the instrument.

This value must correspond to the

current internal group address

from the instrument.

Variable Type: Integer

Valid Range:   0 to 15

Default:       0

ByVal userAddress&

Control Name:  User Adress

Description:   Specifies the user address on the

RS232 (RS485) to be used to

communicate with the instrument.

This value must correspond to the

current internal user address

from the instrument.

Variable Type: Integer

Valid Range:   0 to 15

Default:       0

ByVal instrumentIDString$

Control Name:  Instrument ID String

Description:   Returns the ID-String of the

instrument.

Variable Type: String

ByVal reset&

Control Name:  Reset

Description:   If reset is on, a device clear and

power on reset is done during initialization.

Variable Type: Integer

Valid Range:   0 to 1

0 = off

1 = on

instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   COM-Port is out of range

-2   Baud Rate is out of range

-3   Time-out is out of range

-4   Block Check parameter is out of range

-5   Group Address is out of range

-6   User Address is out of range

-7   Reset Device is out of range

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp6706_local_gpib

Function bp6706_local_gpib& (ByVal instrumentID&)

Purpose

Panel Name:    Local (GPIB only)

Description:   Sets the multimeter temporarily in

local mode.

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1     Invalid Instrument ID

220    Unable to open instrument

221    Unable to close instrument

224    Error clearing instrument

230    Error writing to instrument

231    Error reading from instrument

232    Instrument not initialized

233    Error configuring GPIB address

236    Error interpreting instrument response

237    Error configuring EOS terminator

________________________________________________________________________________

bp6706_receive_scpi

Function bp6706_receive_scpi& (ByVal instrumentID&, ByVal sendSCPIMessage$, ByVal instrumentAnswers$)

Purpose

Panel Name:   Receive a SCPI-Message

Description:  Sends "Query form" from SCPI-

Message and receive a answers

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal sendSCPIMessage$

Control Name:  Send SCPI-Message

Description:   This control is a string parameter

You must input Query-Form from

the SCPI-message without the

Line Feed termination.

Example      : ":SENS:CORR:STATE?"

Variable Type: String

ByVal instrumentAnswers$

Control Name:  Answer from Slave

Description:   This control return a string from

the slave.

Variable Type: String (Length => see the 6706

Manual SCPI-Commands)

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp6706_send_scpi

Function bp6706_send_scpi& (ByVal instrumentID&, ByVal sendSCPIMessage$)

Purpose

Panel Name:   Send a SCPI-Message

Description:  Sends a SCPI-Message to a

slave

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal sendSCPIMessage$

Control Name:  Send SCPI-Message

Description:   This control is a string parameter

You must input the SCPI-message

without the LineFeed termination.

Example      : ":SENS:CORR:STATE ON"

Variable Type: String

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp6706_set_curr_config

Function bp6706_set_curr_config& (ByVal instrumentID&, ByVal currentRange&, ByVal voltageLimitV#)

Purpose

Panel Name:    Sets Current Configure

Description:   Configure the instrument in

following ways:

- Selects the current range of the

instrument

- Input the voltage protection

level

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal currentRange&

Control Name:  Current Range

Description:   Specifices the current range of

the instrument .

Variable Type: Integer

Valid Range:   0 = Auto Range

1 = 2   mA

2 = 20  mA

3 = 200 mA

Default Range: 0 = Auto Range

ByVal voltageLimitV#

Control Name:  Voltage Limit

Description:   Specifices the voltage protection

limit of all current ranges of the

instrument .

Variable Type: double

Valid Range:   0.1V-180V (Current range 2mA-20mA)

0.1V-20V  (Current range 200mA)

Default Value: 20V

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Range

-3   Invalid Limit

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp6706_set_curr_value

Function bp6706_set_curr_value& (ByVal instrumentID&, ByVal currentValue#, ByVal currentUnit&)

Purpose

Panel Name:    Sets Current Configure

Description:   Configure the instrument in

following ways:

- Selects the current range of the

instrument

- Input the voltage protection

level

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal currentValue#

Control Name:  Current Value

Description:   Specifices the current output

value of the instrument .

Variable Type: double

ByVal currentUnit&

Control Name:  Current Unit

Description:   Specifices the current output

unit of the instrument .

Variable Type: Integer

0 = uA

1 = mV

Default Value: 1 = mA

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Unit

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

310   Device Voltage Error

311   Device Current Error

312   Device Over Error

________________________________________________________________________________

bp6706_set_volt_config

Function bp6706_set_volt_config& (ByVal instrumentID&, ByVal voltageRange&, ByVal currentLimitmA#, ByVal voltageSense&)

Purpose

Panel Name:    Set Voltage Configure

Description:   Configure the instrument in

following ways:

- Selects the voltage range of the

instrument

- Switches the sense function

Internal/External

- Input the current protection

level

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal voltageRange&

Control Name:  Range

Description:   Specifices the range of the

instrument .

Variable Type: Integer

Valid Range:   0 = Auto Range

1 = 20  mV

2 = 200 mV

3 = 2   V

4 = 20  V

5 = 180 V

Default Range: 0 = Auto Range

ByVal currentLimitmA#

Control Name:  Current Limit

Description:   Specifices the current protection

limit of all voltage ranges of the

instrument .

Variable Type: double

Valid Range:   0.1mA-200mA (Voltage range

20mV-20V)

0.1mA-20mA (Voltage range 180V)

Default Value: 20 mA

ByVal voltageSense&

Control Name:  Voltage Sense

Description:   Specifices the adjustment of the

sense by output voltage.

Variable Type: Integer

Valid Range:   0 to 1

0 = Internal Sense

1 = External Sense

Default Value: 0 = Internal Sense

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Range

-4   Invalid Sense

-3   Invalid Current Limit

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp6706_set_volt_value

Function bp6706_set_volt_value& (ByVal instrumentID&, ByVal voltageValue#, ByVal voltageUnit&)

Purpose

Panel Name:    Set Voltage Configure

Description:   Configure the instrument in

following ways:

- Selects the voltage range of the

instrument

- Switches the sense function

Internal/External

- Input the current protection

level

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal voltageValue#

Control Name:  Voltage Value

Description:   Specifices the voltage output

value of the instrument .

Variable Type: double

ByVal voltageUnit&

Control Name:  Voltage Unit

Description:   Specifices the voltage output

unit of the instrument .

Variable Type: Integer

0 = uV

1 = mV

2 = V

Default Value: 2 = V

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

-2   Invalid Unit

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

310   Device Voltage Error

311   Device Current Error

312   Device Over Error

________________________________________________________________________________

bp6706_system_error

Function bp6706_system_error& (ByVal instrumentID&, ByVal systemError$)

Purpose

Panel Name:   System Error

Description:  Reads the system error of the

device

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal systemError$

Control Name:  System Error

Description:   Displays the SCPI system error

Variable Type: String

(see manual Resistomat Type 6706)

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1   Invalid Instrument ID

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp6706_system_keyboard

Function bp6706_system_keyboard& (ByVal instrumentID&, ByVal keyboard&)

Purpose

Panel Name:   System Keybroad

Description:  Controls the keybroad of the device

Parameter List

ByVal instrumentID&

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

ByVal keyboard&

Control Name:  Keyboard

Description:   This parameter locks the keyboard

on the front panel of the

instrument

Variable Type: Integer

Valid Range:   0  = Unlocked

1  = Locked

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Value

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT