Burster Process Interface Type 9714

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for Burster Process Interface Type 9714.

It contains functions for opening, configuring, taking measurements from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

For GPIB instrument drivers:

-  the instrument is connected to the GPIB.

-  the GPIB address supplied to the initialize function must match the

GPIB address of the instrument.

For VXI instrument drivers:

-  the instrument is installed in the VXI mainframe and you are

using one of the following controller options:

Embedded controller

MXI

MXI2

GPIB-VXI

-  the logical address supplied to the initialize function must

match the logical address of the instrument.

For RS-232 instrument drivers:

-  the instrument is connected to the RS-232 interface.

-  the COM port, baud rate, parity, and timeout supplied to the

initialize function must match the settings of the instrument.

________________________________________________________________________________

Error and Status Information:

Each function in this instrument driver returns a status code that

either indicates success or describes an error or warning condition.

Your program should examine the status code from each call to an

instrument driver function to determine if an error occurred.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

The description of each instrument driver function lists possible error

codes and their meanings

________________________________________________________________________________

How To Use This Document:

Use this document as a programming reference manual.

It describes each function in the

Burster Process Interface Type 9714

instrument.  The functions appear in alphabetical order, with a description

of the function and its C syntax, a description of each parameter, and a list

of possible error codes.

________________________________________________________________________________

Function Tree Layout:

Class/Panel Name:                     Function Name:

Initialize

Initialize One Device           bp9714_init_device

Initialize All Devices          bp9714_init_all_devices

Init. RS485 (CVI only)          bp9714_init_rs485

Calibrate

Calibration State               bp9714_cal_state

Read Calibration State          bp9714_read_cal_state

Sets Calibration Unit           bp9714_cal_set_unit

Read Calibration Unit           bp9714_read_cal_unit

Calibration Input               bp9714_cal_input

Read Calibration Input          bp9714_read_cal_input

Calibration Ana. Output         bp9714_cal_analogue_output

Read Calibration Ana. Output    bp9714_read_cal_analogue_output

Calibration Input Meas.         bp9714_cal_input_meas

Read Calibration Input Meas.    bp9714_read_cal_input_meas

Accept Calibration Datas        bp9714_cal_accept_datas

Configure

Config Device                   bp9714_config

Read Config Device              bp9714_read_config

Config Output                   bp9714_config_output

Read Config Output              bp9714_read_config_output

Config Analog Output            bp9714_config_analog_output

Read Config Analog Output       bp9714_read_config_analog_output

Config Input                    bp9714_config_input

Read Config Input               bp9714_read_config_input

Comparator

Comparator State                bp9714_comp_state

Read Comparator State           bp9714_read_comp_state

Read_comp_count                 bp9714_read_comp_count

Comparator Config               bp9714_comp_config

Read Comparator Config          bp9714_read_comp_config

Two Limit Comparator            bp9714_comp_two_limit

Read Two Limit Comparator       bp9714_read_comp_two_limit

Four Limit Comparator           bp9714_comp_four_limit

Read Four Limit Comparator      bp9714_read_comp_four_limit

Peak Detection

Peak Detection State            bp9714_peak_detection_state

Read Peak Detection State       bp9714_read_peak_detection_state

Read Peak Values                bp9714_read_peak_values

Tare

Tare State                      bp9714_tare_state

Read Tare State                 bp9714_read_tare_state

Read Tare Value                 bp9714_read_tare_value

Measuring

Start Measure                   bp9714_start_measure

Read Measure Value              bp9714_read_measure

Read Measure String             bp9714_read_string_measure

Stop Measure                    bp9714_stop_measure

System Serial Comm.

Set Serial Parameters           bp9714_system_serial_set_para

Read Serial Parameters          bp9714_read_system_serial_para

Accept New Serial Para          bp9714_system_serial_accept

System

Store Memory No.                bp9714_store_memory

Get Status                      bp9714_get_status

Recall Memory No.               bp9714_recall_memory

Set Display Contrast            bp9714_set_display_contrast

Read Display Contrast           bp9714_read_display_contrast

System Error                    bp9714_system_error

System Keyboard                 bp9714_system_keyboard

System Security                 bp9714_system_security

Read System Security            bp9714_read_system_security

Send a SCPI-Message             bp9714_send_scpi

Receive a SCPI-Message          bp9714_receive_scpi

Datalogger

Logg state                      bp9714_logg_state

Read Logg State                 bp9714_read_logg_state

Clear Block                     bp9714_clear_block

Select Block                    bp9714_select_block

Read Selected Block             bp9714_read_selected_block

Read Count                      bp9714_read_count

Read Data                       bp9714_read_data

Set Block Parameter             bp9714_set_block_parameter

Read Block Parameter            bp9714_read_block_parameter

Read Eval                       bp9714_read_eval

Read Data Block                 bp9714_read_data_block

Close                              bp9714_close

________________________________________________________________________________

Burster Process Interface Type 9714

Instrument Name: burster Process Interface

Type 9714

Description:   This instrument module provides

RS232, RS485 (fullduplex only)

support for Process Interface

Type 9714 from burster.

Functions and Classes:

(1) Initialize :

Initialize the instrument

(2) Calibrate:

Calibrate functions of the Process Interface

(3) Calculate:

Calculate settings of the instrument

(4) Configure:

Configures the instrument

(5) Measuring :

Measuring functions of the Process Interface

(6) Peak Detection:

Peak Detection functions of the Process

Interface

(7) Tare :

Tare functions of the Process Interface

(8) System Serial Communication

Change the Communication Parameters

of the instrument

(9) System:

Display Contrast

Reads the system error

Reads the instrument ID-String

Controls the keyboard

Sends a SCPI-Message

Receives a SCPI-Message

(10) Close :

Takes the instrument off-line.

*********

________________________________________________________________________________

The following functions are in alphabetical order.

________________________________________________________________________________

bp9714_cal_accept_datas

int bp9714_cal_accept_datas (int instrumentID);

Purpose

Panel Name:    Accept calibration datas

Description:   The calibration values from

followed function are accept

- Calibration Input /Meas.

- Calibration Analog Output

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

330   The devicve don't accept the

calibration datas

________________________________________________________________________________

bp9714_cal_analogue_output

int bp9714_cal_analogue_output (int instrumentID, double lowValue,

double low_Scale, double highValue,

double high_Scale);

Purpose

Panel Name:    Calibrate Analogue Output

Description:   Calibrate the instrument in

following ways:

- Calibrate the low and

high value of the analogue

Output

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

lowValue

Variable Type       double

Control Name:  Low  Value

Description:   Specifies the low calibration

value of the analogue output

Variable Type: Double

Default:       0.0

low_Scale

Variable Type       double

highValue

Variable Type       double

Control Name:  High Value

Description:   Specifies the high calibration

value of the analogue output

Variable Type: Double

Default:       0.0

high_Scale

Variable Type       double

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_cal_input

int bp9714_cal_input (int instrumentID, double lowCalValue,

double highCalValue, double lowScaleValue,

double highScaleValue);

Purpose

Panel Name:    Calibrate Input

Description:   Calibrate the instrument in

following ways:

- Input the Calibration low and

high Value

- Input the Scale low and

high Value

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

lowCalValue

Variable Type       double

Control Name:  Low Cal. Value

Description:   Specifies the low calibration

value

Variable Type: Double

Default:       0.0

highCalValue

Variable Type       double

Control Name:  High Cal. Value

Description:   Specifies the high calibration

value

Variable Type: Double

Default:       0.0

lowScaleValue

Variable Type       double

Control Name:  Low Scale value

Description:   Specifies the low scale value

Variable Type: Double

Default:       0.0

highScaleValue

Variable Type       double

Control Name:  High Scale Value

Description:   Specifies the high Scale Value

Variable Type: Double

Default:       0.0

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_cal_input_meas

int bp9714_cal_input_meas (int instrumentID, int measure);

Purpose

Panel Name:    Calibrate Input Meas

Description:   Start to measure the calibration

value

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

measure

Variable Type       int

Control Name:  Measure

Description:   This parameter starts to measure

the calibration Value

Variable Type: Integer

Valid Range:   0 = Low Value

1 = High Value

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Measure

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_cal_set_unit

int bp9714_cal_set_unit (int instrumentID, int calibrationUnit);

Purpose

Panel Name:    Sets Calibration Unit

Description:   Set the Calibration Unit

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

calibrationUnit

Variable Type       int

Control Name:  Unit

Description:   This parameter selects the

calibration unit

Variable Type: Integer

Valid Range:   0 = mV

1 = mV/V

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Calibration Unit

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_cal_state

int bp9714_cal_state (int instrumentID, int state);

Purpose

Panel Name:    Calibration State

Description:   Sets the calibration State ON/OFF

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

state

Variable Type       int

Control Name:  State

Description:   If the State is On the calibration

functions are possible to activate

Variable Type: Integer

Valid Range:   0 = Off

1 = On

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid State

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_clear_block

int bp9714_clear_block (int instrumentID, int block);

Purpose

Panel Name:    Read Comparator State

Description:   Reads the Comparator State

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

block

Variable Type       int

Control Name:  State

Description:   If the State is On the comparator

is active

Variable Type: Integer

Valid Range:   0 = Off

1 = On

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid State

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_close

int bp9714_close (int instrumentID);

Purpose

Panel Name:    Close

Description:   Takes the instrument off-line.

Note: The  instrument must  be reinitialized

to use it again.

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

221   Unable to close instrument

232   Instrument not initialized

________________________________________________________________________________

bp9714_comp_config

int bp9714_comp_config (int instrumentID, int comparatorType,

int comparatorControl);

Purpose

Panel Name:    Comparator Config

Description:   Configure the comparator functions

of the instrument in following

ways:

- Selects the comparator type

- Selects the comparator Control

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

comparatorType

Variable Type       int

Control Name:  Comparator Type

Description:   Switches the comparator type

Variable Type: Integer

Default:       0 = Dynamic

Valid Range:   0 = Dynamic

1 = Static

comparatorControl

Variable Type       int

Control Name:  Comparator Control

Description:   Switches the comparator Control

Variable Type: Integer

Default:       0 = Single

Valid Range:   0 = Single

1 = Average

2 = Minimum

3 = Maximum

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Comparator Type

-3   Invalid Comparator Control

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_comp_four_limit

int bp9714_comp_four_limit (int instrumentID, double lowerLimit1,

double lowerLimit2, double upperLimit1,

double upperLimit2);

Purpose

Panel Name:    Four Limit Comparator

Description:   Configure the four limit

comparator functions

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

lowerLimit1

Variable Type       double

Control Name:  Lower Limit 1

Description:   Specifies the lower limit of the

two limit comparator 1

Variable Type: Double

Default:       0.0

lowerLimit2

Variable Type       double

Control Name:  Lower Limit 2

Description:   Specifies the lower limit of the

two limit comparator 2

Variable Type: Double

Default:       0.0

upperLimit1

Variable Type       double

Control Name:  Upper Limit1

Description:   Specifies the upper limit of the

two limit comparator 1

Variable Type: Double

Default:       0.0

upperLimit2

Variable Type       double

Control Name:  Upper Limit2

Description:   Specifies the upper limit of the

two limit comparator 2

Variable Type: Double

Default:       0.0

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

330   Invalid Comparator Datas

________________________________________________________________________________

bp9714_comp_state

int bp9714_comp_state (int instrumentID, int state);

Purpose

Panel Name:    Comparator State

Description:   Sets the Comparator State ON/OFF

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

state

Variable Type       int

Control Name:  State

Description:   If the State is On the comparator

is active

Variable Type: Integer

Valid Range:   0 = Off

1 = On

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid State

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_comp_two_limit

int bp9714_comp_two_limit (int instrumentID, double lowerLimit,

double upperLimit);

Purpose

Panel Name:    Two Limit Comparator

Description:   Configure the two limit comparator

functions

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

lowerLimit

Variable Type       double

Control Name:  Lower Limit

Description:   Specifies the lower limit of the

two limit comparator

Variable Type: Double

Default:       0.0

upperLimit

Variable Type       double

Control Name:  Upper Limit

Description:   Specifies the upper limit of the

two limit comparator

Variable Type: Double

Default:       0.0

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

330   Invalid Comparator Datas

________________________________________________________________________________

bp9714_config

int bp9714_config (int instrumentID, int averageControl,

int averageCounts, int measurementMode,

int displayFormat, int deviceUnit, int startMode);

Purpose

Panel Name:    Config Device

Description:   Configure the instrument in

following ways:

- Selects the device of the device

- Selects the Display Format

- Sets the Average Counts

- Selects the Average Control

- Selects the measurement mode

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

averageControl

Variable Type       int

Control Name:  Average Control

Description:   This parameter selects the type of

Averaging

Variable Type: Integer

Valid Range:   0 = Moving

1 = Repeat

averageCounts

Variable Type       int

Control Name:  Average Counts

Description:   This parameter inputs the numbers

of average counts

Variable Type: Integer

Valid Range:   1 - 9999

measurementMode

Variable Type       int

Control Name:  Measurement Mode

Description:   This parameter is meant for

changing between single and

repetition measurements.

Variable Type: Integer

Valid Range:   0 = Single

1 = Repetition

displayFormat

Variable Type       int

Control Name:  Display Format

Description:   This parameter selects the display

Format

Variable Type: Integer

Valid Range:   0 - 6

deviceUnit

Variable Type       int

Control Name:  Device Unit

Description:   This parameter selects the display

Format

Variable Type: Integer

Valid Range:   0 - 6

startMode

Variable Type       int

Control Name:  Measurement Mode

Description:   This parameter is meant for

changing between single and

repetition measurements.

Variable Type: Integer

Valid Range:   0 = Single

1 = Repetition

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Average Control

-3   Invalid Average Counts

-4   Invalid Measurement Mode

-5   Invalid Display Format

-6   Invalid Device Unit

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_config_analog_output

int bp9714_config_analog_output (int instrumentID, int analogOutputType);

Purpose

Panel Name:    Config Analog Output

Description:   Configure the instrument in

following ways:

- Selects the analog output type

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

analogOutputType

Variable Type       int

Control Name:   Analogue Output Type

Description:    This parameter switches the

Analogue Output type of the

device

Variable Type:  Integer

Valid Values:   0 = Current

1 = Voltage

Default:        0 = Current

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Anlogue Output type

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_config_input

int bp9714_config_input (int instrumentID, int referencePoint,

int inputFilter, int voltageRange);

Purpose

Panel Name:    Config Intput

Description:   Configure the instrument in

following ways:

- Selects the reference point of

the sensor input

- Switch the input low pass-filter

- Selects the input voltage range

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

referencePoint

Variable Type       int

Control Name:   Reference Point

Description:    This parameter switches the

intput Reference Point

of the device

Variable Type:  Integer

Valid Values:   0 = Grounded

1 = Difference

inputFilter

Variable Type       int

Control Name:   Input Filter

Description:    This parameter switches the

intput low-pass Filter

of the device

Variable Type:  Integer

Valid Values:   0 = Off

1 = On

voltageRange

Variable Type       int

Control Name:   Voltage Range

Description:    This parameter switches the

input Voltage Range of the device

Variable Type:  Integer

Valid Values:   0 = 1.25mV

1 = 2.5mV

2 = 5mV

3 = 10mV

4 = 12.5mV

5 = 25mV

6 = 50mV

7 = 100mV

8 = 125mV

9 = 250mV

10 = 500mV

11 = 1V

12 = 1.25V

13 = 2.5V

14 = 5V

15 = 10V

16 = 12V

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Reference Point

-3   Invalid Filter

-4   Invalid Voltage Range

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_config_output

int bp9714_config_output (int instrumentID, int outputShape,

int outputSupply);

Purpose

Panel Name:    Config Output Supply

Description:   Configure the instrument in

following ways:

- Selects the type of the supply

- Selects the shape of the supply

- Selects the value of the supply

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

outputShape

Variable Type       int

Control Name:   Output Shape

Description:    This parameter switches the

shapee mode of the device

Variable Type:  Integer

Valid Values:   0 = DC

1 = LOW

2 = DCR

3 = ACR

4 = ALTR

Default:        0 = Intern

outputSupply

Variable Type       int

Control Name:   Output Supply

Description:    This parameter switches the

Output Value of the device

Variable Type:  Integer

Valid Values:   0 = Asym. 2.5V

1 = Asym. 5V

2 = Asym. 10V

3 = Sym.  5V

4 = Sym.  10V

5 = Sym.  20V

Default:        0 = Asym. 2.5V

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Output Shape

-3   Invalid Output Type

-3   Invalid Output Supply Value

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_get_status

int bp9714_get_status (int instrumentID);

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

310   Device Voltage Error

311   Device Current Error

350   Measuring not started

________________________________________________________________________________

bp9714_init_all_devices

int bp9714_init_all_devices (int COMPort, int baudRate,

double timeout_seconds, int blockCheck,

int instrumentID[], int groupAddresses[],

int userAddresses[], int *instrNumbers);

Purpose

Panel Name:    Initialize All Devices

Description:   Search all connected devices

Initializes the instrument by:

1) finding an unused device in the device table

using the supplied seriall group and user

address

2) returns an Array of instrument ID's

3) returns an Array of Group and User Addresses

4) During to search the connected Instruments

return the function the Code 360, after the

search is finshed, the function return 0

C  Example:

ret = 1;

while (ret != 0)

bp9714_init_all_devices(...);

Parameter List

COMPort

Variable Type       int

Control Name:   COM-Port

Description:    Specifies the COM-Port to be

used to communicate with the

instrument.

Variable Type:  Integer

Valid Values:   1 - COM1   (=> Adress 0x3f8 IRQ4)

2 - COM2   (=> Adress 0x2f8 IRQ3)

3 - COM3   (=> Adress 0x3e8 IRQ4)

4 - COM4   (=> Adress 0x2e8 IRQ3)

Default:        1

baudRate

Variable Type       int

Control Name:   Baud Rate

Description:    Specifies the baud rate of the

RS232-Port to communicate with

the instrument. This value must

correspond to the current

internal setting of the

instrument.

Variable Type:  Integer

Valid Values:   1200,2400,4800,9600 or 19200 baud

Default:        9600

timeout_seconds

Variable Type       double

Control Name:   Time-out (seconds)

Description:    Specifies  the time-out limit for

I/O operations.

A  value  of  0.0  will  disable

time-out altogether.

Variable Type:  Double

Valid Range:    0.060 to 100.0 seconds

Default:        1.0

blockCheck

Variable Type       int

Control Name:   Bcc-Check

Description:    This parameter switches the block

check function of the driver ON

or OFF.

This value must correspond to the

current internal setting of the

instrument.

Variable Type:  Integer

Valid Values:   0 = OFF, 1 = ON

Default:        0 = OFF

instrumentID

Variable Type       int []

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID-Array, which is

used as an input here.

(The ID is used to select among like-instruments)

Variable Type: Integer Arraqy

groupAddresses

Variable Type       int []

Control Name:  Group Addresses

Description:   The initialize function call

returns an Group Address Array, which is

used as an input here.

Variable Type: Integer Array

userAddresses

Variable Type       int []

Control Name:  User Addresses

Description:   The initialize function call

returns an User Address Array, which is

used as an input here.

Variable Type: Integer Array

instrNumbers

Variable Type       int (passed by reference)

Control Name:  Instr. Numbers

Description:   The initialize function call

returns the number of founded

instruments

Variable Type: Integer

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   COM-Port is out of range

-2   Baud Rate is out of range

-3   Time-out is out of range

-4   Block Check parameter is out of range

-5   Group Address is out of range

-6   User Address is out of range

-7   Reset Device is out of range

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

360   The function sends Code 360 during

to search the instruments

________________________________________________________________________________

bp9714_init_device

int bp9714_init_device (int COMPort, int baudRate,

double timeout_seconds, int blockCheck,

int groupAddress, int userAddress,

char instrumentIDString[], int resetDevice,

int *instrumentID);

Purpose

Panel Name:    Initialize One Device

Description:   Initializes the instrument by:

1) finding an unused device in the device table

using the supplied serial group and user

address

2) device clear and RESET  (if desired)

3) returns the ID-String of the instrument

4) returns an instrument ID

Defaults : 8  Data Bits

1  Stop Bit

No Partiy

Parameter List

COMPort

Variable Type       int

Control Name:   COM-Port

Description:    Specifies the COM-Port to be

used to communicate with the

instrument.

Variable Type:  Integer

Valid Values:   1 - COM1   (=> Adress 0x3f8 IRQ4)

2 - COM2   (=> Adress 0x2f8 IRQ3)

3 - COM3   (=> Adress 0x3e8 IRQ4)

4 - COM4   (=> Adress 0x2e8 IRQ3)

Default:        1

baudRate

Variable Type       int

Control Name:   Baud Rate

Description:    Specifies the baud rate of the

RS232-Port to communicate with

the instrument. This value must

correspond to the current

internal setting of the

instrument.

Variable Type:  Integer

Valid Values:   1200,2400,4800,9600 or 19200 baud

Default:        9600

timeout_seconds

Variable Type       double

Control Name:   Time-out (seconds)

Description:    Specifies  the time-out limit for

I/O operations.

A  value  of  0.0  will  disable

time-out altogether.

Variable Type:  Double

Valid Range:    0.060 to 100.0 seconds

Default:        1.0

blockCheck

Variable Type       int

Control Name:   Bcc-Check

Description:    This parameter switches the block

check function of the driver ON

or OFF.

This value must correspond to the

current internal setting of the

instrument.

Variable Type:  Integer

Valid Values:   0 = OFF, 1 = ON

Default:        0 = OFF

groupAddress

Variable Type       int

Control Name:  Group Address

Description:   Specifies the group address on the

RS232 (RS485) to be used to

communicate with the instrument.

This value must correspond to the

current internal group address

from the instrument.

Variable Type: Integer

Valid Range:   0 to 15

Default:       0

userAddress

Variable Type       int

Control Name:  User Adress

Description:   Specifies the user address on the

RS232 (RS485) to be used to

communicate with the instrument.

This value must correspond to the

current internal user address

from the instrument.

Variable Type: Integer

Valid Range:   0 to 15

Default:       0

instrumentIDString

Variable Type       char []

Control Name:  Instrument ID String

Description:   Returns the ID-String of the

instrument.

Variable Type: String

resetDevice

Variable Type       int

Control Name:  Reset

Description:   If reset is on, a device clear and

power on reset is done during initialization.

Variable Type: Integer

Valid Range:   0 to 1

0 = off

1 = on

instrumentID

Variable Type       int (passed by reference)

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   COM-Port is out of range

-2   Baud Rate is out of range

-3   Time-out is out of range

-4   Block Check parameter is out of range

-5   Group Address is out of range

-6   User Address is out of range

-7   Reset Device is out of range

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_init_rs485

int bp9714_init_rs485 (int RS485CtrlBit, int ctrlBitState);

Purpose

Panel Name:    Initialize RS485 (CVI only)

Description:   This function is usefull in

LabWindows/CVI only, because the

RS485 Halfduplex Mode is only

supported in CVI/V3.1

Parameter List

RS485CtrlBit

Variable Type       int

Control Name:   RS485 Ctrl. Bit State

Description:    This parameter switches the Ctrl.

Bit RTS or DTR from the RS485

halfduplex Ctrl

Variable Type:  Integer

Valid Values:   0 = RTS

1 = DTR

Default:        0 = RTS

ctrlBitState

Variable Type       int

Control Name:   Ctrl. Bit State

Description:    This parameter switches the state

from the RS485 halfduplex Ctrl.

Bit Low or High Active.

Variable Type:  Integer

Valid Values:   0 = Low Active

1 = High Active

Default:        1 = High Active

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Ctrl Bit

-2   Invalid Ctrl State

________________________________________________________________________________

bp9714_logg_state

int bp9714_logg_state (int instrumentID, int datenlogger);

Purpose

Panel Name:    Tare State

Description:   Sets the Tare State ON/OFF

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

datenlogger

Variable Type       int

Control Name:  State

Description:   If the State is On, the calibrate

the input offset

Variable Type: Integer

Valid Range:   0 = Off

1 = On

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid State

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_peak_detection_state

int bp9714_peak_detection_state (int instrumentID, int state);

Purpose

Panel Name:    Peak Detection State

Description:   Switches the peak detection ON/OFF

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

state

Variable Type       int

Control Name:  State

Description:   If the state is On the peak

detection is active

Variable Type: Integer

Valid Range:   0 = Off

1 = On

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid State

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_read_block_parameter

int bp9714_read_block_parameter (int instrumentID, int *size, int block,

char name[], int *filter, int *XVal,

double *time, double *delta);

Purpose

Panel Name:    Read Comparator State

Description:   Reads the Comparator State

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

size

Variable Type       int (passed by reference)

block

Variable Type       int

name

Variable Type       char []

filter

Variable Type       int (passed by reference)

XVal

Variable Type       int (passed by reference)

time

Variable Type       double (passed by reference)

delta

Variable Type       double (passed by reference)

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid State

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_read_cal_analogue_output

int bp9714_read_cal_analogue_output (int instrumentID, double *lowValue,

double *low_Scale,

double *highValue,

double *high_Scale);

Purpose

Panel Name:    Read Calibration Analogue Output

Description:   Reads the Calibration data of the analoque

output:

- Reads the low and high output values

and the low and high scale values

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

lowValue

Variable Type       double (passed by reference)

Control Name:  Low  Value

Description:   Specifies the low calibration

value of the analogue output

Variable Type: Double

Default:       0.0

low_Scale

Variable Type       double (passed by reference)

highValue

Variable Type       double (passed by reference)

Control Name:  High Value

Description:   Specifies the high calibration

value of the analogue output

Variable Type: Double

Default:       0.0

high_Scale

Variable Type       double (passed by reference)

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_read_cal_input

int bp9714_read_cal_input (int instrumentID, double *lowCalValue,

double *highCalValue, double *lowScaleValue,

double *highScaleValue);

Purpose

Panel Name:    Read Calibration Input

Description:   Reads the calibration data of the

the input:

- Reads the Calibration low and

high Value

- Reads the Scale low and

high Value

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

lowCalValue

Variable Type       double (passed by reference)

Control Name:  Low Cal. Value

Description:   Specifies the low calibration

value

Variable Type: Double

Default:       0.0

highCalValue

Variable Type       double (passed by reference)

Control Name:  High Cal. Value

Description:   Specifies the high calibration

value

Variable Type: Double

Default:       0.0

lowScaleValue

Variable Type       double (passed by reference)

Control Name:  Low Scale value

Description:   Specifies the low scale value

Variable Type: Double

Default:       0.0

highScaleValue

Variable Type       double (passed by reference)

Control Name:  High Scale Value

Description:   Specifies the high Scale Value

Variable Type: Double

Default:       0.0

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_read_cal_input_meas

int bp9714_read_cal_input_meas (int instrumentID, double *kal1,

double *kal2);

Purpose

Panel Name:    Read Calibration Input Meas

Description:   Reads the measured values Kal1 and Kal2

of the input calibration

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

kal1

Variable Type       double (passed by reference)

Control Name:  Measure

Description:   This parameter starts to measure

the calibration Value

Variable Type: Integer

Valid Range:   0 = Low Value

1 = High Value

kal2

Variable Type       double (passed by reference)

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Measure

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_read_cal_state

int bp9714_read_cal_state (int instrumentID, int *state);

Purpose

Panel Name:    Calibration State

Description:   Reads the calibration state

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

state

Variable Type       int (passed by reference)

Control Name:  State

Description:   If the State is On the calibration

functions are possible to activate

Variable Type: Integer

Valid Range:   0 = Off

1 = On

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid State

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_read_cal_unit

int bp9714_read_cal_unit (int instrumentID, int *calibrationUnit);

Purpose

Panel Name:    Read Calibration Unit

Description:   Reads the Calibration Unit

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

calibrationUnit

Variable Type       int (passed by reference)

Control Name:  Unit

Description:   This parameter selects the

calibration unit

Variable Type: Integer

Valid Range:   0 = mV

1 = mV/V

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Calibration Unit

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_read_comp_config

int bp9714_read_comp_config (int instrumentID, int *comparatorType,

int *comparatorControl);

Purpose

Panel Name:    Comparator Config

Description:   Configure the comparator functions

of the instrument in following

ways:

- Selects the comparator type

- Selects the comparator Control

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

comparatorType

Variable Type       int (passed by reference)

Control Name:  Comparator Type

Description:   Switches the comparator type

Variable Type: Integer

Default:       0 = Dynamic

Valid Range:   0 = Dynamic

1 = Static

comparatorControl

Variable Type       int (passed by reference)

Control Name:  Comparator Control

Description:   Switches the comparator Control

Variable Type: Integer

Default:       0 = Single

Valid Range:   0 = Single

1 = Average

2 = Minimum

3 = Maximum

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Comparator Type

-3   Invalid Comparator Control

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_read_comp_count

int bp9714_read_comp_count (int instrumentID, int *count);

Purpose

Panel Name:    Read Comparator State

Description:   Reads the Comparator State

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

count

Variable Type       int (passed by reference)

Control Name:  State

Description:   If the State is On the comparator

is active

Variable Type: Integer

Valid Range:   0 = Off

1 = On

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid State

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_read_comp_four_limit

int bp9714_read_comp_four_limit (int instrumentID, double *lowerLimit1,

double *lowerLimit2,

double *upperLimit1,

double *upperLimit2);

Purpose

Panel Name:    Read Four Limit Comparator

Description:   Reads the configuration of the four

limit comparator functions

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

lowerLimit1

Variable Type       double (passed by reference)

Control Name:  Lower Limit 1

Description:   Specifies the lower limit of the

two limit comparator 1

Variable Type: Double

Default:       0.0

lowerLimit2

Variable Type       double (passed by reference)

Control Name:  Lower Limit 2

Description:   Specifies the lower limit of the

two limit comparator 2

Variable Type: Double

Default:       0.0

upperLimit1

Variable Type       double (passed by reference)

Control Name:  Upper Limit1

Description:   Specifies the upper limit of the

two limit comparator 1

Variable Type: Double

Default:       0.0

upperLimit2

Variable Type       double (passed by reference)

Control Name:  Upper Limit2

Description:   Specifies the upper limit of the

two limit comparator 2

Variable Type: Double

Default:       0.0

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

330   Invalid Comparator Datas

________________________________________________________________________________

bp9714_read_comp_state

int bp9714_read_comp_state (int instrumentID, int *state);

Purpose

Panel Name:    Read Comparator State

Description:   Reads the Comparator State

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

state

Variable Type       int (passed by reference)

Control Name:  State

Description:   If the State is On the comparator

is active

Variable Type: Integer

Valid Range:   0 = Off

1 = On

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid State

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_read_comp_two_limit

int bp9714_read_comp_two_limit (int instrumentID, double *lowerLimit,

double *upperLimit);

Purpose

Panel Name:    Read Two Limit Comparator

Description:   Reads the configuration of the

two limit comparator.

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

lowerLimit

Variable Type       double (passed by reference)

Control Name:  Lower Limit

Description:   Specifies the lower limit of the

two limit comparator

Variable Type: Double

Default:       0.0

upperLimit

Variable Type       double (passed by reference)

Control Name:  Upper Limit

Description:   Specifies the upper limit of the

two limit comparator

Variable Type: Double

Default:       0.0

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

330   Invalid Comparator Datas

________________________________________________________________________________

bp9714_read_config

int bp9714_read_config (int instrumentID, int *averageControl,

int *averageCounts, int *measurementMode,

int *displayFormat, int *deviceUnit,

int *start_mode);

Purpose

Panel Name:    Read Config Device

Description:   Reads the Configuration of the

instrument in following ways:

- Reads the unit of the device

- Reads the display format

- Reads the average counts

- Reads the average control

- Reads the measurement mode

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

averageControl

Variable Type       int (passed by reference)

Control Name:  Average Control

Description:   This parameter selects the type of

Averaging

Variable Type: Integer

Valid Range:   0 = Moving

1 = Repeat

averageCounts

Variable Type       int (passed by reference)

Control Name:  Average Counts

Description:   This parameter inputs the numbers

of average counts

Variable Type: Integer

Valid Range:   1 - 9999

measurementMode

Variable Type       int (passed by reference)

Control Name:  Measurement Mode

Description:   This parameter is meant for

changing between single and

repetition measurements.

Variable Type: Integer

Valid Range:   0 = Single

1 = Repetition

displayFormat

Variable Type       int (passed by reference)

Control Name:  Display Format

Description:   This parameter selects the display

Format

Variable Type: Integer

Valid Range:   0 - 6

deviceUnit

Variable Type       int (passed by reference)

Control Name:  Device Unit

Description:   This parameter selects the display

Format

Variable Type: Integer

Valid Range:   0 - 6

start_mode

Variable Type       int (passed by reference)

Control Name:  Measurement Mode

Description:   This parameter is meant for

changing between single and

repetition measurements.

Variable Type: Integer

Valid Range:   0 = Single

1 = Repetition

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Average Control

-3   Invalid Average Counts

-4   Invalid Measurement Mode

-5   Invalid Display Format

-6   Invalid Device Unit

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_read_config_analog_output

int bp9714_read_config_analog_output (int instrumentID,

int *analogOutputType);

Purpose

Panel Name:    Read Config Analog Output

Description:   Reads the analoque output type of

the instrument

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

analogOutputType

Variable Type       int (passed by reference)

Control Name:   Analogue Output Type

Description:    This parameter switches the

Analogue Output type of the

device

Variable Type:  Integer

Valid Values:   0 = Current

1 = Voltage

Default:        0 = Current

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Anlogue Output type

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_read_config_input

int bp9714_read_config_input (int instrumentID, int *referencePoint,

int *inputFilter, int *voltageRange);

Purpose

Panel Name:    Read Config Intput

Description:   Reads the configuration of the

instruments input in the following

ways:

- Reads the reference point of

the sensor input

- Reads the state of the input low

pass-filter

- Reads the input voltage range

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

referencePoint

Variable Type       int (passed by reference)

Control Name:   Reference Point

Description:    This parameter switches the

intput Reference Point

of the device

Variable Type:  Integer

Valid Values:   0 = Grounded

1 = Difference

inputFilter

Variable Type       int (passed by reference)

Control Name:   Input Filter

Description:    This parameter switches the

intput low-pass Filter

of the device

Variable Type:  Integer

Valid Values:   0 = Off

1 = On

voltageRange

Variable Type       int (passed by reference)

Control Name:   Voltage Range

Description:    This parameter switches the

input Voltage Range of the device

Variable Type:  Integer

Valid Values:   0 = 1.25mV

1 = 2.5mV

2 = 5mV

3 = 10mV

4 = 12.5mV

5 = 25mV

6 = 50mV

7 = 100mV

8 = 125mV

9 = 250mV

10 = 500mV

11 = 1V

12 = 1.25V

13 = 2.5V

14 = 5V

15 = 10V

16 = 12V

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Reference Point

-3   Invalid Filter

-4   Invalid Voltage Range

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_read_config_output

int bp9714_read_config_output (int instrumentID, int *outputShape,

int *outputSupply);

Purpose

Panel Name:    Read Config Output

Description:   Reads the configuration of the

instrument excitation output in

following way:

- Reads the type of the excitation

- Reads the shape of the excitation

- Reads the value of the exitation

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

outputShape

Variable Type       int (passed by reference)

Control Name:   Output Shape

Description:    This parameter switches the

shapee mode of the device

Variable Type:  Integer

Valid Values:   0 = DC

1 = LOW

2 = DCR

3 = ACR

4 = ALTR

Default:        0 = Intern

outputSupply

Variable Type       int (passed by reference)

Control Name:   Output Supply

Description:    This parameter switches the

Output Value of the device

Variable Type:  Integer

Valid Values:   0 = Asym. 2.5V

1 = Asym. 5V

2 = Asym. 10V

3 = Sym.  5V

4 = Sym.  10V

5 = Sym.  20V

Default:        0 = Asym. 2.5V

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Output Shape

-3   Invalid Output Type

-3   Invalid Output Supply Value

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_read_count

int bp9714_read_count (int instrument_ID, int *count, int block);

Purpose

Panel Name:    Read Comparator State

Description:   Reads the Comparator State

Parameter List

instrument_ID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

count

Variable Type       int (passed by reference)

Control Name:  State

Description:   If the State is On the comparator

is active

Variable Type: Integer

Valid Range:   0 = Off

1 = On

block

Variable Type       int

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid State

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_read_data

int bp9714_read_data (int instrumentID, int block, int number,

float *value);

Purpose

Panel Name:    Read Comparator State

Description:   Reads the Comparator State

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

block

Variable Type       int

number

Variable Type       int

value

Variable Type       float (passed by reference)

Control Name:  State

Description:   If the State is On the comparator

is active

Variable Type: Integer

Valid Range:   0 = Off

1 = On

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid State

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_read_data_block

int bp9714_read_data_block (int instrumentID, int block, int adresse,

float data[]);

Purpose

Panel Name:    Read Comparator State

Description:   Reads the Comparator State

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

block

Variable Type       int

adresse

Variable Type       int

data

Variable Type       float []

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid State

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_read_display_contrast

int bp9714_read_display_contrast (int instrumentID, double *contrast);

Purpose

Panel Name:    Read Display Contrast

Description:   Reads the display contrast of the

instrument

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

contrast

Variable Type       double (passed by reference)

Control Name:   Display Contrast

Description:    Specifies the display contrast

Variable Type:  Double

Valid Range:    0.0 to 1.0

Default:        0.5

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

-2   Invalid Contrast

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

320   Datalogger is empty

________________________________________________________________________________

bp9714_read_eval

int bp9714_read_eval (int instrumentID, int block, double *minimum,

double *stdDeviation, double *maximum,

double *meanValue);

Purpose

Panel Name:    Read Comparator State

Description:   Reads the Comparator State

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

block

Variable Type       int

minimum

Variable Type       double (passed by reference)

Control Name:  State

Description:   If the State is On the comparator

is active

Variable Type: Integer

Valid Range:   0 = Off

1 = On

stdDeviation

Variable Type       double (passed by reference)

Control Name:  State

Description:   If the State is On the comparator

is active

Variable Type: Integer

Valid Range:   0 = Off

1 = On

maximum

Variable Type       double (passed by reference)

Control Name:  State

Description:   If the State is On the comparator

is active

Variable Type: Integer

Valid Range:   0 = Off

1 = On

meanValue

Variable Type       double (passed by reference)

Control Name:  State

Description:   If the State is On the comparator

is active

Variable Type: Integer

Valid Range:   0 = Off

1 = On

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid State

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_read_logg_state

int bp9714_read_logg_state (int instrumentID, int *data_logger_state);

Purpose

Panel Name:    Read Comparator State

Description:   Reads the Comparator State

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

data_logger_state

Variable Type       int (passed by reference)

Control Name:  State

Description:   If the State is On the comparator

is active

Variable Type: Integer

Valid Range:   0 = Off

1 = On

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid State

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_read_measure

int bp9714_read_measure (int instrumentID, double *value, char unit[],

char comparatorValue[]);

Purpose

Panel Name:    Read Measure Value

Description:   Read the measurment value as

double ,the unit and the

comparator value

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

value

Variable Type       double (passed by reference)

Control Name:  Value

Description:   Returns the Value of the

instrument.

Variable Type: double

unit

Variable Type       char []

Control Name:  Unit

Description:   Returns the Unit of the

instrument.

Variable Type: String

comparatorValue

Variable Type       char []

Control Name:  Comparator Value

Description:   Returns the comparator value of

the instrument.

Variable Type: String

Possible Values:  NC is equal No Comparator

<<, < , = , > , >>

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

310   Device Voltage Error

311   Device Current Error

350   Measuring not started

________________________________________________________________________________

bp9714_read_peak_detection_state

int bp9714_read_peak_detection_state (int instrumentID, int *state);

Purpose

Panel Name:    Read Peak Detection State

Description:   Reads the state of the peak detection

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

state

Variable Type       int (passed by reference)

Control Name:  State

Description:   If the State is On the peak

detection is activated

Variable Type: Integer

Valid Range:   0 = Off

1 = On

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid State

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_read_peak_values

int bp9714_read_peak_values (int instrumentID, double *minValue,

double *maxValue, double *diffValue);

Purpose

Panel Name:    Read Peak Values

Description:   Reads the peak values

values as double

- Minium Value

- Maximum Value

- Difference Value

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

minValue

Variable Type       double (passed by reference)

Control Name:  Min. Value

Description:   Returns the minimum peak value

Variable Type: double

maxValue

Variable Type       double (passed by reference)

Control Name:  Max. Value

Description:   Returns the maximum peak value

Variable Type: double

diffValue

Variable Type       double (passed by reference)

Control Name:  Diff. Value

Description:   Returns the difference

max. peak value - min. peak value

Variable Type: double

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

310   Device Voltage Error

311   Device Current Error

312   Device Temperature Error

313   Device Calibration Error

314   Device In Over Error

315   Device Out Over Error

316   Device Lowbat Error

________________________________________________________________________________

bp9714_read_selected_block

int bp9714_read_selected_block (int instrumentID, int *selectedBlock);

Purpose

Panel Name:    Read Comparator State

Description:   Reads the Comparator State

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

selectedBlock

Variable Type       int (passed by reference)

Control Name:  State

Description:   If the State is On the comparator

is active

Variable Type: Integer

Valid Range:   0 = Off

1 = On

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid State

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_read_string_measure

int bp9714_read_string_measure (int instrumentID, unsigned char *value,

unsigned char *comp_value);

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

value

Variable Type       unsigned char (passed by reference)

Control Name:  Value

Description:   Returns the Value of the

instrument.

Variable Type: double

comp_value

Variable Type       unsigned char (passed by reference)

Control Name:  Value

Description:   Returns the Value of the

instrument.

Variable Type: double

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

310   Device Voltage Error

311   Device Current Error

350   Measuring not started

________________________________________________________________________________

bp9714_read_system_security

int bp9714_read_system_security (int instrumentID, int *code);

Purpose

Panel Name:   Read System Security

Description:  Reads the security code of the device

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

code

Variable Type       int (passed by reference)

Control Name:   Security Code

Description:    Specifies the sequrity code of

the instrument

Variable Type:  integer

Variable Range: 0-9999

Default:        2588

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Code

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_read_system_serial_para

int bp9714_read_system_serial_para (int instrumentID,

int *interfaceType,

int *groupAddress, int *userAddress,

int *baudrate,

int *DBitsParitySBits, int *parity,

int *blockCheck, int *charDelay);

Purpose

Panel Name:    Read Serial Parameters

Description:   Reads the following serial

communication parameters

- Interface Type

- Baudrate

- Group Address

- User Address

- Databits

- Parity

- Blockcheck

- Char Delay

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

interfaceType

Variable Type       int (passed by reference)

Control Name:  Interface Type

Description:   Specifies the Interface Type of

the communicate with the

instrument.

Variable Type: Integer

Valid Range:   0 = RS232

1 = RS485 Fullduplex

1 = RS485 Halfduplex

Default:       0 = RS232

groupAddress

Variable Type       int (passed by reference)

Control Name:  Group Address

Description:   Specifies the group address on the

RS232 (RS485) to be used to

communicate with the instrument.

Variable Type: Integer

Valid Range:   0 to 15

Default:       0

userAddress

Variable Type       int (passed by reference)

Control Name:  User Adress

Description:   Specifies the user address on the

RS232 (RS485) to be used to

communicate with the instrument.

Variable Type: Integer

Valid Range:   0 to 15

Default:       0

baudrate

Variable Type       int (passed by reference)

Control Name:  Baudrate

Description:   Specifies the Baudrate to

communicate with the instrument.

Variable Type: Integer

Valid Range:   0 = 300

1 = 600

2 = 1200

3 = 2400

4 = 4800

5 = 9600

6 = 19200

7 = 38400

Default:       5 = 9600

DBitsParitySBits

Variable Type       int (passed by reference)

Control Name:  DBits/Parity/SBits

Description:   Specifies the Databits the Parity

(0 = Off,1 = ON) and the Stopbits

to communicate with the instrument

Variable Type: Integer

Valid Range:   0 = 8DataBits No Parity 1Stopbit

1 = 7DataBits No Parity 2Stopbit

2 = 7DataBits YesParity 1Stopbit

3 = 8DataBits No Parity 2Stopbit

4 = 8DataBits YesParity 1Stopbit

Default:       0 = 8DataBits No Parity 1Stopbit

parity

Variable Type       int (passed by reference)

Control Name:  Parity

Description:   Specifies the Databits the Parity

Mode to communicate with

the instrument.

Variable Type: Integer

Valid Range:   0 = None

1 = ODD

2 = EVEN

Default:       0 = None

blockCheck

Variable Type       int (passed by reference)

Control Name:   Bcc-Check

Description:    This parameter switches the block

check function of the instrument

Variable Type:  Integer

Valid Values:   0 = OFF, 1 = ON

Default:        0 = OFF

charDelay

Variable Type       int (passed by reference)

Control Name:  Char Delay

Description:   Siwtch the Char Delay Function of

the device ON

Variable Type: Integer

Valid Range:   0 to 1

0 = off

1 = on

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Interface Type

-2   Invalid Group Address

-3   Invalid User Address

-4   Invalid Baudrate

-5   Invalid Databits

-6   Invalid Parity

-7   Invalid Blockcheck

-8   Invalid Chardelay

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_read_tare_state

int bp9714_read_tare_state (int instrumentID, int *state);

Purpose

Panel Name:    Read Tare State

Description:   Read the sate of the tare function

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

state

Variable Type       int (passed by reference)

Control Name:  State

Description:   If the State is On, the calibrate

the input offset

Variable Type: Integer

Valid Range:   0 = Off

1 = On

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid State

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_read_tare_value

int bp9714_read_tare_value (int instrumentID, double *tareValue);

Purpose

Panel Name:    Read Tare Values

Description:   Read the tare value as double

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

tareValue

Variable Type       double (passed by reference)

Control Name:  Value

Description:   Returns the tare value

of the instrument.

Variable Type: double

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

310   Device Voltage Error

311   Device Current Error

312   Device Temperature Error

313   Device Calibration Error

314   Device In Over Error

315   Device Out Over Error

316   Device Lowbat Error

________________________________________________________________________________

bp9714_recall_memory

int bp9714_recall_memory (int instrumentID, int memoryNo);

Purpose

Panel Name:    Recall Memory No.

Description:   Recall the actual Parameters to

the selected memory of the

instrument

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

memoryNo

Variable Type       int

Control Name:   Memory No.

Description:    Specifies the Memory No.

Variable Type:  integer

Valid Range:    1 to 16

Default:        1

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

-2   Invalid Memory No.

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

320   Datalogger is empty

________________________________________________________________________________

bp9714_receive_scpi

int bp9714_receive_scpi (int instrumentID, char sendSCPIMessage[],

char instrumentAnswers[]);

Purpose

Panel Name:   Receive a SCPI-Message

Description:  Sends "Query form" from SCPI-

Message and receive a answers

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

sendSCPIMessage

Variable Type       char []

Control Name:  Send SCPI-Message

Description:   This control is a string parameter

You must input Query-Form from

the SCPI-message without the

Line Feed termination.

Example      : ":SENS:CORR:STATE?"

Variable Type: String

instrumentAnswers

Variable Type       char []

Control Name:  Answer from Slave

Description:   This control return a string from

the slave.

Variable Type: String (Length => see the 6706

Manual SCPI-Commands)

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_select_block

int bp9714_select_block (int instrumentID, int block);

Purpose

Panel Name:    Read Comparator State

Description:   Reads the Comparator State

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

block

Variable Type       int

Control Name:  State

Description:   If the State is On the comparator

is active

Variable Type: Integer

Valid Range:   0 = Off

1 = On

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid State

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_send_scpi

int bp9714_send_scpi (int instrumentID, char sendSCPIMessage[]);

Purpose

Panel Name:   Send a SCPI-Message

Description:  Sends a SCPI-Message to a

slave

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

sendSCPIMessage

Variable Type       char []

Control Name:  Send SCPI-Message

Description:   This control is a string parameter

You must input the SCPI-message

without the LineFeed termination.

Example      : ":SENS:CORR:STATE ON"

Variable Type: String

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_set_block_parameter

int bp9714_set_block_parameter (int instrumentID, int block, int size,

char name[], int filter, int XVal,

double time, double delta);

Purpose

Panel Name:    Read Comparator State

Description:   Reads the Comparator State

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

block

Variable Type       int

size

Variable Type       int

name

Variable Type       char []

filter

Variable Type       int

XVal

Variable Type       int

time

Variable Type       double

delta

Variable Type       double

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid State

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_set_display_contrast

int bp9714_set_display_contrast (int instrumentID, double contrast);

Purpose

Panel Name:    Display Contrast

Description:   Adjust the display contrast of the

instrument

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

contrast

Variable Type       double

Control Name:   Display Contrast

Description:    Specifies the display contrast

Variable Type:  Double

Valid Range:    0.0 to 1.0

Default:        0.5

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

-2   Invalid Contrast

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

320   Datalogger is empty

________________________________________________________________________________

bp9714_start_measure

int bp9714_start_measure (int instrumentID);

Purpose

Panel Name:    Start Measuring

Description:   Start the measuring of instrument

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

310   Device Voltage Error

311   Device Current Error

312   Device Temperature Error

313   Device Calibration Error

314   Device In Over Error

315   Device Out Over Error

316   Device Lowbat Error

________________________________________________________________________________

bp9714_stop_measure

int bp9714_stop_measure (int instrumentID);

Purpose

Panel Name:    Stop Measuring

Description:   Abort the measure

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

310   Device Voltage Error

311   Device Current Error

312   Device Temperature Error

313   Device Calibration Error

314   Device In Over Error

315   Device Out Over Error

316   Device Lowbat Error

________________________________________________________________________________

bp9714_store_memory

int bp9714_store_memory (int instrumentID, int memoryNo);

Purpose

Panel Name:    Store Memory No.

Description:   Store the actual Parameters to

the selected memory of the

instrument

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

memoryNo

Variable Type       int

Control Name:   Memory No.

Description:    Specifies the Memory No.

Variable Type:  integer

Valid Range:    1 to 16

Default:        1

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrumet ID

-2   Invalid Memory No.

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

320   Datalogger is empty

________________________________________________________________________________

bp9714_system_error

int bp9714_system_error (int instrumentID, char systemError[]);

Purpose

Panel Name:   System Error

Description:  Reads the system error of the

device

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

systemError

Variable Type       char []

Control Name:  System Error

Description:   Displays the SCPI system error

Variable Type: String

(see manual Resistomat Type 6706)

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0     Success

-1   Invalid Instrument ID

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_system_keyboard

int bp9714_system_keyboard (int instrumentID, int keyboard);

Purpose

Panel Name:   System Keybroad

Description:  Controls the keybroad of the device

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

keyboard

Variable Type       int

Control Name:  Keyboard

Description:   This parameter locks the keyboard

on the front panel of the

instrument

Variable Type: Integer

Valid Range:   0  = Unlocked

1  = Locked

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Value

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_system_security

int bp9714_system_security (int instrumentID, int code);

Purpose

Panel Name:   System Security

Description:  Controls the security functions

of the device

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

code

Variable Type       int

Control Name:   Security Code

Description:    Specifies the sequrity code of

the instrument

Variable Type:  integer

Variable Range: 0-9999

Default:        2588

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid Code

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_system_serial_accept

int bp9714_system_serial_accept (int instrumentID);

Purpose

Panel Name:    Accept New Serial Para.

Description:   The new Serial Parameters

are accepted.

WARNING :      After this function the new serial

Paramters are active.

Now you must change the same

Parameters by your Personal

Computer Interface

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

330   The devicve don't accept the

calibration datas

________________________________________________________________________________

bp9714_system_serial_set_para

int bp9714_system_serial_set_para (int instrumentID, int interfaceType,

int groupAddress, int userAddress,

int baudrate, int DBitsParitySBits,

int parity, int blockCheck,

int charDelay);

Purpose

Panel Name:    Set Serial Parameters

Description:   Sets the followed Serial

Communication Parameters

- Interface Type

- Baudrate

- Group Address

- User Address

- Databits

- Parity

- Blockcheck

- Char Delay

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

interfaceType

Variable Type       int

Control Name:  Interface Type

Description:   Specifies the Interface Type of

the communicate with the

instrument.

Variable Type: Integer

Valid Range:   0 = RS232

1 = RS485 Fullduplex

1 = RS485 Halfduplex

Default:       0 = RS232

groupAddress

Variable Type       int

Control Name:  Group Address

Description:   Specifies the group address on the

RS232 (RS485) to be used to

communicate with the instrument.

Variable Type: Integer

Valid Range:   0 to 15

Default:       0

userAddress

Variable Type       int

Control Name:  User Adress

Description:   Specifies the user address on the

RS232 (RS485) to be used to

communicate with the instrument.

Variable Type: Integer

Valid Range:   0 to 15

Default:       0

baudrate

Variable Type       int

Control Name:  Baudrate

Description:   Specifies the Baudrate to

communicate with the instrument.

Variable Type: Integer

Valid Range:   0 = 300

1 = 600

2 = 1200

3 = 2400

4 = 4800

5 = 9600

6 = 19200

7 = 38400

Default:       5 = 9600

DBitsParitySBits

Variable Type       int

Control Name:  DBits/Parity/SBits

Description:   Specifies the Databits the Parity

(0 = Off,1 = ON) and the Stopbits

to communicate with the instrument

Variable Type: Integer

Valid Range:   0 = 8DataBits No Parity 1Stopbit

1 = 7DataBits No Parity 2Stopbit

2 = 7DataBits YesParity 1Stopbit

3 = 8DataBits No Parity 2Stopbit

4 = 8DataBits YesParity 1Stopbit

Default:       0 = 8DataBits No Parity 1Stopbit

parity

Variable Type       int

Control Name:  Parity

Description:   Specifies the Databits the Parity

Mode to communicate with

the instrument.

Variable Type: Integer

Valid Range:   0 = None

1 = ODD

2 = EVEN

Default:       0 = None

blockCheck

Variable Type       int

Control Name:   Bcc-Check

Description:    This parameter switches the block

check function of the instrument

Variable Type:  Integer

Valid Values:   0 = OFF, 1 = ON

Default:        0 = OFF

charDelay

Variable Type       int

Control Name:  Char Delay

Description:   Siwtch the Char Delay Function of

the device ON

Variable Type: Integer

Valid Range:   0 to 1

0 = off

1 = on

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Interface Type

-2   Invalid Group Address

-3   Invalid User Address

-4   Invalid Baudrate

-5   Invalid Databits

-6   Invalid Parity

-7   Invalid Blockcheck

-8   Invalid Chardelay

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT

________________________________________________________________________________

bp9714_tare_state

int bp9714_tare_state (int instrumentID, int state);

Purpose

Panel Name:    Tare State

Description:   Sets the Tare State ON/OFF

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   The initialize function call

returns an instrument ID, which is used as an

input here.  (The ID is used to select among

like-instruments).

Variable Type: Integer

Valid Range:   1 to 10 (typical)

The value 10 is set internally in the driver.

state

Variable Type       int

Control Name:  State

Description:   If the State is On, the calibrate

the input offset

Variable Type: Integer

Valid Range:   0 = Off

1 = On

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Invalid Instrument ID

-2   Invalid State

1 - 124   RS-232 error (see RS-232 Library

error codes)

220   Unable to open instrument

221   Unable to close instrument

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

237   Error configuring EOS terminator

300   The instrument sends NAK

301   The instrument sends EOT