====================================

|   PROTEUS  DOCUMENTATION  FILE   |

====================================

B&C Microsystems Inc.

ver. 1.2a    02/04/93

I. HOW TO USE MACROS:

====================

1. Command Line Mode:

To run macros in Command Line Mode press F9 [Run Macro]. On the command

line type the desired macro. PROTEUS automatically reads the macrofile

and executes the macro which might prompt the user for actions.

The normal PROTEUS interface screen can be turned OFF by means of the

SCRNOFF macro command (SCRNON to turn back on).

A few sample macros are provided for demo purposes. To get familiar with

running macros from the Command Line, execute the following sample macros:

PROG1: (to run press F9 from Main Menu and type "prog1<Enter>)

-----

- This macro exemplifies the operation of reading the data from a device and

saving the data from the Proteus data buffer into a file, then loading the

same file into the data buffer and programming a second device with it.

You will be prompted to insert the device to be read and the device to be

programmed/tested but you don't have to actually insert the physical

device in any socket. The devices you will be prompted for are EPROMS

and a blank socket is read by PROTEUS exactly as a blank (fully

erased) EPROM with FF(hex) data in all locations.

- Note: Inspect the PROTEUS.MCR system macro file for details about the

macro implementation

PROG2:

-----

- Same with PROG1 only the normal PROTEUS screen is turned OFF

2. Batch Mode:

To run a macro in Batch Mode you need to type at the DOS prompt PROTEUS

followed by the parameter "\m" and the name of the macro you want to execute.

e.g.  proteus \m macroname<Enter>

The file PROTEUS.MCR contains various sample macros and is loaded

automatically upon running the PROTEUS program. You can specify loading of a

different macro file by typing at the DOS prompt PROTEUS followed by the

parameter "\f" and the name of the file you want to load.

e.g. proteus \f macrofilename<Enter>

A new macro command "MACRO_PROGRAM" was implemented for automatically

executing macros and/or macro command upon loading a macro file. All macros

and/or macrocommands found in the macro file below the command

"MACRO_PROGRAM" will thereby be executed when you start PROTEUS.

To get familiar with the execution of macros in Batch Mode you can exit

PROTEUS and run DEMO.BAT. This batch file starts up PROTEUS with the  \m

parameter and a macro file exemplifying the execution of macro in Batch

Mode.

DEMO: (from DOS prompt type "DEMO" to execute file DEMO.BAT)

----

- This batch file contains the declaration:

proteus \m prog3

This will execute the PROTEUS program and run the macro PROG3 defined in

the macro file PROTEUS.MCR. This macro is the  same as PROG2 just that it

exits the PROTEUS program after termination.

- Inspect the file PROTEUS.MCR for details about the implementation of the

macro "prog3".

3. To edit macros while running the PROTEUS software press ^<F1> (CTRL & F1 key).

This will actually execute the macro "editmcr" which will temporary terminate

the PROTEUS program, run the DOS editor program "edit" in the c:\DOS

subdirectory, and load as file to edit PROTEUS.MCR. If you don't have

the DOS editr "edit" on your hard disk or if you choose to use your own

editor, you can alter the macro EDITFILE by replacing the DOS editor "edit"

with the name of your editor. By doing this you will be able to use your own

editor to write/modify macros from inside the PROTEUS software by typing

from the Main Menu  ^<F1> (Ctrl & F1 key).

II. HOW TO USE ADEL (Algorithm Development System):

==================================================

1. Select Options from COMMANDS menu.

2. Press SPACE to select USER device library.

3. Press ESC to return to COMMANDS menu.

4. Choose part you wish to modify.

5. Select ADEL (Algo).

6. From Device Record screen you may access elements of the algorithm.

a. To view pin record, first move cursor to pin record field, then

press F6.  You will now be in the pin record editor.

b. To view family record, first move cursor to family record field, then

press F6.  You will now be in the family record editor.  The family

record includes the following information:

1. Supply voltages (VPP, VCC, etc).

2. Timing specifications (tD, PW, OPW, etc).

3. Algorithm code (through #load(WAV, ?, ?, ?) directive).

4. Other special parameters.

c. To view algorithm code, first perform step b. to enter family

record editor.  Then move cursor to record number argument of

#load(WAV, ?, ?, ?) directive.  Then press F6.

Example:

#load(WAV, 1, 4, 9, 16)

^

|

Place cursor here then press F6 to view/edit record.

III. NOTES ON TEST PROGRAMS IMPLEMENTED FOR SAMPLE DEVICES IN LIBRARY "TEST":

============================================================================

PROTEUS is very well suited for PCMCIA Memory and I/O Cards testing. The

built-in Algorithm Development Environment (ADEL) makes writing and debug-

ging test software very convenient and efficient.

A new library TEST was added to the PROTEUS software. This library includes

as of version 3.4j two devices: PCMCIA-1MBFlsh.5MBRAM and Turbo-IC. Both of

these devices are offered with the source code for the algorithms.

A. PCMCIA-1MBFlsh.5MBRAM (Manufacturer: PCMCIA-Manuf):

------------------------------------------------------

Note: Although the tests implemented for this device are real, the device

itself and the Manufacturer are fictitious.

To get familiar and view the implementation of test program for a PCMCIA card

select Options from the Main Menu, and from the Options window select the

library Test (tap the <Space Bar> to display available libraries). Type <Enter>

or <Escape> to return to the Main Screen, select Choose Device from the

Main Menu and then select PCMCIA-1MBFlsh.5MBRAM. When returning back to the

Main Screen you will see a custom Main Menu, defined specifically for testing

When returning back to the Main Screen you will see a custom Main Menu defined

the selected card.

To better understand the sample test program implemented, the card in question

needs to be described briefly. The card is a mixed technology PCMCIA card and

has two memory areas: the first area of 1MB is implemented with Flash devices,

more specifically Intel 2MB Flashes EPROMs (28F020s), and the second area is

implemented with Static RAMs.

Flash: bank0 (zone0) 00000-40000     RAM: bank1 (zone2) 80000-a0000

bank1 (zone1) 40000-80000          bank2 (zone3) a0000-c0000

Each area is implemented in two banks (zones). The Test Menu is structured

such as to allow testing of each bank individually. This is helpful in identi-

fying potential manufacturing problems with the card, i.e. ad address pin on

a Flash chip not soldered correctly, a short between two data lines, etc.

The following is a brief description of the different tests that can be per-

formed from the Main Menu when selecting the PCMCIA-1MBFlsh.5MBRAM device:

1. Open and Short Test:

-----------------------

Tests if any address lines and data lines are open or shorted to Vcc and GND.

This test covers the entire address space of the card and therefore both Flash

and RAM devices.

2. Write/Read RAM Bank1 (zone2 80000-a0000) VL (low values for Vcc and Vpp):

------------------------------------------------------------------------------

This test writes a data pattern to RAM location and reads the data back. For

reason of speed and efficiency, not all locations are written to and read back

but only the locations that are relevant for checking each address line (i.e.

locations 80000 + 0,1,2,4,8 etc.). The wordsize of the card is 16 bits. For the

RAM are the  Write/Read test actually consists of writing at pairs of locations

8000 plus 0+1,2+3,4+5,8+9,10+11.... the data ffff+0000.

The VL means stands for testing with the low (min) values for Vcc and Vpp and VH

stands for high (max) values for Vcc and Vpp.

3. Write/Read RAM Bank1 (zone2 80000-a0000) VH (high values for Vcc and Vpp):

------------------------------------------------------------------------------

Same as above but for high Vcc and Vpp.

4. Write/Read RAM Bank2 (zone3 a0000-c0000) VL (lower values for Vcc and Vpp):

------------------------------------------------------------------------------

Same as test 2 but for zone3.

5. Write/Read RAM Bank2 (zone3 a0000-c0000) VH (high values for Vcc and Vpp):

------------------------------------------------------------------------------

Same as test 3 but for zone3.

6. Write Flash Bank1 (zone0 00000-40000) VL (low values for Vcc and Vpp) 0s:

------------------------------------------------------------------------------

This test writes zeroes to the relevant addresses in Flash bank1 (i.e. 00000

plus 0,1,2,4,8,10,20...etc.). Each address is verified after being written.

Low values for Vcc and Vpp are being used.

7. Write Flash Bank1 (zone0 00000-40000) VH (high values for Vcc and Vpp) 0s:

----------------------------------------------------------------------------

Same as test 6 but with high values for Vcc and Vpp.

8. Write Flash Bank2 (zone1 40000-80000) VL (low values for Vcc and Vpp) 0s:

----------------------------------------------------------------------------

Same as test 6 but for zone1.

9. Write Flash Bank2 (zone1 40000-80000) VH (high values for Vcc and Vpp) 0s:

----------------------------------------------------------------------------

Same as test 8 with high values for Vcc and Vpp.

A,B,C,D:

------------

Same as tests 6,7,8,9 but ones being written instead of zeroes.

E,F,G,H:

----------------------------------------------------------------

Write test as in 6,7,8,9 only that a data pattern of AA55 is being written

to each relevant address instead of zeroes or ones.

I,K,L,M:

----------------------------------------------------------------

Same as in E,F,G,H only Reading instead of Writing.

N,O,P,Q:

-------

Reading Data into Data buffer for data inspection purposes

R. Edit Buffer:

--------------

Allows inspection / editing of the data buffer (address for each bank= offset

plus 0 - 13 (buffer is 8 bit, card data is 16 bit).

B. 28C256,28C64 (Manufacturer: TURBO-IC):

-----------------------------------------

Note: Although the tests implemented for this device are real, the device

itself and the Manufacturer are fictitious.

The test programs implemented enable the user to perform Redundancy Repair at

wafer level for a certain type of EPROM. This type of operation becomes more

and more important as memory chips become larger and larger. The operation

consists of basically swapping out a bad memory bank (zone) and connecting

in its place a spare good memory bank (zone)

(Details on the program to be added later. Call B&C Microsystems Technical

Support if you need assistance in writing a similar program for your own

needs.)

HOW TO USE ADEL (Algorithm Development System) WITH THE TEST LIBRARY:

====================================================================

1. Select Options from COMMANDS menu.

2. Tap SPACE to select TEST device library.

3. Press ESC to return to COMMANDS menu.

4. Choose PCMCIA-1MBFlsh.5MBRAM card (inspect/change the algo for this device)

5. Select ADEL (Algo).

6. From Device Record screen you may access elements of the algorithm.

a. To view pin record, first move cursor to pin record field, then

press F6.  You will now be in the pin record editor.

b. To view family record, first move cursor to family record field, then

press F6.  You will now be in the family record editor.

c. To view test  code, first perform step b. to enter family record editor

Then move cursor to record number argument of #load(WAV, ?, ?, ?) directive.

Then press F6.

Example:

#load(WAV, 1, 4, 9, 16)

^

|

Place cursor here then press F6 to view/edit record.

@

Check the PROTEUS.REV file for new devices and features added to the last software release.

$