1. Confirmation of version of firmware

- Press the TEST button on the adapter.

Hold It down until the T(Test) LED comes on. ------ Test print start!

- Please confirm the Firmware version of Setup page.

If Firmware is "14.134.1", the update is unnecessary.

If Firmware is not “14.134.1", the update is necessary. → To clause 2.

(3) Choose [Adapter Settings].

(2) Double-click the printer name on screen.

(1) Double-click the MarkVision icon.

2. Upgrade the firmware by MarkVision

(4) Click [Flash Adapter…].

(8) Choose "pos.10.10.ppn" file.

(9) Click [Accept]. ------ Start updating!

(6) Click [Locate Flash File(s)].

(5) Click [OK].

(7) Select the Drive and the Folder.

(10) Choose "MarkNet XLe" of the Adapter type(s).

(11) Click [Start]. ------ Start updating! ( The update is a few minutes.)

(8) Choose "ra04_134.prp" file.

(9) Click [Accept].

(15) Finish updating.

(14) Click [x] on the upper right corner.

(12) Click [Done].

(13) Click [OK].

(1) Hold down the [TEST] button

3. The board is initialized

(2) When the T(Test) LED comes on, press the [TEST] button.

Release the [TEST] button, but continue to hold the [SE] button.

(3) The LED's will count from 7 to 0 In binary.

When only T(Test) LED comes on, release the SE button.

→ Initialization starts.( about one minute )

(4) When only S(Status)LED comes on,Initialization is completed.

4. Confirmation of version of firmware

- Press the TEST button on the adapter.

Hold It down until the T(Test) LED comes on. ------ Test print start!

- Please confirm the Firmware version.

If Firmware is "14.134.1", the update is completion.

If Firmware is not “14.134.1", the update is necessary. → To clause 2.