The Virtual Disk Driver

for

Fujitsu M2531B

for

Novell Networks

© Copyright 1995, Fujitsu Europe Limited

2, Longwalk Road : Stockley Park : Uxbridge : UB11 1AB

MiniEddy Virtual Disk Driver for Netware

PROGRAM VERSION 1.51

Description

This Network Loadable Module makes the Fujitsu M2531B1 and the M2531B2 optical jukeboxes appear on the host fileserver as 3 virtual disk drives whose capacities are bounded by the media present in each internal magazine. Each magazines can be fully or sparsely populated with 128MB and 230MB media giving the optical library a capacity ranging from 128MB to the maximum of 7.64 Gigabytes. These virtual disks can then be manipulated and configured in exactly the same way as that of a hard disk.

Volumes may span “disks”.
Multiple volumes available within a “disk”.

Here are some examples :

35 × 230MB Magneto-Optical Disks inside the M2531 ...

Cleaning disk

| Cln | 1... |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ... | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

would equate to 3 virtual disks on the system ...

|  | Virtual Disk 1 | Virtual Disk 2 | Virtual Disk 3 |
| --- | --- | --- | --- |
| Cln | (11 MO cartridges) | (12 MO cartridges) | (12 MO cartridges) |

which can be configured as (maximum capacities) ...

|  | Volume A | Volume B | Volume C |
| --- | --- | --- | --- |
| Cln | (2.4 Gigabytes) | (2.62 Gigabytes) | (2.62 Gigabytes) |

OR

|  | Volume A |
| --- | --- |
| Cln | (7.64 Gigabytes) |

OR

|  | Volume A | Volume B |
| --- | --- | --- |
| Cln | (5.02 Gigabytes) | (2.62 Gigabytes) |

OR

|  | Volume A | Vol B | Vol C | Volume D | Vol E |
| --- | --- | --- | --- | --- | --- |
| Cln | (2.4 Gigabytes) | (1.31 GB) | (1.31 GB) | (1.74 GB) | (0.88GB) |

The permutations are endless but some restraint must be exerted to limit them to what is practical. These restraints are detailed later in the Questions & Answers section.

Instructions for Use

1.	Install the SCSI host adapter and configure the disk driver as per the instructions for that device. Make sure that the SCSI card disk driver and the ASPI layer of the SCSI host adapter are loaded.

Refer to your host adapter documentation for full details.

2.	Set the SCSI IDs of the three devices contained within the jukebox. See the following options /S section for further details.

| Device | ID |
| --- | --- |
| Accessor Optical Disk Drive 1 Optical Disk Drive 2 | 2 3 4 |

3.	Set SW-4 of the ODD1 and ODD2 switches at the back of the jukebox to the ON position. This will set the device type to “optical”.

4.	Attach the M2531B2 to the network fileserver with a good quality cable and ensure that the bus is terminated correctly using the terminator supplied with the library if necessary. Note that the M2531 has about a metre of SCSI bus cable within the system.

5.	Load media into each magazine. The jukebox and driver will accept mixed media, either 230MB or 128MB media in any slot of the magazine. Slot 1 (sometimes marked CLN) of magazine 1 is expected to be filled with a cleaning cartridge. Do not insert media in this slot.

6.	Copy the MINIEDDY.NLM from floppy to the server hard disk and boot the fileServer with Netware 3.11, 3.12, 4.01, or 4.1

Type at the console :

LOAD [path[:]]MINIEDDY <options>

SCAN FOR NEW DEVICES (v3.12 and v4 only)

LOAD INSTALL

Options

/Sx:y	This switch is used to force the SCSI id of a data transfer device or ODD. Where: x = device number (1 or 2), y = scsi id ( 0 - 7).  The NLM will attempt to ascertain the SCSI ID of each associated data transfer device (optical disk drive) direct from the miniEddy but if this fails or you experience problems with the device not being recognised then it will default to:
device #1 scsi id = minieddy_ARC scsi id + 1
device #2 scsi id = minieddy_ARC scsi id + 2

The /S switch can be used to override these defaults.

load c:minieddy /s1:1 /s2:2
would force the driver to recognise and use the optical disk drives at IDs 1 and 2. Make sure that the ODDs are set to these IDs.

/Dnn	This switch is used to specify the maximum delay time for a read or write request i.e. the maximum amount of time that a request can be kept waiting while other requests are being processed. The delay value nn is the time in seconds. The default value for this delay is 120 seconds.

/Lnn	This switch is used to specify the minimum amount of time a piece of media should remain loaded while it still has read or write requests to process. The value nn is the time in seconds. The default value for min load time is 60 seconds. This parameter is used to ensure that newly loaded media is not unloaded too quickly due to an old request.

/En 	This switch is used to specify the error reporting level. Errors are reported to the system console.Levels can assume the following values -:

0	disable all messages
1	base level error messages (default)
2	medium level error messages
3	extended error messages

7.	Load INSTALL from the console and MiniEddy will now be seen as 3 virtual disks.  whose capacities are bounded by the number and type of MO disks in each magazine.

From the Install Main Menu....

Select <<Disk Options>>

Select <<Format (optional)>>

Select each miniEddy virtual disk in turn and format.

After each piece of media has had a unique signature written to it then installation of partitions and volumes should continue as per your Novell Netware Installation documentation. It should be noted that once a volume has been defined then its capacity cannot be changed. Media cannot be added to a magazine at a later date.

Special considerations

The MINIEDDY.NLM expects there to be a head cleaning cartridge in the slot marked CLN which is the first slot in magazine 1. Automatic cleaning of the drives will be undertaken by the driver and requires no user intervention. Ensure that the cartridge used is the one that is supplied with the M2531.

Magazine 1 total capacity when fully populated with 230MB media and a volume is specified to occupy all available space: 
Total volume space :2,400,800K Bytes

Magazine 2 & 3 total capacity when fully populated with 230MB media and a volume is specified to occupy all available space:
Total volume space :2,621,212K Bytes

Fileserver memory requirements

Netware Version 3.11 and 3.12

The same rules apply to the M2531 using the Virtual Disk Driver as to any fixed disk with a large capacity. Determine what sort of data you will be storing as to what block size will best suit your needs. Every file stored on the library will occupy at least one block. A small block size (the default in Novell is 4K) will increase your RAM requirements on the server whilst a larger block size of 16k, 32k and 64k will significantly decrease the need for extra RAM.

For each DOS volume :

MEMORY = 0.023 * VolumeSize(MB) / BlockSize

For each volume with ADDED NAME SPACE

MEMORY = 0.032 * VolumeSize(MB) / BlockSize

Add the requirements for each volume and round up to the nearest megabyte not forgetting to add the result of any existing volumes that are likely to be mounted at the same time such as SYS:

Add 2MB for the operating system

Netware Version 4

Netware Vesion 4 is slightly more involved but here are details of the volume requirements. Refer to Appendix A of the Netware installation guide for full details.

FAT = 
	[Volume Blocks] × 8.2 bytes
Block suballocation enabled = 
	(((Blocksize ×2) -1) ×4096) + (5 × Number of Files) bytes
Directory Entry Tables (DET) =
	Number of Files × 10 bytes

The approximate number files can be calculated by dividing the volume size by the average number of files.

Volume Blocks can be calculated by dividing the volume size by the block size.

Tray locking

Tray locking is performed by turning and removing thekey from the control fascia panel. Pressing the EJECT button while the key is in the enable position will cause all media to be returned to their original slots and all volumes to be immediately dismounted.

Questions & Answers

1.	The MO cartridges from one magazine have got mixed up. Will I experience any difficulty in accessing the data.

The data does not need to be recovered. As long as all disks pertaining to that particular volume or volumes are returned to the magazine from whence they came, irrespective of any sequence then the software driver will intelligently attempt to read and associate those disks to that volume.

2.	I've done my calculations and I need another 44MB of RAM on the 3.12 server to be able to see the 7.6GB of storage space available on the optical jukebox! This wasn't budgeted for, what should I do?

The same rules apply to the optical jukebox as to any large capacity fixed disk. Novell Netware needs to cache as much of the file system directory as possible so increasing the fileserver memory would be advantageous in any circumstance. If this is impossible then you may have to define the M2531 into smaller volumes and restrict the number of mounted volumes at any one time. Another solution would be to increase the volume block size from the default of 4k to something like 16k. If this still poses a problem then increase the block size even further to 32k or 64k. The Novell manual detailing Network Concepts is a source for further reading on the subject of volume block sizes and data type.

3	As the magazines in the loading tray are removable, can this driver be used as a backup solution for my fixed disk volumes?

Yes.  In fact you can even backup to the optical disk library without any specialist backup software. Make sure that the Archive bit is set on each file to be backed up then use NCOPY with the /S parameter to copy all subdirectories and /M to copy only those file that have the archive bit set then reset it after copying. Depending on how much data is to be copied, the first backup will take some time but subsequent backups will only copy new files and those files that were changed since the last backup.

NCOPY fs1/data: fs1/mEddy1: /S /M

NCOPY fs1/acct: fs1/mEddy2: /S /M

NCOPY fs1/imgs: fs1/mEddy3: /S /M

The advantage is that those backed up files are now instantly available. If the original volume is unavailable for any reason then the optical jukebox can be used as a direct source for files with only a noticably slight depreciation in effective performance.

4.	Can the jukebox act as an extra online hard disk on the fileserver?

Yes it can but because of the mechanical nature of miniEddy, workstation users may not be aware of the need for media to be manipulated and may be put off by the unusual response times to loading and saving data.

5.	Having loaded the virtual disk driver, I can see the three disks associated with the library but the two internal optical disk drives appear aswell. Is this correct?

Yes it is, but do not try to install a partition on those drives. The best way though to overcome this situation is to specify command line options at the time of loading the SCSI driver that will prevent Netware from reporting those drives. In the case of an Adaptec host the command line option would be REMOVABLE=OFF

6.	Having loaded the virtual disk driver and scanned for new devices, I can’t see the three disks associated with the library.

Make sure that the ASPI layer of the host adapter driver has been loaded. In some cases this is an automatic process but in others the command must be specified in the AUTOEXEC.NCF. In the case of a Buslogic adapter the command would be :

;load host adapter driver and exclude

;the internal optical drives

LOAD [pathname]BT31X.DSK EXCLUDE_IDS=2, 3

;load aspi layer

LOAD [pathname]BTASPI.DSK

7	I have noticed a degradation in performance when more and more volumes are mounted. Is there a solution for this?

If those mounted volumes are spread across the three virtual disks then this is likely to happen because, depending on which model of library you own, there will be either one or two reading devices (optical disk drives) servicing the three virtual disks. Try to keep the number of mounted volumes to a minimum at all times.

8	The jukebox CHECK1 lamp lights and the machine ceases to operate when one particular piece of media is being loaded to the picker arm. I have checked the media and that is OK. What is wrong?

The jukebox has rather sophisticated optical sensors that detect where the media is located at any time. If there are dark markings on the spine of the media then this or the media label is black then this will confuse those sensors. Remove any markings from the spine of the disk or remove the label and try again.

Example  AUTOEXEC.NCF from Netware 3.12

FILE SERVER NAME ARCH_01

IPX INTERNAL NET D00BED00

REGISTER MEMORY 1000000 1000000

LOAD AIC7870 SLOT=33 MEM=F400 INT=9 REMOVABLE=OFF

LOAD ENPCI SLOT=16 FRAME=ETHERNET_802.3 Name=IPX_NET

BIND IPX to IPX_NET NET=E9E90

MOUNT SYS

MOUNT DATA

LOAD MINIEDDY /S1:1 /S2:2 /E3

SCAN FOR NEW DEVICES

MOUNT LIB1

MOUNT LIB2

Notes

Load AIC7870 uses the REMOVABLE=OFF parameter which allows the driver to ignore any removable devices connected to the SCSI bus. This will in effect remove any reference in the operating system to the optical disk drives that are physically inside the optical library.

Volumes SYS and DATA are specifically mounted by name before loading the optical library device driver. Loading and mounting the volumes can take between 25 and 30 minutes depending on the size of the volumes as each piece of media has to be checked , normal access to the server will not be delayed.