GDS-830 PC remote control program (for RS-232)

Version 1.3

Overview

Thank you for using GDS-830 PC remote control program. This program is a demo tool for exploring our superior GDS-830. Users can acquire the waveform data from GDS-830 digitizing oscilloscope to personal computer via RS-232 port. The waveform data can also be saved into several graphic formats, perfect for data presentation use. The totally thirty of measurement readout (channel1 and channel2) can be displayed at same time, suitable for multi-measurement. For this moment, English, Chinese (traditional), French, Italian, and Indonesian are supported; however, more languages support will be included in the near future.

Figure1. This graphic user interface of GDS-830 PC remote control program

Features

- GDS-830 PC remote control program can update the waveform display on personal computer quick quickly. Almost near to real-time display! Best for on line waveform monitoring!

- The graphic user interface is friendly, easy to use. Even un-training person can easily to operate our software.

- This program can save your waveform image as

-BMP,

-PCX,

-TIFF,

-PNG

-JPEG

Users can embedded these images into any application for their own particular purpose. For instance, users can embed the image into Microsoft® Word program and make an impressive document.

- The saved image files can be re-loaded into screen, perfect for waveform evaluation and comparison.

- The wave image can input any text descriptions as you like. This feature is good for users who want to make descriptions on the waveform image files.

- The display area can be enlarged as 4 times of original size, good for waveforms observation! The waveform details can easily to see!

- The traces and background color can be changed as many as possible. Select the favor color as you preferred.

- Thirty measurements (channel 1+channel 2) can be display simultaneously. This feature can display your specific measurement items on the screen in the same time! Users can monitor all necessary measurement items simultaneously. Best for the production lines of manufacturers.

- Multi-languages are supported (English, Chinese (traditional), French, Italian, and Indonesian for this moment). User just operate this software directly, no translation problem!

- The printout data sheet will include the details of horizontal and vertical data. Make your report more precisely, that is great for your data reports!

GDS-830 PC remote control program is plenty of features that make signal measurement as easy as possible!

Requirement

Installation of GDS-830 PC remote control program requires the following:

- An IBM compatible personal computer with a minimum Pentium-200 Class Microprocessor.

- GDS-830 PC remote control program requires Windows 95(OSR2)/Windows 98/ 98SE / ME with 64 MB RAM, Windows 2000 with 128 MB RAM.

- If you use Windows 2000 you must be Administrator to install GDS-830 PC remote control program.

- Roughly 20 Mega-Byte of free disk space for the GDS-830 PC remote control program Installation.

Installation

To install GDS-830 PC remote control program, run the Setup.exe file and follow the instructions.

Removing GDS-830 PC remote control program

To remove GDS-830 PC remote control program from your computer system, go to Start->Settings->Control Panel->Add/Remove Programs, GDS-830 PC remote control program will be listed as "GDS-830 PC remote control program". The removal system is automated and requires very little user intervention (Simply click OK to confirm that you wish to remove GDS-830 PC remote control program).

Starting this program

- Turn on GDS-830

- The PC program’s RS232 setting is following:

a. Go to Setup->RS-232, Click the RS232 setting icon

b. Select the suitable RS232 settings and click “OK”

c. If the RS232 settings are correctly, the scope’s waveform will be displayed. The “Connection Indicator” will also turn green and the message of “Connected with PC; Scope keyboard locked” will be displayed. For this moment, the scope’s keyboards are locked.

d. If click “Connect” button again, the “Connection Indicator” will turn red and the message of “Disconnected with PC; Scope keyboard unlocked” will be displayed. For this moment, user can operate the scope’s keyboards directly.

Operation

- File Menu

- Load/Save Setup：User can save/load the current connection setup as a file.

- Save Waveform：The save your waveform image as a BMP, PCX, TIFF, PNG or JPEG file

File Menu

- Save：Save current display waveform area to a file.

User can easy input any text into the display area. Just click the “Text Input” box and key in the preferred text, move the mouse to a chosen area and click again afterward. The input text will be attached on the display area.

Note：The “Undo” function can only undo text inputs three times, If text removed over three times, please press “Erase All” button to remove all text inputs.

- Load：All the saved waveforms can be recovered from files.

Note：If any text inputs are attached with the saved waveform file and the file is loaded, the attached text will not be shown on the waveform display area .

- Print/Print Setup：Print out the waveform and printer setup.

  - Setup

Setup menu

- Status Report：All the setup for the present will be display on this window. User can printout or save the results

Status Report

- RS-232：Setup the RS232 configuration.

NOTE: For this moment, only 19200 Baud rate is available!

RS-232 setup menu

- Measure：Totally 30 measured data can be displayed on the program. 16 measured data can display simultaneously.

Measurement menu

- Trigger：select the preferred trigger items and press OK

Trigger menu

- Acquisition：only sample mode and 500 memory length are available on this program.

Acquisition menu

- Display Setup：select the preferred setup and press OK

Display setup

- Channel 1 /Channel 2：select the correct setup and press OK

Channel 1/2 setup menu

- GUI display

- Remove：Remove the attached waveforms

- Graphic：Select the color of waveform displays

Graph setup

- Channel Display：Select channel 1/2 display on PC or not

- Cursor：Select the cursor T/V (horizontal/vertical) on PC or not

- Connection Indicator：Please refer to page 5

- Start both communication between GDS-830 and personal computer.

- Autoset：Select the autoset function for GDS-830.

- Close：Quit the program.

- Screen：Click this area twice by mouse will large 4 times the display area.

Large view of waveform display area

- Select the preferred Volt/div and Sec/div ranges.

Volt/div selection window

Sec/div selection window

- Adjust the channel 1/2 vertical and horizontal positions. Also the trigger level. Included

- Multi-language support. English, Chinese, French, Italian and Indonesian are supported now!

- program version information