GRS 60x2 PC Remote Control Program (for RS-232)

Version 1.1

2002/06/11

Overview

Thank you for using GRS-60X2 PC remote control program. This program is a powerful tool for exploring our superior GRS-60X2. Users can acquire the waveform data from GRS-60X2 digitizing oscilloscope to personal computer via RS-232. The waveform data can also be saved into several graphic formats, perfect for data presentation use.

Features：

- GRS-60X2 PC remote control program can update the waveform display on personal computer very quickly (depending on baud rate).

- The graphic user interface is friendly, easy to use.

- This version can save your waveform image as a BMP, PCX, TIFF, PNG or JPEG file.

- The saved image files can be re-loaded into screen, perfect for waveform evaluation.

## Installation:

Unzip the zip-file in a temporary directory and run the setup program.

Run:

Step 1. Connect RS232 cable to GRS-60x2 and computer.

Setp 2. Turn on GRS-60x2

Setp 3. Press UTILITY bottom 3 times and set baud-rate by VARIABLE knob.

Setp 4. Execute GRS.exe in your PC.

Setp 5. Click  to set baud-rate (Max is 9600).

Step 6. Click  to select channels (select one channel at least).

Setp 7. Click  to communicate GRS-60X2 and personal computer.

Setp 8. Click  to Storage mode

Step 7.

Step 8

Step 6

Step 5

# Operation

# A

- Waveform Screen

- Pri-Trigger position control.

# B

- Load：All the saved waveforms can be recovered from files.

- Remove：Remove the attached waveform.

- Save Waveform：The save your

    - waveform image as a BMP, PCX, TIFF, PNG or JPEG file

    - waveform data as a .csv file

- Graphic：Select the color of waveforms

- Print Setup：Print function.

# C

- Channel Display：Select channel 1/2 display on/off.

- Cursor：Select the cursor T/V (horizontal/vertical) on/off

- RS-232：Setup the RS232 configuration.

# D

- Trigger：Select trigger functions.

- Display Setup：Select Text/Illumine intensity, smooth and interpolation types.

- Average：Select average function.

- Start both communication between GRS-60X2 and personal computer or Unlock the keyboard of front panel for GRS-60X2.

- Channel 1 /Channel 2：select the correct setup and press OK

- Run/Stop：Run/Stop switchl

- RealTime/Storage: Switch RealTime/Storage mode.

- Reset: Factory Default Load.

# E

- Select the preferred Volt/div and Sec/div ranges.

# F

- Invert

- ADD

- ALT/CHOP

- MAG(x1, x5,x10 and x20)

# G

Message Box: Including Realtime/Storage mode, Run/Stop, and Time.

# H

- State of received data.

- Waveform transmit Start/Stop button.

- Commands to GRS-60x2.

- Close this program.