Good Will SFG-830 Function Gen. (VISA)

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for Good Will SFG-830 Function Gen. (VISA).

It contains functions for opening, configuring, taking measurements from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

-  the instrument is connected to the GPIB.

-  the GPIB address supplied to the initialize function must match the

GPIB address of the instrument.

________________________________________________________________________________

Error and Status Information:

Error codes and warnings are returned as the return value of each instrument

driver function. A program should examine this value after each call to an

instrument driver function to determine if any error occurred.  The general

meaning of the return value is as follows:

Value                   Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

Possible error codes and their meanings are listed with the corresponding

instrument driver function.

________________________________________________________________________________

How To Use This Document:

This document is intended to be used as a programming reference manual.

It describes each function in the Good Will SFG-830 Function Gen. (VISA)

instrument driver. The functions are listed in alphabetical order with a

description of the function, C syntax of the function, a description of each

parameter, and possible error codes.

________________________________________________________________________________

Function Tree Layout:

Good Will SFG-830 Function Gen. (VISA) Function Name:

Initialize                                  gwsfg830_init

Application Functions

Generate Waveform                        gwsfg830_generateWaveform

Configuration Functions

Configure Waveform                       gwsfg830_configWvfm

Configure AM/FM                          gwsfg830_configAMFM

Configure PM                             gwsfg830_configPM

Configure Sweep Type                     gwsfg830_configLogLin

Action/Status Functions

Set Modulation                           gwsfg830_setmodulation

Data Functions

Read Instrument Setting                  gwsfg830_dataReadSetting

Utility Functions

Write To Instrument                      gwsfg830_writeInstrData

Read Instrument Data                     gwsfg830_readInstrData

Reset                                    gwsfg830_reset

Self-Test                                gwsfg830_selfTest

Error-Query                              gwsfg830_errorQuery

Error Message                            gwsfg830_errorMessage

Revision Query                           gwsfg830_revisionQuery

Close                                       gwsfg830_close

________________________________________________________________________________

Good Will SFG-830 Function Gen. (VISA)

This instrument module provides programming support for the

GOOD WILL SFG-830 Synthesized Function Generator(VISA I/O).The module is

divided into the following functions:

Functions/Classes:

(1) Initialize:

This function initializes the instrument and sets it to a default

configuration.

(2) Application Functions: (Class)

This class contains high-level, test and measurement routines.  These

examples call other instrument driver functions to generate waveform by

setting some conditions.

(3) Configuration Functions: (Class)

This class of functions configures the instrument by setting acquisition

and system configuration parameters.

(4) Action/Status Functions: (Class)

This class of functions begins or terminates an acquisition. It also

provides functions which allow the user to determine the current status

of the instrument.

(5) Data Functions: (Class)

This class of functions transfers data to or from the instrument.

(6) Utility Functions: (Class)

This class of functions provides lower level functions to communicate

with the instrument, and change instrument parameters.

(7) Close:

This function takes the instrument offline.

________________________________________________________________________________

The following functions are in alphabetical order.

________________________________________________________________________________

gwsfg830_close

ViStatus gwsfg830_close (ViSession instrumentHandle);

Purpose

This function performs the following operations:

viClose (instrSession) and viClose (rmSession).

Notes:

(1) The instrument must be reinitialized to use it again.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

gwsfg830_configAMFM

ViStatus gwsfg830_configAMFM (ViSession instrumentHandle,

ViInt16 modulationWaveform,

ViBoolean modulationType,

ViReal64 depthorSpanorHz, ViReal64 sweepRateHz);

Purpose

This function sets up the AM or FM modulation: depth/deviation, sweep

rate, modulation waveform,and then turns the selected modulation on.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

modulationWaveform

Variable Type       ViInt16

The Modulation Waveform control specifies the type of

waveform to generate.

Valid Range:

1 - Sine (Default)

2 - Square

3 - Triangle

4 - Ramp

modulationType

Variable Type       ViBoolean

This control specifies whether the modulation type is AM or FM.

Valid Range:

VI_OFF (0) - AM (Default)

VI_ON  (1) - FM

depthorSpanorHz

Variable Type       ViReal64

This control specifies either the depth or span dependent upon the

setting of the 'Modulation Type'control. If AM is selected then then

Depth specifies the AM modulation depth of the

output waveform. If FM is selected then Span specifies the FM

modulation span of the output waveform

Valid Range: Dependent upon the 'Modulation Type'control

AM (VI_OFF): -100% to 100%

FM (VI_ON):  0.01 Hz to 30 MHz

Default Value: 100%

sweepRateHz

Variable Type       ViReal64

The modulation Rate control specifies the time needed to perform.

Valid Range: 0.01 to 10 kHz

Default Value: 1 kHz

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFC09F0  Warning Low Battery

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Modulation Waveform) out of range.

BFFC0003  Parameter 3 (Modulation Type) out of range.

BFFC0004  Parameter 4 (Depth or Span) out of range.

BFFC0005  Parameter 5 (Sweep Rate) out of range.

BFFC09F2  Error Programming Error

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

gwsfg830_configLogLin

ViStatus gwsfg830_configLogLin (ViSession instrumentHandle, ViInt16 sweepType,

ViInt16 modulationWaveform,

ViReal64 startFrequencyHz,

ViReal64 stopFrequencyHz, ViReal64 sweepRateHz);

Purpose

This function sets up a Linear or Logarithmic Sweep: start and stop

frequencies, sweep rate, and the modulation waveform. The stop frequency

must be greater than the start frequency.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

sweepType

Variable Type       ViInt16

This control specifies whether the sweep type is Linear or

Logarithmic

Valid Range:

VI_OFF (0) - Logarathmic

VI_ON  (1) - Linear (Default)

modulationWaveform

Variable Type       ViInt16

The Modulation Waveform specifies whether the

waveform is Triangle or Ramp.

Valid Range:

3 - Triangle

4 - Ramp (Default)

startFrequencyHz

Variable Type       ViReal64

The Start Frequency control specifies the lowest frequency in the

sweep range. The start frequency must be less than the stop

frequency.

Valid Range: 0.0 to 30.0 MHz

Default Value: 1 kHz

Notes: If the 'Stop Frequency' is not greater than the 'Start

Frequency' the 'Stop Frequency' control will generate an "out of

range" error.

stopFrequencyHz

Variable Type       ViReal64

The Stop Frequency control specifies the highest frequency in the

sweep range. The stop frequency must be greater than the start

frequency.

Valid Range: 0.0 to 30.0 MHz

Default Value: 10 kHz

Notes: If the 'Stop Frequency' is not greater than the 'Start

Frequency' the 'Stop Frequency' control will generate an "out of

range" error.

sweepRateHz

Variable Type       ViReal64

The Sweep Rate control specifies the time needed to perform.

Valid Range: 0.01 to 1 kHz

Default Value: 1 kHz

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFC09F0  Warning Low Battery

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Sweep Type) out of range.

BFFC0003  Parameter 3 (Modulation Waveform) out of range.

BFFC0004  Parameter 4 (Start Frequency) out of range.

BFFC0005  Parameter 5 (Stop Frequency) out of range.

BFFC0006  Parameter 6 (Sweep Rate) out of range.

BFFC09F2  Error Programming Error

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

gwsfg830_configPM

ViStatus gwsfg830_configPM (ViSession instrumentHandle, ViReal64 phasedeg,

ViReal64 sweepRateHz);

Purpose

This function sets up the PM: phase and rate; and then turns the PM

modulation on. The function sets the modulation waveform to  a ramp wave

and the waveform function that is active must be a sine wave.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

phasedeg

Variable Type       ViReal64

The PHASE controls specifies the phase shift of the output waveform.

Valid Range: 0 to 360 degrees

Default Value: 100 degree

sweepRateHz

Variable Type       ViReal64

The Sweep Rate control specifies the time needed to perform.

Valid Range: 0.01 to 10 kHz

Default Value: 1 kHz

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFC09F0  Warning Low Battery

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (phase) out of range.

BFFC0003  Parameter 3 (rate) out of range.

BFFC09F2  Error Programming Error

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

gwsfg830_configWvfm

ViStatus gwsfg830_configWvfm (ViSession instrumentHandle, ViInt16 waveformType,

ViReal64 sampleFrequency, ViReal64 amplitude,

ViReal64 DCOffset);

Purpose

This function causes the instrument to generate a waveform.  This

function sets the Waveform type, Frequency, Amplitude and DC Offset.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

waveformType

Variable Type       ViInt16

This control programs the instrument to generate the specified

waveform.

Valid Range:

1 - Sine (Default)

2 - Square

3 - Triangle

4 - Ramp

5 - Arbitrary

sampleFrequency

Variable Type       ViReal64

The Frequency control specifies the frequency of the output waveform.

Valid Range: Dependent upon selected waveform

Sine/Square/Arbitrary - 0.0 to 30.0 MHz

Triangle/Ramp - 0.0 to 100.0 kHz

Default Value: 1.0E5 Hz

amplitude

Variable Type       ViReal64

The AMPLITUDE control specifies the voltage level of the waveform.The

peak-to-peak voltage is twice the value of the amplitude.

Valid Range: 0.0 to 10.0 Volts

Default Value: 1.0 V

Notes:

(1) The sum of the amplitude & the DC offset voltage cannot exceed

10.0 Volts.

(2) Although the amplitude range for the arbitrary waveform is

specified here, this control is not used to set the arbitrary

waveform amplitude. The Amplitude is sent as data points using an

array or a .WAV file.

DCOffset

Variable Type       ViReal64

The DC OFFSET control specifies the amount of DC voltage

present in the output waveform.

Valid Range: -5.0 to +5.0 Volts

Default Value: 0 V

Notes:

(1) The sum of the amplitude & the DC offset voltage cannot exceed

10.0 Volts.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFC09F0  Warning Low Battery

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Waveform Type) out of range.

BFFC0003  Parameter 3 (Sample Frequency) out of range.

BFFC0004  Parameter 4 (Amplitude) out of range.

BFFC0005  Parameter 5 (DC Offset) out of range.

BFFC09F2  Error Programming Error

BFFC09F4  Amplitude+Offset > 10V

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

gwsfg830_dataReadSetting

ViStatus gwsfg830_dataReadSetting (ViSession instrumentHandle,

ViInt16 messageType, ViChar message[]);

Purpose

This function queries the instrument for the specifed data, status, or

setup information.  The available queries are as follows:

- programming error list

- query frequency value

- query amptiude value

- query offset value

- query phase value

- query rate value

- query span value

- query depth value

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

messageType

Variable Type       ViInt16

This control determines the message to be read from the instrument.

Valid Range:

0 - Error List (Default Value)

1 - Frequency

2 - Amptiude

3 - Offset

4 - Phase

5 - Rate

6 - Span

7 - Depth

message

Variable Type       ViChar[]

This control specifies the name of the array into which the incoming

data from the instrument is placed.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Message Type) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

gwsfg830_errorMessage

ViStatus gwsfg830_errorMessage (ViSession instrumentHandle, ViStatus statusCode,

ViChar message[]);

Purpose

This function takes the Status Code returned by the instrument driver

functions, interprets it and returns it as a user readable string.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  VI_NULL

statusCode

Variable Type       ViStatus

This control accepts the Status Code returned from the instrument

driver functions.

Default Value:

0 - VI_SUCCESS

message

Variable Type       ViChar[]

This control returns the interpreted Status Code as a user readable

message string.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

3FFF0085  VI_WARN_UNKNOWN_STATUS

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

gwsfg830_errorQuery

ViStatus gwsfg830_errorQuery (ViSession instrumentHandle, ViInt32 *errorCode,

ViChar errorMessage[]);

Purpose

This function reads an error code from the instrument's error queue.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

errorCode

Variable Type       ViInt32 (passed by reference)

This control returns the error code read from the instrument's error

queue.

errorMessage

Variable Type       ViChar[]

This control returns the error message string read from the

instrument's error message queue.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

gwsfg830_generateWaveform

ViStatus gwsfg830_generateWaveform (ViSession instrumentHandle,

ViInt16 waveformType,

ViReal64 sampleFrequency,

ViReal64 amplitude, ViReal64 DCOffset);

Purpose

This application function causes the instrument to generate a waveform.

This function sets the Waveform type, Frequency, Amplitude and DC Offset.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

waveformType

Variable Type       ViInt16

This control programs the instrument to generate the specified

waveform.  Selecting "Current Wvfm" leaves the waveform type

unchanged.

Valid Range:

1 - Sine (Default)

2 - Square

3 - Triangle

4 - Ramp

5 - Arbitrary

sampleFrequency

Variable Type       ViReal64

The FREQUENCY control specifies the frequency of the output waveform.

Valid Range: Dependent upon selected waveform

Sine/Square/Arbitrary - 0.0 to 30.0 MHz

Triangle/Ramp - 0.0 to 100.0 kHz

Default Value: 1.0E5 Hz

amplitude

Variable Type       ViReal64

The AMPLITUDE control specifies the voltage level of the waveform.The

peak-to-peak voltage is twice the value of the amplitude.

Valid Range: 0.0 to 10.0 Volts

Default Value: 1.0 V

Notes:

(1) The sum of the amplitude & the DC offset voltage cannot exceed

10.0 Volts.

DCOffset

Variable Type       ViReal64

The DC OFFSET control specifies the amount of DC voltage

present in the output waveform.

Valid Range: -5.0 to +5.0 Volts

Default Value: 0 V

Notes:

(1) The sum of the amplitude & the DC offset voltage cannot exceed

10.0 Volts.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFC09F0  Warning Low Battery

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Waveform Type) out of range.

BFFC0003  Parameter 3 (Sample Frequency) out of range.

BFFC0004  Parameter 4 (Amplitude) out of range.

BFFC0005  Parameter 5 (DC Offset) out of range.

BFFC09F2  Error Programming Error

BFFC09F4  Amplitude+Offset > 10V

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

gwsfg830_init

ViStatus gwsfg830_init (ViRsrc resourceName, ViBoolean IDQuery,

ViBoolean resetDevice, ViSession *instrumentHandle);

Purpose

This function performs the following initialization actions:

- Opens a session to the Default Resource Manager resource and a session

to the specified device using the interface and address specified in the

Resource_Name control.

- Performs an identification query on the Instrument.

- Resets the instrument to a known state.

- Sends initialization commands to the instrument that set any necessary

programmatic variables such as Headers Off, Short Command form, and Data

Transfer Binary to the state necessary for the operation of the

instrument driver.

- Returns an Instrument Handle which is used to differentiate between

different sessions of this instrument driver.

- Each time this function is invoked a Unique Session is opened.  It is

possible to have more than one session open for the same resource.

/**** DELETE THESE NOTES AND STATUS CODES IF SUPPORTED *****/

Notes:

(1) If this instrument does not support an ID Query, and the ID Query

control is set to "Do Query" then this function should return the Warning

Code 0x3FFC0101 - VI_WARN_NSUP_ID_QUERY.

(2) If this instrument does not support a Reset, and the Reset control is

set to "Reset Device" then this function should return the Warning Code

0x3FFC0102 - VI_WARN_NSUP_RESET.

Parameter List

resourceName

Variable Type       ViRsrc

This control specifies the interface and address of the device that

is to be initialized (Instrument Descriptor). The exact grammar to be

used in this control is shown in the note below.

Default Value:  "GPIB::8"

Notes:

(1) Based on the Instrument Descriptor, this operation establishes a

communication session with a device.  The grammar for the Instrument

Descriptor is shown below.  Optional parameters are shown in square

brackets ([]).

Interface   Grammar

------------------------------------------------------

GPIB        GPIB[board]::primary address[::secondary address]

[::INSTR]

The GPIB keyword is used with GPIB instruments.

The default value for optional parameters are shown below.

Optional Parameter          Default Value

-----------------------------------------

board                       0

secondary address           none - 31

IDQuery

Variable Type       ViBoolean

This control specifies if an ID Query is sent to the instrument

during the initialization procedure.

Valid Range:

VI_OFF (0) - Skip Query

VI_ON  (1) - Do Query (Default Value)

Notes:

(1) Under normal circumstances the ID Query ensures that the

instrument initialized is the type supported by this driver. However

circumstances may arise where it is undesirable to send an ID Query

to the instrument.  In those cases; set this control to "Skip Query"

and this function will initialize the selected interface, without

doing an ID Query.

resetDevice

Variable Type       ViBoolean

This control specifies if the instrument is to be reset to its

power-on settings during the initialization procedure.

Valid Range:

VI_OFF (0) - Don't Reset

VI_ON  (1) - Reset Device (Default Value)

Notes:

(1) If you do not want the instrument reset. Set this control to

"Don't Reset" while initializing the instrument.

instrumentHandle

Variable Type       ViSession (passed by reference)

This control returns an Instrument Handle that is used in all

subsequent function calls to differentiate between different sessions

of this instrument driver.

Notes:

(1) Each time this function is invoked a Unique Session is opened.

It is possible to have more than one session open for the same

resource.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFC09F0  Warning Low Battery

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (ID Query) out of range.

BFFC0003  Parameter 3 (Reset Device) out of range.

BFFC0011  Instrument returned invalid response to ID Query

BFFC0803  Error Scanning Response

BFFC09F2  Error Programming Error

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

gwsfg830_readInstrData

ViStatus gwsfg830_readInstrData (ViSession instrumentHandle,

ViInt32 numberBytesToRead, ViChar readBuffer[],

ViInt32 *numBytesRead);

Purpose

This function reads data from the instrument's output buffer and returns

it to the specified variable in memory.

Notes:

(1) Because the instrument may return both numeric and text data in

response to queries this function returns the data in string format.

(2) If valid data is not available at the instrument's output buffer when

this function is called the instrument will hang up and the function will

not return until it times out.  If the time-out is disabled this function

will hang indefinitely and it may be necessary to reboot the computer to

break out.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

numberBytesToRead

Variable Type       ViInt32

The number of bytes specified by this control should be greater than

or equal to the number of bytes which are to be read from the

instrument. If the actual number of bytes to be read is greater than

the number this control specifies then multiple reads will be

required to empty the instrument's output buffer.

If the instrument's output buffer is not emptied the instrument may

return invalid data when future reads are performed.

Default Value:  50 (See NOTE 2)

Notes:

(1) If NO DATA is available at the instrument's output buffer when

this function is called the instrument will hang up and the function

will not return until it times out.  If the time-out is disabled,

this function will hang indefinitely and it may be necessary to

reboot the computer.

(2) If the number of bytes expected is greater than 50 the value of

this control may be increased. If the actual number of bytes read is

larger than the declared size of the read buffer a run-time error

will be generated.

readBuffer

Variable Type       ViChar[]

The incoming data from the instrument is placed into this variable.

Notes:

(1) Because the instrument may return both numeric and text data in

response to queries this function returns the data in string format.

(2) This function does not overwrite any old data left in the string

variable from the last time the function was called. String data in

LabWindows/CVI is terminated with an ASCII null(0x0) and string

manipulation functions will only recognize data before the ASCII

null.

(3) The declared size of the string variable must be greater than the

actual number of bytes read from the instrument if it is not a

run-time error will be generated.

numBytesRead

Variable Type       ViInt32 (passed by reference)

This variable contains the actual number of bytes read from the

instrument. This is the value which is returned by the read function.

Notes:

(1) If the actual number of bytes read is less than the number of

bytes specified in the Number Bytes To Read control then the output

buffer has probably been emptied. If the read function fails and the

number of bytes read is 0, the most probable cause for the failure is

there was no data available at the instrument's output buffer.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

gwsfg830_reset

ViStatus gwsfg830_reset (ViSession instrumentHandle);

Purpose

This function resets the instrument to a known state and the instrument

setup, waveform, and scratchpad memories to the following default values:

Function            - Sine

Sample Frequency    - 1.0 KHz

Amplitude           - 1.0 Vpp

Offset              - 0.0 V

Modulation Waveform - Sine

Modulation Type     - AM

Start Frequency     - 1.0 KHz

Stop Frequency      - 10 KHz

Rate                - 1.0KHz

Modulation          - Off

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

gwsfg830_revisionQuery

ViStatus gwsfg830_revisionQuery (ViSession instrumentHandle,

ViChar instrumentDriverRevision[],

ViChar firmwareRevision[]);

Purpose

This function returns the revision numbers of the instrument driver and

instrument firmware, and tells the user with which  instrument firmware

this revision of the driver is compatible.

This instrument driver's Revision Number is "Rev 1.0, 12/97, CVI 4.0.1"

and is compatible with Instrument Firmware Revision "3.1".

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

instrumentDriverRevision

Variable Type       ViChar[]

This control returns the Instrument Driver Software Revision.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

firmwareRevision

Variable Type       ViChar[]

This control returns the Instrument Firmware Revision.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

gwsfg830_selfTest

ViStatus gwsfg830_selfTest (ViSession instrumentHandle, ViInt16 *selfTestResult,

ViChar selfTestMessage[]);

Purpose

This function runs the instrument's self test routine and returns the

test result(s).

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

selfTestResult

Variable Type       ViInt16 (passed by reference)

This control contains the value returned from the instrument self

test.  Zero means success.  For any other code, see the device's

operator's manual.

selfTestMessage

Variable Type       ViChar[]

This control contains the string returned from the self test. See the

device's operation manual for an explanation of the string's

contents.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

gwsfg830_setmodulation

ViStatus gwsfg830_setmodulation (ViSession instrumentHandle,

ViBoolean setModulationState);

Purpose

This function turns the function generator modulation On or Off.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

setModulationState

Variable Type       ViBoolean

The MODULATION control specifies if the modulation

is on or off.

Valid Range:

VI_OFF (0) - Off (Default)

VI_OFF (1) - On

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Set Modulation State) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

gwsfg830_writeInstrData

ViStatus gwsfg830_writeInstrData (ViSession instrumentHandle,

ViString writeBuffer);

Purpose

This function writes commands and queries to the instrument to modify

parameters and query device settings.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

writeBuffer

Variable Type       ViString

The user can use this control to send common commands and queries to

the instrument. This control can also be used to write any valid

command to the instrument.

Default Value:  "*RST"

Notes:

(1) The command or query to be sent to the instrument may be a

literal enclosed in double quotes i.e. "*RST" or may be contained in

a variable of type string.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.