GM SSP X  Power Source (VISA)

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for GM SSP X  Power Source (VISA).

It contains functions for opening, configuring, taking measurements from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

For GPIB instrument drivers:

-  the instrument is connected to the GPIB.

-  the GPIB address supplied to the initialize function must match the

GPIB address of the instrument.

For RS-232 instrument drivers:

-  the instrument is connected to the RS-232 interface.

-  baud rate, parity which are set in VISA must

corespond with settings of the instrument.

________________________________________________________________________________

Error and Status Information:

Each function in this instrument driver returns a status code that

either indicates success or describes an error or warning condition.

Your program should examine the status code from each call to an

instrument driver function to determine if an error occurred.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

The description of each instrument driver function lists possible error

codes and their meanings

________________________________________________________________________________

How To Use This Document:

Use this document as a programming reference manual.

It describes each function in the

GM SSP X  Power Source (VISA)

instrument.  The functions appear in alphabetical order, with a description

of the function and its C syntax, a description of each parameter, and a list

of possible error codes.

________________________________________________________________________________

Function Tree Layout:

Class/Panel Name:                           Function Name:

Initialize                               gmsspx_init

Function Examples

Application Example                   gmsspx_AppExample

Configuration Functions

Voltage Configuration

Set Voltage Setting                gmsspx_confSetVolt

Get Voltage Setting                gmsspx_confGetVolt

Set Voltage Limit                  gmsspx_confSetVoltLimit

Get Voltage Limit                  gmsspx_confGetVoltLimit

Set OVP Level                      gmsspx_confSetOVPLevel

Get OVP Level                      gmsspx_confGetOVPLevel

Get Max Voltage                    gmsspx_confGetVoltMax

Current Configuration

Set Current Setting                gmsspx_confSetCurrent

Get Current Setting                gmsspx_confGetCurrent

Set Current Limit                  gmsspx_confSetCurrLimit

Get Current Limit                  gmsspx_confGetCurrLimit

Set OCP Delay                      gmsspx_confSetOCPDelay

Get OCP Delay                      gmsspx_confGetOCPDelay

Set OCP State                      gmsspx_confSetOCPState

Get OCP State                      gmsspx_confGetOCPState

Get Max Current                    gmsspx_confGetCurrMax

Output Configuration

Set Output State                   gmsspx_confSetOutputState

Get Output State                   gmsspx_confGetOutputState

Get Output Mode                    gmsspx_confGetOutputMode

Sequence Configuration

Sequence Setting                   gmsspx_confSetSeq

Set Sequence State                 gmsspx_confSetSeqState

Get Sequence State                 gmsspx_confGetSeqState

Set Sequence Repetition            gmsspx_confSetSeqRepetition

Get Sequence Repetition            gmsspx_confGetSeqRepetition

Set Sequence Start Stop Address    gmsspx_confSetSeqRange

Get Sequence Start Stop Address    gmsspx_confGetSeqRange

Set Sequence Default Time          gmsspx_confSetSeqDefaultTime

Get Sequence Default Time          gmsspx_confGetSeqDefaultTime

Set Sequence Step Time             gmsspx_confSetSeqTime

Get Sequence Step Time             gmsspx_confGetSeqTime

Analog Interface Configuration

Set Sig1_Sig2                      gmsspx_confSetSignalFunc

Get Sig1_Sig2                      gmsspx_confGetSignalFunc

Set Trigger Mode                   gmsspx_confSetTriggerMode

Get Trigger Mode                   gmsspx_confGetTriggerMode

Set Sset                           gmsspx_confSetSset

Get Sset                           gmsspx_confGetSset

MinMax Configuration

Set MinMax State                   gmsspx_confSetMinMaxState

Get MinMax State                   gmsspx_confGetMinMaxState

Set Power-On Mode                     gmsspx_confSetPwrOnMode

Get Power-On Mode                     gmsspx_confGetPwrOnMode

Action/Status Functions

Sequence Control                      gmsspx_actSendSeqTrigger

Reset MinMax                          gmsspx_resetMinMax

Data Functions

Measurement Functions

Fetch Min Max Measurement          gmsspx_fetchMinMax

Meas Output Voltage                gmsspx_measOutVolt

Meas Output Current                gmsspx_measOutCurr

Meas Output Power                  gmsspx_measOutPower

Register Functions

Recall Register                    gmsspx_dataRecallRegister

Save Register                      gmsspx_dataSaveRegister

Store Register Data                gmsspx_dataStoreReg

Read Register Data                 gmsspx_dataReadReg

Store Register Data Array          gmsspx_dataStoreRegDataArr

Utility Functions

Set System Register                   gmsspx_utilSetReg

Get Register                          gmsspx_utilGetReg

Check State                           gmsspx_errorCheckState

Write To Instrument                   gmsspx_writeInstrData

Read Instrument Data                  gmsspx_readInstrData

Reset                                 gmsspx_reset

Self-Test                             gmsspx_self_test

Diagnostics                           gmsspx_diag

Error-Query                           gmsspx_error_query

Error Message                         gmsspx_error_message

Revision Query                        gmsspx_revision_query

Set Display State                     gmsspx_utilSetDisplayState

Get Display State                     gmsspx_utilGetDisplayState

Get Instrument Model                  gmsspx_utilGetInstrModel

Set I/O Delay Time                    gmsspx_utilSetIODelay

Get I/O Delay Time                    gmsspx_utilGetIODelay

Address                               gmsspx_utilAddress

Lock Instrument                       gmsspx_utilLockInstr

Close                                    gmsspx_close

________________________________________________________________________________

GM SSP X  Power Source (VISA)

This instrument module provides programming support for the GM SSP X

Power Sources. The module is divided into the following functions:

Functions/Classes:

(1) Initialize:

This function initializes the instrument and sets it to a default

configuration.

(2) Function Examples: (Class)

This class contains high-level, test and measurement routines.  These

examples call other instrument driver functions to configure, start, and

read from the instrument.

(3) Configuration Functions: (Class)

This class of functions configures the instrument by setting acquisition

and system configuration parameters.

(4) Action/Status Functions: (Class)

This class of functions begins or terminates an acquisition. It also

provides functions which allow the user to determine the current status

of the instrument.

(5) Data Functions: (Class)

This class of functions transfers data to or from the instrument.

(6) Utility Functions: (Class)

This class of functions provides lower level functions to communicate

with the instrument, and change instrument parameters.

(7) Close:

This function takes the instrument offline.

________________________________________________________________________________

The following functions are in alphabetical order.

________________________________________________________________________________

gmsspx_actSendSeqTrigger

ViStatus gmsspx_actSendSeqTrigger (ViSession instrumentHandle,

ViInt32 sequenceControl);

Purpose

This function operates the sequence trigger.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

sequenceControl

Variable Type       ViInt32

This control defines Sequence Trigger.

Valid Range:

For 62 X,64 X models:

0 - Go (Default Value)

For 32 X models:

0 - Go (Default Value)

1 - Hold

2 - Cont

3 - Start

4 - Step

5 - Stop

Note:

(1) Go - Start Sequence

(2) Hold - Pause Sequence (Firmware rev. 1.003 only)

(3) Cont - Continue Sequence (Firmware rev. 1.003 only)

(4) Start - Recall Start Register parameters

(5) Step - Recall next register parameters

(6) Stop - Recall Stop register parameters

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Sequence Control) out of range.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_AppExample

ViStatus gmsspx_AppExample (ViSession instrumentHandle,

ViReal64 voltageLimit_inV,

ViReal64 currentLimit_inA,

ViReal64 voltageSetting_inV,

ViReal64 currentSetting_inA,

ViPReal64 outputVoltage_inV,

ViPReal64 outputCurrent_inA);

Purpose

This function sets the voltage/current setpoints and limits and sets the

output state to On.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

voltageLimit_inV

Variable Type       ViReal64

This control defines the upper soft limit for the voltage setpoint,

thus enabling you to prevent the output voltage from being set in

excess of a certain value.

Default Value: 20.0

Valid Range:

-------------------------------------------------------------

Model           |   Valid Range (V)   |   Resolution (mV)   |

-------------------------------------------------------------

32 N 20 RU 10 P |       0...20 V      |          1 mV       |

32 N 40 RU  6 P |       0...40 V      |          1 mV       |

32 N 80 RU  3 P |       0...80 V      |          1 mV       |

32 N 20 RU 20 P |       0...20 V      |          1 mV       |

32 N 40 RU 12 P |       0...40 V      |          1 mV       |

32 N 80 RU  6 P |       0...80 V      |          1 mV       |

32 N 32 RU 18 P |       0...32 V      |          1 mV       |

62 N 40 RU 25 P |       0...40 V      |          1 mV       |

62 N 40 RU 50 P |       0...40 V      |          1 mV       |

64 N 40 RU100 P |       0...40 V      |          1 mV       |

64 N 40 RU150 P |       0...40 V      |          1 mV       |

62 N 80 RU12.5P |       0...80 V      |          1 mV       |

62 N 80 RU 25 P |       0...80 V      |          1 mV       |

64 N 80 RU 50 P |       0...80 V      |          1 mV       |

64 N 80 RU 75 P |       0...80 V      |          1 mV       |

62 N 52 RU 25 P |       0...52 V      |          1 mV       |

62 N 52 RU 50 P |       0...52 V      |          1 mV       |

64 N 52 RU 100P |       0...52 V      |          1 mV       |

64 N 52 RU 150P |       0...52 V      |          1 mV       |

-------------------------------------------------------------

Note:

(1) Value must be larger than voltage setpoint.

currentLimit_inA

Variable Type       ViReal64

This control defines the upper soft limit for the current setpoint,

thus enabling you to prevent the output current from being set in

excess of a certain value.

Default Value: 3.0

Valid Range:

-------------------------------------------------------------

Model           |   Valid Range (A)   |   Resolution (mA)   |

-------------------------------------------------------------

32 N 20 RU 10 P |       0...10 A      |          1 mA       |

32 N 40 RU  6 P |       0....6 A      |          1 mA       |

32 N 80 RU  3 P |       0....3 A      |          1 mA       |

32 N 20 RU 20 P |       0...20 A      |          1 mA       |

32 N 40 RU 12 P |       0...12 A      |          1 mA       |

32 N 80 RU  6 P |       0....6 A      |          1 mA       |

32 N 32 RU 18 P |       0...18 A      |          1 mA       |

62 N 40 RU 25 P |       0...25 A      |          1 mA       |

62 N 40 RU 50 P |       0...50 A      |          1 mA       |

64 N 40 RU100 P |       0..100 A      |          1 mA       |

64 N 40 RU150 P |       0..150 A      |          1 mA       |

62 N 80 RU12.5P |       0.12.5 A      |          1 mA       |

62 N 80 RU 25 P |       0...25 A      |          1 mA       |

64 N 80 RU 50 P |       0...50 A      |          1 mA       |

64 N 80 RU 75 P |       0...75 A      |          1 mA       |

62 N 52 RU 25 P |       0...25 A      |          1 mA       |

62 N 52 RU 50 P |       0...50 A      |          1 mA       |

64 N 52 RU 100P |       0..100 A      |          1 mA       |

64 N 52 RU 150P |       0..150 A      |          1 mA       |

-------------------------------------------------------------

Note:

(1) Value must be larger than current setpoint.

voltageSetting_inV

Variable Type       ViReal64

This control sets the setpoint value of the output voltage.

Default Value: 10.0

Valid Range:

-------------------------------------------------------------

Model           |   Valid Range (V)   |   Resolution (mV)   |

-------------------------------------------------------------

32 N 20 RU 10 P |       0...20 V      |          5 mV       |

32 N 40 RU  6 P |       0...40 V      |         10 mV       |

32 N 80 RU  3 P |       0...80 V      |         20 mV       |

32 N 20 RU 20 P |       0...20 V      |          5 mV       |

32 N 40 RU 12 P |       0...40 V      |         10 mV       |

32 N 80 RU  6 P |       0...80 V      |         20 mV       |

32 N 32 RU 18 P |       0...32 V      |         10 mV       |

62 N 40 RU 25 P |       0...40 V      |         10 mV       |

62 N 40 RU 50 P |       0...40 V      |         10 mV       |

64 N 40 RU100 P |       0...40 V      |         10 mV       |

64 N 40 RU150 P |       0...40 V      |         10 mV       |

62 N 80 RU12.5P |       0...80 V      |         20 mV       |

62 N 80 RU 25 P |       0...80 V      |         20 mV       |

64 N 80 RU 50 P |       0...80 V      |         20 mV       |

64 N 80 RU 75 P |       0...80 V      |         20 mV       |

62 N 52 RU 25 P |       0...52 V      |         20 mV       |

62 N 52 RU 50 P |       0...52 V      |         20 mV       |

64 N 52 RU 100P |       0...52 V      |         20 mV       |

64 N 52 RU 150P |       0...52 V      |         20 mV       |

-------------------------------------------------------------

Note:

(1) Value must be smaller than voltage limit.

currentSetting_inA

Variable Type       ViReal64

This control sets the setpoint value of the output current.

Default Value: 1.0

Valid Range:

-------------------------------------------------------------

Model           |   Valid Range (A)   |   Resolution (mA)   |

-------------------------------------------------------------

32 N 20 RU 10 P |       0...10 A      |        2.5 mA       |

32 N 40 RU  6 P |       0....6 A      |          2 mA       |

32 N 80 RU  3 P |       0....3 A      |          1 mA       |

32 N 20 RU 20 P |       0...20 A      |          5 mA       |

32 N 40 RU 12 P |       0...12 A      |       3.33 mA       |

32 N 80 RU  6 P |       0....6 A      |          2 mA       |

32 N 32 RU 18 P |       0...18 A      |          5 mA       |

62 N 40 RU 25 P |       0...25 A      |       6.25 mA       |

62 N 40 RU 50 P |       0...50 A      |       12.5 mA       |

64 N 40 RU100 P |       0..100 A      |         25 mA       |

64 N 40 RU150 P |       0..150 A      |         40 mA       |

62 N 80 RU12.5P |       0.12.5 A      |      3.125 mA       |

62 N 80 RU 25 P |       0...25 A      |       6.25 mA       |

64 N 80 RU 50 P |       0...50 A      |       12.5 mA       |

64 N 80 RU 75 P |       0...75 A      |         20 mA       |

62 N 52 RU 25 P |       0...25 A      |       6.25 mA       |

62 N 52 RU 50 P |       0...50 A      |      12.5  mA       |

64 N 52 RU 100P |       0..100 A      |         25 mA       |

64 N 52 RU 150P |       0..150 A      |         40 mA       |

-------------------------------------------------------------

Note:

(1) Value must be smaller than current limit.

outputVoltage_inV

Variable Type       ViReal64 (passed by reference)

This control returns output setpoint value of the output voltage.

outputCurrent_inA

Variable Type       ViReal64 (passed by reference)

This control returns output setpoint value of the output current.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Voltage Limit) out of range.

BFFC0003  Parameter 3 (Current Limit) out of range.

BFFC0004  Parameter 4 (Voltage Setting) out of range.

BFFC0005  Parameter 5 (Current Setting) out of range.

BFFC0006  Parameter 6 (Output Voltage) Null pointer.

BFFC0007  Parameter 7 (Output Current) Null pointer.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_close

ViStatus gmsspx_close (ViSession instrumentHandle);

Purpose

This function performs the following operations:

viClose (instrSession) and viClose (rmSession).

Notes:

(1) The instrument must be reinitialized to use it again.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the gmsspx_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confGetCurrent

ViStatus gmsspx_confGetCurrent (ViSession instrumentHandle,

ViPReal64 currentSettinginA);

Purpose

This function queries the setpoint value of the output current.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

currentSettinginA

Variable Type       ViReal64 (passed by reference)

This control returns the setpoint value of the output current.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Current Setting) Null pointer.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confGetCurrLimit

ViStatus gmsspx_confGetCurrLimit (ViSession instrumentHandle,

ViPReal64 currentLimit_inA);

Purpose

This function queries the upper soft limit for the current setpoint.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

currentLimit_inA

Variable Type       ViReal64 (passed by reference)

This control returns the upper soft limit for the current setpoint.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Current Limit) Null pointer.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confGetCurrMax

ViStatus gmsspx_confGetCurrMax (ViSession instrumentHandle,

ViPReal64 maxCurrent_inA);

Purpose

This function returns the upper current limit of the instrument.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

maxCurrent_inA

Variable Type       ViReal64 (passed by reference)

This control returns the upper current limit.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Voltage) Null pointer.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confGetMinMaxState

ViStatus gmsspx_confGetMinMaxState (ViSession instrumentHandle,

ViPBoolean minMaxState);

Purpose

This function queries MINMAX mode.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

minMaxState

Variable Type       ViBoolean (passed by reference)

This control returns MINMAX mode. The MINMAX function allows the MIN

and MAX values of voltage and current to be saved in memory.

VI_FALSE (0) - Off

VI_TRUE  (1) - On

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (MinMax State) Null pointer.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confGetOCPDelay

ViStatus gmsspx_confGetOCPDelay (ViSession instrumentHandle,

ViPReal64 OCPDelay_in_sec);

Purpose

This function queries the OCP delay.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

OCPDelay_in_sec

Variable Type       ViReal64 (passed by reference)

This control returns the OCP delay.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (OCP Delay in sec) Null pointer.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confGetOCPState

ViStatus gmsspx_confGetOCPState (ViSession instrumentHandle,

ViPBoolean OCPState);

Purpose

This function queries OCP state. The OCP function determines the

behaviour of the power output when the load current reaches the setpoint

value, i.e. when the output changes to the current regulation mode CC.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

OCPState

Variable Type       ViBoolean (passed by reference)

This control returns OCP state.

VI_FALSE (0) - Off

VI_TRUE  (1) - On

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (OCP State) Null pointer.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confGetOutputMode

ViStatus gmsspx_confGetOutputMode (ViSession instrumentHandle,

ViPInt32 outputMode);

Purpose

This function queries the current operating mode of the output.

Note:

(1) Under certain circumstances the instrument needs several seconds to

fully switch among the output mode. If you find that the mode obtained

using this function does not correspond to the instrument state, allow

several seconds for the mode setling before calling this function.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

outputMode

Variable Type       ViInt32 (passed by reference)

This control returns the actual operating mode of the output.

0 - OFF

1 - CV

2 - CC

3 - OL

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Output Mode) Null pointer.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confGetOutputState

ViStatus gmsspx_confGetOutputState (ViSession instrumentHandle,

ViPBoolean outputState);

Purpose

This function queries power output state.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

outputState

Variable Type       ViBoolean (passed by reference)

This control returns power output state.

VI_FALSE (0) - Off

VI_TRUE  (1) - On

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Output State) Null pointer.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confGetOVPLevel

ViStatus gmsspx_confGetOVPLevel (ViSession instrumentHandle,

ViPReal64 OVPLevel_inV);

Purpose

This function queries Over Voltage Protection level. OVP Level defines

the threshold value for the over voltage protection. As soon as the

voltage at the output is automatically deactivated.

OVP response time is less than 200 micro seconds.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

OVPLevel_inV

Variable Type       ViReal64 (passed by reference)

This control returns Over Voltage Protection level.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (OVP Level) Null pointer.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confGetPwrOnMode

ViStatus gmsspx_confGetPwrOnMode (ViSession instrumentHandle,

ViPInt32 powerOnMode);

Purpose

This function queries the status of the device setting after power on.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

powerOnMode

Variable Type       ViInt32 (passed by reference)

This control returns the status of the device setting after power on.

0 - Reset

1 - Recall

2 - Stand-by

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Power-On Mode) Null pointer.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confGetSeqDefaultTime

ViStatus gmsspx_confGetSeqDefaultTime (ViSession instrumentHandle,

ViPReal64 defaultTime_in_sec);

Purpose

This function queries Default Sequence Time.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

defaultTime_in_sec

Variable Type       ViReal64 (passed by reference)

This control returns Default Sequence Time.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Default Time) Null pointer.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confGetSeqRange

ViStatus gmsspx_confGetSeqRange (ViSession instrumentHandle,

ViPInt32 startAddress,

ViPInt32 stopAddress);

Purpose

This function queries Sequence Range.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

startAddress

Variable Type       ViInt32 (passed by reference)

This control returns Start Index.

stopAddress

Variable Type       ViInt32 (passed by reference)

This control returns Stop Index.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Start Address) Null pointer.

BFFC0003  Parameter 3 (Stop Address) Null pointer.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confGetSeqRepetition

ViStatus gmsspx_confGetSeqRepetition (ViSession instrumentHandle,

ViPInt32 sequenceRepetition);

Purpose

This function queries Sequence Repetition.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

sequenceRepetition

Variable Type       ViInt32 (passed by reference)

This control returns Sequence Repetition.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Sequence Repetition) Null pointer.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confGetSeqState

ViStatus gmsspx_confGetSeqState (ViSession instrumentHandle,

ViPBoolean sequenceState);

Purpose

This function queries Sequence State.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

sequenceState

Variable Type       ViBoolean (passed by reference)

This control returns Sequence State.

VI_FALSE (0) - Off

VI_TRUE (1) - On

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Sequence State) Null pointer.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confGetSeqTime

ViStatus gmsspx_confGetSeqTime (ViSession instrumentHandle,

ViPReal64 stepTime_in_sec);

Purpose

This function queries Sequence Time.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

stepTime_in_sec

Variable Type       ViReal64 (passed by reference)

This control returns Sequence Time.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Step Time) out of range.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confGetSignalFunc

ViStatus gmsspx_confGetSignalFunc (ViSession instrumentHandle,

ViPInt32 signal1, ViPInt32 signal2);

Purpose

This function queries specific function for Signal1 and Signal2 outputs

on Analog Interface.

Note:

(1) This function is supported only by 32 X models.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

signal1

Variable Type       ViInt32 (passed by reference)

This control returns specific function for Signal1 output on Analog

Interface.

0 - Off

1 - On

2 - Out

3 - Mode

4 - Sequence

5 - Signal Set

6 - U Low

7 - U High

8 - I Low

9 - I High

Note:

(1) This function is supported only by 32 X models.

signal2

Variable Type       ViInt32 (passed by reference)

This control returns specific function for Signal2 output on Analog

Interface.

0 - Off

1 - On

2 - Out

3 - Mode

4 - Sequence

5 - Signal Set

6 - U Low

7 - U High

8 - I Low

9 - I High

Note:

(1) This function is supported only by 32 X models.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Signal1) Null pointer.

BFFC0003  Parameter 3 (Signal2) Null pointer.

BFFC0004  Parameter 4 (State) Null pointer.

BFFC09F0  Instrument status error.

BFFC09F2  Function not supported for this model

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confGetSset

ViStatus gmsspx_confGetSset (ViSession instrumentHandle,

ViPBoolean SSet);

Purpose

This function gets Signal Set (SSet) functionallity in the instrument.

Note:

(1) This function is supported only by 32 X models.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

SSet

Variable Type       ViBoolean (passed by reference)

This control returns the Signal1 and Signal2 Output state.

VI_FALSE (0) - Off

VI_TRUE (1) - On

Note:

(1) This function is supported only by 32 X models.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (SSet) Null pointer.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confGetTriggerMode

ViStatus gmsspx_confGetTriggerMode (ViSession instrumentHandle,

ViPInt32 triggerMode);

Purpose

This function returns Trigger Mode.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

triggerMode

Variable Type       ViInt32 (passed by reference)

This control returns Trigger Mode. The trigger mode determines the

effect of the floating optocoupler input Trigger on the Analog

Interface.

0 - Off

1 - Out

2 - Recall

3 - Sequence

4 - Local Locked

5 - MinMax Mode

Note:

(1) Off - Trigger input disabled

(2) Out - Trigger input controls Output On/Off

-------------------------------------------

| Trigger | Effect                        |

-------------------------------------------

| Low     | Output state depends on       |

|         | manual setting                |

|Low->High| Output remains Off or         |

|         | will be Off after 1..15ms     |

| High    | Output is in Off state   and  |

|         | cannot be activated manually  |

|High->Low| Output will be activated      |

|         | after 1..15ms                 |

-------------------------------------------

(3) Recall - Trigger Input controls Recall of settings from Sequence

memory in single step.

This trigger mode permits step-by-step execution of the Sequence

function time controlled by an external signal. Each pulse

(11ms<t<0.8s) applied to the trigger input causes the voltage/current

pairs of an individual sequence register to be recalled, the sequence

time parameter is of no consequences.

The first pulse applied after Trigger mode has been changed to Recall

mode will recall Start register.

Applying a pulse with t>1.0s will reset the counter for register

number and another pulse will recall the Start register.

(4) Sequence - Trigger input controls execution of the sequence

functions

-------------------------------------------

| Trigger | Effect                        |

-------------------------------------------

|Low->High| 1 to 15 ms later the Sequence |

|         | execution is started (Seq Go) |

|High->Low| 1 to 15 ms later the Sequence |

|         | is discontinued (Seq Stop)    |

-------------------------------------------

(5) Local Locked - Trigger input controls locking of front panel

controls

-------------------------------------------

| Trigger | Effect                        |

-------------------------------------------

| Low     | all front panel controls are  |

|         | enabled                       |

| High    | all front panel controls are  |

|         | disabled                      |

-------------------------------------------

(6) MinMax - Trigger input controls MinMax function

-------------------------------------------

| Trigger | Effect                        |

-------------------------------------------

| Low     | Minmax state depends on       |

|         | manual setting                |

|Low->High| MinMax reset is executed      |

| High    | MinMax On                     |

|High->Low| MinMax Off                    |

-------------------------------------------

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Trigger Mode) Null pointer.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confGetVolt

ViStatus gmsspx_confGetVolt (ViSession instrumentHandle,

ViPReal64 voltageSetting_inV);

Purpose

This function queries the setpoint value of the output voltage.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

voltageSetting_inV

Variable Type       ViReal64 (passed by reference)

This control returns the setpoint value of the output voltage.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Voltage Setting) Null pointer.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confGetVoltLimit

ViStatus gmsspx_confGetVoltLimit (ViSession instrumentHandle,

ViPReal64 voltageLimit_inV);

Purpose

This function queries the upper soft limit for the voltage setpoint.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

voltageLimit_inV

Variable Type       ViReal64 (passed by reference)

This control returns the upper soft limit for the voltage setpoint.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Voltage Limit) Null pointer.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confGetVoltMax

ViStatus gmsspx_confGetVoltMax (ViSession instrumentHandle,

ViPReal64 maxVoltage_inV);

Purpose

This function returns the upper voltage limit of the instrument.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

maxVoltage_inV

Variable Type       ViReal64 (passed by reference)

This control returns the upper voltage limit.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Voltage) Null pointer.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confSetCurrent

ViStatus gmsspx_confSetCurrent (ViSession instrumentHandle,

ViReal64 currentSetting_inA);

Purpose

This function defines the setpoint value of the output current.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

currentSetting_inA

Variable Type       ViReal64

This control sets the setpoint value of the output current.

Default Value: 0.0

Valid Range:

-------------------------------------------------------------

Model           |   Valid Range (A)   |   Resolution (mA)   |

-------------------------------------------------------------

32 N 20 RU 10 P |       0...10 A      |        2.5 mA       |

32 N 40 RU  6 P |       0....6 A      |          2 mA       |

32 N 80 RU  3 P |       0....3 A      |          1 mA       |

32 N 20 RU 20 P |       0...20 A      |          5 mA       |

32 N 40 RU 12 P |       0...12 A      |       3.33 mA       |

32 N 80 RU  6 P |       0....6 A      |          2 mA       |

32 N 32 RU 18 P |       0...18 A      |          5 mA       |

62 N 40 RU 25 P |       0...25 A      |       6.25 mA       |

62 N 40 RU 50 P |       0...50 A      |       12.5 mA       |

64 N 40 RU100 P |       0..100 A      |         25 mA       |

64 N 40 RU150 P |       0..150 A      |         40 mA       |

62 N 80 RU12.5P |       0.12.5 A      |      3.125 mA       |

62 N 80 RU 25 P |       0...25 A      |       6.25 mA       |

64 N 80 RU 50 P |       0...50 A      |       12.5 mA       |

64 N 80 RU 75 P |       0...75 A      |         20 mA       |

62 N 52 RU 25 P |       0...25 A      |       6.25 mA       |

62 N 52 RU 50 P |       0...50 A      |       12.5 mA       |

64 N 52 RU 100P |       0..100 A      |         25 mA       |

64 N 52 RU 150P |       0..150 A      |         40 mA       |

-------------------------------------------------------------

Note:

(1) Value must be smaller than the current limit.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Current Setting) out of range.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confSetCurrLimit

ViStatus gmsspx_confSetCurrLimit (ViSession instrumentHandle,

ViReal64 currentLimit_inA);

Purpose

This function defines the upper soft limit for the current setpoint.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

currentLimit_inA

Variable Type       ViReal64

This control defines the upper soft limit for the current setpoint,

thus enabling you to prevent the output current from being set in

excess of a certain value.

Default Value: 3.0

Valid Range:

-------------------------------------------------------------

Model           |   Valid Range (A)   |   Resolution (mA)   |

-------------------------------------------------------------

32 N 20 RU 10 P |       0...10 A      |          1 mA       |

32 N 40 RU  6 P |       0....6 A      |          1 mA       |

32 N 80 RU  3 P |       0....3 A      |          1 mA       |

32 N 20 RU 20 P |       0...20 A      |          1 mA       |

32 N 40 RU 12 P |       0...12 A      |          1 mA       |

32 N 80 RU  6 P |       0....6 A      |          1 mA       |

32 N 32 RU 18 P |       0...18 A      |          1 mA       |

62 N 40 RU 25 P |       0...25 A      |          1 mA       |

62 N 40 RU 50 P |       0...50 A      |          1 mA       |

64 N 40 RU100 P |       0..100 A      |          1 mA       |

64 N 40 RU150 P |       0..150 A      |          1 mA       |

62 N 80 RU12.5P |       0.12.5 A      |          1 mA       |

62 N 80 RU 25 P |       0...25 A      |          1 mA       |

64 N 80 RU 50 P |       0...50 A      |          1 mA       |

64 N 80 RU 75 P |       0...75 A      |          1 mA       |

62 N 52 RU 25 P |       0...25 A      |          1 mA       |

62 N 52 RU 50 P |       0...50 A      |          1 mA       |

64 N 52 RU 100P |       0..100 A      |          1 mA       |

64 N 52 RU 150P |       0..150 A      |          1 mA       |

-------------------------------------------------------------

Note:

(1) Value must be larger than current setpoint.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Current Limit) out of range.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confSetMinMaxState

ViStatus gmsspx_confSetMinMaxState (ViSession instrumentHandle,

ViBoolean minMaxState);

Purpose

This function selects MINMAX mode.

Notes:

(1) The MINMAX function allows the MIN and MAX values of voltage and

current to be saved in memory.

(2) Resetting the saved MINMAX values is possible at any time by Reset

MinMax function in Action/Status functions.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

minMaxState

Variable Type       ViBoolean

This control selects MINMAX mode.

Valid Range:

VI_FALSE (0) - Off (Default Value)

VI_TRUE  (1) - On

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (MinMax State) out of range.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confSetOCPDelay

ViStatus gmsspx_confSetOCPDelay (ViSession instrumentHandle,

ViReal64 OCPDelay_in_sec);

Purpose

This function defines the OCP delay. Delay defines for the enabled OCP

function (Set OCP State) the delay until the power output is OFF

following a transition to current regulation (actual current = current

setpoint) If the output current drops below the  setpoint value before

timeout, the OFF sequence is interrupted and restarted when a return is

made to current regulation.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

OCPDelay_in_sec

Variable Type       ViReal64

This control defines the OCP delay.

Default Value: 0.0

Valid Range: 0.0 s - 99.99 s

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (OCP Delay in sec.) out of range.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confSetOCPState

ViStatus gmsspx_confSetOCPState (ViSession instrumentHandle,

ViBoolean OCPState);

Purpose

This function selects OCP ON/OFF. The OCP function determines the

behaviour of the power output when the load current reaches the setpoint

value, i.e. when the output changes to the current regulation mode CC.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

OCPState

Variable Type       ViBoolean

This control selects OCP ON/OFF.

Valid Range:

VI_FALSE (0) - Off (Default Value)

VI_TRUE  (1) - On

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (OCP State) out of range.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confSetOutputState

ViStatus gmsspx_confSetOutputState (ViSession instrumentHandle,

ViBoolean outputState);

Purpose

This function selects power output ON/OFF.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

outputState

Variable Type       ViBoolean

This control selects power output ON/OFF.

Valid Range:

VI_FALSE (0) - Off (Default Value)

VI_TRUE  (1) - On

Note:

(1) Internal ON procedure: for the transition from inactive to active

state the setpoints for current and voltage are first set "0" for an

approx. 4 ms duration with the output active, after which the

currently set values are executed.

(2) Internal OFF procedure: for approx. 350 ms the setpoint values

for voltage and current are set to 0 V and 0 A respectively. During

this period the sink is activated and the output capacitor will be

discharged. The sink is then returned OFF so that the output becomes

a high-impedance for externally

connected DC voltages (e.g. when outputs are paralleled) for values

bellow the rated output voltage.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Output State) out of range.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confSetOVPLevel

ViStatus gmsspx_confSetOVPLevel (ViSession instrumentHandle,

ViReal64 OVPLevel_inV);

Purpose

This function defines Over Voltage Protection level. OVP Level defines

the threshold value for the over voltage protection. As soon as the

voltage at the output is automatically deactivated.

OVP response time is less than 200 micro seconds.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

OVPLevel_inV

Variable Type       ViReal64

This control defines Over Voltage Protection level.

Default Value: 25.0

Valid Range:

-------------------------------------------------------------

Model           |   Valid Range (V)   |   Resolution (mV)   |

-------------------------------------------------------------

32 N 20 RU 10 P |       3...25 V      |        100 mV       |

32 N 40 RU  6 P |       3...50 V      |        200 mV       |

32 N 80 RU  3 P |       3..100 V      |        400 mV       |

32 N 20 RU 20 P |       3...25 V      |        100 mV       |

32 N 40 RU 12 P |       3...50 V      |        200 mV       |

32 N 80 RU  6 P |       3..100 V      |        400 mV       |

32 N 32 RU 18 P |       3...40 V      |        200 mV       |

62 N 40 RU 25 P |       3...50 V      |        100 mV       |

62 N 40 RU 50 P |       3...50 V      |        100 mV       |

64 N 40 RU100 P |       3...50 V      |        100 mV       |

64 N 40 RU150 P |       3...50 V      |        100 mV       |

62 N 80 RU12.5P |       3..100 V      |        100 mV       |

62 N 80 RU 25 P |       3..100 V      |        100 mV       |

64 N 80 RU 50 P |       3..100 V      |        100 mV       |

64 N 80 RU 75 P |       3..100 V      |        100 mV       |

62 N 52 RU 25 P |       3...62.5 V    |        100 mV       |

62 N 52 RU 50 P |       3...62.5 V    |        100 mV       |

64 N 52 RU 100P |       3...62.5 V    |        100 mV       |

64 N 52 RU 150P |       3...62.5 V    |        100 mV       |

-------------------------------------------------------------

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (OVP Level) out of range.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confSetPwrOnMode

ViStatus gmsspx_confSetPwrOnMode (ViSession instrumentHandle,

ViInt32 powerOnMode);

Purpose

This function defines the status of the device setting after power on.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

powerOnMode

Variable Type       ViInt32

This control defines the status of the device setting after power on.

Valid Range:

0 - Reset (Default Value)

1 - Recall

2 - Stand-by

Note:

(1) Reset - Device returns to default setting.

(2) Recall - Device settings are the same as before power-off.

(3) Stand-by - Same as for Recall except that power output remains

OFF

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Power-On Mode) out of range.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confSetSeq

ViStatus gmsspx_confSetSeq (ViSession instrumentHandle,

ViBoolean sequenceState,

ViInt32 startAddress, ViInt32 stopAddress,

ViInt32 sequenceRepetition,

ViReal64 defaultTime_in_sec);

Purpose

This function sets the Sequence.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

sequenceState

Variable Type       ViBoolean

This control defines Sequence State. The Sequence function permits

automatic recall of the values of voltage and current setpoint and

sequence time saved in the memory locations 11 through 255 according

to the parameters Sequence repetition, start and stop.

Valid Range:

VI_FALSE (0) - Off (Default Value)

VI_TRUE (1) - On

Note:

(1) Off - Terminate and/or close Sequence function

(2) On - Open Sequence function

startAddress

Variable Type       ViInt32

This control defines Start Index. The setting parameters Start memory

location address and Stop memory location address define the

beginning and end of the Sequence registers to be executed, whereby

Stop address must be >= Start address.

Default Value: 11

Valid Range: 11 - 255

stopAddress

Variable Type       ViInt32

This control defines Stop Index. The setting parameters Start memory

location address and Stop memory location address define the

beginning and end of the Sequence registers to be executed, whereby

Stop address must be >= Start address.

Default Value: 11

Valid Range: 11 - 255

sequenceRepetition

Variable Type       ViInt32

This control defines Sequence Repetition. When the Sequence function

is open, the Repetition parameter defines the number of times the

sequence is to be repeated between the selected Start and Stop

addresses.

Default Value: 0

Valid Range: 0 - 255

Note:

(1) 0 - infinite repetition

(2) 1-255 number of cycles

defaultTime_in_sec

Variable Type       ViReal64

This control defines Default Sequence Time. Default time defines the

dwell default for a pair of voltage/current values to be output by

the Sequence function.

Default Value: 0.01

Valid Range: 0.01s - 99.99s

Resolution: 0.01s

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Sequence State) out of range.

BFFC0003  Parameter 3 (Start Address) out of range.

BFFC0004  Parameter 4 (Stop Address) out of range.

BFFC0005  Parameter 5 (Sequence Repetition) out of range.

BFFC0006  Parameter 6 (Default Time) out of range.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confSetSeqDefaultTime

ViStatus gmsspx_confSetSeqDefaultTime (ViSession instrumentHandle,

ViReal64 defaultTime_in_sec);

Purpose

This function defines Default Sequence Time.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

defaultTime_in_sec

Variable Type       ViReal64

This control defines Default Sequence Time. Default time defines the

dwell default for a pair of voltage/current values to be output by

the Sequence function.

Default Value: 0.01

Valid Range: 0.01s - 99.99s

Resolution: 0.01s

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Default Time) out of range.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confSetSeqRange

ViStatus gmsspx_confSetSeqRange (ViSession instrumentHandle,

ViInt32 startAddress,

ViInt32 stopAddress);

Purpose

This function defines Sequence Range.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

startAddress

Variable Type       ViInt32

This control defines Start Index. The setting parameters Start memory

location address and Stop memory location address define the

beginning and end of the Sequence registers to be executed, whereby

Stop address must be >= Start address.

Default Value: 11

Valid Range: 11 - 255

stopAddress

Variable Type       ViInt32

This control defines Stop Index. The setting parameters Start memory

location address and Stop memory location address define the

beginning and end of the Sequence registers to be executed, whereby

Stop address must be >= Start address.

Default Value: 11

Valid Range: 11 - 255

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Start Address) out of range.

BFFC0003  Parameter 3 (Stop Address) out of range.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confSetSeqRepetition

ViStatus gmsspx_confSetSeqRepetition (ViSession instrumentHandle,

ViInt32 sequenceRepetition);

Purpose

This function defines Sequence Repetition.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

sequenceRepetition

Variable Type       ViInt32

This control defines Sequence Repetition. When the Sequence function

is open, the Repetition parameter defines the number of times the

sequence is to be repeated between the selected Start and Stop

addresses.

Default Value: 0

Valid Range: 0 - 255

Note:

(1) 0 - infinite repetition

(2) 1-255 number of cycles

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Sequence Repetition) out of range.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confSetSeqState

ViStatus gmsspx_confSetSeqState (ViSession instrumentHandle,

ViBoolean sequenceState);

Purpose

This function sets the Sequence State.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

sequenceState

Variable Type       ViBoolean

This control defines Sequence State. The Sequence function permits

automatic recall of the values of voltage and current setpoint and

sequence time saved in the memory locations 11 through 255 according

to the parameters Sequence repetition, start and stop.

Valid Range:

VI_FALSE (0) - Off (Default Value)

VI_TRUE (1) - On

Note:

(1) Off - Terminate and/or close Sequence function

(2) On - Open Sequence function

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Sequence State) out of range.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confSetSeqTime

ViStatus gmsspx_confSetSeqTime (ViSession instrumentHandle,

ViReal64 stepTime_in_sec);

Purpose

This function defines Sequence Time. Sequence time represents the dwell

time specific to each Sequence register for a pair of voltage/current

values to be output by Sequence function.

The sequence time gets saved into the register along with the voltage and

current values with the function Save Register.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

stepTime_in_sec

Variable Type       ViReal64

This control defines Sequence Time.

Default Value: 0.0

Valid Range: 0.00s - 99.99s

Resolution: 0.01s

Note:

(1) If the value is 0.0s, then Sequence time is the same as Default

time.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Step Time) out of range.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confSetSignalFunc

ViStatus gmsspx_confSetSignalFunc (ViSession instrumentHandle,

ViInt32 signal1, ViInt32 signal2);

Purpose

This function defines specific function for Signal1 and Signal2 outputs

on Analog Interface.

Note:

(1) This function is supported only by 32 X models.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

signal1

Variable Type       ViInt32

This control defines specific function for Signal1 output on Analog

Interface.

Valid Range:

0 - Off (Default Value)

1 - On

2 - Out

3 - Mode

4 - Sequence

5 - Signal Set

6 - U Low

7 - U High

8 - I Low

9 - I High

Note:

(1) This function is supported only by 32 X models.

(2) Off - Signal1 passive high

(3) On - active low

(4) Out - Output On - passive high

Output Off - active low

(5) Mode - Off to CV - passive high

CC to OL - active low

(6) Sequence - Go - active low

(7) SignalSet - Off - passive high

On - active low

(8) U Low - Umeas > Umin - passive high

Umeas < Umin - active low

(9) U High - Umeas < Umax - passive high

Umeas > Umax - active low

(10) I Low - Imeas > Imin - passive high

Imeas < Imin - active low

(11) I High - Imeas < Imax - passive high

Imeas > Imax - active low

signal2

Variable Type       ViInt32

This control defines specific function for Signal2 output on Analog

Interface.

Valid Range:

0 - Off (Default Value)

1 - On

2 - Out

3 - Mode

4 - Sequence

5 - Signal Set

6 - U Low

7 - U High

8 - I Low

9 - I High

Note:

(1) This function is supported only by 32 X models.

(2) Off - Signal2 passive high

(3) On - active low

(4) Out - Output On - passive high

Output Off - active low

(5) Mode - Off to CV - passive high

CC to OL - active low

(6) Sequence - Go - active low

(7) SignalSet - Off - passive high

On - active low

(8) U Low - Umeas > Umin - passive high

Umeas < Umin - active low

(9) U High - Umeas < Umax - passive high

Umeas > Umax - active low

(10) I Low - Imeas > Imin - passive high

Imeas < Imin - active low

(11) I High - Imeas < Imax - passive high

Imeas > Imax - active low

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Signal1) out of range.

BFFC0003  Parameter 3 (Signal2) out of range.

BFFC0004  Parameter 4 (State) out of range.

BFFC09F0  Instrument status error.

BFFC09F2  Function not supported for this model

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confSetSset

ViStatus gmsspx_confSetSset (ViSession instrumentHandle, ViBoolean SSet);

Purpose

This function sets Signal Set (SSet) functionallity in the instrument.

Note:

(1) This function is supported only by 32 X models.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

SSet

Variable Type       ViBoolean

This control defines the Signal1 and Signal2 Output state.

Valid Range:

VI_FALSE (0) - Off (Default Value)

VI_TRUE (1) - On

Note:

(1) This function is supported only by 32 X models.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (SSet) out of range.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confSetTriggerMode

ViStatus gmsspx_confSetTriggerMode (ViSession instrumentHandle,

ViInt32 triggerMode);

Purpose

This function defines Trigger Mode. The trigger mode determines the

effect of the floating optocoupler input Trigger on the Analog Interface.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

triggerMode

Variable Type       ViInt32

This control defines Trigger Mode.

Valid Range:

0 - Off (Default Value)

1 - Out

2 - Recall

3 - Sequence

4 - Local Locked

5 - MinMax Mode

Note:

(1) Off - Trigger input disabled

(2) Out - Trigger input controls Output On/Off

-------------------------------------------

| Trigger | Effect                        |

-------------------------------------------

| Low     | Output state depends on       |

|         | manual setting                |

|Low->High| Output remains Off or         |

|         | will be Off after 1..15ms     |

| High    | Output is in Off state   and  |

|         | cannot be activated manually  |

|High->Low| Output will be activated      |

|         | after 1..15ms                 |

-------------------------------------------

(3) Recall - Trigger Input controls Recall of settings from Sequence

memory in single step.

This trigger mode permits step-by-step execution of the Sequence

function time controlled by an external signal. Each pulse

(11ms<t<0.8s) applied to the trigger input causes the voltage/current

pairs of an individual sequence register to be recalled, the sequence

time parameter is of no consequences.

The first pulse applied after Trigger mode has been changed to Recall

mode will recall Start register.

Applying a pulse with t>1.0s will reset the counter for register

number and another pulse will recall the Start register.

(4) Sequence - Trigger input controls execution of the sequence

functions

-------------------------------------------

| Trigger | Effect                        |

-------------------------------------------

|Low->High| 1 to 15 ms later the Sequence |

|         | execution is started (Seq Go) |

|High->Low| 1 to 15 ms later the Sequence |

|         | is discontinued (Seq Stop)    |

-------------------------------------------

(5) Local Locked - Trigger input controls locking of front panel

controls

-------------------------------------------

| Trigger | Effect                        |

-------------------------------------------

| Low     | all front panel controls are  |

|         | enabled                       |

| High    | all front panel controls are  |

|         | disabled                      |

-------------------------------------------

(6) MinMax - Trigger input controls MinMax function

-------------------------------------------

| Trigger | Effect                        |

-------------------------------------------

| Low     | Minmax state depends on       |

|         | manual setting                |

|Low->High| MinMax reset is executed      |

| High    | MinMax On                     |

|High->Low| MinMax Off                    |

-------------------------------------------

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Trigger Mode) out of range.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confSetVolt

ViStatus gmsspx_confSetVolt (ViSession instrumentHandle,

ViReal64 voltageSetting_inV);

Purpose

This function sets the setpoint value of the output voltage.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

voltageSetting_inV

Variable Type       ViReal64

This control sets the setpoint value of the output voltage.

Default Value: 0.0

Valid Range:

-------------------------------------------------------------

Model           |   Valid Range (V)   |   Resolution (mV)   |

-------------------------------------------------------------

32 N 20 RU 10 P |       0...20 V      |          5 mV       |

32 N 40 RU  6 P |       0...40 V      |         10 mV       |

32 N 80 RU  3 P |       0...80 V      |         20 mV       |

32 N 20 RU 20 P |       0...20 V      |          5 mV       |

32 N 40 RU 12 P |       0...40 V      |         10 mV       |

32 N 80 RU  6 P |       0...80 V      |         20 mV       |

32 N 32 RU 18 P |       0...32 V      |         10 mV       |

62 N 40 RU 25 P |       0...40 V      |         10 mV       |

62 N 40 RU 50 P |       0...40 V      |         10 mV       |

64 N 40 RU100 P |       0...40 V      |         10 mV       |

64 N 40 RU150 P |       0...40 V      |         10 mV       |

62 N 80 RU12.5P |       0...80 V      |         20 mV       |

62 N 80 RU 25 P |       0...80 V      |         20 mV       |

64 N 80 RU 50 P |       0...80 V      |         20 mV       |

64 N 80 RU 75 P |       0...80 V      |         20 mV       |

62 N 52 RU 25 P |       0...52 V      |         20 mV       |

62 N 52 RU 50 P |       0...52 V      |         20 mV       |

64 N 52 RU 100P |       0...52 V      |         20 mV       |

64 N 52 RU 150P |       0...52 V      |         20 mV       |

-------------------------------------------------------------

Note:

(1) Value must be smaller than the voltage limit.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Voltage Setting) out of range.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_confSetVoltLimit

ViStatus gmsspx_confSetVoltLimit (ViSession instrumentHandle,

ViReal64 voltageLimit_inV);

Purpose

This function defines the upper soft limit for the voltage setpoint.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

voltageLimit_inV

Variable Type       ViReal64

This control defines the upper soft limit for the voltage setpoint,

thus enabling you to prevent the output voltage from being set in

excess of a certain value.

Default Value: 20.0

Valid Range:

-------------------------------------------------------------

Model           |   Valid Range (V)   |   Resolution (mV)   |

-------------------------------------------------------------

32 N 20 RU 10 P |       0...20 V      |          1 mV       |

32 N 40 RU  6 P |       0...40 V      |          1 mV       |

32 N 80 RU  3 P |       0...80 V      |          1 mV       |

32 N 20 RU 20 P |       0...20 V      |          1 mV       |

32 N 40 RU 12 P |       0...40 V      |          1 mV       |

32 N 80 RU  6 P |       0...80 V      |          1 mV       |

32 N 32 RU 18 P |       0...32 V      |          1 mV       |

62 N 40 RU 25 P |       0...40 V      |          1 mV       |

62 N 40 RU 50 P |       0...40 V      |          1 mV       |

64 N 40 RU100 P |       0...40 V      |          1 mV       |

64 N 40 RU150 P |       0...40 V      |          1 mV       |

62 N 80 RU12.5P |       0...80 V      |          1 mV       |

62 N 80 RU 25 P |       0...80 V      |          1 mV       |

64 N 80 RU 50 P |       0...80 V      |          1 mV       |

64 N 80 RU 75 P |       0...80 V      |          1 mV       |

62 N 52 RU 25 P |       0...52 V      |          1 mV       |

62 N 52 RU 50 P |       0...52 V      |          1 mV       |

64 N 52 RU 100P |       0...52 V      |          1 mV       |

64 N 52 RU 150P |       0...52 V      |          1 mV       |

------------------------------------------------------------

Note:

(1) Value must be larger than voltage setpoint.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Voltage Limit) out of range.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_dataReadReg

ViStatus gmsspx_dataReadReg (ViSession instrumentHandle,

ViInt32 registerNo,

ViPReal64 voltageSetting_inV,

ViPReal64 currentSettinginA,

ViPReal64 stepTime_in_sec, ViPInt32 SSet);

Purpose

This function returns parameters to register.

Note:

(1) This function is supported only by 32 X models.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

registerNo

Variable Type       ViInt32

This control defines the register to read settings.

Default Value: 11

Valid Range: 11 - 255

Note:

(1) This function is supported only by 32 X models.

voltageSetting_inV

Variable Type       ViReal64 (passed by reference)

This control returns the setpoint value of the output voltage.

currentSettinginA

Variable Type       ViReal64 (passed by reference)

This control returns the setpoint value of the output current.

stepTime_in_sec

Variable Type       ViReal64 (passed by reference)

This control returns Sequence Time. Sequence time defines the dwell

time specific to each Sequence register for a pair of voltage/current

values to be output by Sequence function.

SSet

Variable Type       ViInt32 (passed by reference)

This control returns the state of selected register.

Valid Range:

0 : Off - Register is disabled.

1 : On  - Register is enabled.

2 : Clear - Register is cleared.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Register No) out of range.

BFFC0003  Parameter 3 (Voltage Setting) out of range.

BFFC0004  Parameter 4 (Current Setting) out of range.

BFFC0005  Parameter 5 (Step Time) out of range.

BFFC0006  Parameter 6 (SSet) out of range.

BFFC09F0  Instrument status error.

BFFC09F2  Function not supported for this model

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_dataRecallRegister

ViStatus gmsspx_dataRecallRegister (ViSession instrumentHandle,

ViInt32 recallRegisterNo);

Purpose

This function recalls the specified register from the instrument's

memory.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

recallRegisterNo

Variable Type       ViInt32

This control defines the register to recall.

Default Value: 1

Valid Range: 1 - 255

Note:

(1) 1 - 10 Recalled parameters USET, ISET, OVSET, ULIM, ILIM,

OUTPUT Mode, OCP State, OCP Delay, MinMax Mode State,

Sequence Mode, Seq. Time, Seq. Default Time, Seq.

Repetition, Seq. Start, Seq. Stop

(2) 11 - 255 Recalled parameters USET, ISET, Seq. Time

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Recall Register No) out of range.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_dataSaveRegister

ViStatus gmsspx_dataSaveRegister (ViSession instrumentHandle,

ViInt32 saveRegisterNo);

Purpose

This function stores settings to the register in instrument's memory.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

saveRegisterNo

Variable Type       ViInt32

This control defines the register to store settings to it.

Default Value: 1

Valid Range: 1 - 255

Note:

(1) 1 - 10 Saved parameters USET, ISET, OVSET, ULIM, ILIM,

OUTPUT Mode, OCP State, OCP Delay, MinMax Mode State,

Sequence Mode, Seq. Time, Seq. Default Time, Seq.

Repetition, Seq. Start, Seq. Stop

(2) 11 - 255 Saved parameters USET, ISET, Seq. Time

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Save Register No) out of range.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_dataStoreReg

ViStatus gmsspx_dataStoreReg (ViSession instrumentHandle,

ViInt32 registerNo,

ViReal64 voltageSetting_inV,

ViReal64 currentSetting_inA,

ViReal64 stepTime_in_sec, ViInt32 SSet);

Purpose

This function stores parameters to register.

Note:

(1) This function is supported only by 32 X models.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

registerNo

Variable Type       ViInt32

This control defines the register to store settings to it.

Default Value: 11

Valid Range: 11 - 255

Note:

(1) This function is supported only by 32 X models.

voltageSetting_inV

Variable Type       ViReal64

This control sets the setpoint value of the output voltage.

Default Value: 0.0

Valid Range:

-------------------------------------------------------------

Model           |   Valid Range (V)   |   Resolution (mV)   |

-------------------------------------------------------------

32 N 20 RU 10 P |       0...20 V      |          5 mV       |

32 N 40 RU  6 P |       0...40 V      |         10 mV       |

32 N 80 RU  3 P |       0...80 V      |         20 mV       |

32 N 20 RU 20 P |       0...20 V      |          5 mV       |

32 N 40 RU 12 P |       0...40 V      |         10 mV       |

32 N 80 RU  6 P |       0...80 V      |         20 mV       |

32 N 32 RU 18 P |       0...32 V      |         10 mV       |

62 N 40 RU 25 P |       0...40 V      |         10 mV       |

62 N 40 RU 50 P |       0...40 V      |         10 mV       |

64 N 40 RU100 P |       0...40 V      |         10 mV       |

64 N 40 RU150 P |       0...40 V      |         10 mV       |

62 N 80 RU12.5P |       0...80 V      |         20 mV       |

62 N 80 RU 25 P |       0...80 V      |         20 mV       |

64 N 80 RU 50 P |       0...80 V      |         20 mV       |

64 N 80 RU 75 P |       0...80 V      |         20 mV       |

62 N 52 RU 25 P |       0...52 V      |         20 mV       |

62 N 52 RU 50 P |       0...52 V      |         20 mV       |

64 N 52 RU 100P |       0...52 V      |         20 mV       |

64 N 52 RU 150P |       0...52 V      |         20 mV       |

-------------------------------------------------------------

Note:

(1) Value must be smaller than voltage limit.

currentSetting_inA

Variable Type       ViReal64

This control sets the setpoint value of the output current.

Default Value: 0.0

Valid Range:

-------------------------------------------------------------

Model           |   Valid Range (A)   |   Resolution (mA)   |

-------------------------------------------------------------

32 N 20 RU 10 P |       0...10 A      |        2.5 mA       |

32 N 40 RU  6 P |       0....6 A      |          2 mA       |

32 N 80 RU  3 P |       0....3 A      |          1 mA       |

32 N 20 RU 20 P |       0...20 A      |          5 mA       |

32 N 40 RU 12 P |       0...12 A      |       3.33 mA       |

32 N 80 RU  6 P |       0....6 A      |          2 mA       |

32 N 32 RU 18 P |       0...18 A      |          5 mA       |

62 N 40 RU 25 P |       0...25 A      |       6.25 mA       |

62 N 40 RU 50 P |       0...50 A      |       12.5 mA       |

64 N 40 RU100 P |       0..100 A      |         25 mA       |

64 N 40 RU150 P |       0..150 A      |         40 mA       |

62 N 80 RU12.5P |       0.12.5 A      |      3.125 mA       |

62 N 80 RU 25 P |       0...25 A      |       6.25 mA       |

64 N 80 RU 50 P |       0...50 A      |       12.5 mA       |

64 N 80 RU 75 P |       0...75 A      |         20 mA       |

62 N 52 RU 25 P |       0...25 A      |       6.25 mA       |

62 N 52 RU 50 P |       0...50 A      |       12.5 mA       |

64 N 52 RU 100P |       0..100 A      |         25 mA       |

64 N 52 RU 150P |       0..150 A      |         40 mA       |

-------------------------------------------------------------

Note:

(1) Value must be smaller than current limit.

stepTime_in_sec

Variable Type       ViReal64

This control defines Sequence Time. Sequence time defines the dwell

time specific to each Sequence register for a pair of voltage/current

values to be output by Sequence function.

Default Value: 0.0

Valid Range: 0.00s - 99.99s

Resolution: 0.01s

Note:

(1) If the value is 0.0s, then Sequence time is the same as Default

time.

SSet

Variable Type       ViInt32

This control defines the state of selected register.

Valid Range:

0 - Off

1 - On (Default Value)

2 - Clear

Note:

(1) Off - Register is disabled.

(2) On - Register is enabled.

(3) Clear - Register is cleared.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Register No) out of range.

BFFC0003  Parameter 3 (Voltage Setting) out of range.

BFFC0004  Parameter 4 (Current Setting) out of range.

BFFC0005  Parameter 5 (Step Time) out of range.

BFFC0006  Parameter 6 (SSet) out of range.

BFFC09F0  Instrument status error.

BFFC09F2  Function not supported for this model

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_dataStoreRegDataArr

ViStatus gmsspx_dataStoreRegDataArr (ViSession instrumentHandle,

ViInt32 startRegisterNo,

ViReal64 _VI_FAR voltageSetting_inV[],

ViReal64 _VI_FAR currentSetting_inA[],

ViReal64 _VI_FAR stepTime_in_sec[],

ViInt32 _VI_FAR SSet[],

ViInt32 numberOfElements);

Purpose

This function stores parameters to more registers.

Note:

(1) This function is supported only by 32 X models.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

startRegisterNo

Variable Type       ViInt32

This control defines first register to start store settings to memory

from it.

Default Value: 11

Valid Range: 11 - 255

Note:

(1) This function is supported only by 32 X models.

voltageSetting_inV

Variable Type       ViReal64[]

This control sets array of setpoint values of the output voltage.

Default Value: not defined

Valid Range:

-------------------------------------------------------------

Model           |   Valid Range (V)   |   Resolution (mV)   |

-------------------------------------------------------------

32 N 20 RU 10 P |       0...20 V      |          5 mV       |

32 N 40 RU  6 P |       0...40 V      |         10 mV       |

32 N 80 RU  3 P |       0...80 V      |         20 mV       |

32 N 20 RU 20 P |       0...20 V      |          5 mV       |

32 N 40 RU 12 P |       0...40 V      |         10 mV       |

32 N 80 RU  6 P |       0...80 V      |         20 mV       |

32 N 32 RU 18 P |       0...32 V      |         10 mV       |

62 N 40 RU 25 P |       0...40 V      |         10 mV       |

62 N 40 RU 50 P |       0...40 V      |         10 mV       |

64 N 40 RU100 P |       0...40 V      |         10 mV       |

64 N 40 RU150 P |       0...40 V      |         10 mV       |

62 N 80 RU12.5P |       0...80 V      |         20 mV       |

62 N 80 RU 25 P |       0...80 V      |         20 mV       |

64 N 80 RU 50 P |       0...80 V      |         20 mV       |

64 N 80 RU 75 P |       0...80 V      |         20 mV       |

62 N 52 RU 25 P |       0...52 V      |         20 mV       |

62 N 52 RU 50 P |       0...52 V      |         20 mV       |

64 N 52 RU 100P |       0...52 V      |         20 mV       |

64 N 52 RU 150P |       0...52 V      |         20 mV       |

-------------------------------------------------------------

Note:

(1) Value must be smaller than voltage limit.

currentSetting_inA

Variable Type       ViReal64[]

This control sets the array of setpoint values of the output current.

Default Value: not defined

Valid Range of array elements:

-------------------------------------------------------------

Model           |   Valid Range (A)   |   Resolution (mA)   |

-------------------------------------------------------------

32 N 20 RU 10 P |       0...10 A      |        2.5 mA       |

32 N 40 RU  6 P |       0....6 A      |          2 mA       |

32 N 80 RU  3 P |       0....3 A      |          1 mA       |

32 N 20 RU 20 P |       0...20 A      |          5 mA       |

32 N 40 RU 12 P |       0...12 A      |       3.33 mA       |

32 N 80 RU  6 P |       0....6 A      |          2 mA       |

32 N 32 RU 18 P |       0...18 A      |          5 mA       |

62 N 40 RU 25 P |       0...25 A      |       6.25 mA       |

62 N 40 RU 50 P |       0...50 A      |       12.5 mA       |

64 N 40 RU100 P |       0..100 A      |         25 mA       |

64 N 40 RU150 P |       0..150 A      |         40 mA       |

62 N 80 RU12.5P |       0.12.5 A      |      3.125 mA       |

62 N 80 RU 25 P |       0...25 A      |       6.25 mA       |

64 N 80 RU 50 P |       0...50 A      |       12.5 mA       |

64 N 80 RU 75 P |       0...75 A      |         20 mA       |

62 N 52 RU 25 P |       0...25 A      |       6.25 mA       |

62 N 52 RU 50 P |       0...50 A      |       12.5 mA       |

64 N 52 RU 100P |       0..100 A      |         25 mA       |

64 N 52 RU 150P |       0..150 A      |         40 mA       |

-------------------------------------------------------------

Note:

(1) Value must be smaller than current limit.

stepTime_in_sec

Variable Type       ViReal64[]

This control sets the array of Sequence Times. Sequence time defines

the dwell time specific to each Sequence register for a pair of

voltage/current values to be output by Sequence function.

Default Value: not defined

Valid Range: 0.00s - 99.99s

Resolution: 0.01s

Note:

(1) If the value is 0.0s, then Sequence time is the same as Default

time.

SSet

Variable Type       ViInt32[]

This control defines array of states of selected registers.

Valid Range:

0 - Off

1 - On (Default Value)

2 - Clear

Note:

(1) Off - Register is disabled.

(2) On - Register is enabled.

(3) Clear - Register is cleared.

numberOfElements

Variable Type       ViInt32

This control defines the number of points to store in registers.

Default Value: not defined

Valid Range: 0 to 245

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Register No) out of range.

BFFC0003  Parameter 3 (Voltage Setting) Null pointer.

BFFC0004  Parameter 4 (Current Setting) Null pointer.

BFFC0005  Parameter 5 (Step Time) Null pointer.

BFFC0006  Parameter 6 (SSet) Null pointer.

BFFC0007  Parameter 7 (Number of points) out of range.

BFFC09F0  Instrument status error.

BFFC09F2  Function not supported for this model

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_diag

ViStatus gmsspx_diag (ViSession instrumentHandle,

ViChar _VI_FAR diagnosticsMessage[]);

Purpose

This function runs a set of the instrument's internal diagnostic routines

and returns the results.

Note:

(1) This function is supported only by 62 X and 64 X models.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

diagnosticsMessage

Variable Type       ViChar[]

This control contains the string returned from the diagnostics. See

the device's operation manual for an explanation of the string's

contents.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the gmsspx_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Self-Test Result) NULL pointer.

BFFC09F2  Function not supported for this model

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_errorCheckState

ViStatus gmsspx_errorCheckState (ViSession instrumentHandle,

ViBoolean state);

Purpose

This function switches ON/OFF state checking of the instrument (reading

of the Standard Event Register and checking it for error).

All driver functions are using state checking. Set this function to ON

when debugging your application. For better bus throughput and instrument

performance switch it OFF.

State checking is ON by default.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

state

Variable Type       ViBoolean

This control selects ON or OFF instrument state checking.

Valid Range:

VI_FALSE (0) - Off

VI_TRUE  (1) - On (Default Value)

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the gmsspx_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (State) out of range.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_error_message

ViStatus gmsspx_error_message (ViSession instrumentHandle,

ViStatus statusCode,

ViChar _VI_FAR message[]);

Purpose

This function takes the Status Code returned by the instrument driver

functions, interprets it and returns it as a user readable string.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  VI_NULL

statusCode

Variable Type       ViStatus

This control accepts the Status Code returned from the instrument

driver functions.

Default Value:

0 - VI_SUCCESS

message

Variable Type       ViChar[]

This control returns the interpreted Status Code as a user readable

message string.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the gmsspx_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

3FFF0085  Unknown status code - VI_WARN_UNKNOWN_STATUS

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_error_query

ViStatus gmsspx_error_query (ViSession instrumentHandle,

ViPInt32 errorCode,

ViChar _VI_FAR errorMessage[]);

Purpose

This function reads an error code from the instrument's error queue.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

errorCode

Variable Type       ViInt32 (passed by reference)

This control returns the error code read from the instrument's error

queue.

errorMessage

Variable Type       ViChar[]

This control returns the error message string read from the

instrument's error message queue.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the gmsspx_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

3FFC0104  Error query not supported

BFFC0002  Parameter 2 (Error Code) NULL pointer.

BFFC0003  Parameter 3 (Error Message) NULL pointer.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_fetchMinMax

ViStatus gmsspx_fetchMinMax (ViSession instrumentHandle,

ViPReal64 minVoltage_inV,

ViPReal64 maxVoltage_inV,

ViPReal64 minCurrent_inA,

ViPReal64 maxCurrent_inA);

Purpose

This function returns the limit values of the output current and voltage

measured and saved in MINMAX memory during MINMAX ON.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

minVoltage_inV

Variable Type       ViReal64 (passed by reference)

This control returns the minimum value of the output voltage measured

and saved in MINMAX memory during MINMAX ON.

Note:

(1) You may pass VI_NULL if you do not need this value.

maxVoltage_inV

Variable Type       ViReal64 (passed by reference)

This function returns the maximum value of the output voltage

measured and saved in MINMAX memory during MINMAX ON.

Note:

(1) You may pass VI_NULL if you do not need this value.

minCurrent_inA

Variable Type       ViReal64 (passed by reference)

This function returns the minimum value of the output current

measured and saved in MINMAX memory during MINMAX ON.

Note:

(1) You may pass VI_NULL if you do not need this value.

maxCurrent_inA

Variable Type       ViReal64 (passed by reference)

This function returns the maximum value of the output current

measured and saved in MINMAX memory during MINMAX ON.

Note:

(1) You may pass VI_NULL if you do not need this value.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_init

ViStatus gmsspx_init (ViRsrc resourceName, ViBoolean IDQuery,

ViBoolean resetDevice,

ViPSession instrumentHandle);

Purpose

This function performs the following initialization actions:

- Opens a session to the Default Resource Manager resource and a session

to the specified device using the interface and address specified in the

Resource_Name control.

- Performs an identification query on the Instrument.

- Resets the instrument to a known state.

- Sends initialization commands to the instrument that set any necessary

programmatic variables such as Headers Off, Short Command form, and Data

Transfer Binary to the state necessary for the operation of the

instrument driver.

- Returns an Instrument Handle which is used to differentiate between

different sessions of this instrument driver.

- Each time this function is invoked a Unique Session is opened.  It is

possible to have more than one session open for the same resource.

Parameter List

resourceName

Variable Type       ViRsrc

This control specifies the interface and address of the device that

is to be initialized (Instrument Descriptor). The exact grammar to be

used in this control is shown in the note below.

Default Value:  "GPIB::13"

Notes:

(1) Based on the Instrument Descriptor, this operation establishes a

communication session with a device.  The grammar for the Instrument

Descriptor is shown below.  Optional parameters are shown in square

brackets ([]).

Interface   Grammar

------------------------------------------------------

Serial      ASRL<port>::INSTR

GPIB        GPIB[board]::primary address[::secondary address]

[::INSTR]

The ASRL keyword is used with Serial instruments.

The GPIB keyword is used with GPIB instruments.

The default value for optional parameters are shown below.

Optional Parameter          Default Value

-----------------------------------------

board                       0

secondary address           none - 31

IDQuery

Variable Type       ViBoolean

This control specifies if an ID Query is sent to the instrument

during the initialization procedure.

Valid Range:

VI_FALSE (0) - Skip Query

VI_TRUE  (1) - Do Query (Default Value)

Notes:

(1) Under normal circumstances the ID Query ensures that the

instrument initialized is the type supported by this driver. However

circumstances may arise where it is undesirable to send an ID Query

to the instrument.  In those cases; set this control to "Skip Query"

and this function will initialize the selected interface, without

doing an ID Query.

resetDevice

Variable Type       ViBoolean

This control specifies if the instrument is to be reset to its

power-on settings during the initialization procedure.

Valid Range:

VI_FALSE (0) - Don't Reset

VI_TRUE  (1) - Reset Device (Default Value)

Notes:

(1) If you do not want the instrument reset. Set this control to

"Don't Reset" while initializing the instrument.

instrumentHandle

Variable Type       ViSession (passed by reference)

This control returns an Instrument Handle that is used in all

subsequent function calls to differentiate between different sessions

of this instrument driver.

Notes:

(1) Each time this function is invoked a Unique Session is opened.

It is possible to have more than one session open for the same

resource.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the gmsspx_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (ID Query) out of range.

BFFC0003  Parameter 3 (Reset Device) out of range.

BFFC0011  Instrument returned invalid response to ID Query

BFFC09F1  Unknown instrument model

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_measOutCurr

ViStatus gmsspx_measOutCurr (ViSession instrumentHandle,

ViPReal64 outputCurrent);

Purpose

This function queries the actual measured value of the output current.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

outputCurrent

Variable Type       ViReal64 (passed by reference)

This control returns the actual measured value of the output current.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Output Current) Null pointer.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_measOutPower

ViStatus gmsspx_measOutPower (ViSession instrumentHandle,

ViPReal64 outputPower_inW);

Purpose

This function queries the actually measured power on the output.

The time required to acquire and process the reading is approx. 90 ms.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

outputPower_inW

Variable Type       ViReal64 (passed by reference)

This control returns the actually measured power on the output.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Output Power) Null pointer.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_measOutVolt

ViStatus gmsspx_measOutVolt (ViSession instrumentHandle,

ViPReal64 outputVoltage);

Purpose

This function queries the actual measured value of the output voltage.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

outputVoltage

Variable Type       ViReal64 (passed by reference)

This control returns the actual measured value of the output voltage.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Output Voltage) Null pointer.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_readInstrData

ViStatus gmsspx_readInstrData (ViSession instrumentHandle,

ViInt32 numberBytesToRead,

ViChar _VI_FAR readBuffer[],

ViPInt32 numBytesRead);

Purpose

This function reads data from the instrument's output buffer and returns

it to the specified variable in memory.

Notes:

(1) Because the instrument may return both numeric and text data in

response to queries this function returns the data in string format.

(2) If valid data is not available at the instrument's output buffer when

this function is called the instrument will hang up and the function will

not return until it times out.  If the time-out is disabled this function

will hang indefinitely and it may be necessary to reboot the computer to

break out.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

numberBytesToRead

Variable Type       ViInt32

The number of bytes specified by this control should be greater than

or equal to the number of bytes which are to be read from the

instrument. If the actual number of bytes to be read is greater than

the number this control specifies then multiple reads will be

required to empty the instrument's output buffer.

If the instrument's output buffer is not emptied the instrument may

return invalid data when future reads are performed.

Default Value:  50 (See NOTE 2)

Notes:

(1) If NO DATA is available at the instrument's output buffer when

this function is called the instrument will hang up and the function

will not return until it times out.  If the time-out is disabled,

this function will hang indefinitely and it may be necessary to

reboot the computer.

(2) If the number of bytes expected is greater than 50 the value of

this control may be increased. If the actual number of bytes read is

larger than the declared size of the read buffer a run-time error

will be generated.

readBuffer

Variable Type       ViChar[]

The incoming data from the instrument is placed into this variable.

Notes:

(1) Because the instrument may return both numeric and text data in

response to queries this function returns the data in string format.

(2) This function does not overwrite any old data left in the string

variable from the last time the function was called. String data in

LabWindows/CVI is terminated with an ASCII null(0x0) and string

manipulation functions will only recognize data before the ASCII

null.

(3) The declared size of the string variable must be greater than the

actual number of bytes read from the instrument if it is not a

run-time error will be generated.

numBytesRead

Variable Type       ViInt32 (passed by reference)

This variable contains the actual number of bytes read from the

instrument. This is the value which is returned by the read function.

Notes:

(1) If the actual number of bytes read is less than the number of

bytes specified in the Number Bytes To Read control then the output

buffer has probably been emptied. If the read function fails and the

number of bytes read is 0, the most probable cause for the failure is

there was no data available at the instrument's output buffer.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the gmsspx_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

BFFC09F0  Instrument status error.

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_reset

ViStatus gmsspx_reset (ViSession instrumentHandle);

Purpose

This function resets the instrument to a known state and sends

initialization commands to the instrument that set any necessary

programmatic variables.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the gmsspx_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC09F0  Instrument status error.

BFFC09F1  Unknown instrument model

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_resetMinMax

ViStatus gmsspx_resetMinMax (ViSession instrumentHandle);

Purpose

This function resets values in MinMax memory.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_revision_query

ViStatus gmsspx_revision_query (ViSession instrumentHandle,

ViChar _VI_FAR instrumentDriverRevision[],

ViChar _VI_FAR firmwareRevision[]);

Purpose

This function returns the revision numbers of the instrument driver and

instrument firmware, and tells the user with which  instrument firmware

this revision of the driver is compatible.

This instrument driver's Revision Number is

"Rev 1.1, 07/2000, CVI 5.5" and is compatible with Instrument Firmware

Revision "1.002" for 32 X model and "2.012" for 62 X and 64 X model.

This instrument driver was developed using instruments returning the

following identification strings:

model 32 X:

GOSSEN-METRAWATT,SSP32N040RU012P,000000000,01.002

model 62 X and 64 X:

GOSSEN-METRAWATT,62N080RU025P,ET 0003  ,02.012

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

instrumentDriverRevision

Variable Type       ViChar[]

This control returns the Instrument Driver Software Revision.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

firmwareRevision

Variable Type       ViChar[]

This control returns the Instrument Firmware Revision.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the gmsspx_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

3FFC0105  Revision query not supported - VI_WARN_NSUP_REV_QUERY

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_self_test

ViStatus gmsspx_self_test (ViSession instrumentHandle,

ViPInt16 selfTestResult,

ViChar _VI_FAR selfTestMessage[]);

Purpose

This function runs the instrument's self test routine and returns the

test result(s).

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

selfTestResult

Variable Type       ViInt16 (passed by reference)

This control contains the value returned from the instrument self

test.  Zero means success.  For any other code, see the device's

operator's manual.

selfTestMessage

Variable Type       ViChar[]

This control contains the string returned from the self test. See the

device's operation manual for an explanation of the string's

contents.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the gmsspx_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

3FFC0103  Self Test not supported - VI_WARN_NSUP_SELF_TEST

BFFC0002  Parameter 2 (Self-Test Result) NULL pointer.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_utilAddress

ViStatus gmsspx_utilAddress (ViSession instrumentHandle,

ViInt32 addressOfInstrument);

Purpose

This function address the appropriate instrument when several instruments

are chained via serial cable.

Note:

(1) This function is supported only by 32 X models.

(2) This function is supported only via serial interface.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

addressOfInstrument

Variable Type       ViInt32

Address of the instrument in the serial cable chain.

Default Value: 11

Valid Range: 1 - 32

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Address Of Instrument) out of range.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_utilGetDisplayState

ViStatus gmsspx_utilGetDisplayState (ViSession instrumentHandle,

ViPBoolean displayState);

Purpose

This function queries display mode.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

displayState

Variable Type       ViBoolean (passed by reference)

This control returns display mode.

VI_FALSE (0) - Off

VI_TRUE  (1) - On

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Display State) Null pointer.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_utilGetInstrModel

ViStatus gmsspx_utilGetInstrModel (ViSession instrumentHandle,

ViChar _VI_FAR instrumentModel[],

ViPInt32 instrModelNumber);

Purpose

This function returns the instrument model as a string and a number.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

instrumentModel

Variable Type       ViChar[]

This control returns the Instrument Model string.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

instrModelNumber

Variable Type       ViInt32 (passed by reference)

This control returns the instrument model as a number. You can pass

VI_NULL if you don't need to know this number.

Return Values:

0 - 32N020RU010

1 - 32N040RU006

2 - 32N080RU003

3 - 32N020RU020

4 - 32N040RU012

5 - 32N080RU006

6 - 32N032RU018

7 - 62N040RU025

8 - 62N040RU050

9 - 64N040RU100

10 - 64N040RU150

11 - 62N080RU125

12 - 62N080RU025

13 - 64N080RU050

14 - 64N080RU075

15 - 62N052RU025

16 - 62N052RU050

17 - 64N052RU100

18 - 64N052RU150

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Voltage) Null pointer.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_utilGetIODelay

ViStatus gmsspx_utilGetIODelay (ViSession instrumentHandle,

ViPReal64 IODelayTime);

Purpose

This function returns the instrument model as a string and a number.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

IODelayTime

Variable Type       ViReal64 (passed by reference)

This control returns the delay between I/O operations.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (IO Delay Time) Null pointer.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_utilGetReg

ViStatus gmsspx_utilGetReg (ViSession instrumentHandle,

ViInt32 registerName, ViPInt32 value);

Purpose

This function gets specified value from register.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

registerName

Variable Type       ViInt32

Register name to get.

Valid Range:

0 - ESR (Default Value)

1 - ESE

2 - CRA

3 - ERA

4 - ERAE

5 - ERB

6 - ERBE

7 - STB

8 - SRE

9 - PRE

value

Variable Type       ViInt32 (passed by reference)

Value of the register.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Register No) out of range.

BFFC0003  Parameter 3 (Voltage) Null pointer.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_utilLockInstr

ViStatus gmsspx_utilLockInstr (ViSession instrumentHandle,

ViBoolean instrument);

Purpose

This function locks/unlocks the instrument.

Note:

(1) This function is supported only via GPIB interface.

(2) You have to have NI-VISA version 2.01 or greather properly installed

on your machine.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

instrument

Variable Type       ViBoolean

This control lock/unlock the instrument.

Valid Range:

VI_FALSE (0) - Unlock

VI_TRUE  (1) - Lock  (Default Value)

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Instrument) out of range.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_utilSetDisplayState

ViStatus gmsspx_utilSetDisplayState (ViSession instrumentHandle,

ViBoolean displayState);

Purpose

This function selects display mode. This function enables the digital

display to be signalled OFF if remote control is active.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

displayState

Variable Type       ViBoolean

This control lock/unlock the instrument.

Valid Range:

VI_FALSE (0) - Unlock

VI_TRUE  (1) - Lock  (Default Value)

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (Display State) out of range.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_utilSetIODelay

ViStatus gmsspx_utilSetIODelay (ViSession instrumentHandle,

ViReal64 IODelayTime);

Purpose

This function defines the delay between I/O operations.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

IODelayTime

Variable Type       ViReal64

This value specifies the delay between I/O operations.

Valid Range: => 0.0 seconds

Default Value: 0.002 seconds

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (IO Delay Time) out of range.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_utilSetReg

ViStatus gmsspx_utilSetReg (ViSession instrumentHandle,

ViInt32 systemRegisterName, ViInt32 value);

Purpose

This function sets specified value to register.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

systemRegisterName

Variable Type       ViInt32

Register name to set.

Valid Range:

0 - ESE (Default Value)

1 - ERAE

2 - ERBE

4 - SRE

5 - PRE

value

Variable Type       ViInt32

New value of the register.

Valid Range: 0 - 255

Default Value: 0

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the rsamiq_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

BFFC0002  Parameter 2 (System Register No) out of range.

BFFC0003  Parameter 3 (Voltage) out of range.

BFFC09F0  Instrument status error.

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

gmsspx_writeInstrData

ViStatus gmsspx_writeInstrData (ViSession instrumentHandle,

ViString writeBuffer);

Purpose

This function writes commands and queries to the instrument to modify

parameters and query device settings.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

writeBuffer

Variable Type       ViString

The user can use this control to send common commands and queries to

the instrument. This control can also be used to write any valid

command to the instrument.

Default Value:  "*RST"

Notes:

(1) The command or query to be sent to the instrument may be a

literal enclosed in double quotes i.e. "*RST" or may be contained in

a variable of type string.

Return Value

Returns the status code of this operation. The status code  either

indicates success or describes an error or warning condition. You

examine the status code from each call to an instrument driver

function to determine if an error occurred. To obtain a text

description of the status code, call the gmsspx_error_message

function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources. The following table defines the ranges of additional

status codes that this driver can return. The table lists the

different include files that contain the defined constants for the

particular status codes:

BFFC09F0  Instrument status error.

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors