U8903B firmware upgrade procedures

The U8903B firmware update file can be obtained from the Keysight U8903B firmware update Web site at www.keysight.com/find/audioanalyzer_firmware.

Perform the following procedure to update the U8903B firmware.

1 Save the firmware update files in an external USB flash storage.

2 Connect the external USB flash storage to the U8903B.

3 Press on the SYSTEM panel.

4 From the System menu page, press the Update softkey. The Update menu page is displayed as shown in figure 1-6.

Figure 1-6 System > Update menu page

5 Press the Load File softkey and the recall menu page is displayed to enable you to select the source location.

6 Select the firmware file (xxx.bin) to be updated, and press the Recall softkey as shown in Figure 1-7.

Table 1-7 System > Update menu page

Menu Description

The firmware update process may take 20 minutes to 40 minutes to complete. DO NOT power off the U8903B.