The complete board and the used parts are not very critcal. If you can not get the 3V3 regulator,you can use two diodes in series like a 1N4148. This lowers the 5 Volt to a useful value. Also possible are

Some other types of voltageregulators like the LP2950CZ33 or the LM2931AZ33...

Because the Board is only in use for seconds - until the loader is copied to the Ram - a little too high voltage does not matter. Also the chance that the normal "C" type of the 247C256/512 will work, is higher at higher voltages...

So if you can not get this 3V3 refulator, it is possible to build the board. Also the condensators are not critical - any other type and value will work.This condensators ar only used, to prevent the regulator from oscillating.... In other words: you can use any part you can get to build this board as long the parameters are "near" the right ones....

If you put two diodes in series, on each diode there is a voltage loss of about 0.7 Volts. So on these two diodes there is a loss of about 1.4 Volt. Because the board has an inputvoltage from the connector JP250 of about 5 Volts, the voltage AFTER these two diodes is about 3.6 Volts. Because the 5V is already regulated, there is no urgent need to do this also for the 3V3.

For a first test, you can use the board without programmed eproms on it. If you have soldered everything correctly, the your humax should start with the plugged in board normally. If this is so, then you can measure the voltage on the pins 28 of the ic-sockets. It should be about 3.6 Volts. After that

you can test the Switch (single pole momentary on). If the switch is on, the your Humax should be "dead" - not really - only the sreen is blank an there is no sign on the frontdisplay. This shows you, that the original flash is switched off an there is no code to boot.

Then you should program your Eproms (U2 left one begins with 08 40 09 00 00 00 09 25 .. .. ..,  and U3 the right one with 3C 00 40 60 00 00 01 48 .. .. ) If the RomBoard is complete, you can give it a try in the posted manner. It should show you a H208 Loader in the frontdisplay and a blue screen with the downloading percentage. After the first percentage of downloading you must release the button on the board and everything should work in the expected manner... By the way, if something went wrong with the download, you can give it a second try without the romboard! Some times the loader is

Written correctly, but a checksum over the application is not calculated new. If this happens, you will get an errorAx and you must download the complete +CAM file once more - without the plugged romboard.... But this is very seldom and not worth of thinking too much about it (:-)

You can also go this way directly. For the first time WITH the plugged in RomBoard, you can download the enclosed H208 loader alone (this takes only some seconds) and in the second step you can download in the same way the complete firmware. If you go this way, there is no chance to get an errormessage about a wrong CRC...

To find out if you have a culprit you have to check every pin, every solderpoint and so on with a magnifier and an ohmmeter. Only one Adress or Dataline without the right contact makes the

board useless!

There Are Mayby some problems with 27C256 with a very slow access time? In my board I am using parts with 100nS. As far as I know, the 27C256 in older maskrevisions can have up to 250nS. Some of my friends have build this board with flashmemorys 28F040, 27C512, 27C256 - but the

slowest device has 100nS (the fastes 45nS).