LP6DUTIL

USER’S  GUIDE

1.  GENERAL:

LP6DUTIL is a DOS-based program providing facilities to obtain information about Emulex Host Bus Adapters, and to test different areas of these adapters.

2.  SUPPORTED  ADAPTERS:

This version of LP6DUTIL supports the following adapters:

LP6000, LP7000, LP8000, LP850, LP9000, LP950, LP9802/LP9802DC, LP982, LP10000/LP10000DC/LP1050, LP11000/LP11002/LP1150, LP101, LPe11000/ LPe11002/LPe1150.

3.  PROGRAM  ACTIVATION:

Bootup your system with DOS if it has not been running under DOS. From the directory where LP6DUTIL.EXE file resides, enter  LP6DUTIL<Enter>. You cannot start LP6DUTIL at the Command Prompt of Windows. All adapters in your system are detected and reset. A brief Introduction message is displayed with a menu bar on top of the screen, and prompt lp> at the bottom:

# File       Adapter        Info        Test        Flash        Debug        Help

LP6DUTIL version 9.2A6

Copyright © 2004, Emulex Corporation

lp>

Notes:

# For debugging, you can activate the program without restarting the adapter by entering

LP6DUTIL /nr

or LP6DUTIL /nf (to bypass the Flash Loadlist search at the beginning of the program).

You can start your desired process now through either the menu bar or the command line.

- USE OF COMMAND LINE:

At the lp> prompt enter the desired command with correct syntax then hit Enter.

If you want to see all the available commands, enter help and Enter:

Available Commands:

@		download		extloopback		help

intloopback	pciloopback		reset			sramtest

If you want to see the syntax for a particular command, enter help followed by that command name then hit Enter;  help download will yield this display:

Syntax: download <n=adapter> <i=imagepath>

OR

Syntax: download <a=adaptertype> <i=imagepath>

Notes: The additional set of commands is described in the separate document, New Set of Commands.doc.

Please note that the program simple parsing scheme requires only one blank space between adjacent items in the command line, and no blank space between the = sign and the two letters before and after this sign. In case of multiple arguments, order in which an argument appears does not have to be fixed.

4.1 @

This @ sign is followed by the pathname of the script file containing a set of commands.

Ex:  @a:\script1.txt

4.2  download

This command has two formats:

The first format is for downloading the firmware image file to ONE adapter specified by adapter number (base 1):

Ex:  download n=2 i=c:\lp6dutil\dd381a3.awc

The second format is for downloading the firmware image file to all adapters of the same type in your system:

Ex:  download a=lp8000 i=c:\dd381x2.awc

4.3  extloopback:

1:  LP8K(v2)

2:  LP9K(2GB)

## OK

This is to run the external loopback test. It requires the loopback plug in place to be successful. With one command, you can run the test on ONE or ALL adapters in your system. In the following examples, r is the repeat count, o is the test action option on error with o=1 for STOP, o=2 for REPEAT, o=3 for IGNORE.

Ex:  extloopback n=1 r=50 o=1

Ex:  extloopback n=all r=20 o=3

4.4  intloopback:

This is to run internal loopback test. Same format as extloopback.

This test does not run on the IBM Blade LP1005DC-CM2.

4.5  pciloopback:

This is to run pci loopback test. Same format as extloopback.

4.6  reset:

This is to reset the adapters. This command has a s option to skip the post test in Restart (1=yes;  0=no):

Ex:  reset n=all s=0

4.7  sramtest:

This test does not run on the IBM Blade LP1005DC-CM2.

This is to run SRAM test. It has some more arguments:

Number of sequences:			s

Number of concurrent sequences:	c

Sequence length in bytes:		l

Pattern index:				p

Pattern index p:

0:	random bytes

1:	all 0

2:	all 1

3:	all 5

4:	all A

5:	random words

6:	repeating bytes

7:	increment 1

8:	increment 2

9:	increment 3

10:	walk 0

11:	walk 1

12:	loop all

13:	steve 1

14:	steve 2

15:	steve alternate

Ex:  sramtest n=all r=10 o=1 s=20 c=5 l=1024 p=4

NOTES:  You can set Repeat Count r=0 to have a test run forever. In this case, hit s key when you want to stop the test.

- SCRIPT FILE:

Commands can be grouped together in a script file so that a set of commands can be carried out. A sample script file labeled samplescript.txt is provided for your convenience. Comment line starts with a semicolon. Each line follows the same parsing rules as described earlier. You can make copy of this sample file to create your own script file, take the semicolons off, make some value changes to fit your need, then run the combined tests via @ command or via the menu bar as described later.

NOTES: You can have the series of all test commands in the Script file to run again and again forever by adding the command repeat at the last line of the Script file. In this case, you can hit the s key any time you want to stop all tests.

- OUTPUT FILE:

When you run tests with script file, an output file is automatically created to log the test results so that you can view them at a later time. This file, labeled lp6dlog.txt, is placed in c:\lp6dutil directory. Due to the limited scrolling buffer size (for about 12 screen full), you can copy and save lp6dlog.txt and start a new one at appropriate times.

- USE OF MENU BAR:

# File       Adapter        Info        Test        Flash        Debug        Help

The menu bar has 7 items as shown above. When you hold down the ALT key, each of the 7 items has a highlighted letter as shown below:

# File       Adapter        Info        Test        Flash        Debug        Help

To obtain the pop-up menu for one menu item, hold down the Alt key and hit the corresponding highlighted letter key.

7.1  To Quit LP6DUTIL:

Hold down the Alt key, hit F key for the pop-up menu of the File menu item.

# Open Input Script…

Read Logfile…

Exit

Hit x key to exit LP6DUTIL.

7.2  The File Menu Item:

Pop-up menu is shown above. We will describe this File menu item with great details. You can handle other menu items in similar manner.

To make an entry selection in the pop-up menu, move the block cursor to that entry then hit Enter or you can hit the corresponding highlighted letter. To move the block cursor, you can use the Up/Down arrow key. In case of this pop-up menu, hitting the o key will have the Select Input Script dialog box as shown below.

To make an entry selection in a dialog box, move the cursor to that entry then hit Enter. There are two ways to move cursor within a displayed dialog box:

Either you hit Tab key

Or you hit Alt + the highlighted letter of the desired box title. Please note that for buttons such as OK, Alt + o has the double effect as the cursor is moved to this OK button and Enter key is hit.

Within a scrollbox,  the Up/Down arrow key is used to move the block cursor.

- Open Input Script:

Select Input Script

File Name:

Drives:                               Files:

Directories:

A:

C:

OK

Cancel

Select the desired drive and directory. All the existing files in the selected directory will be displayed in the Files box. Select the desired file. Next hit Tab key to move the cursor to the OK button and hit Enter (or hit Alt + o).

All tests listed in the script file will run, and results are logged into the output file as described earlier.

- Read Logfile:

Hit l key or move the block cursor to this entry and hit Enter. A Test Logfile message box appears with some general information about the log file. Move the cursor to the OK button and hit Enter. The log file contents will be displayed. Scrolling is supported.

7.3  The Adapter Menu Item:

Pop-up menu:

Display…

Set Status Word…

Reset…

Warmstart/NoRam…

DC Bridges…

Display and DC Bridges are self explanatory.

Restart has different types and custom options as shown in its dialog box.

7.4  The Info Menu Item:

Pop-up menu:

# PCI Info

Revisions

Service Parameters

Counters

Link Status

Link Attention

Select desired entry for getting information on different aspects of the adapters. Dialog boxes are self explanatory.

7.5  The Test Menu Item:

Pop-up menu:

# Select Adapters…

Select Tests…

Select Options…

Start Tests

With Select Adapters dialog box, you can select one, multi, or all detected adapters for testing.

With Select Tests dialog box, you can select one, multi, or all tests.

With Select Options dialog box as shown below, you select some options common to all tests, and some options that are applied only to a specific test:

# Configure Test Parameters

Common Options:                                   Test Specific Options:

Repeat Count: [00000050]

Error Action:

(.)  Stop

( )  Repeat

( )  Ignore

# PCI Loopback

Internal Loopback

External Loopback

SRAM Test

Done

Only SRAM test has specific options.

Start Tests selection displays the dialog box as shown below.

Hit Alt + s to start selected tests on selected adapters.

As tests progress, you will see summarized test result messages displayed in the Messages box.

Hit Alt + u then select the desired adapter in the Under Tests box to see the test results in the Pass Count box.

# Test Results

Under Tests:                       Pass Count:

Messages:

# PCI Loopback Test:                   00000050

Internal Loopback Test:             00000050

External Loopback Test:            00000000

SRAM Test:                               00000050

1:  LP8K(v2)

Start

**Start tests on adapter 1

Run PCI Loopback test

Pass 50 test(s)

*Complete tests on adapter 1, NO error

Stop

Done

7.6  The Flash Menu Item:

Pop-up menu:

# Display Flash…

Download…

Direct Download

Boot Bios…

Soft Jumpers…

Wakeup Parms…

Configuration Regions…

We are going to discuss the download process. Others are self explanatory.

The Download dialog box is shown below.

Select the desired drive and directory. All the existing files in the selected directory will be displayed in the Files box. Select the desired file. Next select the adapter then hit Alt + o to start downloading.

Download Image

File Name:

Drives:                               Files:

Directories:                        Adapters:

Reset After Download:

(.)  Yes

( )  No

A:

C:

1:  LP8K(v2)

2:  LP9K(2GB)

OK

Cancel

Various steps of download process will be displayed on the screen with the success or error result.

7.7  The Debug Menu Item:

Pop-up menu:

# SLIM

Mailbox

Registers

Dump

Run Program

Run Diag

Recovery

Use these as debugging tools.

7.8  The Help Menu Item:

Pop-up menu:

# Help

About

Help entry currently shows nothing.

About entry shows the LP6DUTIL version and the Emulex Copyrights.