AWSICE: S/390 Parallel Channel

==============================

The AWSICE manager allows real S/370 I/O control units to be attached to a

P/390 system via the S/390 Parallel Channel Adapter (AKA PARCA).

Configuring devices

===================

WARNING: Care should be used in assigning P/390 device addresses for AWSICE

devices.  Mixing emulated devices and "real" AWSICE devices on the

same control unit address range for example,

can cause problems in system operation that may only

show up in error recover situations.  Likewise, AWSICE allows you

to remap the physically plugged device address to a different S/390

device address.  This feature is primarily to aid migration -- careless

use of the feature, for example, by mapping devices on the same

controller to different S/390 channels, or mixing up devices

between controllers can cause errors that will be difficult to

detect and resolve.  It is best to assign AWSICE devices to

contiguous ranges that match the device/controller characteristics.

To configure a device, go to the P/390 Configuration "Update System Devices"

(F2) screen.  Use the manager code for AWSICE in the Mgr field.  The FN/P field

can contain the following:

(blank)  P/390 device address is the same as the hardware unit address.

Assumes the device is attached to P/390 Parallel Channel

Adapter 0.

c      where c is a the adapter number to which the device is

attached.  This number is assigned by AIX Configuration Services.

Use "smit(ty)" or "lsdev -C" to determine the Adapter Number of the

desired card by checking the corresponding slot ID.

(ie. "ricio1  Available 04-B0" indicates that slot 4 holds adapter 1).

cdd    Remap P/390 device address to hardware unit address "dd".

Assign to Adapter number "c".

Device Speed

------------

/Dn       Turns on high-speed Data Streaming

"n" is data streaming speed:

/D3 gives 3 MB/sec

/D4 gives 4.5 MB/sec

Note: If /D is not specified, the default is DC Interlock (DCI), but the

device can force the channel into Data Streaming 3 MB/sec.

Similarly, if /D3 is specified, the device can force the channel

into DCI.  So unless you want to run at 4.5 MB/sec, you need not

specify any /D parameter.

Device Mode

-----------

/B        Run device in Byte Multiplexor mode

Note: If /B is not specified, the default is Block Multiplexor mode.

Honor Prefetch

--------------

/P       Makes AWSICE honor the prefetch setting of the operating

system (in the ORB).  Not all operating systems can set this,

and it cannot be set in 370 mode.

The default (no /P) is to prefetch all the time, which

improves performance but may prevent some software from

running correctly, specifically software that dynamically

modifies channel programs. Don't specify /P unless you

experience problems.

No Merge

--------

/M       Prevents merging of small data-chained CCWs into a single CCW.

Merge is normally ON, because this results in better performance

and is required for certain devices and channel programs.

Examples:

--------

1. This DEVMAP entry remaps P/390 address 580 to hardware unit address 60.

Default speed is DCI (but can run at 3 MB/sec) and Block Multiplexor.

Type    Addr    Mgr   FN/P

----    ----    ---   ------------

3420    580      x    060

2. Assign device 580 to adapter number 0, Data Streaming at 3 MB/sec,

Block Multiplexor.

Assign 590 to adapter number 1, Data Streaming at 4.5 MB/sec,

Block Multiplexor.

Type    Addr    Mgr   FN/P

----    ----    ---   ------------

3480    580      x    0 /d3

3480    590      x    1 /d4

3. Assign device 580 to adapter number 1 and remap to unit

address to 60.  Default speed is DCI, mode is Byte Multiplexor.

Type    Addr    Mgr   FN/P

----    ----    ---   ------------

3420    580      x    160 /b

Utilities

=========

Help on these commands is available by specifying a "?" after the name.

icestat   Display the LIC levels of the specified ICE card.

icemap    Display the devices claimed by AWSICE and their settings.

Valid only when the system is IPL'ed.

icetrace  Get an internal ICE card I/O trace dump.

The devices must be enabled for tracing as specified in the

section "Tracing AWSICE Device I/O", above.

icedev    Standalone scan of the channel for devices. Valid when system is

no IPL'e, however, the P/390 must have been

IPL'ed at least once since the last AIX re-boot. IPL'ing the P/390

initializes the S/390 Channel Adapter(s).  Use 'ipl390 noipl' followed

by 'stop390' to initialize the adpaters if you have just re-booted AIX.

See User's Guide or for more utilities...

INSTALLATION NOTES:

==================

1. Install ARTIC960 support programs from installation diskettes  "RS/6000

S/390 ARTIC960 Device Drivers for AWSICE".  Type 'smitty p390' and select

the appropriate install option.

2. Uncomment the AWSICE Device Manager section in /usr/lpp/r390/bin/ipl390

if you are at r390.obj 4.2.1.0 or an older version.

Uncomment everything except the "ricinit" statements for adapters that

are not installed.

NOTE: Starting at r390.obj 4.2.1.5, modifications to ipl390 are NOT needed.

AWSICE is started by changing the "nostart" keyword to "noparms" on the

"Start Device Manager Parameter Input/Display" panel of the Configurator

(press F12 from the Update System Devices panel).

3. Change the Secondary Arbitration level of "Available" adapter

to "ENABLE" from SMIT. NOTE: This option  may not exist depending on

the level of RIC code installed.  If it does not exist, skip this step.

run smit & choose following options

- Devices - Communications - IBM ARTIC960 Adapter - Adapter

- Change/Show Characteristics of a Artic960 Adapter

From Popup List Select Available IBM ARTIC960 Device Driver

ENABLE the Secondary Arbitration Level & Commit the change

4. Append /usr/lpp/devices.artic960/bin to the system PATH variable. This is

ussually done in /.profile, /.dtprofile, or /etc/environment.

5. Re-boot the AIX system.

Tracing AWSICE Device I/O

=========================

I/O tracing can be enabled for up to four devices per ICE card at a time.

Specify the device addresses in MANOPs:

While P/390 I/O subsystem is running:

- Click on "P/390 Manual Operations" icon (MANOPS),

- Click on "Control", then "Device Debug"

- Type in the desired device address

- Check the box labelled "I/O Adapter Trace"