LAN3172: 3172 Emulator

=====================

The 3172 emulator lets VTAM running on the P/390 think it is talking

to a real channel-attached 3172.

DEVICE TYPE: 3088

=================

CONFIGURING A 3172

==================

To configure a 3172, go to the 390 Configuration "Update System Devices"

(F2) screen and configure a single 3088 device address.  Use the manager code

for LAN3172 in the Mgr field.  Do not put anything in the FN/P field.

Example:

______________________________________________________________________________

|Addr Device Label  Atype  Size     Mgr  FN/P                 Path:>         |

|-------------------------------------- -------------------------------------|

| 448 >3088 >      >     >          9                                        |

|------                            ---                                       |

|____________________________________________________________________________|

MGR Codes: 1=AWSFBA   2=AWSCKD   3=AWS3274  4=LAN3274  5=AWS3215  6=AWS2821

7=AWS2540  8=LAN3088  9=LAN3172  A=LCS3172  B=AWS2703  C=AWSICA   D=AWSTAPE

Where 448 is the device address and 9 is the LAN3172 manager code.

Only ONE 3172 device address (VTAM CUADDR) can be specified.  Only VTAM/SNA

(LSA or Link Services Architecture) connections are supported.  Multiple

adapters are supported by configuring multiple XCA VTAM Lists with unique

SAPs and ADAPNOs (see example below).

LAN3172 can share and adapter with LCS3172 or with AIX TCP/IP.

INSTALLATION NOTES:

==================

1) DLC Support

LAN3172 uses the following 3 services provided by AIX:

dlctoken - Token Ring Data Link Control

dlcether - standard (DIX) ethernet Data Link Control

dlc8023  - IEEE 802.3 Data Link Control

NOTE: It is recommended that the entire DLC fileset group be installed

from the AIX Installation CDROM.

To find out whether these DLCs are Available, type from shell prompt,

ls -C |grep dlc

If the required DLCs are not present and available, they must be installed

be configured using smit.  Refer to SMIT configuration below.

IMPORTANT: Standard Ethernet usage requires that both /dev/dlcether and

/dev/dlc8023 are available.

2) Very old levels of LAN3172 may require the bos.compat.lan fileset to be

installed on AIX 4.1 for COMIO compatibility.  This is not required on

R/390 filesets "r390.obj 4.0.15.0" and higher.  Prior to 4.0.15.0, only

LAN adapters that support COMIO will work.  This interface was changed

in 4.0.15.0 so that COMIO is no longer a dependency. Such older levels are

no longer supported.

Invocation of the 3172 device manager (dm3172)

----------------------------------------------

NOTE: Starting at r390.obj 4.3.x.x, modifications to ipl390 are NOT needed.

These parameters (eg tok0, ent1, etc) should be entered on the Configurator's

"Start Device Manager  Parameter Input/Display" panel.

Uncomment the 3172 device manager statement (dm3172) in ipl390 script,

Example 1 (multiple adapters):

------------------------------

$DMDIR/dm3172 tok1 ent0 &

where,

dm3172 -->  is the executable file for 3172 emulator.

tok1 --> the Token Ring network device name (token ring 1)

ent0 --> the Ethernet network device name (ethernet 0)

The above example assumes 2 LAN adapter cards are to be used in the

VTAM XCA configuration.

NOTE: The VTAM ADAPNO parameter should be set to the position (starting at

zero) of the desired adapter device name in the dm3172 statement:

tok1 will be ADAPNO  0

ent0 will be ADAPNO  1

Example 2:

----------

If there is only one adapter is to be used, say token ring 1,

then the ipl390 statement would read,

$DMDIR/dm3172 tok1 &

In this case, only one VTAM XCA definition is needed with

ADAPNO=0 (tok1 is in position 0 since no other adapters used).

CONFIGURATIONS ... :

====================

The RS6000/AIX related configurations are to be done through "SMIT"

interface provided by AIX. Refer to appropriate manuals of AIX for more

detail.

SMIT CONFIGURATION:

==================

To configure LAN parameters necessary for LAN3172 device manager,

1) Configuring DLC device managers.

-----------------------------------

use smit options in the following order to configure DLC.

For Ethernet networks,

"smit"

"Devices",   "Communications",     "Ethernet",

"Services",  "Data Link Controls", "Add an Ethernet Data Link Control"

and select the following from the list,

"dlc ethernet dlc Standard Ethernet Data Link Control"

"dlc IEEE_ethernet dlc IEEE Ethernet (802.3) Data Link Control"

NOTE: For Standard Ethernet and 802.3, it is recommended that the Maximum

IP Packet	size is set to 1492.  From the "Services" panel, go to

"Change/Show Characteristics of a Network Interface Driver".

For Token Ring networks,

"smit"

"Devices",   "Communications",     "Tokenring",

"Services",  "Data Link Controls", "Add a Token ring Data Link Control"

and select the following from the list,

"dlc tokenring dlc Token-Ring Data Link Control"

2) Configuring the Network Adapters / Network

----------------------------------------------

For the Network adapter configuration, say Token Ring/Ethernet

adapters, refer to appropriate AIX manuals.

Once the adapters are configured, follow the steps below to change the

Adapter address so that the VTAM configuration and AIX system configuration

match.  LAN3172 can use only the alternate adapter address, so this step

is required for both Ethernet and Token Ring (example shown for tok0 only):

"smit"

"Devices",   "Communications",     "Tokenring",

"Adapters", "Change / Show Characteristics of a Token Ring Adapter"

select the desired adapter (example "tok0...")

Modify only the following three fields from the menu displayed.

Enable ALTERNATE TOKEN RING address                 yes             +

ALTERNATE TOKEN RING address                       [0x400074700001] +

Apply change to DATABASE only                       yes

The above address 0x400074700001 is a LOCALLY ADMINISTERED ADDRESS:

This address must start with hex 4000 and must be 12 digits.  The

remaining 8 digits can be whatever you like as long as the address

is unique on the Local Area Network.

NOTE: This address will be available only on the next System BOOT.

The above example matches the P/390 VM Starter System node "SYS1",

The address entered here will be used by a remote VTAM node as the

DESTINATION MAC ADDRESS (see XCA VTAMLST below).

VTAM CONFIGURATION

==================

DISCLAIMER:  The following examples may not be accurate depending

on your particular VTAM and SNA client and peer

software.  Always consult your VTAM Resource Definitions

and client/peer configuration and customization manuals.

VTAM CONFIGURATION - SAMPLE PEER TO PEER XCA CONNECTION

==================

_____________

_______________     |           |     _______________

|             |     |           |     |             |

| SYS1 (SA=1) |_____|    LAN    |_____| SYS2 (SA=2) |

|             |     |           |     |             |

|_____________|     |___________|     |_____________|

400074700001                          400074700002

SYS1: XCA VTAMLST

-----------------

SYS1XCA VBUILD TYPE=XCA

SYS1448 PORT CUADDR=448,  * SYS1 LAN3172 DEVMAP 370 Address

ADAPNO=0,    * LAPS Adapter Number

MEDIUM=RING,

SAPADDR=8,   * Use SAP(8) since CM uses SAP(4)

TIMER=60

SYS1XCA2 GROUP DIAL=NO

SYS1XLS2 LINE USER=SNA,ISTATUS=ACTIVE

SYS1XP2 PU ISTATUS=ACTIVE,

MACADDR=400074700002,  * SYS2 LAPS MAC Destination Address

PUTYPE=5,

SAPADDR=8,             * Use SAP(8) since CM uses SAP(4)

SUBAREA=02,

TGN=1

SYS2: XCA VTAMLST

-----------------

SYS2XCA VBUILD TYPE=XCA

SYS2448 PORT CUADDR=448,   * SYS2 LAN3172 DEVMAP 370 Address

ADAPNO=0,     * LAPS Adapter Number

MEDIUM=RING,

SAPADDR=8,    * Use SAP(8) Since CM uses SAP(4)

TIMER=60

SYS2XCA1 GROUP DIAL=NO

SYS2XLS1 LINE USER=SNA,ISTATUS=ACTIVE

SYS2XP1 PU ISTATUS=ACTIVE,

MACADDR=400074700001, * SYS1 LAPS MAC Destination Address

PUTYPE=5,

SAPADDR=8,            * Use SAP(8) since CM uses SAP(4)

SUBAREA=01,

TGN=1

Multiple adapters are supported via additional PORT statements with a

different ADAPNO specification (only one global 370 CUADDR is allowed)

PC/3270 CONFIGURATION -  Setting up a Client PC for LOGON sessions

---------------------

These instructions are for the IBM PC/3270 product. If you are using another

emulator product, read through the steps and then use the summary information

that follows to customize your emulator parameters.

On each DOS client PC:

1. Install the IBM LAN Support Program. You MUST install the 802.2 support.

This will place two statements in your CONFIG.SYS similar to this:

DEVICE=C:\LSP\DXMA0MOD.SYS

DEVICE=C:\LSP\DXMC0MOD.SYS

2. Install PC/3270. Here are abbreviated instruction steps:

- Insert PC/3270 DOS diskette 1 and type A:INSTALL

- Choose "Quick Installation"

- For "Attachment type" choose "LAN via 802.2 protocol"

- For "Number of host sessions" select how many you want (1 to 8).

- On menu screen "LAN via 802.2 Protocol" fill in:

>   Destination Address:  400074700001 <--- e.g. for P/390 SYS1

Use the Network Adapter address of the P/390 you want to connect to

via the LAN.

The Network Adapter address is also used by

other systems to connect via VTAM to your P/390 (see the MACADDR

parameter in the appropriate VTAMLST).

>   Physical Unit ID: nnnnn

This number uniquely identifies your PC workstation. It can be any five

digit hexadecimal number that is unique across the entire LAN.

>   Block ID: 05D

The block ID is used to preface the Physical Unit ID to form an exchange

identification. Use the default value of 05D.  If you change it, then

it must match the IDBLK parameter in the SLAN VTAMLST.

>  PIU Size: 0265

PIU specifies the maximum number of receive data bytes. KEEP THE DEFAULT

OF 0265 FOR NOW. We will change this later manually! The desired value

that will match the P/390 Starter System VTAM cannot be entered on this

menu. It is equal to the MAXDATA parameter in SLAN VTAMLST plus 36 bytes

(4905 + 36 = 4941).

- Follow the directions on screen to save the PC/3270 configuration, and

exit the install program. DO NOT REBOOT the PC yet!

- Edit the file c:\PC3270\PC3270.BAT.

- Locate the line that starts "PCSTKR LU= ..."

- Change "RS=265" to "RS=4941" <-- MAXDATA + 36

- Change "DS=04"  to "DS=08"   <-- SAPADDR value in SLAN VTAMLST

- Save the file.

- Reboot your PC and type $PC3270 to start the emulator

(or c:\PC3270\PC3270 to skip the setup program each time).

SUMMARY OF EMULATOR PARAMETERS:

Using the P/390 VM Starter System, for node ID "SYS1", set up the

following parameters in each client's 3270 emulator:

Destination Address (TIC):  400074700001

ID Number: 05Dnnnnn

Receive Buffer Size: 4941

EXAMPLES:

========

����������������������������Ŀ

�Ĵ VTAM XCA TO XCA CONNECTION �������������������������������������������Ŀ

� ������������������������������                                           �

�                                                                          �

�                                                                          �

�                       �����������Ŀ                                      �

�  ��������������Ŀ     �           �     ��������������Ŀ                 �

�  � VMA   SA=193 �����Ĵ TokenRing �����Ĵ VMB   SA=194 �                 �

�  ����������������     �           �     ����������������                 �

�    4000749000C1       �������������       4000749000C2                   �

�                                                                          �

�                                                                          �

� ����������������������Ŀ                                                 �

�Ĵ VMA � XCA MAJOR NODE �������������������������������������������������Ĵ

� ������������������������                                                 �

� NGAXCA VBUILD  TYPE=XCA                                                  �

� *                                                                        �

� NGAX280 PORT   CUADDR=280,ADAPNO=0,SAPADDR=4,MEDIUM=RING,              - �

|                DELAY=0.1,TIMER=20                                        �

� *                                                                        �

� *------------------------------------------------ NGAXCA SUBAREA NODES   �

� *---------------------------------------------------------------------   �

� *                                                                        �

� *--------------------------------------------------------- SUBAREA VMB   �

� NGAXSNGB GROUP DIAL=NO                                                   �

� *              ----------------------------------------------- LAN3172   �

� NGAXSNLB  LINE USER=SNA,ISTATUS=ACTIVE                                   �

� *                                                                        �

� NGAXSNPB    PU PUTYPE=5,SUBAREA=194,TGN=1,                             - �

|                MACADDR=4000747000C2,SAPADDR=4                            �

|                                                                          �

� ����������������������Ŀ                                                 �

�Ĵ VMB � XCA MAJOR NODE �������������������������������������������������Ĵ

� ������������������������                                                 �

� NGBXCA VBUILD  TYPE=XCA                                                  �

� *                                                                        �

� NGBX280 PORT   CUADDR=280,ADAPNO=0,SAPADDR=4,MEDIUM=RING,              - �

|                DELAY=0.1,TIMER=20                                        �

� *                                                                        �

� *------------------------------------------------ NGBXCA SUBAREA NODES   �

� *---------------------------------------------------------------------   �

� *                                                                        �

� *--------------------------------------------------------- SUBAREA VMA   �

� NGBXSNGA GROUP DIAL=NO                                                   �

� *              ----------------------------------------------- LAN3172   �

� NGBXSNLA  LINE USER=SNA,ISTATUS=ACTIVE                                   �

� *                                                                        �

� NGBXSNPA    PU PUTYPE=5,SUBAREA=193,TGN=1,                             - �

|                MACADDR=4000747000C1,SAPADDR=4                            �

|                                                                          �

����������������������������������������������������������������������������

NOTES: (XCA >< XCA) (TYPE5 >< TYPE5) CONNECTION

. each P/390 has a single TokenRing Adapter installed (ADAPTER NUMBER 0)

. each VTAM in each P/390 is using SAP('04'x)

. each PU MACADDR= address is the destination (remote) LAN3172 LAN Network

Adapter Address (VTAM obtains the local LAN3172 LAN Network Address

by query from LAN3172).

. each PU SAPADDR= address is the destination (remote) LAN3172 SAP Address

(VTAM passes the local LAN3172 SAP Address (from PORT SAPADDR=) to

LAN3172 by command).

. PORT parameter DELAY= was added to VM by APAR VM58851 and will not be

valid without this VTAM APAR (added by PTF UV58746, 11/09/94)

��������������������������������Ŀ

�Ĵ VTAM XCA PERPHERIAL CONNECTION ���������������������������������������Ŀ

� ����������������������������������                                       �

� �����������������Ŀ                                                      �

� �  P/390          �                                                      �

� �����������������Ĵ                                                      �

� �  LAPS ADAPTER0  � �����������Ŀ                                        �

� ���������������Ŀ � �           �   ������������Ŀ                       �

� �� VMA   SA=193 ���Ĵ TokenRing ���Ĵ TYPE2.1 PU �  OS2/CM2 05D0000      |

� ����������������� � �           �   ��������������                       �

� �  4000749000C1   � �������������                                        �

� �                 �                                                      �

� �                 �                                                      �

� �  LAPS ADAPTER1  � �����������Ŀ                                        �

� ���������������Ŀ � �           �   ������������Ŀ                       �

� �� VMA   SA=193 ���Ĵ EtherNet  ���Ĵ TYPE2.1 PU �  DOS/PC3270 05D0001   �

� ����������������� � �           �   ��������������                       �

� �  4000749001C1   � �������������                                        �

� �������������������                                                      �

� ���������������������������Ŀ                                            �

�Ĵ XCA MAJOR NODE /TOKENRING ��������������������������������������������Ĵ

� �����������������������������                                            �

� NGAXCAR VBUILD TYPE=XCA                                                  �

� *                                                                        �

� NGAX280R PORT  CUADDR=280,ADAPNO=0,SAPADDR=4,MEDIUM=RING,              - �

|                DELAY=0.1,TIMER=20                                        �

� *                                                                        �

� *--------------------------------------------- NGAXCA PERIPHERAL NODES   �

� *---------------------------------------------------------------------   �

� *                                                                        �

� NGAXRNG  GROUP DIAL=YES,CALL=INOUT,ANSWER=ON,ISTATUS=ACTIVE              �

� *                                                                        �

� NGBXRNL0 LINE                                                            �

� NGBXRNP0 PU                                                              �

�                                                                          �

� ��������������������������Ŀ                                             �

�Ĵ XCA MAJOR NODE /ETHERNET ���������������������������������������������Ĵ

� ����������������������������                                             �

� NGAXCAE VBUILD TYPE=XCA                                                  �

� *                                                                        �

� NGAX280E PORT  CUADDR=280,ADAPNO=1,SAPADDR=4,MEDIUM=CSMACD,            - �

|                DELAY=0.1,TIMER=20                                        �

� *                                                                        �

� *--------------------------------------------- NGAXCA PERIPHERAL NODES   �

� *---------------------------------------------------------------------   �

� *                                                                        �

� NGAXENG  GROUP DIAL=YES,CALL=INOUT,ANSWER=ON,ISTATUS=ACTIVE              �

� *                                                                        �

� NGBXENL0 LINE                                                            �

� NGBXENP0 PU                                                              �

�                                                                          �

� ���������������������Ŀ                                                  �

�Ĵ SWITCHED MAJOR NODE ��������������������������������������������������Ĵ

� �����������������������                                                  �

� NGASLAN VBUILD TYPE=SWNET                                                �

� *                                                                        �

� *        -  -- ---------------------------------------------- NGASXP00   �

� *                                                                        �

� NGASXPP0    PU PUTYPE=2,ADDR=C1,MAXDATA=2042,                          - �

�                IDBLK=05D,IDNUM=00000,CPNAME=NGASXP00,                  - �

�                DLOGMOD=HTPCMD2E,MODETAB=NGMODE,USSTAB=NGUSSS             �

� *                                                                        �

� NGASXL01    LU LOCADDR=1                                                 �

� NGASXL02    LU LOCADDR=2                                                 �

� NGASXL03    LU LOCADDR=3                                                 �

� NGASXL04    LU LOCADDR=4                                                 �

� *                                                                        �

� *        -  -- ---------------------------------------------- NGASXP01   �

� *                                                                        �

� NGASXPP1    PU PUTYPE=2,ADDR=C1,MAXDATA=1490,                          - �

�                IDBLK=05D,IDNUM=00001,CPNAME=NGASXP01,                  - �

�                DLOGMOD=HTPCMD2E,MODETAB=NGMODE,USSTAB=NGUSSS             �

� *                                                                        �

� NGASXL11    LU LOCADDR=1                                                 �

� NGASXL12    LU LOCADDR=2                                                 �

� NGASXL13    LU LOCADDR=3                                                 �

� NGASXL14    LU LOCADDR=4                                                 �

�                                                                          �

����������������������������������������������������������������������������

NOTES: (XCA PERIPHERAL) (TYPE2) CONNECTION

. single P/390 with two LAN Adapters installed

. a TokenRing Adapter installed (ADAPTER NUMBER 0)

. an EtherNet Adapter installed (ADAPTER NUMBER 1)

. two XCA MajorNodes (one for TokenRing, one for EtherNet)

. each XCA MajorNode references the same LAN3172 device (CUADDR=280)

. only one LAN3172 SubChannel address in DEVMAP (a single LAN3172 with

multiple LAN Adapters)

. each LINE/PU pair under the GROUP DIAL=YES requires one LAPS 802.2

link station (these link stations are allocated to LAN3172 when VTAM

activates the XCA MajorNode) so keep that in mind when deciding how

many LINE/PU pairs to code in the XCA MajorNode.

. note that MAXDATA= for EtherNet connections is limited to 1490 bytes

(EtherNet Adapters from vendors other than IBM may have a slightly

lower limit) -- a lower limit may imposed by the remote end depending

upon the EtherNet protocol stack and/or 3270 emulator used.