Release Notes

1.08a 			12-10-98

# Here is a list of the changes between ATM code 1.05 and 1.08a

1. Corrected a number of bugs that could result in "stuck" LE_ARP table entries, with an ATM address of 0ff0ff0ff.... and add a checking task to the software to remove any entries that are found to remain in a non-operable state for an exceptionally long time (about 1 minute).

2.  Corrected the counting of AAL5 errors, previously the count could occasionally increment by 65535, rather than 1!

3.  Detected the movement of MAC addresses between switches and clear obsolete entries in LE_ARP table.  This permits movement of devices between switches without the need to wait for the 5-minute LE_ARP table entry aging timeout.

4.  Fixed interoperability issue with Bay Centillion switch.

5.  Fixed memory leak in NARP processing code that resulted in 009085E.

6.  Fixed error in ILMI code that occasionally caused failure to respond to ILMI GET requests from ATM switch.  This could result in an ATM port reset, requiring all LEC connections to be re-established (only seen with IBM 8265 core ATM switch).

7.  A number of minor errors were found and fixed by the developers and some performance improvements were made to the ATM code.

8. Fixed fault in code that could occasionally result in error 090603f while a LEC is attempting to join an ELAN.

9.  Diagnostic code has been included to help resolve future ATM reset problems.  The state of the stack at the time of the failure is stored in the MIB: LsAtmElanTable, which is not used by the ATM module.

10.  Corrected several problems that could occur if the LE_ARP table became full - this could result in module resets or loss of connectivity due to the incorrect deletion of entries in the table.

11.  A problem was corrected that caused the incorrect addition of a false permanent entry in the LE_ARP table.  This was invisible on the VT100 screens, but could eventually result in the software reporting CAM full and the problems outlined in (10) above.

12.  Occasional resets with error code 00806303 or 00902107 were corrected.