IBM Corporation

IBM Nways RouteSwitch Release Notes

Version 3.2.6

- Rev. G

October 28, 1998

Nways RouteSwitch Release Notes

Version 3.2.6

- IBM Networking Home Page:

- http://www.networking.ibm.com/

PTF		APAR

8273 RouteSwitch Software				NP00952	NA04880

8273 Advanced Routing Software				NP00953	NA04881

8273 LANE Services Software				NP00950	NA04877

8274 RouteSwitch Software				NP00954	NA04882

8274 Advanced Routing Software				NP00955	NA04883

8274 RouteCell Software					NP00956	NA04884

8274 LANE Services Software				NP00957	NA04885

8277 RouteSwitch Software				NP00947	NA04866

8277 Advanced Routing Software				NP00948	NA04871

8277 LANE Services Software				NP00949	NA04872

IMPORTANT INFORMATION FOR IBM CUSTOMERS:

Before reading the remainder of this document, please read the following:

Notice: This document may contain information about features, models, and/or functions, not currently offered for sale by IBM.  The inclusion of this information should NOT be interpreted as indicative that these features/models/functions currently exist in any IBM product or that IBM plans to offer them in the future.

- Table of Contents

- 1	Introduction	7

2	Technical Support Contacts	7

3	Release 3.2 Feature Set Supported	7

4	Unsupported in this release (3.2.6):	11

4.1	IBM 8277 Nways Ethernet RouteSwitch	11

4.2	IBM Nways 8274 LAN RouteSwitch	11

4.3	IBM Nways 8273 Ethernet RouteSwitch	11

4.4	SNMP Agent does not support the following features:	11

5	General Instructions and Notes for IBM Nways 8274 LAN RouteSwitch and IBM 8277 Nways Ethernet RouteSwitch.	12

6	Group Mobility	15

7	IBM 8277 Nways Ethernet RouteSwitch Reboot Routine	15

8	Binding VLANs	15

9	Content Addressable Memory (CAM): Advanced Uses (IBM Nways 8274 LAN RouteSwitch)	15

10	Duplicate MAC Support on IBM Nways 8274 LAN RouteSwitch	17

11	Authenticated VLANs	17

11.1	Server	17

11.2	AMC Server Setup	17

11.3	Authentication Server Configuration	17

11.4	Authentication Agent Setup and Configuration	18

11.5	Authenticated VLAN Client	18

11.6	Troubleshooting	18

11.7	Helpful Hints and other Notable Information	19

12	IBM Support for Functional Address (IBM Spanning Tree)	20

13	Memory Requirements for IBM Nways 8274 LAN RouteSwitch, IBM 8277 Nways Ethernet RouteSwitch and IBM Nways 8273 Ethernet RouteSwitch	21

14	Dynamic DRAM allocation (mpm.img) Requirements	23

15	Multiple Image Requirements	24

16	ATM Services	24

17	Release 3.2.6 IBM Nways 8274 LAN RouteSwitch FLASH Requirements	25

18	Release 3.2.6 IBM 8277 Nways Ethernet RouteSwitch FLASH Requirements	26

19	Release 3.2.6 IBM 8277 Nways Ethernet RouteSwitch Images Required	26

20	Release 3.2.6 IBM Nways 8273 Ethernet RouteSwitch FLASH Requirements	27

21	MPM Upgrade	28

22	Simm Flash Module	28

23	HSM2 Modules	28

24	ESM-100C-12, ESM-100F-8, and ESM-C-32 modules	28

25	Optimized Ports on ESM-100C-12, ESM-100F-8, ESM-C-32 and IBM 8277 Nways Ethernet RouteSwitches	28

26	ATM LANE	29

26.1	802.3 LEC	29

26.2	802.5 LEC	29

26.3	ATM 802.3 LEC to Ethernet Segment Translations	29

26.4	LAN Emulated Client Start–up and Back-off Timers	29

26.5	Plug and Play LECS	30

26.6	LANE Service Module	30

27	PNNI - Configuring Node-Specific Parameters	30

28	PNNI Identifiers: ATM Address and Node ID	31

29	PNNI and Redundant MPM's	31

30	CSM/FCSM Hot Swap Instructions	31

31	CSM-PNNI MTU Size Configuration Guidelines	32

32	IBM Nways 8274 LAN RouteSwitch Version 3.1 Problem Resolutions	33

33	IBM Nways 8274 LAN RouteSwitch Version 3.2 Problem Resolutions	35

34	Known Problems	41

34.1	Known Advanced Routing, GateD & Routing Problems	41

34.2	Known ATM Problems	42

34.3	Known Authenticated VLANs Problems	43

34.3.1	Client	43

34.3.2	Switch	43

34.4	Known Nways RouteTracker Manager Problems	43

34.5	Known BootP Problems	43

34.6	Known Bridging Problems	43

34.7	Known CSM Problems	43

34.8	Known Duplicated MAC Problems	44

34.9	Known Ethernet Problems	45

34.10	Known FDDI Problems	45

34.11	Known Firewall Problems	45

34.11.1	Firewall Server	45

34.11.2	Firewall Switch	45

34.12	Known Group Mobility Problems	45

34.13	Known Health MIB Problems	46

34.14	Known Hot Swap Problems	46

34.15	Known HRE Problems	46

34.16	Known LANE Service Module Problems	46

34.17	Known LSM Problems	46

34.18	Known Mammoth Problems	46

34.19	Known MPM Redundancy Problems	47

34.20	Known IBM 8277 Nways Ethernet RouteSwitch Problems	47

34.21	Known IBM Nways 8273 Ethernet RouteSwitch Problems	47

34.22	Known Port Manager Problems	48

34.23	Known Port Mirroring Problems	48

34.24	Known Port Monitoring Problems	48

34.25	Known SNMP Problems	48

34.26	Known System Problems	50

34.27	Known Token Ring Problems	50

34.28	Known Translation Problems	50

34.29	Known User Interface and General Problems	50

34.30	Known WSM Problems	51

34.31	Known X-Cell/FCSM Problems	51

35	Route-Vision Compatibility & Minimum Requirements Matrices	55

35.1	Compatibility Matrix	55

35.2	Minimum Requirements	56

36	IBM Nways 8274 LAN RouteSwitch, IBM 8277 Nways Ethernet RouteSwitch and IBM Nways 8273 Ethernet RouteSwitch	58

37	Hardware/Software Release Compatibility Matrices	58

37.1	IBM Nways 8274 LAN RouteSwitch Hardware/Software Release Compatibility Matrix	58

37.2	IBM 8277 Nways Ethernet RouteSwitch Hardware/Software Release Compatibility Matrix	73

37.3	IBM Nways 8273 Ethernet RouteSwitchSwitch Hardware/Software Release Compatibility Matrix	74

Revision History

| Release | Revision | Date | Description of Changes |
| --- | --- | --- | --- |
| 3.2.1 | A | 3/6/98 | Early Availability Release - No IBM Nways 8273 Ethernet RouteSwitch Support |
| 3.2.1 | B | 3/6/98 | Documentation Change Only |
| 3.2.2 | C | 4/22/98 | 2nd Early Availability Release - No IBM Nways 8273 Ethernet RouteSwitch Support |
| 3.2.3 | D | 5/30/98 | General Availability Release - No IBM Nways 8273 Ethernet RouteSwitch Support |
| 3.2.4 | E | 7/6/98 | Maintenance Release to provide IBM Nways 8273 Ethernet RouteSwitch Support and to fix problem reports in earlier release |
| 3.2.5 | F | 8/31/98 | Maintenance Release to fix previous problems. |
| 3.2.6 | G | 10/28/98 | Maintenance Release to fix previous problems. |

# Introduction

These release notes cover the basic feature set supported, known limitations, bugs and known work-arounds for IBM Nways 8274 LAN RouteSwitch Version 3.2.6 software.  Please read this document in its entirety as it contains important operational aspects that may impact your network.

# Technical Support Contacts

This section provides contacts for help if you have questions about the IBM RouteSwitch products or if a RouteSwitch product is not working correctly.  It also explains how to access the IBM electronic sites to obtain the latest version of microcode and release notes.

- Electronic Support
This section explains how to access the IBM electronic site to obtain the latest version of microcode, drivers, and software by using the Internet World Wide Web or FTP.

a.	WWW:	 http://www.networking.ibm.com/ 
This is the IBM Networking home page.  From here, you can access product announcements, publications, and other information regarding hardware and software updates, and a technical support information database.  The direct path to the support area is:

http://www.networking.ibm.com/support

b.	FTP:  	lansupport.raleigh.ibm.com\

Voice Support

IBM Network Hardware Support: 1-800-IBM-SERV.  Follow the menu prompts for Network Hardware.

# Release 3.2 Feature Set Supported

The following hardware and software features are supported subject to the restrictions in this note with release 3.2.6:

Notice: This document may contain information about features, models, and/or functions, not currently offered for sale by IBM.  The inclusion of this information should NOT be interpreted as indicative that these features/models/functions currently exist in any IBM product or that IBM plans to offer them in the future.

- Hardware:

- IBM Nways 8274 LAN RouteSwitch:

- 8274-W33  IBM Nways 8274 LAN RouteSwitch with built-in 150W power supply, supports Frame/Cell Bus.

- 8274-500 Chassis

- 8274-PS5 Power Supply

- 8274-513 Chassis

- 8274-PS5-250  8274-513 power supply

- 8274-913 Chassis

- 8274-PS9-500  8274-913 Power Supply

- 8274-900 / 8274-900e Chassis

- 8274-PS9 8274-900 Power Supply

- 8274-W53 Wide, 5 slot IBM Nways 8274 LAN RouteSwitch

- 8274-PS5-DC250, 250W, -48V DC power supply for 8274-W53

- 8274-900wx Wide, 9 slot IBM Nways 8274 LAN RouteSwitch

- 8274-PS9-500P, 500W power supply for 9e, 9x and 9wx

- MPM-1G, or MPM-1GW; MPM 16/32/64 Mbytes DRAM, 4 Mbytes Flash, HRE socket, 960M VBUS for narrow or wide chassis

- MPM II-16/32/64Mbytes

- MPM 16/32/64Mbytes

- HRE Hardware Routing Engine

- ASM-E3-1/2 One or two port ATM 500Kbytes SRAM, 34 Mbps E3, BNC connectors

- ASM2-155FM-1/2  One or two port ASM-SAHI multimode fiber

- ASM2-155FS-1/2  One or two; port ASM-SAHI single mode fiber

- CSM-155-8 or 8W; Eight port, 155Mbps OC3c/STM-1 CSM, multimode fiber for narrow or wide chassis

- CSM-155-8S or 8SW; Eight port, 155Mbps OC3c/STM-1 CSM single mode fiber for narrow or wide chassis

- CSM-155-6M2S six port multimode and two ports single mode 155 Mbps SONET ST3c/STM-1

- CSM-155-6M2SL

- CSM-155C-8, 8 port 155 Mbps OC-3/STM-1 copper

- CSM-155C-8W  8 port 155 Mbps OC-3/STM-1 copper module

- CSM-155-FSL-8

- CSM-622-2E  two port 622 Mbps OC12c/STM-4c CSM multimode fiber, extended memory

- CSM-622-2SE  two port 622 Mbps OC12c/STM-4c, CSM single mode fiber, extended memory

- CSM-A25-12/24, 12/24 port ATM25 module, RJ-45 connectors

- CSM-A25-12/24   12/24 port ATM25 module, RF-45 connectors

- FCSM Frame to Cell Switching Module (Narrow and wide module)

- ESM-U-6 6-port Ethernet Universal Module

- ESM-AB-FL 10BaseFL Adapter Card for ESM-U-6

- ESM-AB-T 10BaseT RJ-45 Adapter Card for ESM-U-6

- ESM-AB-AT 10BaseT RJ-45 and AUI Adapter Card for ESM-U-6

- ESM-AB-B 10Base2 Adapter Card for ESM-U6

- ESM-AB-FL-S  Single-mode 10BaseFL Adapter Card for ESM-U-6

- ESM-AB-AFD Adapter board for Ethernet Universal Switching Module

- ESM-F-8  10BaseFL 8 port Ethernet Module

- ESM-C-8 10BaseT RJ-45 8-port Ethernet Module

- ESM-C-12 10BaseT RJ-45 12 port Ethernet Module

- ESM-T-12 10BaseT, 50 Pin Telco Connectors, 12 port Ethernet Module

- ESM-100C-FD-1/2 One or two port full duplex 100BaseTx, (MM)

- ESM-100-FM-FD-1/2 One or two port full duplex 100BaseFx

- ESM-100-FS-FD-1/2 One or two port full duplex 100Base Fx, (SM)

- ESM-100C-5 4 port shared 100BaseT + 1 full duplex 100BaseTx

- ESM-100CFM-5 4 port shared 100BaseTx + 1 full duplex 100BaseFx (MM)

- ESM-100CFS-5 4 port shared 100BaseTx + 1 full duplex 100BaseFx (SM)

- ESM-100C-12   12 10/100 BaseT, RJ45 ports

- ESM-100FM-8   8-port 100BaseFx full duplex, 8 SC pair, multimode fiber

- ESM-C-32W   32 port 10BaseT, RJ45

- ESM-C-16W  16 port 10BaseT switch module

- FSM-SH-1/2 FDDI Switching Module, SC connectors, supports single mode(category 2)

- TSM-CD-6 Token Ring Dual Mode Module

- WSM-4/8-SC Four or 8 Port WAN Switching Module, Universal Serial External DTE/DCE Wide Area Network Interface Daughter Card

- WSM-SC-4 or 4W; WAN Switching module, 4 ports Universal Serial External Interface.  Supports the five serial cable types (V.35, RS-232, X.21, RS-530, and RS449) for wide chassis

- WSM-SC-8 or 8W; WAN Switching module, 8 ports Universal Serial External Interface.  Supports the five serial cable types (V.35, RS-232, X.21, RS-530, and RS449) for wide chassis

- 8277-BPS-250  Backup Power System 250W, AC for 8274-W33

- IBM Nways 8273 Ethernet RouteSwitchSwitch:

- IBM Nways 8273 Ethernet RouteSwitch 10E

- IBM Nways 8273 Ethernet RouteSwitch 10

- IBM Nways 8273 Ethernet RouteSwitch 10U

- RASM-DS3-1 DS3 sub-module.  One port, 45 Mbps, BNC connectors

- RASM-E3-1 E3 sub-module. One port, 34 Mbps, BNC connectors

- RASM-155FM-1 ½ and 2 Mbyte ATM sub-module-Multi-mode

- RASM-155-FS-1 ½ and 2 Mbyte ATM sub-module- Single-mode

- RESM-AB-AFD One full duplex AUI port for IBM Nways 8273 Ethernet RouteSwitch 10U

- RESM-AB-FL 10BaseFL Adapter Card for the IBM Nways 8273 Ethernet RouteSwitch 10U

- RESM-AB-T 10BaseT Adapter Card for the IBM Nways 8273 Ethernet RouteSwitch 10U

- RESM-AB-AT 10BaseT RJ-45 and AUI Adapter Card for the IBM Nways 8273 Ethernet RouteSwitch 10U

- RESM-AB-B 10Base2 Adapter Card for the IBM Nways 8273 Ethernet RouteSwitch 10U

- RESM-AB-FL-S  10BaseFL Adapter Card for the IBM Nways 8273 Ethernet RouteSwitch 10U

- RESM-100C-4 4 Port 100BaseT sub-module

- RESM-100C-FD-1 One 100BaseTx  full duplex port sub-module

- RESM-100FM-FD-1 One 100BaseFx full duplex port sub-module (MM)

- RESM-100FS-FD-1 One 100BaseFx full duplex port sub-module (SM)

- RFSM-S-1 FDDI sub-module. One DAS port

- RFSM-M-1 FDDI sub-module Multi-mode

- RFSM-C-4 CDDI sub-module four CDDI ports

- RWSM-S-2 Frame Relay Sub-module.  Two universal external DTE/DCE WAN connectors.

- RWSM-SC-4 Frame Relay Sub-module with compression. Four universal external DTE/DCE WAN connectors.

- The following include modules which contain either HSM or HSM2* boards.

- ASM-155FM-1/2 Multi-mode ATM Module ½ and 2 Mbyte SRAM

- ASM-155FS-1/2  Single Mode ATM Module ½ and 2 Mbyte SRAM

- ASM-155C-1/2 One or two port, 155 Mbps UTP 5, RJ45 connectors ½ Mbyte

- ASM-155C-1/2E One or two port, 155 Mbps UTP 5, RJ45 connectors 2 Mbytes

- ASM-DS3-1/2 ATM ½ Mbyte w/BNC connectors

- ESM-100C-4/8 100BaseT Switching Module

- FSM-M-1/2 FDDI Module Multi-mode

- FSM-C-4/8 CDDI Module

- FSM-M-C FDDI/CDDI Module

- FSM-S-1/2 FDDI Single Mode sub-module

- TSM-C-6 Module (4Mbps or 16 Mbps)

- TSM-F-6 Module (4 or 16 Mbps Rev B4 and above,  16 Mbps only for revisions)

- TSM-CD-6 Module (4 Mbps or 16 Mbps)

- Version 2.0.4 and subsequent releases support HSM2.  See section 10 for additional information.  Version 3.2.6 and previous software versions support HSM.

Software Major Features:

- 2K CAMS

- 802.5 LAN Emulation Client

- Adaptive MAC learning Algorithm

- Advanced Routing including OSPF and RIPII

- ATM Services

- Ethernet LAN Emulation Client

- X-Lane with PVC’s

- RFC 1483 (LLC Encapsulation over ATM)

- RFC 1577 (Classical IP over ATM)

- IBM Nways RouteSwitch point to point bridging

- VLAN Trunking over ATM

- ATM UNI 3.0/3.1

- Nways RouteTracker Manager

- Banyan Vines Translations

- DHCP BootP Relay/Translations

- Distributed CAM Utilization

- Domain Bridging

- Duplicate MAC Support

- Enhanced Diagnostics

- Ethernet RMON (Groups: Ethernet Statistics, History Control, Ethernet History, Alarm and Event)

- Flooding Limits

- Flood Limits Override

- Frame Relay

- Group Mobility

- Hot Swap of like modules

- HRE Layer 3 frame forwarding ASIC, IP, IPX route forwarding

- IBM RI/RO Format interoperability

- IBM Token Ring Support Functional Address

- IEEE 802.10 VLAN Tagging over FDDI

- IP Firewall

- IP Multicast Routing

- IP Routing

- IPX Any-to-Any Switching

- LLC Output Option for Translations

- IPX Client Network Policy refinement

- IPX Raw over FDDI

- IPX RIP/SAP Filtering

- IPX Routing

- LANE Service Module (LSM) (IBM Nways 8274 LAN RouteSwitch Only)

- MPM Port Monitoring

- Multicast Virtual LAN’s

- Optional Token Ring Copy Bit Setting

- Port Forwarding Policy

- Port Mirroring (10BaseT)

- PVCs and SVCs

- Router VLAN Management

- SNMP Trap Support

- Source Route Bridging

- STM-1 Coded in accordance with ITU Ch. 4 of I.432 (Geneva July 1995)

- SVC/PVC Recovery Improvements

- Token Ring Source Route RIF Stripping

- Virtual Rings and Virtual Rings over Trunks (backbones)

- VLAN Trunking over FDDI

- X-Cell

- X-LANE Enhancements

- New Hardware in 3.2 Release*:

- IBM 8277 Nways Ethernet RouteSwitch:

8277-13C  32 port 10BaseT switch, 2 fixed 100BaseTx uplinks with 1K/4K CAM

8277-13Z  32 port 10BaseT switch, 1 fixed 100BaseTx & 1 fixed 100BaseFx uplink with 1K/4K CAM

8277-13F  32 port 10BaseT switch, 2 fixed 100BaseFx uplinks with 1K/4K CAM

8277-23Z  32 port 10BaseT switch, 1 uplink slot for a fast Ethernet or ATM sub-module with 2K/4K CAM

8277-33Z  32 to 96 port 10BaseT switch, 1 uplink slot, 3K/8K CAM

8277-416  16 Port 10/100 BaseT switch, no uplink slot, 1K/4K CAM

8277-524  24-port 10/100 BaseT switch, 1 uplink slot, 3K/8K CAM

RSASM2-155FM-1, OSASM2-155FS-1, OSASM2-155FSH-1  ATM uplink modules

RSASM2-155RFM-1, OSASM2-155RFS-1  ATM uplink modules with physical port redundancy.

RSESM-100FM/FS-2  100BaseFx sub-module, 2-PORT 100 Mbps, multi/single mode, SC connectors

RSESM-100C-4  100 BaseTx sub-module, 4-port, 100 Mbps, RJ45 connectors

IBM Nways 8274 LAN RouteSwitch:

ASM-155FSH-1EW/2EW-4C  ASM wide module, 1 or 2 ports OC-3, long reach single mode fiber with 4K CAM

ASM-CE-155FM-2S2EW-4C  2 E1/2 serial circuit emulation ports + 155 Mbps ATM uplink, multimode fiber, 4K CAM

ASM-CE-155FS-2S2EW-4C  2 T1/2 serial circuit emulation ports + 155 Mbps ATM uplink, single mode fiber, 4K CAM

ASM-CE-155FM-2S2TW-4C  2 E1/2 serial circuit emulation ports + 155 Mbps ATM uplink, multimode fiber, 4K CAM

ASM-CE-155FS-2S2TW-4C  2 T1/2 serial circuit emulation ports + 155 Mbps ATM uplink, single mode fiber, 4K CAM

ASM-CE-DS3-2S2TW-4C  2 T1/2 serial circuit emulation ports + 45 Mbps DS-3 ATM uplink, 4K CAM

ASM-CE-E3-2S2EW-4C  2 E1/2 serial circuit emulation ports + 34 Mbps E3 ATM uplink, 1K CAMCSM-UW  Universal Cell Switching module, 3 adapter board slots

CSM-AB-155FM/FS-2W  2 port 155Mbps OC-3/STM-1 adapter for CSM-U multimode or single mode fiber

CSM-AB-155FSH-2W 2 port OC3 adapter board, long reach single mode fiber for CSM-UW

CSM-AB-155C-2W  2 port 155Mbps OC-3/STM-1 adapter for CSM-U, single mode fiber

CSM-AB-DS1-4W  4 port DS-1 adapter for CSM-U

CSM-AB-DS3-2W  2 port 45Mbps DS-3 adapter for CSM-U

CSM-AB-E1-4W  4 port E1 adapter for CSM-UW (RJ-45).  Standard memory supports 2K VCs/port & 4,096 cell buffers/port.

CSM-AB-E3-2W  2 port 34Mbps E3 adapter for CSM-UW (BNC). Standard memory supports 4K VCs/port & 8,192 cell buffers/port.

CSM-AB-CE-E1/T1-4W  4 port E1 or T1 AL-1 circuit emulation module for X-Cell, adapter for CSM-U

CSM-UW  Universal Cell Switching Module.  3 NNI adapter board slots available

ESM-100-FS-8W-4C  8 100BaseFx (single-mode) switch ports and CAM support for 4096 MAC addresses.  Fast Ethernet Switching Module.

ESM-FM-8W-2C/16W-2C  8 dual/16 dual ST connectors to 10BaseFL hubport or device., multimode fiber and cam support for 2048 MAC addresses.WSM-BRI-SC-1W/2W  1 or 2 serial ports and 1 or 2 ISDN BRI(U and ST interface), with compression

WSM-BRI-SC-1W/2W  1/2 serial port(s) supporting PPP or Frame Relay, 1 BRI (U and S/T) port supporting PPP.

WSM-FT1/FE1-SC-1W  1 serial port and 1 Fractional T1 or E1(integral DSU/CSU) port, with compression

WSM-FT1/FE1-SC-2W  2 serial port and 1 Fractional T1 or E1(integral DSU/CSU) port, with compression

IBM Nways 8273 Ethernet RouteSwitch:

RASM-CE-155FM-2S2E  2 E1/2 serial circuit emulation ports + 155 Mbps ATM uplink, multimode fiber

RASM-CE-155FM-2S2T  2 T1/2 serial circuit emulation ports + 155 Mbps ATM uplink, multimode fiber

RASM-CE-155FS-2S2E  2 E1/2 serial circuit emulation ports + 155 Mbps ATM uplink, single mode fiber

RASM-CE-155FS-2S2T  2 T1/2 serial circuit emulation ports + 155 Mbps ATM uplink, single mode fiber

RASM-CE-DS3-2S2T 2 T1/2 serial circuit emulation ports + 45 Mbps DS-3 ATM uplink

RASM-CE-E3-2S2E  2 E1/2 serial circuit emulation ports + 34 Mbps E3 ATM uplink

- *New hardware must be supported by version 3.2 MPM software. Route-Vision applications must also be version 3.2 or higher to support new hardware.

New Software Major Features in 3.2 Release:

- 802.1Q Trunking on 10/100

- ASM2 Traffic Shaping

- ATM Signaling Performance

- ATM Statistical Enhancements

- Auto-Switch

- Bi-directional Traffic Parameters

- Binding VLANs

- Default bridge mode change

- DHCP VLANs

- Dual Homing on FDDI

- Dynamic CAC

- Dynamic LANE

- E.164 address support

- Group Mobility II

- ICMP Router Discovery (part of Advanced Routing)

- LANE Service Module (LSM) (IBM 8277 Nways Ethernet RouteSwitch & IBM Nways 8273 Ethernet RouteSwitch)

- Multiple Users’ Sessions (Simultaneous Telnet)

- RouteChannel

- Port Mapping

- PPP support

- Soft PVCs

- TR Port Mirroring & Monitoring

- User Authenticated VLANs

- VP Tunnelling

- Virtual port increase to 32

- WAN Routing Enhancements

- IP RIP Filters

- IPX Watchdog Spoofing

- NetWare Serialization Packet Filters

- SPX Keep-alive Spoofing

- Triggered IPX

New software features are only supported in version 3.2.6 IBM Nways 8274 LAN RouteSwitch software and are only supported by Route-Vision Version 3.2.6 applications.

Note:  For ATM interoperability testing results, please contact IBM Technical Support.

# Unsupported in this release (3.2.6):

## IBM 8277 Nways Ethernet RouteSwitch

- ATM Traffic Shaping

- Group Mobility over Source-routing

- ISDN

- NHRP

- Token Ring, Source Route or FDDI encapsulations are not supported on IBM 8277 Nways Ethernet RouteSwitch Router Ports.

## IBM Nways 8274 LAN RouteSwitch

- ISDN

- NHRP

- The VPI specification selection in the PRPADD command is now specified on tunnel creation instead of the route property.

- UDP relay is not supported with Nways RouteTracker Manager VLANs.

## IBM Nways 8273 Ethernet RouteSwitch

- 802.1q Trunking

- Group Mobility over Source-routing

- ISDN

- NHRP

- RouteChannel

- UDP relay is not supported with Nways RouteTracker Manager VLANs.

## SNMP Agent does not support the following features:

- ATM Traffic Shaping

- 802.1q Trunking

- NHRP

- LSM

- LANE QoS

- Ethernet 10/100 (Speed, Modes)

- Token Ring Fiber

- CD6 Port Modes

- Port Mapping

- VP Tunneling

- Port Monitor

# General Instructions and Notes for IBM Nways 8274 LAN RouteSwitch and IBM 8277 Nways Ethernet RouteSwitch.

- If you are upgrading from IBM Nways 8274 LAN RouteSwitch Version 2.0 or later to 3.2.6, download the 3.2.6 software and then reboot the IBM Nways 8274 LAN RouteSwitch – see Chapter 6 of the IBM Nways 8274 LAN RouteSwitch User’s Guide for details on software downloading and configuration.

- IBM Nways 8274 LAN RouteSwitch/IBM 8277 Nways Ethernet RouteSwitch Version 3.2.6 requires a minimum of 16 MB SIMM and 4 MB Flash.  (See Section 16-17)  IBM 8277 Nways Ethernet RouteSwitch 8277-13 and 416 with 1K Cam will work with 8MB SIMM.

- To perform IBM Nways 8274 LAN RouteSwitch (version 3.2.6) installation with floppy diskette(s) which is required on a PC, follow these instructions:

- Using a PC, insert the first floppy, CD to the directory where software installation is to occur and type X:install (X = the drive letter for installation).  Instructions are provided for the remainder of the installation.  After installation of the first two (2) diskettes is completed, copy the files from the third diskette onto the same directory.

Warning:  File names handled by Windows95 32-bit application programs change the first letter to uppercase, which will not be recognized by IBM Nways 8274 LAN RouteSwitch operating system.  All filenames must be lowercase.

- Use ProComm 3.0 or any program that makes the target file names in all lowercases.

- If not, make sure to rename all file names with lowercases before down loading them.

- IBM 8277 Nways Ethernet RouteSwitch/IBM Nways 8274 LAN RouteSwitch/IBM Nways 8273 Ethernet RouteSwitch Passwords

The following are defined users:  diag, admin, user.

The default password for all users is "switch".

- Caution:  DO NOT power down the IBM Nways 8274 LAN RouteSwitch during boot-up process.  Wait for the Login Prompt before powering down.  If the Login prompt does not appear after approx. 6 minutes, then power down and reboot.

- Caution:  DO NOT power down during system file compaction.  Message will be displayed "File System Compaction in progress."  Powering down during file compaction will cause corruption of flash memory.

- When upgrading from version 2.0 or 2.1 software to versions 3.1 or higher, if user has Nways RouteTracker Manager VLANs configured with router ports, user must first upgrade the software, remove those Nways RouteTracker Manager VLANs, reboot the switch and reconfigure those Nways RouteTracker Manager VLANs.

- Route-Vision 3.2.6 is required for managing the new features contained in IBM Nways 8274 LAN RouteSwitch version 3.2.6 software.  See Route-Vision 3.2.6 release notes for more detail.

- When upgrading to 3.2.6, the mpm.cmd file must be set to rifStripping=1 to enable "RIF Stripping",

- If a PC is directly connected to the IBM Nways 8274 LAN RouteSwitch, the IBM Nways 8274 LAN RouteSwitch Spanning Tree will reconfigure when installed or rebooted (this is generally not a problem).  However if a network layer protocol is started at boot time (via the autoexec.bat) of the PC and is dependent upon a connection (such as an IPX connection), then the network layer process will not be able to start until the Spanning Tree process has completed.  In the event that the IPX client is unable to attach to its server the following steps may be used to solve the problem:

- If only 1 client is attached, the port type can be changed to optimize device switching using the modvp command.  This configuration will turn off spanning tree.

- If the system constraints allow, the spanning tree forwarding timer can be changed to a lower value.  This should not be done without careful consideration for the network needs of spanning tree.

- If necessary either the NETX or VLM commands can be re-entered.  In some versions of NETX the driver is loaded regardless of whether a connection is made.  When this occurs the driver must be unloaded then rebooted.  NETX version 3.32 and VLM version 1.20 were used in the IBM testing and do not require an unload between commands.

- In release 3.2.6, the IBM Nways 8274 LAN RouteSwitch supports a maximum of 16,384 MAC addresses instead of the current maximum support of 12,282 MAC addresses.  This means that an 8274-9SLOT, equipped with Network Switching Modules (NSM) of each 2K CAM, each will be able to address all the CAM space in each NSM without reconfiguration.

- A modification to the aging timer for transparent bridging will only take place on newly learned entries.  Any entries learned on the previous timer will time-out on that value. Once they time-out, and the entry is re-learned, the correct aging timer will be used.

- Please see the Hardware Compatibility Matrix in section 36 to verify your IBM Nways 8274 LAN RouteSwitch platform is compatible with IBM Nways 8274 LAN RouteSwitch software version 3.2.6.

- When changing the encapsulation type (e.g., 802.3 to Ethernet II), you must reset the physical port (either by pulling the physical cable out and pushing it back in or using the User Interface commands to disable and then enable port).

- When downloading files using ftp, FAT filenames are changed to uppercase which makes files unreadable.  The user must change filenames to lowercase.  The following versions do not display this problem: WFWG of ftp version 32b,  ChameleonNFS, and SUN’s PCNFS.

- Upon initial start-up (or after removal of mpm.cnf file) of a system with large numbers of physical ports, primarily the 9 slot chassis, the message:  “File system compaction in progress…..” may be seen.  This is not a problem, unless it repeats indefinitely.  This message may be seen from time to time if network management functions are occurring.  One time is not a problem, immediate repetitions are a problem and if they occur contact IBM Technical support.

- Each group within the IBM Nways 8274 LAN RouteSwitch executes the IEEE 802.1d spanning tree algorithm.  Spanning tree defines its bridge ID as the lowest MAC address of all the virtual ports attached to the given Group.  If a virtual port is moved from “Group A” to “Group B”, then a spanning tree reconfiguration may result.  If ‘Group A’s” bridge ID is the MAC address of that virtual port, then its bridge ID must change.  This will require a spanning tree reconfiguration.  If the virtual port added to “Group B” has a MAC address lower than all other virtual ports attached to “Group B”, a spanning tree reconfiguration must occur.  If the user performs such port moves via a Telnet session, they will experience a temporary lockout (up to 30 seconds) in their session, if spanning tree reconfigures on the Group supporting their Telnet session.

- Translations from source routing to transparent bridging are not supported (e.g. Token Ring source routing to Ethernet transparent).

- Use of the “break” key on a terminal/PC will change the baud rate of the serial port connected to the IBM Nways 8274 LAN RouteSwitch.  The baud rate will cycle to the next highest baud rate. When it gets to 38,400, it will cycle back to 1200 baud. The cycle is as follows:  9600  19200  38400  1200  9600.

- When a destination node is moved to a different location in Nways RouteTracker Manager VLAN, the source does not learn the new location until the destination node sends a packet.

- There is no limit on the number of times a TSM-CD-6 port can detect a difference in ring speed and reset itself.  This may be a problem when more than one port in a ring has ring speed auto detect capability and the ports are configured at different speeds.  Under these circumstances, it is important to know what the ring speed should be and to configure it explicitly.  The default speed is 16 Mbps.  In the future, a limit will be placed on the number of times a TSM-CD-6 port will detect ring speed and reset itself.

- If a group is declared to be a Default Group for Frame Relay Virtual Circuit creation, then it cannot be deleted, even if it is not currently being used.  Rmgp will display the message “Cannot delete”.  Group has active entries.”   Work-around:  “frm slot/port” to remove the group as a Default and then rmgp. (PR 4549).

- Vrouter port does not support FDDI-raw frame – when vrouter ports are configured as Ethernet 802.3 and FDDI port as FDDI-raw, FDDI-raw packets received on the other side of router are not routed.

- SNA traffic on Token Ring and Nways RouteTracker Manager VLANs: If the user wants to configure Nways RouteTracker Manager vlans for SNA traffic based on DSAP/SSAP protocol rules, it is recommended that the user also configures port rules for ports that connect the IBM Nways 8274 LAN RouteSwitch to the server.  The reason being that the servers normally will not generate traffic unless a client talks to it.  But before the client initiates the SNA session, it sends out the “Duplicate MAC test” frames which carry the SNA dsap/ssap values.  As a result, the client gets assigned to the SNA vlan and subsequent SNA traffic will not go out on the server port as that port is not still part of the SNA vlan.

- In order to use Autoencaps feature user must create a dummy Nways RouteTracker Manager vlan.

- Autoencapsulation will not work properly if IPX server is down when IPX client boots up.

- This version of software supports ATM SVCs on up to 10 ASM ports per switch.  Up to 16 LANE LECS are also supported per port and a maximum of 64 LECS per IBM Nways 8274 LAN RouteSwitch has been tested.  Only one service (either Elan or PTOP) is supported per group on an IBM 8277 Nways Ethernet RouteSwitch ATM uplink.

- Multicast Routing is not supported over ATM CIP connections in Version 3.0 initial release.  If this option is selected, the traffic will not be sent to these connections.  Multicast routing is not supported over frame relay connections that have multiple virtual circuits.  In order to use Multicast routing over a frame relay connection only one virtual circuit for the connection must be made.

- Caution should be used when issuing the mrrtab command in the user interface.  There is a known problem when there are large quantities of routes defined in the system (>3000 routes).  Use the mrrnum command to display the number of known routes in the system.  Use of the mrrtab with large quantities of routes may cause the switch to hang, or crash due to lack of memory.

- For IBM Nways 8273 Ethernet RouteSwitch10U with RESM-AB-AFD (full duplex): Source learning will not flush the MAC table for the port when the cable is moved.  This occurs because the link never goes down on these full duplex ports.  This will cause traffic to continue to go to the old port and not flood.  The MAC will not move correctly to the new port. Wait until the source learning timer expires (normally 5 minutes) after removing the cable before reinstalling to the new port.

- The ATM Forum IISP Standard does not support a dynamic routing protocol for ATM NSAP addresses. Consequently, when connecting X-cells together via IISP in a LANE environment, The path to the LANE well known address  (wka = 47.00.79.00.00.00.00.00.00.00.00.00.00.00.A0.3E.00.00.01.00) needs to be statically configured on all X-cell’s not directly connected to the IBM Nways RouteSwitchMSS (or any LANE server).  The X-cell’s which have the directly connected server will learn the correct path via the ILMI address registration process.  You may abbreviate this to a shorter prefix and if a LANE Server becomes available, PNNI will favor that over the shorter prefix.

Additionally, addresses required for all ATM server connectivity must be statically configured.  This includes any LES/BUS  requirements.   The “prpadd” and “pradd” are used to add these routes.

Note: PNNI does not require that these addresses be added statically

- IBM Fiber RI/RO Support:

When connecting to the Ring-In or Ring-Out ports of an IBM token-ring concentrator (8230, 8260, etc.) using the TSM-F-6, the ports of the TSM-F-6 should be configured for IBM RO operation.   In addition, if the ports are configured to support either source-routing or source-route transparent (SRT) operation, the TSM should be connected to Ring-In or Ring-out of the concentrator, but not to both.

- All IBM mibs need to be reloaded for the 3.2.6 version release.

- In a topology where a loop exists, do not configure static routes on each of the switches (in the loop) such that each switch points to another switch (in the same loop).

- There is a label “use_mrouted” which must be set to a non-zero value for mrouted to load and run.  Originally the presence of the file “mrd.img” was sufficient to activate multicast routing functionality.  Now the file must be present, and the “use_mrouted” flag must be set.

This flag can be set by inserting the line: "use mrouted=1" into the mpm command file before the line which invokes cmInit.

- When changing your UNI port to PNNI or IISP and vice versa, you must change PNNI port to UNI and reboot the switch to have the changes become effective.

- Spanning Tree must be OFF in order for the Enhanced Diagnostics program to function properly.  This applies to version 3.0.2 and later.  The switch defaults to Spanning Tree ON.  Set it to OFF through stc command, save the change, and power recycle.  If the problem persists, remove cfg/cnf files, and repeat the steps turning the Spanning Tree off.

- There is a restriction on the trunking protocol which does not allow specific routed frames to get to FDDI trunks or ATM trunks when coming from TR LANE side.

- There is a flag that can be placed in the mpm.cmd file for adjusting the MTU (Maximum Transmission Unit) of our OSPF implementation upwards from the default of 4K (4544 bytes). The syntax is as follows; gated_recv_mtu=x where x equals a value greater than 4544. This line MUST be placed after the line containing cmInit and the switch must then be rebooted in order for the change to be effective. A recommended size would be 200 bytes greater than the maximum frame size expected by OSPF. This flag simply reserves buffer space, so care should be exercised not to use a value too large as wasted buffer space (memory) will be consumed.

- Systems using the Advanced IP Routing software on media types with MTUs greater than 4K (Token Ring, ATM, etc) must set the “gated_recv_mtu” flag in the mpm.cmd file to indicate the MTU.  The syntax is “gated_recv_mtu = X” where X is the MTU size in bytes.  This flag MUST go AFTER the “cmInit” call in mpm.cmd.  It is advisable to add 20 or 30 extra bytes to the MTU size so that the receive buffers are guaranteed to be large enough.  There are no restrictions on the values of this parameter.

- When making changes in port mapping menu, user must save and quit before changes are effective.

- For the IBM 8277 Nways Ethernet RouteSwitch 8277-13 and IBM 8277 Nways Ethernet RouteSwitch 416, the Port Monitoring feature requires 32 Mbytes of DRAM.  Also, it is not recommended to use Port Monitoring on high bandwidth ports, such as a backbone uplink.

- Hot swap of uplink interfaces for 8277-33X and 8274-524 are not supported

- In a redundant MPM configuration, do not remove the primary MPM.  Do a renounce command first.

- 802.1q not supported over RouteChannel.

- The very first packet on the switch with RouteChannel is sent out on all RouteChannel ports.

- Do not use port monitor on any port used for high backbone traffic.  Port Monitoring should only be used for monitoring stations. This could freeze telnet sessions under high traffic.

- Firewall Status between switch and NT station get out of sync.

- Available MAC addresses could run out during trunking setup that leads to failure of creating services with the error message “pmPortCreate(): couldn't allocate a mac address".  Ensure there are enough base MAC addressed assigned.

- All parameters in mpm.cmd have to be located before cmInit unless explicitly specified.

- Spanning tree on the IBM 8277 Nways Ethernet RouteSwitch is supported only if the ports are in "bridged mode".  Spanning tree will not work if the ports are in "AutoSW" mode.

- Statistics on 802.1q ports is not recorded on IBM 8277 Nways Ethernet RouteSwitches

- To make sure that the proper switch negotiates to become the spanning tree root bridge always set the priority of this switch accordingly.  Do not rely on the MAC address to determine which switch becomes the root bridge.

- Nways RouteTracker Manager VLANs and the multicast router code

Due to the nature of Nways RouteTracker Manager VLANs and the multicast router code, we cannot support multicast routing with Nways RouteTracker Manager VLANs.  Since a port can be located in multiple VLANs, it is impossible for the multicast router to be certain of the source network of the traffic stream.  This restriction will be lifted with the release of IP Multicast Switching which is currently scheduled for release 4.0.

- Maximum number of groups supported for ATM trunking is 31.

- Can not use NetWare login with authentication using XVSS client.

- XVSS client does not work over Token Ring.

# Group Mobility

- This instruction only applies if user is currently using group-mobility in 3.1 and user wants to upgrade to 3.2 using group-mobility.

Follow these instructions:

Remove the vlans that have been configured for group-mobility in 3.1.

- Take "group_mobility=1" out of the mpm.cmd file.

When loading 3.2:

Create the mobile groups desired and reconfigure the policies for those groups.

Perform the gmcfg command to enable group-mobility on the switch.

- It is not required to do "rm mpm.cnf" on the UI shell when loading 3.2.

# IBM 8277 Nways Ethernet RouteSwitch Reboot Routine

ERROR MESSAGE:   cmDoStateP4Done(3): Download failed

This most likely cause of this error message is a missing image (code) file needed for IBM 8277 Nways Ethernet RouteSwitch operation.  The files which may be missing from the flash are:

em4.img    All IBM 8277 Nways Ethernet RouteSwitches need this file

- ae4.img    IBM 8277 Nways Ethernet RouteSwitches with ATM uplink need this file.

# Binding VLANs

The IBM Nways 8274 LAN RouteSwitch and IBM 8277 Nways Ethernet RouteSwitch support a feature called Binding VLANs.  Binding VLANs allow the user to define policies for mapping a

specific traffic from a specific port to a specified VLAN.

Network Administrators are always looking for the right balance of security for their networks. For some, a firewall at their Internet access

point is sufficient. Others want access lists or additional firewalls within their Intranet routing functions.  However in some networks,

network administrators want to go one step further and are seeking to restrict user access by binding (or configuring) a port with one or

more specifically allowed device addresses (i.e. MAC, IP address). This binding enables an additional level of security, because only

specifically defined devices have the privilege of access to the network through a particular switch port. Users are configured for access

through one or more ports and optionally across a set of VLANs.

IBM calls this feature Binding VLANs. More often than not, users need to be in more than one VLAN. This is simply accomplished by

configuring the user as a member of more than one VLAN.

A device (or user) can be identified by: 1) Port, MAC and IP address; or 2) Port, MAC address and protocol type; or 3) Port, and protocol.

In fact, when this type of policy is used, one and more devices can be attached to the port. As well as each device can be a member of

more than one VLAN, if desired. When traffic is received and the originating device does not match the any of the binding policies for the

port, the traffic will be discarded and network management is notified.

Some users require that a single device be a member of multiple VLANs.  This is possible by defining a binding VLAN policy for each

VLAN in which a device wants to be a member.

When using the third option, port and protocol, the policy is in effect creating a filter applied to traffic of the specified protocol type from the

incoming port.  The filtered traffic can be grouped into a VLAN that can be propagated or even discarded.

# Content Addressable Memory (CAM): Advanced Uses (IBM Nways 8274 LAN RouteSwitch)

- This overview assumes the reader has a basic knowledge of the CAM's function in an IBM Nways 8274 LAN RouteSwitch. It is intended to address advanced uses of CAM, such as restrictions, configuration, and the page architecture of CAM. It also provides several examples to illustrate how CAM is allocated to switching modules.

- The Goal of CAM

- Without any user configuration, every switching module--regardless of its number of CAMs--should come up upon a reset or after a reboot.  There are restrictions to CAM usage. The main restriction is that only 16K of CAM is available in the entire switch due to limited VRAM memory. However, when the number of CAMs exceeds the maximum allowed, the user should be able to configure the proper CAM usage on each board (via the camcfg command) to get as close to the maximum limitation as possible.

- CAM's Page Architecture

- Structurally, the switch's CAM consists of 16 x 1K pages of memory numbered from 0 to 15. A page id identifies each CAM page and is also used to differentiate the CAM allocated to each slot.

- Switching modules with 2K or 4K of CAM must start CAM allocation on an even page (i.e., page 0, 2, 4, etc,). In addition, if multiple CAM pages are needed for a 2K or 4K module, then those pages must be contiguous.

- Boards with 1K of CAM are first initialized on page 15, then page 14, then page 13, etc. Boards with 2K or 4K of CAM are first initialized on Page 0. So 1K CAM boards start at the bottom of the CAM page structure and 2K and 4K boards start at the top of the CAM page structure. A 4K CAM module in slot 2 may use Page 0 (an even page) through Page 3, and a 1K CAM module in slot 3 may use Page 15. Modules cannot share MAC entries because each MAC entry is unique in the switch.

- The MPM and Page 0

- Since the CAM database is only used for MAC learning, only 128 entries are needed by the MPM to store router port MACs. Page ID 0 is always used by the MPM CAM. The extra 896 entries in Page 0 that the MPM does not need can be used by modules in other slots. Any board using Page 0 may have 128 entries deducted from its CAM usage because those entries are used by the MPM.

- Since the MPM uses 128 entries of the total 16K available in the switch, the maximum number of entries that can be learned by switching modules is actually 16226 instead of 16786.

- CAM Initialization

- A successful initialization of CAM is important. If CAM initialization on a board fails, then that board will not come up properly. With no user-configuration of CAM, the switch software tries to initialize and set up all CAM available on each module. As long as there is available memory, this is possible.

- If there is not enough memory for the entire CAM or the configured amount of CAM on a module, the switch will attempt to find room for 64 contiguous CAM entries.  This minimum amount of CAM is enough to initialize the CAM and allow the board to come up.

- If 64 contiguous entries are not available, switch software will take 64 entries way from another board, which will usually be the board in the last slot. In this case, the switch will reinitialize the CAM so the board can come up. A warning message similar to the following will be displayed to notify the user during the reboot or reset:

- slot x: Cam usage was reduced to y so it can be properly initialized!

- The user can see the CAM use by each module through the camstat UI command or through the howCamPage dshell command.

During a reboot, the switch does not know the number of CAMs on a board until the Chassis Manager software initializes the board. Also, boards do not necessarily come up in sequential order (e.g., slot 8 may come up before slot 7). Therefore, CAM page assignments may be altered after hot swapping multiple boards, and hot swapping may also affect the CAM usage by each board.

- Restrictions for 2K and 4K CAM Boards

- The restrictions for 2K and 4K CAM boards (i.e., CAMs must start on an even page and those pages must be contiguous) can mean that every CAM entry may not be used.

- The camcfg command

- The camcfg command is used only when the total number CAMs in the IBM Nways 8274 LAN RouteSwitch exceeds the limits. This command allows you to configure the amount of CAM you want a board to use regardless of how much physical CAM is present on the board. You must reboot the IBM Nways 8274 LAN RouteSwitch before a new CAM configuration takes effect. The camcfg command is described in Chapter 12 of the IBM Nways 8274 LAN RouteSwitch User's Manual.

- An Example using the camstat Command

- The camstat command allows you to view the number of CAMs and the configured CAM usage for each slot.  The following is a sample of a camstat display:

Slot  	# of CAMs  	Cfg Usage		Adj Usage 	Max Avail 	Actual Usage

----  	---------  		--------- 		--------- 		--------- 		------------

2       	1          	 	0       		960     		954        		100

3      	2        		1500      		1536    		1503          	0

4       	4           		0      		4096    		4085         		0

5       	4        		3000      		3008    		2997          	0

6       	4        		2800      		2816    		2807          	0

7       	1          		64        		64      		47         		0

8       	4        		1500        		64      		51         		0

9       	4          		0        		64      		59          		0

- Slot  - The physical slot number where the module is located.

- # of CAMs - The actual size of the CAM installed on this module.

- Cfg Usage - The amount of MAC entries configured for this slot through the camcfg command. A value of '0' indicates no configuration has been performed and the entire CAM is used.

Adj Usage - The number of entries actually allocated for this CAM.  The number of MAC entries is always rounded to a multiple of 64. If Page 0 is being used, then 128 entries may be deducted from the amount. These 128 entries are used by the MPM.

- Max Avail - Some entries in the CAM are reserved for internal use. This column shows the number of entries available for MAC learning.

- Actual usage - The total number of MACs learned on this slot.

- Some highlights in the example:

- Slot 2 has 1K of CAM with no configuration. Only 960 entries are allocated because 128 entries are being deducted for use by the MPM. Only 954 entries are available for MAC learning and 100 MACs have already been learned.

- Slot 3 has 2K of CAM and is configured to use 1500 entries. The Adj Usage column shows that the number of CAM entries used is rounded up to 1536.

- Slot 8 has only 1K of CAM but is configured to use 2500 entries. The software will not allocate more entries than the size of an actual physical CAM. In addition, the Adj Usage is reduced to only 64 entries because there was not enough memory to use all available CAM on this board.

- Slot 9 has 4K of CAM with no configuration. The Adj Usage is also reduced to only 64 entries because there was not enough memory.

# Duplicate MAC Support on IBM Nways 8274 LAN RouteSwitch

In some network configurations, the IBM Nways 8274 LAN RouteSwitch must interoperate with servers and routers that carry the same MAC address on multiple interfaces.   Switch software supports these duplicate MAC addresses. Chapter 11 of your IBM Nways 8274 LAN RouteSwitch Users' Manual describes this feature.   Support for duplicate MACs can be limited by the lack of internal VRAM memory and CAM space.  However, since there are usually a small number of  servers or routers in a given network, this limitation usually is not a  problem. Therefore, it is not recommended that you bridge multiple VLAN Groups together as this causes most MAC addresses learned to become duplicate entries.

# Authenticated VLANs

- In order to use Authenticated VLANs, you must purchase the Authenticated Management Console: AMC-CP-1, AMC-CP-5, AMC-CP-U.

## Server

- The authentication server or AMC runs on an NT server or NT workstation 4.0.  The AMC software is produced by Check Point Software Technologies Ltd.

## AMC Server Setup

- Start the installation of the AMC.  You will be asked about licenses.  The license that is already installed may have already expired.

- You can order a 30-day evaluation license from IBM (see price list) or the actual product which will include a permanent license for the AMC.  In order to obtain a 30-day evaluation license, use the key provided by IBM and log on to Check Point’s license web page.  You must select the option to get a license for the AMC.   The license you receive must include the word ‘vlan’ in the features.  You cannot use a FireWall-1 evaluation license for the AMC. A couple of licensing areas may be confusion but license the AMC as a management server. When asked about FireWall-1 version, just say “3.0”.

- Add the new license.  Then add an administrator.  Next, you will be asked to add GUI clients.   GUI clients are not required if you only manage the AMC from the NT system on which the AMC is installed.  If you wish to add a GUI client, add the name of the NT system on which you installed the GUI client portion of the AMC.  Verify that the name of the NT system running the GUI client software exists in the host file of the NT system running the AMC.

- You are asked to add Remote Switches.  A remote switch is an authentication agent or in other words, the switch running the user authentication software.  Add each switch (using the name that is configured in the NT system's HOSTS file for that switch) that will be in the user authentication configuration.  Upon adding the remote switch, you will be prompted for a password.  This password is the S/KEY password you will be prompted for when you configure the user authentication software on the switch.  After adding a remote switch in the AMC configuration for each of the switches involved in the user authentication configuration, the installation is complete.

- The following information is in the Authentication Management Console Administrator Guide: User Authentication VLAN Server Software.

## Authentication Server Configuration

You will find the Authentication Management console in the Program Menu.  Select Policy Editor.  You will be prompted for a user name, password and AMC Server. The AMC Server name must match the NT system's configured name which can be found in Control Panel->Network under the identification tab.  The user name and password to enter are the user name and password you entered for the administrator during the server installation.

You must configure network objects and users.  The network objects consist of a workstation object for the NT system on which the authentication server is running and a switch object for each of the switches involved in the user authentication  configuration.  You must also configure a workstation object for any system running as a RADIUS or Defender server.

Select Manage->Network Objects.  From New, select Workstation.  Enter the name of the NT system on which the server is running.  Now press the Get Address button (this only works if you have the IP address in the hosts file - found in winnt/system32/drivers/etc) or enter the IP address.  It is strongly recommended that you enter each of the systems in the HOSTS file.  Click on the 'DataBase Manager' check box in the lower left corner.  Then click on OK.

To configure a network object for the systems hosting a RADIUS or Defender server, follow the same directions as above but clicking on 'DataBase Manager' is unnecessary.  The only thing you need to configure is the name and address.

To configure a network object for each switch, from Manage->Network Objects, press the 'New' button and select switch.  Enter the switch name and click on 'Get Address' or enter the address.  Then click on the 'Interfaces' tab.  Click on 'SNMP Get' to get all the interfaces from the switch.  You must already have all the authenticated groups and the client group configured on the switch at this point so the network object on the server will match the configuration on the switch.  Click on the SNMP tab and click on 'Get' to retrieve the information about the switch from the switch. Click on the 'VLANs" tab and click on 'SNMP Get' to retrieve information about all of the user authenticated groups on the switch.  You should see every network configured for user authentication on the switch displayed in this list.  Executing the 'vag' command on the switch produces the same list of information.  Continue this until you have configured a switch network object for each switch.

To configure a RADIUS or Defender server network object, select Manage->Servers.  Click on the 'New' button and select the type of server you wish to configure.  Enter all of the related information.  The only tricky thing here is to know that the Host configured for the server is the system on which the RADIUS or Defender server resides and not the system the AMC is running on.

Now you can configure users.  Select 'Users' from the Manage menu.  You will see one object in the list.  This 'Default' is the default  template.  To configure each user, select one of the templates available below 'Group' and 'Template' when you click on the 'New' button.  If you do not create other defaults, you will click on the only one available, 'Default'.  Enter the user name.  Select the Authentication tab.  Many authentication options are available.  Refer to the documentation or contact us for details on the various authentication mechanisms.  Under the 'Time' tab, you can restrict the time the user is able to authenticate.  Be aware that this does not restrict the time the user remains authenticated but only restricts the actual time the user can connect and attempt authentication.  Next, select the 'VLANs' tab to select the authenticated VLAN the user will be a member of.  From the list of VLANs, select the network and click OK to finish configuring the user.

Finally, install the database.  This can be done from the user network object configuration dialog or from the main menu on the top of the main Policy Editor dialog.  This installs all of the changes in the database on the authentication server.  This must be done after any changes are made to incorporate the new changes into the daemon running under NT.

## Authentication Agent Setup and Configuration

You should already have all of the groups/VLANs configured for authentication.  Verify that group mobility is enabled on the switch by running the gmcfg command. Now start the authentication agent on the switch.  To do so, execute the 'fwconfig' command on the switch.  (The fwd.img file must already be on the switch for this command to work.)  Executing this command, set the mode to 'authentication only', enable the  state, enter the primary management station address (the IP address of the NT system running the AMC), enter the S/KEY password (the password entered for the corresponding remote switch either during the AMC installation or from the Configuration dialog), and the time zone.

After the command finishes, you will see a message indicating that the loadable modules have been successfully loaded and the corresponding tasks successfully spawned.  The message looks like

fwdSpawn done! fwdTaskId = 0x481BB1F8

vauthdSpawn done! vauthdTaskId = 0x481A6D40

Executing the fwconfig command again and choosing to view the existing configuration you will see the state of the connection to the server listed as "The state of the connection to this manager is:”.  It should show “CONNECTED”.  (If “NOT CONNECTED” displayed, see TROUBLESHOOTING below for help.)

Now the agent is configured, running and connected to the authentication server.

## Authenticated VLAN Client

The client software can be installed on Windows 95 or NT.  Before installing, verify that the DLC protocol is installed.  When using DLC on Windows 95, you must obtain an updated DLC from Microsoft.  The file is called MSDLC32.EXE and can be obtained on ftp.microsoft.com under the Softlib/Mslfiles directory.

## Troubleshooting

- Be aware that though the image fwd.img is named the same for both the IBM 8277 Nways Ethernet RouteSwitch and the IBM Nways 8274 LAN RouteSwitch platform, the files are different.  Make sure you have the correct images.

2.	If SNMP Get button does not work during the network object configuration for the switch, the NT system running the AMC cannot see the switch.  Verify that the correct IP address is configured for the switch in the HOSTS file and verify that you can ping the switch from the NT system running the AMC.

3.	If the switch continues to report the log authentication failed, you are experiencing a problem with S/KEY password negotiation between the authentication agent and the server.  This occurs when the authentication server (AMC) and the switch cannot sync up their connection using the configured S/KEY password.  Go into the Configuration dialog and enter a new password for the remote modules corresponding to each of the switches.  Verify that each of the switch names used in the configuration of each Remote Switch and for each Switch Network Object is the same as the name that is found in the HOSTS file on the NT system running the AMC.  Then reboot the server.   On the switch, run the 'fwconfig' command again and change the S/KEY password to the new one just set on the AMC.  Occasionally, it takes a couple of tries, but this will enable the AMC and the switch to sync up the S/KEY passwords.

- If telnet authentication does not work at all, make sure your telnet port is configured as a port accepting telnet authentication requests (using the avlports and avlsports commands).  Verify that you are telnetting to the correct address and port.  For example, your authenticated network is group 2 configured with an IP router port of  10.0.0.100 in the 10.0.0.0 network.  The client is configured with the address of 10.0.0.1 and is attached to port 2/12 that is configured as a port in group 3, the client group with no IP router port.  The port 2/12 is configured as a telnet authentication port and is displayed when I execute the 'avlsports' command.  The default telnet authentication address is 10.0.0.253. You would telnet from your client to 10.0.0.253, port 259.

- Token Ring clients currently do not work with the MAC client.  The telnet client works over telnet, but the MAC client only works on Ethernet.

- The clients are able to authenticate correctly and seem to function correctly for a short period of time but do not remain in the authenticated group.  The unintentional moving of clients out of the authenticated group can be caused by two things: one, the regular timeout was reached, or two, group mobility is not enabled on the switch.  Use the gmcfg command.

7.	Under RADIUS server configuration, the version pull-down menu only lists RADIUS version 1.0.  This does not imply that only version 1.0 works.  Check Point has successfully tested with many versions of RADIUS from various vendors with no problems.  The AMC functions with RADIUS challenge and response methods implemented by various RADIUS server vendors including RACAL.  See Check Point’s web page for RADIUS servers tested and supported.

8.	Using the 'mag' command you can add a port binding rule to an authenticated group.  You then can add, modify or remove port binding rules from the group using the 'modatvl' command.  You cannot use the 'mag' command again to add a new port binding rule to the same group, you must use 'modatvl'.  The 'mag' command simply allows you to add the first port binding rule to an authenticated group that previously had no port binding rules.

- When starting the Policy Editor, you cannot connect to the server.  After the timeout, it says that it cannot connect to the server yet in the services dialog, it shows that the firewall daemon is running.  If you see this, verify that the name you entered for the AMC server matches the name of your NT system.

10.	You can authenticate, but you still do not belong to the authenticated group you are configured into.  Double check that you used the correct license.  Licenses obtained from the Check Point web page are only usable on the AMC if the features of the license include 'vlanl', 'vlan5' or vlans'.  If you use a Firewall-1 license, the AMC will appear to work but will not grant the user the actual authentication into an authenticated group configured on the switch.

## Helpful Hints and other Notable Information

- Use the 'macinfo' command to verify that the MAC address of the client belongs to the correct group.

2. 	Turning off Spanning Tree on the client ports eliminates the wait for the port to become active.  Setting the port to Optimized will eliminate this delay BUT be aware that setting the port to optimized will eliminate the ability to time the MAC addresses out.  In order to maintain the timeout on the ports, spanning tree must be running on the port.   If timeouts are required and you do not want to run spanning tree, contact customer support for details on how to configure timeouts on optimized ports.

- When creating a new group, all currently authenticated clients will be removed from their authenticated group and put back in the group their port is configured in.  When creating or modifying a group, the groups are flushed and rules are relearned.  This causes all of the authenticated clients to be flushed from their authenticated groups.

- Changing group configuration on the switch causes all of the groups to be flushed.  The result is that all authenticated clients are flushed back to their original groups requiring re-authentication.  Always remember to update the switches configuration on the AMC when changes in group configuration have been made.

- Make sure DNS is working correctly on the NT system running the AMC.  If NT tries to use DNS and cannot reach the DNS server, the installation will take a very long time to finish.  Disabling DNS completely during installation will allow the installation to proceed much faster.

- Verify that the NT system's name and the name used in the AMC's configuration for the NT system (the workstation running the AMC) match.  These two names must be the same and the name must be in the hosts file or the Policy Editor application gets confused.

# IBM Support for Functional Address (IBM Spanning Tree)

- The user can enable on a per Group basis the IBM spanning tree algorithm.  This capability is provided by one of the menu items under the ‘stc [Group ID]’ command.  When the user selects IBM spanning tree, the IBM Nways 8274 LAN RouteSwitch automatically sets the default values for maximum age, forward delay, and hello time (6, 4, and 2 seconds, respectively) as seen in the subsequent menu items.  Unlike the IEEE spanning tree algorithm, in the IBM spanning tree algorithm these values are fixed.  However, for consistency in menu displays, IBM chose to not only display these values as in the IEEE spanning tree case but, in addition, enable the user to change them.  In fact, the user should not change these values.

- The default spanning tree algorithm for each Group is IEEE.  If the user wishes to change spanning tree parameters on a per Group basis through use of the ‘stc [Group ID]’ command, at least 1 virtual port needs to be attached to the Group.  An error message is displayed to this effect if the user selects the ‘stc [Group ID]’ command and there are no virtual ports attached to the specified Group.

- When the user selects IBM spanning tree, additional default settings are carried out as follows:

- all virtual ports attached to the Group in question whose physical port is either 4Mb or 16MB token ring are initialized to use Functional rather than Group addresses;

- all virtual ports attached to the Group in question whose physical port is neither 4Mb nor 16Mb are set to manual forwarding mode; and,

- as additional virtual ports are attached to the given Group, rules a) and b) are applied.

Virtual ports that are in manual forwarding state means that they are not participating in any flavor of spanning tree (neither IEEE nor IBM).  Since this state occurs automatically as a result of rule b), IBM decided to forward any IEEE spanning tree frame received on such a port to all other virtual ports in the Group that are in manual forwarding state.  This is to prevent a loop in the topology as a result of applying rule b) mechanically.

- For a Group configured for IBM spanning tree, the user can override the default settings at his or her discretion.  However, as a general rule, overriding the manual forwarding state to enable IEEE spanning tree on that port only is not recommended.  The user is now running a “hybrid” of IBM and IEEE spanning tree within the same Group.  This may have some unintended consequences, and, although not forbidden, is not recommended.

- It appears that IBM SRT bridges send on token ring an IEEE-style BPDU with an STE RIF (0xC200).  We do not support this mode of operation.  The IBM Nways 8274 LAN RouteSwitch will neither process nor forward these frames.

- IBM Spanning Tree uses the following 48-bit Functional Address (FA) given in canonical form to transmit BPDUs: hexadecimal 0300 0000 8000.  IBM has discovered that IBM clients and servers are permitted to use this address to transmit data.

- With the current architecture of the IBM Nways 8274 LAN RouteSwitch, we cannot support the same FA for data and Spanning Tree.  The FA for IBM Spanning Tree is programmed into our MPM CAM; therefore, all data frames with this FA are claimed by the MPM!  This prevents the data from passing through our switch. However, we provide the following workarounds:

- There exists an ‘mpm.cmd’ settable global variable ‘faBpGrpDisable’ such that when set to 1, Spanning Tree will not program the FA into the MPM CAM.  This works for those environments that are not running IBM Spanning Tree.  The setting of the variable ‘faBpGrpDisable’ should be placed BEFORE the ‘cmInit’ command in file mpm.cmd.

- There exists an ‘mpm.cmd’ settable global variable ‘faBpGrpOverride’ which allows the user to set the lower 32-bits of the FA to some value other than hexadecimal 0000 8000 which is the lower 32 bits of the IBM Spanning Tree FA.  The upper 16 bits are always hexadecimal 0300.  This works for those environments whose applications use FA hexadecimal 0300 0000 8000, who need IBM Spanning Tree, and whose switches are all IBM or third-party bridges / switches whereby their IBM Spanning Tree FA is configurable.  The setting of the variable ‘faBpGrpOverride’ should be placed BEFORE the ‘cmInit’ command in file mpm.cmd.

# Memory Requirements for IBM Nways 8274 LAN RouteSwitch, IBM 8277 Nways Ethernet RouteSwitch and IBM Nways 8273 Ethernet RouteSwitch

Version 3.2.6 requires 16 Mbytes of DRAM and 4 Mbytes of Flash memory. Configuration information is stored in FLASH in addition to the compressed file images.  See the following table for the flash memory required for each module.

| Filename* | Image Size (Bytes) | Module Type |
| --- | --- | --- |
| mpm.img | 1,595,201 | MPM, MPM II or MPM-1G image. |
| mpm.cmd | 18 | MPM, MPM II or MPM-1G command file containing system commands for booting. |
| mpm.cfg | 1k | MPM, MPM II or MPM-1G configuration file fixes at 1K to allow the chassis bootstrap to initialize. |
| mpm.cnf | 32k minimum- 512k maximum | MPM, MPM II, or MPM-1G configuration file (size indicated is the default (or minimum) size. As configurations are added, the size will increase. |
| tmpmpm.cnf | 32k minimum- 512k maximum | Temporary shadow file of the MPM configuration file during expansion. It remains the same size as the mpm.cnf. |
| diag.img | 174,537 | Off-line Enhanced Diagnostics image for the MPM. |
| dni.img | 17,936 | Off-line Enhanced Diagnostics image for the NI. |
| gated.img | 501,406 | Advanced Routing – Dynamically loadable GateD image for OSPF, and RIP II. |
| isdn.img | 201,718 | Dynamically loadable module which contains the ISDN signaling code, and the WSM-BRI daughtercard code. (Daughtercard code takes up flash space only) Needed only if WSM-BRI is present. |
| mrd.img | 66,296 | Advanced Routing – Dynamically loadable image for Multicast Routing function. |
| t1e1drv.img | 74,566 | Image for the following switching modules: CSM-AB-E1, CSM-AB-CE-E1, CSM-AB-CE-T1, WSM-FT1, WSM-FE1. |
| asm.img | 812,690 | ASM-155-F, ASM-155-C, ASM-DS3, ASM-E3, ASM2-155-F switching module loadable image. Includes MPM code to support modules, signaling stack and ATM Services (see Section 15). |
| asmce.img | 46,288 | Image for the following switching modules: ASM-CE |
| asmcedrv.img | 152,679 | Image for the following switching modules: ASM-CE, ASM2-CE, CSM-CE, RASM-CE. |
| cell.img | 819,096 | X-Cell – Dynamically loadable image for the MPM to manage all types of CSM. |
| ds3e3drv.img | 53,715 | Image for the following switching modules: ASM-DS3, ASM-E3, ASM-CE-DS3, ASM-CE-E3, ASM2-DS3, ASM2-E3, CSM-AB-DS3, CSM-AB-E3, RASM-DS3, RASM-E3, RASM-CE-DS3, RASM-CE-E3. |
| mesm.img | 24,337 | Image for the following switching modules: ESM-100C-12, ESM-100F-8, and ESM-C-32. |
| e12.img | 26,895 | Image for the following switching modules: ESM-C-12, ESM-T-12, ESM-F-8. |
| esm.img | 26,438 | Image for the following switching modules: ESM-8, ESM-U. |
| fesm.img | 30,912 | Image for the following switching modules: ESM-100-C with HSM motherboard. |
| fesm2.img | 30,944 | Image for the following switching modules: ESM-100-C with HSM2 motherboard. |
| fsm.img | 253,428 | Image for the following switching modules: FSM-M, FSM-S, FSM-C, FSM-M-C. Includes MPM code to support modules. |
| tsm.img | 125,864 | Image for the following switching modules: TSM-C-6, TSM-F-6. |
| tsm.pga | 74,410 | Includes trbyr6.pga, tryrr.pga, trlobe.pga, trstatn.pga, trsyr6.pga, trsyrr.pga, trxyln3.pga, tryln6.pga. |
| wsm.img | 419,586 | Includes WSM Driver, PPP Code (Telenetworks), Frame Relay, LMI, and WSM daughtercard (Daughtercard only takes up flash space only.). |
| fwd.img | 137,775 | Firewall image for IBM Nways 8274 LAN RouteSwitch |
| fwd.img | 137,523 | Firewall image for IBM Nways 8273 Ethernet RouteSwitch |
| lsm.img | 175,037 | LANE Service Module image |
| mp4.img | 1,543,529 | Management Processor Image for the IBM 8277 Nways Ethernet RouteSwitch 8277-13 & 5024. |
| mp4.cmd | 18 | Management Processor Image command file containing system commands for booting for the IBM 8277 Nways Ethernet RouteSwitch 2032, 3032, 5024. |
| mp4.cmd | 41 | Management Processor Image command file containing system commands for booting for the IBM 8277 Nways Ethernet RouteSwitch.8277-13 & 416. |
| ae4.img | 793,394 | Image for the following subswitch modules: ATM and Ethernet for the IBM 8277 Nways Ethernet RouteSwitch 2032, 3032 & 5024. |
| em4.img | 26,929 | Image for the Ethernet subswitch module for all IBM 8277 Nways Ethernet RouteSwitch units. |
| p4diag.img | 84,877 | Image for Enhanced Diagnostics for the IBM 8277 Nways Ethernet RouteSwitch 8277-13 & 5024. |
| mp.img | 1,488,211 | Management Processor Image for the IBM Nways 8273 Ethernet RouteSwitch. |
| mp.cmd | 18 | Management Processor Image command file containing system commands for booting for the IBM Nways 8273 Ethernet RouteSwitch. |
| as.img | 753,870 | ATM combined with Ethernet for the IBM Nways 8273 Ethernet RouteSwitch |
| fs.img | 274,882 | FDDI combined with Ethernet for the IBM Nways 8273 Ethernet RouteSwitch |
| pdiag.img | 66,611 | Image for Enhanced Diagnostics for the IBM Nways 8273 Ethernet RouteSwitch. |
| ws.img | 444,140 | WSM image for IBM Nways 8273 Ethernet RouteSwitch. |
| wsdrv.img | 302,270 | WSM image for IBM Nways 8273 Ethernet RouteSwitch. |
| asce.img | 68,587 | Image for the following switching modules: RASM-CE |

- Note:  Release 3.0 and higher require 4 Mbytes of FLASH for the IBM Nways 8274 LAN RouteSwitch and IBM Nways 8273 Ethernet RouteSwitch.

Due to the increasing number of image files and varying sizes of the image files, the user has an enormous amount of flexibility in configuring an IBM Nways 8274 LAN RouteSwitch or IBM Nways 8273 Ethernet RouteSwitch.  The table above shows the images with a brief description of what each image is used for. All images will fit into 4 Mbytes of FLASH, however the FLASH is also used to store configuration information so in most cases not all images are used and additional space will be required for configuration information. Therefore it is highly recommended that users remove all images not required for operation on a given switch (make sure you back them up on another device e.g. PC or Unix endstation). Once they are backed up remove (rm command) any Network Interface Module files that are not used to conserve flash memory usage.   For example, if the token ring module (TSM) is not installed in the IBM Nways 8274 LAN RouteSwitch, the tsm.img file and all .pga files may be removed.

IMPORTANT:  The chassis requires a minimum amount of space available in FLASH for booting. All the resident images in FLASH can not exceed 3.520 Mbytes when booting the IBM Nways 8274 LAN RouteSwitch or IBM Nways 8273 Ethernet RouteSwitch with the default configuration. If your images exceed this amount you MUST delete some images from FLASH.  Also if you are running with any configuration other than the default, you will need additional available FLASH.

# Dynamic DRAM allocation (mpm.img) Requirements

It should be noted that real-time processing forces the stack to grow and shrink based on events, therefore additional DRAM should be considered for the elasticity of the stack.

Software functions (or processes) may also allocate memory for data structure or tables, as needed.  For instance when RIP or OSPF receive routing updates from peer routers additional routes may be learned.  For each additional route, memory is allocated.

All of the dynamic allocation will not be described below due to the complexity involved.  Instead key functional areas where allocation may require additional memory are described.

Not all images impact the MPM memory.  Below is a table indicating which images will impact the MPM memory allocation.

| Filename | MPM Only | MPM and NI | NI Only |
| --- | --- | --- | --- |
| mpm.img | X |  |  |
| gated.img | X |  |  |
| isdn.img |  | X |  |
| mrd.img | X |  |  |
| nhrpmgr.img | X |  |  |
| t1e1drv.img | X |  |  |
| asm.img |  | X |  |
| asmce.img |  |  | X |
| asmcedrv.img | X |  |  |
| cell.img | X |  |  |
| ds3e3drv.img |  | X |  |
| mesm.img |  |  | X |
| e12.img |  |  | X |
| esm.img |  |  | X |
| fesm.img |  |  | X |
| fesm2.img |  |  | X |
| fsm.img |  |  | X |
| tsm.img |  | X |  |
| wsm.img |  | X |  |
| fwd.img | X |  |  |
| lsm.img | X |  |  |

# Multiple Image Requirements

Multiple Images are required for some modules.  The WSM modules and some of the ASM and CSM modules require more than one image. Below is a table showing which board types require more than one image besides the mpm.img.

| #### IBM Nways 8274 LAN RouteSwitch |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Board Type |  |  | Images (besides mpm.img) |  |  |  |
| CSM-AB | DS1 | cell.img | asm.img | t1e1drv.img |  |
|  | E1 | cell.img | asm.img | t1e1drv.img |  |
|  | CE-T1 | cell.img | asm.img | t1e1drv.img | asmcedrv.img |
|  | CE-E1 | cell.img | asm.img | t1e1drv.img | asmcedrv.img |
|  | DS3 | cell.img | asm.img | ds3e3drv.img |  |
|  | E3 | cell.img | asm.img | ds3e3drv.img |  |
|  | 155x | cell.img | asm.img |  |  |
| ASM-CE | T1/OC3 | t1e1drv.img | asmce.img | asm.img | asmcedrv.img |
|  | E1/OC3 | t1e1drv.img | asmce.img | asm.img | asmcedrv.img |
|  | T1/DS3 | t1e1drv.img | asmce.img | asm.img | asmcedrv.img |
|  | E1/E3 | t1e1drv.img | asmce.img | asm.img | asmcedrv.img |
| WSM | wsm.img |  |  |  |  |
| WSM-BRI | wsm.img | isdn.img |  |  |  |
| WSM-FT1 | wsm.img | t1e1drv.img |  |  |  |
| WSM-FE1 | wsm.img | t1e1drv.img |  |  |  |
| #### IBM 8277 Nways Ethernet RouteSwitch |  |  |  |  |  |
| Board Type |  |  | Images (besides mpm.img) |  |  |
| OSASM2-155 | ae4.img | em4.img |  |  |  |
| OSESM | em4.img |  |  |  |  |

# ATM Services

| ATM Services | ASM | ASM2 |
| --- | --- | --- |
| 1482 | X | X |
| LEC | X | X |
| CIP | X | X |
| X-LANE | X |  |
| Trunking | X |  |
| PTOP | X |  |

# Release 3.2.6 IBM Nways 8274 LAN RouteSwitch FLASH Requirements

| Images |  |  | Grand Total Flash Size | ATM Edge Flash Size | ATM Edge w/ Adv. Rtg Flash Size | Ethernet / FDDI Flash Size | Ethernet / FDDI w/Adv. Rtg. Flash Size | Cell Flash Size | Cell with LSM Flash Size | Wide Area Edge Flash Size | Wide Area Edge w/Adv. Rtg. Flash Size |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Files | Basic Images |  |  |  |  |  |  |  |  |  |  |
|  | BOOTROM |  | 262,144 | 262,144 | 262,144 | 262,144 | 262,144 | 262,144 | 262,144 | 262,144 | 262,144 |
|  | FFS Spare |  | 262,144 | 262,144 | 262,144 | 262,144 | 262,144 | 262,144 | 262,144 | 262,144 | 262,144 |
| 1 | MPM Image |  | 1.595,201 | 1.595,201 | 1.595,201 | 1.595,201 | 1.595,201 | 1.595,201 | 1.595,201 | 1.595,201 | 1.595,201 |
| 2, 3 | MPM Config (Potentially 2 files) |  | 65,536 | 65,536 | 65,536 | 65,536 | 65,536 | 65,536 | 65,536 | 65,536 | 65,536 |
|  | Potential Config information |  | 200,000 | 200,000 | 200,000 | 200,000 | 200,000 | 200,000 | 200,000 | 200,000 | 200,000 |
| 4 | MPM Log |  | 12,048 | 12,048 | 12,048 | 12,048 | 12,048 | 12,048 | 12,048 | 12,048 | 12,048 |
| 5 | MPM Misc |  | 4,096 | 4,096 | 4,096 | 4,096 | 4,096 | 4,096 | 4,096 | 4,096 | 4,096 |
|  |  | Subtotal | 2,401,169 | 2,401,169 | 2,401,169 | 2,401,169 | 2,401,169 | 2,401,169 | 2,401,169 | 2,401,169 | 2,401,169 |
| Network Interface Images |  |  |  |  |  |  |  |  |  |  |  |
| 6 | asm.img | ASM Driver | 812,690 | 812,690 | 812,690 |  |  | 812,690 | 812,690 |  |  |
| 7 | asmce.img | ASM CE | 46,288 |  |  |  |  |  |  |  |  |
| 8 | asmcedrv.img | ASM CE Driver | 152,679 |  |  |  |  |  |  |  |  |
| 9 | e12.img | ESM 12-port | 26,895 |  |  |  |  |  |  |  |  |
| 10 | esm.img | ESM 8-port | 26,438 |  |  |  |  |  |  |  |  |
| 11 | fesm.img | FAST ESM (HSM1) | 30,912 |  |  |  |  |  |  |  |  |
| 12 | fesm2.img | FAST ESM (HSM2) | 30,944 |  |  |  |  |  |  |  |  |
| 13 | fsm.img | FSM (MPM & NI) | 253,428 | 253,428 | 253,428 | 253,428 | 253,428 |  |  | 253,428 | 253,428 |
| 14 | mesm.img | Mammoth based ESM | 24,337 | 24,337 | 24,337 | 24,337 | 24,337 |  |  | 24,337 | 24,337 |
| 15 | tsm.img | TSM (MPM & NI) | 125,864 | 125,864 | 125,864 |  |  |  |  | 125,864 | 125,864 |
| 16 | t1e1drv.img | T1/E1 NI driver | 74,566 |  |  |  |  |  |  |  |  |
| 17 | ds3edrv.img | DS3/E3 NI driver | 53,715 |  |  |  |  |  |  |  |  |
|  |  | Subtotal | 1,658,756 | 1,216,319 | 1,216,319 | 277,767 | 277,767 | 812,690 | 812,690 | 403,629 | 403,629 |
| Dynamically Loadable Images |  |  |  |  |  |  |  |  |  |  |  |
| 18 | Cell.img | IBM Nways RouteSwitchCell image | 819,096 |  |  |  |  | 819,096 | 819,096 |  |  |
| 19 | Diag.img* | Diagnostic MPM image | 174,537 |  |  |  |  |  |  |  |  |
| 20 | Dni.img | Diagnostic NI image | 17,936 |  |  |  |  |  |  |  |  |
| 21 | Wsm.img | FR NI image | 419,586 |  |  |  |  |  |  | 419,586 | 419,586 |
| 22 | Isdn.img | ISDN BRI | 201,718 |  |  |  |  |  |  |  |  |
| 23 | Gated.img | Advanced Routing | 501,406 |  | 501,406 |  | 501,406 |  |  |  | 501,406 |
| 24 | Mrd.img | IP Multicast Routing | 66,296 |  |  |  |  |  |  |  |  |
| 25 | Lsm.img* | LANE Server | 175,037 |  |  |  |  |  | 175,037 |  |  |
| 26 | Fwd.img* | Firewalls | 137,775 |  | 137,774 |  | 137,774 |  |  |  | 137,775 |
|  |  | Subtotal | 2,513,387 | 0 | 639,180 | 0 | 639,180 | 819,096 | 994,134 | 419,586 | 1,057,767 |
|  |  | Total | 6,573,312 | 3,617,488 | 4,256,668 | 2,678,936 | 3,318,116 | 4,032,955 | 4,207,993 | 3,224,384 | 3,863,565 |
|  | 4 MegBytes = 4,194,304 |  | 4,194,304 | 4,194,304 | 4,194,304 | 4,194,304 | 4,194,304 | 4,194,304 | 4,194,304 | 4,194,304 | 4,194,304 |
|  |  |  | 2,379,008 | -576,816 | 62,364 | -1,515,368 | -876,188 | -161,347 | 13,689 | -969,920 | -330,739 |

* When utilizing these dynamically loadable images, we recommend 32MB DRAM.

WARNING:

You will have to remove unused network interface images from flash to make room for Loadable images if the amount of remaining flash is less than 512,288 bytes.

# Release 3.2.6 IBM 8277 Nways Ethernet RouteSwitch FLASH Requirements

| Images |  |  | IBM 8277 Nways Ethernet RouteSwitch Grand Total Flash Size | ATM Uplinks Flash Size | Fast Ethernet Uplinks Flash Size |
| --- | --- | --- | --- | --- | --- |
| Files | Basic Images |  |  |  |  |
|  | BOOTROM |  | 524,288 | 524,288 | 524,288 |
|  | FFS Spare |  | 262,144 | 262,144 | 262,144 |
| 1 | MP4 Image |  | 1,543,529 | 1,543,529 | 1,543,529 |
| 2, 3 | MPM Config (potentially 2 files) |  | 65,536 | 65,536 | 65,536 |
|  | Potential Config information |  | 200,000 | 200,000 | 200,000 |
| 4 | MPM Log |  | 12,048 | 12,048 | 12,048 |
| 5 | MPM Misc |  | 4,096 | 4,096 | 4,096 |
|  |  | Subtotal | 2,611,641 | 2,611,641 | 2,611,641 |
|  | Network Interface Images |  |  |  |  |
| 6 | ae4.img* | ATM & Ethernet Driver | 793,394 | 793,394 |  |
| 7 | em4.img | Ethernet Driver | 26,929 | 26,929 | 26,829 |
|  |  | Subtotal | 820,323 | 820,323 | 26,829 |
|  | Advance Feature Images |  |  |  |  |
| 10 | p4diag.img* | Diagnostic MPM image | 84,877 |  |  |
| 11 | gated.img* | Advanced Routing | 501,406 |  |  |
| 12 | mrd.img* | IP Multicast Routing | 66,296 |  |  |
|  |  | Subtotal | 652,579 | 0 | 0 |
|  | Total |  | 4,084,543 | 3,431,964 | 2,638,470 |
|  | 4 MegBytes = 4,194,304 |  | 4,194,304 | 4,194,304 | 4,194,304 |
|  |  |  | 109,761 | -762,340 | -1,555,834 |

*These images are dynamically loadable images.  When running lsm.img and fwd.img, it is recommended that the user install 32MB DRAM.

# Release 3.2.6 IBM 8277 Nways Ethernet RouteSwitch Images Required

|  | OS-8277-13/416 | 8277-524/3032/2032 |
| --- | --- | --- |
| mp4.cmd | YES | YES |
| em4.img | YES | YES |
| mp4.img | YES | YES |
| ae4.img | NO | YES |
| p4diag.img | YES | YES |
| ws4.img | NO | YES |
| isdn.img | NO | YES |

OS-8277-13 and OS-416 are shipped with Diagnostics program disabled from the factory to save DRAM spaces.  It must be kept that way during the normal operation of these IBM 8277 Nways Ethernet RouteSwitch models.  Otherwise, low memory error may occur.  If Diag program is needed, it must be enabled by modifying the MPM command parameter in the mp4.cmd file as shown below.  It should be disabled after the Diag program and before using the IBM 8277 Nways Ethernet RouteSwitch for normal operation.

Disable Diag (factory default)

diagnostics_disabled=1

Enable Diag

diagnostics_disabled=0

# Release 3.2.6 IBM Nways 8273 Ethernet RouteSwitch FLASH Requirements

| Images |  |  | Grand Total Flash Size | IBM Nways 8273 Ethernet RouteSwitch with ATM Flash Size | IBM Nways 8273 Ethernet RouteSwitch without ATM Flash Size |
| --- | --- | --- | --- | --- | --- |
| Files | Basic Images |  |  |  |  |
|  | BOOTROM |  | 524,288 | 524,288 | 524,288 |
|  | FFS Spare |  | 262,144 | 262,144 | 262,144 |
| 1 | MP Image |  | 1,488,211 | 1,488,211 | 1,488,211 |
| 2, 3 | MPM Config (potentially 2 files) |  | 65,536 | 65,536 | 65,536 |
|  | Potential Config information |  | 200,000 | 200,000 | 200,000 |
| 4 | MPM Log |  | 12,048 | 12,048 | 12,048 |
| 5 | MPM Misc |  | 4,096 | 4,096 | 4,096 |
|  |  | Subtotal | 2,556,427 | 2,556,427 | 2,556,427 |
|  | Network Interface Images |  |  |  |  |
| 6 | as.img | ATM & Ethernet image | 753,870 | 753,870 |  |
| 7 | asce.img |  | 68,587 |  |  |
| 8 | asmcedrv.img |  | 152,679 |  |  |
| 9 | fs.img | FDDI & Ethernet image | 274,882 | 274,882 | 274,882 |
| 10 | t1e1drv.img | T1/E1 NI Driver | 74,566 |  |  |
| 11 | ds3e3drv.img | DS3/E3 NI Driver | 53,715 |  |  |
|  |  | Subtotal | 1,378,299 | 1,028,752 | 274,882 |
|  | Dynamically Loadable Images |  |  |  |  |
| 12 | pdiag.img | Diagnostic MPM image | 66,611 |  |  |
| 13 | gated.img | Advanced Routing | 501,406 |  |  |
| 14 | mrd.img | IP Multicast Routing | 66,296 |  |  |
| 15 | ws.img | FR NI image | 444,140 |  |  |
| 16 | wsdrv.img | FR NI Driver | 302,270 |  |  |
| 17 | fwd.img | Firewall image | 137,523 |  |  |
|  |  | Subtotal | 1,518,246 | 0 | 0 |
|  |  | Total | 5,452,972 | 3,585,179 | 2,831,309 |
|  | 4 MegBytes = 4,194,304 |  | 4,194,304 | 4,194,304 | 4,194,304 |
|  |  |  | 1,258,668 | -609,125 | -1,362,995 |

WARNING:

You will have to remove unused network interface images from flash to make room for Dynamically Loadable images if the amount of remaining flash is less than 512,288 bytes.

# MPM Upgrade

Release 3.0 and greater requires 16 Mbytes of DRAM memory.  For units in the field, there are two field upgradable memory upgrades for the MPM.  One Model no. is MPM-UK-16MB, this upgrade kit expands the Management Processor Module DRAM memory to 16 Mbytes.  The upgrade includes one 16 Mbyte simm and installation instructions. The other one is MPM-UPGR-32MB, this upgrade kit expands the MPM module DRAM memory to 16 Mbytes, it also expands the FLASH to a total of 8 Mbytes.  The upgrade includes one 32 Mbyte simm and installation instructions.  For MPM upgrades IBM memory is required.

# Simm Flash Module

The following files MUST be in /flash:

The boot file	-	mpm.img	See below.

The startup script	-	mpm.cmd	See below.

Configuration files	-	mpm.cfg AND mpm.cnf

The Log file	-	mpm.log (this may change)

Redundant MPM issues w/32MB SIMM/flash:

A file in /flash can only be transferred to /flash on another MPM.

A file in /simm can only be transferred to /simm on another MPM.

This applies to imgsync, mpmstore, mpmreplace, mpmload and mpmget.

# HSM2 Modules

The HSM is the base switching intelligence for all IBM switching modules except for the 10BaseT switching modules.  The HSM2 is an enhanced version of the original HSM, which provides improved performance, specifically for two-way traffic.

HSM2 support requires software release 2.0.5. or later.  If any modules in the chassis are equipped with HSM2’s, software release 2.0.5 or later must be loaded in the chassis’ MPM.  Additionally, user’s transitioning from 1.x software also requires additional memory.

In software release 3.0 all network interface images, except for FESM2 and WSM2 are compatible with HSM and HSM2.

# ESM-100C-12, ESM-100F-8, and ESM-C-32 modules

The ESM-100C-12 and ESM-100F-8 do not have adequate bandwidth to provide full-wire throughput with full-duplex enabled on all the ports at 100 Mbps.  A rule of thumb on ESM-100C-12: Ports 1 to 6 have approximately 700 Mbps bandwidth to use, and ports 7 to 12 also have 700 Mbps of bandwidth. Use these to calculate the expected available full throughput (100 Mbps used for half duplex ports and 200 Mbps used for full duplex ports).  Similarly, on ESM-100F-8 ports 1 to 4 have approximately 700Mbps bandwidth and port 4 to 8 also have 700MBps bandwidth.  Note that this limitation does not apply to ESM-C-32 module.

For example, following configuration will only consume 630 Mbps out of 700 Mbps available.

PORTS	SPEED		DUPLEX

Port 1 through 3	100Mbit	Full

Port 4 through 6	 10Mbit	Half

Another example show below consumes all available 700Mbps bandwidth.

PORTS	SPEED		DUPLEX

Port 1 	100Mbit	Full

Port 2 through 6	100Mbit	Half

Configurations with more than 700 Mbps allocated will likely result in lost and errored packets depending on traffic load.

- The 10/100 and Full/Half Duplex Configuration: The 10/100 and half/full duplex port configuration is only available from the UI only.  The SNMP management objects for these variables will be included in later releases.

- ESM-100C-12, ESM-100F-8, and ESM-C-32 modules do not maintain statistics on the number of frames discarded due to the flood limit.  The statistic shows as 0.

- ESM-C-32 card will not operate outside of 100ppm tolerance of 16 Megahertz specified in Ethernet Specifications. ESM-C-32 will operate up to 200 ppm.  However, we have observed some NIC cards which are in the tolerance range of  500 ppm.

- The IBM Nways 8274 LAN RouteSwitch supports up to 253 virtual bridge ports, therefore, an IBM Nways 8274 LAN RouteSwitch with 8 ESM-C32 modules has a total of 256 physical ports but will not be able to use more than 253 ports.   The last 3 ports in slot 8 are not useable.

- Port mapping is supported on the following Mammoth based Ethernet boards:  ESM-100C-12, ESMC-FD, ESM-100FM-8, ESM-100FS-8W-4C, ESM-100-FS-FD, ESM-100FS-FD-1

# Optimized Ports on ESM-100C-12, ESM-100F-8, ESM-C-32 and IBM 8277 Nways Ethernet RouteSwitches

The ESM-100C-12, ESM-100F-8 AND ESM-C-32 and IBM 8277 Nways Ethernet RouteSwitches will flood packets with unknown destination addresses on ports configured for optimized device mode.

To prevent this from occurring, the following can be entered in the mpm.cmd file:

MamOptSwtchPorts=1

NOTE:  If the port is set to optimized and has not learned a MAC address, it will flood these packets out regardless if the above command is used.

# ATM LANE

## 802.3 LEC

All translations performed on outbound 802.3 LECS services are the same translations that are performed on outbound physical Ethernet ports.  Under the UI commands “mas”, and ”cas”, there is a field called use_translations.  This field controls whether outbound 802.3 LEC services should enable or disable translations when transmitting frames originating from a physical Ethernet segment. It should also be noted that when the field use_translation is set to 0, all translations from dissimilar medias are still performed, and only translations on data frames from physical Ethernet media to 802.3 LECS  will be disabled.  This option is provided to provide increased performance when moving data between physical Ethernet media and 802.3 LANE.

## 802.5 LEC

All translations performed on outbound 802.5 LECS services are the same translations that are performed on outbound physical Token Ring ports.  Under the UI commands “mas”, and ”cas”, there is a field called use_translations.  This field should control whether outbound 802.5 LEC services should enable or disable translations when transmitting frames originating from a physical Token Ring segment. However, this option is not applicable to 802.5 LEC services.  Please disregard this field.

## ATM 802.3 LEC to Ethernet Segment Translations

When translating data frames received from an ATM 802.3 LEC service to be output on an Ethernet segment, the user has the option to enable or disable translations. There is a parameter called atm_ethernet_xlat under the UI “cas” and “mas” command which defaults to a 1, meaning translations are enabled. For details on actual protocol translations and meaning of various translations options please refer to the IBM Nways 8274 LAN RouteSwitch User Manual.

## LAN Emulated Client Start–up and Back-off Timers

The IBM IBM Nways 8274 LAN RouteSwitch LEC provides the capability of backing-off if the LEC notices calls being released or when multiple LECS on the same IBM Nways 8274 LAN RouteSwitch are being enabled at the same time (e.g. physical cable is disconnected/reconnected).  The amount (time in ticks) and nature (fixed/random) of the back-off period can be controlled using the variables described in the table below.  It should be noted that the adjustment of these values needs to be done only if the ATM Network (which the LECS are connected) is not capable of processing the total number of calls that could be generated by all the LECS connected to the ATM network.  This scenario is most likely to occur under the following two situations:

- During a global reboot of all the LECS due to power on/power failure.

- During a global attempt to join an ELAN when a central LANE server resource such as LECS/LES/BUS is disconnected/reconnected or some failure.

Each LEC needs to setup a call to LEC/ LES and BUS before it stops retrying.  If any fails the LEC backs off and continues the process of attempting to join the ELAN.

The default values are chosen so that calls to the LECS/LES/BUS are individually throttled by a time period of 16.66 – 266.56 milliseconds.

For 3.2.6, the default timers with comments are as follows;

| Default Timers | Description | Value |
| --- | --- | --- |
| atmlec_randomize_throttle = 1 | This flag controls if the procedure uses random back-off or fixed back-off. <= 1 (random back-off) =0 (fixed back-off) | 0 or 1 |
| atmlec_backoff = 500 | The % back-off in both random and fixed can be controlled using the flag atmlec_backoff,, by default + 500 (for 500%). | > = 100 |
| atmlec_LECS_throttle_ticks = 16 | All values for ticks should be power of two (example 8, 16, 32, 64). Each tick is 16.66 milliseconds. | Power of 2 and > = 2 |
| atmlec_les_throttle_ticks = 16 | All values for ticks should be power of two (example 8, 16, 32, 64). Each tick is 16.66 milliseconds. | Power of 2 and > = 2 |
| atmlec_bus_throttle_ticks = 16 | All values for ticks should be power of two (example 8, 16, 32, 64). Each tick is 16.66 milliseconds. | Power of 2 and > = 2 |
| atmlec_LECS_retry = 3 | Retries before reboot | > = 3 |
| atmlec_les_retry = 3 | Retries before reboot | > = 3 |
| atmlec_bus_retry = 3 | Retries before reboot | > = 3 |

To modify these default timers, the mpm.cmd files needs to be modified. An example mpm.cmd file with modified timer values is provided below;

cmDoDump=1

atmlec_LECS_throttle_ticks=64

atmlec_les_throttle_ticks=64

atmlec_bus_throttle_ticks=64

atmlec_LECS_retry=7

atmlec_les_retry=7

atmlec_bus_retry=7

cmInit

## Plug and Play LECS

From builds 3.2.1.5, when an IBM Nways 8274 LAN RouteSwitch is booted up with no configuration information or when an ASM is newly (was not an ASM slot before) hot inserted into a chassis, the default ATM service created on an ATM access interface will be a 802.3 LEC.

This means that all ASM ports (SAHI and MIDWAY based) and the FCSM LAN port will now automatically be in SVC mode (will run SSCOP and ILMI protocol) and each of the ATM interface will come up with a 802.3 LEC trying to join the elan named "default".

*** There will no longer be the PTOP PVC of VC 100 by default ***

If configuration information for each of the ATM interface is already present, then you should see no change on these ATM interfaces, i.e. no new services are created.

To ensure proper operation, an ASM port should be now connected to CSM ports and not back to back to another ASM port.

Also the LES/BUS being used (MSS or IBM Nways 8274 LAN RouteSwitch) should have the elan named "default" configured.  To make this configuration easy, enter the following command on "the" IBM Nways 8274 LAN RouteSwitch to be setup as the LES/BUS.

autolesbus slot/intf.     ==> this will automatically setup an elan named default.

Although a 802.3 LEC is created by default on each ATM interface as described above, only those LECS on an enabled (cable connected) physical interface will try to join. Also if the chosen LES/BUS is more than a hop away, please do not forget to change the CSM ports to PNNI type as needed.

All other service operations on this default LEC remains the same as in previous builds.

## LANE Service Module

The following is the memory used in the LANE Service Module (LSM) assuming one LEC and one LES-BUS (one ELAN) is configured with the ELAN_NAME policy:

LSM code image (un-compressed):	500,000 bytes

LECS runtime memory	:	5,200  bytes

LES-BUS runtime memory	:	4,300  bytes

Message queue for AM	:	504,000  bytes

Message queue for Signaling	:	335,000  bytes

Message queue for Timer	:	2,000 bytes

Message queue for VSE	:	1,200 bytes

Total memory used:		1,351,700 bytes

- Each LEC will consume 4,618 bytes.

If you have 60 LEC's joined in the same ELAN then it will be 60*4,618=277,080 bytes plus the 1,351,700 above which will come up with 1,628,780 bytes usage. If there are 60 LECS and 4 ELANs, divided into two ELANs you need an additional 4,300 bytes for the second ELAN.

A single LSM will support up to 80 LAN Emulation Clients (LECS) and 4 ELANs (LES/BUS) which is an average of 20 LECS per ELAN. The BUS forwarding rate measured on SAHI is 50,000 packets per second.

The LECS Policy Value of ESI/Selector under the ATM Address Prefix is not supported in release 3.2.6. Any user input will have no effect in filtering, any LECS Configuration Request will pass this criteria if it is selected.

LSM LES/BUS redundancy cannot be configured on the same switch.

# PNNI - Configuring Node-Specific Parameters

- If a "pncfg" command has been issued on a 3.1.X switch, it will store the user's node id/address configured.  If the administrator did not modify the node id or address, pncfg will store the factory default address.

- If the user then upgrades to 3.2, the node id will be as it was in 3.1 and prior builds.  Once the user upgrades their 3.1.X switch to 3.2.6, they need to issue the "pncfg" command and change it to the new prefix.  When PNNI configuration has been changed using the pncfg, there is a flag that gets set, which will tell PNNI whether it should use the 3.2.6 factory defaults or the user configured prefix.

- Also, after upgrading to 3.2, the user should verify that the address summary matches the node id (in the first 13 bytes).

# PNNI Identifiers: ATM Address and Node ID

- In releases prior to 3.2, the default node level was 96(in decimal notation). In release 3.2, the default level is 80(in decimal notation). Therefore, if you have an existing IBM Nways 8274 LAN RouteSwitch network with PNNI nodes residing at the old default level, you will have to reconfigure the Node IDs for either the new nodes or the old nodes if you want all nodes to be in the same peer group.

- The ATM address for a node in a Single Peer Group Network for the 3.2.X switch is as follows:

- 3903488001bc90000101xxyyzz0020daxxyyzz00

- where xxyyzz is the 3-byte IBM Nways 8274 LAN RouteSwitch-specific ESID for the mpm in this chassis.

- For example the following are the steps for reconfiguring the 3.1.x switch Node ID and Peer Group ID in order for the 3.1.X switch to be in the same peer group as the 3.2.X switch.

- pncfg slot/port

- ATM PNNI Node-specific Configuration

- (Will be defaulted to bracketed values if unspecified)

- 1)	ATM address of this node(hex)   : Unspecified

- [3903488001bc900001000100010020da7f6bc000]

- 2)	Node ID (without level and rsvd): Unspecified

- [3903488001bc900001000100010020da7f6bc000]

- 3)  	Local node level            [96]: Unspecified

- 4)  	Admin status (up,down)      [up]: Unspecified

- 5)  	Timer configuration sub-menu

- To configure a parameter, type "item = value"  (as in 3=100)

- To quit out of configuration, type "quit"

- To save the configured info, type "save"

- :1=3903488001bc900001017f6bc00020da7f6bc000

- :2=3903488001bc900001017f6bc00020da7f6bc000

- :3=80

- :save

- :quit

- Enter pnin command to verify the current configuration values for this node, such as its Node ID and ATM address after changes are made.

- ATM PNNI Node Information

- Node ID:	50a0 3903488001bc900001017f6bc00020da7f6bc000

- ATM address of this Node:	3903488001bc900001017f6bc00020da7f6bc000

- Node level:	50   (80 decimal)

- Peer group ID: 	50   3903488001bc90000101000000

- PNNI node index:	1

- Administrative status:	ENABLED	Operational status: 	UP

- Lowest level node: 	True	Restricted transit:	False

- Complex representation: 	False 	Restricted branching: 	False

- Address summarization:	Configured (and operational)

- Summary address:	3903488001bc900001017f6bc0

# PNNI and Redundant MPM's

When the node ID is unspecified, the node Id changes when the secondary MPM takes over.  To prevent this failover problem, one must specify the default, save it to the configuration file, and then perform an image sync as follows:

- Execute pncfg on the primary and set items 1,2,3, and 6 to the defaults (from unspecified …)

- Save the data and force the changes to take immediate effect.

- Execute a configsync. (immediately …)

Optional:

- Execute renounce (to verify that the values are on the redundant MPM).

- Login to the (now) primary

- Execute pncfg or pninfo and verify items 1,2 3, and 6…..

# CSM/FCSM Hot Swap Instructions

The following instructions are recommended when performing hot swap with CSM and FCSM modules:

- “Swap” command should be on when pulling or inserting CSM and FCSM modules.

- Before pulling the CSM module, disconnect all CSM ports on that CSM module.

- Pull the CSM Module.

- Wait for OK2 LED on MPM to return to blinking green

- Insert CSM module.

- Wait for OK2 LED to return to blinking green on the MPM module and the inserted CSM module.

- Reconnect any CSM ports.

Currently if an FCSM is pulled, most likely the LECS attached (externally) to that chassis will not rejoin the ELAN and stay in the initial state (PR 10780).  This is an external ASM LEC problem.  One of the following workarounds must be used to bring the attached external ASM LEC back up:

- Disconnect the LECS before hot swapping the FCSM or

- Use ‘mas’ to bring the LEC back up after hot swapping the FCSM.

When FCSM is Hot Swapped in large networks, ASM side of signalling stack releases all the new call setup requests.  PR 13961 & 13960

The following workaround must be used:

1)	Before pulling FCSM module, disconnect all CSM ports on all CSM modules on that switch.

2) 	Wait for all the calls to be released.

3) 	Pull the FCSM module.

4) 	Wait for OK2 LED on MPM to return to blinking green.

5) 	Insert the FCSM module.

6) 	Wait for FCSM initialization to complete.

7) 	Wait for OK2 LED to return to blinking green on MPM and inserted FCSM module.

8) 	Reconnect all CSM ports.

# CSM-PNNI MTU Size Configuration Guidelines

This document is being provided as additional design guidelines for building a core ATM-based network running PNNI, when using the IBM Nways 8274 LAN RouteSwitch with X-Cell modules.

The design guidelines take into consideration that the PNNI frame sizes used have a direct impact on the ability to have a certain number of CSM (Cell Switching Module) ports operational in one IBM Nways 8274 LAN RouteSwitch. PNNI frame sizes will vary depending on what type of information is being sent between PNNI nodes. It ranges from 16 bytes to 8192 byte sizes. The most frequently sent message is the “hello” message – it consists roughly of about 100 bytes. However, database summaries and PTSE’s (PNNI Topology State Elements) that are exchanged between switches can run from 64 to 8K frame sizes. As the support for frame sizes become larger, the total number of CSM ports you can have on a switch becomes smaller.

The chart above illustrates that if you need to support PNNI frames of up to 8192 bytes, the total number of CSM ports that you can have in the chassis is limited to 21 CSM ports.

It is also recommended that the maximum number of PNNI neighbors per PNNI node should be less than, or equal to 30 neighbors. Having more than 30 neighbors will trigger higher CPU utilization on the MPM (Management Processor Module), unless the Hello timers are re-configured [using the pmcfg command] with higher values. As an example, if the PNNI node is required to support up to 8K frame sizes, you can configure up to 21 CSM ports running PNNI.

The frame sizes are configurable [using the command MAP slot # / 2 on the FCSM] and these are our frame size recommendations:

- Small to Medium networks (up to 20 PNNI nodes ) : 	2048 to 6144 bytes

- Large size networks (more than 20 nodes):	       8192 bytes

By default, IBM switches will not originate frames larger than 2K bytes. Therefore in an all-IBM network, re-configuration of the MTU’s is not required. However, if you plan to have a multi-vendor ATM network, it is recommended that you use the 8K frame size, and also limit the number of CSM port connections to less than or equal to 21 per IBM Nways 8274 LAN RouteSwitch.

[Note: this recommendation applies only when the FCSM-155 module is used; this is not applicable to the FCSM-622 and MPM-CW modules].

# IBM Nways 8274 LAN RouteSwitch Version 3.1 Problem Resolutions

The IBM Nways 8274 LAN RouteSwitch Version 3.1.6 known problems have been fixed in this release.

- When using the fportc command to configure FDDI SMT, a user can enter more than one assignment statement.  If the user opts to use more than one assignment then there will be one prompt per assignment.  This will not cause any problem, except for visual display. (PR 284)

- Static MAC cannot be assigned on any Service/Instance besides the first bridge port.  (PR 2383)

- Macinfo may report more number of MACs than actual in a source routed environment. The actual MAC addresses shown are correct as is the number of MACS shown. (PR 3157)

- Change mirroring ports group disables mirroring. (PR 5090)

- Invalid network parameter within the network statement of the Gated.conf may crash the switch. (PR 6279)

- An SNMP get of the atTable returns one extra byte that is erroneous.  This is an inconvenience only, as the rest of the data is correct. (PR 6669)

- No virtual link information is available within the UI.  If virtual links are configured, the user has no way of finding out information about the status, etc.  (PR 6793)

- RIPflush does not clear the RIP routes in GateD. (PR 6900)

- Deleting an active xlat entry causes error number (errno) 0X2 message to be output. (PR 6975)

- Failure in csmSigtask related to PNNI routing.  In some rare conditions, the csmSigTask may generate an exception at boot time, causing LECS to not join the LES.  This will result in a dump file being produced on the MPM. (PR 6982)

- When LANE is in intermediate state and administratively is disabled, VCs are still displayed. (PR 7138)

- Services may get corrupted on invalid SNMP modify attempts. (PR 7185)

- User side of an IISP is not supported. (PR 7422)

- Call control does not participate correctly in the master/slave dialogue.  IBM Nways RouteSwitch PNNI requires itself to be the master.  As such the IBM Nways RouteSwitch ESI must always be of greater value than the other vendors.  Workaround: Set IBM Nways RouteSwitch to higher value or the other vendor to lower value. (PR 7423)

- Changing admin weight in ppcfg for a port causes all the default values to go to zero. (PR 7990)

- Firewall does not work with HRE. (PR 8007)

- Multicast routing will not operate if ESM-100C-12, ESM-100F-8, and ESM-C-32 (with optimized ports) and Multicast claiming turned on. (PR 8033)

- In certain cases, when using variable length subnet masks where the mask is not a continuous string of ones (i.e. has 0’s in the mask) , OSPF will abort. (PR 8089)

- Delete Mobile Group Net IP Rule does not the return port to Group 1. (PR 8105)

- When performing a get next on pnnixNodeTable, the getnext operation goes into a loop. (PR 8115)

- When RI/RO is set, src shows six ports for fiber card instead of three. (PR 8130)

- Token Ring IEEE 802.5 MIB is not supported  (PR 8327)

- LED display of ESM-100C-5 is not displayed properly.  (PR 8380)

- Changing vpi/vci bits on OC12 ports does not work. (PR 8475)

- The status LED’s are not reflected for banks C & D on CSM-A25-24. (PR 8557)

- The max vpi/vci bits for oc12 modules in the map command should be 2 bits vpi and 10 bits vci respectively. (PR 8626)

- Flash parameters of a reverse connection are not deleted after a connection is deleted. (PR 8681)

- Diagnostics test may fail when ATM E3/DS3 and/or OC3 modules are tested due to insufficient transmit/receive delay. (PR 8923)

- Previously used Mirroring Port on ESM-12 causes CPU utilization to remain at 55-65%. (PR 9011)

- AppleTalk Phase 1 (EtherII) is not supported on Nways RouteTracker Manager. (PR 9031)

The IBM Nways 8274 LAN RouteSwitch Version 3.1.7 known problems have been fixed in this release.

- SNMP agent returns NO_SUCH_NAME error when any dsx3ConfigTable objects were set.  (PR 6902)

- Under ptomp connections, if a party is removed, the reverse connection is not removed automatically.  The user needs to remove the reverse connection in a 2 step process. (PR 6968/6091)

- The Auto Switch Timer does not reflect the same value when set via SNMP.  i.e. A user set the timer as 30000 from NMS but the next query of that value comes back with another number. (PR 7543)

- Frame relay crashes or VC fails to pass data after line is put into loop back.  (PR 10492)

- RWSM-SC-4 with compression enabled may have CRC errors under heavy traffic.  (PR 11089)

- Serial port will not come down if multiple cables are unplugged at the same time.  (PR 11091)

- First time configured PPP serial port will not come up with cisco.  (PR 11153)

- Multicast claiming causes issues with learning duplicate mac addresses.  (PR 11239)

- Prevent Signaling suspend from memPart free command.  (PR 11328)

- High MPM utilization caused by SR over multiple groups.  (PR 11339)

- Resolve switch crash in cmSigTask by PNNI routines.  (PR 11354)

- Feature to filter SR traffic on a TB port.  (PR 11441)

- Unnecessary AT processing of Source Learned packets when AT is not configured.  (PR 11451)

- blsMgr runs high utilization when config file is removed.  (PR 11498)

- Proper initialization of CSM-622 IOP's to prevent high CRC error and loss of SSCOP.  (PR 11569)

- Continuous SNMP polling of the routing table crashes switch.  (PR 11594)

- Addition of diagnostic tests for CSM Modules.   (PR 11608, 11611, 11644)

- Entire CAM table billed with Multicast entries cause no source learning for regular mac addresses.  (PR 11609)

- Memory corruption caused by signaling stack.  (PR 11612)

- TSM-CD-6 fails port test after 50 iterations.  (PR 11635)

- Consistent FCSM crash if user removes configuration information created prior to 3.1.6.  (PR 11639)

The IBM Nways 8274 LAN RouteSwitch Version 3.1.8 known problems have been fixed in this release.

- Certain IBM Token Ring devices, that attempt to use express buffers will have problems since our TR LEC uses FC = 0x45 (5). (PR 8249)

- CIP client configured as SVC fails to retry the call setup to the ARP Server when the first call setup got a call proceeding/connect message with a VPI/VCI that is already in our connection record table. Such as VCI=100 for PTOP. (PR 11648)

- Using the MAS command to modify a service that has source routing configured causes the inbound and outbound hop counts in the source route configuration to be set back to the default value of 7. (PR 11959)

- Magic MAC address frames, used to communicate between the MPM and NI's, often have special formats that are not network compatible. When these frames are parsed by the HRE they can crash. If the magic frame is sent with a media type equal to 0x10, the HRE will ignore these frames. (PR 12007)

- 2 CIP SVC clients calling each other get two SVC established but traffic is not forwarded out the correct VC for the other side to receive. The two clients are unable to communicate with each other. (PR 12013)

- When a client sends out a DHCP request over a source routed environment, the reply from the server has the Client's MAC address in canonical format instead of non-canonical. (PR 12098)

- Increase logging capability to add VSE capture information to help diagnose problems with MPM, particularly in the matter of MPM high CPU utilization. (PR 12174)

- rifStripping not working on Sahi (hsm2 &hsm3) modules. (PR 12407)

- Source Routed FDDI frames cause the ASM to crash. Added additional debug & Fix. (PR 12445)

- IBM Nways 8274 LAN RouteSwitch with Token Ring LEC defined on each ASM port (different SR Segment numbers). A Magic MAC address setup by ATM manager during initialization is disappearing from the MPM CAM, so the frame is claimed by every slot including the AS and it is being bounced back to the MPM again and again. (PR 12447)

- Make RxBusy configurable on mammoth for ATM CAM problem. (PR 12450)

- Added validity check on FDDI SMT frames to insure that the buffer pointer is word-aligned. On rare occasions this address is not aligned and we need to discard the frame and increment the counter. (PR 12459)

- The IBM Nways 8274 LAN RouteSwitch allows a TB frame from a Source Route Bridge port out to a Source Route Transparent port and this causes the clients RIF to be lost causing connectivity to be lost. (PR 12544)

- Under certain unknown load/traffic conditions, the IBM Nways 8274 LAN RouteSwitch telnet client may drop the connection. (PR 12557)

- IBM Nways 8273 Ethernet RouteSwitch with multiple Nways RouteTracker Manager VLANs and an external router attached does not flag the IP router as an IP device and router in the Nways RouteTracker Manager software. (PR 12704)

- Added hysteresys for CSM ports. Loss of signal to upper layer is delayed for x periods before notification of link down. Default value is one period or one second. Variable can be set in the mpm.cmd for extending this period. (PR 12744)

- 't_csmumeTask' runs out or very low on space and causes the switch to crash.  (PR 12762)

- IBM Nways 8274 LAN RouteSwitch with TR_LEC configured over redundant cell switches. In redundant link connected switches, if the secondary link takes over the connection to the TR-LEC still uses the primaries VCI pointing to the other switch, not the VCI setup for the secondary link. (PR 12766)

- The switch crashes in UI after modifying the port rule for the ATM Service. (PR 12770)

- rifStripping is not working on IBM Nways 8273 Ethernet RouteSwitch, it was found that the software was missing in the IBM Nways 8273 Ethernet RouteSwitch Ethernet module. (PR 12771)

- Need to add a sap filter to allow for separation of traffic on the edge switches. This would allow the source routed traffic and IP traffic to not go where it isn't wanted. (PR 12775)

- Many MACs are being flushed by ATM multiple times when the primary link is being restored from the secondary link, because some switches still hold the MAC addresses on the BLOCKED port because of the Source Route frame. (PR 12796)

- Occasionally a MAC has been deleted from the Source Learning database but is still in the NI CAM, when this happens the two are out of sync and the connection to the MAC will never be recovered. (PR 12820)

- Same PR as 13009, and 13064. IPR table has not AF_INET in the routing table, with a bogus metric. (PR 12854)

- When one cell switch is turned off the secondary path is taken. When the cell switch is turned on again, everything switches back to the primary path. However, there are situations where the port is updated in the source learning table but the vci is still the one used for the secondary path's ATM port. (PR 12870)

- Switches uses senders hardware address instead on the switches Vrouter port address, in a gatewayless client the ARP reply will not have the correct address. (PR 12902)

- If the IP cached RIF changes, the IP route cache in the HRE or MPM route cache should be flushed to allow this change to end up in the route cache. (PR 12905)

- This is a PER to allow SAP control of NET side ILMI polling. (PR 12914)

- Two switches are connected with a PTOP service and a LANE service over the same media. LANE spanning tree cost is 8 and PTOP is 10. Based on cost the PTOP service goes into blocking and the LANE is forwarding. When the LANE connection is broken the failover to the PTOP service works. When the connection is reestablished, the PTOP goes back to blocking, and the LANE service is in forwarding state, but no traffic is forwarded. (PR 12988)

- Two switches connected via PNNI and IISP. Creating a PTOP SVC (Private or RFC1483) on either switch will put their ports in "enabling" mode. (PR 12996)

- RIP version 1 is failing to advertise static routes. (PR 13009)

- In redundant Token Ring environments Token Ring CAMs fills up and duplicate MACs are caused in the same group. As a result, all of the MAC addresses in the Token Ring CAMs, instead of only the local addresses. Setting 'ARE blocking' to Yes on all redundant TR ports, to prevent load balancing. (PR 13048)

- IPR command returns "Bad rtentry at 0x4bff648c". This goes on and on… (PR 13064)

- Non-ATM ports Logical layer's admin status are down. Shows as disabled via the vi command. (PR 13078)

- Null RIF in FDDI/Trunking environment cause CPU to have high utilization. (PR 13083)

- Bridged Token Ring to Ethernet DHCP request causes the ARP entry for the DHCP client being in non-canonical format when it should be canonical. (PR 13093)

- Race condition exists when the Spanning Tree blocks the LANE port the LEC will delete all connections which includes some incomplete connection setups. (PR 13104)

- Two Token Ring ports, each in separate groups, defined as different virtual rings, are bridged together via an external Source Route bridge. Unicast Source Route traffic will loop from 1 port to another then through the external SRB over and over causing a storm. (PR 13145)

- When trying to delete a WAN routing service the switch can crash if one of the PVC’s is either non-existent or inactive. (PR 13192)

# IBM Nways 8274 LAN RouteSwitch Version 3.2 Problem Resolutions

The IBM Nways 8274 LAN RouteSwitch Version 3.2.1 known problems have been fixed in this release.

- The command fwtvl shows all host addresses twice. (PR 7093)

- Dynamic elans do not switch over when primary link goes down. (PR 10137)

- On-going sessions do not resume upon physical reconnect of the PC. (PR 10271)

- Both Fast and SlowPath does not handle IP EthII to EthSnap translations. (PR 10489)

- Failed to receive multicast traffic within the same multicast vlan. (PR 10506)

- "modvp,"change bridge mode parameter, get error message, cannot exit. (PR 10538)

- Start avlans, no connection is made and all activity seems to stop. (PR 10558)

- Unplugging the client's port does not remove the client from the authenticated group. (PR 10591)

- Occasionally, can not "unmirror" ports on 32-port mammoth modules. (PR 10666)

- Port mirroring on Mammoth card will start up, then halt. (PR 10764)

- Nways RouteTracker Manager rules remain for authenticated group (PR 10805)

- Duplicate mac on RouteChannel does not work. (PR 10831)

- Nways RouteTracker Manager on RouteChannel does not work. (PR 10832)

- pmon and port mirroring causes packets to flood. (PR 10838)

- Multicast traffic floods all ports in Group; not just MC VLAN ports. (PR 10891)

- Logging out from authentication flushes all clients on the same port (PR 10918)

- Setting gmMoveToDefGroup to off fails. (PR 11008)

- Setting gmDefGroup to off fails. (PR 11009)

- Setting groupMobilityStatus to off fails. (PR 11010)

- Upon deleting a leaf on svvc, received "error obtaining the indices…"& leafs connection. (PR 11031)

- Broadcast with incrementing source is not flooding. (PR 11118)

- Soft PVCs calls are not going through the second advertised route when first route disappears. (PR 11145)

- 8277-13 SlowPath IP or IPX routing very slow. (PR 11214)

The IBM Nways 8274 LAN RouteSwitch Version 3.2.2 known problems have been fixed in this release.

- Port Monitoring byte count displayed is not accurate. (PR 6211)

- In certain networks where MAC address time out of the MPM CAM frequently (several timeouts per second), there can arise situations where the VLAN memberships of remaining MAC addresses are inconsistent across the network until the next VAP update is generated.  This will usually occur on large networks or networks with a lot of instability.  The result is that some VLAN leakage can occur for short periods of time. (PR 7809)

- Port assignment changes on modifies instead of when MAC ages out. (PR 9907)

- Addqgrp 1,2,3 puts the q port in 2 and 3 only. (PR 10670)

- Breaking cable on q port gpmobility moves the port to new group and primary group is lost. (PR 10675)

- Observed Flooded frames come out on 1 out of 4 ports in a group. (PR 10676)

- Can not use multiple IPX encapsulations with authenticated VLANs. (PR 10804)

- "mpmload" caused primary to hang. (PR 10819)

- DVA command causes tui_shell to crash and switch to reboot. (PR 11018)

- Token Ring software will not load with new 32MByte simm. (PR 11089)

- LinkMonitor crashes when ports are looped back. (PR 11101)

- Crashing in snmp_agent with polling applications running. (PR 11124)

- Performing a imgsync after a imgcl will freeze UI. (PR 11151)

- Inability to define mac address in gp 1, when port where MAC is attached is in another group. (PR 11200)

- ILMI goes into race condition between MSS and IBM Nways 8274 LAN RouteSwitch. (PR 11218)

- The switch may reboot with CSM’s and Redundant MPMs. (PR 11444)

- Pmon gives "xMsgQSend failed, 2." Interspersed with data displays. (PR 11508)

- When switch CPU utilization is at or near 100%, and the firewall rules are being downloaded, on rare occasion the switch may abort. (PR 11599)

- Intermittently the Data Direct Vcc can not be established between the LECS in the same group of the same IBM Nways 8274 LAN RouteSwitch, all traffic will go through the BUS via the Multicast Send/Forward Vcc. (PR 11715)

- MRS ipxflush not flushing correctly across triggered connection. (PR 11828)

- A problem exists with the interaction between PNNI and clients registered via ILMI whereby the client may register properly but not be advertised.  This can be seen by PNNI's padj command when the address is present in the display but the Advertised flag is set to "False - on port up".  In reality, the port is up, but PNNI will not advertise it due to a timing problem. (PR 11954)

- ILMI responds with nosuchname for uni version request. (PR 12003)

The IBM Nways 8274 LAN RouteSwitch Version 3.2.3 known problems have been fixed in this release.

- Configuration of a Group without assigning any ports to the Group will result in the routing tables displaying the Group’s IP network as being available. (PR 549/551/699)

- Token Ring IP Client loses connection when IP virtual router port frame type is set to Ethernet SNAP (8).  (PR 3982)

- VLS and VLRS show port zero on a CSM module with dropped cells. (PR 8000)

- The representation of the LEC in the MIB-II if Table is not working properly. (PR 8159)

- When neighbor node have their Ids reconfigured, their old Nodal Information Group persist for up to 60 mins. in the net. (PR 8632)

- The status viewer of Check Point’s EMC (console) aborts if the automatic update selection value is set to a small interval, such as 1 minute.  (PR 9134)

- User Plane connection & bearer class do not get set according to user. (PR 9611)

- MVC does not change the end point ID. (PR 9933)

- SNMP doesn't allow a set of a non-well known address, where the default is the well known address, for either the LECS or LES. (PR 10018)

- After deleting AT rule, you cannot create another port binding rule using mag command. (PR 10076)

- Soft PVCs will not work if secondary MPM becomes primary. The Soft PVC stays in connecting state. (PR 10129)

- When using the NMS station, selecting “manual mode”, and a manually inputting the LES/BUS address, the SNMP agent doesn't use it correctly.  Instead it uses zeros.  An entry exists with zeros in it, and it is displayed using the VVA command. (PR 10181)

- Neighbor Port Type (fport) does not reset after disconnect. (PR 10630)

- Automatic network object updates crash the FW. (PR 10737)

- ASM LEC stays in initial state after hot swap of attached Xcell’s FCSM. (PR 10780)

- Command svvc shows a configured Soft PVC as a PVC connection when leaf is not connecting. (PR 10855)

- If the MAC address of a device changes, the old MAC address may not be flushed from the MPM CAM.  This can occur if the router framing type is Token Ring, FDDI or Source Route Token Ring. (PR 10909)

- Duplicate Mac, if Cam table fills up, macinfo will show wrong slots. (PR 11079)

- In IBM 8277 Nways Ethernet RouteSwitch, router device packets incorrectly trigger VLAN 1 assignment.  VLAN 2 is configured with an IP Network Address rule only.  NT and Novell servers join both VLAN 1 and VLAN 2 when RIP/SAP is active, even though they only match the IP Network Address rule. (PR 11248)

- When configuring a secondary LES/BUS, it is not necessary to enter the primary LES/BUS address as required in the configuration menu. If you do so, the address will be different the next time you want to modify it. This will not affect the normal functionality. (PR 11375)

- "vib" command does not show LES/BUS security status. (PR 11432)

- Port Monitoring over token ring doesn't filter out multicast packets. (PR 11500)

- modvp - using next  (n) will cause changes to mirroring to be automatically saved. (PR 11539)

- When a station is moved to a different slot, the connection could be lost.  If a device assigned to multiple mobile groups is moved from one slot to another, and transmits traffic for only one of the groups, it gets moved for only that group.  It still remains learned on the old port for the remaining groups, until it generates traffic for those groups also.  As a result, this station is not accessible by these remaining groups. (PR 11743)

- When user exits from the mas command on 802.5 LANE, a long line of zeros is displayed. (PR 11749)

- ATM Port Properties BWG returns invalid values. (PR 11937)

- When 2 IBM Nways RouteSwitch CIP SVC clients call each other at the same time, 2 SVC's get established, but traffic is not forwarded out of the correct VC for the other side to receive.  This results in the 2 CIP clients being unable to communicate to each other.  (PR 12013)

- UI displays all SVC connections as UBR irrespective of their actual QoS. (PR 12099)

- Console error messages appear after changing FDDI Operation Mode and then virtual port parameters. (PR 12114)

- Frames transmitted on a mirrored port from a port with 802.1q enabled will have 4 extra bytes. (PR 12173)

- IP Routing table, which is retrieved with IPR command, displays "NO ROUTING" entry for active IP route for VLAN router port. (PR 12262)

- "pmp" gives wrong prompt to resume monitoring after pausing. (PR 12372)

- With Autoencaps on, swchmac command only shows the translation for the first instance of the MAC. (PR 12385)

- Ethernet/DHCP client cannot obtain IP address from a Token Ring/DHCP server when source routing is enabled. (PR 12419)

- The vpi field in prpadd shouldn't be available to user to change when the route property is added over a tunnel.  This is because the VPI is specified on the tunnel creation and the PRPadd command should not allow it to be changed. (PR 12461)

- It is not possible to set/view the pncfg summary via snmp. (PR 12469)

- The IBM-port.mib defines 16 as the maximum number of ports on a module, however  the switch will allow up to 32 ports.  Therefore, the MIB needs corrected. (PR 12486)

- Deleting a mobile group with VLANs does not clear all the components. (PR 12507)

- Adding a trap handler entry to the trap Table disables broadcast traps if they were enabled. (PR 12531)

- When there is not enough space on secondary MPM, primary gets MPM exception when utilizing "imgsync". (PR 12541)

- Hot swapping FCSM module may result in data exception errors with secondary MPM. (PR 12556)

The IBM Nways 8274 LAN RouteSwitch Version 3.2.4 known problems have been fixed in this release.

- Setting Frame Relay Phys Ports Adm Status = Disabled has no effect on Virtual Ports. (PR 4068)

- When the UNI version is changed and the option to reset the board is chosen, the change does not become effective.  User must reboot the switch to make a change in UNI take effect. (PR 4464)

- Vvc and vcs do not work for individual connections on FCSM board. (PR 7380)

- Using a SNMP GetNext command to retrieve an empty ATM table can result in the switch searching through all the subsequent entries, resulting in the switch appearing to be locked out. (PR 8043)

- Certain IBM Token Ring devices, that attempt to use express buffers, will have problems since our R LEC uses FC = 0x45 (5). (PR 8249)

- Upon removing OC12 connection, the signaling channel stays in the hardware and does not get flushed. (PR 8268)

- ISDN call will not drop if IP is not configured or configured on both ends. It will be necessary to admindown and adminup the interface to disconnect the call. (PR 8339)

- No support for terminal display text.  It will inhibit calls from being connected. (PR 8715)

- After removing a CSM module, all CSM commands show either ports or channels on a slot. (PR 8812)

- Using RouteDirector, metric tags of route addresses are always returned as 0. (PR 8850)

- User RouteDirector, the ifIndex field is always returned as '1 if a static route is created by the Director.  This prevents addition or deletion of routes by RouteDirector. (PR 8851)

- Can not use NetWare login with authentication using XVSS client. (PR 9300)

- Modifying SVC service and saving can cause display of large number on exit.  This doesn't affect the actual operation of the SVC. (PR 10132)

- New SNMP management agent support for 10/100 modules through Route-Vision. (PR 10387)

- Added new ATM DS3 and E3 port type definitions to the IBM-port.mib (phyPortTable). (PR 10535)

- When a WSM Universal Serial Port (USP) is connected to a CSU/DSU, in some setups, power-cycling the CSU/DSU can cause the WSM card to reset, and in some cases result in the switch auto-rebooting. (PR 11002)

- pmon, if traffic load high, display to screen will display nothing. (PR 11108)

- RARP request hardware type incorrectly translated from FDDI to Ethernet. (PR 11195)

- The serial port on the BRI daughter card does not properly implement the EIA signalling with the DCE cable.  It may be necessary to unplug the cable in order to bring the cable up. (PR 11475)

- Multicast routing packets are not forwarded to Gated on the IBM 8277 Nways Ethernet RouteSwitch. (PR 11692)

- Source routed DHCP reply has destination MAC in canonical format instead of non-canonical, resulting in the client not receiving the reply. (PR 12098)

- End station on SR port will not get connectivity if SRT in path. (PR 12105)

- The flag "atm_use_Mbus-3" is not functional in IBM Nways 8273 Ethernet RouteSwitch only. (PR 12182)

- Group Mobility over Source Routing IBM Nways 8274 LAN RouteSwitch environment does not work. (PR 12294)

- PNNI does not always use the preferred Shortest Path during Load Balancing when topology changes (such as an unplugged link). (PR12492)

- VE command shows error discards out on LSM port. (PR 12516)

- CIP service using SVC does not come ack up when the cable is disconnected and then reconnected between the switches. (PR 12761)

- Directly connected Ethernet IPX interfaces don't time out.  When the Ethernet cable that connects the two switches is physically removed, all remotely-see networks in the IPX routing table (w/ipxr) are immediately set to 16 by the switch. (PR 12837)

- IPX RIP table takes 9 minutes to time out. (PR 12838)

- If router encapsulation is set to FDDI-SNAP or TOKEN RING-SNAP and the output ethernet port encapsulation is set to ETHER2, the packet wrongly gets translated to 802.3. (PR 12877)

- No trap message is sent for detection of our own IP address being duplicated. (PR 12879)

- When a duplicate MAC address for a router port is detected, we need to send an SNMP trap. (PR 12880)

- This is a PER to allow SAP control of NET side ILMI polling. (PR 12914)

- Can't view ILMI registered addresses via Route-Vision.  Added MIB support for 'padj' command in the Route-Vision (PR 12942)

- Source routing does not work over Token Ring LEC with group mobility enabled. (PR 12944)

- RIF stripping not working on 802.3 LANE when Token Ring-port configured for Source Route Transparent. (PR 12965)

- Imagesync command displays and error saying that there is not enough memory on the second mpm, but the ls command shows 1.99M free. (PR 12966)

- Two switches are connected with a PTOP service and a LANE service over the same media. LANE spanning tree cost is 8 and PTOP is 10. Based on cost the PTOP service goes into blocking and the LANE is forwarding. When the LANE connection is broken the failover to the PTOP service works. When the connection is reestablished, the PTOP goes back to blocking, and the LANE service is in forwarding state, but no traffic is forwarded. (PR 12988)

- Exiting from the dshell/ui at an inopportune time can cause the excTask to die due to an unaligned access in the ui_do_dshell_demise routine. (PR 12994)

- RMON probes supports 99 maximum unique Entry Identifiers (3 ESM-32 cards and 3 more ports). (PR 12997)

- RIP version 1 is failing to advertise static routes. (PR 13009)

- ESM-F-16 cards lockup when running group mobility. (PR 13015)

- ASM portion of FCSM stops transmitting frames on all VCs, when An Ethernet MAC was learned using a cam index that was previously used by the ATM magic MAC. (PR 13017)

- Fake VCI on signaling portion of FCSM. (PR 13018)

- Default heap sizes for RMON are not sufficient for all supported interfaces. When 256 Ethernet ports are activated, system will only generate 59 Ethernet probes. Memory is quickly exhausted. (PR 13021)

- In redundant Token Ring environment Token Ring CAM fills up and dup macs are caused in same group. (PR 13048)

- Add support for demux reset h/w change on CSM622 boards. (PR 13049)

- "mpmload" request causes tCMMaster to dump on MPM. (Scopus 20309) (PR 13072)

- Non-ATM ports Logical layer's admin status are down. Shows as disabled via the vi command. (PR 13078)

- Null RIF in FDDI/Trunking environment cause CPU to have high utilization. (PR 13083)

- Multiple ping sessions can cause a crash. (PR 13084)

- Crash with 100FX card and long system names. (Scopus 20274) (PR 13086)

- LANE crash when changing traffic from mac ports. (Scopus 20324) (PR 13104)

- Switch with Mammoth E12 card crashes when the ifTable is polled. (Scopus 20275) (PR 13111)

- PNNI Call Distribution does not work, when equal cost paths to the same address. PNNI always takes the first path in the routing table. (PR 13113)

- PNNI path selection is not working. PNNI takes longer route paths. (PR 13114)

- Round Robin Call Distribution over redundant IISP links does not work. (PR 13115)

- When changing configuration parameters in the export statement, Gated takes an assertion error and aborts. (PR 13116)

- Add LE_NARP support. (PR 13120)

- HPUX stations not responding to LE-ARP requests when configured as Optimized mode and using group mobility. Bridge mode works fine. (PR 13127)

- LANE never comes up by setting atm_use_mbus=3. (PR 13139)

- Two Token Ring ports, each in separate groups, defined as different virtual rings, are bridged together via an external Source Route bridge. Unicast Source Route traffic will loop from 1 port to another then through the external SRB over and over causing a storm. (PR 13145)

- The value returned for the SNMP variable atmLeArpMacAddress is ASCII, should be hex. (PR 13151)

- Selector 0 not allowed when creating a ATM service. (Scopus 20315) (PR 13194)

- The 8277-13/416 do not have a vbus and it should not be turned on. This can causes CAM initialization failures, and the switch reboots. (PR 13203)

- Add software keep alive for LSI Gore voltage leakage problem. (PR 13207)

- Connectivity problem on some FDDI ports due to Virtual Port overlap memory map. (PR 13211)

- UI crash if user configures duplicate IPX network number for router ports. (PR 13212)

- Wrong board type (ESM-T24) included in 3.2.4.6.A. (PR 13213)

- DS3 LED don't initialize on boot up. (PR 13243)

- Loss connectivity with SRT and SR only 802.5 elan when spanning tree is off (Scopus 20391) (PR 13245)

- In Source Route environment Virtual Rings do not work properly. (PR 13248)

- Moving a port from one group to another may cause spanning tree topology change. (PR 13261)

- Gated reconfig doesn't work when reconfigure more that 10 interfaces (Scopus 20418) (PR 13264)

- Static IP routes disappearing on reboot due to split route information being saved in the configuration file. (PR 13271)

- Duplicate MAC entries caused ATM Magic MAC’s to get corrupted which cause all ATM services to go down when only half the 4K CAM of the FCSM is filled. (PR 13275)

- Allocated memory block header corrupted, need some test code to diagnose further. (PR 13276)

- Allow disabling firewall filters on outbound traffic, to improve performance. (PR 13281)

- Spanning Tree becomes disabled when TRSR port moves into new Mobile Group. (PR 13283)

- LeArp displaying invalid values in SNMP. (PR 13284)

- Link Table (via SNMP fails to display all links when first link is removed) (PR 13301)

- Redundant MSS fail-back process. LEC’s from secondary to primary MSS, not all LEC’s rejoin. (PR 13308)

- Redundant MPM 1Gs w/32 meg causes a data exception error on boot up when the diag.img file gets loaded. (Scopus 20379) (PR 13331)

- Client telnet session does not check to see if it is trying to setup the session using a null pointer in the call to the function set_listitem (). (PR 13356)

The IBM Nways 8274 LAN RouteSwitch Version 3.2.5 known problems have been fixed in this release.

- Configuration of a Group without assigning any ports to the Group will result in the routing tables displaying the Group’s IP network as being available. (PR 549)

- Configured 2 or 3 ISDN backup for only one primary. When primary is down, all ISDN configured back up for that primary should be triggered. (PR 8717)

- When the Check Point Management Station (EMC) and the switch firewall are attempting to communicate through a router, the switch will sometimes fail to establish connectivity with the EMC.  (PR 8722)

- User RouteDirector, the ifIndex field is always returned as –1 if a static route is created by the Director.  This prevents addition or deletion of routes by RouteDirector. (PR 8851)

- QOS Classes for metric tags are created by value number instead of bit position as mib states.  This allows for only UBM classes to be successfully put into a tag. (PR 8852)

- SNMP query of gmGroupListTable returns primary mobile group values only when multiple groups are defined. (PR 8946)

- RMON stats under swch command are overwritten when Transmitted Good Octets exceeds 10 digits. (PR 10844)

- Setting the Switch system time back will cause the system uptime to reflect incorrect date/time. ILMI Polling sees a decrease in SysUpTime ticks and sends cold starts to the ATM attached LEC. (PR 11346 & 13598)

- Failure counter under frs stays at 1. (PR 11552)

- Changing settings with SER command are not kept after reboot. (PR 11659)

- Saving a change in pgcfg and pncfg may result in the system running out of memory and crashing on live networks. (PR 11725)

- UDP relay does not function with Nways RouteTracker Manager VLANS (3.4 Feature).  (PR 11848)

- Unable to add static dup mac to multiple lane client service instances (PR 11990)

- If set FR default bridge mode to zero, wipes out all WSM services. (PR 12259)

- Make RxBusy configurable on Mammoth for ATM CAM problem. (PR 12450)

- PNNI does not always use the preferred Shortest Path during Load Balancing when topology changes (such as an unplugged link). (PR 12492)

- When the primary MSS is disconnected, the secondary MSS maintains the redundant VC with the primary MSS. (PR 12614)

- Need to add a sap filter to allow for separation of traffic on the edge switches. This would allow the source routed traffic and IP traffic to not go where it isn't wanted. (PR 12775)

- Gated.conf is not copied during configsync. (PR 12889)

- Not able to ping end-station when Source Routing is enabled and the group is being trunked over IBM FDDI trunking. (PR 12922)

- After changing to the '/simm' directory, with logging enabled, issuing commands causes utmp errors to console. (PR 12963)

- Make soft PVC retry timer resolution to 10th of a second. (PR 12964)

- When an Ethernet MAC was learned using a cam index that was previously used by the ATM magic MAC, the ASM portion of FCSM stops transmitting frames on all VCs, when (PR 13017)

- In a redundant Token Ring environment, Token Ring CAM fills up and duplicate MACs are caused in same group. (PR 13048/20307)

- PNNI Call Distribution does not work.  PNNI always takes the first path in the routing table even if it's loaded and another redundant equal cost path is available. (PR 13113)

- When changing configuration parameters in the export statement, Gated takes an assertion error and aborts. (PR 13116)

- FDDI-SH, part number 050108-15, Module type 51 is not recognized by chassis manager. (PR 13141)

- CIP goes into "enabling" when reboot or disconnect. (PR 13175)

- GmAutoServiceTable does not have value for active slot/port. (PR 13189)

- GmGroupListTable has entries of non-exit ports. (PR 13190)

- After a reboot, the ATM LEC's join, release, rejoin; the Delay timer is suspended; and the LEC gets torn down.  This appears as an ILMI timing issue. (PR 13176)

- Upgrading WSM flash ROM fails to load.  WSM ROMS should be verified against the current version and upgraded, if necessary, before loading 3.3.1. (PR 13225)

- SNMP "Get" fails on pnnixRouteAddrTable, returns "Error code set in packet - Return packet too big". (PR 13238)

- VSE initialization failed on older ESM-C-8 card in even slots. (PR 13240)

- PNNI Race Condition w/50 PNNI neighbors. (PR 13266)

- ISDN with ST interface, second time the cable is unplugged it stops working. (PR 13272)

- Setting next-hop for generic IP services causes MPM crash with dump file. (PR 13278)

- From the ‘relay’ command, when a forwarded port number greater than 255 is configured, port number gets truncated to char. (PR 13279)

- LeArp displaying invalid values in SNMP agent. (PR 13284)

- Inactive and active ports should move into Mobile Groups with port rules for those ports.  However, after reboot of switch, port was back in Group 1 and did not move into Mobile Group, even though port rules were still defined. (PR 13309)

- Add capability to configure locally administered MAC addresses on virtual ports. (PR 13355)

- ATM service staying in Initialized until triggered. (PR 13367)

- PNNI Agent - Static Routes created on ports that are not active via Route-Vision do not appear if not advertised. (PR 13368)

- PNNI agent - IISP routes can't be deleted or modified. (PR 13369)

- PNNI agent Route Node and Addr Tables: Info should be 0.0.  (PR 13370)

- In presence of IISP link failures, PNNI should re-route over viable paths of greater cost. (PR 13384)

- PNNI task (tPNNI_1) intermittently crashes in cmPrcTmr(). (PR 13398)

- PTOP bridging with SAHI boards result in TX SDU discards and impacting performance. (PR 13401)

- After a period of time, 'ipmac' command reveals an IP associated MAC table to be corrupted. (PR 13406)

- IPX Ethernet routes have the same delay as ISDN, even though the ISDN line is a lot slower. Delays should be configurable in order to direct traffic across the most desirable route. (PR 13408)

- Using Route-Vision, IISP Static routes are displayed on the wrong Slot/Port. (PR 13412)

- When deciding to send 'duplicate MAC' trap, ignore frames that are AREs or STEs. (PR 13421)

- Routers with high amount of RIPs and SAP can consume system utilization. (PR 13426)

- For token ring DHCP server, the client hardware address in the DHCP packet gets swapped to canonical on the server side which causes a problem for Solaris DHCP server. (PR 13431)

- Empty atmxaddresstable causes NMS to timeout. (PR 13439)

- SNMP-10/100 Autonegotiation changing to fixed speed is not the same as the UI. (PR 13443)

- Support Internal Loopback Test for CSM, Mammoth, ASM2 and IBM 8277 Nways Ethernet RouteSwitch. (PR 13453)

- Add port stress test to diagnostics. (PR 13459)

- Call setup code is accessing a little past end of structure preventing proper H.310.  LEC video standards to me met. (PR 13463)

- Software fails to detect port up/down on CSM-AB-155 ports. (PR 13468)

- IPX over PPP won't negotiate with 3Com NetBuilder. (PR 13475)

- Unstructured T1 circuit emulation circuit intermittently gets bit errors. (PR 13479)

- Add MIB support for Designated Transit List. (PR 13480)

- Add code to handle CR and slot/port 0 input in ui_do_csm_cmvc(). (PR 13488)

- Add parameters to tweak SNMP CPU-based backoff. (PR 13490)

- MIB Support for trap request related to Route mis-configuration. (PR 13497)

- We need to be able to use a specific port for each radius server since some may be using different ports.  The default is 1812/1813 but most use 1645/1646. (PR 13503)

- cratvl' can create non-routing atvlan. (PR 13506)

- Bogus mac addresses learned on ATM uplink upon initialization. (PR 13508)

- dshell created by '^W^W' crashes the switch. (PR 13509)

- Changing password for admin on primary MPM does not roll to secondary MPM until reboot. (PR 13510)

- IBM Nways 8273 Ethernet RouteSwitch with WSM and 100BaseT does not boot up. (PR 13519)

- AMD Flash Defect - Last Row Charge Gain enhancement. (PR 13523)

- Current config_dump() adds actual symbol names to the dump. (PR 13525)

- No longer able to telnet to switch when ioctl call fails. (PR 13531)

- Cannot ping in/out of several ATM edge switch. (PR 13532)

- Setting Rx Best Effort is not allowed on ASM modules, but the UI menu option is displayed. (PR 13536)

- WSM daughter card generates inaccurate timer ticks to WSM NI. (PR 13537)

- PPP Peers created on WSM ports can cause the switch to crash. (PR 13552)

- ATM PTOP Bridging services disappear after ATM-SAHI hotswap. (PR 13559)

- “imgsync” command  will return (FILE SYNC incomplete ... try again). (PR 13567)

- DHCP requests are not being forwarded out of 802.1Q ports. (PR 13582)

- Prevention of TX processor crashes resulting from "odd byte alignment" boundary conditions while running 802.1Q packets. (PR 13583)

- Without configuring the Radius server with the L2 Authentication II software, I attempted to authenticate with a Radius password/userid. The switch would reboot. (PR 13603)

- With the firewall enabled on the switch at the same time L2 Authentication II is active, the switch reboots. There needs to be a check so that the switch doesn't reboot. (PR 13605)

- Support is needed for IBM 8277 Nways Ethernet RouteSwitch 5024 with Mammoth II's. (PR 13624)

- Extra "Enters" when using the ping command can cause error message to be displayed. "Ping_info". (PR 13630)

- "avlsAuthTime" has "user" misspelled as "uer" in the provided description. This should be fixed prior to release of this level. (PR 13645)

- Under some configurations of ATM, the box will sometimes lock up or crash. (PR13651)

- Frame relay services don't work with vport over 128. (PR 13676)

- OSPF doesn't come up to full for 10 min. in simple case or over 30 min with virtual link on PNNI. (PR 13677)

- Diagnostics test for SUNI fails on CSMU-CAB-E1. (PR 13695)

- 802.1q does not recognize MAC address when moved to another q port. (PR 13707)

- Das modified LANE Service wipes out services/ports. (PR 13709)

- 8021Q port transmits two corrupted bytes in packets from an invalid pointer when bridging from an FCSM II to 8021q port.  Packets are corrupted on tranmit. (PR 13723)

- CTRL-C does not about the UI command processing of the "avlr1" command. CTRL-D will abort the processing, but so should CTRL-C. (PR 13761)

- TSM modules reset with NI dumps. (PR 13776)

- IfAdminStatus always down for CSM ports. (PR 13787)

- Increase default value for IP-MAC aging timer to match Route-Vision find timer. (PR 13826)

- Need to the ability to turn off the task real credit algorithm.  (PR 13835/20847)

- Chassis manager reports the cost of the RESM-100FM-FD-1 is set to 100 instead of 10. (PR 13844)

- Adding DHCP MAC rules, through Route-Vision, for a group (verify with UI command atvirl that Route-Vision was successful). Reboot the switch and observe that the second and the third DHCP MAC rules have disappeared. (PR 13846)

- Filtering-database not updated correctly in SRB environment causing packets not to be forwarding.  Must be in a redundant environment. (PR 13871)

- Edge switch looses its IP routes when the PNNI core switch is rebooted, and never regains them, even though the all VC's are reestablished. (PR 13885)

- IP ping over ISDN-PPP often crashes "tNetTask". (PR 13896)

- When creating new groups with IP routing enabled, they come up as routing disabled. (PR 13923)

- UDP packet received counter not advancing when receiving RIP packets. (PR 13925)

- DDVCC maintained until MAC ages out of VLAN. (PR 13930)

- Change default clocking mode of circuit emulation to Adaptive clocking. (PR 13935)

- Logging in to secondary MPM as Diag causes UI to hang. (PR 13947/20922)

- When creating a Mobile Group, user is prompted to enter a VLAN Description if they are adding rules to the group. However, the description entered does not appear as VLAN description in atvl table, or anywhere else. (PR 13998)

- Rip active on Vlan does not trigger ISDN call anymore. (PR 14004)

- Problems bringing up 125 dlci's on a frame relay port. (PR 14019)

- Port Mapping not functional after reboot although the config seems to be intact from UI.(PR 14028)

- With Rif stripping=on the cmd file, FDDI over ATM is broken. (PR 14032)

- After running a while under high load, users are unable to login to the switch.  You can enter your username and password but nothing ever happens.  The switch continues to run but the UI shell hangs. (PR 14061)

- Various msgQ variables are dereferenced, but never initialized in vse recv. msgQ messages are displayed on the console. (PR 14066)

- Crash dump fixed for tRiptimer(). (PR 20240)

# Known Problems

The problems listed here include problems known at the time of the product’s release.  Any problems not discussed in this section should be brought to the attention of the IBM Technical Support organization as soon as possible.  They may be contacted at (818) 880-3500 or 800-IBM-96 (99526-96), International (818) 878-4507.

## Known Advanced Routing, GateD & Routing Problems

- Problem Report  4122

The total datagrams received and total PDU’s requested for transmit statistics displayed in the ips command can be incorrect.  The switch uses TCP packets to enforce paging for the user interface displays, which causes these statistics to increase even when no external IP traffic is being received or transmitted.

Work-around:	Turn off the switch “more” function, which allows multiple page user interface output to be displayed one page at a time.  This is accomplished by issuing the “nomore” UI command.

- Problem Report 5611

Using mask with exact or refines as parameters in route filter do not work together. When the gated.conf file contains:

import proto rip interface xxxx {192.206.184.0 mask 255.255.255.192 exact;};

It should match only the network and not the subnet or hosts of the network.  But the network address is not accepted.  Replacing exact with refines should match all subnets and hosts but only matches hosts.  None of the subnet addresses are accepted.  This is explained in section 4.25.2 of gated conf. Guide.  The route filters do not work correctly.  A class C address with the mask of 255.255.255.240 only accepts subnet 0 and its hosts.  It does not accept subnets 16, 32, etc.

- Work-around:	NONE

- Problem Report 6060

- OSPF does not pass on the route tag in its external LS advertisement.

- Work-around:	NONE

Problem Report 6534

OSPF generates with wrong next hop for routes over Frame Relay.

Three switches within the same network, running GateD routing environment, connected via Frame Relay.  When the three switches are connected without a loop via Frame Relay, the top switch (A) advertises the route from its neighbor (B) to the other neighbor (C) with the next hop being switch B.  Since there is no physical connection between switch B and C, the advertised network becomes unreachable.

- Work-Around:	This configuration cannot be expected to work in normal IP routing environments.  This requires changes to the way routing works in a non-meshed frame relay network.  Those changes have been made in the RIP routing supported by the base IBM Nways 8274 LAN RouteSwitch software.  If this configuration is required, use the IBM Nways 8274 LAN RouteSwitch RIP implementation, not the Advanced Routing package.

Problem Report 6947, 6943

IPX clients were unable to connect to any IPX server when the router port encapsulation was set to SR_FDDI_LLC or SR_FDDI_SNAP and the client and server did not have source routing enabled.

- Work-Around:	Do not set router port encapsulation to either SR_FDDI_LLC or SR_FDDI_SNAP if source routing is not configured on the bridge ports.

Problem Report 7063

ipxping does not work over FDDI trunking with TR_SNAP & TR_LLC router port encapsulation.

- Work-Around:	ipxping does work using TR_SNAP & TR_LLC router port encapsulations over other media types.

- Problem Report 7448

- IBM Nways 8274 LAN RouteSwitch switches RIP II multicast packets instead of consuming them entirely.  This causes no noticeable problem.  This only causes slight increase in traffic on the network.

- Work-Around:	NONE

- Problem Report 7534

- OSPF stops when a group is removed from DR Other.  It happens after a Link State Update due to the DR Other not sending any more Hello packets resulting in the lost of adjacency.

- Work-Around:	Do gated reconfig DR Other, after having removed the group.

Problem Report 9801

OSPF neighbor list is not displayed with nonbroadcast interfaces.

- Work-around:	NONE

Problem Report 10012

Gated RIP v1 and v2 never remove peers from "rip peers" table.

- Work-around:	NONE

Problem Report 10380/12016

Routing Tables are not updated with PPP connection to CISCO (OSPF).

- Work-around:	Configure nonbroadcast media type for your PPP interfaces.

Problem Report 11560

OSPF ASE routes show up as STATIC with IPR command.

- Work-around:	NONE

Problem Report 12847

Can not das FR trunk from mobile.

- Work-around:	Delete the Nways RouteTracker Manager rule(s), before attempting the das.

Problem Report 13044

Sometimes GateD aborts during gated reconfig on the IBM Nways 8273 Ethernet RouteSwitch.

- Work-around:	NONE

Problem Report 13124/13133

Commands ipxr and ipxsap routing table result error message: IPX display routing table xMsgQSend failed.

- Work-around:	Insure that all boards in switch that do not load correctly are removed, and then reboot.

Problem Report 13132

ipxping does not work for the specified destination network IPX address.

- Work-around:	NONE.

Problem Report 13152

When a 802.1q group is deleted from a port, other 802.1q groups on that port also get deleted.

- Work-around:	NONE

Problem Report 13153

Error message "Error! Vport…" continuously displayed when more than 16 groups are added to a Q port.

- Work-around:	NONE

Problem Report 13167

Incorrect virtuallink display.  When virtuallinks are configured within OSPF, the "o vl" display shows the state as 'P2P' when it should display 'Full'.

- Work-around:	Do not configure more than 15 802.1Q groups on a port.

Problem Report 13217

Disabling IPX serialization filter does not take effect until after reboot.

Work-around:	NONE

Problem Report  13708

When multiple telnet sessions perform an IPXSAP command, the switch crashes.

- Work-around:	NONE

Problem Report 20050

- Laptop clients using the Xircom or Megahertz ethernet PCMCIA cards and running solely the IPX stack with the framing type set to autonegotiation might experience intermittent connectivity issues connecting to IPX servers.

- Work-around:	NONE

- Problem Report 20909

After a reboot, RIP doesn't advertise internal router instance.

Work-around:	Turn rip off on each routing instance that does not work.  Then turn each of them on.  They will function from that point on until the next switch reboots.

Problem Report  IBM5871

A Router using ATM on one interface and Ethernet on the other can cause MPM utilization of 100% if moderate levels of traffic exist on the Ethernet Port while the ATM interface is down.

Work-Around:  Stop traffic on Ethernet ports  - Reboot switch

## Known ATM Problems

- Problem Report  2332/11277/13090

When configuring an IPX router port for source routing, the IPX ping across a trunk port will not work.

- Work-around:	Disable source routing on router port.  Use a non source routed station to manage the IBM Nways 8274 LAN RouteSwitch through a non source routed VLAN group.

- Problem Report 4335

No mib support for atmAddArpEntry for CIP services.

- Work-around:	NONE

- Problem Report 4730

Peak cell rates do not get initialized on an ATM-DS3 module if ATM-OC3 module was previously in the same slot.

- Work-around:	NONE

Problem Report 11691

- When an ATM module is running out of MAC addresses, user cannot create new ATM service.  There are times for trunking or x-lane service where the user may want to trunk more than one group.  Each group requires a separate MAC address.  The service will not be successfully created when trying to create a trunking service to trunk 5 groups with only 3 MAC addresses left on the slot.  But if user does a vas command, this partially created service will remain and will not be able to delete this service.

- Work-around:	Delete the mpm.cnf file and start from scratch.  To prevent this, always check to see if there are enough MAC address available before creating any new service or to add groups to trunking services.

Problem Report 11736

Command masrt does not support removal of ATM addresses.

- Work-around: 	Use only if using registered LECS address.

Problem Report 13066

ATM-DS3 and E3 modules may cause a memory protection error and lock up after sublayer changes.

- Work-around:	NONE.  If memory protection error occurs, user has to reboot the switch.

- Problem Report 13392

With a large number of LECS, the switchover between primary and secondary is slow.

Work-around:	NONE

- Problem Report 13942

When a virtual connection is created with an out of range VCI, an entry is created in the ATM connections table anyway.

Work-around:	NONE

- Problem Report 20884

Switch hangs after issuing memshow 1 command from dshell.

Work-around:	NONE

## Known Authenticated VLANs Problems

### 34.3.1	Client

Problem Report 8321

If a user is defined with a third party authentication mechanism such as SecureID and it’s not in place, the telnet client hangs.  The AVLAN debug message on the switch shows "Trying to send to client SecureID request failed".

- Work-around:	NONE

- Problem Report 13502

Avlr1* and avlrm* commands need defaults defined.

Work-around:	NONE

### 34.3.2	Switch

Problem Report 9177

Switch remains in connected state long after AMC goes down. If only one switch is connected to the AMC, it will "unconnect".  If there is more than one switch connected to the AMC, then only one switch "unconnects" and the other switches stay "connected".

Work-around:	NONE

Problem Report 10126

When a switch is configured to run VLAN Authentication only, the EMC Firewall can still get control of the switch and download rules.

Work-around:	NONE

Problem Report 11786

Toggle switch for the *status bar* doesn't work properly.  If you toggle the status bar and tool bar, the status bar does not *hide*.  If you toggle on the tool bar while the status bar is still toggled off, then the status bar will *hide*.

Work-around:	NONE

## Known Nways RouteTracker Manager Problems

Problem Report 10618

The virtual router port for a group is not assigned when VLAN ASCII configuration file is used.

Work-around: 	NONE

Problem Report 10623

The group name and group administrative status' are not read in from VLAN ASCII configuration file.

Work-around: 	NONE

## Known BootP Problems

Problem Report 12098

Source routed DHCP reply has destination MAC in canonical format instead of non-canonical, resulting in the client not receiving the reply.

Work-around:	NONE

## Known Bridging Problems

- Problem Report 5854

Modifying the spanning tree forward delay parameter using the stc command may cause the forwarding table entries not to be flushed and relearned properly.

Work-around:	NONE. If you change the forward delay, it is recommended that the switch be rebooted.

## Known CSM Problems

- Problem Report  6806

When using PNNI, and the vap command, the display erroneously shows 65535 configured VPCs on a PNNI link.

Work-around:	Display problem, ignore

Problem Report  9209

PVCs are not removed fully when CSM board is removed. If user creates a PVC connection via SNMP and then later on removes the board which has the outgoing slot/port vpi/vci of the original PVC. The information stays in flash and will appear on all CSM commands with port number equal to -1. This problem also exists for PNNI ports.

Work-around:	NONE

Problem Report  9424

The SVC connections created on tunneled ports appear as PVC connections. Since they appear as PVC connections, the user can delete them. The connections could be the control distribute VC of an ELAN.

Work-around:	Do not delete connections.

Problem Report 9510

The Release Cause in VVC menu doesn't reflect the actual cause code for the release.  It always shows 0.

- Work-around:	Display problem, ignore.

Problem Report 9547

When changing user priority to 0 on a channel, the VVC show user priority of 8.

- Work-around: 	Display problem, ignore.

Problem Report 9664

If a large value for VPI/VCI is entered in scvc for a second try after an out of range value for VPI/VCI, vpi/vci config could show the status "OTHER".

- Work-around:	NONE

Problem Report 10223

LS1010 does not accept Soft PVC setup request from IBM Nways RouteSwitchCell and releases the call with cause code 73.

Work-around:	NONE

Problem Report 10225

User cannot disable a Soft PVC configuration.

Work-around:	Delete Soft PVC

Problem Report 10408

If user mvpt a VP Tunnel multiple times the neighbor state could go to rcc unavailable and never recover unless the VP Tunnel is removed and reconstructed or booting the switches.

Work-around:	NONE

Problem Report 10477

User must not be able to configure VP Tunnels on CSM port of FCSM.  System will allow user to configure it then puts out an error message: "Invalid param - programmer error."

Work-around: 	Do not try to configure VP tunnels on CSM port of FCSM.

Problem Report 11034

Receive the following message when modifying soft pvc connentions:

"Any additions and deletions of Multicast  parties are permanent, cancelling the menu will not cancel multicast changes! "

- Work-around:	NONE, Display Only

Problem Report 11158

The UI does not show the correct traffic descriptors for SVC calls.  It shows UBR for all connectors (cosmetic only, it is correct internally).  (PR 5952)

- Work-around:	NONE

Problem Report 11238

Prevent user from creating conn w/end pt of an existing pvc.

- Work-around: 	Check if existing PVC before creating conn.

Problem Report 11753

If at anytime a crankback occurs on an IISP link, all alternate links (PNNI/ISP) on that node will not be used.

Work-around:	Enable PNNI on alternate link.

Problem Report 12128

Unable to change vpi/vci due to channels not getting deleted from the system software.

Work-around:	NONE

Problem Report 13811

When the user NIC which has a function UniAutoConfig is connected to one port of CSM, the switch does not reply with the right parameters.

Work-around:	NONE

Problem Report 13874

Mvc of SPVC traffic parameters doesn't take effect.  The SETUP message for the latest change has the old values for the modified parameter.

Work-around:	NONE

Problem Report 13960

When the FCSM is hot swapped and swapped back in, some connections are left in memory and not cleared.

Work-around:	NONE

## Known Duplicated MAC Problems

- Problem Report 13129

Duplicate occurrences of MAC are learned.  When all but one occurrence exists, the entry remains marked as a duplicate when only one entry is left.

Work-around:	NONE

## Known Ethernet Problems

- Problem Report 5042

SNMP shows 100BaseT ports as regular Ethernet. There is no distinction between 10 BaseT and 100BaseT ports.

Work-around:	NONE

- Problem Report 13210

IPX packets coming from a 100Mbps NIC causes the whole 10Mbps 32port ESM to hang.

Work-around:	Make sure that all connected hosts are configured for 10Mbps and not configured for 100Mbps.

## Known FDDI Problems

- Problem Report 10759

FDDI client failed to get IP address from DHCP Server.

Work-around:	NONE.

Problem Report 14088

FDDI client using SNAP encapsulation fails to get IP address when DHCP Server is on TR using Source Routing and SNAP encapsulation.

Work-around:	NONE

- Problem Report 20876

RIP frames contain bogus data in fields that 'should be' zero fields for RIP V.1 frames.

Work-around:	Reboot required to solve unreachable networks.

## Known Firewall Problems

### 34.11.1	Firewall Server

Problem Report 11572

When several switches are connected to the NT Management Server, the fw.exe aborts when trying to start the Log view.

- Work-around:	Try to initialize again.

Problem Report 11946

EMC aborts with an access violation when one deactivates the Firewall service while the status window is up and running, and then activates the service again.

- Work-around: 	Bring the status window down before reactivating the EMC.

### 34.11.2	Firewall Switch

- Problem Report 7027

SNMP Fetch must be done twice sometimes on the Check Point Management Station (EMC) when configuring the Switch network object’s interfaces.

- Work-around:	NONE

Problem Report 9322

Firewall display on switch says connected while EMC is down.

- Work-around:	NONE

Problem Report 9876

Changing secondary EMC setting on switch requires reboot.

Work-around:	NONE

## Known Group Mobility Problems

Problem Report 12178

When group-mobility is enabled and the router is configured, the dynamic LANE should be automatically activated for that router.

Work-around: 	NONE

- Problem Report 13309

Port rules don't work after switch is rebooted.

Work-around:	NONE

- Problem Report 13424

ASCII file changes for Sun.

Work-around:	NONE

- Problem Report 13941

When a port is mirrored that has Group Mobility enabled, the port becomes non-functional.

Work-around:	Do not enable port mirroring on a port running Group Mobility.

- Problem Report 20785

When adding a static MAC to a mobile group, station with the MAC address does not receive frames.

Work-around:	NONE

## Known Health MIB Problems

Problem Report 13750

Add Health MIB branch to IBM MIB for compatibility.

Work-around: 	This is for use with 3.4 switches only.

## Known Hot Swap Problems

Problem Report 13012

Hot swapping CSM-A25 while pinging across two CSM-155 modules will result ping time out.

Work-around: 	NONE

Problem Report 13533

When two switches are connected and a hot swap of an ASM occurs, the data direct is lost.

Work-around: 	Reboot remote switch.

Problem Report 13659

After a TSM-CD-6 is hot swapped, the board will reboot itself every time IPX is started.

Work-around: 	NONE

Problem Report 13698

When performing a hot swap on a WSM, FR bridging is disabled and pings timed out.

Work-around: 	wpmodify timing and save to bring back FR bridging.

Problem Report 13840

Need to reboot MPM to enable LEC after hot swap of FCSM.

Work-around: 	Use renounce command or reboot switch.

## Known HRE Problems

- Problem Report 6121

HRE is unable to route IP FDDI data through FDDI trunking.

- Work-around:	NONE

- Problem Report 8040

- If HRE is installed in second slot, routing performance is significantly reduced if the MPM in the second slot becomes the primary MPM.

- Work-around:	Use HRE in slot 1 of MPM only or use HRE-PLUS in Primary and Secondary MPM.

## Known LANE Service Module Problems

- Problem Report 9180

ESI/Sel type policy under ATM prefix policy does not work.

- Work-around:	NONE

## Known LSM Problems

Problem Report 10389

Occasionally the counter that displays the number of proxy and non-proxy LEC's joined to a particular ELAN does not reset.

- Work-around:	NONE

Problem Report 12246

When LSM is configured on Midway ASM module, the number of LECS can be supported is limited depending on the size of Tx/Rx SAR Seg size. If the number of LECS exceeds the number of that port can support then some of the LECS will stuck in the LECS configuration state and the VCCs can not be released.

Work-around:	Configure the port SAR segment (seg) size to be a smaller number (follow the instruction in the manual) when there are more than 232 VCCs will be set up in the same ASM port (this number is calculated based on a 2Mb port).

Problem Report 12661

When adding LES/BUS to LECS, UI allows adding no-name ELAN to LECS database, and there is no way to delete the no-name ELAN from LECS.

- Work-around:	NONE

## Known Mammoth Problems

- Problem Report 12452

When IBM Nways 8274 LAN RouteSwitch or IBM 8277 Nways Ethernet RouteSwitch Mammoth ethernet ports receive misaligned frames, the transmitting port will generate CRC errors.

Work-around:	NONE

Problem Report 12547

RouteChannel is supported on Mammoth based modules. Before swapping mammoth modules with non-mammoth modules disable RouteChannel.

Work-around:	Delete RouteChannel prior to changing the module with one that does.

Problem Report 13696

Fully loaded Chassis with ESM-C-32 will return "pmHndlCrsp()" messages.  Switch will come up after awhile.

Work-around:	NONE

## Known MPM Redundancy Problems

Problem Report 12490

"mpmstore" only works with full path name to the file.

Work-around:	Use the full path name.

Problem Report 12829

When the secondary MPM becomes primary, the MPM menu still displays the local MPM as secondary.

- Work-around:	NONE

Problem Report 12969

When the secondary fails to come up due to missing mpm.img, the primary MPM may display the error message "cmMasterStates(1): START state, without Watchdog" then cannot log in.

Work-around:	NONE

Problem Report 13088

Command mpmreplace gated.img will display error message "there is no file gated.img on this MPM" when the file is in /simm directory.

Work-around:	Enter the command mpmreplace  /simm/gated.img.

Problem Report 13173

Swap on with FCSM, primary is to busy to bring up FCSM and fails to send heartbeat to 2nd MPM.

Work-around:	NONE

Problem Report 13226

Mpmload never gave back prompt after retrieving the file mpm.cmd.

Work-around:	Hit <return> key.

Problem Report 13567

When doing an "imgsync", as soon as it starts replacing the first file "mpm.cnf" you receive the following the following message:

"FILE SYNC incomplete ... try again.

Work-around:	NONE

Problem Report 13612

If the mpm.cmd file is removed and then a renounce is performed on the primary mpm.  The new primary mpm, issues command "sls" to check on the new secondary mpm.  Then the terminal hangs for about 3 minutes and then the prober crashes and reboots the switch.

Work-around:	NONE

Problem Report 13674

The configsync command does not operate correctly after enabling the syncctl command.

Work-around:	Manually do a configsync command.  Afterwards, the configsyc command starts to sync on intervals.

Problem Report 13720

Sometimes, the syncctl command does not sync the files completely right after booting up.

Work-around:	Make sure the syncctl command is off.  Manually do the configsync and imgsync commands.

Problem Report 13733

imgsync repeats that there is one more file to sync when files are updated. It should report that all files are currently synchronized.

Work-around:	NONE

## Known IBM 8277 Nways Ethernet RouteSwitch Problems

Problem Report 11108

pmon, if traffic load high, display to screen will display nothing.

Work-around:	NONE.

## Known IBM Nways 8273 Ethernet RouteSwitch Problems

- Problem Report 10759

RESM-100C-FD and RESM-100C-4 combined causes more than 50% timeout on ping using 100cfd port.

Work-around:	NONE.

## Known Port Manager Problems

- Problem Report  7646

Power off an IBM Nways RouteSwitch and changed board types, for example take out a Token Ring CD6 Board and replace it with a E12 board.  When the system comes up, other boards contained in the chassis may have problems initializing.  This is manifested by the system message:

Problem slot #, <board type> not responding to INIT in state VSE_UP

The other boards usually do complete initialization as the system re-attempts the initialization.  However, occasionally, the initialization fails in all attempts and the board needed to be manually reset.

NOTE:  This PR replaces PR 7541.

Work-around:	Manually reset.

Problem Report 13029

Maxing out available ATM port MAC addresses will result DETACHED ports.  However, after removing all the services, those DETACHED ports still appear.  Can not remove the groups belonging to those detached ports.  Must remove config file to clean up.

Work-around:	Avoid creating more ATM services than are available on that board (typically 16).

## Known Port Mirroring Problems

Problem Report 9901

RMON with port mirroring over Mammoth does not show RMON statistics.

- Work-around:	NONE

Problem Report 13105

Port mirroring does not function on ESM-C-32 cards.

Work-around:	NONE

## Known Port Monitoring Problems

Problem Report 13005

When Port Monitor with display to the screen, the display will not end on the last byte of the packets but will fill out the remainder with garbage.  The file created by the port monitor session will be correct.

- Work-around:	NONE, Display Only.

Problem Report 13892

Terminating a port monitoring session via "pmd" after killing the user  with write/read privilege causes Switch to crash.

Work-around:	Perform a 'pmd" on that session and then the switch will then reboot.

## Known SNMP Problems

Problem Report 5042/10164/11087

SNMP "ifSpeed" info is not correct for ESM-100C-12, ESM-100F-8, ESM-100-C2 & ESM-100-F2.

Work Around:	NONE

Problem Report 6248

OSPF Routes are displayed as RIP when using SNMP.

Work Around:	NONE

- Problem Report 8106

The SNMP agent will return a False value regardless of whether a Token Ring Port in participating as an Active Monitor or not.

- Work-around:	NONE

- Problem Report 8188

- Entries in the bridge forwarding database returned by the SNMP agent should be in numeric order.

Work-Around:	NONE. All entries are returned.

Problem Report 9112

CSM xylnatmInterfaceCutOverSlot/Port will not set.

Work Around:	NONE

Problem Report 10014

Cannot modify ATM vlan cluster connection pairs.

Work Around:	NONE

Problem Report 10709

Four IBM-supplied MIB files will not compile with some off-the shelf compilers: 1850v1, nhrp, pnhrp, softpvc.  The SNMPv2 IMPORT statements must be commented out if this is the case.

Work Around:	NONE

Problem Report 11733

All non-direct routes are incorrectly displayed as RIP routes.

Work Around:	NONE

Problem Report 11807

phyPortTable statistics of ATM ports are not correct.

Work Around:	NONE

Problem Report 11817

ATM Port Config must set Buffer size before Segment size if lowering values.

Work Around:	NONE

Problem Report 11904

The IPX Routing table does not show entries for VLANs.

Work Around:	NONE

Problem Report 11905

Can not set ifAdminStatus.

Work Around:	Use IBM-vport AdminStatus instead.

Problem Report 11975

IfDescr for ESM-C-16 are not correct.

Work Around:	NONE

Problem Report 12025

Dot5RingOpenStatus can return an invalid value of 0.

Work Around:	NONE

Problem Report 12132

IfSpeed does not always match dot5RingSpeed.

Work Around:	NONE

Problem Report 12064

The ifTable shows LSM virtual ports as UNKNOWN type.

Work Around:	NONE

Problem Report 12139

The Source Routing type is not allowed to be Set to a value other than the default (ARE) on the creation of an IPX Router.  It is necessary to Modify the Router to change the SR Type and to set the value in SNMP.

Work Around:	NONE

Problem Report 12189

IP multicast routing MIB doesn't include IGMP version, it always defaults to Version 2.

Work Around:	NONE

Problem Report 12352

LECS entries for Auto Activated LANE service for mobile groups are not included in SNMP.

Work Around:	NONE

Problem Report 12453

Console debug message appears after creating a LANE Service using a TR LEC via SNMP.

Work Around:	NONE

Problem Report 12897

PhyPort table has dummy values for ATM.

Work Around:	NONE

Problem Report 12898

IfSpeed shows 0 on LANE ports.

Work Around:	NONE

Problem Report 12971

TSnmp_agent may crash after modifying the ATM Service.

Work Around:	Do not use HOPV to recreate.  Use NMS application under which the problem does not occur.

Problem Report 13053

AtmxVccOperStats will occasionally return invalid value 0.

Work Around:	NONE

Problem Report 13063

ATM SVC PTOP Service virtual ports show 0 for vporlfIndex while the interfaces table showed 3001 and 3002.

Work Around:	NONE

Problem Report 13137

Enable/Disable VLAN Cluster Service via SNMP wipes out connections.

Work Around:	Use UI instead of IBM Nways RouteSwitch Network Manager or HOPV to disable/enable a vlan cluster service.

Problem Report 13185

AtmxServiceType returns 8 for LSM but it's not defined in MIB.

Work Around:	NONE

Problem Report 14092

GETNEXT of vatIPRouting Table and vatIPXRouting Table goes into an infinite loop.

Work Around:	NONE

Problem Report 14093

Cell statistics stop accumulating when they reach 2 billion.

Work Around:	NONE

Problem Report 14094

SVC CIP Service will not allow the Address Index to be changed.

Work Around:	NONE

Problem Report 14095

Sets on Advanced E3 parameters fail.

Work Around:	NONE

Problem Report 14101

Transparent Bridge needs duplicate MAC aging timer.

Work Around:	NONE

## Known System Problems

- Problem Report 10759

Switch may reset when modifying configurations when the flash file system is full (trying to expand mpm.cnf file).

Work-around:	Do not let flash get full.  Always make sure tat there is at least (2*size of config file) bytes free.

- Problem Report 13791

Switch giving "tUi-shellt0" error messages..

Work-around:	NONE

## Known Token Ring Problems

- Problem Report 4944

Hot swapping of the dual mode token ring card will cause “Changed ring speed detection” messages to be displayed.

Work-around:	NONE

Problem Report 8139

Fiber RIRO-BYTEX will not load onto fiber card properly and ports do not initialize correctly.

- Work-around:	Use Ring-out only.

Problem Report 8753

IBM Nways 8274 LAN RouteSwitch participates as an active monitor even when not configured to participate as an active monitor.

- Work-around:	NONE

Problem Report 9993

Token Ring fiber interfaces using 802.5j does not interoperate with other vendors 802.5j equipment.

Work-around:	NONE

Problem Report 12965

RIF stripping not working on 802.3 LANE when TR-port configured for SRT.

- Work-around:	NONE

Problem Report 13788

TSM dump file locations change causing the dump files to be empty.

- Work-around:	NONE

## Known Translation Problems

Problem Report 10788

IPX translations -  unable to use TR router encaps with ETH media.

Work-around:	NONE

## Known User Interface and General Problems

- Problem Report 1969

The command srs, which displays source routing parameters, shows counters for duplicate LAN ID’s and duplicate segments which increase when there is an external loop.

Work-around:	NONE REQUIRED. Visual only.

- Problem Report 2600

Cannot change FDDI connection rejection policy via user interface.

Work-around:	Use IBM Nways RouteSwitch Network Manager.

Problem Report 6261

Packet discards due to insufficient buffers are not properly displayed in the vs command.

Work-Around:	NONE.

Problem Report 13028

"Monitor" directory not referenced in networking menu screen.

Work-Around:	NONE.

Problem Report 13052

Debug message "ui_do_atm: sel"0x12" appears after SEL change in cas SVC.

Work-Around:	NONE, Ignore message.

Problem Report 13069

The cas command does not allow to set up VLAN cluster if the connection type was changed to PVC from SVC.

Work-Around:	NONE.

Problem Report 13119

Unknown VPI/Unknown VCI fields in the vlrs command do not display last unknown VPI/VCI per port basis.

Work-Around:	NONE.

Problem Report 13182

vvc on an individual slot/port vpi/vci on OC-12 port show two entries on UI.

Work-Around:	NONE.

## Known WSM Problems

Problem Report 13193/13205/13218/13219

Admin through PPPm does not work on WSM-BRI (ISDN)

Work-around:	NONE

## Known X-Cell/FCSM Problems

Problem Report 6721

Switch needs to be rebooted after changing the signalling version on CSM port(s).

- Work-Around:	NONE

- Problem Report 6844

- When a connection is created and multicast child, or when reverse connections are created, transmit bandwidth is not decremented.

- Work-Around:	NONE

Problem Report 6901

When selecting the max vci of 255, the vci is not included and system creates a vpc.

- Work-Around:	Do not select maximum vci of 255.

- Problem Report 7126

- ILMI and signaling do not go out CSM port when FCSM is in slot 1.

- Work-Around:	Install FCSM in any other slot.

- Problem Report 7203

- Switch may not reboot properly after primary MPM-1G is removed from slot 2.

- Work-Around:	NONE

- Problem Report 7380

- Vvc and vcs do not work for individual connections on FCSM board.

- Work-Around:	List all connections.

- Problem Report 7869

- The calculation of PCR for CBR in lower than 1k values is incorrect.

- Work-Around:	NONE

- Problem Report 8268

- Upon removing OC12 connection, the signaling channel stays in the hardware and does not get flushed.

- Work-Around:	NONE, one extra channel left on the system is not a problem.

- Problem Report 8812

- After removing a CSM module, all CSM commands show either ports or channels on a slot.

- Work-Around:	NONE, this occurs only when a board is removed.

- Problem Report 9213

- If the user changes the peak cell rate in the traffic description table usage the cvc or mvc commands and then changes the sustaining cell rate or the maximum burst size, the available bandwidth will be decremented again.

- Work-Around:	Do not save the channel if this occurs, cancel the menu and always enter the sustaining cell rate and maximum burst size before changing the peak cell rate (i.e. change the peak cell rate last).

Problem Report 12128

FCSM-622 IOP discarding cells due to congestion when there is no congestion.

Work-around:	NONE

Problem Report 12155

IPX data corruption seen when using FCSM-622.

Work-around:	NONE

Problem Report 13001

The vap command shows incorrect number of configured vc on FCSM-622.

Work-around:	NONE (Cosmetic does not affect operation.)

Problem Report 13148

Ping timeout approx 60% when traversing through FCSM PTOP & FCSM LANE services.

Work-around:	Use FCSM-IIW-4C

Problem Report 13961

When FCSM is hot swapped, ASM side of signalling stack releases all  the call requests.

Work-around:	NONE

Problem Report 14017

FCSM crashes after switch reboot and creating an NI dump.

Work-around:	NONE

Problem Report  IBM5512

FCSM crashes under certain conditions when installed with a TSM and a LANE Service is in the same group as Token Ring ports.

Work-Around:  NONE

Problem Report  IBM5585

Cannot use 8274 as a Cell Switch with PNNI in an environment where there are 8265's running PNNI full hierarchy.

Work-Around:  NONE

MPM/HRE Compatibility Matrix

|  |  | HRE |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HRE-PLUS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2 | 3 | 4 | 5 | 6 | 7 | A | A1 | A2 | A3 | A4 | B | B1 | B2 | B3 | A4 | A5 | B | B1 | B2 | B3 | B4 | B5 | B6 |
| MPM-II | A |  |  |  |  |  |  | N | N | N | N | U | U | U | U | U | N | N | N | N | N | N | N | N | N |
| 050120-13 | A1 |  |  |  |  |  |  | N | N | N | N | U | U | U | U | U | N | N | N | N | N | N | N | N | N |
|  | A2 |  |  |  |  |  |  | N | N | N | N | U | U | U | U | U | N | N | N | N | N | N | N | N | N |
|  | A3 |  |  |  |  |  |  | N | N | N | N | U | U | U | U | U | N | N | N | N | N | N | N | N | N |
|  | A4 |  |  |  |  |  |  | N | N | N | N | U | U | U | U | U | N | N | N | N | N | N | N | N | N |
|  | A5 |  |  |  |  |  |  | N | N | N | N | OK | OK | OK | OK | OK | N | N | N | N | N | N | N | N | N |
|  | A6 |  |  |  |  |  |  | N | N | N | N | OK | OK | OK | OK | OK | N | N | N | N | N | N | N | N | N |
|  | A7 |  |  |  |  |  |  | N | N | N | N | OK | OK | OK | OK | OK | N | N | N | N | N | N | N | N | N |
|  | A8 |  |  |  |  |  |  | N | N | N | N | OK | OK | OK | OK | OK | N | N | N | N | N | N | N | N | N |
|  | A9 |  |  |  |  |  |  | N | N | N | N | OK | OK | OK | OK | OK | N | N | N | N | N | N | N | N | N |
|  | A10 |  |  |  |  |  |  | N | N | N | N | OK | OK | OK | OK | OK | N | N | N | N | N | N | N | N | N |
|  | C17 |  |  |  |  |  |  | N | N | N | N | OK | OK | OK | OK | OK | N | N | N | N | N | N | N | N | N |
|  | C18 |  |  |  |  |  |  | N | N | N | N | OK | OK | OK | OK | OK | N | N | N | N | N | N | N | N | N |
|  | C19 |  |  |  |  |  |  | N | N | N | N | OK | OK | OK | OK | OK | N | N | N | N | N | N | N | N | N |
|  | C20 |  |  |  |  |  |  | N | N | N | N | OK | OK | OK | OK | OK | N | N | N | N | N | N | N | N | N |
|  | C21 |  |  |  |  |  |  | N | N | N | N | OK | OK | OK | OK | OK | N | N | N | N | N | N | N | N | N |
| MPM-II | B |  |  |  |  |  |  | N | N | N | N | U | U | U | U | U | N | N | N | N | N | N | N | N | N |
| 050120-06 | C |  |  |  |  |  |  | N | N | N | N | U | U | U | U | U | N | N | N | N | N | N | N | N | N |
|  | C1 |  |  |  |  |  |  | N | N | N | N | U | U | U | U | U | N | N | N | N | N | N | N | N | N |
|  | C2 |  |  |  |  |  |  | N | N | N | N | U | U | U | U | U | N | N | N | N | N | N | N | N | N |
|  | C3 |  |  |  |  |  |  | N | N | N | N | U | U | U | U | U | N | N | N | N | N | N | N | N | N |
|  | C4 |  |  |  |  |  |  | N | N | N | N | U | U | U | U | U | N | N | N | N | N | N | N | N | N |
|  | C5 |  |  |  |  |  |  | N | N | N | N | U | U | U | U | U | N | N | N | N | N | N | N | N | N |
|  | C6 |  |  |  |  |  |  | N | N | N | N | U | U | U | U | U | N | N | N | N | N | N | N | N | N |
|  | C7 |  |  |  |  |  |  | N | N | N | N | U | U | U | U | U | N | N | N | N | N | N | N | N | N |
|  | C8 |  |  |  |  |  |  | N | N | N | N | U | U | U | U | U | N | N | N | N | N | N | N | N | N |
|  | C9 |  |  |  |  |  |  | N | N | N | N | U | U | U | U | U | N | N | N | N | N | N | N | N | N |
|  | C10 |  |  |  |  |  |  | N | N | N | N | U | U | U | U | U | N | N | N | N | N | N | N | N | N |
|  | C11 |  |  |  |  |  |  | N | N | N | N | OK | OK | OK | OK | OK | N | N | N | N | N | N | N | N | N |
|  | C12 |  |  |  |  |  |  | N | N | N | N | OK | OK | OK | OK | OK | N | N | N | N | N | N | N | N | N |
|  | C13 |  |  |  |  |  |  | N | N | N | N | OK | OK | OK | OK | OK | N | N | N | N | N | N | N | N | N |
|  | C14 |  |  |  |  |  |  | N | N | N | N | OK | OK | OK | OK | OK | N | N | N | N | N | N | N | N | N |
|  | C15 |  |  |  |  |  |  | N | N | N | N | OK | OK | OK | OK | OK | N | N | N | N | N | N | N | N | N |
|  | C16 |  |  |  |  |  |  | N | N | N | N | OK | OK | OK | OK | OK | N | N | N | N | N | N | N | N | N |
|  | C17 |  |  |  |  |  |  | N | N | N | N | OK | OK | OK | OK | OK | N | N | N | N | N | N | N | N | N |
|  | C18 |  |  |  |  |  |  | N | N | N | N | OK | OK | OK | OK | OK | N | N | N | N | N | N | N | N | N |
|  | C19 |  |  |  |  |  |  | N | N | N | N | OK | OK | OK | OK | OK | N | N | N | N | N | N | N | N | N |
|  | C20 |  |  |  |  |  |  | N | N | N | N | OK | OK | OK | OK | OK | N | N | N | N | N | N | N | N | N |
|  | C21 |  |  |  |  |  |  | N | N | N | N | OK | OK | OK | OK | OK | N | N | N | N | N | N | N | N | N |
| MPM-1G | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | U | U | U | U | U | U | U | U | U |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | U | U | U | U | U | U | U | U | U |
|  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | U | U | U | U | U | U | U | U | U |
|  | A1 |  |  |  |  |  |  | N | N | N | N | U | U | U | U | U | U | U | U | U | U | U | U | U | U |
|  | A2 |  |  |  |  |  |  | N | N | N | N | U | U | U | U | U | U | U | U | U | U | U | U | U | U |
|  | A3 |  |  |  |  |  |  | N | N | N | N | U | U | U | U | U | U | U | U | U | U | U | U | U | U |
|  | A4 |  |  |  |  |  |  | N | N | N | N | U | U | U | U | U | U | U | U | U | U | U | U | U | U |
|  | A5 |  |  |  |  |  |  | N | N | N | N | U | U | U | U | U | U | U | U | U | U | U | U | U | U |
|  | A6 |  |  |  |  |  |  | N | N | N | N | U | U | U | U | U | U | U | U | U | U | U | U | U | U |
|  | A7 |  |  |  |  |  |  | N | N | N | N | OK | OK | OK | OK | OK | U | U | U | U | U | U | U | U | U |
|  | A8 |  |  |  |  |  |  | N | N | N | N | OK | OK | OK | OK | OK | U | U | U | U | U | U | U | U | U |
|  | A9 |  |  |  |  |  |  | N | N | N | N | OK | OK | OK | OK | OK | U | U | U | U | U | U | U | U | U |
|  | A10 |  |  |  |  |  |  | N | N | N | N | OK | OK | OK | OK | OK | U | U | U | U | U | U | U | U | U |
|  | A11 |  |  |  |  |  |  | N | N | N | N | OK | OK | OK | OK | OK | U | U | U | U | U | U | U | U | U |
|  | A12 |  |  |  |  |  |  | N | N | N | N | OK | OK | OK | OK | OK | U | U | U | U | U | U | U | U | U |

MPM/HRE Compatibility Matrix (Cont'd)

|  |  |  | HRE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HRE-PLUS |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2 | 3 | 4 | 5 | 6 | 7 | A | A1 |  | A2 | A3 | A4 | B | B1 | B2 | B3 | A4 | A5 | B | B1 |  |  | B2 | B3 | B4 | B5 | B6 |
| MPM-1G |  | A13 |  |  |  |  |  |  | N | N |  | N | N | OK | OK | OK | OK | OK | U | U | U | U |  |  | U | U | U | U | U |
| (Cont'd) |  | A14 |  |  |  |  |  |  | N | N |  | N | N | U | U | U | U | U | U | U | U | U |  |  | U | U | U | U | U |
|  |  | A15 |  |  |  |  |  |  | N | N |  | N | N | U | U | U | U | U | U | U | U | U |  |  | U | U | U | U | U |
|  |  | A16 |  |  |  |  |  |  | N | N |  | N | N | OK | OK | OK | OK | OK | OK | OK | OK | OK |  |  | OK | OK | OK | OK | OK |
|  |  | A17 |  |  |  |  |  |  | N | N |  | N | N | OK | OK | OK | OK | OK | OK | OK | OK | OK |  |  | OK | OK | OK | OK | OK |
|  |  | B |  |  |  |  |  |  | N | N |  | N | N | OK | OK | OK | OK | OK | U | U | U | U |  |  | U | U | U | U | U |
|  |  | B1 |  |  |  |  |  |  | N | N |  | N | N | OK | OK | OK | OK | OK | U | U | U | U |  |  | U | U | U | U | U |
|  |  | B2 |  |  |  |  |  |  | N | N |  | N | N | OK | OK | OK | OK | OK | U | U | U | U |  |  | U | U | U | U | U |
|  |  | B3 |  |  |  |  |  |  | N | N |  | N | N | OK | OK | OK | OK | OK | U | U | U | U |  |  | U | U | U | U | U |
|  |  | B4 |  |  |  |  |  |  | N | N |  | N | N | U | U | U | U | U | U | U | U | U |  |  | U | U | U | U | U |
|  |  | B5 |  |  |  |  |  |  | N | N |  | N | N | U | U | U | U | U | U | U | U | U |  |  | U | U | U | U | U |
|  |  | B6 |  |  |  |  |  |  | N | N |  | N | N | U | U | U | U | U | U | U | U | U |  |  | U | U | U | U | U |
|  |  | B7 |  |  |  |  |  |  | N | N |  | N | N | U | U | U | U | U | U | U | U | U |  |  | U | U | U | U | U |
|  |  | B8 |  |  |  |  |  |  | N | N |  | N | N | U | U | U | U | U | U | U | U | U |  |  | U | U | U | U | U |
|  |  | B9 |  |  |  |  |  |  | N | N |  | N | N | U | U | U | U | U | U | U | U | U |  |  | U | U | U | U | U |
|  |  | B10 |  |  |  |  |  |  | N | N |  | N | N | OK | OK | OK | OK | OK | OK | OK | OK | OK |  |  | OK | OK | OK | OK | OK |
|  |  | B11 |  |  |  |  |  |  | N | N |  | N | N | OK | OK | OK | OK | OK | OK | OK | OK | OK |  |  | OK | OK | OK | OK | OK |
|  |  | B12 |  |  |  |  |  |  | N | N |  | N | N | OK | OK | OK | OK | OK | OK | OK | OK | OK |  |  | OK | OK | OK | OK | OK |
|  |  | B13 |  |  |  |  |  |  | N | N |  | N | N | OK | OK | OK | OK | OK | OK | OK | OK | OK |  |  | OK | OK | OK | OK | OK |
|  |  | B14 |  |  |  |  |  |  | N | N |  | N | N | OK | OK | OK | OK | OK | OK | OK | OK | OK |  |  | OK | OK | OK | OK | OK |
|  |  | B15 |  |  |  |  |  |  | N | N |  | N | N | OK | OK | OK | OK | OK | OK | OK | OK | OK |  |  | OK | OK | OK | OK | OK |
|  |  | B16 |  |  |  |  |  |  | N | N |  | N | N | OK | OK | OK | OK | OK | OK | OK | OK | OK |  |  | OK | OK | OK | OK | OK |
|  |  | B17 |  |  |  |  |  |  | N | N |  | N | N | OK | OK | OK | OK | OK | OK | OK | OK | OK |  |  | OK | OK | OK | OK | OK |
|  |  | B18 |  |  |  |  |  |  | N | N |  | N | N | OK | OK | OK | OK | OK | OK | OK | OK | OK |  |  | OK | OK | OK | OK | OK |
|  |  | B19 |  |  |  |  |  |  | N | N |  | N | N | OK | OK | OK | OK | OK | OK | OK | OK | OK |  |  | OK | OK | OK | OK | OK |
|  |  | B20 |  |  |  |  |  |  | N | N |  | N | N | OK | OK | OK | OK | OK | OK | OK | OK | OK |  |  | OK | OK | OK | OK | OK |
|  | OK=ACCEPTABLE CONFIGURATION |  |  |  |  |  |  |  |  |  | U=MPM UPGRADE REQUIRED |  |  |  |  |  |  |  |  |  |  |  |  | N=NOT POSSIBLE |  |  |  |  |  |

# Route-Vision Compatibility & Minimum Requirements Matrices

## Compatibility Matrix

| Hardware Platform | INTEL | Sun | HP | IBM |  |
| --- | --- | --- | --- | --- | --- |
| Operating System | W95 / 98, WinNTv4.0 | Solaris 2.5 Solaris 2.6 | HPUX 10.x | AIX 4.x |  |
| Management Platform |  |  |  |  |  |
| v3.2 |  |  |  |  | XOS Version |
| Route-Vision Enterprise Win95/NT |  |  |  |  | Switch firmware v3.2 or v3.1.5 |
| IBM Nways RouteSwitch Network Manager Win95/NT |  |  |  |  | Switch firmware v3.1.5 / v3.1.6 / v3.2 |
| IBM Nways RouteSwitch Network Manager UNIX |  |  |  |  | Switch firmware v3.1.5 / v3.1.6 / v3.2 |
| Switch Start |  |  |  |  | Supports IBM 8277 Nways Ethernet RouteSwitch or IBM Nways 8273 Ethernet RouteSwitch only; firmware v3.2 or v3.1.5 |
| v3.3 |  |  |  |  | XOS Version |
| Route-Vision UNIX (IBM Nways RouteSwitch Network Manager & Nways RouteTracker Manager UNIX) |  |  |  |  | Switch firmware v3.1.6 / v3.2 / v3.3 |
| Route-Vision Workgroup Win95/NT |  |  |  |  | IBM 8277 Nways Ethernet RouteSwitch or IBM Nways 8273 Ethernet RouteSwitch only; firmware v3.2 or v3.1.5 |
| Switch Start |  |  |  |  | Switch firmware v3.2 / v3.3 |
| v3.4 |  |  |  |  | XOS Version |
| Route-Vision UNIX (All functional capabilities) |  |  |  |  | Switch firmware v3.2 or above |
| Route-Vision Enterprise Win95/NT |  |  |  |  | Switch firmware v3.2 or above |
| Route-Vision Workgroup Win95/NT |  |  |  |  | IBM 8277 Nways Ethernet RouteSwitch or IBM Nways 8273 Ethernet RouteSwitch only; v3.2 or above |
| Switch Start |  |  |  |  | Supports all IBM devices; firmware v3.3 or above; 3rd party NMS integration is limited to launching of Switch Start only |

| Hardware Platform | INTEL | Sun | Sun | Sun | HP | IBM |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Operating System | W95 / 98, WinNTv4.0 | Solaris 2.5 / 2.6 | Solaris 2.5 / 2.6 | Solaris 2.5 / 2.6 | HPUX 10.x | AIX 4.3 |  |
| Management Platform | HPOV B.05.0.2 HPOV D.01.02 | HPOV 4.x HPOV 5.x | Sunnet Mgr | Spectrum | HPOV 5.x | NV6000 4.x |  |
| v3.2 |  |  |  |  |  |  | XOS Version |
| Route-Vision Enterprise Win95/NT |  |  |  |  |  |  | Switch firmware v3.2 or v3.1.5 |
| IBM Nways RouteSwitch Network Manager Win95/NT |  |  |  |  |  |  | Switch firmware v3.1.5 / v3.1.6 / v3.2 |
| IBM Nways RouteSwitch Network Manager UNIX |  |  |  |  |  |  | Switch firmware v3.1.5 / v3.1.6 / v3.2 |
| Switch Start |  |  |  |  |  |  | Supports IBM 8277 Nways Ethernet RouteSwitch or IBM Nways 8273 Ethernet RouteSwitches only; firmware v3.2 or v3.1.5 |
| v3.3 |  |  |  |  |  |  | XOS Version |
| Route-Vision UNIX (IBM Nways RouteSwitch Network Manager & Nways RouteTracker Manager UNIX) |  |  |  |  |  |  | Switch firmware v3.1.6 / v3.2 / v3.3 |
| Route-Vision Workgroup Win95/NT |  |  |  |  |  |  | IBM 8277 Nways Ethernet RouteSwitch or IBM Nways 8273 Ethernet RouteSwitches only; firmware v3.2 or v3.1.5 |
| Switch Start |  |  |  |  |  |  | Switch firmware v3.2 / v3.3 |
| v3.4 |  |  |  |  |  |  | XOS Version |
| Route-Vision UNIX (All functional capabilities) |  |  |  |  |  |  | Switch firmware v3.2 or above |
| Route-Vision Enterprise Win95/NT |  |  |  |  |  |  | Switch firmware v3.2 or above |
| Route-Vision Workgroup Win95/NT |  |  |  |  |  |  | IBM 8277 Nways Ethernet RouteSwitch or IBM Nways 8273 Ethernet RouteSwitches only; v3.2 or above |
| Switch Start |  |  |  |  |  |  | Supports all IBM devices; firmware v3.3 or above; 3rd party NMS integration is limited to launching of Switch Start only |

## Minimum Requirements

| Minimum Requirements for Release v3.4 | Route-Vision for Enterprise / Workgroups (Win95/NT) v3.4 | Route-Vision UNIX v3.3 (Solaris) | Route-Vision UNIX v3.3 (HP-UX) | Route-Vision UNIX v3.3 (AIX) |
| --- | --- | --- | --- | --- |
| HW or CPU | Intel Pentium 166 MHz | Sun Sol2 workstation (such as SPARCstation 4 or 5, or ULTRA-1) | HP 9000 | RS/6000 Workstation/Server 39 |
| Memory | 64 MB RAM (128 MB RAM recommended) | 64 MB RAM (128 MB RAM recommended) | 64 MB RAM (128 MB RAM recommended) | 64 MB RAM (128 MB RAM recommended) |
| Video Display | SVGA Video Card (hi-color) |  |  |  |
| Video HW | 17" Monitor, 1024 x 768 Resolution recommended | XIIR5/Motif 1.2 | XIIR5/Motif 1.2 | XIIR5/Motif 1.2 |
| Disk space | 300 MB free disk space | 100 MB free disk space |  |  |
| OS | Microsoft Windows NT 4.0 with Service Pack 3 or Microsoft Windows 95 OSR 2.1 (i.e. 950B) with DCOM or Microsoft 98 | Solaris 2.51/SunOS 5 | HP UX v10.x | AIX v4.1, AIX v4.3 |
| Shell | N/A |  |  |  |
| Media | CD-ROM Drive | CD ROM | CD ROM | CD ROM |
| NIC | Network Interface Card | Network Interface Card | Network Interface Card | Network Interface Card |
| Protocol | TCP/IP Stack | TCP/IP Stack | TCP/IP Stack | TCP/IP Stack |
| 3rd party NMS support or Special instructions |  | HP OpenView | NetView |  |

| Minimum Requirements for Release v3.4 | Switch Start v3.3 (Windows95/NT) | Switch Start v3.3 (Solaris) | Switch Start v3.3 (HP-UX) | Switch Start v3.3 (AIX) |
| --- | --- | --- | --- | --- |
| HW or CPU | Intel Pentim 166 MHz | Sun Sol2 workstation (such as SPARCstation 4 or 5, or ULTRA-1) | (HP 9000, 700 series) | RS/6000 Workstation/Server 39 |
| Memory | 64 MB RAM (128 MB RAM recommended) | 64 MB RAM (128 MB RAM recommended) | 64 MB RAM (128 MB RAM recommended) | 64 MB RAM (128 MB RAM recommended) |
| Video Display | SVGA Video Card | XIIR6/Motif 1.2 |  |  |
| Video HW | 17" Monitor, 800 x 600 Resolution recommended | 17" or 20" Monitor | 17" or 20" Monitor | 17" or 20" Monitor |
| Disk space | 30 MB free disk space | 30 MB free disk space | 30 MB free disk space | 10 MB free disk space |
| OS | Windows 95 OSR 2 or Windows NT 4.0 (Service Pack 3) or Windows98 | Solaris 2.5.1 / SunOS 5.5.1 | HP-UX 10.20 | IBM AIX 4.1.4 or 4.2, 4.3 |
| Shell | N/A | N/A | N/A | N/A |
| Media | CD ROM | CD ROM | CD ROM | CD ROM |
| NIC | Network Interface Card | Network Interface Card | Network Interface Card | Network Interface Card |
| Protocol | TCP/IP Stack | TCP/IP Stack | TCP/IP Stack | TCP/IP Stack |
| 3rd party NMS support or Special instructions | Requires "Java Runtime Environment" from Sun Microsystems | Requires "Java Runtime Environment" from Sun Microsystems | Requires the "HP-UX Java Developer's Kit and Virtual Machine for Java" HP using the following link: | AIX v4.3 includes the required JAVA Runtime Engine (JRE) v1.1 |
| 3rd party NMS support or Special instructions | http://www.javasoft.com/products/jdk/1.1/jre/index.html | http://www.javasoft.com/products/jdk/1.1/jre/index.html | http://www.hp.com/go/java |  |

# IBM Nways 8274 LAN RouteSwitch, IBM 8277 Nways Ethernet RouteSwitch and IBM Nways 8273 Ethernet RouteSwitch Hardware/Software Release Compatibility Matrices

## IBM Nways 8274 LAN RouteSwitch Hardware/Software Release Compatibility Matrix

| IBM Nways 8274 LAN RouteSwitch Hardware |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Software Release |  |  |  |  |  |  |  |
| Product | 3.1.8 | 3.1.9 | 3.2.1 | 3.2.2 | 3.2.3 | 3.2.4 | 3.2.5 | 3.2.6 |
| MPM (P/N 050026-00) Requires 16 Meg for 3.0 | DISCONTINUED |  |  |  |  |  |  |  |
| Rev. C (2.10) | x | x | x | x | x | x | x | x |
| Rev. D (2.11) | x | x | x | x | x | x | x | x |
| Rev. E (2.12) | x | x | x | x | x | x | x | x |
| Rev. F (2.13) | x | x | x | x | x | x | x | x |
| Rev. G (2.14) | x | x | x | x | x | x | x | x |
| Rev. H (2.15) | x | x | x | x | x | x | x | x |
| Rev. I (2.16) | x | x | x | x | x | x | x | x |
| Rev. J (2.17) | x | x | x | x | x | x | x | x |
| Rev. K (2.18) | x | x | x | x | x | x | x | x |
| Rev. L (2.19) | x** | x** | x** | x** | x** | x** | x** | x** |
| Rev. L1 (2.20) | x | x | x | x | x | x | x | x |
| Rev. L2 (2.21) | x | x | x | x | x | x | x | x |
| Rev. L3 (2.22) | x | x | x | x | x | x | x | x |
| Rev. L4 (2.23) | x | x | x | x | x | x | x | x |
| Rev. L5 (2.24) | x | x | x | x | x | x | x | x |
| Rev. L6 (2.25) | x | x | x | x | x | x | x | x |
| MPM (P/N 050026-01) Requires 16 Meg for 3.0 |  |  |  |  |  |  |  |  |
| Rev. C (2.10) | x | x | x | x | x | x | x | x |
| Rev. D (2.11) | x | x | x | x | x | x | x | x |
| Rev. E (2.12) | x | x | x | x | x | x | x | x |
| Rev. F (2.13) | x | x | x | x | x | x | x | x |
| Rev. G (2.14) | x | x | x | x | x | x | x | x |
| Rev. H (2.15) | x | x | x | x | x | x | x | x |
| Rev. I (2.16) | x | x | x | x | x | x | x | x |
| Rev. J (2.17) | x | x | x | x | x | x | x | x |
| Rev. K (2.18) | x | x | x | x | x | x | x | x |
| Rev. L (2.19) | x** | x** | x** | x** | x** | x** | x** | x** |
| Rev. L1 (2.20) | x | x | x | x | x | x | x | x |
| Rev. L2 (2.21) | x | x | x | x | x | x | x | x |
| Rev. L3 (2.22) | x | x | x | x | x | x | x | x |
| Rev. L4 (2.23) | x | x | x | x | x | x | x | x |
| Rev. L5 (2.24) | x | x | x | x | x | x | x | x |
| Rev. L6 (2.25) | x | x | x | x | x | x | x | x |
| Rev. L7 (2.26) | x | x | x | x | x | x | x | x |
| Rev. L8 (2.27) | x | x | x | x | x | x | x | x |
| MPM-1G (P/N 050143-06) |  |  |  |  |  |  |  |  |
| Rev. A1 (1.01) | x** | x** | x** | x** | x** | x** | x** | x** |
| Rev. A2 (1.02) | x | x | x | x | x | x | x | x |
| Rev. A3 (1.03) | x | x | x | x | x | x | x | x |
| Rev. A4 (1.04) | x | x | x | x | x | x | x | x |
| Rev. A5 (1.05) | x | x | x | x | x | x | x | x |
| Rev. A6 (1.06) | x | x | x | x | x | x | x | x |
| Rev. A7 (1.07) | x | x | x | x | x | x | x | x |
| Rev. A8 (1.08) | x | x | x | x | x | x | x | x |
| Rev. A9 (1.09) | x | x | x | x | x | x | x | x |
| Rev. A10 (1.10) | x | x | x | x | x | x | x | x |
| Rev. A11 (1.11) | x | x | x | x | x | x | x | x |
| Rev. A12 (1.12) | x | x | x | x | x | x | x | x |
| Rev. A13 (1.13) | x | x | x | x | x | x | x | x |
| Rev. A14 (1.14) | x | x | x | x | x | x | x | x |
| Rev. A15 (1.15) | x | x | x | x | x | x | x | x |
| Rev. A16 (1.16) | x | x | x | x | x | x | x | x |
| Rev. A17 (1.17) | x | x | x | x | x | x | x | x |
| Rev. A18 (1.18) | x | x | x | x | x | x | x | x |
| Rev. A19 (1.19) | x | x | x | x | x | x | x | x |
| Rev. A20 (1.20) | x | x | x | x | x | x | x | x |
| Rev. A21 (1.21) | x | x | x | x | x | x | x | x |
| Rev. A22 (1.22) | x | x | x | x | x | x | x | x |

| 36.1 | IBM Nways 8274 LAN RouteSwitch Hardware/Software Release Compatibility Matrix (Cont'd) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Software Release |  |  |  |  |  |  |  |  |
| Product |  | 3.1.8 | 3.1.9 | 3.2.1 | 3.2.2 | 3.2.3 | 3.2.4 | 3.2.5 | 3.2.6 |  |
| MPM-II (P/N 050120-06) Requires 16 Meg for 3.0 |  |  |  |  |  |  |  |  |  |  |
| Rev. C1 (2.02) |  | x** | x** | x** | x** | x** | x** | x** | x** |  |
| Rev. C2 (2.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. C3 (2.04/3.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. C4 (2.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. C5 (2.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. C4 (3.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. C5 (3.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. C6 (3.07) |  | x | x | x | x | x | x | x | x |  |
| Rev. C7(3.08) |  | x | x | x | x | x | x | x | x |  |
| Rev. C8 (3.09) |  | x | x | x | x | x | x | x | x |  |
| Rev. C9 (3.10) |  | x | x | x | x | x | x | x | x |  |
| Rev. C10 (3.11) |  | x | x | x | x | x | x | x | x |  |
| Rev. C11 (3.12) |  | x | x | x | x | x | x | x | x |  |
| Rev. C12 (3.13) |  | x | x | x | x | x | x | x | x |  |
| Rev. C13 (3.14) |  | x | x | x | x | x | x | x | x |  |
| Rev. C14 (3.15) |  | x | x | x | x | x | x | x | x |  |
| Rev. C15 (3.16) |  | x | x | x | x | x | x | x | x |  |
| Rev. C16 (3.17) |  | x | x | x | x | x | x | x | x |  |
| Rev. C17 (3.18) |  | x | x | x | x | x | x | x | x |  |
| Rev. C18 (3.19) |  | x | x | x | x | x | x | x | x |  |
| Rev. C19 (3.20) |  | x | x | x | x | x | x | x | x |  |
| Rev. C20 (3.21) |  | x | x | x | x | x | x | x | x |  |
| Rev. C21 (3.22) |  | x | x | x | x | x | x | x | x |  |
| MPM 11-16GB (P/N 050120-13) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) |  | x** | x** | x** | x** | x** | x** | x** | x** |  |
| Rev. A1 (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. A5 (1.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. A6 (1.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. A7 (1.07) |  | x | x | x | x | x | x | x | x |  |
| Rev. A8 (1.08) |  | x | x | x | x | x | x | x | x |  |
| Rev. A9 (1.09) |  | x | x | x | x | x | x | x | x |  |
| Rev. A10 (1.10) |  | x | x | x | x | x | x | x | x |  |
| Rev. C17 (1.17) |  | x | x | x | x | x | x | x | x |  |
| Rev. C18 (1.18) |  | x | x | x | x | x | x | x | x |  |
| Rev. C19 (1.19) |  | x | x | x | x | x | x | x | x |  |
| Rev. C20 (1.20) |  | x | x | x | x | x | x | x | x |  |
| Rev. C21 (1.21) |  | x | x | x | x | x | x | x | x |  |
| HRE Hardware Routing Engine (P/N 050125-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. A1 (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. A5 (1.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. A6 (1.06) |  | x | x | x | x | x | x | x | x |  |
| HRE Plus (P/N 050125-08) |  |  |  |  |  |  |  |  |  |  |
| Rev. A4 (1.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. A5 (1.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. A6 (1.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. A7 (1.07) |  | x | x | x | x | x | x | x | x |  |
| ESM-C-8 (P/N 050014-00) |  | DISCONTINUED |  |  |  |  |  |  |  |  |
| Rev. D (1.09) |  | x | x | x | x | x | x | x | x |  |
| Rev. E (1.10) |  | x | x | x | x | x | x | x | x |  |
| Rev. F (1.11) |  | x** | x** | x** | x** | x** | x** | x** | x** |  |
| Rev. F1 (1.12) Error: Reference source not found |  | x | x | x | x | x | x | x | x |  |
| Rev. F2 (1.13) |  | x | x | x | x | x | x | x | x |  |
| Rev. F3 (1.14) |  | x | x | x | x | x | x | x | x |  |
| ESM-C-12 (P/N 050112-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. D (4.00) |  | x** | x** | x** | x** | x** | x** | x** | x** |  |
| Rev. D1 (4.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. D2 (4.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. D3 (4.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. D4 (4.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. E (5.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. E1 (5.01) Error: Reference source not found |  | x | x | x | x | x | x | x | x |  |
| Rev. E2 (5.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. E3 (5.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. E4 (5.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. E5 (5.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. E6 (5.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. E7 (5.07) |  | x | x | x | x | x | x | x | x |  |
| Rev. E8 (5.08) |  | x | x | x | x | x | x | x | x |  |
| Rev. E9 (5.09) |  | x | x | x | x | x | x | x | x |  |
| ESM-C-16W (P/N 050154-17) |  |  |  |  |  |  |  |  |  |  |
| Rev. C5 (3.05) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. C6 (3.06) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. C7 (3.07) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. C8 (3.08) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. C9 (3.09) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. C10 (3.10) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. C11 (3.11) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. C12 (3.12) Initial Release |  | x | x | x | x | x | x | x | x |  |
| ESM-C-16W (P/N 050154-18) |  |  |  |  |  |  |  |  |  |  |
| Rev. C (3.00) |  |  |  |  |  |  |  |  |  |  |
| Rev. C1 (3.01) |  |  |  |  |  |  |  |  |  |  |
| Rev. C2 (3.02) |  |  |  |  |  |  |  |  |  |  |
| Rev. C3 (3.03) |  |  |  |  |  |  |  |  |  |  |
| Rev. C4 (3.04) |  |  |  |  |  |  |  |  |  |  |
| Rev. C5 (3.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. C6 (3.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. C7 (3.07) |  | x | x | x | x | x | x | x | x |  |
| ESM-C-32 (P/N 050154-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. B (2.00) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. B1 (2.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. B2 (2.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. B3 (2.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. B4 (2.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. B5 (2.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. B6 (2.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. B7 (2.07) 2 |  | x | x | x | x | x | x | x | x |  |
| Rev. B8 (2.08) |  | x | x | x | x | x | x | x | x |  |
| Rev. B9 (2.09) |  | x | x | x | x | x | x | x | x |  |
| ESM-C-32 (P/N 050154-16) |  |  |  |  |  |  |  |  |  |  |
| Rev. C (3.00) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. C1 (3.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. C2 (3.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. C3 (3.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. C4 (3.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. C5 (3.05) Error: Reference source not found |  | x | x | x | x | x | x | x | x |  |
| Rev. C6 (3.06) |  | x | x | x | x | x | x | x | x |  |
| ESM-F-8 (P/N 050118-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) |  | x** | x** | x** | x** | x** | x** | x** | x** |  |
| Rev. A1 (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. A5 (1.05) Error: Reference source not found |  | x | x | x | x | x | x | x | x |  |
| Rev. B (2.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. B1 (2.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. B2 (2.02) Error: Reference source not found |  | x | x | x | x | x | x | x | x |  |
| Rev. B3 (2.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. B4 (2.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. B5 (2.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. B6 (2.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. B7 (2.07) |  | x | x | x | x | x | x | x | x |  |
| Rev. B8 (2.08) |  | x | x | x | x | x | x | x | x |  |
| Rev. B9 (2.09) |  | x | x | x | x | x | x | x | x |  |
| Rev. B10 (2.10) |  | x | x | x | x | x | x | x | x |  |
| ESM-FM-8W-2C (P/N 050197-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A1 (1.01) Initial Release |  |  |  |  |  | x | x | x | x |  |
| Rev. A2 (1.02) |  |  |  |  |  | x | x | x | x |  |
| ESM-FM-16W-2C (P/N 050197-08) |  |  |  |  |  |  |  |  |  |  |
| Rev. A1 (1.01) Initial Release |  |  |  |  |  | x | x | x | x |  |
| Rev. A2 (1.02) |  |  |  |  |  | x | x | x | x |  |
| ESM-T-12 (P/N 050140-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. B (2.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. B1 (2.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. B2 (2.02)Error: Reference source not found |  | x | x | x | x | x | x | x | x |  |
| Rev. B3 (2.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. B4 (2.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. B5 (2.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. B6 (2.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. B7 (2.07) |  | x | x | x | x | x | x | x | x |  |
| ESM-U-6 (P/N 050067-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. C (1.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. D (1.04) |  | x** | x** | x** | x** | x** | x** | x** | x** |  |
| Rev. E (1.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. F (2.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. F1 (2.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. F2 (2.02) 1 |  | x | x | x | x | x | x | x | x |  |
| Rev. F3 (2.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. F4 (2.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. F5 (2.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. F6 (2.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. F7 (2.07) |  | x | x | x | x | x | x | x | x |  |
| ESM-AB-AFD (P/N 050144-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. A1 (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  | x | x | x | x | x | x | x | x |  |
| ESM-AB-AT (P/N 050079-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. B |  | x | x | x | x | x | x | x | x |  |
| Rev. B1 |  | x | x | x | x | x | x | x | x |  |
| ESM-AB-B (P/N 050097-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. B |  | x | x | x | x | x | x | x | x |  |
| Rev. C |  | x | x | x | x | x | x | x | x |  |
| Rev. C1 |  | x | x | x | x | x | x | x | x |  |
| Rev. C2 |  | x | x | x | x | x | x | x | x |  |
| Rev. C3 |  | x | x | x | x | x | x | x | x |  |
| Rev. C4 |  | x | x | x | x | x | x | x | x |  |
| ESM-AB-FL (P/N 050085-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. B |  | x | x | x | x | x | x | x | x |  |
| Rev. C |  | x | x | x | x | x | x | x | x |  |
| Rev. D |  | x | x | x | x | x | x | x | x |  |
| Rev. E |  | x | x | x | x | x | x | x | x |  |
| Rev. E1 |  | x | x | x | x | x | x | x | x |  |
| Rev. E2 |  | x | x | x | x | x | x | x | x |  |
| Rev. E3 |  | x | x | x | x | x | x | x | x |  |
| Rev. E4 |  | x | x | x | x | x | x | x | x |  |
| ESM-AB-FL-S (P/N 050123-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. A1 (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  | x | x | x | x | x | x | x | x |  |
| ESM-AB-FL-SH (P/N 050123-13) |  |  |  |  |  |  |  |  |  |  |
| Rev. A1 (1.00) Initial Release |  | x | x | x | x | x | x | x | x |  |
| ESM-AB-T (P/N 050091-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A |  | x | x | x | x | x | x | x | x |  |
| Rev. A1 |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 |  | x | x | x | x | x | x | x | x |  |
| Rev. A3 |  | x | x | x | x | x | x | x | x |  |
| Rev. A4 |  | x | x | x | x | x | x | x | x |  |
| ESM-100C-12 (P/N 050158-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. B2 (2.02) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. B3 (2.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. B4 (2.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. B5 (2.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. B6 (2.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. B7 (2.07) |  | x | x | x | x | x | x | x | x |  |
| Rev. B8 (2.08) |  | x | x | x | x | x | x | x | x |  |
| Rev. B9 (2.09) |  | x | x | x | x | x | x | x | x |  |
| Rev. B10 (2.10) |  | x | x | x | x | x | x | x | x |  |
| Rev. B11 (2.11) |  | x | x | x | x | x | x | x | x |  |
| Rev. B12 (2.12) |  | x | x | x | x | x | x | x | x |  |
| ESM-100C-FD (P/N 050122-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. B (2.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. B1 (2.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. B2 (2.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. B3 (2.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. B4 (2.04) |  | x | x | x | x | x | x | x | x |  |
| ESM-100C-FD, AE (P/N 050122-15) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. B1 (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. B2 (1.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. B3 (1.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. B4 (1.04) |  | x | x | x | x | x | x | x | x |  |
| ESM-100FM-8 (P/N 050159-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. A1 (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. A5 (1.05)2 |  | x | x | x | x | x | x | x | x |  |
| Rev. A6 (1.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. A7 (1.07) |  | x | x | x | x | x | x | x | x |  |
| Rev. A8 (1.08) |  | x | x | x | x | x | x | x | x |  |
| Rev. A9 (1.09) |  | x | x | x | x | x | x | x | x |  |
| Rev. A10 (1.10) |  | x | x | x | x | x | x | x | x |  |
| Rev. A11 (1.11) |  | x | x | x | x | x | x | x | x |  |
| ESM-100FM-FD (P/N 050122-13) |  |  |  |  |  |  |  |  |  |  |
| Rev. B (2.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. B1 (2.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. B2 (2.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. B3 (2.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. B4 (2.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. B5 (2.05) |  | x | x | x | x | x | x | x | x |  |
| ESM-100FM-FD (P/N 050122-17) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. A1 (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. B3 (2.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. B6 (2.06) |  | x | x | x | x | x | x | x | x |  |
| ESM-100FS-8W-4C (P/N 050159-16) |  |  |  |  |  |  |  |  |  |  |
| Rev. B4 (2.04) Initial Release |  |  |  |  |  | x | x | x | x |  |
| Rev. B5 (2.05) |  |  |  |  |  | x | x | x | x |  |
| Rev. B6 (2.06) |  |  |  |  |  | x | x | x | x |  |
| ESM-100FS-FD (P/N 050122-16) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. A1 (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. B3 (2.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. B6 (2.06) |  | x | x | x | x | x | x | x | x |  |
| ESM-100FS-FD-1 (P/N 050122-14) |  |  |  |  |  |  |  |  |  |  |
| Rev. B (2.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. B1 (2.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. B2 (2.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. B3 (2.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. B4 (2.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. B5 (2.05) |  | x | x | x | x | x | x | x | x |  |
| FESM AE Daughter Card (P/N 050109-13) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. A1 (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  | x | x | x | x | x | x | x | x |  |
| FESM II 100BTX (P/N 050139-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. A1 (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. A5 (1.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. A6 (1.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. A7 (1.07) |  | x | x | x | x | x | x | x | x |  |
| Rev. A8 (1.08) |  | x | x | x | x | x | x | x | x |  |
| Rev. A9 (1.09) |  | x | x | x | x | x | x | x | x |  |
| FESM II 100BTX (P/N 050139-13) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. A1 (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. A5 (1.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. A6 (1.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. A7 (1.07) |  | x | x | x | x | x | x | x | x |  |
| CDDI Daughter Card (P/N 050073-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. B (1.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. C (1.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. D (1.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. E (1.05) |  | x**** | x**** | x**** | x**** | x**** | x**** | x**** | x**** |  |
| Rev. E1 (1.06) |  | x**** | x**** | x**** | x**** | x**** | x**** | x**** | x**** |  |
| Rev. E2 (1.07) |  | x**** | x**** | x**** | x**** | x**** | x**** | x**** | x**** |  |
| HSM (P/N 050031-06) |  | DISCONTINUED |  |  |  |  |  |  |  |  |
| Rev. E (2.05) “ “ |  | x | x | x | x | x | x | x | x |  |
| Rev. F (2.06) “ “ |  | x | x | x | x | x | x | x | x |  |
| Rev. G (2.07) “ “ |  | x | x | x | x | x | x | x | x |  |
| Rev. H (2.08) “ “ |  | x | x | x | x | x | x | x | x |  |
| Rev. J (2.09) “ “ |  | x | x | x | x | x | x | x | x |  |
| Rev. K (3.10) |  | x** | x** | x** | x** | x** | x** | x** | x** |  |
| Rev. L (3.11) |  | x** | x** | x** | x** | x** | x** | x** | x** |  |
| Rev. M (4.12) |  | x | x | x | x | x | x | x | x |  |
| Rev. M1 (4.13) |  | x | x | x | x | x | x | x | x |  |
| Rev. M2 (4.14) |  | x | x | x | x | x | x | x | x |  |
| HSM2 (P/N 050110-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. C (3.00) |  | x** | x** | x** | x** | x** | x** | x** | x** |  |
| Rev. C1 (3.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. C2 (3.02)Error: Reference source not found |  | x | x | x | x | x | x | x | x |  |
| Rev. C3 (3.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. D (4.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. D1 (4.01) Error: Reference source not found |  | x | x | x | x | x | x | x | x |  |
| Rev. D2 (4.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. D3 (4.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. D4 (4.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. D5 (4.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. D6 (4.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. D7 (4.07) |  | x | x | x | x | x | x | x | x |  |
| HSM2 4K (P/N 050168-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. A1 (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. A5 (1.05) |  | x | x | x | x | x | x | x | x |  |
| TSM Copper (P/N 050049-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. B (1.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. C (1.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. D (1.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. D1 (4.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. D2 (4.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. D3 (4.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. D4 (4.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. D5 (4.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. D6 (4.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. D7 (4.07) |  | x | x | x | x | x | x | x | x |  |
| TSM Fiber (P/N 050111-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. B. (2.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. B1 (2.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. B2 (2.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. B3 (2.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. B4 (2.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. B5 (2.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. B6 (2.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. B7 (2.07) |  | x | x | x | x | x | x | x | x |  |
| Rev. B8 (2.08) |  | x | x | x | x | x | x | x | x |  |
| TSM Dual Mode (P/N 050121-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. A1 (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. A5 (1.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. A6 (1.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. A7 (1.07) |  | x | x | x | x | x | x | x | x |  |
| ATM OC3 Daughter Card (P/N 050044-00) |  | DISCONTINUED |  |  |  |  |  |  |  |  |
| Rev. A (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. B (2.00) |  | x | x | x | x | x | x | x | x |  |
| ATM OC3 Daughter Card (P/N 050116-26) Multimode, ½ MB SRAM |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. A1 (1.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. A3 (1.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. A4 (1.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. A5 (1.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. A6 (1.07) |  | x | x | x | x | x | x | x | x |  |
| ATM OC3 Daughter Card (P/N 050116-24) Multimode, 2 MB SRAM |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. A1 (1.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. A3 (1.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. A4 (1.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. A5 (1.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. A6 (1.07) |  | x | x | x | x | x | x | x | x |  |
| Rev. A7 (1.08) |  | x | x | x | x | x | x | x | x |  |
| ATM OC3 Daughter Card (P/N 050116-25) Single-mode, 2 MB SRAM |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. A1 (1.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. A3 (1.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. A4 (1.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. A5 (1.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. A6 (1.07) |  | x | x | x | x | x | x | x | x |  |
| Rev. A7 (1.08) |  | x | x | x | x | x | x | x | x |  |
| ATM OC3 Daughter Card (P/N 050116-27) Single-mode, ½ MB SRAM |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. A1 (1.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. A3 (1.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. A4 (1.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. A5 (1.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. A6 (1.07) |  | x | x | x | x | x | x | x | x |  |
| ATM DS3 Daughter Card (P/N 050104-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. A1 (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. B (2.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. B1 (2.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. B2 (2.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. B3 (2.03) |  | x | x | x | x | x | x | x | x |  |
| ATM E3 Daughter Card (P/N 050104-13) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. A1 (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. A5 (1.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. A6 (1.06) |  | x | x | x | x | x | x | x | x |  |
| ATM-UTP Daughter Card (P/N 050115-06) one UTP |  |  |  |  |  |  |  |  |  |  |
| Rev. B (2.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. B1 (2.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. B2 (2.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. B3 (2.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. B4 (2.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. B5 (2.05) |  | x | x | x | x | x | x | x | x |  |
| ATM2-155-FM (P/N 050146-06) Multimode |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. A1 (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. A5 (1.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. A6 (1.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. A7 (1.07) |  | x | x | x | x | x | x | x | x |  |
| Rev. A8 (1.08) |  | x | x | x | x | x | x | x | x |  |
| ATM2-155-FS (P/N 050146-13) Single-mode |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. A1 (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. A5 (1.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. A6 (1.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. A7 (1.07) |  | x | x | x | x | x | x | x | x |  |
| Rev. A8 (1.08) |  | x | x | x | x | x | x | x | x |  |
| ASM-155FSH-1EW-4C (PN 050116-28) |  |  |  |  |  |  |  |  |  |  |
| Rev. A2 (1.02) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  |  |  | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  |  |  | x | x | x | x | x | x |  |
| Rev. A5 (1.05) |  |  |  | x | x | x | x | x | x |  |
| Rev. A6 (1.06) |  |  |  | x | x | x | x | x | x |  |
| Rev. A7 (1.07) |  |  |  | x | x | x | x | x | x |  |
| ASM-CE-155FM-2S2EW (PN 050110-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. D4 (4.04) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. D5 (4.05) |  |  |  | x | x | x | x | x | x |  |
| Rev. D6 (4.06) |  |  |  | x | x | x | x | x | x |  |
| Rev. D7 (4.07) |  |  |  | x | x | x | x | x | x |  |
| ASM-CE-155FM-2S2EW-4C (PN 050110-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. D4 (4.04) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. D5 (4.05) |  |  |  | x | x | x | x | x | x |  |
| Rev. D6 (4.06) |  |  |  | x | x | x | x | x | x |  |
| Rev. D7 (4.07) |  |  |  | x | x | x | x | x | x |  |
| ASM-CE-FM-2S2TW (PN 050110-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. D4 (4.04) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. D5 (4.05) |  |  |  | x | x | x | x | x | x |  |
| Rev. D6 (4.06) |  |  |  | x | x | x | x | x | x |  |
| Rev. D7 (4.07) |  |  |  | x | x | x | x | x | x |  |
| ASM-CE-155FM-2S2TW-4C (PN 050168-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A2 (1.02) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  |  |  | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  |  |  | x | x | x | x | x | x |  |
| Rev. A5 (1.05) |  |  |  | x | x | x | x | x | x |  |
| ASM-CE-155FS-2S2EW (PN 050110-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. D4 (4.04) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. D5 (4.05) |  |  |  | x | x | x | x | x | x |  |
| Rev. D6 (4.06) |  |  |  | x | x | x | x | x | x |  |
| Rev. D7 (4.07) |  |  |  | x | x | x | x | x | x |  |
| ASM-CE-155FS-2S2EW-4C (PN 050168-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A2 (1.02) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  |  |  | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  |  |  | x | x | x | x | x | x |  |
| Rev. A5 (1.05) |  |  |  | x | x | x | x | x | x |  |
| ASM-CE-155FS-2S2TW (PN 050110-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. D4 (4.04) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. D7 (4.07) |  |  |  | x | x | x | x | x | x |  |
| Rev. D5 (4.05) |  |  |  | x | x | x | x | x | x |  |
| Rev. D6 (4.06) |  |  |  | x | x | x | x | x | x |  |
| ASM-CE-155FS-2S2TW-4C (PN 050168-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A2 (1.02) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  |  |  | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  |  |  | x | x | x | x | x | x |  |
| Rev. A5 (1.05) |  |  |  | x | x | x | x | x | x |  |
| ASM-CE-DS3-2S2TW (PN 050110-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. D4 (4.04) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. D5 (4.05) |  |  |  | x | x | x | x | x | x |  |
| Rev. D6 (4.06) |  |  |  | x | x | x | x | x | x |  |
| Rev. D7 (4.07) |  |  |  | x | x | x | x | x | x |  |
| ASM-CE-DS3-2S2TW-4C (PN 050168-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A2 (1.02) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  |  |  | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  |  |  | x | x | x | x | x | x |  |
| Rev. A5 (1.05) |  |  |  | x | x | x | x | x | x |  |
| ASM-CE-E3-2S2EW (PN 050110-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. D4 (4.04) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. D5 (4.05) |  |  |  | x | x | x | x | x | x |  |
| Rev. D6 (4.06) |  |  |  | x | x | x | x | x | x |  |
| Rev. D7 (4.07) |  |  |  | x | x | x | x | x | x |  |
| ASM-CE-E3-2S2EW-4C (PN 050168-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A2 (1.02) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  |  |  | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  |  |  | x | x | x | x | x | x |  |
| Rev. A5 (1.05) |  |  |  | x | x | x | x | x | x |  |
| ASM-CE-155FM-2S2EW-4C (PN 050168-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A2 (1.02) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  |  |  | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  |  |  | x | x | x | x | x | x |  |
| Rev. A5 (1.05) |  |  |  | x | x | x | x | x | x |  |
| CSM-155-6M2S (P/N 050113-25) |  |  |  |  |  |  |  |  |  |  |
| Rev. G1 (7.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. G2 (7.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. G3 (7.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. G4 (7.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. G5 (7.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. G6 (7.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. G7 (7.07) |  | x | x | x | x | x | x | x | x |  |
| CSM-155-6M2S (P/N 050113-85) |  |  |  |  |  |  |  |  |  |  |
| Rev. G4 (7.04) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. G6 (7.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. G7 (7.07) |  | x | x | x | x | x | x | x | x |  |
| Rev. G8 (7.08) |  | x | x | x | x | x | x | x | x |  |
| CSM-155-6M2SL (P/N 050113-27) |  |  |  |  |  |  |  |  |  |  |
| Rev. F5 (6.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. G1 (7.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. G2 (7.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. G3 (7.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. G4 (7.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. G5 (7.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. G6 (7.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. G7 (7.07) |  | x | x | x | x | x | x | x | x |  |
| CSM-155-6M2SL (P/N 050113-87) |  |  |  |  |  |  |  |  |  |  |
| Rev. G4 (7.04) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. G5 (7.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. G6 (7.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. G7 (7.07) |  | x | x | x | x | x | x | x | x |  |
| Rev. G8 (7.08) |  | x | x | x | x | x | x | x | x |  |
| CSM-155-8 (P/N 050113-14) |  |  |  |  |  |  |  |  |  |  |
| Rev. F (6.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. F1 (6.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. F2 (6.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. F3 (6.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. F4 (6.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. F5 (6.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. F6 (6.06) |  | x | x | x | x | x | x | x | x |  |
| CSM-155-8 (P/N 050113-24) |  |  |  |  |  |  |  |  |  |  |
| Rev. G (7.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. G1 (7.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. G2 (7.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. G3 (7.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. G4 (7.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. G5 (7.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. G6 (7.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. G7 (7.07) |  | x | x | x | x | x | x | x | x |  |
| CSM-155-8 (P/N 050113-84) |  |  |  |  |  |  |  |  |  |  |
| Rev. G4 (7.04) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. G5 (7.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. G6 (7.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. G7 (7.07) |  | x | x | x | x | x | x | x | x |  |
| Rev. G8 (7.08) |  | x | x | x | x | x | x | x | x |  |
| CSM-155C-8 (P/N 050113-83) |  |  |  |  |  |  |  |  |  |  |
| Rev. G4 (7.04) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. G6 (7.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. G7 (7.07) |  | x | x | x | x | x | x | x | x |  |
| Rev. G8 (7.08) |  | x | x | x | x | x | x | x | x |  |
| CSM-155C-8W (P/N 050113-23) |  |  |  |  |  |  |  |  |  |  |
| Rev. G (7.00) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. G1 (7.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. G2 (7.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. G3 (7.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. G4 (7.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. G6 (7.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. G7 (7.07) |  | x | x | x | x | x | x | x | x |  |
| CSM-155-8S (P/N 050113-08) |  |  |  |  |  |  |  |  |  |  |
| Rev. F (6.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. F1 (6.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. F2 (6.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. F3 (6.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. F4 (6.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. F5 (6.05) |  | x | x | x | x | x | x | x | x |  |
| CSM-155-8S (P/N 050113-18) |  |  |  |  |  |  |  |  |  |  |
| Rev. G1 (7.01) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. G2 (7.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. G3 (7.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. G4 (7.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. G5 (7.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. G6 (7.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. G7 (7.07) |  | x | x | x | x | x | x | x | x |  |
| CSM-155-8S (P/N 050113-78) |  |  |  |  |  |  |  |  |  |  |
| Rev. G4 (7.04) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. G5 (7.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. G6 (7.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. G7 (7.07) |  | x | x | x | x | x | x | x | x |  |
| Rev. G8 (7.08) |  | x | x | x | x | x | x | x | x |  |
| CSM-155-FSL-8 (P/N 050113-86) |  |  |  |  |  |  |  |  |  |  |
| Rev. G4 (7.04) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. G5 (7.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. G6 (7.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. G7 (7.07) |  | x | x | x | x | x | x | x | x |  |
| Rev. G8 (7.08) |  | x | x | x | x | x | x | x | x |  |
| CSM-622-2E (P/N 050133-13 ) |  |  |  |  |  |  |  |  |  |  |
| Rev. A1 (1.01) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. A5 (1.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. A6 (1.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. A7 (1.07) |  | x | x | x | x | x | x | x | x |  |
| Rev. A10 (1.10) |  | x | x | x | x | x | x | x | x |  |
| Rev. A11 (1.11) |  | x | x | x | x | x | x | x | x |  |
| Rev. A12 (1.12) |  | x | x | x | x | x | x | x | x |  |
| Rev. A13 (1.13) |  | x | x | x | x | x | x | x | x |  |
| Rev. A14 (1.14) |  | x | x | x | x | x | x | x | x |  |
| CSM-622-2E (P/N 050133-33 ) |  |  |  |  |  |  |  |  |  |  |
| Rev. B (2.00) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. B1 (2.01) |  | x | x | x | x | x | x | x | x |  |
| CSM-622-2E (P/N 050133-53 ) |  | DISCONTINUED |  |  |  |  |  |  |  |  |
| Rev. C (3.00) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. C1 (3.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. C2 (3.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. C3 (3.03) |  | x | x | x | x | x | x | x | x |  |
| CSM-622-2E (P/N 050133-73 ) |  |  |  |  |  |  |  |  |  |  |
| Rev. C1 (3.01) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. C2 (3.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. C3 (3.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. C4 (3.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. C5 (3.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. C6 (3.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. C7 (3.07) |  | x | x | x | x | x | x | x | x |  |
| Rev. C8 (3.08) |  | x | x | x | x | x | x | x | x |  |
| Rev. C9 (3.09) |  | x | x | x | x | x | x | x | x |  |
| Rev. C10 (3.10) |  | x | x | x | x | x | x | x | x |  |
| CSM-622-2E (P/N 050133-93) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. A1 (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. A8 (1.08) |  | x | x | x | x | x | x | x | x |  |
| Rev. A9 (1.09) |  | x | x | x | x | x | x | x | x |  |
| Rev. A10 (1.10) |  | x | x | x | x | x | x | x | x |  |
| Rev. A11 (1.11) |  | x | x | x | x | x | x | x | x |  |
| Rev. A12 (1.12) |  | x | x | x | x | x | x | x | x |  |
| Rev. A13 (1.13) |  | x | x | x | x | x | x | x | x |  |
| Rev. A14 (1.14) |  | x | x | x | x | x | x | x | x |  |
| Rev. A15 (1.15) |  | x | x | x | x | x | x | x | x |  |
| Rev. A16 (1.16) |  | x | x | x | x | x | x | x | x |  |
| CSM-622-2SE (P/N 050133-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A1 (1.01) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. A5 (1.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. A6 (1.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. A7 (1.07) |  | x | x | x | x | x | x | x | x |  |
| Rev. A10 (1.10) |  | x | x | x | x | x | x | x | x |  |
| Rev. A11 (1.11) |  | x | x | x | x | x | x | x | x |  |
| Rev. A12 (1.12) |  | x | x | x | x | x | x | x | x |  |
| Rev. A13 (1.13) |  | x | x | x | x | x | x | x | x |  |
| Rev. A14 (1.14) |  | x | x | x | x | x | x | x | x |  |
| CSM-622-2SE (P/N 050133-26 ) |  |  |  |  |  |  |  |  |  |  |
| Rev. B (2.00) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. B1 (2.01) |  | x | x | x | x | x | x | x | x |  |
| CSM-622-2SE (P/N 050133-46 ) |  | DISCONTINUED |  |  |  |  |  |  |  |  |
| Rev. C (3.00) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. C1 (3.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. C2 (3.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. C3 (3.03) |  | x | x | x | x | x | x | x | x |  |
| CSM-622-2SE (P/N 050133-66 ) |  |  |  |  |  |  |  |  |  |  |
| Rev. C1 (3.01) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. C2 (3.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. C3 (3.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. C4 (3.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. C5 (3.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. C6 (3.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. C7 (3.07) |  | x | x | x | x | x | x | x | x |  |
| Rev. C8 (3.08) |  | x | x | x | x | x | x | x | x |  |
| Rev. C9 (3.09) |  | x | x | x | x | x | x | x | x |  |
| Rev. C10 (3.10) |  | x | x | x | x | x | x | x | x |  |
| CSM-622-2SE (P/N 050133-86) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. A1 (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. A8 (1.08) |  | x | x | x | x | x | x | x | x |  |
| Rev. A9 (1.09) |  | x | x | x | x | x | x | x | x |  |
| Rev. A10 (1.10) |  | x | x | x | x | x | x | x | x |  |
| Rev. A11 (1.11) |  | x | x | x | x | x | x | x | x |  |
| Rev. A12 (1.12) |  | x | x | x | x | x | x | x | x |  |
| Rev. A13 (1.13) |  | x | x | x | x | x | x | x | x |  |
| Rev. A14 (1.14) |  | x | x | x | x | x | x | x | x |  |
| Rev. A15 (1.15) |  | x | x | x | x | x | x | x | x |  |
| Rev. A16 (1.16) |  | x | x | x | x | x | x | x | x |  |
| CSM-A25-12 (P/N 050134-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. A1 (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.02) |  | x | x | x | x | x | x | x | x |  |
| CSM-A25-12 (P/N 050134-66) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.01) Initial Release |  |  | x |  |  | x | x | x | x |  |
| Rev. A1 (1.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.03) |  | x | x | x | x | x | x | x | x |  |
| CSM-A25-24 (P/N 050134-13) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. A1 (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.02) |  | x | x | x | x | x | x | x | x |  |
| CSM-A25-24 (P/N 050134-73) |  |  |  |  |  |  |  |  |  |  |
| Rev. A1(1.01) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.02) |  | x | x | x | x | x | x | x | x |  |
| CSM-AB-155C-2W (P/N 050164-10) |  |  |  |  |  |  |  |  |  |  |
| Rev. A4 (1.04) Initial Release |  |  |  |  |  | x | x | x | x |  |
| Rev. A5 (1.05) |  |  |  |  |  | x | x | x | x |  |
| CSM-AB-155FM-2W (P/N 050164-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A2 (1.02) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  |  |  | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  |  |  | x | x | x | x | x | x |  |
| Rev. A5 (1.05) |  |  |  | x | x | x | x | x | x |  |
| CSM-AB-155FS-2W (P/N 050164-08) |  |  |  |  |  |  |  |  |  |  |
| Rev. A2 (1.02) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  |  |  | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  |  |  | x | x | x | x | x | x |  |
| Rev. A5 (1.05) |  |  |  | x | x | x | x | x | x |  |
| CSM-AB-155FSH-2W (P/N 050164-09) |  |  |  |  |  |  |  |  |  |  |
| Rev. A2 (1.02) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  |  |  | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  |  |  | x | x | x | x | x | x |  |
| Rev. A5 (1.05) |  |  |  | x | x | x | x | x | x |  |
| CSM-AB-CE-E1-4W (P/N 050187-08) |  |  |  |  |  |  |  |  |  |  |
| Rev. A6 (1.06) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. A7 (1.07) |  |  |  | x | x | x | x | x | x |  |
| CSM-AB-CE-T1-4W (P/N 050187-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A5 (1.05) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. A6 (1.06) |  |  |  | x | x | x | x | x | x |  |
| CSM-AB-DS1-4W (P/N 050179-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A3 (1.03) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  |  |  | x | x | x | x | x | x |  |
| Rev. A5 (1.05) |  |  |  | x | x | x | x | x | x |  |
| CSM-AB-DS3-2W (P/N 050169-26) |  |  |  |  |  |  |  |  |  |  |
| Rev. b (2.00) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. b1 (2.01) |  |  |  | x | x | x | x | x | x |  |
| CSM-AB-E1-4W (P/N 050179-13) |  |  |  |  |  |  |  |  |  |  |
| Rev. A3 (1.03) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  |  |  | x | x | x | x | x | x |  |
| Rev. A5 (1.05) |  |  |  | x | x | x | x | x | x |  |
| CSM-AB-E3-2W (P/N 050169-28) |  |  |  |  |  |  |  |  |  |  |
| Rev. B (2.00) Initial Release |  |  |  | x | x | x | x | x | x |  |
| CSM-UW (P/N 050157-86) |  |  |  |  |  |  |  |  |  |  |
| Rev. b1 (2.01) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. B2 (2.02) |  |  |  | x | x | x | x | x | x |  |
| FCSM-OC3 (P/N 050129-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. A1 (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. B (2.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. B1 (2.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. B2 (2.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. B3 (2.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. B4 (2.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. B5 (2.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. B6 (2.06) |  | x | x | x | x | x | x | x | x |  |
| FCSM-OC3-4C (P/N 050129-13) |  |  |  |  |  |  |  |  |  |  |
| Rev. B1 (2.01) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. B2 (2.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. B3 (2.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. B4 (2.04) |  | x | x | x | x | x | x | x | x |  |
| FCSM-OC3-4C (P/N 050129-73) |  |  |  |  |  |  |  |  |  |  |
| Rev. B1 (2.01) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. B2 (2.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. B3 (2.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. B4 (2.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. B5 (2.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. B6 (2.06) |  | x | x | x | x | x | x | x | x |  |
| FCSM-OC12 (P/N 050181-06) |  | DISCONTINUED |  |  |  |  |  |  |  |  |
| Rev. A (1.00) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. A1 (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.02) |  | x | x | x | x | x | x | x | x |  |
| FCSM-OC12 (P/N 050181-66) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.04) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. A5 (1.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. A6 (1.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. A7 (1.07) |  | x | x | x | x | x | x | x | x |  |
| Rev. A8 (1.08) |  | x | x | x | x | x | x | x | x |  |
| FSM Daughter Card (P/N 050037-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. C (1.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. D (1.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. E (1.05) |  | x*** | x*** | x*** | x*** | x*** | x*** | x*** | x*** |  |
| Rev. E1 (1.06) |  | x*** | x*** | x*** | x*** | x*** | x*** | x*** | x*** |  |
| FSM Daughter Card (P/N 050108-06) Multimode |  |  |  |  |  |  |  |  |  |  |
| Rev. C (2.03) |  | x*** | x*** | x*** | x*** | x*** | x*** | x*** | x*** |  |
| Rev. C1 (2.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. C2 (2.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. C3 (2.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. C4 (2.07) |  | x | x | x | x | x | x | x | x |  |
| Rev. C5 (2.08) |  | x | x | x | x | x | x | x | x |  |
| FSM Daughter Card (P/N 050108-13) Single-mode |  |  |  |  |  |  |  |  |  |  |
| Rev. B (2.00) Initial Release |  | x | x | x | x | x | x | x | x |  |
| Rev. C3 (3.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. C4 (3.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. C5 (3.05) |  | x | x | x | x | x | x | x | x |  |
| FDDI/2-S-ATM Cat 2 Laser (P/N 050108-15) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. A1 (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. C3 (3.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. C4 (3.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. C5 (3.02) |  | x | x | x | x | x | x | x | x |  |
| 100BaseT Daughter Card (P/N 050109-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. C (3.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. C1 (3.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. C2 (3.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. C3 (3.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. C4 (3.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. C5 (3.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. C6 (3.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. C7 (3.07) |  | x | x | x | x | x | x | x | x |  |
| Rev. C8 (3.08) |  | x | x | x | x | x | x | x | x |  |
| Rev. C9 (3.09) |  | x | x | x | x | x | x | x | x |  |
| Rev. C10 (3.10) |  | x | x | x | x | x | x | x | x |  |
| Rev. C11 (3.11) |  | x | x | x | x | x | x | x | x |  |
| Rev. C12 (3.12) |  | x | x | x | x | x | x | x | x |  |
| Rev. C13 (3.13) |  | x | x | x | x | x | x | x | x |  |
| Rev. C14 (3.14) |  | x | x | x | x | x | x | x | x |  |
| Rev. C15 (3.15) |  | x | x | x | x | x | x | x | x |  |
| WSM-SC-4/8 WAN (P/N 050117-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) |  |  |  |  |  |  |  |  |  |  |
| Rev. A1 (1.01) |  |  |  |  |  |  |  |  |  |  |
| Rev. A2 (1.02) |  |  |  |  |  |  |  |  |  |  |
| Rev. A3 (1.03) |  |  |  |  |  |  |  |  |  |  |
| Rev. A4 (1.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. A5 (1.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. A6 (1.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. A7 (1.07) |  | x | x | x | x | x | x | x | x |  |
| Rev. A8 (1.08) |  | x | x | x | x | x | x | x | x |  |
| Rev. A9 (1.09) |  | x | x | x | x | x | x | x | x |  |
| Rev. A10 (1.10) |  | x | x | x | x | x | x | x | x |  |
| WSM-S-2 WAN (P/N 050117-13) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) |  | x | x | x | x | x | x | x | x |  |
| Rev. A1 (1.01) |  | x | x | x | x | x | x | x | x |  |
| Rev. A2 (1.02) |  | x | x | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  | x | x | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  | x | x | x | x | x | x | x | x |  |
| Rev. A5 (1.05) |  | x | x | x | x | x | x | x | x |  |
| Rev. A6 (1.06) |  | x | x | x | x | x | x | x | x |  |
| Rev. A7 (1.07) |  | x | x | x | x | x | x | x | x |  |
| WSM-BRI-SC-1W, WSM-BRI-SC-2W (P/N 050160-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A1(1.01) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. A2 (1.02) |  |  |  | x | x | x | x | x | x |  |
| Rev. A3 (1.03) |  |  |  | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  |  |  | x | x | x | x | x | x |  |
| WSM-FE1-SC-1W, WSM-FE1-SC-2W (P/N 050148-08) |  |  |  |  |  |  |  |  |  |  |
| Rev. A3 (1.03) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. A4 (1.04) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. A5 (1.05) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. A6 (1.06) |  |  |  | x | x | x | x | x | x |  |
| WSM-FT1-SC-1W, WSM-FT1-SC-2W (P/N 050148-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A3 (1.03) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. A4 (1.04) |  |  |  | x | x | x | x | x | x |  |
| Rev. A5 (1.05) |  |  |  | x | x | x | x | x | x |  |
| Rev. A6 (1.06) |  |  |  | x | x | x | x | x | x |  |

## IBM 8277 Nways Ethernet RouteSwitch Hardware/Software Release Compatibility Matrix

|  | Software Release |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Product | 3.1.8 | 3.1.9 | 3.2.1 | 3.2.2 | 3.2.3 | 3.2.4 | 3.2.5 | 3.2.6 |
| 13X, 13X-4C (P/N 050188-06) |  |  |  |  |  |  |  |  |
| Rev. A4 (1.04) Initial Release |  |  | x | x | x | x | x | x |
| Rev. A5 (1.05) |  |  | x | x | x | x | x | x |
| Rev. A6 (1.06) |  |  | x | x | x | x | x | x |
| Rev. A7 (1.07) |  |  | x | x | x | x | x | x |
| Rev. A8 (1.08) |  |  | x | x | x | x | x | x |
| Rev. A9 (1.09) |  |  | x | x | x | x | x | x |
| Rev. A10 (1.10) |  |  | x | x | x | x | x | x |
| 13XF, 13XF-4C (P/N 050188-09) |  |  |  |  |  |  |  |  |
| Rev. A4 (1.04) Initial Release |  |  | x | x | x | x | x | x |
| Rev. A5 (1.05) |  |  | x | x | x | x | x | x |
| Rev. A6 (1.06) |  |  | x | x | x | x | x | x |
| Rev. A7 (1.07) |  |  | x | x | x | x | x | x |
| Rev. A8 (1.08) |  |  | x | x | x | x | x | x |
| Rev. A9 (1.09) |  |  | x | x | x | x | x | x |
| Rev. A10 (1.10) |  |  | x | x | x | x | x | x |
| 8277-13F, 8277-13F-4C (P/N 050188-08) |  |  |  |  |  |  |  |  |
| Rev. A4 (1.04) Initial Release |  |  | x | x | x | x | x | x |
| Rev. A5 (1.05) |  |  | x | x | x | x | x | x |
| Rev. A6 (1.06) |  |  | x | x | x | x | x | x |
| Rev. A7 (1.07) |  |  | x | x | x | x | x | x |
| Rev. A8 (1.08) |  |  | x | x | x | x | x | x |
| Rev. A9 (1.09) |  |  | x | x | x | x | x | x |
| 8277 MODEL 232, 8277 MODEL 232-4C (P/N 050180-08) |  |  |  |  |  |  |  |  |
| Rev. B7 (2.07) Initial Release |  |  |  | x | x | x | x | x |
| Rev. B8 (2.08) |  |  |  | x | x | x | x | x |
| Rev. B9 (2.09) |  |  |  | x | x | x | x | x |
| Rev. B10 (2.10) |  |  |  | x | x | x | x | x |
| Rev. B11 (2.11) |  |  |  | x | x | x | x | x |
| Rev. B12 (2.12) |  |  |  | x | x | x | x | x |
| Rev. B13 (2.13) |  |  |  | x | x | x | x | x |
| 8277 MODEL 332, 8277 MODEL 332-8C, 8277 MODEL 332E (P/N 050180-06) |  |  |  |  |  |  |  |  |
| Rev. B6 (2.06) Initial Release |  |  |  | x | x | x | x | x |
| Rev. B7 (2.07) |  |  |  | x | x | x | x | x |
| Rev. B8 (2.08) |  |  |  | x | x | x | x | x |
| Rev. B9 (2.09) |  |  |  | x | x | x | x | x |
| Rev. B10 (2.10) |  |  |  | x | x | x | x | x |
| Rev. B11 (2.11) |  |  |  | x | x | x | x | x |
| OS-416, OS-416-4C (P/N 050190-06) |  |  |  |  |  |  |  |  |
| Rev. A5 (1.05) Initial Release |  |  |  | x | x | x | x | x |
| Rev. A6 (1.06) Initial Release |  |  |  | x | x | x | x | x |
| Rev. A7 (1.07) |  |  |  | x | x | x | x | x |
| Rev. A8 (1.08) |  |  |  | x | x | x | x | x |
| Rev. A9 (1.09) |  |  |  | x | x | x | x | x |
| 8277-524, 8277-524-8C (P/N 050196-06) |  |  |  |  |  |  |  |  |
| Rev. B3 (2.03) Initial Release |  |  | x | x | x | x | x | x |
| Rev. B4 (2.04) |  |  | x | x | x | x | x | x |
| Rev. B5 (2.05) |  |  | x | x | x | x | x | x |
| Rev. B6 (2.06) |  |  | x | x | x | x | x | x |
| Rev. B7 (2.07) |  |  | x | x | x | x | x | x |
| Rev. B8 (2.08) |  |  | x | x | x | x | x | x |
| Rev. B9 (2.09) |  |  | x | x | x | x | x | x |
| Rev. B10 (2.10) |  |  | x | x | x | x | x | x |
| Rev. B11 (2.11) |  |  | x | x | x | x | x | x |
| OSASM2-155FM-1 (P/N 050175-12) |  |  |  |  |  |  |  |  |
| Rev. A2 (1.02) Initial Release |  |  |  | x | x | x | x | x |
| Rev. A3 (1.03) |  |  |  | x | x | x | x | x |
| Rev. A4 (1.04) |  |  |  | x | x | x | x | x |
| OSASM2-155FS-1 (P/N 050175-13) |  |  |  |  |  |  |  |  |
| Rev. A2 (1.02) Initial Release |  |  |  | x | x | x | x | x |
| Rev. A3 (1.03) |  |  |  | x | x | x | x | x |
| Rev. A4 (1.04) |  |  |  | x | x | x | x | x |
| OSASM2-155FSH-1 (P/N 050175-14) |  |  |  |  |  |  |  |  |
| Rev. A2 (1.02) Initial Release |  |  |  | x | x | x | x | x |
| Rev. A3 (1.03) |  |  |  | x | x | x | x | x |
| Rev. A4 (1.04) |  |  |  | x | x | x | x | x |
| OSASM2-155RFM-1 (P/N 050175-09) |  |  |  |  |  |  |  |  |
| Rev. A3 (1.03) Initial Release |  |  |  | x | x | x | x | x |
| Rev. A4 (1.04) |  |  |  | x | x | x | x | x |
| Rev. A5 (1.05) |  |  |  | x | x | x | x | x |

| 36.2 | IBM 8277 Nways Ethernet RouteSwitch Hardware/Software Release Compatibility Matrix (Cont'd) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Software Release |  |  |  |  |  |  |  |  |
| Product |  | 3.1.8 | 3.1.9 | 3.2.1 | 3.2.2 | 3.2.3 | 3.2.4 | 3.2.5 | 3.2.6 |  |
| OSASM2-155RFS-1 (P/N 050175-10) |  |  |  |  |  |  |  |  |  |  |
| Rev. A3 (1.03) Initial Release |  |  |  |  | x | x | x | x | x |  |
| Rev. A4 (1.04) |  |  |  |  | x | x | x | x | x |  |
| Rev. A5 (1.05) |  |  |  |  | x | x | x | x | x |  |
| OSESM-100C-4 (P/N 050171-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A2 (1.02) Initial Release |  |  |  |  |  | x | x | x | x |  |
| Rev. A3 (1.03) |  |  |  |  |  | x | x | x | x |  |
| OSESM-100FM-2 (P/N 050170-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. A1 (1.01) |  |  |  | x | x | x | x | x | x |  |
| Rev. A2 (1.02) |  |  |  | x | x | x | x | x | x |  |
| OSESM-100FS-2 (P/N 050170-08) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) Initial Release |  |  |  | x | x | x | x | x | x |  |
| Rev. A1 (1.01) |  |  |  | x | x | x | x | x | x |  |
| Rev. A2 (1.02) |  |  |  | x | x | x | x | x | x |  |

## IBM Nways 8273 Ethernet RouteSwitchSwitch Hardware/Software Release Compatibility Matrix

|  | Software Release |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Product | 3.1.8 | 3.1.9 | 3.2.1 | 3.2.2 | 3.2.3 | 3.2.4 | 3.2.5 | 3.2.6 |
| IBM Nways 8273 Ethernet RouteSwitch10 (P/N 050152-06) |  |  |  |  |  |  |  |  |
| Rev. D (4.00) | x | x |  |  |  | x | x | x |
| Rev. D1 (4.01) | x | x |  |  |  | x | x | x |
| Rev. D2 (4.02) | x | x |  |  |  | x | x | x |
| Rev. D3 (4.03) | x | x |  |  |  | x | x | x |
| Rev. D4 (4.04) | x | x |  |  |  | x | x | x |
| Rev. D5 (4.05) | x | x |  |  |  | x | x | x |
| Rev. D6 (4.06) | x | x |  |  |  | x | x | x |
| Rev. D7 (4.07) | x | x |  |  |  | x | x | x |
| Rev. D8 (4.08) | x | x |  |  |  | x | x | x |
| Rev. D9 (4.09) | x | x |  |  |  | x | x | x |
| Rev. D10 (4.10) | x | x |  |  |  | x | x | x |
| Rev. D11 (4.11) | x | x |  |  |  | x | x | x |
| Rev. D12 (4.12) | x | x |  |  |  | x | x | x |
| Rev. D13 (4.13) | x | x |  |  |  | x | x | x |
| Rev. D14 (4.14) | x | x |  |  |  | x | x | x |
| Rev. D15 (4.15) | x | x |  |  |  | x | x | x |
| Rev. D17 (4.17) | x | x |  |  |  | x | x | x |
| Rev. D18 (4.18) | x | x |  |  |  | x | x | x |
| IBM Nways 8273 Ethernet RouteSwitch10U (P/N 050130-06) |  |  |  |  |  |  |  |  |
| Rev. A (1.00) | x | x |  |  |  | x | x | x |
| Rev. A1 (1.01) | x | x |  |  |  | x | x | x |
| Rev. A2 (1.02) | x | x |  |  |  | x | x | x |
| Rev. A3 (1.03) | x | x |  |  |  | x | x | x |
| Rev. A4 (1.04) | x | x |  |  |  | x | x | x |
| Rev. A5 (1.05) | x | x |  |  |  | x | x | x |
| Rev. A6 (1.06) | x | x |  |  |  | x | x | x |
| Rev. A7 (1.07) | x | x |  |  |  | x | x | x |
| Rev. A8 (1.08) | x | x |  |  |  | x | x | x |
| Rev. A9 (1.09) | x | x |  |  |  | x | x | x |
| Rev. A10 (1.10) | x | x |  |  |  | x | x | x |
| Rev. A11 (1.11) | x | x |  |  |  | x | x | x |
| Rev. A12 (1.12) | x | x |  |  |  | x | x | x |
| Rev. A13 (1.13) | x | x |  |  |  | x | x | x |
| Rev. A14 (1.14) | x | x |  |  |  | x | x | x |
| Rev. A15 (1.15) | x | x |  |  |  | x | x | x |
| Rev. A16 (1.16) | x | x |  |  |  | x | x | x |
| Rev. A17 (1.17) | x | x |  |  |  | x | x | x |
| Rev. A18 (1.18) | x | x |  |  |  | x | x | x |
| Rev. A19 (1.19) | x | x |  |  |  | x | x | x |
| Rev. A20 (1.20) | x | x |  |  |  | x | x | x |
| Rev. A22 (1.22) | x | x |  |  |  | x | x | x |
| Rev. A23 (1.23) | x | x |  |  |  | x | x | x |
| Rev. A24 (1.24) | x | x |  |  |  | x | x | x |

| 36.3 | IBM Nways 8273 Ethernet RouteSwitchSwitch Hardware/Software Release Compatibility Matrix (Cont'd) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Software Release |  |  |  |  |  |  |  |  |
| Product |  | 3.1.8 | 3.1.9 | 3.2.1 | 3.2.2 | 3.2.3 | 3.2.4 | 3.2.5 | 3.2.6 |  |
| IBM Nways 8273 Ethernet RouteSwitch10UE (P/N 050130-08) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) |  | x | x |  |  |  | x | x | x |  |
| Rev. A1 (1.01) |  | x | x |  |  |  | x | x | x |  |
| Rev. A2 (1.02) |  | x | x |  |  |  | x | x | x |  |
| Rev. A3 (1.03) |  | x | x |  |  |  | x | x | x |  |
| Rev. A4 (1.04) |  | x | x |  |  |  | x | x | x |  |
| Rev. A5 (1.05) |  | x | x |  |  |  | x | x | x |  |
| Rev. A6 (1.06) |  | x | x |  |  |  | x | x | x |  |
| Rev. A7 (1.07) |  | x | x |  |  |  | x | x | x |  |
| Rev. A8 (1.08) |  | x | x |  |  |  | x | x | x |  |
| Rev. A9 (1.09) |  | x | x |  |  |  | x | x | x |  |
| Rev. A10 (1.10) |  | x | x |  |  |  | x | x | x |  |
| Rev. A11 (1.11) |  | x | x |  |  |  | x | x | x |  |
| Rev. A13 (1.13) |  | x | x |  |  |  | x | x | x |  |
| Rev. A14 (1.14) |  | x | x |  |  |  | x | x | x |  |
| Rev. A15 (1.15) |  | x | x |  |  |  | x | x | x |  |
| IBM Nways 8273 Ethernet RouteSwitch10E (P/N 050152-08) |  |  |  |  |  |  |  |  |  |  |
| Rev. D (4.00) |  | x | x |  |  |  | x | x | x |  |
| Rev. D1 (4.01) |  | x | x |  |  |  | x | x | x |  |
| Rev. D2 (4.02) |  | x | x |  |  |  | x | x | x |  |
| Rev. D3 (4.03) |  | x | x |  |  |  | x | x | x |  |
| Rev. D4 (4.04) |  | x | x |  |  |  | x | x | x |  |
| Rev. D5 (4.05) |  | x | x |  |  |  | x | x | x |  |
| Rev. D6 (4.06) |  | x | x |  |  |  | x | x | x |  |
| Rev. D7 (4.07) |  | x | x |  |  |  | x | x | x |  |
| Rev. D8 (4.08) |  | x | x |  |  |  | x | x | x |  |
| Rev. D9 (4.09) |  | x | x |  |  |  | x | x | x |  |
| Rev. D10 (4.10) |  | x | x |  |  |  | x | x | x |  |
| Rev. D11 (4.11) |  | x | x |  |  |  | x | x | x |  |
| Rev. D12 (4.12) |  | x | x |  |  |  | x | x | x |  |
| Rev. D13 (4.13) |  | x | x |  |  |  | x | x | x |  |
| RESM-AB-AFD (P/N 050144-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) |  | x | x |  |  |  | x | x | x |  |
| Rev. A1 (1.01) |  | x | x |  |  |  | x | x | x |  |
| Rev. A2 (1.02) |  | x | x |  |  |  | x | x | x |  |
| Rev. A3 (1.03) |  | x | x |  |  |  | x | x | x |  |
| RESM-AB-FL (P/N 050085-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. B |  | x | x |  |  |  | x | x | x |  |
| Rev. C |  | x | x |  |  |  | x | x | x |  |
| Rev. D |  | x | x |  |  |  | x | x | x |  |
| Rev. E |  | x | x |  |  |  | x | x | x |  |
| Rev. E1 |  | x | x |  |  |  | x | x | x |  |
| Rev. E2 |  | x | x |  |  |  | x | x | x |  |
| Rev. E3 |  | x | x |  |  |  | x | x | x |  |
| Rev. E4 |  | x | x |  |  |  | x | x | x |  |
| RESM-AB-FL-S (P/N 050123-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) |  | x | x |  |  |  | x | x | x |  |
| Rev. A1 (1.01) |  | x | x |  |  |  | x | x | x |  |
| Rev. A2 (1.02) |  | x | x |  |  |  | x | x | x |  |
| Rev. A3 (1.03) |  | x | x |  |  |  | x | x | x |  |
| Rev. A4 (1.04) |  | x | x |  |  |  | x | x | x |  |
| RESM-AB-T (P/N 050091-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A |  | x | x |  |  |  | x | x | x |  |
| Rev. A1 (1.01) |  | x | x |  |  |  | x | x | x |  |
| Rev. A2 (1.02) |  | x | x |  |  |  | x | x | x |  |
| Rev. A3 (1.03) |  | x | x |  |  |  | x | x | x |  |
| Rev. A4 (1.04) |  | x | x |  |  |  | x | x | x |  |
| RESM-AB-B (P/N 050097-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. B |  | x | x |  |  |  | x | x | x |  |
| Rev. C |  | x | x |  |  |  | x | x | x |  |
| Rev. C1 |  | x | x |  |  |  | x | x | x |  |
| Rev. C2 |  | x | x |  |  |  | x | x | x |  |
| Rev. C3 |  | x | x |  |  |  | x | x | x |  |
| Rev. C4 |  | x | x |  |  |  | x | x | x |  |
| RESM-AB-AT (P/N 050079-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. B |  | x | x |  |  |  | x | x | x |  |
| Rev. B1 |  | x | x |  |  |  | x | x | x |  |
| RASM Daughter Card (P/N 050116-26) (1/2 MB SRAM) MM |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.01) |  | x | x |  |  |  | x | x | x |  |
| Rev. A 1 (1.01) |  | x | x |  |  |  | x | x | x |  |
| Rev. A2 (1.02) |  | x | x |  |  |  | x | x | x |  |
| Rev. A3 (1.03) |  | x | x |  |  |  | x | x | x |  |
| Rev. A4 (1.04) |  | x | x |  |  |  | x | x | x |  |
| Rev. A5 (1.05) |  | x | x |  |  |  | x | x | x |  |
| Rev. A6 (1.06) |  | x | x |  |  |  | x | x | x |  |
| RASM Daughter Card (P/N 050116-27) (1/2 MB SRAM) SM |  |  |  |  |  |  |  |  |  |  |
| Rev. A1 (1.01) |  | x | x |  |  |  | x | x | x |  |
| Rev. A2 (1.02) |  | x | x |  |  |  | x | x | x |  |
| Rev. A3 (1.03) |  | x | x |  |  |  | x | x | x |  |
| Rev. A4 (1.04) |  | x | x |  |  |  | x | x | x |  |
| Rev. A5 (1.05) |  | x | x |  |  |  | x | x | x |  |
| Rev. A6 (1.06) |  | x | x |  |  |  | x | x | x |  |
| RASM Daughter Card (P/N 050116-24) (2 MB SRAM) MM |  |  |  |  |  |  |  |  |  |  |
| Rev. A1 (1.01) |  | x | x |  |  |  | x | x | x |  |
| Rev. A2 (1.02) |  | x | x |  |  |  | x | x | x |  |
| Rev. A3 (1.03) |  | x | x |  |  |  | x | x | x |  |
| Rev. A4 (1.04) |  | x | x |  |  |  | x | x | x |  |
| Rev. A5 (1.05) |  | x | x |  |  |  | x | x | x |  |
| Rev. A6 (1.06) |  | x | x |  |  |  | x | x | x |  |
| Rev. A7 (1.07) |  | x | x |  |  |  | x | x | x |  |
| RASM Daughter Card (P/N 050116-25) (2 MB SRAM) SM |  |  |  |  |  |  |  |  |  |  |
| Rev. A1 (1.01) |  | x | x |  |  |  | x | x | x |  |
| Rev. A2 (1.02) |  | x | x |  |  |  | x | x | x |  |
| Rev. A3 (1.03) |  | x | x |  |  |  | x | x | x |  |
| Rev. A4 (1.04) |  | x | x |  |  |  | x | x | x |  |
| Rev. A5 (1.05) |  | x | x |  |  |  | x | x | x |  |
| Rev. A6 (1.06) |  | x | x |  |  |  | x | x | x |  |
| Rev. A7 (1.07) |  | x | x |  |  |  | x | x | x |  |
| RFESM II 100BTX (P/N 050139-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) |  | x | x |  |  |  | x | x | x |  |
| Rev. A1 (1.01) |  | x | x |  |  |  | x | x | x |  |
| Rev. A2 (1.02) |  | x | x |  |  |  | x | x | x |  |
| Rev. A3 (1.03) |  | x | x |  |  |  | x | x | x |  |
| Rev. A4 (1.04) |  | x | x |  |  |  | x | x | x |  |
| Rev. A5 (1.05) |  | x | x |  |  |  | x | x | x |  |
| Rev. A6 (1.06) |  | x | x |  |  |  | x | x | x |  |
| Rev. A7 (1.07) |  | x | x |  |  |  | x | x | x |  |
| Rev. A8 (1.08) |  | x | x |  |  |  | x | x | x |  |
| RFESM II 100BTX (P/N 050139-13) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) |  | x | x |  |  |  | x | x | x |  |
| Rev. A1 (1.01) |  | x | x |  |  |  | x | x | x |  |
| Rev. A2 (1.02) |  | x | x |  |  |  | x | x | x |  |
| Rev. A3 (1.03) |  | x | x |  |  |  | x | x | x |  |
| Rev. A4 (1.04) |  | x | x |  |  |  | x | x | x |  |
| Rev. A5 (1.05) |  | x | x |  |  |  | x | x | x |  |
| Rev. A6 (1.06) |  | x | x |  |  |  | x | x | x |  |
| RESM-100FM-FD (P/N 050122-13) |  |  |  |  |  |  |  |  |  |  |
| Rev. B (2.00) |  | x | x |  |  |  | x | x | x |  |
| Rev. B1 (2.01) |  | x | x |  |  |  | x | x | x |  |
| Rev. B2 (2.02) |  | x | x |  |  |  | x | x | x |  |
| Rev. B3 (2.03) |  | x | x |  |  |  | x | x | x |  |
| Rev. B4 (2.04) |  | x | x |  |  |  | x | x | x |  |
| Rev. B5 (2.05) |  | x | x |  |  |  | x | x | x |  |
| RESM-100C-FD (P/N 050122-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. B (2.00) |  | x | x |  |  |  | x | x | x |  |
| Rev. B1 (2.01) |  | x | x |  |  |  | x | x | x |  |
| Rev. B2 (2.02) |  | x | x |  |  |  | x | x | x |  |
| Rev. B3 (2.03) |  | x | x |  |  |  | x | x | x |  |
| Rev. B4 (2.04) |  | x | x |  |  |  | x | x | x |  |
| RESM-100FS-FD-1 (P/N 050122-14) |  |  |  |  |  |  |  |  |  |  |
| Rev. B (2.00) |  | x | x |  |  |  | x | x | x |  |
| Rev. B1 (2.01) |  | x | x |  |  |  | x | x | x |  |
| Rev. B2 (2.02) |  | x | x |  |  |  | x | x | x |  |
| Rev. B3 (2.03) |  | x | x |  |  |  | x | x | x |  |
| Rev. B4 (2.04) |  | x | x |  |  |  | x | x | x |  |
| Rev. B5 (2.05) |  | x | x |  |  |  | x | x | x |  |
| RWSM-SC-4 WAN (P/N 050117-06) |  |  |  |  |  |  |  |  |  |  |
| Rev. A2 (1.02) |  |  |  |  |  |  |  |  |  |  |
| Rev. A3 (1.03) |  |  |  |  |  |  |  |  |  |  |
| Rev. A4 (1.04) |  | x | x |  |  |  | x | x | x |  |
| Rev. A5 (1.05) |  | x | x |  |  |  | x | x | x |  |
| Rev. A6 (1.06) |  | x | x |  |  |  | x | x | x |  |
| Rev. A7 (1.07) |  | x | x |  |  |  | x | x | x |  |
| Rev. A8 (1.08) |  | x | x |  |  |  | x | x | x |  |
| Rev. A9 (1.09) |  | x | x |  |  |  | x | x | x |  |
| Rev. A10 (1.10) |  | x | x |  |  |  | x | x | x |  |
| RWSM-S-2 WAN (P/N 050117-13) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.00) |  | x | x |  |  |  | x | x | x |  |
| Rev. A1 (1.01) |  | x | x |  |  |  | x | x | x |  |
| Rev. A2 (1.02) |  | x | x |  |  |  | x | x | x |  |
| Rev. A3 (1.03) |  | x | x |  |  |  | x | x | x |  |
| Rev. A4 (1.04) |  | x | x |  |  |  | x | x | x |  |
| Rev. A5 (1.05) |  | x | x |  |  |  | x | x | x |  |
| Rev. A6 (1.06) |  | x | x |  |  |  | x | x | x |  |
| Rev. A7 (1.07) |  | x | x |  |  |  | x | x | x |  |
| RFSM Daughter Card (P/N 050037-06) (MM only) |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.01) |  | x | x |  |  |  | x | x | x |  |
| Rev. B (1.02) |  | x | x |  |  |  | x | x | x |  |
| Rev. C (1.03) |  | x | x |  |  |  | x | x | x |  |
| Rev. D (1.04) |  | x | x |  |  |  | x | x | x |  |
| Rev. E (1.05) |  | x | x |  |  |  | x | x | x |  |
| Rev. E1 (1.06) |  | x | x |  |  |  | x | x | x |  |
| RFSM Daughter Card (P/N 050108-06) (MM) |  |  |  |  |  |  |  |  |  |  |
| Rev. C (2.03) |  | x*** | x*** |  |  |  | x*** | x*** | x*** |  |
| Rev. C1 (2.04) |  | x | x |  |  |  | x | x | x |  |
| Rev. C2 (2.05) |  | x | x |  |  |  | x | x | x |  |
| Rev. C3 (2.06) |  | x | x |  |  |  | x | x | x |  |
| Rev. C4 (2.07) |  | x | x |  |  |  | x | x | x |  |
| Rev. C5 (2.08) |  | x | x |  |  |  | x | x | x |  |
| RFSM Daughter Card (P/N 050108-13) (SM) |  |  |  |  |  |  |  |  |  |  |
| Rev. B (2.00) Initial Release |  | x | x |  |  |  | x | x | x |  |
| Rev. C3 (2.06) |  | x | x |  |  |  | x | x | x |  |
| Rev. C4 (2.07) |  | x | x |  |  |  | x | x | x |  |
| Rev. C5 (2.08) |  | x | x |  |  |  | x | x | x |  |
| RFSM Daughter Card (P/N 050073-06) UTP |  |  |  |  |  |  |  |  |  |  |
| Rev. A (1.01) |  | x | x |  |  |  | x | x | x |  |
| Rev. B (1.02) |  | x | x |  |  |  | x | x | x |  |
| Rev. C (1.03) |  | x | x |  |  |  | x | x | x |  |
| Rev. D (1.04) |  | x | x |  |  |  | x | x | x |  |
| Rev. E (1.05) |  | x | x |  |  |  | x | x | x |  |
| Rev. E1 (1.06) |  | x | x |  |  |  | x | x | x |  |
| Rev. E2 (1.06) |  | x | x |  |  |  | x | x | x |  |

**	All FSM-M-2 modules require REV K or greater HSM or any released HSM2.

*** 	This revision and above is required for MPM redundancy.

****	This revision and above is required for Hot Swap.

This revision and above is required to support the 960 VBUS.

This revision or above (within the same alpha range) are required to support 960 VBUS Speed.

2 This revision or later is required for hot swap of modules.  Contact Customer Support  for upgrade.

1

2