EX-1489  ICF-3 (Icom Cloning Format-3) cloning software ver. 1.1

PMR DOCUMENT

For Icom's PMR PORTABLE RADIOS

IC-F30LT, IC-F40LT, IC-F30, IC-F40

Copyright 1991-1994 Icom Inc.

Portions of the software copyrighted

by Microsoft Corporation 1982-1988

**                                                       **

**  IMPORTANT: There are 3 document files on this disk:  **

**  LMR.DOC, PMR.DOC and COMMON.DOC. The COMMON.DOC file **

**  MUST be read first as it contains basic startup,     **

**  operating and shutdown procedures common to all      **

**  types of F30/F40 series transceivers. If you have    **

**  not read that file yet, quit this file and open the  **

**  COMMON.DOC file before proceeding any further.       **

**                                                       **

-------------------------------------------------------------------------------

DIFFERENCES BETWEEN THE IC-F30LT/40LT AND IC-F30/F40 PROGRAMMING SOFTWARE

-------------------------------------------------------------------------------

This document file explains each of the screens contained in the programming

software for both the IC-F30LT/F40LT(PMR) and IC-F30/F40(PMR) transceivers. The

differences between these two programs are as follows.

Number of memory channels     IC-F30/F40: 16     IC-F30LT/F40LT: 96 or 99

Programmable text channels    IC-F30/F40: none   IC-F30LT/F40LT: 32

Although this document file refers mainly to the IC-F30LT/F40LT(PMR), basic

programming for the IC-F30/F40(PMR) is the same.  However, keep in mind the

above differences between the two types of transceivers.

- 1 -

-------------------------------------------------------------------------------

TABLE OF CONTENTS

-------------------------------------------------------------------------------

1.  SCREEN DESCRIPTIONS ..............................   3

MEMORY CH SCREEN ...............................   3

TX CODE CH SCREEN ..............................  12

RX CODE CH SCREEN ..............................  15

DTMF CH SCREEN .................................  19

SCAN LIST SCREEN ...............................  20

TEXT CHANNEL SCREEN ............................  22

USER TONE SCREEN ...............................  23

COMMON SCREEN ..................................  24

2.  CLONING OPERATION ................................  39

3.  FILE SAVING and LOADING ..........................  41

4.  PRINTING .........................................  43

- 2 -

-------------------------------------------------------------------------------

1. SCREEN DESCRIPTIONS

-------------------------------------------------------------------------------

////// MEMORY CHANNEL SCREEN //////

NOTE: There are 16 channels only, without BANKs, for the IC-F30/F40; 6 BANK

screens (BANK1 to BANK 6) or 10 FREE CHANNEL screens for programming of channel

data for the IC-F30LT/F40LT.  Which of these sets appears depends on how the

"Rotary Selector" is programmed in the COMMON screen (p. 24).  The following

example shows the IC-F30/F40 screen; when selecting the IC-F30LT/F40LT, 6 banks

or up to 99 channels can be used.

+[Memory CH]---------------+---------------+----+-----+----+-----+-----+

|CH  |      Frequency (MHz)| CTCSS/DTCS    |Text|Power|TOT |Lock |RF   |

| No.|Rx        |Tx        |RX     |TX     |CH  |Save |    |-out |Power|

+----+----------+----------+-------+-------+----+-----+----+-----+-----+

|  1 |146.25000 |146.25000 |OFF    |OFF    |OFF |ON   |OFF |OFF  |High |

|  2 |160.25000 |160.25000 |OFF    |OFF    |OFF |ON   |OFF |OFF  |High |

|  3 |173.75000 |173.75000 |OFF    |OFF    |OFF |ON   |OFF |OFF  |High |

~    |          |          |       |       |    |     |    |     |     ~

~    |          |          |       |       |    |     |    |     |     ~

| 15 |173.75000 |173.75000 |023N   |023N   |OFF |ON   |OFF |OFF  |High |

| 16 |136.05000 |136.05000 |OFF    |OFF    |OFF |ON   |OFF |OFF  |High |

+----+----------+----------+-------+-------+----+-----+----+-----+-----+

<continued from the right side of the above screen>

+----++------------------+---------+-----------+----------+-----------------+

|CH  ||    LOG IN/OFF    |User Scan|CH Mute    |CH Select | Monitor Key Act |

| No.||Audible |Inaudible|Include  |Operation  |Action    |Audible|Inaudible|

+----++--------+---------+---------+-----------+----------+-------+---------+

|  1 ||OFF     |OFF      |Enable   |Single     |Inaudible |ON     |ON       |

|  2 ||OFF     |OFF      |Enable   |Single     |Inaudible |ON     |ON       |

|  3 ||OFF     |OFF      |Enable   |Single     |          |ON     |ON       |

~    ||        ~         |         |           |          |       |         ~

~    ||        ~         |         |           |          |       |         ~

| 15 ||OFF     |OFF      |Enable   |Single     |Inaudible |ON     |ON       |

| 16 ||OFF     |OFF      |Enable   |Single     |Inaudible |ON     |ON       |

+----++--------+---------+---------+-----------+----------+-------+---------+

<continued from the right side of the above screen>

+----++---------+--------+--------+------+--------------+----+--------+------+

|CH  || Audible |Auto    |Call    |5-Tone|5-Tone CH     |ID  |R-NR    |2-Tone|

| No.||Call|PTT |Reset   |Inactive|Format|RPT |STN |ID  |Pos.|12345678|Decode|

+----++----+----+--------+--------+------+----+----+----+----+--------+------+

|  1 ||ON  |OFF |Timer B |OFF     |CCIR  |OFF | 1  |OFF |OFF |12345678|OFF   |

|  2 ||ON  |OFF |Timer B |OFF     |CCIR  |OFF | 2  |OFF |OFF |12345678|OFF   |

|  3 ||ON  |OFF |Timer B |OFF     |CCIR  |OFF | 3  |OFF |OFF |12345678|OFF   |

~     ~                                ~                                     ~

~     ~                                ~                                     ~

| 15 ||ON  |OFF |Timer B |OFF     |CCIR  |OFF |OFF |OFF |OFF |12345678|OFF   |

| 16 ||ON  |OFF |Timer B |OFF     |CCIR  |OFF |OF  |OFF |OFF |12345678|OFF   |

+----++----+----+--------+--------+------+------+----------+----+----+-------+

- 3 -

The Memory CH screen automatically appears when selecting "IC-F30/40 5-TONE"

or "IC-F30LT/F40LT 5-TONE" in the opening screen.  This screen allows you to

set the attributes of each channel.

1) Select the desired channel line and desired item column using the [Arrow]

keys.

- The selection is highlighted.

2) Enter the desired digits or characters, if applicable, then push [Enter];

or push [Enter] then select an item in a submenu; or push [Space] or

[Backspace] to  toggle the selection.

- See the descriptions of each column item for function and setting

details.

Frequency RX and TX(MHz)

Receive and transmit frequencies for each channel.  Enter receive or transmit

frequencies starting from the 100 MHz digit.  Usable frequency coverage differs

depending on version as follows:

**                                                    **

**  Usable frequency coverage                         **

**                                                    **

**  IC-F30   L-band type     136.00000 to 150.00000   **

**           H-band type     146.00000 to 174.00000   **

**  IC-F40   L-band type     400.00000 to 430.00000   **

**           M-band type     430.00000 to 470.00000   **

**           H-band type     470.00000 to 520.00000   **

**                                                    **

Selectable value:    000.00000 to 799.99500 (MHz)

in 0.00500 (5 kHz) or 0.00625 (6.25 kHz) steps

Selection method:    Type the desired frequency in directly, then

push [Enter].

**                                                   **

** NOTE: Even though out-of-frequency values can be  **

**       set in this program, the transceiver may    **

**       not accept such frequencies.                **

**                                                   **

- 4 -

CTCSS/DTCS RX and TX (MHz)

These two items are used to select tone frequencies when using CTCSS

(continuous tone coded squelch system) or DTCS (digital tone coded squelch) or

to turn either system off for a given channel.

Selectable value:    67.0    103.5    159.8    203.5

(CTCSS)     69.3    107.2    162.2    206.5

71.9    110.9    165.5    210.7

74.4    114.8    167.9    218.1

77.0    118.8    171.3    225.7

79.7    123.0    173.8    229.1

82.5    127.3    177.3    233.6

85.4    131.8    179.9    241.8

88.5    136.5    183.5    250.3

91.5    141.3    186.2    254.1

94.8    146.2    189.9

97.4    151.4    196.6

100.0   156.7    199.5

- 5 -

(DTCS)      000(I) to 777(I)   (Some codes are redundant, refer to

"I" means inverse   the table below.)

++-----+-----------------++-----+-----------------++-----+-----------------+

|| 000 | 352             || 105 | 204 247 420 710 || 252 | 661             |

|| 001 | 476 760         || 106 | 221 241 304 424 || 254 | 314 612 706     |

|| 002 | 522 540         || 107 | 365             || 255 | 425             |

|| 003 | 100             || 110 | 126 302 430     || 257 | 705             |

|| 004 | 300 344         || 112 | 250 505 512     || 262 | 316 431 730     |

|| 005 | 044 400         || 113 | 573             || 266 | 266 655         |

|| 006 |                 || 114 | 327 615         || 271 | 427 510 762     |

|| 007 | 670             || 115 | 534 674         || 272 |                 |

|| 010 | 033 600         || 117 | 411 756         || 274 | 652             |

|| 012 | 215 320         || 122 | 535             || 275 |                 |

|| 013 | 063 700         || 123 | 632 657         || 276 | 326 432         |

|| 014 | 450 500 544     || 125 | 173             || 307 | 362 565         |

|| 015 | 740 747         || 127 | 412 441 711     || 311 | 330 456 561     |

|| 016 | 154 206         || 130 | 364 641         || 312 | 515 663 743     |

|| 017 | 200             || 131 | 572 702         || 315 | 321 673         |

|| 020 | 170 230 601     || 132 | 605 634 714     || 317 | 546 614 751     |

|| 021 | 277 402         || 133 | 413 620         || 324 | 343 570         |

|| 022 | 264 461 613     || 134 | 273             || 325 | 550 626         |

|| 023 | 340 766         || 135 | 205 610         || 331 | 372 507         |

|| 024 | 120 260         || 136 | 502 712         || 332 | 433 552         |

|| 025 |                 || 141 | 177 541         || 335 |                 |

|| 026 | 566             || 142 | 174 270         || 336 | 770             |

|| 027 | 201 242         || 143 | 333             || 337 |                 |

|| 030 | 055             || 144 | 466 666         || 344 | 471 664 715     |

|| 031 | 374             || 145 | 525             || 346 | 616 635 724     |

|| 032 |                 || 146 | 220 414 422 442 || 347 | 434 776         |

|| 034 | 103 140 410     || 147 | 303 306 761     || 351 | 353 43562       |

|| 035 | 124 403         || 150 | 256 703         || 356 | 521             |

|| 036 | 137             || 152 | 366 415         || 357 | 477 774         |

|| 037 | 560 627         || 153 | 606 630         || 361 | 373             |

|| 040 | 052 404         || 155 | 233 660         || 363 | 436 443 444 662 |

|| 041 | 111 451 514 602 || 156 | 517 741         || 367 | 676             |

|| 042 | 160 216 341     || 157 | 322 503         || 376 | 617 763         |

|| 043 | 355             || 161 | 345 532         || 446 | 467 511 672     |

|| 045 | 240 305 543     || 162 | 416 553         || 447 | 473 474 731 744 |

|| 046 | 202 210 421 644 || 163 | 460 607 654     || 452 | 524 765         |

|| 047 | 375 707         || 164 | 207 732         || 454 | 513 545 564     |

|| 050 | 167             || 165 | 354             || 455 | 533 551         |

|| 051 | 520 771         || 166 | 773             || 462 | 472 623 725     |

|| 053 |                 || 171 | 265 426         || 463 | 637 775         |

|| 054 | 405 675         || 175 |                 || 523 | 647 726         |

|| 056 | 465 656         || 176 | 244 417         || 526 | 562 645         |

|| 057 | 172             || 212 | 253             || 527 | 764             |

|| 060 | 116 737         || 213 | 263 736         || 537 | 735             |

|| 061 | 211 232 650     || 214 | 310 377 437     || 547 | 757             |

|| 062 | 070 101 407     || 217 | 371 453 530     || 555 | 571             |

|| 064 | 151 406 440     || 222 | 445 457 575     || 556 |                 |

|| 065 | 301             || 223 | 350 475 750     || 576 | 722             |

|| 066 | 737             || 224 | 313 506 574     || 577 |                 |

|| 067 | 516 720         || 225 | 536             || 633 | 667             |

|| 071 | 603 717 746     || 227 | 261 567         || 646 | 665             |

|| 072 | 470 701         || 231 | 504 631 636 745 || 651 | 677             |

|| 073 | 640             || 234 | 423 563 621 713 || 716 | 727             |

|| 074 | 360 721         || 235 | 611 671 723     || 733 |                 |

|| 075 | 501 624         || 236 | 251 704 742     || 752 | 755             |

|| 076 | 203 754         || 237 | 464 642 772     || 753 |                 |

|| 077 |                 || 243 | 267 342         || 767 |                 |

|| 102 | 121 323 604     || 245 | 370 554         || 777 |                 |

|| 104 | 226 557         || 246 | 542 653         ||     |                 |

++-----+-----------------++-----+-----------------++-----+-----------------+

- 6 -

Selection method:    Type the desired tone frequency in directly, then

push [Enter]; or

Push [Enter] to select the submenu, then make

a selection from the submenu; or

Push [Space] or [Backspace] to toggle the

selection.

Text CH

This item is used to set pre-programmed text to a channel for indication in

the function display. For a list of the factory pre-programmed text (text

channels 33 to 99, refer to p. 22.

**                                           **

** NOTE: IC-F30/F40 have no LCD, therefore   **

**       text channels cannot be displayed   **

**       even though this software allows    **

**       them to be programmed.              **

**                                           **

Selectable value:    01 to 99 or OFF

When 01 to 32 is used, text can be programmed in

the TEXT CHANNEL screen (p. 22).

Selection method:    Type the desired selection in directly, then

push [Enter].

Power Save

This item is used to turn the power save function ON or OFF for a particular

channel. When the power save function is activated, the current drain for the

stand by condition is reduced by approx. 25% compared to when the power save

function is OFF.

Selectable value:    ON or OFF

Selection method:    Push [Space] or [Backspace] to toggle the

selection.

TOT

This item is used to turn the time-out timer function ON or OFF for a

particular channel. The time-out time can be specified in the COMMON screen

(p. 32).

Selectable value:    ON or OFF

Selection method:    Push [Space] or [Backspace] to toggle the

selection.

- 7 -

Lockout

This item is used to set the lockout condition for a particular channel. OFF

sets the channel as a non-lockout channel (transmit is possible at any time);

Busy sets the channel so that transmit is inhibited while receiving a signal;

and Rpt(repeater) sets the channel so that transmit is normally inhibited while

receiving a signal but is not inhibited while receiving a signal with a matched

CTCSS tone. Refer to "Audible Call/PTT" which functions similarly to the

lockout function.

Selectable value:    OFF, Busy or Rpt(repeater)

Selection method:    Push [Enter] to select the submenu, then

make a selection; or

Push [Space] or [Backspace] to toggle the

selection.

RF power

This item sets the RF output power for a particular channel. RF output level

can be selected from one of 3 levels (1 high and 2 low). These output power

levels are dealer-programmable from their default values (refer to the

Programming Manual for details). Make sure "TX Power Adjust" (p. 31) in

the COMMON  screen is set to "Enable" in order to set an output power as

described in the Programming Manual.

Selectable value:    Low1 (default= 1 W)

Low2 (default= 3 W)

High (default= 5 W)

Selection method:    Push [Enter] to select the submenu, then

make a selection; or

Push [Space] or [Backspace] to toggle the

selection.

Log IN/OFF Audible/Inaudible

These items are used for code transmission related to 5-Tone operation.

When ON is selected, for one or both of the items, an ID code is sent on

a pre-assigned TX code channel at the moment the PTT is pushed, (Log IN) or

released, (Log OFF).  See p. 10 for TX code channel assignment and p. 12 for TX

code programming.

Selectable value:     Log IN, Log OFF, both or OFF

Selection method:     Push [Enter] to select the submenu, then make

a selection; or push [Space] or [Backspace] to

toggle the selection.

**                                                         **

** Audible condition: 5-Tone receive mute is cancelled.    **

**                                                         **

** Inaudible condition: 5-Tone receive mute is activated.  **

**                      This means that only signals which **

**                      contain the correct code can set   **

**                      the transceiver to an audible      **

**                      condition.                         **

**                                                         **

- 8 -

User Scan Include

This item is used to enable or inhibit users from modifying scan lists. When

enabled, a user can lockout or add channels to preprogrammed scan lists by

pushing and holding a SCAN switch; when inhibited, scan lists cannot be

modified by users.

Selectable value:     Enable or Inhibit

Selection method:     Push [Space] or [Backspace] to toggle the

selection.

CH Mute Operation

Each channel has two mute conditions, continuous tone (CTCSS/DTCS) and Single

tone (5-Tone/2-Tone).  This item selects the audio mute method as continuous

only or single with continuous. When selecting "Single" both CTCSS (or DTCS)

and 5-Tone (or 2-Tone) must be matched in order to open the audio mute. In

addition, the PTT switch functions only for the audible condition when "Single"

is selected, with no restriction when "Continuous" is selected.

Selectable value:     Continuous or Single

Selection method:     Push [Space] or [Backspace] to toggle the

selection.

CH Select Action

This item sets the audible/inaudible condition (without pushing the monitor

switch), when selecting a channel. "Audible" or "Inaudible" selects

the condition; "_____" does not change the condition even when selecting a

channel.

Selectable value:    Inaudible, Audible or _______

Selection method:    Push [Space] or [Backspace] to toggle the

selection.

Monitor Key Action Audible/Inaudible

These items activate and deactivate the monitor switch. When OFF is selected

for both the "Audible" and "Inaudible" items, the monitor switch does not

function on the specified channel. When OFF is selected for one of the items

only, for example "Inaudible" only, the monitor switch activates "ON" only.

Selectable value:     ON or OFF

Selection method:     Push [Space] or [Backspace] to toggle the

selection.

Audible Call/PTT

This item allows you to use the Call/PTT switch to temporarily change to

audible mode from inaudible mode when ON is selected.

Selectable value:     ON or OFF

Selection method:     Push [Space] or [Backspace] to toggle the

selection.

- 9 -

Auto Reset

This item sets the time period before the transceiver automatically switches

back to inaudible mode from audible mode. When audible mode is selected after

receiving a call, the time period before switching back to inaudible mode is

dependent on this item and the following item, Call Inactive. The shorter of

the two takes priority. Refer to p. 34 for setting the Timer A and Timer B

values.

Selectable value:     Timer A or Timer B

Selection method:     Push [Space] or [Backspace] to toggle the

selection.

Call Inactive

This item sets the time period before the transceiver automatically switches

back to inaudible mode after transmitting or receiving a 5-Tone or 2-Tone call.

See previous item and p. 34 for setting Inactive timer.

Selectable value:      ON or OFF

Selection method:      Push [Space] or [Backspace] to toggle the

selection.

5-Tone Format

This item sets the 5-Tone format for a particular channel.

Selectable value:      CCIR, ZVEI1, ZVEI2, DZVE1, EEA, EEA2, DAPL,

EIA, DTMF and User

Selection method:      Push [Enter] to select the submenu, then

make a selection; or

Push [Space] or [Backspace] to toggle the

selection.

5-Tone CH RPT/STN/ID

The transceiver can transmit up to 3 kinds of 5-Tone codes in sequence.

1. RPT (repeater code), 2. STN (station code), 3. ID (ID code).

Assign TX code channel numbers to these codes in this item.

Note that the order of code transmission can be changed in the "ID Pos." item.

Selectable value:      1 to 16, 18, ..., 30, 32; or OFF

* after a value indicates long tone is ON.

(maximum number differs depending on "DTMF:

TX Code CH" selection in the COMMON screen.)

Selection method:      Type the desired channel code in directly, then

push [Enter], or;

Push [Space] to turn the code OFF.

**                                                           **

** NOTE: When STN (station) is set to OFF, the [CALL] switch **

**       has no function.                                    **

**       ID codes should not be set to OFF, otherwise ID     **

**       transmission is impossible.  If you do not need an  **

**       ID code, set "ID Pos." to OFF in the next item.     **

**                                                           **

- 10 -

ID Pos.

This item sets the position of the transceiver's ID code in relation to the

station code when making a 5-Tone call. When Top is selected, the Repeater

code, ID code and Station code are transmitted in sequence. When BTM (bottom)

is selected, the Repeater code, Station code and ID code are transmitted in

sequence. When OFF is selected, the Repeater code and Station code only are

transmitted in sequence. Decoding also functions in the set sequence. Note

that the repeater code is not decoded.

Selectable value:      OFF, BTM(bottom) or Top

Selection method:      Push [Enter] to select the submenu, then

make a selection; or

Push [Space] or [Backspace] to toggle the

selection.

R-NR (receive-nonreceive)

12345678

This item is used to toggle the receive code channels between effective

and non-effective (there are 8 receive code channels, refer to p. 15).

Selectable value:      [1] to [8]

(Indicated numbers are effective and non-

indicated numbers are not effective.)

Selection method:      Push a digit key to toggle the corresponding

receive channel ON and OFF.

2-Tone Decode

This item enables selected channels to receive 2-Tone calls. Note that even

when ON is selected, 2-Tone codes cannot be decoded unless an optional UT-80

2-TONE DECODER UNIT is installed in the transceiver. Refer to "RX Code CH

SCREEN" (p. 15) for operation after receiving a 2-Tone signal. Decode

codes are adjusted on the UT-80.

Selectable value:      ON or OFF

Selection method:      Push [Space] or [Backspace] to toggle the

selection.

- 11 -

///// TX Code CH SCREEN /////

+[TX Code CH]+-------+----+----+-------+

|CH  |Code   |Input  |Up- |Text|ABC    |

| No.|       |Digit  |date|CH  |Dec|Aud|

+----+-------+-------+----+----+---+---+

|  1 |12345  |12345--|OFF |OFF |OFF|OFF|

|  2 |67890  |12345--|OFF |OFF |OFF|OFF|

|  3 |14725  |12345--|OFF |OFF |OFF|OFF|

~                                      ~

~                                      ~

| 30 |-------|-------|--- |--- |---|---|

| 31 |-------|-------|--- |--- |---|---|

| 32 |-------|-------|--- |--- |---|---|

+----+-------+-------+----+----+---+---+

The TX CODE CH screen is accessed from the Screen item in the TOP menu. This

screen allows you to set 5-Tone code transmit conditions.

1) Select the desired line (for the corresponding TX channel) and the desired

item using the [Arrow] keys.

- The selection is highlighted.

2) Push [Enter] to select a submenu, then select the desired data with the

[Arrow] and [Enter] keys; or

enter the desired digits or characters if applicable, then push [Enter].

- See the descriptions of each column item for function and setting

details.

Code

This item sets a 5-Tone code for transmission on the selected channel.

Selectable value:      [0] to [9], [A] to [E], [G] and [*]

(G and * are the group tones and are the same

tones as specified in the COMMON screen.)

Selection method:      Push the appropriate digit and/or character

keys for the code desired. Up to 7 digits

can be entered.

**                                            **

** NOTE: DO NOT use special tones for regular **

**       codes, otherwise correct 5-Tone      **

**       detection may not be possible.       **

**       Special tones can be set in the      **

**       COMMON screen.                       **

**                                            **

**                                            **

** NOTE: Before setting this item, set a      **

**       desired special tone (group) in the  **

**       COMMON screen.                       **

**                                            **

- 12 -

Input Digit

When the [TX Code] function is assigned to a key, transmit codes can be

input manually. In this case, set the allowable keys for input using this

item.  For the IC-F30/F40, an optional UT-82 keypad is necessary to input

digits.

Selectable value:      [1] to [7]

(Indicated numbers mean key entry accept; non-

indicated numbers mean key entry inhibit.)

Selection method:       Push a digit key (1-7) to toggle the

corresponding input digit ON and OFF.

Update

This item allows you to set the transceiver so that transmit codes for a

given memory channel can be overwritten with the [TX Code] key. When this item

is set to OFF, TX code channels can still be changed with the [TX Code] key,

however, when power is turned OFF, the channel reverts to its previous setting.

For the IC-F30/F40, an optional UT-82 keypad is necessary to input a 5-Tone

code manually.

Selectable value:       ON or OFF

Selection method:       Push [Enter] to select the submenu, then

make a selection; or

Push [Space] or [Backspace] to toggle the

selection.

Text CH

This item is used to select text for display when a TX code channel is

selected and the "LCD Display" item in the COMMON screen is set to "MR CH +

TX Code."

Selectable value:      01 to 99 or OFF

Selection method:      Type the desired text channel in directly,

then push [Enter] or push [Space] to select

OFF.

**                                           **

** NOTE: IC-F30/F40 have no LCD, therefore   **

**       text channels cannot be displayed   **

**       even though this software allows    **

**       them to be programmed.              **

**                                           **

- 13 -

ABC Dec

This item is used to decode the answer back code for display indication after

transmitting a code on a particular channel.

Selectable value:       ON or OFF

Selection method:       Push [Space] or [Backspace] to toggle the

selection.

**                                           **

** NOTE: IC-F30/F40 have no LCD, therefore   **

**       text channels cannot be displayed   **

**       even though this software allows    **

**       them to be programmed.              **

**                                           **

ABC Aud

This item sets the transceiver to audible mode after an answer back code is

received on a particular channel.

Selectable value:       ON or OFF

Selection method:       Push [Space] or [Backspace] to toggle the

selection.

- 14 -

///// RX Code CH SCREEN /////

All RX code channels are used for the IC-F30LT/F40LT.  An optional UT-83 5-TONE

UNIT is necessary for the IC-F30/F40.  When an optional UT-80 2-TONE UNIT is

installed, only RX code channels G and 1 are used for group and individual

codes, respectively.

+[RX Code CH]-+-------+-------------------------------------------------------+

|CH  |Code    |Status |Decode Action               ID   Audible               |

| No.|        |Digit  |TX  |LED |Scan   |Beep     |Dec.|Mode   |Text|ABC |Stun|

+----+--------+-------+----+----+-------+---------+----+-------+----+----+----+

|  G |------- |-------|OFF |ON  |------ |PiPi     |OFF |Audible|OFF |OFF |----|

|  1 |14725   |-------|OFF |ON  |------ |PiPi     |OFF |Audible| 36 |OFF |----|

~                                                                             ~

~                                                                             ~

|  7 |        |-------|OFF |ON  |------ |PiPi     |OFF |Audible| 36 |OFF |----|

|  8 |        |-------|OFF |ON  |------ |PiPi     |OFF |Audible| 36 |OFF |----|

+----+--------+-------+----+----+-------+---------+----+-------+----+----+----+

The RX CODE CH screen is accessed from the Screen item in the TOP menu. This

screen allows you to set 5-Tone or 2-Tone code receive conditions.

1) Select the desired channel number line (G is for group code) and desired

item column using the [Arrow] keys.

- The selection is highlighted.

2) Push [Enter] to select a submenu, then select the desired data with the

[Arrow] and [Enter] keys; or

enter the desired digits or characters if applicable, then push [Enter].

- See the descriptions of each column item for function and setting

details.

Code

This item is used to set the code for one of the 8 individual receive code

channels (codes cannot be input for the group code channel).

Selectable value:      [0] to [9], [A] to [E], [G] and [*]

Selection method:      Type in the appropriate digits and/or characters

for the desired code (up to 7 digits).

- 15 -

Status Digit

This item toggles the status digits ON and OFF. When a matched 5-Tone code has

been detected and decoded, the status digits are displayed on the function

display. These digits are only for display (for user-defined signals, etc.)

and are not needed for a matched tone.

Selectable value:       ON or OFF

Selection method:       Enter a digit (1-7) to toggle the corresponding

status digit ON and OFF.

Indicated digits are displayed, non-indicated

digits are not displayed.

**                                           **

** NOTE: IC-F30/F40 have no LCD, therefore   **

**       text channels cannot be displayed   **

**       even though this software allows    **

**       them to be programmed.              **

**                                           **

Decode Action TX

This item is used to trigger the transceiver into the transmit condition when

a matched 5-Tone or 2-Tone code is received. This is useful in emergency

situations, allowing a system operator, etc. to monitor the surrounding area

of the transceiver without the transceiver's PTT being pushed. The transceiver

remains in the transmit condition for a period determined by the Auto TX

timer (see p. 34).

Selectable value:       ON or OFF

Selection method:       Push [Space] or [Backspace] to toggle the

selection.

Decode Action LED

This item sets an LED condition (LED flashes orange) to alert the user when a

matched 5-Tone or 2-Tone code is received.

Selectable value:      ON or OFF

Selection method:      Push [Space] or [Backspace] to toggle the

selection ON and OFF.

Decode Action Scan

This item is used to trigger a scan condition when the transceiver receives

a matched 5-Tone code or; _____ means scan conditions are unaffected by 5-Tone

codes.

Selectable value:      Cancel, Start or _____ (no action)

Selection method:      Push [Space] or [Backspace] to toggle the

selection ON and OFF.

- 16 -

Decode Action Beep

This item sets a beep condition to alert the user when the transceiver receives

a matched 5-Tone or 2-Tone code. Note that repeating tones continue for 30 sec.

Selectable value:      OFF, Pi, PiPi, PiRo, Pi (RPT),

PiPi (RPT), PiRo (RPT)

**                                                     **

** NOTE: Pi        = 1 high beep tone                  **

**       PiPi      = 2 high beep tones                 **

**       PiRo      = 1 high and 1 low beep tone        **

**       Pi (RPT)  = 1 high, repeating beep tone       **

**       PiPi (RPT)= 2 high, repeating beep tones      **

**       PiRo (RPT)= 1 high, 1 low repeating beep tone **

**                                                     **

Selection method:      Push [Enter] to select the submenu, then

make a selection; or

Push [Space] or [Backspace] to toggle the

selection.

Decode Action ID Dec.

This item sets the selected channel to display the sending station's ID when

a matched 5-Tone code is received.

Selectable value:      ON or OFF

Selection method:      Push [Space] or [Backspace] to toggle the

selection ON and OFF.

Decode Action Audible Mode

This item is used to set a channel to audible or inaudible mode after receiving

a matched 5-Tone or 2-Tone code or do nothing (_____).

Selectable value:       Audible, Inaudible or _____

Selection method:       Push [Enter] to select the submenu, use the

arrow keys to highlight the desired selection,

then push [Enter].

Decode Action Text

This item sets a pre-programmed text message for display when a matched 5-Tone

or 2-Tone code is received.

Selectable value:       01 to 99 or OFF

Selection method:       Type the desired text channel number, then

push [Enter]; or, push [Space] to select OFF.

**                                           **

** NOTE: IC-F30/F40 have no LCD, therefore   **

**       text channels cannot be displayed   **

**       even though this software allows    **

**       them to be programmed.              **

**                                           **

- 17 -

Decode Action ABC(answer back code)

This item sends a station code or a 1 kHz single signal for 2 sec. as an answer

back after receiving a matched 5-Tone or 2-Tone code.

Selectable value:       OFF, SGL (single), STN (displayed station code)

or TX Code CH (1 to 16, ...32)

Selection method:       Push [Enter] to select the submenu, then

select the desired condition; or

Push [Space] or [Backspace] to toggle the

selection.

Decode Action Stun

This item sets the transceiver to be disabled after receiving a specific

5-Tone or 2-Tone code. This function is convenient in case the transceiver is

stolen, etc. There are 2 levels of disablement: stun - requires the user to

re-enter the appropriate password (one chance only); and kill - the transceiver

cannot be activated without reprogramming.

Selectable value:     ___(None), Stun or Kill

Selection method:     Push [Enter] to select the submenu, then

select the desired condition; or

Push [Space] or [Backspace] to toggle the

selection.

- 18 -

////// DTMF CH SCREEN //////

+[DTMF CH]----------------------+

| CH  |Code                     |

|  No.|                         |

+-----+-------------------------+

|  1  |OFF                      |

|  2  |OFF                      |

|  3  |OFF                      |

|  4  |OFF                      |

|  5  |OFF                      |

|  6  |OFF                      |

|  7  |OFF                      |

|  8  |OFF                      |

+-----+-------------------------+

The DTMF CH screen allows you to program the transceiver's 8 DTMF channels

initially. Note that the number of DTMF channels available depends on how many

5-Tone TX code channels have been programmed. 0 to 8 DTMF channels may be

available for programming in the COMMON screen (p. 29).

1) Select the code line for the DTMF channel you wish to program.

- The selection is highlighted.

2) Enter the desired digits and/or characters, then push [Enter]; or

push [Space] or [Backspace] to turn the DTMF function OFF.

Selectable value:       [0] to [9] and [A] to [F] or OFF

**                                         **

** NOTE: Up to 24 digits can be programmed **

**       into each DTMF channel.           **

**       [E] and [F] refer to [*] and [#]  **

**       on the keypad, respectively.      **

**                                         **

Selection method: Same as steps 1) and 2) above.

- 19 -

///// SCAN LIST SCREEN /////

The following screen appears for the IC-F30LT/F40LT.  The corresponding

screen for the IC-F30/F40 has no "BANK" numbers and therefore only 1 column

for programming of channels.

+[Scan List]---------+----+----+----------------++----------------+

|List|Scan           |Text|PWR |[Bank 1] 1111111||[Bank 6] 1111111|

| No.|Mode           |  CH|Save|1234567890123456||1234567890123456|

+----+---------------+----+----+----------------++----------------+

|  1 |Mode 1(Normal) | 91 | ON |1234567890ABCDEF||________________|

|  2 |Mode 1(Normal) | 92 | ON |________________||________________|

|  3 |Mode 1(Normal) | 93 | ON |________________||________________|

|  4 |Mode 1(Normal) | 94 | ON |________________||________________|

|  5 |Mode 1(Normal) | 95 | ON |________________||________________|

|  6 |Mode 1(Normal) | 96 | ON |________________||1234567890ABCDEF|

|  7 |Mode 1(Normal) | 89 | ON |________________||________________|

|  8 |Mode 1(Normal) | 89 | ON |________________||________________|

|  9 |Mode 1(Normal) | 89 | ON |________________||________________|

| 10 |Mode 1(Normal) | 90 | ON |1234567890ABCDEF||1234567890ABCDEF|

+----+---------------+----+----+----------------++----------------+

The SCAN LIST screen is accessed from the Screen item in the TOP menu. This

screen allows you to set various conditions for each of the 10 scan lists.

1) Select the desired scan list line and desired item column using the

[Arrow] keys.

- The selection is highlighted.

2) Enter the desired digits or characters if applicable, then push [Enter];

or push [Enter] to select a submenu; or toggle the selection using [Space]

or [Backspace].

- See the descriptions of each column item for function and setting

details.

**                                                                    **

**  A scan list number can be selected by pushing a digit key during  **

**  scanning. However, the scan list is selected according to the     **

**  initially selected bank number (during bank operation).           **

**  For the IC-F30/F40 without an optional UT-82 Keypad, only list    **

**  number 1 is used.                                                 **

**                                                                    **

Scan Mode

This item sets a particular scan list for one of several scan modes or turns

scanning OFF for the selected list.

Mode 0(OFF)-    scan is turned OFF;

Mode 1(Normal)- scan proceeds in sequence from lower to higher channel numbers;

Mode 2(Prio-A)- scan proceeds in sequence from lower to higher channels while

checking the priority channel;

Cancelling the scan, for example pushing a [SCAN] key or [PTT],

selects the channel last paused on during the scan;

Mode 3(Prio-A)- same as Mode 2(Prio-A) except, when cancelling the scan the

priority channel is selected.

Mode 2(R.Sel)-  Scan proceeds in sequence from lower to higher channels while

checking the rotary selector's channel. Cancelling the scan,

such as by pushing a [SCAN] key or [PTT], selects the channel

last paused on during the scan.

Mode 3(R.Sel)-  same as Mode 2(R.Sel) except, when cancelling the scan, the

rotary selector's channel is selected.

- 20 -

Selectable value:       Mode 0(OFF), Mode 1(Normal), Mode 2(Prio-A),

Mode 3(Prio-A), Mode 2(R.Sel), Mode 3(R.Sel)

Selection method:       Push [Enter] to select the submenu, then

select the desired condition; or

Push [Space] or [Backspace] to toggle the

selection.

**                                                             **

**  NOTE: When "MR+SCAN is assigned to the rotary selector in  **

**        the COMMON screen (p. 26), Mode 2 (R.Sel) and Mode 3 **

**        (R.Sel) cannot be used, otherwise there would be no  **

**        no way to stop the scan.                             **

**                                                             **

Text CH

This item sets a pre-programmed text channel for display indication when

selecting a particular scan list. For the IC-F30LT/F40LT, programmable text

channels are available. See p. 22 for TEXT screen details.

Selectable value:       01 to 99 or OFF

Selection method:       Enter the desired selection directly, then

push [Enter]; or, push [Space] to select OFF.

**                                           **

** NOTE: IC-F30/F40 have no LCD, therefore   **

**       text channels cannot be displayed   **

**       even though this software allows    **

**       them to be programmed.              **

**                                           **

PWR Save

This item sets the power saver to OFF or ON when a particular scan list is

selected.

Selectable value:       ON or OFF

Selection method:       Push [Space] or [Backspace] to toggle the

selection.

1111111  (Channel setting)

1234567890123456

This item sets the channels to be scanned in a particular list.

Selectable value:       [0] to [9] and [A] to [F]

[A] to [F] correspond to channels 11 to 16,

respectively.

Indicated channels are scanned, non-indicated

channels are not scanned.

Selection method:       Push the corresponding digit key to toggle a

particular channel ON and OFF.

- 21 -

///// TEXT CHANNEL SCREEN /////

The IC-F30/F40 have channels 33 to 99 only, although they cannot be displayed.

+[TEXT CH]-----+----+---------+----+---------+----+---------+

|CH  |Text     |CH  |Text     |CH  |Text     |CH  |Text     |

| No.|         | No.|         | No.|         | No.|         |

+----+---------+----+---------+----+---------+----+---------+

|  1 |         |  2 |         |  3 |         |  4 |         |

|  5 |         |  6 |         |  7 |         |  8 |         |

|  9 |         | 10 |         | 11 |         | 12 |         |

| 13 |         | 14 |         | 15 |         | 16 |         |

| 17 |         | 18 |         | 19 |         | 20 |         |

| 21 |         | 22 |         | 23 |         | 24 |         |

| 25 |         | 26 |         | 27 |         | 28 |         |

| 29 |         | 30 |         | 31 |         | 32 |         |

| 33 |AIRPORT  | 34 |  BASE   | 35 |  BOSS   | 36 |  BUS    |

| 37 |  CALL   | 38 | CHIEF   | 39 |DELIVERY | 40 |DIRECTOR |

| 41 |DISPATCH | 42 | EDITOR  | 43 |FACTORY  | 44 | FIELD   |

| 45 |  FIRE   | 46 | GARAGE  | 47 |  GATE   | 48 |GENERAL  |

| 49 | GUARD   | 50 |  HELP   | 51 | HOUSE   | 52 | LEADER  |

| 53 |MANAGER  | 54 |MESSAGE  | 55 | MOBILE  | 56 | OFFICE  |

| 57 |OPERATOR | 58 | PLANT   | 59 | POLICE  | 60 |  PORT   |

| 61 |PRIVATE  | 62 |PRODUCER | 63 |  RAIL   | 64 | REMOTE  |

| 65 | SALES   | 66 |SECURITY | 67 |SERVICE  | 68 |SPECIAL  |

| 69 |STATION  | 70 |  TAXI   | 71 | TRANS   | 72 |WELCOME  |

| 73 |  YARD   | 74 |         | 75 |         | 76 |         |

| 77 |         | 78 |         | 79 |         | 80 |         |

| 81 |         | 82 |         | 83 |         | 84 |         |

| 85 |         | 86 | PRIO A  | 87 | PRIO B  | 88 |PRIOSCAN |

| 89 |SCANNING | 90 | SCAN10  | 91 | SCAN1   | 92 | SCAN2   |

| 93 | SCAN3   | 94 | SCAN4   | 95 | SCAN5   | 96 | SCAN6   |

| 97 | SCAN7   | 98 | SCAN8   | 99 | SCAN9   |    |-------- |

+----+---------+----+---------+----+---------+----+---------+

The TEXT CH screen is accessed from the Screen item in the TOP menu. This

screen allows you to set the text for text channels (except for the

factory programmed text channels, 33 to 99).  The IC-F30/F40 have no

programmable channels, 1 to 32.

1) Select the desired text channel using the [Arrow] keys.

- The selection is highlighted.

2) Enter the desired characters, then push [Enter].

- A maximum of 8 characters can be input per channel (including spaces).

- 22 -

///// USER TONE SCREEN /////

+[User Tone]----+----------------+

|Tone|Encode(Hz)|Decode Frequency|

| No.|Frequency |Low(Hz)|High(Hz)|

+----+----------+-------+--------+

|  0 | 1980     | 1941  | 2020   |

|  1 | 1124     | 1102  | 1148   |

|  2 | 1197     | 1174  | 1220   |

|  3 | 1274     | 1249  | 1301   |

|  4 | 1358     | 1331  | 1386   |

|  5 | 1446     | 1417  | 1474   |

|  6 | 1539     | 1509  | 1571   |

|  7 | 1639     | 1608  | 1673   |

|  8 | 1747     | 1712  | 1781   |

|  9 | 1860     | 1822  | 1896   |

|  A | 2400     | 2352  | 2449   |

|  B | 930      |  911  | 948    |

|  C | 2245     | 2200  | 2291   |

|  D |  991     |  971  | 1010   |

|  E | 2109     | 2068  | 2154   |

+----+----------+-------+--------+

The User Tone screen is accessed from the Screen item in the TOP menu. This

screen allows you to set user tones.

1) Select the desired tone number and frequency column using the [Arrow] keys.

- The selection is highlighted.

2) Enter the desired frequency, then push [Enter].

Selectable value:    400 to 3000

(Nearest available frequency is selected

automatically.)

**                                                          **

** NOTE: When Low and High frequencies are mixed up, "*"    **

**       appears on the right side of the "High" frequency. **

**       Re-set the frequencies in this case.               **

**                                                          **

- 23 -

///// COMMON SCREEN /////

+[Common]--------------+-----------------++----------------------+--------+

| Title                |                 ||Title                 |        |

+----------------------+-----------------++----------------------+--------+

|Key Assign(Red)       |Emergency        ||Time Out Timer        | 30.000 |

|Key Assign(.)         |Monitor          ||TOT Penalty Timer     | 20.000 |

|Key Assign(..)        |Call             ||Lockout Penalty Timer |  5.000 |

|Key Assign(P0)*       |Re-Dial          ||VOX Delay Timer       |  1.000 |

|Key Assign(P1)*       |Priority A       ||Man Down Timer        |  5.000 |

|Key Assign(P2)*       |Talk Around      ||Stop Scan Timer       |  5.000 |

|Key Assign(Ent)*      |TX Code          ||Resume Scan Timer     |  3.000 |

|RF Power Hold         |OFF              ||Auto Reset Timer A    | 30.000 |

|Rotary Selector       |Code CH          ||Auto Reset Timer B    |OFF     |

|One touch DTMF        |OFF              ||Inactive Timer        | 60.000 |

|PTT Call(Inaudible)   |OFF              ||Auto TX Timer         | 30.000 |

|Opening Text**        |              69 ||DTMF Timer            |  0.100 |

|Priority CH A         |              16 ||2-Tone Group Timer    |  3.000 |

|Priority CH B         |               1 ||ID Decode Timer       |  1.600 |

|Emergency CH          |               1 ||ABC Decode Timer      |  1.600 |

|LCD Display**         |MR CH            ||Link R Timer          |  0.800 |

|TOT ID Out            |OFF              ||Link 1 Timer          |  0.800 |

|TOT Beep Out          |OFF              ||Link 2 Timer          |  0.800 |

|Power ON Scan         |OFF              ||Link A Timer          |  0.800 |

|Man Down              |ON               ||Long Tone Timer       |  0.700 |

|Clone Comment         |ICOM IC-F30 PMR  ||Lead Out Delay Timer  |  0.100 |

|User Password*        |            1234 ||User Tone Period      |  0.100 |

|Password*             |OFF              ||User Notone Timer     |  0.160 |

|Dealer Passcode*      |          159357 ||                      |        |

|DTMF : TX Code CH     |8 : 16           ||                      |        |

|Low Battery Beep      |ON               ||                      |        |

|Operation Beep        |ON               ||                      |        |

|Beep Couple           |Linked           ||                      |        |

|Light*                |Auto             ||TX DTCS Inverse       |Normal  |

|VOX                   |ON               ||RX DTCS Inverse       |Normal  |

|Set Mode Access*      |Enable           ||AF Minimum Level      |      2 |

|Cloning*              |Enable           ||Beep Level            |    128 |

|TX Power Adjust       |Enable           ||Squelch Level         |    118 |

|Display Digit**       |12345__          ||VOX Gain              |    184 |

|Compare Digit         |12345__          ||BUSY Action           |Low     |

|Special Tone(Group)   |A                ||OPT1 Action           |Input   |

|Special Tone(Repeat)  |E                ||OPT2 Action           |Input   |

|Special Tone(Link2)   |F                ||OPT3 Action           |Input   |

+----------------------+-----------------++----------------------+--------+

*These items are not effective for the IC-F30/F40 without an optional

UT-82 Keypad installed.

**These items are not effective for the IC-F30/F40 because they have no LCD.

The COMMON screen is accessed from the Screen item in the TOP menu. This

screen allows you to set conditions common to all channels.

1) Select the desired item using the [Arrow] keys.

- The selected position is highlighted.

2) Enter the desired letters and/or digits where applicable, then push

[Enter]; or, push [Enter] to select a submenu, then select a condition;

or, push [Space] or [Backspace] to toggle a selection.

- 24 -

Key Assign(Red/./../P0/P1/P2/Ent)

These items assign a function to the dealer-programmable switches/keys.

Selectable value:       Null(Light)     -No function. LCD lights when

pushed.

Monitor         -Monitor switch. Toggles

between audible and inaudible.

Talk Around     -Selects simplex temporarily.

Priority A      -Selects priority channel A.

Priority B      -Selects priority channel B.

Bank            -Increments bank number.

Does not appear when

"IC-F30/F40" is started up.

High            -Selects transmit high power.

Low2            -Selects transmit low power2.

Low1            -Selects transmit low power1.

Lock*           -Electronically locks keypad.

Lock Override*  -Activates keypad only when

this key is pushed and held.

Keypad operation is locked

normally when this function is

assigned.

Scan            -Scan start/stop.

DTMF*           -Transmits a DTMF channel.

Re-Dial*        -Transmits most recently

transmitted DTMF code again.

Call            -Transmits a 5-Tone code.

TX Code*        -TX code input.

TX Code CH*     -Selects a TX code channel.

Emergency       -Sends an emergency ID code

signal.

Memory CH*      -Accepts memory channel number

entry via the digit keys.

OPT1 Out, OPT2 Out, OPT3 Out, OPT1 Out(Moment),

OPT2 Out(Moment), OPT3 Out(Moment)

*-marked items require an optional UT-82 Keypad

for the IC-F30/F40.

Selection method:       Push [Enter] to select the submenu, then

select the desired condition; or push [Space]

or [Backspace] to toggle a selection.

RF Power Hold

This item affects the output power settings of individual channels after

power is turned OFF. When this item is set to ON, individual channels retain

any user-assigned power outputs. When this item is set to OFF, all output power

values return to their pre-programmed settings.

Selectable value:      ON or OFF

Selection method:      Push [Space] or [Backspace] to toggle the

selection.

- 25 -

Rotary Selector

This item is used to vary the action of the rotary selector. The rotary

selector can be programmed to select memory channels, memory channels and

the scan function or, 5-Tone TX code channels. When MR+SCAN is selected, the

home position (ch 1) starts scan.

Selectable value:       MR, MR+SCAN or Code CH

Selection method:       Push [Enter] to select the submenu, then

select the desired condition; or

Push [Space] or [Backspace] to toggle the

selection.

One Touch DTMF

This item is used to toggle the one touch DTMF function ON and OFF. This

setting is effective for the IC-F30LT/F40LT.  This function is also available

for the IC-F30/F40 when an optional UT-82 Keypad is installed.  This setting

also selects the keypad function as below.

+----------------------------+--------------------------------------+

|  COMMON SCREEN SETTING     |                                      |

+----------------+-----------+            KEYPAD                    |

| ROTARY         | One Touch |            FUNCTION                  |

| SELECTOR       | DTMF      |                                      |

+----------------+-----------+--------------------------------------+

| [MR :BANK] or  |   OFF     | [1] to [6]: One touch bank selection |

| [MR+SCAN :BANK]|           | [7] to [0]: No function.             |

|                +-----------+--------------------------------------+

|                |   ON      | [1] to [0]: DTMF transmission        |

+----------------+-----------+--------------------------------------+

| [DTMF :FREE]   |   OFF     | [1] to [0]: Direct channel number    |

|                |           |             input for operating      |

|                |           |             channels, ch1 to ch10    |

|                +-----------+--------------------------------------+

|                |   ON      | [1] to [0]: DTMF transmission        |

+----------------+-----------+--------------------------------------+

Selectable value:       ON or OFF

Selection method:       Push [Space] or [Backspace] to toggle the

selection.

- 26 -

Opening Text

This item is used to program the message that appears in the function display

when first applying power to the transceiver. See p. 22 for text channel

programming.

Selectable value:      01 to 99 or OFF

Selection method:      Enter the desired text channel number directly;

or

Push [Space] or [Backspace] to turn the opening

text OFF.

**                                           **

** NOTE: IC-F30/F40 have no LCD, therefore   **

**       text channels cannot be displayed   **

**       even though this software allows    **

**       them to be programmed.              **

**                                           **

Priority CH A/Priority CH B/Emergency CH

These items are used to assign a channel to Priority A, Priority B and

the Emergency channels.

Selectable value:      IC-F30LT/F40LT  1-1 to 6-16 (Bank-type)

1 to 99 (Channel free-type)

IC-F30/F40      1 to 16

Selection method:      Enter the desired channel directly, then push

[Enter].

TOT ID OUT

This item is used to send an ID code when the TOT period has elapsed. If the

channel's ID code is set to OFF, this item has no function. Similarly, if the

TOT setting in the MEMORY CH screen is set to OFF, this item has no function.

Selectable value:       ON or OFF

Selection method:       Push [Space] or [Backspace] to toggle the

selection.

TOT Beep Out

This item is used to set beeps to alert the user 10 secs. before the TOT is to

be engaged (1 long beep) and when the TOT has elapsed (5 short beeps). If the

TOT ID OUT item is set to ON, it takes priority and no beeps will be emitted

at the end of the TOT period.

Selectable value:       ON or OFF

Selection method:       Push [Space] or [Backspace] to toggle the

selection.

- 27 -

Power ON Scan

This item is used to toggle the Power ON Scan function ON and OFF. When turned

ON, scan automatically starts when the transceiver power is turned ON.

Selectable value:      ON or OFF

Selection method:      Push [Space] or [Backspace] to toggle the

selection.

Man Down

This item toggles the emergency man down function ON and OFF. However, an

internal mercury switch must be installed before this function will work

properly. Otherwise, an emergency call is transmitted even when the transceiver

is not left in a horizontal position.

Selectable value:      ON or OFF

Selection method:      Push [Space] or [Backspace] to toggle the

selection.

Clone Comment

This item allows you to program a comment for reference purposes. When you

connect a transceiver to a PC using the OPC-427 Cloning Cable, you can check

the transceiver's comment by selecting "information" in the Clone menu. See

p. 40 for more information about the Clone menu. Also, this comment is used

as a header when printing out data.

Selectable value:       Any character (16 max. including spaces)

Selection method:       Type in the desired comment, then push

[Enter].

User Password

This item is used to program 4 digits as a user input code at power ON.  This

item is only effective when the following item, "Password," is set to ON.  For

the IC-F30/F40 with an optional UT-82 Keypad installed, it may be possible to

use this function.  However, no LCD is available to check whether input digits

are correct or not.

Selectable value:       0000 to 99999

Selection method:       Enter the desired password, then push

[Enter].

Password

This item is used to toggle the user password ON and OFF for the IC-F30LT/F40LT

only. When the password is turned OFF, a user password is not needed to access

the transceiver functions even when a password is programmed in the previous

item above.

Selectable value:       ON or OFF

Selection method:       Push [Space] or [Backspace] to toggle the

selection.

- 28 -

Dealer Passcode

This item is used to program the code needed to access the transceiver's

SET MODE (for the IC-F30LT/F40LT only). Many of the transceiver's functions

can be adjusted in SET MODE.  Refer to the Programming Manual for details.

Make sure that "Set Mode Access"  is set to "Enable" when you want to be able

to access SET MODE. For the IC-F30/F40 with an optional UT-82 Keypad installed,

it may be possible to use this function.  However, no LCD is available to

check digit input.

Selectable value:       000000 to 999999

Selection method:       Enter the desired passcode, then push

[Enter].

DTMF : TX Code CH

This item sets the ratio of usable DTMF channels to usable TX Code channels.

Selectable value:       0:32, 1:30, 2:28, 3:26, ......., 8:16

Selection method:       Push [Space] or [Backspace] to toggle the

the selection; or push [Enter] to select the

submenu, then select the desired value.

Low Battery Beep

This item is used to used to toggle the low battery beep function ON and OFF.

When this function is turned ON, the transceiver emits beep tones to alert

the user when the attached battery pack needs recharging.

Selectable value:      ON or OFF

Selection method:      Push [Space] or [Backspace] to toggle the

selection.

Operation Beep

This item is used to toggle the transceiver's confirmation beeps ON and OFF.

Selectable value:       ON or OFF

Selection method:       Push [Space] or [Backspace] to toggle the

selection.

- 29 -

Beep Couple

This item is used to toggle the beep couple function between Independent mode

and  Link mode. In Independent mode, the beep volume is constant and

independent of the transceiver volume control; in Link mode, the beep volume

is directly proportional to the transceiver volume control.

Selectable value:       Indep. or Linked

Selection method:       Push [Space] or [Backspace] to toggle the

selection.

Light

This item is used to adjust the action of the display and keypad backlighting.

OFF deactivates the lighting; Auto turns the lighting on for 5 secs. every

time a key or switch is activated; and, Continuous is used to turn the lighting

on continuously while the transceiver power is ON.

Selectable value:      OFF, Auto and Continuous

Selection method:      Push [Enter] to select the submenu, then

select the desired condition; or

Push [Space] or [Backspace] to toggle the

selection.

VOX

This item toggles the VOX (voice activated transmit) function ON and OFF.

Note that the VOX function can only be used when an optional headset is

connected to the transceiver.

Selectable value:      ON or OFF

Selection method:      Push [Space] or [Backspace] to toggle the

selection.

Set Mode Access

This item toggles Set Mode access ON and OFF. When enabled a user is able to

program non-Set Mode items via the transceiver.  See the Programming Manual for

details. Although this item can be set for the IC-F30/F40, programming in SET

mode may not be possible because there is no LCD.

Selectable value:      Enable or Inhibit

Selection method:      Push [Space] or [Backspace] to toggle the

selection.

- 30 -

Cloning

This item toggles cloning (transceiver to transceiver) ON and OFF. When

enabled, a transceiver's programmed contents can be cloned to another

without the need for this cloning software.  See the Programming Manual for

cloning operation.  For the IC-F30/F40, transceiver-to-transceiver cloning may

be impossible.

Selectable value:       Enable or Inhibit

Selection method:       Push [Space] or [Backspace] to toggle the

selection.

TX Power Adjust

This item toggles TX power adjustment ON and OFF. When enabled, the

transceiver's pre-programmed transmit output power settings can be adjusted

(refer to the Programming Manual).

Selectable value:       Enable or Inhibit

Selection method:       Push [Space] or [Backspace] to toggle the

selection.

Display Digit

This item toggles the display digits ON and OFF. The display digits determine

which digits of a 5-Tone code appear on the function display. Note that during

transmit, all digits of a code are transmitted, regardless of whether they are

displayed or not.

Selectable value:       [1] to [7]

Selection method:       Push a digit key (1-7) to toggle the

corresponding  display digit ON and OFF.

Only indicated digits appear on the LCD.

**                                           **

** NOTE: IC-F30/F40 have no LCD, therefore   **

**       text channels cannot be displayed   **

**       even though this software allows    **

**       them to be programmed.              **

**                                           **

Compare Digit

This item toggles the compare digits ON and OFF. When receiving a tone, if

the compare digits match the received tone, the programmed decode action

(pgs.15-17) is carried out. Note that if the receive tone has more digits than

the activated compare digits, a match will still occur as long as the compare

digits are matched.

Selectable value:       [1] to [7]

Selection method:       Push a digit key (1-7) to toggle the

corresponding  compare digit ON and OFF.

Only indicated digits appear on the LCD.

- 31 -

Special Tone(Group)

This item sets a group tone. Group tones are used to contact all individual

transceivers in the same group. For example, 1234* would be used to contact

transceivers with the station codes 12340 to 1234E. The group tone is usually

set to "A."

Selectable value:       [0] to [9], [A] to [E]

Selection method:       Enter the desired digit or character, then

push [Enter].

Special Tone(Repeat)

This item sets a repeating tone. When a 5-Tone code contains 2 or more

identical tones in succession, a repeat tone (usually E) is used. For example,

11111 would be sent as 1E1E1.

Selectable value:       [0] to [9], [A] to [E]

Selection method:       Enter the desired digit or character, then

push [Enter].

Special Tone(Link2)

This item sets a tone between the station code and the ID code in a 5-Tone

code. This tone is normally set to F (no tone).

Selectable value:       [0] to [9], [A] to [F]

Selection method:       Enter the desired digit or character, then

push [Enter].

Time Out Timer

This item allows you to adjust the time-out timer setting. This setting

is the period of continuous transmission required before transmit is

automatically terminated. The time-out timer operates independently for each

channel. Refer to "MEMORY CHANNEL screen," p. 7.

Selectable value:      0 to 255 seconds

Selection method:      Enter the desired time in seconds, then push

[Enter].

**                                         **

** A time-out time of 0 means transmitting **

** is not possible.                        **

**                                         **

- 32 -

TOT Penalty Timer

This item allows you to adjust the time of the TOT penalty timer. This is the

period from when the time-out timer is activated (transmit is automatically

terminated) during which transmitting is further inhibited.

Selectable value:      OFF to 255 seconds

Selection method:      Enter the desired time in seconds, then push

[Enter]. If 0 is input, OFF appears; or

Push [Space] or [Backspace] to select OFF.

Lockout Penalty Timer

This item sets a penalty time (transmit is inhibited) when the user attempts

to transmit on a busy channel. Transmit is inhibited for a period determined

by this timer.

Selectable value:      OFF to 255 seconds

Selection method:      Enter the desired time in seconds, then push

[Enter]. If 0 is input, OFF appears; or

Push [Space] or [Backspace] to select OFF.

VOX Delay Timer

This item adjusts the time period for switching from transmit to receive

after a transmission is completed.

Selectable value:      0 to 25.5 seconds

Selection method:      Enter the desired time seconds, then push

[Enter].

Man Down Timer

This item adjusts the time required for the Man Down emergency function to

activate from when the transceiver is placed in a horizontal position. Refer to

p. 28 for Man Down setting and operation note.

Selectable value:      0 to 25.5 seconds

Selection method:      Enter the desired time in seconds, then push

[Enter].

Stop Scan Timer

This item adjusts the interval at which the priority channel is checked while

scan is paused on a signal. When 0 is selected, scan does not check for

signals on the priority channel while it is paused on a signal.

Selectable value:      0 to 25.5 seconds

Selection method:      Enter the desired time in seconds, then push

[Enter].

- 33 -

Resume Scan Timer

This item adjusts the time a scan pauses after a signal disappears on the

stopped channel.

Selectable value:      0 to 25.5 seconds

Selection method:      Enter the desired time in seconds, then push

[Enter].

Auto Reset Timer A

This item sets the time for the "Auto Reset" in the MEMORY CHANNEL screen.

See p. 10.

Selectable value:      0 to 255 seconds

Selection method:      Enter the desired time in seconds, then push

[Enter].

Auto Reset Timer B

This item sets the time for the "Auto Reset" in the MEMORY CHANNEL screen.

See p. 10.

Selectable value:      0 to 255 seconds

Selection method:      Enter the desired time in seconds, then push

[Enter].

Inactive Timer

This item sets the time for the "Call Inactive" item in the MEMORY CHANNEL

screen. See p. 10.

Selectable value:      0 to 255 seconds

Selection method:      Enter the desired time in seconds, then push

[Enter].

Auto TX Timer

This item sets the time for "auto TX" when receiving a correct 5-Tone or 2-Tone

signal. Decode Action TX must be set to ON in the RX code screen. See p. 15.

Selectable value:      0 to 255 seconds

Selection method:      Enter the desired time in seconds, then push

[Enter].

- 34 -

DTMF Timer

This item determines the period "t" as shown below. Some repeaters, etc.

require slower transmission of DTMF codes than others do.

Selectable value:      0 to 2.55 seconds

Selection method:      Enter the time in seconds, then push [Enter].

------        ------        ------        ------

|Tone  |      | Tone |      |Tone  |      |Tone

|      |      |      |      |      |      |

-----        ------        ------        ------

Interval     Interval       Interval

<--t--><--t--><--t--><--t--><--t--><--t--><--t-->

2-Tone Group Timer

This item sets the 2-Tone detection delay time for starting a group tone

detection. When the received code is longer than this setting, the decoder

detects the code as a group code.

Selectable value:      0 to 25.5 seconds

Selection method:      Enter the desired time in seconds, then

push [Enter].

ID Decode Timer

This item varies depending on whether the ID code is set as ID TOP or

ID BOTTOM (see p. 11).

When ID BOTTOM is set, the ID decode time is the period from when the last

tone of the station code is received.

When ID TOP is set, the ID decode time is the period from when the last

tone of the ID code is received. In this case, the decode action (see p. 16)

has no effect. Also, note that if no ID code is sent in this case, the call

cannot be decoded.

Selectable value:      0 to 25.5 seconds

Selection method:      Type the desired time, then push

[Enter].

ABC Decode Timer

This item sets the time from when the last tone of a call is received to when

an answer back code is sent. This setting has no meaning if "ABC Decode" in

the TX Code CH screen is set to OFF (p. 14).

Selectable value:      0 to 25.5 seconds

Selection method:      Type the desired time, then push

[Enter].

- 35 -

Link R/Link 1/Link2/Link A Timers

These items set the periods between each Repeater, ID and Station code. Link R,

Link 1 and Link 2 are used when [CALL] is pushed; Link A is used when a 5-tone

call is received.

Selectable value:      0 to 2.55 seconds

Selection method:      Type the desired time in seconds, then push

[Enter].

CALL is        RPT code        ID code          STN code

pushed                         (STN)            (ID)

_______         _______          _______

|       |       |       |        |       |

|       |       |       |        |       |

_______|       |_______|       |________|       |

Link R          Link 1           Link 2

Answer back code

|        -------

|       |       |

Transmitting |       |       |

|-------         ------------------

|Link A

Long Tone Timer

This item sets the time period for the 1st tone of a 5-Tone code. To contact

a transceiver that is scanning, the first tone of a 5-Tone code can be

transmitted for a longer time than the subsequent tones. Long tones are used

for repeater codes when a long tone is required to open a repeater.

Selectable value:      0 to 2.55 seconds

Selection method:      Enter the desired time in milliseconds, then

push [Enter].

Lead Out Delay Timer

This item sets the time from when a code (such as DTMF, single tone, 5-Tone,

etc.) is sent to when the transceiver returns to the receive condition.

Selectable value:      0 to 2.55 seconds

Selection method:      Enter the desired time in seconds, then

push [Enter].

User Tone Period

This item sets the tone period for user tones. Tone periods for tones other

than user tones are set automatically.

Selectable value:      0.255 seconds

Selelction method:     Enter the desired time in seconds, then push

[Enter].

- 36 -

User Notone Timer

This item sets the period necessary for receive of a notone before the

notone action is carried out.

Selectable value:      0.020 to 0.300 seconds (in 0.02 sec. steps)

Selection method:      Enter the desired time in seconds, then push

[Enter].

**                                                      **

** NOTE: Generally, the notone timer is set to 1.5x     **

**       the tone period. The tone period depends on    **

**       the selected tone set (p. 10).  However, if    **

**       an optional 5-Tone encoder is not installed,   **

**       20 ms is the only selectable value for the     **

**       notone timer.  If the notone timer is set too  **

**       short, weak signal noise will interfere with   **

**       proper detection of tones; if the notone timer **

**       is set for too long of a period, the link time **

**       must also be lengthened.                       **

**                                                      **

TX DTCS Inverse

This item sets the polarity of the tranceiver's transmit DTCS codes. In order

for transceivers to communicate using DTCS codes, the polarity of the

receiving transceiver's receive code must be the same as the polarity of the

transmitting transceiver's transmit code.

Selectable value:      Normal or Inverse

Selection method:      Push [Space] or [Backspace] to toggle the

selection.

RX DTCS Inverse

This item sets the polarity of the tranceivers receive DTCS codes. In order

for transceivers to communicate using DTCS codes, the polarity of the

receiving transceiver's receive code must be the same as the polarity of the

transmitting transceiver's transmit code.

Selectable value:      Normal or Inverse

Selection method:      Push [Space] or [Backspace] to toggle the

selection.

AF Minimum Level

This item adjusts the minimum level of AF output when rotating the

transceiver's volume control to its minimum setting. Higher values correspond

to higher minimum adjustable volume levels.

Selectable value:      0 to 255

Selection method:      Enter the desired value, then push [Enter].

- 37 -

Beep Level

This item adjusts the transceiver's confirmation beep volume. Higher values

correspond to higher beep volume levels.

Selectable value:      0 to 255

Selection method:      Enter the desired value, then push [Enter].

Squelch Level

This item adjusts the signal level needed to open the transceiver's noise

squelch circuit. Higher values correspond to stronger signals needed to open

the squelch.

Selectable value:      0 to 255

Selection method:      Enter the desired value, then push [Enter].

VOX Gain

This item adjusts the gain of the transceiver's VOX circuit. Higher values

correspond to lower audio levels needed to activate the VOX circuit.

Selectable value:      0 to 255

Selection method:      Enter the desired value, then push [Enter].

Busy Action

The transceiver's CPU has a "BUSY" port to detect a receive mute condition.

When this item's set value is applied to the BUSY port, the CPU opens the

squelch (receive mute).

Selectable value:      Low or High

Selection method:      Push [Space] or [Backspace] to toggle the

selection.

OPT1/OPT2/OPT3 Action

The transceiver's CPU has OPT1, OPT2 and OPT3 ports.

Selectable value:       Input, Out(L) or Out(H)

Selection method:       Push [Enter] to select the submenu, then

select the desired condition; or

Push [Space] or [Backspace] to toggle the

selection.

- 38 -

-------------------------------------------------------------------------------

2. CLONING OPERATION

------------------------------------------------------------------------------

Programmed data can be cloned to an F30/F40 series transceiver instantly. All

data is clone sent (PC to transceiver) or clone read (transceiver to PC)

in approx. 15 sec. A cloning cable, OPC-427, is necessary to connect between

a PC's RS-232C port and the transceiver. For convenience, when changing a small

portion of the programmed contents, clone read the data from the transceiver,

modify the data, then clone send it back to the transceiver.

CLONE READ

The pre-set data in the transceiver is copied to the PC.

1) Connect the PC's RS-232C port and transceiver's [MULTI] connector

using the OPC-427.

2) Turn the PC and transceiver power ON.

3) Start up this program.

4) Push [Esc], then select "Clone" in the TOP menu with the [C] key, or the

[Arrow] and [Enter] keys.

- The Clone submenu appears.

5) Select "PC<-TR" in the submenu with the [T] key, or the [Arrow] and

[Enter] keys.

- "CLONE" appears in the transceiver display and "Receiving..." appears

at the bottom of the screen.

- When cloning is finished, "Receiving" disappears from the bottom of the

screen.

- Clone read is now complete.

**                                                                       **

** NOTE: For convenience, before cloning, select the Information item in **

**       the Clone submenu, with the transceiver connected to the PC     **

**       and the power turned ON. This item will tell you the version of **

**       the transceiver and also any comments that have been included   **

**       for reference.                                                  **

**                                                                       **

- 39 -

CLONE SEND

The programmed data in the PC is copied to the transceiver.

1) Connect the PC's RS-232C port and transceiver's [MULTI] connector

using the OPC-427.

2) Turn the PC and transceiver power ON.

3) Start up this program.

4) Push [Esc], then select "Clone" in the TOP menu with the [C] key, or the

[Arrow] and [Enter] keys.

- The Clone submenu appears.

5) Select "PC->TR" in the submenu with the [P] key, or [Arrow] and

[Enter] keys.

- "CLONE" appears in the transceiver display and Sending...appears at the

bottom of the screen.

- When cloning is finished, "Sending" disappears from the bottom of the

screen.

6) Turn the transceiver OFF and ON again.

- Clone send is now complete.

**                                                                        **

** NOTE: Before cloning a transceiver, make sure that the correct RS-232C **

**       port is selected in the Clone submenu (Port 1 or Port 2).        **

**       Select RS-232C in the submenu, then select the desired port      **

**       using [Space] or [Backspace]. If the wrong port is selected,     **

**       cloning will not be successful.                                  **

**                                                                        **

- 40 -

-------------------------------------------------------------------------------

3. FILE SAVING and LOADING

-------------------------------------------------------------------------------

The set data can be saved on the same floppy disk (or directly to a hard disk)

as this program for future maintenance or modification purposes.

FILE SAVING OPERATION

1) Push [Esc] to indicate the TOP menu.

2) Select "File" in the TOP menu with the [F] key, or the [Arrow] and

[Enter] keys.

- The File submenu appears.

3) Select "Save" in the submenu with the [S] key, or the [Arrow] and

[Enter] keys.

- A file name box appears and shows "FILE: NONAME.ICF".

4) Type the desired file name to be saved.

- ICF is recommended as a file extension for loading convenience.

- When no file extension is used, ".ICF" is automatically added to the

file name.

- The TOP menu and the file name disappear and the previous menu appears.

**                                                                        **

**  SAVING NOTE:                                                          **

**                                                                        **

**  When starting up the program and no file name has been loaded,        **

**  "NONAME.ICF" appears in the file name box.                            **

**                                                                        **

**  To indicate the previously saved file name, "*" (wild card) may be    **

**  helpful to check the file name.                                       **

**                                                                        **

FILE LOADING OPERATION

1) Push [Esc] to indicate the TOP menu.

2) Select "File" in the TOP menu with the [F] key, or the [Arrow] and

[Enter] keys.

- The File submenu appears.

3) Select "Load" in the submenu with the [L] key, or the [Arrow] and

[Enter] keys.

- A file name box appears and shows "FILE: *.ICF".

4) Type the desired file name, then push [Enter];

Or, push [Enter] to indicate a list of existing file names, select the

desired file name with an [Arrow] key, then push the [Enter] key twice.

- The typed or selected file is loaded, then the file name box

disappears.

- The loaded file name appears in the bottom left corner of the screen.

- 41 -

DOS COMMAND

In the "File" submenu, a "Dos" item appears.  This command pauses the

program so that a DOS command can be used.  When you want to return to

normal operation of the program, type "EXIT", then push [Enter].

- 42 -

-------------------------------------------------------------------------------

4. PRINTING

-------------------------------------------------------------------------------

All of the program screens can be printed out for convenience and reference

for subsequent programming.

1) Push [Esc] to indicate the TOP menu.

2) Select "Print" in the TOP menu with the [P] key, or the [Arrow] and

[Enter] keys.

- The Print submenu appears and shows "Current" and "All."

3) Push [C] or the [Arrow] and [Enter] keys to select "Current," or push

[A] or the [Arrow] and [Enter] keys to select "All."

-"Current" prints out the data for the currently displayed screen only;

"All" prints out the data for all of the screens of the currently loaded

file.

4) After printing is complete, the TOP and submenus disappear.

- 43 -