CTM-05 Register Level Programming Necessary sequence of events.

The CTM-10 has similar structure as CTM-05.

Function:                         If Written                         If Read

Base Addr                       Data Out                            Data In

Base +1                            Command Reg                   Status Reg

Base +2                            Interrupt Control Reg      Digital Inputs

Base +3                            Digital Out                         Interrupt Status

If a CTM-10 then a second AM9513 chip is present at:

Base +4                            Data Out                            Data In

Base +5                            Command Reg                   Status Reg

Base +6                            Interrupt Control Reg      Digital Inputs

Base +7                            Digital Out                         Interrupt Status

Refer also to section D of the CTM-10 and CTM-05A User Guide for description of the registers.

In order to program the AM9513 to do event counting on its 5 counters issue the following sequence of events:

1.  Initialize the 5 counters on each 9513 chip

2.  Zero and Enable counting

3.  Read the counters.  Note:  the counters will count to the max 65536 and then reset to zero and keep counting.

'*** initialize 9513 on CTM-05 board ***

'You must call this routine in the initializing section of your program.

'It sets up each counter to count up with SRC(n) as each clock input.

'SETTING THE MASTER MODE REGISTER

'First set the data pointer register through the command register

'The command code is as follows:-

'  1. C7-5 = 000 (Load data pointer register)

'  2. C4-3 = 10  (Element pointer = master mode register)

'  3. C2-0 = 111 (Group pointer = control)

'This decodes to a byte 0001 0111 or &H17

DASErr = DASByteOut(BaseAddress + 1, &H17)

'Next write to the master mode register

'We are setting up for the following conditions:-

'(Conditions marked X don't matter as we are not using these functions)

'  1. BCD division on scaler        MM15 = 1 (X)

'  2. Data pointer control          MM14 = 1 (enabled)

'  3. Data bus width = 8 bits       MM13 = 0

'  4. FOUT on                       MM12 = 0 (X)

'  5. FOUT divider = 10             MM11-8 = 1010 (X)

'  6. FOUT source = F5 (100Hz)      MM7-4 = 1111 (X)

'  7. Compare 2 disabled            MM3 = 0

'  8. Compare 1 disabled            MM2 = 0

'  9. Time of day mode disabled     MM1-0 = 00

'This makes a master mode word of 1100 1010 1111 0000 or &HCAF0

'This is written as 2 sequential bytes lo/hi &HF0 followed by &HCA

DASErr = DASByteOut(BaseAddress, &HF0)

DASErr = DASByteOut(BaseAddress, &HCA)

'SETTING INDIVIDUAL COUNTER MODE REGISTERS

'Now we can write to each individual counter mode register

'To do this we set the data pointer first through the command register

DASErr = DASByteOut(BaseAddress + 1, &H1)               'data pointer for Ctr. 1

'Now set counter 1 mode register as follows:-

'   1. No gating                    CM15-13 = 000

'   2. Count on rising edge         CM12 = 0

'   3. Input = SRC1                 CM11-8 = 0001

'   4. Disable special gate         CM7 = 0

'   5. Reload from load             CM6 = 0

'   6. Count repetitively           CM5 = 1 (lets you cascade ctrs.)

'   7. Binary count                 CM4 = 0

'   8. Count up                     CM3 = 1

'   9. Active low T/C pulse         CM2-0 = 101

'Counter 1 mode word = 0000 0001 0010 1101 or &H012D

DASErr = DASByteOut(BaseAddress, &H2D)

DASErr = DASByteOut(BaseAddress, &H1)        'mode for Ctr. 1

'Finally set modes of all other counters

DASErr = DASByteOut(BaseAddress + 1, &H2)               'data pointer for Ctr. 2

DASErr = DASByteOut(BaseAddress, &H2D):

DASErr = DASByteOut(BaseAddress, &H2)   'mode for Ctr. 2, SRC2 selected

DASErr = DASByteOut(BaseAddress + 1, &H3)               'data pointer for Ctr. 3

DASErr = DASByteOut(BaseAddress, &H2D):

DASErr = DASByteOut(BaseAddress, &H3)   'mode for Ctr. 3, SRC3 selected

DASErr = DASByteOut(BaseAddress + 1, &H4)              'data pointer for Ctr. 4

DASErr = DASByteOut(BaseAddress, &H2D):

DASErr = DASByteOut(BaseAddress, &H4)   'mode for Ctr. 4, SRC4 selected

DASErr = DASByteOut(BaseAddress + 1, &H5)               'data pointer for Ctr. 5

DASErr = DASByteOut(BaseAddress, &H2D)

DASErr = DASByteOut(BaseAddress, &H5)  'mode for Ctr. 5, SRC5 selected

Anytime you want to zero the counters issue:

'*** reset (zero) all counters ***

'Disarm all counters (stop counting)

DASErr = DASByteOut(BaseAddress + 1, &HDF)

'Enter zero to each counter load register

DASErr = DASByteOut(BaseAddress + 1, &H9)        'point to load register Ctr.1

DASErr = DASByteOut(BaseAddress, 0)

DASErr = DASByteOut(BaseAddress, 0) 'write zero to it

DASErr = DASByteOut(BaseAddress + 1, &HA)        'point to load register Ctr.2

DASErr = DASByteOut(BaseAddress, 0)

DASErr = DASByteOut(BaseAddress, 0) 'write zero to it

DASErr = DASByteOut(BaseAddress + 1, &HB)        'point to load register Ctr.3

DASErr = DASByteOut(BaseAddress, 0)

DASErr = DASByteOut(BaseAddress, 0) 'write zero to it

DASErr = DASByteOut(BaseAddress + 1, &HC)        'point to load register Ctr.4

DASErr = DASByteOut(BaseAddress, 0)

DASErr = DASByteOut(BaseAddress, 0) 'write zero to it

DASErr = DASByteOut(BaseAddress + 1, &HD)       'point to load register Ctr.5

DASErr = DASByteOut(BaseAddress, 0)

DASErr = DASByteOut(BaseAddress, 0) 'write zero to it

'Now load (reset) and arm (enable counting) on all counters

DASErr = DASByteOut(BaseAddress + 1, &H7F)                 <-------------------- IMPORTANT !!

In order to read the Counters:

'This does not interfere with counting or change data

'Data is returned in ReturnData(n)

DASErr = DASByteOut(BaseAddress + 1, &HBF) 'save data to all hold latches

'Now read data from each hold latch

DASErr = DASByteOut(BaseAddress + 1, &H11)                  'point to Ctr. 1 hold register

CL = DASByteIn(BaseAddress)

CH = DASByteIn(BaseAddress)  'read hold latch

ReturnData(1) = 256 * CH + CL         'calculate count

DASErr = DASByteOut(BaseAddress + 1, &H12)                  'point to Ctr. 2 hold register

CL = DASByteIn(BaseAddress)

CH = DASByteIn(BaseAddress)  'read hold latch

ReturnData(2) = 256 * CH + CL         'calculate count

DASErr = DASByteOut(BaseAddress + 1, &H13)                  'point to Ctr. 3 hold register

CL = DASByteIn(BaseAddress)

CH = DASByteIn(BaseAddress)  'read hold latch

ReturnData(3) = 256 * CH + CL         'calculate count

DASErr = DASByteOut(BaseAddress + 1, &H14)                     'point to Ctr. 4 hold register

CL = DASByteIn(BaseAddress)

CH = DASByteIn(BaseAddress)  'read hold latch

ReturnData(4) = 256 * CH + CL         'calculate count

DASErr = DASByteOut(BaseAddress + 1, &H15)                  'point to Ctr. 5 hold register

CL = DASByteIn(BaseAddress)

CH = DASByteIn(BaseAddress)  'read hold latch

ReturnData(5) = 256 * CH + CL         'calculate count