Purpose:  7242 and 7242A will not work in a 7200A without this modification.

Description:

1)  Remove module cover

2)  On 7242ICON make cut a cut at J33 C2 between the pin and the trace that runs next to it.

3)  Place a jumper between J33 C2 and J33 B2.

4)  Reinstall module cover.