EIP 1230A/1231A VXIbus Frequency Counter________________________________________________________________________________  Introduction:  This instrument driver provides programming support for EIP 1230A/1231A VXIbus Frequency Counter.  It contains functions for opening, configuring, taking measurement from, and  closing the instrument.________________________________________________________________________________  Assumptions:  To successfully use this module, the following conditions must be met:  For GPIB instruments drivers:         -  the instrument is connected to the GPIB.         -  the GPIB address supplied to the initialize function must match the            GPIB address of the instrument.  For VXI instruments drivers:         -  the instrument is installed in the VXI mainframe and you are            using one of the following controller options:            Embedded controller                  AT-MXI                  MC-MXI                  GPIB-VXI         -  the logical address supplied to the initialize function must            match the logical address of the instrument.         -  if you are using GPIB-VXI, there must be at least one unused            device name in the handler.________________________________________________________________________________  Error Codes:  Error codes are returned as the return value of each instrument driver  function. A program should examine this value after each call to an instrument  driver function to determine if any error occurred. Possible error codes and  their meanings are listed with the corresponding instrument driver function.________________________________________________________________________________  How To Use This Document:  This document is intended to be used as a programming reference manual.  It describes each function in the                   EIP 1230A/1231A VXIbus Frequency Counter  instrument driver. The functions are listed in alphabetical order with  a description of the function, C syntax of the function, a description of each parameter  parameter, and possible error codes.________________________________________________________________________________Function Tree Layout:   EIP 1230A/1231A VXIbus Frequency Counterunction Name:      Initialize                                  eip123x_init      Configuration         Bands (1230 & 1231)            Bands for EIP1230 A                   eip123x_band_eip1230            Band for EIP1231 A                    eip123x_band_eip1231            Center Frequency                      eip123x_center_freq            Frequency Window Limits               eip123x_freq_win_lim         Sample Rate                              eip123x_sample_rate         Resolution                               eip123x_resolution         Frequency Average                        eip123x_freq_avr         Frequency Offset                         eip123x_freq_offset         Min Pulse Repetition                     eip123x_min_freq_rep         Inhibit Input                            eip123x_inhibit_input         Time Base Source                         eip123x_time_base         Trigger Set Up                           eip123x_trig_setup      Special Functions         Special Function Select                  eip123x_special_funct         Terminate All Specials                   eip123x_terminate_spec      Measurement         Hold Function                            eip123x_hold_func         Set Measurement Type                     eip123x_set_meas_type         Trigger Function                         eip123x_trigger_func         Get Measurement                          eip123x_get_measurement      Save/Recall Instr Setup         Save/Recall Instr Setup                  eip123x_counter_setup         Save Default Setup                       eip123x_save_default_setup         Get General Setup Info                   eip123x_get_gen_setup_info      Low Level         Write to instrument                      eip123x_write         Read Instrument Data                     eip123x_read         Reset instrument                         eip123x_reset         Error Clear/Query                        eip123x_error_query         Revision Query                           eip123x_revision_query      Close                                       eip123x_close________________________________________________________________________________  EIP 1230A/1231A VXIbus Frequency Counter       Instrument Name: EIP123xA Pulsed Microwave                        Frequency Counter       Description:     This instrument module provides                        programming support for the                        EIP1230A and EIP1231A Microwave                        Frequency counters. The module                        is divided into the following                        Classes:       Functions/Classes:       (1) Initialize           Initialize the instrument and set to default           configuration.       (2) Configuration           This class contains the functions required to           configure the instrument.           (2a) Bands (EIP1230A & EIP1231A)                This sub-class contains the band                configuration functions for both                instruments. It also contains the                Frequency Windows function which may                only be used with Band 2, and the                Center Frequency Function which is                available for Bands 2 & 3.       (3)  Special Functions            This class contains the Special Functions            which this instrument driver supports.       (4)  Measurement Functions            This class contains the functions necessary            to capture and display measurements.       (5)  Save and Recall Instrument Setup            This class contains functions that store and            recall the current instrument setup       (6)  Close            Take the instrument offline.________________________________________________________________________________                Following Functions are in alphabetical order.________________________________________________________________________________  eip123x_band_eip1230      int eip123x_band_eip1230 (int instrumentID, int primaryBand, int subBandGHz);  Purpose       Panel Name:    Bands for EIP1230 A       Description:   This function contains the band                      configuration functions for the                      EIP1230A  Parameter List      instrumentID          Variable Type       int           Control Name:  Instrument ID           Description:   Accepts the Instrument ID returned                          by the Initialize function to                          select the desired instrument.           Variable Type: Integer           Valid Range:   1  to  Maximum Instances of                          this Instrument                          The Maximum Instances of this                          Instrument is the maximum number                          instruments that can be controlled                          by this driver in a single system.                          The value is set internally in the                          driver. The typical value is 10.      primaryBand          Variable Type       int           Control Name:  Primary Band (EIP 1230A)           Description:   This parameter controls the                          frequency measurement range for                          the EIP1230A model.                          Select a specific band according                          to the following table.                          0: 100 Hz - 250 MHz (CW only)                          1: 250 MHz - 1 GHz                          2: 950 MHz - 26.5 GHz                          3: 26.5 GHz - 170 GHz (Optional)           Variable Type: Integer           Valid Range:   0 - 100 Hz - 250 MHz (CW only)                          1 - 250 MHz - 1 GHz                          2 - 950 MHz - 26.5 GHz                          3 - 26.5 GHz - 170 GHz (Optional)           Default Value: 2 - 950 MHz - 26.5 GHz      subBandGHz          Variable Type       int           Control Name:  Sub Band (FOR BAND 3 ONLY)           Description:   This parameter controls the                          frequency measurement range of                          Band 3 and is set according to the                          remote sensor being used.  Select                          the appropriate subband according                          to the following table.                          1:  26.5 to 40 GHz                          2:  33 to 50 GHz                          3:  40 to 60 GHz                          4:  50 to 75 GHz                          5:  60 to 90 GHz                          6:  75 to 110 GHz                          7:  90 to 140 GHz                          8:  110 to 170 GHz           Variable Type: Integer           Valid Range:   1 -  26.5 to 40 GHz                          2 -  33 to 50 GHz                          3 -  40 to 60 GHz                          4 -  50 to 75 GHz                          5 -  60 to 90 GHz                          6 -  75 to 110 GHz                          7 -  90 to 140 GHz                          8 -  110 to 170 GHz           Default Value: 1  Return Value           Control Name:  Error Code           Description:   Displays status relating to the                          function call.           Variable Type: Integer           Error Codes:                  0   Success                 -1   Instrument ID out of range                 -2   Primary Band out of range                 -3   Sub Band out of range                230   Error

writing to instrument                231   Error reading from instrument                232   Instrument not initialized                330   Sub Band parameter valid only                      for the optional Band 26.5 - 170 GHz                      Set Sub Band back to default 26.5 -                      40 GHz                350   This Panel valid only for EIP 1230A________________________________________________________________________________  eip123x_band_eip1231      int eip123x_band_eip1231 (int instrumentID, int bandEIP1231A);  Purpose       Panel Name:    Bands for EIP1231 A       Description:   This function contains the band                      configuration functions for the                      EIP1231A  Parameter List      instrumentID          Variable Type       int           Control Name:  Instrument ID           Description:   Accepts the Instrument ID returned                          by the Initialize function to                          select the desired instrument.           Variable Type: Integer           Valid Range:   1  to  Maximum Instances of                          this Instrument                          The Maximum Instances of this                          Instrument is the maximum number                          instruments that can be controlled                          by this driver in a single system.                          The value is set internally in the                          driver. The typical value is 10.      bandEIP1231A          Variable Type       int           Control Name:  Band (EIP 1231A)           Description:   This parameter controls the                          frequency measurement range                          for the EIP 1231A model.                          Select a specific band according                          to the following table.                          0: 100 Hz - 250 MHz (CW only)                          1: 250 MHz - 1 GHz                          2: 950 MHz - 20 GHz           Variable Type: Integer           Valid Range:   0 - 100 Hz - 250 MHz (CW only)                          1 - 250 MHz - 1 GHz                          2 - 950 MHz - 20 GHz           Default Value: 2  Return Value           Control Name:  Error Code           Description:   Displays status relating to the                          function call.           Variable Type: Integer           Error Codes:                  0   Success                 -1   Instrument ID out of range                 -2   Band out of range                230   Error writing to instrument                231   Error reading from instrument                232   Instrument not initialized                360   This Panel Valid only for EIP 1231A________________________________________________________________________________  eip123x_center_freq      int eip123x_center_freq (int instrumentID, double centerFreqMHz);  Purpose       Panel Name:    Center Frequency       Description:   Controls the center frequency mode                      This function controls the center                      frequency mode of operation in                      which the counter looks for a                      signal in the vicinity of the                      center frequency value. This mode                      can be used to reduce acquisition                      time or when measuring a                      particular signal in a multiple                      signal environment. This mode of                      operation is available in Bands 2                      and 3 only. Select Band 2 center                      frequency in the range of 900 MHz                      to 26.7 GHz for the model 1230A,                      and in the range of 900 MHz to                      20.5 GHz for the model 1231A. The                      counter will lock on signals                      50 MHz from the entered center                      frequency. Select Band 3 center                      frequency in the range of the                      subband currently selected. The                      counter will lock on to signals                      2 GHz from the entered frequency,                      but will not reject signals                      outside of this range. If a signal                      is applied that is more than 2 GHz                      from the entered frequency an                      erroneous reading may result. The                      value entered by the user is                      truncated to 10 MHz.  Parameter List      instrumentID          Variable Type       int           Control Name:  Instrument ID           Description:   Accepts the Instrument ID returned                          by the Initialize function to                          select the desired instrument.           Variable Type: Integer           Valid Range:   1  to  Maximum Instances of                                 this Instrument                          The Maximum Instances of this                          Instrument is the maximum number                          instruments that can be controlled                          by this driver in a single system.                          The value is set internally in the                          driver. The typical value is 10.      centerFreqMHz          Variable Type       double           Control Name:  Center Frequency           Description:   This parameter controls the center                          frequency mode of operation in                          which the counter looks for a                          signal in the vicinity of the                          center frequency value.  This mode                          can be used to reduce acquisiton                          time when measuring a particular                          signal in a multiple signal                          environment.  This mode is                          available in either Band 2 or                          Band 3.           Variable Type: Real           Valid Range:   Dependent upon the selected band -                          See below.                   - Band 2: 900 MHz - 26,700 MHz (EIP1230A)                             900 MHz - 20,500 MHz (EIP1231A)                   - Band 3: The range of the Sub Band                               currently selected:                  Subband 1:  26,500 MHz to 40,000 MHz                  Subband 2:  33,000 MHz to 50,000 MHz                  Subband 3:  40,000 MHz to 60,000 MHz                  Subband 4:  50,000 MHz to 75,000 MHz                  Subband 5:  60,000 MHz to 90,000 MHz                  Subband 6:  75,000 MHz to 110,000 MHz                  Subband 7:  90,000 MHz to 140,000 MHz                  Subband 8:  110,000 MHz to 170,000 MHz           Default Value: 900  Return Value           Control Name:  Error Code           Description:   Displays status relating to the                          function call.           Variable Type: Integer           Error Codes:                  0   Success                 -1   Instrument ID out of range                 -2   Center Frequency out of range                230   Error writing to instrument                231   Error reading from instrument                232   Instrument not initialized                236   Error interpreting instrument response                340   Function valid only for bands 2 & 3                      in the EIP 1230 A, and band 2 in EIP                      1231 A.________________________________________________________________________________  eip123x_close      int eip123x_close (int instrumentID);  Purpose       Panel Name:     Close       Description:    The close routine removes the                       entry for the instrument from the                       Instrument Table.       Note: The instrument must be reinitialized to use             it again.  Parameter List      instrumentID          Variable Type       int           Control Name:  Instrument ID           Description:   Accepts the Instrument ID returned                          by the Initialize function to                          select the desired instrument.           Variable Type: Integer           Valid Range:   1  to  Maximum Instances of                          this Instrument                          The Maximum Instances of this                          Instrument is the maximum number                          instruments that can be controlled                          by this driver in a single system.                          The value is set internally in the                          driver. The typical value is 10.  Return Value           Control Name:  Error Code           Description:   Displays status relating to the                          function call.           Variable Type: Integer           Error Codes:                  0   Success                 -1   Instrument ID out of range                221   Unable to close device                232   Device not initialized________________________________________________________________________________  eip123x_counter_setup      int eip123x_counter_setup (int instrumentID, int mode, int setupRegister);  Purpose       Panel Name:    Save/Recall Instr Setup       Description:   This function selects and                      implements the Store and Recall                      functions. The Store command                      causes the instrument to save it's                      current setup in one of its 10                      internal storage registers (0-9).                      After power-up or initialization,

the instrument restores itself                      from register 0.  Parameter List      instrumentID          Variable Type       int           Control Name:  Instrument ID           Description:   Accepts the Instrument ID returned                          by the Initialize function to                          select the desired instrument.           Variable Type: Integer           Valid Range:   1  to  Maximum Instances of                          this Instrument                          The Maximum Instances of this                          Instrument is the maximum number                          instruments that can be controlled                          by this driver in a single system.                          The value is set internally in the                          driver. The typical value is 10.      mode          Variable Type       int           Control Name:  Mode           Description:   This function causes the counter's                          setup to be stored into or                          recalled from one of its                          10 internal storage registers.                          Register 0 is the instrument's                          default register.  At turn-on                          the instrument will be configured                          per setup data from register 0.                  Store:  Saves the current counter setup                          in the storage register specified                          in the register box.                  Recall: Recalls a counter setup which                          has been previously stored                          in the specified storage register.           Variable Type: Integer           Valid Range:   1 - Store                          0 - Recall           Default Value: 0 - Recall      setupRegister          Variable Type       int           Control Name:  Setup Register           Description:   Specifies which instrument                          storage register is to be                          stored or recalled.           Variable Type: Integer           Valid Range:   0 through 9           Default Value: 0  Return Value           Control Name:  Error Code           Description:   Displays status relating to the                          function call.           Variable Type: Integer           Error Codes:                  0   Success                 -1   Instrument ID out of range                 -2   Mode out of range                 -3   Setup Register out of range                230   Error writing to instrument                232   Instrument not initialized________________________________________________________________________________  eip123x_error_query      int eip123x_error_query (int instrumentID, int action, int *_123xErrorNumber,                               char errorMessage[]);  Purpose       Panel Name:    Error Clear/Query       Description:   Either clears pending error                      indications or reads the                      current error number. An error                      number of 0 means that there                      are no errors.  Parameter List      instrumentID          Variable Type       int           Control Name:  Instrument ID           Description:   Accepts the Instrument ID returned                          by the Initialize function to                          select the desired instrument.           Variable Type: Integer           Valid Range:   1 to  Maximum Instances of                                this Instrument                          The Maximum Instances of this                          Instrument is the maximum number                          of instruments that can be                          controlled by this driver in a                          single system. The value is set                          internally in the driver. The                          typical value is 10.      action          Variable Type       int           Control Name:  Action           Description:   This switch determines the action                          performed by the function.           Variable Type: Integer           Valid Range:   0 - Read last 123x error                          1 - Cause the 123x to clear                              its internal error                              indications.           Default Value: 0 - Read      _123xErrorNumber          Variable Type       int (passed by reference)           Control Name:  Error Code           Description:   This output returns the error code                          read from the instrument error                          queue.           Variable Type: Integer *      errorMessage          Variable Type       char []           Control Name:  Error Message           Description:   This output returns the error                          message string read from the                          instrument's error message queue.           Variable Type: String (char *)  Return Value           Control Name:  Error Code           Description:   Displays status relating to the                          function call.           Variable Type: Integer           Error Codes:                  0   Success                 -1   Instrument ID out of range                230   Error writing to instrument                231   Error reading from instrument                232   Instrument not initialized                236   Error interpreting instrument response________________________________________________________________________________  eip123x_freq_avr      int eip123x_freq_avr (int instrumentID, int frequencyAverage);  Purpose       Panel Name:    Frequency Average       Description:   This function controls the number                      of measurements to be averaged on                      frequency and pulse measurements.                      Select in the range of 1 to 99.  Parameter List      instrumentID          Variable Type       int           Control Name:  Instrument ID           Description:   Accepts the Instrument ID returned                          by the Initialize function to                          select the desired instrument.           Variable Type: Integer           Valid Range:   1  to  Maximum Instances of                          this Instrument                          The Maximum Instances of this                          Instrument is the maximum number                          instruments that can be controlled                          by this driver in a single system.                          The value is set internally in the                          driver. The typical value is 10.      frequencyAverage          Variable Type       int           Control Name:  Frequency Average           Description:   This parameter controls the number                          of measurements to be averaged on                          frequency and pulse parameters.                          Frequency averaging is typically                          used to reduce random measurement                          error.           Variable Type: Integer           Valid Range:   01  to 99           Default Value: 01  Return Value           Control Name:  Error Code           Description:   Displays status relating to the                          function call.           Variable Type: Integer           Error Codes:                  0   Success                 -1   Instrument ID out of range                 -2   Frequency Average out of range                230   Error writing to instrument                231   Error reading from instrument                232   Instrument not initialized________________________________________________________________________________  eip123x_freq_offset      int eip123x_freq_offset (int instrumentID, int freqMultiplier,                               double freqOffsetGHz);  Purpose       Panel Name:    Frequency Offset       Description:   Configures frequency measurement                      parameters. This function allows                      the entry of a positive or                      negative frequency to 1 kHz                      resolution into the offset                      frequency register. This parameter                      controls the contant B in the                      formula: Output Freq                      = (M * measured freq) + B where M                      is the frequency multiplier and B                      is the frequency offset. Select in                      the range -99.99  999 GHz to                      +99.99 999 GHz.  Parameter List      instrumentID          Variable Type       int           Control Name:  Instrument ID           Description:   Accepts the Instrument ID returned                          by the Initialize function to                          select the desired instrument.           Variable Type: Integer           Valid Range:   1  to  Maximum Instances of                          this Instrument                          The Maximum Instances of this                          Instrument is the maximum number                          instruments that can be controlled                          by this driver in a single system.                          The value is set internally in the                          driver. The typical value is 10.      freqMultiplier          Variable Type       int           Control Name:  Multiplier           Description:   This parameter controls the value                          of the constant M in the formula:                          Output Frequency

= (M * measured                          frequency) + B                          Where M is the frequency                          multiplier and B is the frequency                          offset.           Variable Type: Integer           Valid Range:   1 to 99           Default Value: 1      freqOffsetGHz          Variable Type       double           Control Name:  Offset Frequency           Description:   Allows the entry of a positive or                          negative frequency to 1 kHz                          resolution into the offset                          frequency register.  This                          parameter controls the constant B                          in the formula:                          Output Frequency = (M * measured                          frequency) + B                          Where M is the frequency                          multiplier and B is the frequency                          offset.           Variable Type: real           Valid Range:   -99.99 999 GHz to +99.999 999 GHz           Default Value: 0  Return Value           Control Name:  Error Code           Description:   Displays status relating to the                          function call.           Variable Type: Integer           Error Codes:                  0   Success                 -1   Instrument ID out of range                 -2   Frequency Multiplier out of range                 -3   Frequency Offset out of range                230   Error writing to instrument                231   Error reading from instrument                232   Instrument not initialized________________________________________________________________________________  eip123x_freq_win_lim      int eip123x_freq_win_lim (int instrumentID, double lowLimitMHz,                                double highLimitMHz);  Purpose       Panel Name:    Frequency Window Limits       Description:   This function controls the high                      and low limits of the frequency                      window that is searched in Band 2.                      Select Low and High Limit in the                      range of 900 MHz to 26.7 GHz for                      the model 1230A, and in the range                      of 900 MHz to 20.5 GHz for the                      model 1230A. The value entered by                      the user is truncated to 10 MHz                      resolution. This function is only                      available in Band 2, and Low limit                      must always be less than High                      limit.  Parameter List      instrumentID          Variable Type       int           Control Name:  Instrument ID           Description:   Accepts the Instrument ID returned                          by the Initialize function to                          select the desired instrument.           Variable Type: Integer           Valid Range:   1  to  Maximum Instances of                          this Instrument                          The Maximum Instances of this                          Instrument is the maximum number                          instruments that can be controlled                          by this driver in a single system.                          The value is set internally in the                          driver. The typical value is 10.      lowLimitMHz          Variable Type       double           Control Name:   Low Limit           Description:   This parameter controls the low                          end of the frequency window that                          is searched for a signal in Band                          2.  Select Low Limit in the                          range of 900 MHz to 26700 MHz for                          Model 1230A and in the range of                          900 MHz to 20500 MHz for Model                          1231A.                          The value entered by the user is                          trincated to 10 MHz resolution.                          This function is only available                          in Band 2.  Low Limit must always                          be lower than High Limit.           Variable Type: Real           Default Value: 900 MHz      highLimitMHz          Variable Type       double           Control Name:  High Limit           Description:   This parameter controls the high                          end of the frequency window that                          is searched for a signal in Band                          2.  Select the High Limit in the                          range of 900 MHz to 26,700 MHz for                          Model 1230A and in the range of                          900 MHz to 20,500 MHz for Model                          1231A.                          The value entered by the user is                          trincated to 10 MHz resolution.                          This function is only available                          in band 2.  Low Limit must always                          be lower than High Limit.           Variable Type: Real           Default Value: 26,700 MHz for EIP 1230A           - This is the value that the control is set to if             you are using a 1231A change the default             setting to: 20,500 MHz for EIP 1231A  Return Value           Control Name:  Error Code           Description:   Displays status relating to the                          function call.           Variable Type: Integer           Error Codes:                  0   Success                 -1   Instrument ID out of range                 -2   Low Limit out of range                 -3   High Limit out of range                230   Error writing to instrument                231   Error reading from instrument                232   Instrument not initialized                236   Error interpreting instrument response                380   This panel vaid only for Band 2.________________________________________________________________________________  eip123x_get_gen_setup_info      int eip123x_get_gen_setup_info (int instrumentID, int setup, char display[]);  Purpose       Panel Name:    Get General Setup Info:       Description:   This panel returns the current                      value of the parameter selected.                      Parameter definitions are below.       Average Val:   This parameter controls the number                      of measurements to be averaged on                      frequency and pulse parameters.       Band:          This parameter controls the                      frequency measurement range for                      the EIP1230A/1231A models.                      Select a specific band according                      to the following table.                      0: 100 Hz - 250 MHz (CW only)                      1: 250 MHz - 1 GHz                      2: 950 MHz - 26.5 GHz (EIP1230A)                         950 MHz - 20 GHz (EIP1231A)                      3: 26.5 GHz - 170 GHz (Optional)       Sub Band:      This parameter controls the                      frequency measurement range of                      Band 3 and is set according to the                      remote sensor being used.  Select                      the apprpriate sybband according                      to the following table.                      1:  26.5 GHz to 40 GHz                      2:  33 GHz to 50 GHz                      3:  40 GHz to 60 GHz                      4:  50 GHz to 75 GHz                      5:  60 GHz to 90 GHz                      6:  75 GHz to 110 GHz                      7:  90 GHz to 140 GHz                      8:  110 GHz to 170 GHz       Center Freq:   This parameter controls the center                      frequency mode of operation in                      which the counter looks for a                      signal in the vicinity of the                      center frequency value.  This mode                      can be uded to reduce acquisiton                      time or when measuring a                      particylar signal in a multiple                      signal environment.  This mode is                      available in either Band 2 or Band                      3.  The value is returned in Hz.       Resolution:    This parameter controls the                      frequency measurement resolution.                      Select the desired resolution                      according ro the following table:                      2: 100 Hz                      3: 1 kHz                      4: 10 kHz                      5: 100 kHz                      6: 1 MHz                      7: 10 MHz                      8: 100 MHz                      9: 1 GHz       Low/high limit:This parameter controls the lo/hi                      end of the frequency window that                      is searched for a signal in Band                      2.  Select Low Limit in the                      range of 900 MHz to 26700 MHz for                      Model 1230A and in the range of                      900 MHz to 20500 MHz for Model                      1231A.  The values are returned in                      Hz.       Synch Line:    Selects the TTL TRG line to be                      used in the Synchronous Trigger                      Protocol.  The SYNC message                      enables the SYNCHRONOUS TTLRG                      trigger protocol       Asynch Line:   Selects the TTL TRG line pair to                      be used in the Asynchroneous                      Trigger Protocol as follows:                      0: 0 & 1                      1: 2 & 3                      2: 4 & 5                      3: 6 & 7       Inhibit Line:  Selects the input

to be used for                      the measurement inhibit function                      as follows:                      0: VXIbus ECLTRG0 line                      1: VXIbus ECLTRG1 line                      2: Front Panel       MinPRF:        This parameter controls the                      minimum pulse repetition frequency                      of the pulsed signals that can be                      acquired and measured by the                      counter.  For example, if 500 Hz                      is selected, the counter will                      measure signals with a pulse                      repetition frequency of 500 Hz or                      greater.  The value is returned in                      Hz.       Multiplier:    This parameter controls the value                      of the constant M in the formula:                      Output Frequency = (M * measured                      frequency) + B                      Where M is the frequency                      multiplier and B is the frequency                      offset.       Offset:        Allows the entry of a positive or                      negative frequency to 1 kHz                      resolution into the offset                      frequency register.  This                      parameter controls the constant B                      in the formula:                      Output Frequency = (M * measured                      frequency) + B                      Where M is the frequency                      multiplier and B is the frequency                      offset.  Parameter List      instrumentID          Variable Type       int           Control Name:  Instrument ID           Description:   Accepts the Instrument ID returned                          by the Initialize function to                          select the desired instrument.           Variable Type: Integer           Valid Range:   1  to  Maximum Instances of                          this Instrument                          The Maximum Instances of this                          Instrument is the maximum number                          instruments that can be controlled                          by this driver in a single system.                          The value is set internally in the                          driver. The typical value is 10.      setup          Variable Type       int           Control Name:  Setup           Description:   Selects the Setup Parameter to be                          viewed, returns the current                          setting of the selected parameter           Variable Type: Integer           Valid Range:   0 - Avg Value                          1 - Band                          2 - Subband                          3 - Center Freq                          4 - Resolution                          5 - Low Limit                          6 - High Limit                          7 - Async Line                          9 - Sync Line                         10 - MIN PRF                         11 - Multiplier                         12 - Offset           Default Value: 12 - Offset      display          Variable Type       char []           Control Name:  Display           Description:   This function displays the current                          status of the parameter selected.                          Parameter definitions are below.           Average Val:   This parameter controls the number                          of measurements to be averaged on                          frequency and pulse parameters.           Band:          This parameter controls the                          frequency measurement range for                          the EIP1230A/1231A models.                          Select a specific band according                          to the following table.                          0: 100 Hz - 250 MHz (CW only)                          1: 250 MHz - 1 GHz                          2: 950 MHz - 26.5 GHz (EIP1230A)                             950 MHz - 20 GHz (EIP1231A)                          3: 26.5 GHz - 170 GHz (Optional)           Sub Band:      This parameter controls the                          frequency measurement range of                          Band 3 and is set according to the                          remote sensor being used.  Select                          the apprpriate sybband according                          to the following table.                          1:  26.5 GHz to 40 GHz                          2:  33 GHz to 50 GHz                          3:  40 GHz to 60 GHz                          4:  50 GHz to 75 GHz                          5:  60 GHz to 90 GHz                          6:  75 GHz to 110 GHz                          7:  90 GHz to 140 GHz                          8:  110 GHz to 170 GHz           Center Freq:   This parameter controls the center                          frequency mode of operation in                          which the counter looks for a                          signal in the vicinity of the                          center frequency value.  This mode                          can be uded to reduce acquisiton                          time or when measuring a                          particylar signal in a multiple                          signal environment.  This mode is                          available in either Band 2 or Band                          3.  The value is returned in Hz.           Resolution:    This parameter controls the                          frequency measurement resolution.                          Select the desired resolution                          according ro the following table:                          2: 100 Hz                          3: 1 kHz                          4: 10 kHz                          5: 100 kHz                          6: 1 MHz                          7: 10 MHz                          8: 100 MHz                          9: 1 GHz           Low/high limit:This parameter controls the lo/hi                          end of the frequency window that                          is searched for a signal in Band                          2.  Select Low Limit in the                          range of 900 MHz to 26700 MHz for                          Model 1230A and in the range of                          900 MHz to 20500 MHz for Model                          1231A.  The values are returned in                          Hz.           Synch Line:    Selects the TTL TRG line to be                          used in the Synchronous Trigger                          Protocol.  The SYNC message                          enables the SYNCHRONOUS TTLRG                          trigger protocol           Asynch Line:   Selects the TTL TRG line pair to                          be used in the Asynchroneous                          Trigger Protocol as follows:                          0: 0 & 1                          1: 2 & 3                          2: 4 & 5                          3: 6 & 7           Inhibit Line:  Selects the input to be used for                          the measurement inhibit function                          as follows:                          0: VXIbus ECLTRG0 line                          1: VXIbus ECLTRG1 line                          2: Front Panel           MinPRF:        This parameter controls the                          minimum pulse repetition frequency                          of the pulsed signals that can be                          acquired and measured by the                          counter.  For example, if 500 Hz                          is selected, the counter will                          measure signals with a pulse                          repetition frequency of 500 Hz or                          greater.  The value is returned in                          Hz.           Multiplier:    This parameter controls the value                          of the constant M in the formula:                          Output Frequency = (M * measured                          frequency) + B                          Where M is the frequency                          multiplier and B is the frequency                          offset.           Offset:        Allows the entry of a positive or                          negative frequency to 1 kHz                          resolution into the offset                          frequency register.  This                          parameter controls the constant B                          in the formula:                          Output Frequency = (M * measured                          frequency) + B                          Where M is the frequency                          multiplier and B is the frequency                          offset.  Return Value           Control Name:  Error Code           Description:   Displays status relating to the                          function call.           Variable Type: Integer           Error Codes:                  0   Success                 -1   Instrument ID out of range                 -2   Setup out of range                230   Error writing to instrument                231   Error reading from instrument                232   Instrument not initialized                236   Error interpreting instrument response________________________________________________________________________________  eip123x_get_measurement      int eip123x_get_measurement (int instrumentID, double *measurement);  Purpose       Panel Name:    Get Measurement       Description:   This function reads the                      measurement back from the                      instrument. All measurement                      configuration

and triggering must                      be implemented before this                      function is called to insure                      accurate data is returned from the                      instrument.  Parameter List      instrumentID          Variable Type       int           Control Name:  Instrument ID           Description:   Accepts the Instrument ID returned                          by the Initialize function to                          select the desired instrument.           Variable Type: Integer           Valid Range:   1  to  Maximum Instances of                          this Instrument                          The Maximum Instances of this                          Instrument is the maximum number                          instruments that can be controlled                          by this driver in a single system.                          The value is set internally in the                          driver. The typical value is 10.      measurement          Variable Type       double (passed by reference)           Control Name:  Result           Description:   Outputs the measurement result,                          all measurement configuration and                          triggering must be implemented                          before this function is called to                          insure accurate data is returned                          from the instrument.           Variable Type: Real  Return Value           Control Name:  Error Code           Description:   Displays status relating to the                          function call.           Variable Type: Integer           Error Codes:                  0   Success                 -1   Instrument ID out of range                231   Error reading from instrument                232   Instrument not initialized                236   Error interpreting instrument response________________________________________________________________________________  eip123x_hold_func      int eip123x_hold_func (int instrumentID, int hold);  Purpose       Panel Name:    Hold Function       Description:   This function selects the status                      of the Hold command.                      When Hold is On, the counter takes                      one reading and waits for a Reset                      or Trigger command.  Upon receipt                      of one of these commands, the                      counter will perform another                      measurement and, when requested,                      output the new reading over the                      VXI bus.  Note that a measurement                      result will be output only once;                      thus, when the instrument is in                      Hold, the user must trigger the                      counter before requesting another                      reading.  If no signal is found,                      the counter will output a zero.                      When the Hold command is Off, data                      is read out to the bus in the                      normal way.  The reading is                      automatically updated according to                      the specified sample rate, and the                      counter can output successive                      readings without requiring a Reset                      or Trigger command each time.  Parameter List      instrumentID          Variable Type       int           Control Name:  Instrument ID           Description:   Accepts the Instrument ID returned                          by the Initialize function to                          select the desired instrument.           Variable Type: Integer           Valid Range:   1  to  Maximum Instances of                          this Instrument                          The Maximum Instances of this                          Instrument is the maximum number                          instruments that can be controlled                          by this driver in a single system.                          The value is set internally in the                          driver. The typical value is 10.      hold          Variable Type       int           Control Name:  Hold           Description:   Holds the last result if On.                          Hold On puts the counter into                          triggered operational mode.           Variable Type: Integer           Valid Range:   1 - On                          0 - Off           Default Value: 0 - Off  Return Value           Control Name:  Error Code           Description:   Displays status relating to the                          function call.           Variable Type: Integer           Error Codes:                  0   Success                 -1   Instrument ID out of range                 -2   Hold out of range                230   Error writing to instrument                231   Error reading from instrument                232   Instrument not initialized________________________________________________________________________________  eip123x_inhibit_input      int eip123x_inhibit_input (int instrumentID, int inhibitInputSource);  Purpose       Panel Name:    Inhibit Input       Description:   This function selects the source                      of the signal controlling when the                      counter can detected a signal.                      This feature allows the counter                      to perform frequency measurements                      at specific points in time.                      Arguments 0, 1, and 2 correspond                      to the VXIbus ECLTRG0 line, VXIbus                      ECLTRG1 line, and front panel,                      respectively.                      Note: The Inhibit Input is active                      high (i.e., the counter is                      inhibited by a logic high signal                      and enabled by a logic low                      signal).  Parameter List      instrumentID          Variable Type       int           Control Name:  Instrument ID           Description:   Accepts the Instrument ID returned                          by the Initialize function to                          select the desired instrument.           Variable Type: Integer           Valid Range:   1  to  Maximum Instances of                          this Instrument                          The Maximum Instances of this                          Instrument is the maximum number                          instruments that can be controlled                          by this driver in a single system.                          The value is set internally in the                          driver. The typical value is 10.      inhibitInputSource          Variable Type       int           Control Name:  Inhibit Input           Description:   Selects the input to be used for                          the measurement inhibit function.           Variable Type: Integer           Valid Range:   0 - VXIbus ECLTRG0 line                          1 - VXIbus ECLTRG1 line                          2 - Front Panel           Default Value: 2 - Front Panel  Return Value           Control Name:  Error Code           Description:   Displays status relating to the                          function call.           Variable Type: Integer           Error Codes:                  0   Success                 -1   Instrument ID out of range                 -2   Inhibit Input out of range                230   Error writing to instrument                231   Error reading from instrument                232   Instrument not initialized________________________________________________________________________________  eip123x_init      int eip123x_init (int logicalAddress, int interface, int IDQuery,                        int resetDevice, int *instrumentID, int *modelNumber);  Purpose       Panel Name:    Initialize       Description:   This routine performs the                      following initialization actions:                      -  Opens the instrument by                         storing information about the                         instrument in the Instrument                         Table.                      -  Initializes the VXI interface                         (NI-VXI or NI-GPIB)                         accordingly.                      -  Performs an identification                         query on the Instrument to                         determine the model number.                      -  Sends initialization commands                         to the instrument.                      -  Returns an Instrument ID which                         is used to differentiate                         between instruments of the same                         model type.  This value will be                         used to identify the instrument                         in subsequent calls.  Parameter List      logicalAddress          Variable Type       int           Control Name:  Logical Address           Description:   Specifies the Logical Address of                          the VXI instrument.  This value                          must corrrespond to the current                          internal setting of the                          instrument.           Variable Type: Integer           Valid Range:   0 - 255           Default Value: 18      interface          Variable Type       int           Control Name:  Interface           Description:   Specifies which protocol is used                          to communicate with the VXI                          mainframe.                          NI-VXI should be selected for

MC-MXI, AT-MXI, or VXIpc-386.                          NI-GPIB should be selected for the                          GPIB-VXI.           Variable Type: Integer           Valid Range:   1 - NI VXI                          2 - NI GPIB           Default Value: 1 - NI VXI      IDQuery          Variable Type       int           Control Name:  ID Query           Description:   Specifies if an ID Query is sent                          to the instrument during the                          initialization procedure.           Variable Type: Integer           Valid Range:   1 - Do Query                          0 - Skip Query           Default Value: 1 - Do Query           NOTE:  Under normal circumstances the ID Query                  insures that the instrument initialized                  is the type supported by this driver.                  However circumstances may arise where it                  is undesirable to send an ID Query to the                  instrument.  In those cases; set this                  control to "Skip Query" and this function                  will initialize the selected interface and                  the command arrays in the driver, without                  doing an ID Query.      resetDevice          Variable Type       int           Control Name:  Reset Device           Description:   Specifies if the instrument is to                          be reset to its power-on settings                          during the initialization                          procedure.           Variable Type: Integer           Valid Range:   1 - Reset Device                          0 - Don't Reset           Default Value: 1 - Reset Device           NOTE: If you do not want the instrument reset.                 Set this control to "Don't Reset" while                 initializing the instrument.      instrumentID          Variable Type       int (passed by reference)           Control Name:  Instrument ID           Description:   Returns an Instrument ID that is                          used in all subsequent function                          calls to select the instrument at                          the address specified.                          If more than one instrument of the                          same model type is used, this ID                          will be used to differentiate                          between them. The first instrument                          of this model type initialized                          will be assigned an Instrument ID                          of 1.           Variable Type: Integer      modelNumber          Variable Type       int (passed by reference)           Control Name:  Model Number           Description:   Returns the instrument model                          number. If the user opted to                          skip the ID query, the instrument                          is assumed to be a 1230,                          otherwise the value is determined                          from the instrument.           Variable Type: Integer  Return Value           Control Name:  Error Code           Description:   Displays status relating to the                          function call.           Variable Type: Integer           Error Codes:                  0   Success                 -1   Logical Address out of range                 -2   Interface out of range                 -3   ID Query out of range                 -4   Reset Device out of range                220   Unable to open instrument                221   Unable to close instrument                222   Unable to initialize VXI library                223   Instrument identification query failed                230   Error writing to instrument                231   Error reading from instrument                232   Instrument not initialized                233   Error configuring GPIB address                236   Error interpreting instrument response                239   Error setting time out                241   Error configuring repeat addressing________________________________________________________________________________  eip123x_min_freq_rep      int eip123x_min_freq_rep (int instrumentID, double minPulseRepFreq,                                int minPRFUnits);  Purpose       Panel Name:    Min Pulse Repetition:       Description:   This function controls the minimum                      pulse repetition frequency of the                      pulsed signals that can be                      acquired and measured by the                      counter. For example if a MINPRF                      of 500 Hz is selected, the counter                      will only measure signals with a                      pulse repetition frequency of                      500 Hz or greater.  Parameter List      instrumentID          Variable Type       int           Control Name:  Instrument ID           Description:   Accepts the Instrument ID returned                          by the Initialize function to                          select the desired instrument.           Variable Type: Integer           Valid Range:   1  to  Maximum Instances of                          this Instrument                          The Maximum Instances of this                          Instrument is the maximum number                          instruments that can be controlled                          by this driver in a single system.                          The value is set internally in the                          driver. The typical value is 10.      minPulseRepFreq          Variable Type       double           Control Name:  Min Pulse Repetition           Description:   This parameter controls the                          minimum pulse repetition frequency                          of the pulsed signals that can be                          acquired and measured by the                          counter.  For example, if 500 Hz                          is selected, the counter will                          measure signals with a pulse                          repetition frequency of 500 Hz or                          greater.                          This parameter affects the                          acquisition speed indirectly via                          two internal processes: the                          waiting time for a pulse at each                          frequency step in the frequency                          range being searched, and the                          waiting time for a pulse when                          taking measurements before                          declaring a "signal lost"                          condition.                          When Min Pulse Repetition                          is set to a low value either                          center frequency of frequency                          limits to minimize signal                          aquisition time.           Variable Type: Real           Valid Range:   1.0 Hz to 2.0 kHz           Default Value: 2.0 kHz      minPRFUnits          Variable Type       int           Control Name:  Minimum PRF Units           Description:   Selects the units for the                          Minimum Pulse Repetition Frequency                          specified by the Min PRF control.           Valid Range:   0 - Hz                          1 - kHz                          2 - MHz                          3 - GHz           Default Value: 0 - Hz  Return Value           Control Name:  Error Code           Description:   Displays status relating to the                          function call.           Variable Type: Integer           Error Codes:                  0   Success                 -1   Instrument ID out of range                 -2   Min Pulse Rep Freq out of range                 -3   MinPRF Units out of range                230   Error writing to instrument                231   Error reading from instrument                232   Instrument not initialized________________________________________________________________________________  eip123x_read      int eip123x_read (int instrumentID, int numberBytesToRead, char readBuffer[],                        long *numBytesRead);  Purpose       Panel Name:    Read Instrument Data       Description:   Reads data from instrument buffer                      and returns it to the specified                      variable in memory.       NOTES: (1) Because the instrument may return both                  numeric and text data in response to                  queries this function returns the data                  in string format.              (2) If valid data is not available at the                  instrument's output buffer when this                  function is called the instrument will                  hang up and the function will not                  return until it times out.  If the                  time-out is disabled this function                  will hang indefinitely and it may be                  neccessary to reboot the computer to                  break out.  Parameter List      instrumentID          Variable Type       int           Control Name:  Instrument ID           Description:   Accepts the Instrument ID returned                          by the Initialize function to                          select the desired instrument.           Variable Type: Integer           Valid Range:   1 to  Maximum Instances of                                this Instrument                          The Maximum Instances of this                          Instrument is the maximum number                          of instruments

that can be                          controlled by this driver in a                          single system. The value is set                          internally in the driver. The                          typical value is 10.      numberBytesToRead          Variable Type       int           Control Name:  Number of Bytes to Read           Description:   The number of bytes specified by                          this control should be greater                          than or equal to the number of                          bytes which are to be read from                          the instrument. If the actual                          number of bytes to be read is                          greater than the number this                          control specifies then multiple                          reads will be necessary to empty                          the instrument's output buffer.                          If the instrument's output buffer                          is not emptied the instrument may                          return invalid data when future                          reads are performed.           Variable Type: Integer           Default Value: 50 (See NOTE 2)           NOTES: (1) If NO DATA is available at the                      instrument's output buffer when this                      function is called the instrument will                      hang up and the function will not                      return until it times out.  If the                      time-out is disabled, this function                      will hang indefinitely and it may be                      neccessary to reboot the computer.                  (2) If the number of bytes expected is                      greater than 50 the value of this                      control may be increased. If the                      actual number of bytes read is larger                      than the declared size of the read                      buffer a run-time error will be                      generated.      readBuffer          Variable Type       char []           Control Name:  Read Buffer           Description:   The incoming data from the                          instrument is placed into this                          variable.           Variable Type: String           NOTES: (1) Because the Instrument may return both                      numeric and text data in response to                      queries this function returns the data                      in string form.                  (2) This function does not overwrite any                      old data left in the string variable                      from the last time the function was                      called. String data in LabWindows is                      terminated with an ASCII null(Hex 0)                      and string manipulation functions will                      only recognize data before the ASCII                      null.                  (3) The declared size of the string                      variable must be greater than the                      actual number of bytes read from the                      instrument if it is not a run-time                      error will be generated.      numBytesRead          Variable Type       long (passed by reference)           Control Name:  Number of Bytes Read           Description:   This variable contains the actual                          number of bytes read from the                          instrument. This is the value                          which is returned by the read                          function. The global variable                          "ibcnt" holds the result of GPIB                          reads and the global variable                          "bytes" holds the result of VXI                          word serial reads.           Variable Type: Long           NOTE: If the actual number of bytes read is less                 than the number of bytes specified in                 number of bytes to read then the output                 buffer is probably empty. If the number of                 bytes read is 0 and the instrument returns                 a 231 error code the read function failed.                 The most probable cause for the read to                 fail is because there was no data available                 at the instrument's output buffer.  Return Value           Control Name:  Error Code           Description:   Displays status relating to the                          function call.           Variable Type: Integer           Error Codes:                  0   Success                 -1   Instrument ID out of range                230   Error writing to instrument                231   Error reading from instrument                232   Instrument not initialized                236   Error interpreting instrument response________________________________________________________________________________  eip123x_reset      int eip123x_reset (int instrumentID);  Purpose       Panel Name:    Reset       Description:   This function resets the                      instrument to its default state.  Parameter List      instrumentID          Variable Type       int           Control Name:  Instrument ID           Description:   Accepts the Instrument ID returned                          by the Initialize function to                          select the desired instrument.           Variable Type: Integer           Valid Range:   1 to  Maximum Instances of                                this Instrument                          The Maximum Instances of this                          Instrument is the maximum number                          of instruments that can be                          controlled by this driver in a                          single system. The value is set                          internally in the driver. The                          typical value is 10.  Return Value           Control Name:  Error Code           Description:   Displays status relating to the                          function call.           Variable Type: Integer           Error Codes:                  0   Success                 -1   Instrument ID out of range                230   Error writing to instrument                231   Error reading from instrument                232   Instrument not initialized                236   Error interpreting instrument response________________________________________________________________________________  eip123x_resolution      int eip123x_resolution (int instrumentID, int resolution);  Purpose       Panel Name:    Resolution       Description:   This function controls the                      frequency measurement resolution.                      Select the desired resolution                      according to the following table:            Resolution    Freq        Gate Time                2         100 Hz      10 ms                                      (Band 0 only)                3         1 kHz       1 ms                4         10 kHz      100 ys                5         100 kHz     10 ys                6         1 MHz       1ys                7         10 MHz      1ys                8         100 MHz     1ys                9         1 GHz       1ys  Parameter List      instrumentID          Variable Type       int           Control Name:  Instrument ID           Description:   Accepts the Instrument ID returned                          by the Initialize function to                          select the desired instrument.           Variable Type: Integer           Valid Range:   1  to  Maximum Instances of                          this Instrument                          The Maximum Instances of this                          Instrument is the maximum number                          instruments that can be controlled                          by this driver in a single system.                          The value is set internally in the                          driver. The typical value is 10.      resolution          Variable Type       int           Control Name:  Resolution           Description:   This parameter controls the                          frequency measurement resolution.                          Select the desired resolution                          according to the following table:                Resolution    Freq        Gate Time                    2         100 Hz      10 ms                                          (Band 0 only)                    3         1 kHz       1 ms                    4         10 kHz      100 us                    5         100 kHz     10 us                    6         1 MHz       1 us                    7         10 MHz      1 us                    8         100 MHz     1 us                    9         1 GHz       1 us           Variable Type: Integer           Valid Range:   2 - 100 Hz                          3 - 1 kHz                          4 - 10 kHz                          5 - 100 kHz                          6 - 1 MHz                          7 - 10 MHz                          8 - 100 MHz                          9 - 1 GHz           Default Value: 3 - 1 kHz  Return Value           Control Name:  Error Code           Description:   Displays status relating to the                          function call.           Variable Type: Integer           Error Codes:                  0   Success                 -1   Instrument ID out of range                 -2   Resolution out of range                230   Error writing to instrument                231   Error reading from instrument                232   Instrument not initialized________________________________________________________________________________

eip123x_revision_query      int eip123x_revision_query (int instrumentID, char driverRevision[],                                  char firmwareRevision[]);  Purpose       Panel Name:    Revision Query       Description:   This function returns the revision                      numbers of the instrument driver                      and instrument firmware, and tells                      the user what firmware the driver                      is compatible with, i.e., Driver                      Revision "A1.0" is compatible                      with Firmware Revision(s) "3.00".  Parameter List      instrumentID          Variable Type       int           Control Name:  Instrument ID           Description:   Accepts the Instrument ID returned                          by the Initialize function to                          select the desired instrument.           Variable Type: Integer           Valid Range:   1 to  Maximum Instances of                                this Instrument                          The Maximum Instances of this                          Instrument is the maximum number                          of instruments that can be                          controlled by this driver in a                          single system. The value is set                          internally in the driver. The                          typical value is 10.      driverRevision          Variable Type       char []           Control Name:  Driver Revision           Description:   This control returns the                          Instrument Driver Revision.           Variable Type: String (char *)      firmwareRevision          Variable Type       char []           Control Name:  Firmware Revision           Description:   This control returns the                          Instrument's Firmware                          Revision.           Variable Type: String (char *)  Return Value           Control Name:  Error Code           Description:   Displays status relating to the                          function call.           Variable Type: Integer           Error Codes:                  0   Success                 -1   Instrument ID out of range                230   Error writing to instrument                231   Error reading from instrument                232   Instrument not initialized                236   Error interpreting instrument response________________________________________________________________________________  eip123x_sample_rate      int eip123x_sample_rate (int instrumentID, long sampleRatemSec);  Purpose       Panel Name:    Sample Rate       Description:   This function sets a delay between                      measurement values (0 to 100 S                      to a 10 mS resolution).  Signal                      tracking is performed before each                      measurement.  This process adds                      approximately 100 ms to the delay                      between.  This means that even if                      the Sample Rate is set to 0 there                      will still be a 100 ms delay                      between measurements.  Parameter List      instrumentID          Variable Type       int           Control Name:  Instrument ID           Description:   Accepts the Instrument ID returned                          by the Initialize function to                          select the desired instrument.           Variable Type: Integer           Valid Range:   1  to  Maximum Instances of                          this Instrument                          The Maximum Instances of this                          Instrument is the maximum number                          instruments that can be controlled                          by this driver in a single system.                          The value is set internally in the                          driver. The typical value is 10.      sampleRatemSec          Variable Type       long           Control Name:  Sample Rate           Description:   Sets a delay between measurement                          values           Variable Type: Long           Valid Range:   0 to 100,000 ms, by 10 ms                          resolution           Default Value: 0 ms (maximum sampling rate)  Return Value           Control Name:  Error Code           Description:   Displays status relating to the                          function call.           Variable Type: Integer           Error Codes:                  0   Success                 -1   Instrument ID out of range                 -2   Sample Rate out of range                230   Error writing to instrument                231   Error reading from instrument                232   Instrument not initialized________________________________________________________________________________  eip123x_save_default_setup      int eip123x_save_default_setup (int instrumentID);  Purpose       Panel Name:    Save Default Setup       Description:   The counter's setup is recalled                      from Save/Recall register 0 at                      power-up or upon receipt of the                      the initialize command. The                      register can be changed by the                      user in order to alter the                      instrument's default settings.                      This function restores register                      0 to the instrument's factory                      default settings.  Parameter List      instrumentID          Variable Type       int           Control Name:  Instrument ID           Description:   Accepts the Instrument ID returned                          by the Initialize function to                          select the desired instrument.           Variable Type: Integer           Valid Range:   1  to  Maximum Instances of                          this Instrument                          The Maximum Instances of this                          Instrument is the maximum number                          instruments that can be controlled                          by this driver in a single system.                          The value is set internally in the                          driver. The typical value is 10.  Return Value           Control Name:  Error Code           Description:   Displays status relating to the                          function call.           Variable Type: Integer           Error Codes:                  0   Success                 -1   Instrument ID out of range                 -2   Mode out of range                 -3   Setup Register out of range                230   Error writing to instrument                232   Instrument not initialized________________________________________________________________________________  eip123x_set_meas_type      int eip123x_set_meas_type (int instrumentID, int measurementType);  Purpose       Panel Name:    Set Measurement Type       Description:   This function selects which type                      of measurement will be output                      by the instrument when the Get                      Measurement function is used:                      Frequency, Width, or Period.                      Also, use Set Measurement Type                      before using the trigger                      if a triggered mode is selected.  Parameter List      instrumentID          Variable Type       int           Control Name:  Instrument ID           Description:   Accepts the Instrument ID returned                          by the Initialize function to                          select the desired instrument.           Variable Type: Integer           Valid Range:   1  to  Maximum Instances of                          this Instrument                          The Maximum Instances of this                          Instrument is the maximum number                          instruments that can be controlled                          by this driver in a single system.                          The value is set internally in the                          driver. The typical value is 10.      measurementType          Variable Type       int           Control Name:  Measurement           Descriptor:    This parameter controls the                          measurement to output.           Variable Type: Integer           Valid Range:   0 - Frequency                          1 - Width                          2 - Period           Default Value: 0 - Frequency  Return Value           Control Name:  Error Code           Description:   Displays status relating to the                          function call.           Variable Type: Integer           Error Codes:                  0   Success                 -1   Instrument ID out of range                 -2   Measurement Type out of range                230   Error writing to instrument                232   Instrument not initialized________________________________________________________________________________  eip123x_special_funct      int eip123x_special_funct (int instrumentID, int selectSpecial,                                 double *returnedReading);  Purpose       Panel Name:    Special Functions       Description:   This function selects the special                      function from the special                      functions which this driver                      supports.                      Call the Terminate all specials                      function to terminate all active                      special special functions.                      Functions supported by this driver                      are:      Read Relative Frequency:

When this function is activated,                      the counter assigns a negative                      value to the last frequency                      reading and enters it into the                      frequency offset register                      (overwriting any existing                      frequency offset value). The last                      input frequency in this case means                      the actual frequency of the input                      signal, not including the effects                      of a frequency multiplier or other                      special function. The counter                      outputs the the relationship, i.e.                      the difference, between the last                      input frequency and the current                      one, subject to any other                      functions activated. It continues                      to do so until the function is                      terminated.      Display IF Readings:                      When this function is activated,                      the counter assigns a negative                      value to the local oscillator (LO)                      frequency and enters it into the                      frequency offset register                      (overwriting any existing                      frequency offset value). The                      counter then subtracts the LO                      frequency from the input frequency                      and outputs the resulting IF. It                      continues to do so until the                      function is terminated.      100 MHz Self-Test                      When this function is activated,                      the counter will count its                      internal VCO.  This function is                      use to verify that the  count                      chain, gate generator, and the VCO                      are operational.  Parameter List      instrumentID          Variable Type       int           Control Name:  Instrument ID           Description:   Accepts the Instrument ID returned                          by the Initialize function to                          select the desired instrument.           Variable Type: Integer           Valid Range:   1  to  Maximum Instances of                          this Instrument                          The Maximum Instances of this                          Instrument is the maximum number                          instruments that can be controlled                          by this driver in a single system.                          The value is set internally in the                          driver. The typical value is 10.      selectSpecial          Variable Type       int           Control Name:  Select Special           Description:   Selects or terminates the Special                          functions. Functions which are                          supported are:                          100 MHz Self-Test: When this                          function is activated, the counter                          counts an internally generated                          test signal.  The counter returns                          a frequency reading of 100 MHz to                          indicate it passed Self-Test.                          Read Relative Frequency: When this                          function is activated, the counter                          assigns a negative value to the                          last frequency reading and enters                          it into the frequency offset                          register (overwriting any existing                          frequency offset value). The last                          input frequency in this case means                          the actual frequency of the input                          signal, not including the effects                          of a frequency multiplier or other                          special function. The counter                          outputs the the relationship, i.e.                          the difference, between the last                          input frequency and the current                          one,subject to any other functions                          activated. It continues to do so                          until the function is terminated.                          Display IF Readings:                          When this function is activated,                          the counter assigns a negative                          value to the local oscillator (LO)                          frequency and enters it into the                          frequency offset register                          (overwriting any existing                          frequency offset value). The                          counter then subtracts the LO                          frequency from the input frequency                          and outputs the resulting IF. It                          continues to do so until the                          function is terminated.           Variable Type: Integer           Valid Range:   0 - 100 MHz Self-Test                          1 - Read Relative Frequency                          2 - Display IF Readings           Default Value: 0 - 100 MHz Self-Test      returnedReading          Variable Type       double (passed by reference)           Control Name:  Returned Reading           Description:   Outputs the Self-Test, Relative                          or IF frequency.           Variable type: Double  Return Value           Control Name:  Error Code           Description:   Displays status relating to the                          function call.           Variable Type: Integer           Error Codes:                  0   Success                 -1   Instrument ID out of range                 -2   Select Special out of range                230   Error writing to instrument                231   Error reading from instrument                232   Instrument not initialized                236   Error interpreting instrument response________________________________________________________________________________  eip123x_terminate_spec      int eip123x_terminate_spec (int instrumentID);  Purpose       Panel Name:    Terminate All Specials       Description:   This Function terminates all                      previously implemented special                      functions.  Parameter List      instrumentID          Variable Type       int           Control Name:  Instrument ID           Description:   Accepts the Instrument ID returned                          by the Initialize function to                          select the desired instrument.           Variable Type: Integer           Valid Range:   1  to  Maximum Instances of                          this Instrument                          The Maximum Instances of this                          Instrument is the maximum number                          instruments that can be controlled                          by this driver in a single system.                          The value is set internally in the                          driver. The typical value is 10.  Return Value           Control Name:  Error Code           Description:   Displays status relating to the                          function call.           Variable Type: Integer           Error Codes:                  0   Success                 -1   Instrument ID out of range                230   Error writing to instrument                231   Error reading from instrument                232   Instrument not initialized________________________________________________________________________________  eip123x_time_base      int eip123x_time_base (int instrumentID, int timeBaseSource);  Purpose       Panel Name:    Time Base Source       Description:   This function selects the source                      of the 10 MHz frequency reference                      signal.  Parameter List      instrumentID          Variable Type       int           Control Name:  Instrument ID           Description:   Accepts the Instrument ID returned                          by the Initialize function to                          select the desired instrument.           Variable Type: Integer           Valid Range:   1  to  Maximum Instances of                          this Instrument                          The Maximum Instances of this                          Instrument is the maximum number                          instruments that can be controlled                          by this driver in a single system.                          The value is set internally in the                          driver. The typical value is 10.      timeBaseSource          Variable Type       int           Control Name:  Time Base Source           Description:   Controls the source of the                          10 MHz frequency reference                          signal.           Variable Type: Integer           Valid Range:   0 - Internal                          1 - External           Default Value: 0 - Internal  Return Value           Control Name:  Error Code           Description:   Displays status relating to the                          function call.           Variable Type: Integer           Error Codes:                  0   Success                 -1   Instrument ID out of range                 -2   Time Base Source out of range                230   Error writing to instrument                231   Error reading from instrument                232   Instrument not initialized________________________________________________________________________________

eip123x_trig_setup      int eip123x_trig_setup (int instrumentID, int syncTrigLine, int asyncTrigLine);  Purpose       Panel Name:    Trigger Set Up       Description:   This function selects the                      triggering mode, configures                      the INT/EXT time base reference,                      and selects the TTLTRG line pair                      to be used in the Asynchronous                      Trigger Protocol. The valid range                      is 0 to 3, corresponding to TTLTRG                      pairs 0 & 1, 2 & 3, 4 & 5, and                      6 & 7 respectively. Issuing                      ASYNC 9 disables the protocol.                      Or it selects the TTLTRG line                      to be used in the Synchronous                      Trigger Protocol. the valid range                      is 0 to 7. Issuing SYNC 9 disables                      the protocol.  Parameter List      instrumentID          Variable Type       int           Control Name:  Instrument ID           Description:   Accepts the Instrument ID returned                          by the Initialize function to                          select the desired instrument.           Variable Type: Integer           Valid Range:   1  to  Maximum Instances of                          this Instrument                          The Maximum Instances of this                          Instrument is the maximum number                          instruments that can be controlled                          by this driver in a single system.                          The value is set internally in the                          driver. The typical value is 10.      syncTrigLine          Variable Type       int           Control Name:  Sync Trig Line           Description:   Selects the TTL TRG line to be                          used in the Synchronous Trigger                          Protocol.  The SYNC message                          enables the SYNCHRONOUS TTLRG                          trigger protocol           Variable Type: Integer           Valid Range:   0 - Line 0                          1 - Line 1                          2 - Line 2                          3 - Line 3                          4 - Line 4                          5 - Line 5                          6 - Line 6                          7 - Line 7                          9 - Protocol Disabled           Default Value: 9 - Protocol Disabled      asyncTrigLine          Variable Type       int           Control Name:  Async Trig Line           Description:   Selects the TTL TRG line pair to                          be used in the Asynchroneous                          Trigger Protocol as follows:                          0: 0 & 1                          1: 2 & 3                          2: 4 & 5                          3: 6 & 7           Variable Type: Integer           Valid Range:   0 - Line pair 0 & 1                          1 - Line pair 2 & 3                          2 - Line pair 4 & 5                          3 - Line pair 6 & 7                          9 - Protocol Disabled           Default Value: 9 - Protocol Disabled  Return Value           Control Name:  Error Code           Description:   Displays status relating to the                          function call.           Variable Type: Integer           Error Codes:                  0   Success                 -1   Instrument ID out of range                 -2   Sync Trig Line out of range                 -3   Async Trig Line out of range                230   Error writing to instrument                231   Error reading from instrument                232   Instrument not initialized________________________________________________________________________________  eip123x_trigger_func      int eip123x_trigger_func (int instrumentID, int function);  Purpose       Panel Name:    Trigger Function       Description:   This function selects and                      implements one of the two internal                      trigger functions, trigger or                      reset.                      Trigger triggers a new                      measurement cycle. If a                      measurement cycle is in progress                      it will be aborted.                      Reset resets                      the converter and restarts the                      measurement process. If a                      measurement is found, a                      measurement will be taken, even if                      the counter is in Hold.  Parameter List      instrumentID          Variable Type       int           Control Name:  Instrument ID           Description:   Accepts the Instrument ID returned                          by the Initialize function to                          select the desired instrument.           Variable Type: Integer           Valid Range:   1  to  Maximum Instances of                          this Instrument                          The Maximum Instances of this                          Instrument is the maximum number                          instruments that can be controlled                          by this driver in a single system.                          The value is set internally in the                          driver. The typical value is 10.      function          Variable Type       int           Control Name:  Function           Description:                          1. Reset: Resets the converter and                             restarts the signal acquisition                             process.  If a signal is found,                             a measurement will be taken,                             even if the counter is in Hold.                          2. Trigger: Triggers a new                             measurement cycle, If a                             measurement cycle is in                             progress, it will be aborted.           Variable Type: Integer           Valid Range:   1 - Trigger                          0 - Reset           Default Value: 0 - Reset  Return Value           Control Name:  Error Code           Description:   Displays status relating to the                          function call.           Variable Type: Integer           Error Codes:                  0   Success                 -1   Instrument ID out of range                 -2   Function out of range                230   Error writing to instrument                232   Instrument not initialized________________________________________________________________________________  eip123x_write      int eip123x_write (int instrumentID, char writeBuffer[]);  Purpose       Panel Name:    Write To Instrument       Description:   Writes commands and queries to the                      instrument to modify parameters                      and query device settings.  Parameter List      instrumentID          Variable Type       int           Control Name:  Instrument ID           Description:   Accepts the Instrument ID returned                          by the Initialize function to                          select the desired instrument.           Variable Type: Integer           Valid Range:   1 to  Maximum Instances of                                this Instrument                          The Maximum Instances of this                          Instrument is the maximum number                          of instruments that can be                          controlled by this driver in a                          single system. The value is set                          internally in the driver. The                          typical value is 10.      writeBuffer          Variable Type       char []           Control Name:  Write Buffer           Description:   The user can use this control to                          send common commands and queries                          to the instrument. This control                          can also be used to write any                          valid command to the instrument.           Variable Type: String           Default Value: "*RST"           NOTES: (1) The command or query to be sent to the                      instrument may be a literal enclosed                      in double quotes i.e. "*RST" or may be                      contained in a variable of type                      string.                  (2) If the number of bytes to be written                      is greater than the size of "cmd" in                      the source code then the size of cmd                      must be increased. The original size                      of cmd is 50 bytes (cmd[50]) the                      largest cmd may be declared as is                      32,767 bytes.  Return Value           Control Name:  Error Code           Description:   Displays the results of the                          function call.           Variable Type: Integer           Error Codes:                  0   Success                 -1   Instrument ID out of range                230   Error writing to instrument                232   Instrument not initialized