EIP 1230A/1231A VXIbus Frequency Counter________________________________________________________________________________  Introduction:  This instrument driver provides programming support for EIP 1230A/1231A VXIbus  Frequency Counter.  It contains functions for opening, configuring, taking measurement from, and  closing the instrument.________________________________________________________________________________  Assumptions:  To successfully use this module, the following conditions must be met:  For GPIB instruments drivers:         -  the instrument is connected to the GPIB.         -  the GPIB address supplied to the initialize function must match the            GPIB address of the instrument.         -  there must be at least one unused device name in the handler.  For VXI instruments drivers:         -  the instrument is installed in the VXI mainframe and you are            using one of the following controller options:            Embedded controller                  AT-MXI                  MC-MXI                  GPIB-VXI         -  the logical address supplied to the initialize function must            match the logical address of the instrument.         -  if you are using GPIB-VXI, there must be at least one unused            device name in the handler.  For RS-232 instruments drivers:         -  the instrument is connected to the RS-232 interface.         -  the COM port, baud rate, parity, and timeout supplied to the            initialize function must match the settings of the instrument.________________________________________________________________________________  LabWindows Libraries:  In order to use this instrument driver outside of the LabWindows  environment, it must be linked with the following LabWindows libraries:  For GPIB instruments drivers:         - 	GPIB Library         - 	Format and I/O Library         -	LabWindows System Library  For VXI instruments drivers:         - 	The appropriate VXI Library:                  VXI386.LIB for embedded controllers                  VXIAT.LIB for the AT-MXI                  VXIMC.LIB for the MC-MXI                  GPIB_C.LIB for GPIB-VXI (C users)                  GPIB_QB.LIB for GPIB-VXI (QuickBASIC users)         -	Format and I/O Library         -	LabWindows System Library  For RS-232 instruments drivers:         - 	RS-232 Library         - 	Format and I/O Library         -	LabWindows System Library  National Instruments strongly recommends that you use the LWMake utility to  build any standalone executable program based on this intrument driver.________________________________________________________________________________  Error Codes:  Error codes are returned as the return value of each instrument driver  function. A program should examine this value after each call to an instrument  driver function to determine if any error occurred. Possible error codes and  their meanings are listed with the corresponding instrument driver function.________________________________________________________________________________  How To Use This Document:  This document is intended to be used as a programming reference manual.  It describes each function in the                    EIP 1230A/1231A VXIbus Frequency Counter  instrument driver. The functions are listed in alphabetical order with  a description of the function, C and QuickBASIC syntax of the function,  a description of each parameter, and possible error codes.________________________________________________________________________________Function Tree Layout:   EIP 1230A/1231A VXIbus Frequency Counter     Function Name:      Initialize                                eip123x_init      Configuration         Bands (1230 & 1231)            Bands for EIP1230 A                 eip123x_band_eip1230            Band for EIP1231 A                  eip123x_band_eip1231            Center Frequency                    eip123x_center_freq            Frequency Window Limits             eip123x_freq_win_lim         Sample Rate                            eip123x_sample_rate         Resolution                             eip123x_resolution         Frequency Average                      eip123x_freq_avr         Frequency Offset                       eip123x_freq_offset         Min Pulse Repetition                   eip123x_min_freq_rep         Inhibit Input                          eip123x_inhibit_input         Time Base Source                       eip123x_time_base         Trigger Set Up                         eip123x_trig_setup      Special Functions         Special Function Select                eip123x_special_funct         Terminate All Specials                 eip123x_terminate_spec      Measurement         Hold Function                          eip123x_hold_func         Set Measurement Type                   eip123x_set_meas_type         Trigger Function                       eip123x_trigger_func         Get Measurement                        eip123x_get_measurement      Save/Recall Instr Setup         Save/Recall Instr Setup                eip123x_counter_setup         Save Default Setup                     eip123x_save_default_setup         Get General Setup Info                 eip123x_get_gen_setup_info      Low Level         Write to Instrument                    eip123x_write         Read Instrument Data                   eip123x_read         Reset instrument                       eip123x_reset         Error Clear/Query                      eip123x_error_query         Revision Query                         eip123x_revision_query      Close                                     eip123x_close________________________________________________________________________________  Instrument         : EIP 1230A/1231A VXIbus Frequency Counter                      Instrument Name: EIP123xA Pulsed Microwave                                       Frequency Counter                                           Description:     This instrument module provides                                       programming support for the                                       EIP1230A and EIP1231A Microwave                                       Frequency counters. The module                                       is divided into the following                                       Classes:                                           Functions/Classes:                                           (1) Initialize                          Initialize the instrument and set to default                          configuration.                                           (2) Configuration                          This class contains the functions required to                          configure the instrument.                                               (2a) Bands (EIP1230A & EIP1231A)                               This sub-class contains the band                               configuration functions for both                               instruments. It also contains the                               Frequency Windows function which may                               only be used with Band 2, and the                               Center Frequency Function which is                               available for Bands 2 & 3.                                           (3)  Special Functions                           This class contains the Special Functions                           which this instrument driver supports.                                           (4)  Measurement Functions                           This class contains the functions necessary                           to capture and display measurements.                                           (5)  Save and Recall Instrument Setup                           This class contains functions that store and                           recall the current instrument setup                                           (6)  Low Level Functions:                           This Class provides lower level functions to                           communicate with the instrument, and change                           instrument parameters.                                           (7)  Close                           Take the instrument offline.________________________________________________________________________________                 Following functions are in alphabetical order.________________________________________________________________________________  Function Name      : eip123x_band_eip1230  C Syntax           : int eip123x_band_eip1230(Instrument_ID, Primary_Band,                                Sub_Band_GHz);                       int Instrument_ID;                       int Primary_Band;                       int Sub_Band_GHz;  QuickBASIC Syntax  : Error.Code% = eip123x.band.eip1230(Instrument.ID%, _                                Primary.Band%, Sub.Band.GHz%)  Purpose            :  Panel Name:    Bands for EIP1230 A                                               Description:   This function contains the band                                       configuration functions for the                                       EIP1230A  Parameter List     : Instrument ID                            Control Name:  Instrument ID                                                       Description:   Accepts the Instrument ID returned                                           by the Initialize function to                                           select the desired instrument.                                                       Variable Type: Integer                                                       Valid Range:   1  to  Maximum Instances of

this Instrument                                                                      The Maximum Instances of this                                           Instrument is the maximum number                                           instruments that can be controlled                                           by this driver in a single system.                                           The value is set internally in the                                           driver. The typical value is 10.                       Primary Band                           0 - 250 MHz -                            1 - 1 GHz -                            2 - 26.5 GHz - (Default)                           3 - 170 GHz -                             Control Name:  Primary Band (EIP 1230A)                                                       Description:   This parameter controls the                                           frequency measurement range for                                           the EIP1230A model.                                           Select a specific band according                                           to the following table.                                                                      0: 100 Hz - 250 MHz (CW only)                                           1: 250 MHz - 1 GHz                                           2: 950 MHz - 26.5 GHz                                           3: 26.5 GHz - 170 GHz (Optional)                                                       Variable Type: Integer                                                       Valid Range:   0 - 100 Hz - 250 MHz (CW only)                                           1 - 250 MHz - 1 GHz                                           2 - 950 MHz - 26.5 GHz                                           3 - 26.5 GHz - 170 GHz (Optional)                                                       Default Value: 2 - 950 MHz - 26.5 GHz                       Sub Band (GHz)                           1 - 26.5 - 40 - (Default)                           2 - 33 - 50 -                            3 - 40 - 60 -                            4 - 50 - 75 -                            5 - 60 - 90 -                            6 - 75 - 110 -                            7 - 90 - 140 -                            8 - 110 - 170 -                             Control Name:  Sub Band (FOR BAND 3 ONLY)                                                       Description:   This parameter controls the                                           frequency measurement range of                                           Band 3 and is set according to the                                           remote sensor being used.  Select                                           the appropriate subband according                                           to the following table.                                                                      1:  26.5 to 40 GHz                                           2:  33 to 50 GHz                                           3:  40 to 60 GHz                                           4:  50 to 75 GHz                                           5:  60 to 90 GHz                                           6:  75 to 110 GHz                                           7:  90 to 140 GHz                                           8:  110 to 170 GHz                                                       Variable Type: Integer                                                       Valid Range:   1 -  26.5 to 40 GHz                                           2 -  33 to 50 GHz                                           3 -  40 to 60 GHz                                           4 -  50 to 75 GHz                                           5 -  60 to 90 GHz                                           6 -  75 to 110 GHz                                           7 -  90 to 140 GHz                                           8 -  110 to 170 GHz                                                       Default Value: 1  Return Value       :  Control Name:  Error Code                                               Description:   Displays status relating to the                                       function call.                                               Variable Type: Integer                                               Error Codes:                                                      0   Success                                                     -1   Instrument ID out of range                              -2   Primary Band out of range                              -3   Sub Band out of range                                                    230   Error writing to instrument                             231   Error reading from instrument                             232   Instrument not initialized                                                    330   Sub Band parameter valid only                                   for the optional Band 26.5 - 170 GHz                                   Set Sub Band back to default 26.5 -                                   40 GHz                                                    350   This Panel valid only for EIP 1230A________________________________________________________________________________  Function Name      : eip123x_band_eip1231  C Syntax           : int eip123x_band_eip1231(Instrument_ID, Band_EIP_1231A);                       int Instrument_ID;                       int Band_EIP_1231A;  QuickBASIC Syntax  : Error.Code% = eip123x.band.eip1231(Instrument.ID%, _                                Band.EIP.1231A%)  Purpose            :  Panel Name:    Bands for EIP1231 A                                               Description:   This function contains the band                                       configuration functions for the                                       EIP1231A  Parameter List     : Instrument ID                            Control Name:  Instrument ID                                                       Description:   Accepts the Instrument ID returned                                           by the Initialize function to                                           select the desired instrument.                                                       Variable Type: Integer                                                       Valid Range:   1  to  Maximum Instances of                                           this Instrument                                                                      The Maximum Instances of this                                           Instrument is the maximum number                                           instruments that can be controlled                                           by this driver in a single system.                                           The value is set internally in the                                           driver. The typical value is 10.                       Band (EIP 1231A)                           0 - 250 MHz -                            1 - 1 GHz -                            2 - 20 GHz - (Default)                            Control Name:  Band (EIP 1231A)                                                       Description:   This parameter controls the                                           frequency measurement range                                           for the EIP 1231A model.                                           Select a specific band according                                           to the following table.                                                                      0: 100 Hz - 250 MHz (CW only)                                           1: 250 MHz - 1 GHz                                           2: 950 MHz - 20 GHz                                                       Variable Type: Integer                                                       Valid Range:   0 - 100 Hz - 250 MHz (CW only)                                           1 - 250 MHz - 1 GHz                                           2 - 950 MHz - 20 GHz                                                       Default Value: 2  Return Value       :  Control Name:  Error Code                                               Description:   Displays status relating to the                                       function call.                                               Variable Type: Integer                                               Error Codes:                                                      0   Success                                                     -1   Instrument ID out of range                              -2   Band out of range                                                    230   Error writing to instrument                             231   Error reading from instrument                             232   Instrument not initialized                                                    360   This Panel Valid only for EIP 1231A________________________________________________________________________________  Function Name      : eip123x_center_freq  C Syntax           : int eip123x_center_freq(Instrument_ID, Center_Freq_MHz);                       int Instrument_ID;                       double Center_Freq_MHz;  QuickBASIC Syntax  : Error.Code% = eip123x.center.freq(Instrument.ID%, _                                Center.Freq.MHz#)  Purpose            :  Panel Name:    Center Frequency                                               Description:   Controls the center frequency mode                                       This function controls the center                                       frequency mode of operation in                                       which the counter looks for a

signal in the vicinity of the                                       center frequency value. This mode                                       can be used to reduce acquisition                                       time or when measuring a                                       particular signal in a multiple                                       signal environment. This mode of                                       operation is available in Bands 2                                       and 3 only. Select Band 2 center                                       frequency in the range of 900 MHz                                       to 26.7 GHz for the model 1230A,                                       and in the range of 900 MHz to                                       20.5 GHz for the model 1231A. The                                       counter will lock on signals                                       50 MHz from the entered center                                       frequency. Select Band 3 center                                       frequency in the range of the                                       subband currently selected. The                                       counter will lock on to signals                                       2 GHz from the entered frequency,                                       but will not reject signals                                       outside of this range. If a signal                                       is applied that is more than 2 GHz                                       from the entered frequency an                                       erroneous reading may result. The                                       value entered by the user is                                       truncated to 10 MHz.                         Parameter List     : Instrument ID                            Control Name:  Instrument ID                                                       Description:   Accepts the Instrument ID returned                                           by the Initialize function to                                           select the desired instrument.                                                       Variable Type: Integer                                                       Valid Range:   1  to  Maximum Instances of                                                  this Instrument                                                                      The Maximum Instances of this                                           Instrument is the maximum number                                           instruments that can be controlled                                           by this driver in a single system.                                           The value is set internally in the                                           driver. The typical value is 10.                       Center Freq (MHz)                            Control Name:  Center Frequency                                                       Description:   This parameter controls the center                                           frequency mode of operation in                                           which the counter looks for a                                           signal in the vicinity of the                                           center frequency value.  This mode                                           can be used to reduce acquisiton                                           time when measuring a particular                                           signal in a multiple signal                                           environment.  This mode is                                           available in either Band 2 or                                           Band 3.                                                       Variable Type: Real                                                       Valid Range:   Dependent upon the selected band -                                           See below.                                                               - Band 2: 900 MHz - 26,700 MHz (EIP1230A)                                              900 MHz - 20,500 MHz (EIP1231A)                                                               - Band 3: The range of the Sub Band                                                currently selected:                                                              Subband 1:  26,500 MHz to 40,000 MHz                                   Subband 2:  33,000 MHz to 50,000 MHz                                   Subband 3:  40,000 MHz to 60,000 MHz                                   Subband 4:  50,000 MHz to 75,000 MHz                                   Subband 5:  60,000 MHz to 90,000 MHz                                   Subband 6:  75,000 MHz to 110,000 MHz                                   Subband 7:  90,000 MHz to 140,000 MHz                                   Subband 8:  110,000 MHz to 170,000 MHz                                                                                  Default Value: 900  Return Value       :  Control Name:  Error Code                                               Description:   Displays status relating to the                                       function call.                                               Variable Type: Integer                                               Error Codes:                                                      0   Success                                                     -1   Instrument ID out of range                              -2   Center Frequency out of range                                                    230   Error writing to instrument                             231   Error reading from instrument                             232   Instrument not initialized                             236   Error interpreting instrument response                                                    340   Function valid only for bands 2 & 3                                   in the EIP 1230 A, and band 2 in EIP                                   1231 A.________________________________________________________________________________  Function Name      : eip123x_close  C Syntax           : int eip123x_close(Instrument_ID);                       int Instrument_ID;  QuickBASIC Syntax  : Error.Code% = eip123x.close(Instrument.ID%)  Purpose            :  Panel Name:     Close                                               Description:    The close routine removes the                                        entry for the instrument from the                                        Instrument Table.                                               Note: The instrument must be reinitialized to use                              it again.  Parameter List     : Instrument ID                            Control Name:  Instrument ID                                                       Description:   Accepts the Instrument ID returned                                           by the Initialize function to                                           select the desired instrument.                                                       Variable Type: Integer                                                       Valid Range:   1  to  Maximum Instances of                                           this Instrument                                                                      The Maximum Instances of this                                           Instrument is the maximum number                                           instruments that can be controlled                                           by this driver in a single system.                                           The value is set internally in the                                           driver. The typical value is 10.  Return Value       :  Control Name:  Error Code                                               Description:   Displays status relating to the                                       function call.                                               Variable Type: Integer                                               Error Codes:                                                      0   Success                                                     -1   Instrument ID out of range                                                    221   Unable to close device                             232   Device not initialized________________________________________________________________________________  Function Name      : eip123x_counter_setup  C Syntax           : int eip123x_counter_setup(Instrument_ID, Mode,                                Setup_Register);                       int Instrument_ID;                       int Mode;                       int Setup_Register;  QuickBASIC Syntax  : Error.Code% = eip123x.counter.setup(Instrument.ID%, _                                Mode%, Setup.Register%)  Purpose            :  Panel Name:    Save/Recall Instr Setup                                               Description:   This function selects and                                       implements the Store and Recall                                       functions. The Store command                                       causes the instrument to save it's                                       current setup in one of its 10                                       internal storage registers (0-9).                                       After power-up or initialization,                                       the instrument restores itself                                       from register 0.                         Parameter List     : Instrument ID                            Control Name:

Instrument ID                                                       Description:   Accepts the Instrument ID returned                                           by the Initialize function to                                           select the desired instrument.                                                       Variable Type: Integer                                                       Valid Range:   1  to  Maximum Instances of                                           this Instrument                                                                      The Maximum Instances of this                                           Instrument is the maximum number                                           instruments that can be controlled                                           by this driver in a single system.                                           The value is set internally in the                                           driver. The typical value is 10.                       Mode                           1 - Store                            0 - Recall (Default)                            Control Name:  Mode                                                       Description:   This function causes the counter's                                           setup to be stored into or                                           recalled from one of its                                           10 internal storage registers.                                           Register 0 is the instrument's                                           default register.  At turn-on                                           the instrument will be configured                                           per setup data from register 0.                                                              Store:  Saves the current counter setup                                           in the storage register specified                                           in the register box.                                                              Recall: Recalls a counter setup which                                           has been previously stored                                           in the specified storage register.                                                       Variable Type: Integer                                                       Valid Range:   1 - Store                                           0 - Recall                                                       Default Value: 0 - Recall                       Setup Register                            Control Name:  Setup Register                                                       Description:   Specifies which instrument                                           storage register is to be                                           stored or recalled.                                                       Variable Type: Integer                                                       Valid Range:   0 through 9                                                       Default Value: 0  Return Value       :  Control Name:  Error Code                                               Description:   Displays status relating to the                                       function call.                                               Variable Type: Integer                                               Error Codes:                                                      0   Success                                                     -1   Instrument ID out of range                              -2   Mode out of range                              -3   Setup Register out of range                                                    230   Error writing to instrument                             232   Instrument not initialized________________________________________________________________________________  Function Name      : eip123x_error_query  C Syntax           : int eip123x_error_query(Instrument_ID, Action,                                123x_Error_Number, Error_Message);                       int Instrument_ID;                       int Action;                       int *123x_Error_Number;    !! Invalid control label name.                       char *Error_Message;  QuickBASIC Syntax  : Error.Code% = eip123x.error.query(Instrument.ID%, _                                Action%, 123x.Error.Number%, Error.Message$)  Purpose            :  Panel Name:    Error Clear/Query                                               Description:   Either clears pending error                                       indications or reads the                                       current error number. An error                                       number of 0 means that there                                       are no errors.                         Parameter List     : Instrument ID                            Control Name:  Instrument ID                                                       Description:   Accepts the Instrument ID returned                                           by the Initialize function to                                           select the desired instrument.                                                       Variable Type: Integer                                                       Valid Range:   1 to  Maximum Instances of                                                 this Instrument                                                                      The Maximum Instances of this                                           Instrument is the maximum number                                           of instruments that can be                                           controlled by this driver in a                                           single system. The value is set                                           internally in the driver. The                                           typical value is 10.                       Action                           1 - Clear Error                            0 - Read Error (Default)                            Control Name:  Action                                                       Description:   This switch determines the action                                           performed by the function.                                                       Variable Type: Integer                                                       Valid Range:   0 - Read last 123x error                                           1 - Cause the 123x to clear                                               its internal error                                               indications.                                                       Default Value: 0 - Read                       123x Error Number                            Control Name:  Error Code                                                       Description:   This output returns the error code                                           read from the instrument error                                           queue.                                                       Variable Type: Integer *                       Error Message                            Control Name:  Error Message                                                       Description:   This output returns the error                                           message string read from the                                           instrument's error message queue.                                                       Variable Type: String (char *)  Return Value       :  Control Name:  Error Code                                               Description:   Displays status relating to the                                       function call.                                               Variable Type: Integer                                               Error Codes:                                                      0   Success                                                     -1   Instrument ID out of range                                                    230   Error writing to instrument                             231   Error reading from instrument                             232   Instrument not initialized                             236   Error interpreting instrument response________________________________________________________________________________  Function Name      : eip123x_freq_avr  C Syntax           : int eip123x_freq_avr(Instrument_ID, Frequency_Average);                       int Instrument_ID;                       int Frequency_Average;  QuickBASIC Syntax  : Error.Code% = eip123x.freq.avr(Instrument.ID%, _                                Frequency.Average%)  Purpose            :  Panel Name:    Frequency Average                                               Description:   This function controls the number                                       of measurements to be averaged on                                       frequency and pulse measurements.                                       Select in the range of 1 to 99.  Parameter List     : Instrument ID                            Control Name:  Instrument ID                                                       Description:   Accepts the Instrument ID returned                                           by the Initialize function to                                           select the desired instrument.                                                       Variable Type: Integer                                                       Valid Range:   1  to  Maximum Instances of                                           this Instrument

The Maximum Instances of this                                           Instrument is the maximum number                                           instruments that can be controlled                                           by this driver in a single system.                                           The value is set internally in the                                           driver. The typical value is 10.                       Frequency Average                            Control Name:  Frequency Average                                                       Description:   This parameter controls the number                                           of measurements to be averaged on                                           frequency and pulse parameters.                                           Frequency averaging is typically                                           used to reduce random measurement                                           error.                                                       Variable Type: Integer                                                       Valid Range:   01  to 99                                                       Default Value: 01  Return Value       :  Control Name:  Error Code                                               Description:   Displays status relating to the                                       function call.                                               Variable Type: Integer                                               Error Codes:                                                      0   Success                                                     -1   Instrument ID out of range                              -2   Frequency Average out of range                                                    230   Error writing to instrument                             231   Error reading from instrument                             232   Instrument not initialized________________________________________________________________________________  Function Name      : eip123x_freq_offset  C Syntax           : int eip123x_freq_offset(Instrument_ID, Freq_Multiplier,                                Freq_Offset__GHz);                       int Instrument_ID;                       int Freq_Multiplier;                       double Freq_Offset__GHz;  QuickBASIC Syntax  : Error.Code% = eip123x.freq.offset(Instrument.ID%, _                                Freq.Multiplier%, Freq.Offset..GHz#)  Purpose            :  Panel Name:    Frequency Offset                                               Description:   Configures frequency measurement                                       parameters. This function allows                                       the entry of a positive or                                       negative frequency to 1 kHz                                       resolution into the offset                                       frequency register. This parameter                                       controls the contant B in the                                       formula: Output Freq                                       = (M * measured freq) + B where M                                       is the frequency multiplier and B                                       is the frequency offset. Select in                                       the range -99.99  999 GHz to                                       +99.99 999 GHz.  Parameter List     : Instrument ID                            Control Name:  Instrument ID                                                       Description:   Accepts the Instrument ID returned                                           by the Initialize function to                                           select the desired instrument.                                                       Variable Type: Integer                                                       Valid Range:   1  to  Maximum Instances of                                           this Instrument                                                                      The Maximum Instances of this                                           Instrument is the maximum number                                           instruments that can be controlled                                           by this driver in a single system.                                           The value is set internally in the                                           driver. The typical value is 10.                       Freq Multiplier                            Control Name:  Multiplier                                                       Description:   This parameter controls the value                                           of the constant M in the formula:                                                                      Output Frequency = (M * measured                                           frequency) + B                                                                      Where M is the frequency                                           multiplier and B is the frequency                                           offset.                                                       Variable Type: Integer                                                       Valid Range:   1 to 99                                                       Default Value: 1                       Freq Offset  (GHz)                            Control Name:  Offset Frequency                                                       Description:   Allows the entry of a positive or                                           negative frequency to 1 kHz                                           resolution into the offset                                           frequency register.  This                                           parameter controls the constant B                                           in the formula:                                                                      Output Frequency = (M * measured                                           frequency) + B                                                                      Where M is the frequency                                           multiplier and B is the frequency                                           offset.                                                       Variable Type: real                                                       Valid Range:   -99.99 999 GHz to +99.999 999 GHz                                                       Default Value: 0  Return Value       :  Control Name:  Error Code                                               Description:   Displays status relating to the                                       function call.                                               Variable Type: Integer                                               Error Codes:                                                      0   Success                                                     -1   Instrument ID out of range                              -2   Frequency Multiplier out of range                              -3   Frequency Offset out of range                                                    230   Error writing to instrument                             231   Error reading from instrument                             232   Instrument not initialized________________________________________________________________________________  Function Name      : eip123x_freq_win_lim  C Syntax           : int eip123x_freq_win_lim(Instrument_ID, Low_Limit_MHz,                                High_Limit_MHz);                       int Instrument_ID;                       double Low_Limit_MHz;                       double High_Limit_MHz;  QuickBASIC Syntax  : Error.Code% = eip123x.freq.win.lim(Instrument.ID%, _                                Low.Limit.MHz#, High.Limit.MHz#)  Purpose            :  Panel Name:    Frequency Window Limits                                               Description:   This function controls the high                                       and low limits of the frequency                                       window that is searched in Band 2.                                       Select Low and High Limit in the                                       range of 900 MHz to 26.7 GHz for                                       the model 1230A, and in the range                                       of 900 MHz to 20.5 GHz for the                                       model 1230A. The value entered by                                       the user is truncated to 10 MHz                                       resolution. This function is only                                       available in Band 2, and Low limit                                       must always be less than High                                       limit.  Parameter List     : Instrument ID                            Control Name:  Instrument ID                                                       Description:   Accepts the Instrument ID returned                                           by the Initialize function to                                           select the desired instrument.                                                       Variable Type: Integer                                                       Valid Range:   1  to  Maximum Instances of                                           this Instrument                                                                      The Maximum Instances of this                                           Instrument is the maximum number                                           instruments that can be controlled                                           by this

driver in a single system.                                           The value is set internally in the                                           driver. The typical value is 10.                       Low Limit (MHz)                            Control Name:   Low Limit                                                       Description:   This parameter controls the low                                           end of the frequency window that                                           is searched for a signal in Band                                           2.  Select Low Limit in the                                           range of 900 MHz to 26700 MHz for                                           Model 1230A and in the range of                                           900 MHz to 20500 MHz for Model                                           1231A.                                                                      The value entered by the user is                                           trincated to 10 MHz resolution.                                           This function is only available                                           in Band 2.  Low Limit must always                                           be lower than High Limit.                                                       Variable Type: Real                                                       Default Value: 900 MHz                                                  High Limit (MHz)                            Control Name:  High Limit                                                       Description:   This parameter controls the high                                           end of the frequency window that                                           is searched for a signal in Band                                           2.  Select the High Limit in the                                           range of 900 MHz to 26,700 MHz for                                           Model 1230A and in the range of                                           900 MHz to 20,500 MHz for Model                                           1231A.                                                                      The value entered by the user is                                           trincated to 10 MHz resolution.                                           This function is only available                                           in band 2.  Low Limit must always                                           be lower than High Limit.                                                       Variable Type: Real                                                       Default Value: 26,700 MHz for EIP 1230A                                                       - This is the value that the control is set to if                              you are using a 1231A change the default                              setting to: 20,500 MHz for EIP 1231A  Return Value       :  Control Name:  Error Code                                               Description:   Displays status relating to the                                       function call.                                               Variable Type: Integer                                               Error Codes:                                                      0   Success                                                     -1   Instrument ID out of range                              -2   Low Limit out of range                              -3   High Limit out of range                                                    230   Error writing to instrument                             231   Error reading from instrument                             232   Instrument not initialized                             236   Error interpreting instrument response                                                    380   This panel vaid only for Band 2.________________________________________________________________________________  Function Name      : eip123x_get_gen_setup_info  C Syntax           : int eip123x_get_gen_setup_info(Instrument_ID, Setup,                                Display);                       int Instrument_ID;                       int Setup;                       char *Display;  QuickBASIC Syntax  : Error.Code% = eip123x.get.gen.setup.info(Instrument.ID%, _                                Setup%, Display$)  Purpose            :  Panel Name:    Get General Setup Info:                                               Description:   This panel returns the current                                       value of the parameter selected.                                       Parameter definitions are below.                                               Average Val:   This parameter controls the number                                       of measurements to be averaged on                                       frequency and pulse parameters.                                               Band:          This parameter controls the                                       frequency measurement range for                                       the EIP1230A/1231A models.                                       Select a specific band according                                       to the following table.                                                              0: 100 Hz - 250 MHz (CW only)                                       1: 250 MHz - 1 GHz                                       2: 950 MHz - 26.5 GHz (EIP1230A)                                          950 MHz - 20 GHz (EIP1231A)                                       3: 26.5 GHz - 170 GHz (Optional)                                               Sub Band:      This parameter controls the                                       frequency measurement range of                                       Band 3 and is set according to the                                       remote sensor being used.  Select                                       the apprpriate sybband according                                       to the following table.                                                              1:  26.5 GHz to 40 GHz                                       2:  33 GHz to 50 GHz                                       3:  40 GHz to 60 GHz                                       4:  50 GHz to 75 GHz                                       5:  60 GHz to 90 GHz                                       6:  75 GHz to 110 GHz                                       7:  90 GHz to 140 GHz                                       8:  110 GHz to 170 GHz                                               Center Freq:   This parameter controls the center                                       frequency mode of operation in                                       which the counter looks for a                                       signal in the vicinity of the                                       center frequency value.  This mode                                       can be uded to reduce acquisiton                                       time or when measuring a                                       particylar signal in a multiple                                       signal environment.  This mode is                                       available in either Band 2 or Band                                       3.  The value is returned in Hz.                                               Resolution:    This parameter controls the                                       frequency measurement resolution.                                       Select the desired resolution                                       according ro the following table:                                                              2: 100 Hz                                       3: 1 kHz                                       4: 10 kHz                                       5: 100 kHz                                       6: 1 MHz                                       7: 10 MHz                                       8: 100 MHz                                       9: 1 GHz                                               Low/high limit:This parameter controls the lo/hi                                       end of the frequency window that                                       is searched for a signal in Band                                       2.  Select Low Limit in the                                       range of 900 MHz to 26700 MHz for                                       Model 1230A and in the range of                                       900 MHz to 20500 MHz for Model                                       1231A.  The values are returned in                                       Hz.                                               Synch Line:    Selects the TTL TRG line to be                                       used in the Synchronous Trigger                                       Protocol.  The SYNC message                                       enables the SYNCHRONOUS TTLRG                                       trigger protocol                                               Asynch Line:   Selects the TTL TRG line pair to                                       be used in the Asynchroneous                                       Trigger Protocol as follows:                                                              0: 0 & 1                                       1: 2 & 3                                       2: 4 & 5                                       3: 6 & 7                                               Inhibit Line:  Selects the input to be used for                                       the measurement inhibit function                                       as follows:

0: VXIbus ECLTRG0 line                                       1: VXIbus ECLTRG1 line                                       2: Front Panel                                               MinPRF:        This parameter controls the                                       minimum pulse repetition frequency                                       of the pulsed signals that can be                                       acquired and measured by the                                       counter.  For example, if 500 Hz                                       is selected, the counter will                                       measure signals with a pulse                                       repetition frequency of 500 Hz or                                       greater.  The value is returned in                                       Hz.                                               Multiplier:    This parameter controls the value                                       of the constant M in the formula:                                                              Output Frequency = (M * measured                                       frequency) + B                                                              Where M is the frequency                                       multiplier and B is the frequency                                       offset.                                               Offset:        Allows the entry of a positive or                                       negative frequency to 1 kHz                                       resolution into the offset                                       frequency register.  This                                       parameter controls the constant B                                       in the formula:                                                              Output Frequency = (M * measured                                       frequency) + B                                                              Where M is the frequency                                       multiplier and B is the frequency                                       offset.  Parameter List     : Instrument ID                            Control Name:  Instrument ID                                                       Description:   Accepts the Instrument ID returned                                           by the Initialize function to                                           select the desired instrument.                                                       Variable Type: Integer                                                       Valid Range:   1  to  Maximum Instances of                                           this Instrument                                                                      The Maximum Instances of this                                           Instrument is the maximum number                                           instruments that can be controlled                                           by this driver in a single system.                                           The value is set internally in the                                           driver. The typical value is 10.                       Setup                            0 - Average Val-                             1 - Band -                             2 - Subband -                             3 - Center Freq-                             4 - Resolution -                             5 - Low Limit -                             6 - High Limit -                             7 - Asynch Line-                             8 - Synch Line -                             9 - Inhibit Line                            10 - MinPRF -                            11 - Multiplier -                            12 - Offset - (Default)                            Control Name:  Setup                                                       Description:   Selects the Setup Parameter to be                                           viewed, returns the current                                           setting of the selected parameter                                                       Variable Type: Integer                                                       Valid Range:   0 - Avg Value                                           1 - Band                                           2 - Subband                                           3 - Center Freq                                           4 - Resolution                                           5 - Low Limit                                           6 - High Limit                                           7 - Async Line                                           9 - Sync Line                                          10 - MIN PRF                                          11 - Multiplier                                          12 - Offset                                                       Default Value: 12 - Offset                       Display                            Control Name:  Display                                                       Description:   This function displays the current                                           status of the parameter selected.                                           Parameter definitions are below.                                                       Average Val:   This parameter controls the number                                           of measurements to be averaged on                                           frequency and pulse parameters.                                                       Band:          This parameter controls the                                           frequency measurement range for                                           the EIP1230A/1231A models.                                           Select a specific band according                                           to the following table.                                                                      0: 100 Hz - 250 MHz (CW only)                                           1: 250 MHz - 1 GHz                                           2: 950 MHz - 26.5 GHz (EIP1230A)                                              950 MHz - 20 GHz (EIP1231A)                                           3: 26.5 GHz - 170 GHz (Optional)                                                       Sub Band:      This parameter controls the                                           frequency measurement range of                                           Band 3 and is set according to the                                           remote sensor being used.  Select                                           the apprpriate sybband according                                           to the following table.                                                                      1:  26.5 GHz to 40 GHz                                           2:  33 GHz to 50 GHz                                           3:  40 GHz to 60 GHz                                           4:  50 GHz to 75 GHz                                           5:  60 GHz to 90 GHz                                           6:  75 GHz to 110 GHz                                           7:  90 GHz to 140 GHz                                           8:  110 GHz to 170 GHz                                                       Center Freq:   This parameter controls the center                                           frequency mode of operation in                                           which the counter looks for a                                           signal in the vicinity of the                                           center frequency value.  This mode                                           can be uded to reduce acquisiton                                           time or when measuring a                                           particylar signal in a multiple                                           signal environment.  This mode is                                           available in either Band 2 or Band                                           3.  The value is returned in Hz.                                                       Resolution:    This parameter controls the                                           frequency measurement resolution.                                           Select the desired resolution                                           according ro the following table:                                                                      2: 100 Hz                                           3: 1 kHz                                           4: 10 kHz                                           5: 100 kHz                                           6: 1 MHz                                           7: 10 MHz                                           8: 100 MHz                                           9: 1 GHz                                                       Low/high limit:This parameter controls the lo/hi                                           end of the frequency window that                                           is searched for a signal in Band                                           2.  Select Low Limit in the                                           range of 900 MHz to 26700 MHz for                                           Model 1230A and in the range of                                           900 MHz to 20500 MHz for Model                                           1231A.  The values are returned in                                           Hz.                                                       Synch Line:    Selects the TTL TRG line to be

used in the Synchronous Trigger                                           Protocol.  The SYNC message                                           enables the SYNCHRONOUS TTLRG                                           trigger protocol                                                       Asynch Line:   Selects the TTL TRG line pair to                                           be used in the Asynchroneous                                           Trigger Protocol as follows:                                                                      0: 0 & 1                                           1: 2 & 3                                           2: 4 & 5                                           3: 6 & 7                                                       Inhibit Line:  Selects the input to be used for                                           the measurement inhibit function                                           as follows:                                                                      0: VXIbus ECLTRG0 line                                           1: VXIbus ECLTRG1 line                                           2: Front Panel                                                       MinPRF:        This parameter controls the                                           minimum pulse repetition frequency                                           of the pulsed signals that can be                                           acquired and measured by the                                           counter.  For example, if 500 Hz                                           is selected, the counter will                                           measure signals with a pulse                                           repetition frequency of 500 Hz or                                           greater.  The value is returned in                                           Hz.                                                       Multiplier:    This parameter controls the value                                           of the constant M in the formula:                                                                      Output Frequency = (M * measured                                           frequency) + B                                                                      Where M is the frequency                                           multiplier and B is the frequency                                           offset.                                                       Offset:        Allows the entry of a positive or                                           negative frequency to 1 kHz                                           resolution into the offset                                           frequency register.  This                                           parameter controls the constant B                                           in the formula:                                                                      Output Frequency = (M * measured                                           frequency) + B                                                                      Where M is the frequency                                           multiplier and B is the frequency                                           offset.  Return Value       :  Control Name:  Error Code                                               Description:   Displays status relating to the                                       function call.                                               Variable Type: Integer                                               Error Codes:                                                      0   Success                                                     -1   Instrument ID out of range                              -2   Setup out of range                                                    230   Error writing to instrument                             231   Error reading from instrument                             232   Instrument not initialized                             236   Error interpreting instrument response________________________________________________________________________________  Function Name      : eip123x_get_measurement  C Syntax           : int eip123x_get_measurement(Instrument_ID, Measurement);                       int Instrument_ID;                       double *Measurement;  QuickBASIC Syntax  : Error.Code% = eip123x.get.measurement(Instrument.ID%, _                                Measurement#)  Purpose            :  Panel Name:    Get Measurement                                               Description:   This function reads the                                       measurement back from the                                       instrument. All measurement                                       configuration and triggering must                                       be implemented before this                                       function is called to insure                                       accurate data is returned from the                                       instrument.  Parameter List     : Instrument ID                            Control Name:  Instrument ID                                                       Description:   Accepts the Instrument ID returned                                           by the Initialize function to                                           select the desired instrument.                                                       Variable Type: Integer                                                       Valid Range:   1  to  Maximum Instances of                                           this Instrument                                                                      The Maximum Instances of this                                           Instrument is the maximum number                                           instruments that can be controlled                                           by this driver in a single system.                                           The value is set internally in the                                           driver. The typical value is 10.                       Measurement                            Control Name:  Result                                                       Description:   Outputs the measurement result,                                           all measurement configuration and                                           triggering must be implemented                                           before this function is called to                                           insure accurate data is returned                                           from the instrument.                                                       Variable Type: Real  Return Value       :  Control Name:  Error Code                                               Description:   Displays status relating to the                                       function call.                                               Variable Type: Integer                                               Error Codes:                                                      0   Success                                                     -1   Instrument ID out of range                                                    231   Error reading from instrument                             232   Instrument not initialized                             236   Error interpreting instrument response________________________________________________________________________________  Function Name      : eip123x_hold_func  C Syntax           : int eip123x_hold_func(Instrument_ID, Hold);                       int Instrument_ID;                       int Hold;  QuickBASIC Syntax  : Error.Code% = eip123x.hold.func(Instrument.ID%, Hold%)  Purpose            :  Panel Name:    Hold Function                                               Description:   This function selects the status                                       of the Hold command.                                                              When Hold is On, the counter takes                                       one reading and waits for a Reset                                       or Trigger command.  Upon receipt                                       of one of these commands, the                                       counter will perform another                                       measurement and, when requested,                                       output the new reading over the                                       VXI bus.  Note that a measurement                                       result will be output only once;                                       thus, when the instrument is in                                       Hold, the user must trigger the                                       counter before requesting another                                       reading.  If no signal is found,                                       the counter will output a zero.                                                              When the Hold command is Off, data                                       is read out to the bus in the                                       normal way.  The reading is                                       automatically updated according to                                       the specified sample rate, and the                                       counter can output successive                                       readings without requiring a Reset                                       or Trigger command each time.  Parameter List     : Instrument ID                            Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned                                           by the Initialize function to                                           select the desired instrument.                                                       Variable Type: Integer                                                       Valid Range:   1  to  Maximum Instances of                                           this Instrument                                                                      The Maximum Instances of this                                           Instrument is the maximum number                                           instruments that can be controlled                                           by this driver in a single system.                                           The value is set internally in the                                           driver. The typical value is 10.                       Hold                           1 - On                            0 - Off (Default)                            Control Name:  Hold                                                       Description:   Holds the last result if On.                                           Hold On puts the counter into                                           triggered operational mode.                                                       Variable Type: Integer                                                       Valid Range:   1 - On                                           0 - Off                                                       Default Value: 0 - Off  Return Value       :  Control Name:  Error Code                                               Description:   Displays status relating to the                                       function call.                                               Variable Type: Integer                                               Error Codes:                                                      0   Success                                                     -1   Instrument ID out of range                              -2   Hold out of range                                                    230   Error writing to instrument                             231   Error reading from instrument                             232   Instrument not initialized________________________________________________________________________________  Function Name      : eip123x_inhibit_input  C Syntax           : int eip123x_inhibit_input(Instrument_ID,                                Inhibit_Input_Source);                       int Instrument_ID;                       int Inhibit_Input_Source;  QuickBASIC Syntax  : Error.Code% = eip123x.inhibit.input(Instrument.ID%, _                                Inhibit.Input.Source%)  Purpose            :  Panel Name:    Inhibit Input                                               Description:   This function selects the source                                       of the signal controlling when the                                       counter can detected a signal.                                                              This feature allows the counter                                       to perform frequency measurements                                       at specific points in time.                                                              Arguments 0, 1, and 2 correspond                                       to the VXIbus ECLTRG0 line, VXIbus                                       ECLTRG1 line, and front panel,                                       respectively.                                                              Note: The Inhibit Input is active                                       high (i.e., the counter is                                       inhibited by a logic high signal                                       and enabled by a logic low                                       signal).  Parameter List     : Instrument ID                            Control Name:  Instrument ID                                                       Description:   Accepts the Instrument ID returned                                           by the Initialize function to                                           select the desired instrument.                                                       Variable Type: Integer                                                       Valid Range:   1  to  Maximum Instances of                                           this Instrument                                                                      The Maximum Instances of this                                           Instrument is the maximum number                                           instruments that can be controlled                                           by this driver in a single system.                                           The value is set internally in the                                           driver. The typical value is 10.                       Inhibit Input Source                           0 - VXI ECLTRG0-                            1 - VXI ECLTRG1-                            2 - Front Panel- (Default)                            Control Name:  Inhibit Input                                                       Description:   Selects the input to be used for                                           the measurement inhibit function.                                                       Variable Type: Integer                                                       Valid Range:   0 - VXIbus ECLTRG0 line                                           1 - VXIbus ECLTRG1 line                                           2 - Front Panel                                                                                  Default Value: 2 - Front Panel  Return Value       :  Control Name:  Error Code                                               Description:   Displays status relating to the                                       function call.                                               Variable Type: Integer                                               Error Codes:                                                      0   Success                                                     -1   Instrument ID out of range                              -2   Inhibit Input out of range                                                    230   Error writing to instrument                             231   Error reading from instrument                             232   Instrument not initialized________________________________________________________________________________  Function Name      : eip123x_init  C Syntax           : int eip123x_init(Logical_Address, Interface, ID_Query,                                Reset_Device, Instrument_ID, Model_Number);                       int Logical_Address;                       int Interface;                       int ID_Query;                       int Reset_Device;                       int *Instrument_ID;                       int *Model_Number;  QuickBASIC Syntax  : Error.Code% = eip123x.init(Logical.Address%, Interface%, _                                ID.Query%, Reset.Device%, Instrument.ID%, _                                Model.Number%)  Purpose            :  Panel Name:    Initialize                                               Description:   This routine performs the                                       following initialization actions:                                                              -  Opens the instrument by                                          storing information about the                                          instrument in the Instrument                                          Table.                                                              -  Initializes the VXI interface                                          (NI-VXI or NI-GPIB)                                          accordingly.                                                              -  Performs an identification                                          query on the Instrument to                                          determine the model number.                                                              -  Sends initialization commands                                          to the instrument.                                                              -  Returns an Instrument ID which                                          is used to differentiate                                          between instruments of the same                                          model type.  This value will be                                          used to identify the instrument                                          in subsequent calls.  Parameter List     : Logical Address                            Control Name:  Logical Address                                                       Description:   Specifies the Logical Address of                                           the VXI instrument.  This value                                           must corrrespond to the current                                           internal setting of the                                           instrument.                                                       Variable Type: Integer                                                       Valid Range:   0 - 255                                                       Default Value: 18                       Interface                           1 - NI-VXI (Default)                           2 - NI-GPIB                             Control Name:  Interface

Description:   Specifies which protocol is used                                           to communicate with the VXI                                           mainframe.                                                                      NI-VXI should be selected for                                           MC-MXI, AT-MXI, or VXIpc-386.                                                                      NI-GPIB should be selected for the                                           GPIB-VXI.                                                       Variable Type: Integer                                                       Valid Range:   1 - NI VXI                                           2 - NI GPIB                                                       Default Value: 1 - NI VXI                       ID Query                           1 - Do Query (Default)                           0 - Skip Query                             Control Name:  ID Query                                                       Description:   Specifies if an ID Query is sent                                           to the instrument during the                                           initialization procedure.                                                       Variable Type: Integer                                                       Valid Range:   1 - Do Query                                           0 - Skip Query                                                       Default Value: 1 - Do Query                                                       NOTE:  Under normal circumstances the ID Query                                   insures that the instrument initialized                                   is the type supported by this driver.                                   However circumstances may arise where it                                   is undesirable to send an ID Query to the                                   instrument.  In those cases; set this                                   control to "Skip Query" and this function                                   will initialize the selected interface and                                   the command arrays in the driver, without                                   doing an ID Query.                       Reset Device                           1 - Reset Device (Default)                           0 - Don't Reset                             Control Name:  Reset Device                                                       Description:   Specifies if the instrument is to                                           be reset to its power-on settings                                           during the initialization                                           procedure.                                                       Variable Type: Integer                                                       Valid Range:   1 - Reset Device                                           0 - Don't Reset                                                       Default Value: 1 - Reset Device                                                       NOTE: If you do not want the instrument reset.                                  Set this control to "Don't Reset" while                                  initializing the instrument.                       Instrument ID                            Control Name:  Instrument ID                                                       Description:   Returns an Instrument ID that is                                           used in all subsequent function                                           calls to select the instrument at                                           the address specified.                                                                      If more than one instrument of the                                           same model type is used, this ID                                           will be used to differentiate                                           between them. The first instrument                                           of this model type initialized                                           will be assigned an Instrument ID                                           of 1.                                                       Variable Type: Integer                       Model Number                            Control Name:  Model Number                                                       Description:   Returns the instrument model                                           number. If the user opted to                                           skip the ID query, the instrument                                           is assumed to be a 1230,                                           otherwise the value is determined                                           from the instrument.                                                       Variable Type: Integer  Return Value       :  Control Name:  Error Code                                               Description:   Displays status relating to the                                       function call.                                               Variable Type: Integer                                               Error Codes:                                                      0   Success                                                     -1   Logical Address out of range                              -2   Interface out of range                              -3   ID Query out of range                              -4   Reset Device out of range                                                    220   Unable to open instrument                             221   Unable to close instrument                             222   Unable to initialize VXI library                             223   Instrument identification query failed                             230   Error writing to instrument                             231   Error reading from instrument                             232   Instrument not initialized                             233   Error configuring GPIB address                             236   Error interpreting instrument response                             239   Error setting time out                             241   Error configuring repeat addressing________________________________________________________________________________  Function Name      : eip123x_min_freq_rep  C Syntax           : int eip123x_min_freq_rep(Instrument_ID,                                Min_Pulse_Rep_Freq, MinPRF_Units);                       int Instrument_ID;                       double Min_Pulse_Rep_Freq;                       int MinPRF_Units;  QuickBASIC Syntax  : Error.Code% = eip123x.min.freq.rep(Instrument.ID%, _                                Min.Pulse.Rep.Freq#, MinPRF.Units%)  Purpose            :  Panel Name:    Min Pulse Repetition:                                               Description:   This function controls the minimum                                       pulse repetition frequency of the                                       pulsed signals that can be                                       acquired and measured by the                                       counter. For example if a MINPRF                                       of 500 Hz is selected, the counter                                       will only measure signals with a                                       pulse repetition frequency of                                       500 Hz or greater.  Parameter List     : Instrument ID                            Control Name:  Instrument ID                                                       Description:   Accepts the Instrument ID returned                                           by the Initialize function to                                           select the desired instrument.                                                       Variable Type: Integer                                                       Valid Range:   1  to  Maximum Instances of                                           this Instrument                                                                      The Maximum Instances of this                                           Instrument is the maximum number                                           instruments that can be controlled                                           by this driver in a single system.                                           The value is set internally in the                                           driver. The typical value is 10.                       Min Pulse Rep Freq                            Control Name:  Min Pulse Repetition                                                       Description:   This parameter controls the                                           minimum pulse repetition frequency                                           of the pulsed signals that can be                                           acquired and measured by the                                           counter.  For example, if 500 Hz                                           is selected, the counter will                                           measure signals with a pulse                                           repetition frequency of 500 Hz or                                           greater.                                                                      This parameter affects the                                           acquisition speed indirectly via                                           two internal processes: the                                           waiting time for a pulse at each

frequency step in the frequency                                           range being searched, and the                                           waiting time for a pulse when                                           taking measurements before                                           declaring a "signal lost"                                           condition.                                                                      When Min Pulse Repetition                                           is set to a low value either                                           center frequency of frequency                                           limits to minimize signal                                           aquisition time.                                                                                  Variable Type: Real                                                       Valid Range:   1.0 Hz to 2.0 kHz                                                       Default Value: 2.0 kHz                       MinPRF Units                           0 - Hz -                            1 - kHz - (Default)                           2 - MHz -                            3 - GHz -                             Control Name:  Minimum PRF Units                                                       Description:   Selects the units for the                                           Minimum Pulse Repetition Frequency                                           specified by the Min PRF control.                                                       Valid Range:   0 - Hz                                           1 - kHz                                           2 - MHz                                           3 - GHz                                                       Default Value: 0 - Hz  Return Value       :  Control Name:  Error Code                                               Description:   Displays status relating to the                                       function call.                                               Variable Type: Integer                                               Error Codes:                                                      0   Success                                                     -1   Instrument ID out of range                              -2   Min Pulse Rep Freq out of range                              -3   MinPRF Units out of range                                                    230   Error writing to instrument                             231   Error reading from instrument                             232   Instrument not initialized________________________________________________________________________________  Function Name      : eip123x_read  C Syntax           : int eip123x_read(Instrument_ID, Number_Bytes_To_Read,                                Read_Buffer, NumBytes_Read);                       int Instrument_ID;                       int Number_Bytes_To_Read;                       char *Read_Buffer;                       long *NumBytes_Read;  QuickBASIC Syntax  : Error.Code% = eip123x.read(Instrument.ID%, _                                Number.Bytes.To.Read%, Read.Buffer$, _                                NumBytes.Read&)  Purpose            :  Panel Name:    Read Instrument Data                                               Description:   Reads data from instrument buffer                                       and returns it to the specified                                       variable in memory.                                               NOTES: (1) Because the instrument may return both                                   numeric and text data in response to                                   queries this function returns the data                                   in string format.                                                      (2) If valid data is not available at the                                   instrument's output buffer when this                                   function is called the instrument will                                   hang up and the function will not                                   return until it times out.  If the                                   time-out is disabled this function                                   will hang indefinitely and it may be                                   neccessary to reboot the computer to                                   break out.  Parameter List     : Instrument ID                            Control Name:  Instrument ID                                                       Description:   Accepts the Instrument ID returned                                           by the Initialize function to                                           select the desired instrument.                                                       Variable Type: Integer                                                       Valid Range:   1 to  Maximum Instances of                                                 this Instrument                                                                      The Maximum Instances of this                                           Instrument is the maximum number                                           of instruments that can be                                           controlled by this driver in a                                           single system. The value is set                                           internally in the driver. The                                           typical value is 10.                       Number Bytes To Read                            Control Name:  Number of Bytes to Read                                                       Description:   The number of bytes specified by                                           this control should be greater                                           than or equal to the number of                                           bytes which are to be read from                                           the instrument. If the actual                                           number of bytes to be read is                                           greater than the number this                                           control specifies then multiple                                           reads will be necessary to empty                                           the instrument's output buffer.                                                                      If the instrument's output buffer                                           is not emptied the instrument may                                           return invalid data when future                                           reads are performed.                                                       Variable Type: Integer                                                       Default Value: 50 (See NOTE 2)                                                       NOTES: (1) If NO DATA is available at the                                       instrument's output buffer when this                                       function is called the instrument will                                       hang up and the function will not                                       return until it times out.  If the                                       time-out is disabled, this function                                       will hang indefinitely and it may be                                       neccessary to reboot the computer.                                                              (2) If the number of bytes expected is                                       greater than 50 the value of this                                       control may be increased. If the                                       actual number of bytes read is larger                                       than the declared size of the read                                       buffer a run-time error will be                                       generated.                       Read Buffer                            Control Name:  Read Buffer                                                       Description:   The incoming data from the                                           instrument is placed into this                                           variable.                                                       Variable Type: String                                                       NOTES: (1) Because the Instrument may return both                                       numeric and text data in response to                                       queries this function returns the data                                       in string form.                                                              (2) This function does not overwrite any                                       old data left in the string variable                                       from the last time the function was                                       called. String data in LabWindows is                                       terminated with an ASCII null(Hex 0)                                       and string manipulation functions will                                       only recognize data before the ASCII                                       null.                                                              (3) The declared size of the string                                       variable must be greater than the                                       actual number of bytes read from the                                       instrument if it is not a run-time                                       error will be generated.                       NumBytes Read

Control Name:  Number of Bytes Read                                                       Description:   This variable contains the actual                                           number of bytes read from the                                           instrument. This is the value                                           which is returned by the read                                           function. The global variable                                           "ibcnt" holds the result of GPIB                                           reads and the global variable                                           "bytes" holds the result of VXI                                           word serial reads.                                                       Variable Type: Long                                                       NOTE: If the actual number of bytes read is less                                  than the number of bytes specified in                                  number of bytes to read then the output                                  buffer is probably empty. If the number of                                  bytes read is 0 and the instrument returns                                  a 231 error code the read function failed.                                  The most probable cause for the read to                                  fail is because there was no data available                                  at the instrument's output buffer.  Return Value       :  Control Name:  Error Code                                               Description:   Displays status relating to the                                       function call.                                               Variable Type: Integer                                               Error Codes:                                                      0   Success                                                     -1   Instrument ID out of range                                                    230   Error writing to instrument                             231   Error reading from instrument                             232   Instrument not initialized                             236   Error interpreting instrument response________________________________________________________________________________  Function Name      : eip123x_reset  C Syntax           : int eip123x_reset(Instrument_ID);                       int Instrument_ID;  QuickBASIC Syntax  : Error.Code% = eip123x.reset(Instrument.ID%)  Purpose            :  Panel Name:    Reset                                               Description:   This function resets the                                       instrument to its default state.  Parameter List     : Instrument ID                            Control Name:  Instrument ID                                                       Description:   Accepts the Instrument ID returned                                           by the Initialize function to                                           select the desired instrument.                                                       Variable Type: Integer                                                       Valid Range:   1 to  Maximum Instances of                                                 this Instrument                                                                      The Maximum Instances of this                                           Instrument is the maximum number                                           of instruments that can be                                           controlled by this driver in a                                           single system. The value is set                                           internally in the driver. The                                           typical value is 10.  Return Value       :  Control Name:  Error Code                                               Description:   Displays status relating to the                                       function call.                                               Variable Type: Integer                                               Error Codes:                                                      0   Success                                                     -1   Instrument ID out of range                                                    230   Error writing to instrument                             231   Error reading from instrument                             232   Instrument not initialized                             236   Error interpreting instrument response________________________________________________________________________________  Function Name      : eip123x_resolution  C Syntax           : int eip123x_resolution(Instrument_ID, Resolution);                       int Instrument_ID;                       int Resolution;  QuickBASIC Syntax  : Error.Code% = eip123x.resolution(Instrument.ID%, _                                Resolution%)  Purpose            :  Panel Name:    Resolution                                               Description:   This function controls the                                       frequency measurement resolution.                                       Select the desired resolution                                       according to the following table:                                                    Resolution    Freq        Gate Time                                                        2         100 Hz      10 ms                                                       (Band 0 only)                                 3         1 kHz       1 ms                                 4         10 kHz      100 ys                                 5         100 kHz     10 ys                                 6         1 MHz       1ys                                 7         10 MHz      1ys                                 8         100 MHz     1ys                                 9         1 GHz       1ys  Parameter List     : Instrument ID                            Control Name:  Instrument ID                                                       Description:   Accepts the Instrument ID returned                                           by the Initialize function to                                           select the desired instrument.                                                       Variable Type: Integer                                                       Valid Range:   1  to  Maximum Instances of                                           this Instrument                                                                      The Maximum Instances of this                                           Instrument is the maximum number                                           instruments that can be controlled                                           by this driver in a single system.                                           The value is set internally in the                                           driver. The typical value is 10.                       Resolution                           2 - 100 Hz -                            3 - 1 kHz - (Default)                           4 - 10 KHz -                            5 - 100 KHz -                            6 - 1 MHz -                            7 - 10 MHz -                            8 - 100 MHz -                            9 - 1 GHz -                             Control Name:  Resolution                                                       Description:   This parameter controls the                                           frequency measurement resolution.                                           Select the desired resolution                                           according to the following table:                                                            Resolution    Freq        Gate Time                                                                2         100 Hz      10 ms                                                           (Band 0 only)                                     3         1 kHz       1 ms                                     4         10 kHz      100 us                                     5         100 kHz     10 us                                     6         1 MHz       1 us                                     7         10 MHz      1 us                                     8         100 MHz     1 us                                     9         1 GHz       1 us                                                       Variable Type: Integer                                                       Valid Range:   2 - 100 Hz                                           3 - 1 kHz                                           4 - 10 kHz                                           5 - 100 kHz                                           6 - 1 MHz                                           7 - 10 MHz                                           8 - 100 MHz                                           9 - 1 GHz                                                       Default Value: 3 - 1 kHz  Return Value       :  Control Name:  Error Code                                               Description:   Displays status relating to the                                       function call.                                               Variable Type: Integer                                               Error Codes:                                                      0   Success                                                     -1   Instrument ID out of range                              -2   Resolution out of range                                                    230   Error writing to instrument

231   Error reading from instrument                             232   Instrument not initialized________________________________________________________________________________  Function Name      : eip123x_revision_query  C Syntax           : int eip123x_revision_query(Instrument_ID,                                Driver_Revision, Firmware_Revision);                       int Instrument_ID;                       char *Driver_Revision;                       char *Firmware_Revision;  QuickBASIC Syntax  : Error.Code% = eip123x.revision.query(Instrument.ID%, _                                Driver.Revision$, Firmware.Revision$)  Purpose            :  Panel Name:    Revision Query                                               Description:   This function returns the revision                                       numbers of the instrument driver                                       and instrument firmware, and tells                                       the user what firmware the driver                                       is compatible with, i.e., Driver                                       Revision "A1.0" is compatible                                       with Firmware Revision(s) "3.00".  Parameter List     : Instrument ID                            Control Name:  Instrument ID                                                       Description:   Accepts the Instrument ID returned                                           by the Initialize function to                                           select the desired instrument.                                                       Variable Type: Integer                                                       Valid Range:   1 to  Maximum Instances of                                                 this Instrument                                                                      The Maximum Instances of this                                           Instrument is the maximum number                                           of instruments that can be                                           controlled by this driver in a                                           single system. The value is set                                           internally in the driver. The                                           typical value is 10.                       Driver Revision                            Control Name:  Driver Revision                                                       Description:   This control returns the                                           Instrument Driver Revision.                                                       Variable Type: String (char *)                       Firmware Revision                            Control Name:  Firmware Revision                                                       Description:   This control returns the                                           Instrument's Firmware                                           Revision.                                                       Variable Type: String (char *)  Return Value       :  Control Name:  Error Code                                               Description:   Displays status relating to the                                       function call.                                               Variable Type: Integer                                               Error Codes:                                                      0   Success                                                     -1   Instrument ID out of range                                                    230   Error writing to instrument                             231   Error reading from instrument                             232   Instrument not initialized                             236   Error interpreting instrument response________________________________________________________________________________  Function Name      : eip123x_sample_rate  C Syntax           : int eip123x_sample_rate(Instrument_ID,                                Sample_Rate_mSec);                       int Instrument_ID;                       long Sample_Rate_mSec;  QuickBASIC Syntax  : Error.Code% = eip123x.sample.rate(Instrument.ID%, _                                Sample.Rate.mSec&)  Purpose            :  Panel Name:    Sample Rate                                               Description:   This function sets a delay between                                       measurement values (0 to 100 S                                       to a 10 mS resolution).  Signal                                       tracking is performed before each                                       measurement.  This process adds                                       approximately 100 ms to the delay                                       between.  This means that even if                                       the Sample Rate is set to 0 there                                       will still be a 100 ms delay                                       between measurements.                                                Parameter List     : Instrument ID                            Control Name:  Instrument ID                                                       Description:   Accepts the Instrument ID returned                                           by the Initialize function to                                           select the desired instrument.                                                       Variable Type: Integer                                                       Valid Range:   1  to  Maximum Instances of                                           this Instrument                                                                      The Maximum Instances of this                                           Instrument is the maximum number                                           instruments that can be controlled                                           by this driver in a single system.                                           The value is set internally in the                                           driver. The typical value is 10.                       Sample Rate (mSec)                            Control Name:  Sample Rate                                                       Description:   Sets a delay between measurement                                           values                                                       Variable Type: Long                                                       Valid Range:   0 to 100,000 ms, by 10 ms                                           resolution                                                       Default Value: 0 ms (maximum sampling rate)  Return Value       :  Control Name:  Error Code                                               Description:   Displays status relating to the                                       function call.                                               Variable Type: Integer                                               Error Codes:                                                      0   Success                                                     -1   Instrument ID out of range                              -2   Sample Rate out of range                                                    230   Error writing to instrument                             231   Error reading from instrument                             232   Instrument not initialized________________________________________________________________________________  Function Name      : eip123x_save_default_setup  C Syntax           : int eip123x_save_default_setup(Instrument_ID);                       int Instrument_ID;  QuickBASIC Syntax  : Error.Code% = eip123x.save.default.setup(Instrument.ID%)  Purpose            :  Panel Name:    Save Default Setup                                               Description:   The counter's setup is recalled                                       from Save/Recall register 0 at                                       power-up or upon receipt of the                                       the initialize command. The                                       register can be changed by the                                       user in order to alter the                                       instrument's default settings.                                       This function restores register                                       0 to the instrument's factory                                       default settings.                         Parameter List     : Instrument ID                            Control Name:  Instrument ID                                                       Description:   Accepts the Instrument ID returned                                           by the Initialize function to                                           select the desired instrument.                                                       Variable Type: Integer                                                       Valid Range:   1  to  Maximum Instances of                                           this Instrument                                                                      The Maximum Instances of this                                           Instrument is the maximum number                                           instruments that can be controlled                                           by this driver in a single system.                                           The value is set internally in the                                           driver. The typical value is 10.  Return Value       :  Control Name:  Error Code                                               Description:

Displays status relating to the                                       function call.                                               Variable Type: Integer                                               Error Codes:                                                      0   Success                                                     -1   Instrument ID out of range                              -2   Mode out of range                              -3   Setup Register out of range                                                    230   Error writing to instrument                             232   Instrument not initialized________________________________________________________________________________  Function Name      : eip123x_set_meas_type  C Syntax           : int eip123x_set_meas_type(Instrument_ID,                                Measurement_Type);                       int Instrument_ID;                       int Measurement_Type;  QuickBASIC Syntax  : Error.Code% = eip123x.set.meas.type(Instrument.ID%, _                                Measurement.Type%)  Purpose            :  Panel Name:    Set Measurement Type                                               Description:   This function selects which type                                       of measurement will be output                                       by the instrument when the Get                                       Measurement function is used:                                       Frequency, Width, or Period.                                       Also, use Set Measurement Type                                       before using the trigger                                       if a triggered mode is selected.  Parameter List     : Instrument ID                            Control Name:  Instrument ID                                                       Description:   Accepts the Instrument ID returned                                           by the Initialize function to                                           select the desired instrument.                                                       Variable Type: Integer                                                       Valid Range:   1  to  Maximum Instances of                                           this Instrument                                                                      The Maximum Instances of this                                           Instrument is the maximum number                                           instruments that can be controlled                                           by this driver in a single system.                                           The value is set internally in the                                           driver. The typical value is 10.                       Measurement Type                           0 - Frequency - (Default)                           1 - Width -                            2 - Period -                             Control Name:  Measurement                                                       Descriptor:    This parameter controls the                                           measurement to output.                                                       Variable Type: Integer                                                       Valid Range:   0 - Frequency                                           1 - Width                                           2 - Period                                                       Default Value: 0 - Frequency                             Return Value       :  Control Name:  Error Code                                               Description:   Displays status relating to the                                       function call.                                               Variable Type: Integer                                               Error Codes:                                                      0   Success                                                     -1   Instrument ID out of range                              -2   Measurement Type out of range                                                    230   Error writing to instrument                             232   Instrument not initialized________________________________________________________________________________  Function Name      : eip123x_special_funct  C Syntax           : int eip123x_special_funct(Instrument_ID, Select_Special,                                Returned_Reading);                       int Instrument_ID;                       int Select_Special;                       double *Returned_Reading;  QuickBASIC Syntax  : Error.Code% = eip123x.special.funct(Instrument.ID%, _                                Select.Special%, Returned.Reading#)  Purpose            :  Panel Name:    Special Functions                                               Description:   This function selects the special                                       function from the special                                       functions which this driver                                       supports.                                                              Call the Terminate all specials                                       function to terminate all active                                       special special functions.                                                              Functions supported by this driver                                       are:                                              Read Relative Frequency:                                       When this function is activated,                                       the counter assigns a negative                                       value to the last frequency                                       reading and enters it into the                                       frequency offset register                                       (overwriting any existing                                       frequency offset value). The last                                       input frequency in this case means                                       the actual frequency of the input                                       signal, not including the effects                                       of a frequency multiplier or other                                       special function. The counter                                       outputs the the relationship, i.e.                                       the difference, between the last                                       input frequency and the current                                       one, subject to any other                                       functions activated. It continues                                       to do so until the function is                                       terminated.                                              Display IF Readings:                                       When this function is activated,                                       the counter assigns a negative                                       value to the local oscillator (LO)                                       frequency and enters it into the                                       frequency offset register                                       (overwriting any existing                                       frequency offset value). The                                       counter then subtracts the LO                                       frequency from the input frequency                                       and outputs the resulting IF. It                                       continues to do so until the                                       function is terminated.                                              100 MHz Self-Test                                       When this function is activated,                                       the counter will count its                                       internal VCO.  This function is                                       use to verify that the  count                                       chain, gate generator, and the VCO                                       are operational.  Parameter List     : Instrument ID                            Control Name:  Instrument ID                                                       Description:   Accepts the Instrument ID returned                                           by the Initialize function to                                           select the desired instrument.                                                       Variable Type: Integer                                                       Valid Range:   1  to  Maximum Instances of                                           this Instrument                                                                      The Maximum Instances of this                                           Instrument is the maximum number                                           instruments that can be controlled                                           by this driver in a single system.                                           The value is set internally in the                                           driver. The typical value is 10.                       Select Special                           0 - Self Test (Default)                           1 - Relative Frq                            2 - Display IF                             Control Name:  Select Special                                                       Description:   Selects or terminates the Special                                           functions. Functions which are                                           supported are:

100 MHz Self-Test: When this                                           function is activated, the counter                                           counts an internally generated                                           test signal.  The counter returns                                           a frequency reading of 100 MHz to                                           indicate it passed Self-Test.                                                                      Read Relative Frequency: When this                                           function is activated, the counter                                           assigns a negative value to the                                           last frequency reading and enters                                           it into the frequency offset                                           register (overwriting any existing                                           frequency offset value). The last                                           input frequency in this case means                                           the actual frequency of the input                                           signal, not including the effects                                           of a frequency multiplier or other                                           special function. The counter                                           outputs the the relationship, i.e.                                           the difference, between the last                                           input frequency and the current                                           one,subject to any other functions                                           activated. It continues to do so                                           until the function is terminated.                                                                      Display IF Readings:                                           When this function is activated,                                           the counter assigns a negative                                           value to the local oscillator (LO)                                           frequency and enters it into the                                           frequency offset register                                           (overwriting any existing                                           frequency offset value). The                                           counter then subtracts the LO                                           frequency from the input frequency                                           and outputs the resulting IF. It                                           continues to do so until the                                           function is terminated.                                                       Variable Type: Integer                                                       Valid Range:   0 - 100 MHz Self-Test                                           1 - Read Relative Frequency                                           2 - Display IF Readings                                                       Default Value: 0 - 100 MHz Self-Test                       Returned Reading                            Control Name:  Returned Reading                                                       Description:   Outputs the Self-Test, Relative                                           or IF frequency.                                                       Variable type: Double  Return Value       :  Control Name:  Error Code                                               Description:   Displays status relating to the                                       function call.                                               Variable Type: Integer                                               Error Codes:                                                      0   Success                                                     -1   Instrument ID out of range                              -2   Select Special out of range                                                    230   Error writing to instrument                             231   Error reading from instrument                             232   Instrument not initialized                             236   Error interpreting instrument response________________________________________________________________________________  Function Name      : eip123x_terminate_spec  C Syntax           : int eip123x_terminate_spec(Instrument_ID);                       int Instrument_ID;  QuickBASIC Syntax  : Error.Code% = eip123x.terminate.spec(Instrument.ID%)  Purpose            :  Panel Name:    Terminate All Specials                                               Description:   This Function terminates all                                       previously implemented special                                       functions.  Parameter List     : Instrument ID                            Control Name:  Instrument ID                                                       Description:   Accepts the Instrument ID returned                                           by the Initialize function to                                           select the desired instrument.                                                       Variable Type: Integer                                                       Valid Range:   1  to  Maximum Instances of                                           this Instrument                                                                      The Maximum Instances of this                                           Instrument is the maximum number                                           instruments that can be controlled                                           by this driver in a single system.                                           The value is set internally in the                                           driver. The typical value is 10.  Return Value       :  Control Name:  Error Code                                               Description:   Displays status relating to the                                       function call.                                               Variable Type: Integer                                               Error Codes:                                                      0   Success                                                     -1   Instrument ID out of range                                                    230   Error writing to instrument                             231   Error reading from instrument                             232   Instrument not initialized________________________________________________________________________________  Function Name      : eip123x_time_base  C Syntax           : int eip123x_time_base(Instrument_ID, Time_Base_Source);                       int Instrument_ID;                       int Time_Base_Source;  QuickBASIC Syntax  : Error.Code% = eip123x.time.base(Instrument.ID%, _                                Time.Base.Source%)  Purpose            :  Panel Name:    Time Base Source                                               Description:   This function selects the source                                       of the 10 MHz frequency reference                                       signal.  Parameter List     : Instrument ID                            Control Name:  Instrument ID                                                       Description:   Accepts the Instrument ID returned                                           by the Initialize function to                                           select the desired instrument.                                                       Variable Type: Integer                                                       Valid Range:   1  to  Maximum Instances of                                           this Instrument                                                                      The Maximum Instances of this                                           Instrument is the maximum number                                           instruments that can be controlled                                           by this driver in a single system.                                           The value is set internally in the                                           driver. The typical value is 10.                       Time Base Source                           1 - External                            0 - Internal (Default)                            Control Name:  Time Base Source                                                       Description:   Controls the source of the                                           10 MHz frequency reference                                           signal.                                                                                  Variable Type: Integer                                                       Valid Range:   0 - Internal                                           1 - External                                                       Default Value: 0 - Internal  Return Value       :  Control Name:  Error Code                                               Description:   Displays status relating to the                                       function call.                                               Variable Type: Integer                                               Error Codes:                                                      0   Success                                                     -1   Instrument ID out of range                              -2   Time Base Source out of range                                                    230   Error writing to instrument

231   Error reading from instrument                             232   Instrument not initialized________________________________________________________________________________  Function Name      : eip123x_trig_setup  C Syntax           : int eip123x_trig_setup(Instrument_ID, Sync_Trig_Line,                                Async_Trig_Line);                       int Instrument_ID;                       int Sync_Trig_Line;                       int Async_Trig_Line;  QuickBASIC Syntax  : Error.Code% = eip123x.trig.setup(Instrument.ID%, _                                Sync.Trig.Line%, Async.Trig.Line%)  Purpose            :  Panel Name:    Trigger Set Up                                               Description:   This function selects the                                       triggering mode, configures                                       the INT/EXT time base reference,                                       and selects the TTLTRG line pair                                       to be used in the Asynchronous                                       Trigger Protocol. The valid range                                       is 0 to 3, corresponding to TTLTRG                                       pairs 0 & 1, 2 & 3, 4 & 5, and                                       6 & 7 respectively. Issuing                                       ASYNC 9 disables the protocol.                                       Or it selects the TTLTRG line                                       to be used in the Synchronous                                       Trigger Protocol. the valid range                                       is 0 to 7. Issuing SYNC 9 disables                                       the protocol.  Parameter List     : Instrument ID                            Control Name:  Instrument ID                                                       Description:   Accepts the Instrument ID returned                                           by the Initialize function to                                           select the desired instrument.                                                       Variable Type: Integer                                                       Valid Range:   1  to  Maximum Instances of                                           this Instrument                                                                      The Maximum Instances of this                                           Instrument is the maximum number                                           instruments that can be controlled                                           by this driver in a single system.                                           The value is set internally in the                                           driver. The typical value is 10.                       Sync Trig Line                           0 - 0 -                            1 - 1 -                            2 - 2 -                            3 - 3 -                            4 - 4 -                            5 - 5 -                            6 - 6 -                            7 - 7 -                            9 - Disabled - (Default)                            Control Name:  Sync Trig Line                                                       Description:   Selects the TTL TRG line to be                                           used in the Synchronous Trigger                                           Protocol.  The SYNC message                                           enables the SYNCHRONOUS TTLRG                                           trigger protocol                                                       Variable Type: Integer                                                       Valid Range:   0 - Line 0                                           1 - Line 1                                           2 - Line 2                                           3 - Line 3                                           4 - Line 4                                           5 - Line 5                                           6 - Line 6                                           7 - Line 7                                           9 - Protocol Disabled                                                       Default Value: 9 - Protocol Disabled                       Async Trig Line                           0 - 0 & 1 -                            1 - 2 & 3 -                            2 - 4 & 5 -                            3 - 6 & 7 -                            9 - Disabled - (Default)                            Control Name:  Async Trig Line                                                       Description:   Selects the TTL TRG line pair to                                           be used in the Asynchroneous                                           Trigger Protocol as follows:                                                                      0: 0 & 1                                           1: 2 & 3                                           2: 4 & 5                                           3: 6 & 7                                                       Variable Type: Integer                                                       Valid Range:   0 - Line pair 0 & 1                                           1 - Line pair 2 & 3                                           2 - Line pair 4 & 5                                           3 - Line pair 6 & 7                                           9 - Protocol Disabled                                                       Default Value: 9 - Protocol Disabled  Return Value       :  Control Name:  Error Code                                               Description:   Displays status relating to the                                       function call.                                               Variable Type: Integer                                               Error Codes:                                                      0   Success                                                     -1   Instrument ID out of range                              -2   Sync Trig Line out of range                              -3   Async Trig Line out of range                                                    230   Error writing to instrument                             231   Error reading from instrument                             232   Instrument not initialized________________________________________________________________________________  Function Name      : eip123x_trigger_func  C Syntax           : int eip123x_trigger_func(Instrument_ID, Function);                       int Instrument_ID;                       int Function;  QuickBASIC Syntax  : Error.Code% = eip123x.trigger.func(Instrument.ID%, _                                Function%)  Purpose            :  Panel Name:    Trigger Function                                               Description:   This function selects and                                       implements one of the two internal                                       trigger functions, trigger or                                       reset.                                                              Trigger triggers a new                                       measurement cycle. If a                                       measurement cycle is in progress                                       it will be aborted.                                                              Reset resets                                       the converter and restarts the                                       measurement process. If a                                       measurement is found, a                                       measurement will be taken, even if                                       the counter is in Hold.  Parameter List     : Instrument ID                            Control Name:  Instrument ID                                                       Description:   Accepts the Instrument ID returned                                           by the Initialize function to                                           select the desired instrument.                                                       Variable Type: Integer                                                       Valid Range:   1  to  Maximum Instances of                                           this Instrument                                                                      The Maximum Instances of this                                           Instrument is the maximum number                                           instruments that can be controlled                                           by this driver in a single system.                                           The value is set internally in the                                           driver. The typical value is 10.                       Function                           1 - Trigger                            0 - Reset (Default)                            Control Name:  Function                                                       Description:                                           1. Reset: Resets the converter and                                              restarts the signal acquisition                                              process.  If a signal is found,                                              a measurement will be taken,                                              even if the counter is in Hold.                                                                      2. Trigger: Triggers a new                                              measurement cycle, If a                                              measurement cycle is in                                              progress,

it will be aborted.                                                       Variable Type: Integer                                                       Valid Range:   1 - Trigger                                           0 - Reset                                                       Default Value: 0 - Reset  Return Value       :  Control Name:  Error Code                                               Description:   Displays status relating to the                                       function call.                                               Variable Type: Integer                                               Error Codes:                                                      0   Success                                                     -1   Instrument ID out of range                              -2   Function out of range                                                    230   Error writing to instrument                             232   Instrument not initialized________________________________________________________________________________  Function Name      : eip123x_write  C Syntax           : int eip123x_write(Instrument_ID, Write_Buffer);                       int Instrument_ID;                       char *Write_Buffer;  QuickBASIC Syntax  : Error.Code% = eip123x.write(Instrument.ID%, _                                Write.Buffer$)  Purpose            :  Panel Name:    Write To Instrument                                               Description:   Writes commands and queries to the                                       instrument to modify parameters                                       and query device settings.  Parameter List     : Instrument ID                            Control Name:  Instrument ID                                                       Description:   Accepts the Instrument ID returned                                           by the Initialize function to                                           select the desired instrument.                                                       Variable Type: Integer                                                       Valid Range:   1 to  Maximum Instances of                                                 this Instrument                                                                      The Maximum Instances of this                                           Instrument is the maximum number                                           of instruments that can be                                           controlled by this driver in a                                           single system. The value is set                                           internally in the driver. The                                           typical value is 10.                       Write Buffer                            Control Name:  Write Buffer                                                       Description:   The user can use this control to                                           send common commands and queries                                           to the instrument. This control                                           can also be used to write any                                           valid command to the instrument.                                                       Variable Type: String                                                       Default Value: "*RST"                                                       NOTES: (1) The command or query to be sent to the                                       instrument may be a literal enclosed                                       in double quotes i.e. "*RST" or may be                                       contained in a variable of type                                       string.                                                              (2) If the number of bytes to be written                                       is greater than the size of "cmd" in                                       the source code then the size of cmd                                       must be increased. The original size                                       of cmd is 50 bytes (cmd[50]) the                                       largest cmd may be declared as is                                       32,767 bytes.  Return Value       :  Control Name:  Error Code                                               Description:   Displays the results of the                                       function call.                                               Variable Type: Integer                                               Error Codes:                                                      0   Success                                                     -1   Instrument ID out of range                                                    230   Error writing to instrument                             232   Instrument not initialized