The protocol between Power supply and PC

Power supply can be connected to RS-232 interface by the DB9 plug on the back panel through the level switching circuit. The following instructions can help you to understand how to control the output of power supply by PC.

- RS-232 Communication Set

You can set the baud rate and address of the power supply by the MENU on the LCD panel.

  - Address: (0—31)

  - Baud rate: 9600 (4800, 9600, 19200, 38400)

  - Data bit: 8

  - Stop bit: 1

  - Verify: None

- DB9 Serial interface

The output of DB9 interface on the back panel is TTL, and you need to level switching it by the auxiliary 3311 to transfer and connect it to the PC Com port.

Power          Level Converter              3110         PC

- Frame format

The length of the frame is 26 bytes (compatible with FAB). The form is as the following:

| AAH | Address | Command | Byte 4 to 25 are relevant information | Check |
| --- | --- | --- | --- | --- |

Directions:

- AAH occupies a byte

- Address (0~31) occupies a byte

- Command occupies a byte. And the contents are as the following:

  - 80H------Set the upper limit of current, power and the voltage level.

  - 81H------Read the current, voltage, power and the current status. The statuses include ON/OFF, over current and overpower status of the power supply.

  - 82H------Control the ON/OFF of the power supply

  - 83H------Set the protection of power supply

  - 84H------Read the protection status of power supply

  - 85H------Demarcate the power command

  - 86H------Return the actual output voltage to power supply

  - 87H------Demarcate the current command

  - 88H------Return the actual output voltage to power supply

  - 89H------Set the demarcating information of power supply

  - 8AH------Read the demarcating information

  - 8BH------Set the serial number of power supply

  - 8CH-----Read the serial number, product model and software version of power supply

  - 12H-------Check

- If you want to control the output of the power supply by PC, you have to set the power supply as PC controlled status. And the command is 82H. If you want to demarcate the output of the power supply and set the demarcating information and serial number of the power supply, you have to set the protection status as OFF first.

- The 26th byte is check. It’s the sum of the previous twenty-five bytes.

- The instructions of the Command list

1．Set Max current, Max power and voltage levgel (80H)

| Byte 1 | AAH |
| --- | --- |
| Byte 2 | Addresses (0—31) |
| Byte 3 | Command (80H) |
| Byte 4 | Low byte of the Max current |
| Byte 5 | High byte of the Max current |
| Byte 6 | Low byte of the low word of the Max voltage |
| Byte 7 | High byte of the low word of the Max voltage |
| Byte 8 | Low byte of the high word of the Max voltage |
| Byte 9 | High byte of the high word of the Max voltage |
| Byte 10 | Low byte of the Max power |
| Byte 11 | High byte of the Max power |
| Byte 12 | Low byte of the low word of the voltage set |
| Byte 13 | High byte of the low word of the voltage set |
| Byte 14 | Low byte of the high word of the voltage set |
| Byte 15 | High byte of the high word of the voltage set |
| Byte 16 | New address of the power supply |
| Byte 17~25 | Keep for the system |
| Byte 26 | Check |

The Current, Voltage and Power are all expressed in two bytes, with low byte in the front and the high byte behind.

For example: The current value 3589H is showed as

| 89H | 35H |
| --- | --- |

The range of current set：0-3000mA

The range of voltage set：0-36000mV

The range of power set: 0-108 (Enlarge 100 times)

2．Read the current, voltage, power value and the status of the power supply (81H)

| Byte 1 | AAH |
| --- | --- |
| Byte 2 | Address (0—31) |
| Byte 3 | Command（81H） |
| Byte 4 | Low byte of current |
| Byte 5 | High byte of current |
| Byte 6 | Low byte of the low word of the voltage |
| Byte 7 | High byte of the low word of the voltage |
| Byte 8 | Low byte of the high word of the voltage |
| Byte 9 | High byte of the high word of the voltage |
| Byte 10 | Low byte of power |
| Byte 11 | High byte of power |
| Byte 12 | Low byte of Max current |
| Byte 13 | High byte of Max current |
| Byte 14 | Low byte of the low word of the Max voltage |
| Byte 15 | High byte of the low word of the Max voltage |
| Byte 16 | Low byte of the High word of the Max voltage |
| Byte 17 | High byte of the High word of the Max voltage |
| Byte 18 | Low byte of Max power |
| Byte 19 | High byte of Max power |
| Byte 20 | Low byte of the low word of the voltage set |
| Byte 21 | High byte of the low word of the voltage set |
| Byte 22 | Low byte of the High word of the voltage set |
| Byte 23 | High byte of the High word of the voltage set |
| Byte 24 | Power supply status |
| Byte 25 | Keep for the system |
| Byte 26 | Checking code |

The current, voltage and power are all expressed in two bytes, with low byte in the front and high byte behind

Power supply status occupies one byte, each bit is defined as:

From high to low

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Bit 0：Power supply status，0 for OFF，1 for ON

Bit 1：Over current status，0 for Normal，1 for abnormal

Bit 2：Over power status，0 for Normal，1 for abnormal

Bit 3：Operation status，0 for Keyboard，1 for PC

Notes: The frame format from Power supply to PC is as the above.

3．Control the ON/OFF of the power supply (82H)

| Byte 1 | AAH |
| --- | --- |
| Byte 2 | Address (0—31) |
| Byte 3 | Command (82H) |
| Byte 4 | The status of the power supply |
| Byte 5~25 | Keep for system |
| Byte 266 | Check6 |

The status of the power supply is expressed in one byte. Each bit is defined as the following:

From high to low

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Bit 0： The status of the power supply，0 for OFF，1 for ON

Bit 1： The power supply controlled by PC，0 for Self-controlling of the power itself，1 for PC controlling the power

Explanation：Only under the situation of PC controlling, you can set the parameters of the power.

Power supply automatically upper-transmits the max current, the max power and the voltage levels to the PC.

The frame is the same as frame of the setup of the max current, the max power and the voltage levels.

4. To set the power calibration protection state (83H)

| Byte 1 | Lock head (AAH) |
| --- | --- |
| Byte 2 | Power Address (0-31) |
| Byte 3 | Command word（83H） |
| Byte 4 | Power calibration protection state |
| Byte 5 | Calibration password (0X28H) |
| Byte 6 | Calibration password (0X01H) |
| Byte 7 to Byte 25 | Preserved by the system |
| Byte 26 | Check code |

Calibration protection state is expressed by one byte. The definition of each bit unit is as the following:

From the high to the low

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Bit 0: protection state, 0 for protection making the ability and 1 for protection out of protection.

5. To read the power calibration protection state (84H)

| Byte 1 | Lock head (AAH) |
| --- | --- |
| Byte 2 | Power Address (0-31) |
| Byte 3 | Command Word (84H) |
| Byte 4 | Power calibration protection state |
| Byte 5 to Byte 25 | Preserved by the system |
| Byte 26 | Check code |

Calibration protection state is expressed by one byte. The definition of each bit unit is as the following:

From the high to the low

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Bit 0: protection state, 0 for protection making the ability and 1 for protection out of protection

6. To calibrate the voltage of the power (85H)

| Byte 1 | Lock head (AAH) |
| --- | --- |
| Byte 2 | Power Address (0-31) |
| Byte 3 | Command word (85H) |
| Byte 4 | Voltage calibration point (1~4) |
| Byte 5 to Byte 25 | Preserved by the system |
| Byte 26 | Check code |

7. To return the power supply the current actual output voltage (86H)

| Byte 1 | Lock head (AAH) |
| --- | --- |
| Byte 2 | Power Address (0-31) |
| Byte 3 | Command word (86H) |
| Byte 4 | Low byte of the front byte of the actual voltage |
| Byte 5 | High byte of the front byte of the actual voltage |
| Byte 6 | Low byte of the back byte of the actual voltage |
| Byte 7 | High byte of the back byte of the actual voltage |
| Byte 8 to Byte 25 | Preserved by the system |
| Byte 26 | Check code |

8. To calibrate the current of the power (87H)

| Byte 1 | Lock head (AAH) |
| --- | --- |
| Byte 2 | Power Address (0-31) |
| Byte 3 | Command word (87H) |
| Byte 4 | Current calibration point (1~2) |
| Byte 5 to Byte 25 | Preserved by the system |
| Byte 26 | Check code |

9. To return the power supply the current actual output current (88H)

| Byte 1 | Lock head (AAH) |
| --- | --- |
| Byte 2 | Power Address (0-31) |
| Byte 3 | Command word (88H) |
| Byte 4 | Low byte of the actual current |
| Byte 5 | High byte of the actual current |
| Byte 5 to Byte 25 | Preserved by the system |
| Byte 26 | Check code |

10. To set the calibration information of the power (89H)

| Byte 1 | Lock head (AAH) |
| --- | --- |
| Byte 2 | Power Address (0-31) |
| Byte 3 | Command word (89H) |
| Byte 4 to byte 23 | Calibration information (ASCII Code) |
| Byte 24 | Preserved by the system |
| Byte 25 | Preserved by the system |
| Byte 26 | Check code |

11. To read the calibration information of the power (8AH)

| Byte 1 | Lock head (AAH) |
| --- | --- |
| Byte 2 | Power Address (0-31) |
| Byte 3 | Command Word (8AH) |
| Byte 4 to Byte 23 | Calibration Information (ASCII Code) |
| Byte 24 | Preserved by the system |
| Byte 25 | Preserved by the system |
| Byte 26 | Check code |

12. To read the product serial No, product type and software version No of the power (8CH)

| Byte 1 | Lock head (AAH) |
| --- | --- |
| Byte 2 | Power Address (0-31) |
| Byte 3 | Command word (8CH) |
| Byte 4 to Byte 9 | Product serial No (ASCII Code) |
| Byte 10 to Byte 14 | Product type (ASCII Code) |
| Byte 15 | Low byte of the software version |
| Byte 16 | High byte of the software version |
| Byte 16 to Byte 25 | Preserved by the system |
| Byte 26 | Check code |

Press the 1 key while starting the machine, then the LCD will display the product serial No, product type and software version No of the power.

For example:

If the product serial No is 000045, the product type is 3645A and the software version No is V2.03, the data the power returning is as the following:

| AA | 00 | 30 | 30 | 30 | 30 | 30 | 34 | 35 | 33 | 36 | 35 | 41 | CB | 00 | XX | XX | XX | XX | XX | XX | XX | XX | XX | XX | 57 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

13. Check command (12H)

| Byte 1 | Lock head (AAH) |
| --- | --- |
| Byte 2 | Power Address (0-31) |
| Byte 3 | Command word (12H) |
| Byte 4 | 正确为80H，错误为90H |
| Byte 5 to byte 25 | Preserved by the system |
| Byte 26 | Check code |

When the power receives a frame of set command, it will check this frame of command and return the relative checked result.

When the power receives a frame of reading command, it will check this frame of command. If it checks correctly, it will return the relative read data. And if it checks wrongly, it will return the check command (90H).

Examples:

1. Set the parameters:

3000mA,36000mV,10800mW(108W) ,3000mV

AA 00 80 B8 0B A0 8C 00 00 30 2A B8 0B 00 00 00 00 00 00 00 00 00 00 00 00 36

2. Read the parameters:

AA 00 81 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 2B

3. Set control status:

A: PC control, output ON

AA 00 82 03 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 2F

B: PC control, output OFF

AA 00 82 02 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 2E

4.Self-controlled

### AA 00 82 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 2C

# Power Calibration

1. Structure of the system

- Procedure of calibration

  - To make the power calibration mode be out of ability

| AA | 00 | 83 | 01 | 28 | 01 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 57 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - Load being constant current mode and the output being OFF

  - To calibrate the voltage of the first point

| AA | 00 | 85 | 01 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 30 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - To wait the outputs of the power to be stable and return to the power the current actual testing voltage value

| AA | 00 | 86 | XX | XX | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | XX |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - To calibrate the voltage of the second point

| AA | 00 | 85 | 02 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - To wait the outputs of the power to be stable and return to the power the current actual testing voltage value

| AA | 00 | 86 | XX | XX | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | XX |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - To calibrate the voltage of the third point

| AA | 00 | 85 | 03 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - To wait the outputs of the power to be stable and return to the power the current actual testing voltage value

| AA | 00 | 86 | XX | XX | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | XX |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - To calibrate the voltage of the fourth point

| AA | 00 | 85 | 04 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - To wait the outputs of the power to be stable and return to the power the current actual testing voltage value

| AA | 00 | 86 | XX | XX | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | XX |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - To make the load be short circuit

  - To calibrate the current of the first point

| AA | 00 | 87 | 01 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - To wait the outputs of the power to be stable and return to the power the current actual testing voltage value

| AA | 00 | 88 | XX | XX | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | XX |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - To calibrate the current of the second point

| AA | 00 | 87 | 02 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - To wait the outputs of the power to be stable and return to the power the current actual testing voltage value

| AA | 00 | 88 | XX | XX | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | XX |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - To make the power calibration protection mode be ability

| AA | 00 | 83 | 00 | 28 | 01 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 56 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

q.	To finish the power calibration

Example Program 1

Explanation:

The following program is edited and passed in Delphi5.0;

TComm32 control file can be downloaded from www.array.com;

The demonstration program can be downloaded from www.array.com;

The other programming tool is the similar;

unit Main;

interface

uses

Windows, Messages, SysUtils, Classes, Graphics, Controls, Forms, Dialogs,

StdCtrls, Comm32, ExtCtrls;

//To definite the constant

const

POWER_ADDRESS  = $00; //Power Address

ORDER_WRITE    = $80; //To set order

ORDER_READ     = $81; //To read the parameter

ORDER_CONTROL  = $82; //Control order

PC_CONTROL     = $02; //PC controlling

SELF_CONTROL   = $00; //Power self-control

POWER_ON       = $03; //Open the output

POWER_OFF      = $02; //Close the output

type

TForm1 = class(TForm)

Comm232: TComm32;

cbCOMM: TComboBox;

cbBaud: TComboBox;

Label1: TLabel;

Label2: TLabel;

btnOpen: TButton;

Memo1: TMemo;

cbOrder: TComboBox;

btnSend: TButton;

Bevel1: TBevel;

procedure btnOpenClick(Sender: TObject);

procedure FormCreate(Sender: TObject);

procedure cbOrderClick(Sender: TObject);

procedure btnSendClick(Sender: TObject);

procedure Comm232RequestHangup(Sender: TObject);

procedure Comm232ReceiveData(Buffer: Pointer; BufferLength: Word);

private

{ Private declarations }

SendBuf:array[0..25] of Byte;   //Sending data buffer definition

ReceBuf:array[0..25] of Byte;   //Receiving data buffer fefinition

Order   :Byte;              //Order

AddOrder:Byte;              //Attached order

procedure TotalBytes;         //Counting the checking sum

procedure ShowSendBuf;      //Displaying the sending data

procedure ShowReceBuf;      //Displaying the receiving data

public

{ Public declarations }

end;

var

Form1: TForm1;

implementation

{$R *.DFM}

//Event of Opening the COM

procedure TForm1.btnOpenClick(Sender: TObject);

begin

Comm232.StopComm;

Comm232.CommPort:=cbComm.Text;

Comm232.BaudRate:=StrToInt(cbBaud.Text);

Comm232.ByteSize:=8;  //8-digit data bit

Comm232.Parity:=0;    //Non-checking

Comm232.StopBits:=0;  //Bit 1; the stopping bit

try

Comm232.StartComm;

btnSend.Enabled:=True;

except

ShowMessage(Format('Falied to open %s',[cbComm.Text]));

btnSend.Enabled:=False;

end;

end;

//To initialize the parameter

procedure TForm1.FormCreate(Sender: TObject);

begin

cbOrder.ItemIndex:=0;

Order:=Order_Read;

end;

//To select the order

procedure TForm1.cbOrderClick(Sender: TObject);

begin

case cbOrder.ItemIndex of

0 :  Order:=Order_Read;     //Read Params

1 :  Order:=Order_Write;    //Params Setting

2 :  begin        //PC Control

Order:=Order_Control;

AddOrder:=Pc_Control;

end;

3 :  begin        //Control By Self

Order:=Order_Control;

AddOrder:=Self_Control;

end;

4 :  begin        //Power Off

Order:=Order_Control;

AddOrder:=Power_Off;

end;

5 :  begin        //Power On

Order:=Order_Control;

AddOrder:=Power_On;

end;

end;

end;

//Event of sending data

procedure TForm1.btnSendClick(Sender: TObject);

var

CurrentMax:Word; //The max current (proportion coefficient being 1000)

VoltageMax:Word; // The max voltage (proportion coefficient being 1000)

PowerMax  :Word; // The max power (proportion coefficient being 1000)

CurVoltage:Word; // The current voltage value (proportion coefficient being 1000)

begin

CurrentMax:=3000;  //3A

VoltageMax:=36000; //36V

PowerMax  :=10800; //108W

CurVoltage:=10000; //10V

FillChar(SendBuf,26,0);

SendBuf[0]:=$AA;

SendBuf[1]:=Power_Address;

SendBuf[2]:=Order;

if Order = Order_Write then //To set the parameter

begin

SendBuf[3]:=CurrentMax mod 256;

SendBuf[4]:=CurrentMax div 256;

SendBuf[5]:=VoltageMax mod 256;

SendBuf[6]:=VoltageMax div 256;

SendBuf[7]:=PowerMax mod 256;

SendBuf[8]:=PowerMax div 256;

SendBuf[9]:=CurVoltage mod 256;

SendBuf[10]:=CurVoltage div 256;

SendBuf[11]:=Power_Address;

end;

if Order = Order_Control then

SendBuf[3]:=AddOrder;

TotalBytes;

ShowSendBuf;

Comm232.WriteCommData(@SendBuf,26);

end;

//Event of hanging the COM port

procedure TForm1.Comm232RequestHangup(Sender: TObject);

begin

Comm232.StopComm;

Comm232.StartComm;

end;

//Event of receiving data

procedure TForm1.Comm232ReceiveData(Buffer: Pointer; BufferLength: Word);

var

i:Byte;

Byte25:Byte;

begin

if BufferLength <> 26 then Exit;

CopyMemory(@ReceBuf,Buffer,26);

if ReceBuf[0] <> $AA then Exit;

if not (ReceBuf[2] in [Order_Write..Order_Control]) then Exit;

Byte25:=0;

for i:=0 to 24 do

Byte25:=Byte25+ReceBuf[i];

if Byte25 <> ReceBuf[25] then Exit;

ShowReceBuf;

end;

//To count the checking sum

procedure TForm1.TotalBytes;

var

i:Byte;

begin

SendBuf[25]:=0;

for i:=0 to 24 do

SendBuf[25]:=SendBuf[25]+SendBuf[i];

end;

//To display the sending data

procedure TForm1.ShowSendBuf;

var

i:Byte;

Str:String;

begin

for i:=0 to 25 do

Str:=Str+' '+IntToHex(SendBuf[i],2);

Memo1.Lines.Add('Send :'+Str);

end;

//To display the receiving data

procedure TForm1.ShowReceBuf;

var

i:Byte;

Str:String;

begin

for i:=0 to 25 do

Str:=Str+' '+IntToHex(ReceBuf[i],2);

Memo1.Lines.Add('Rece :'+Str);

end;

end.

Example program 2

Explanation:

1. The following program is edited and passed in VC6.0;

2. The demonstration program can be downloaded from www.array.com;

Procedure:

To definite the variable and the function:

public:

BYTE Cur_Order;       //Command word

BYTE Add_Order;      //Attached command word

int  Rece_Count;       //The total sum of the received characters

CByteArray SendBuf;    //To sending the buffer

CByteArray ReceBuf;    //To receiving the buffer

void InitData();         //To buffer storage the initial data

void CalDataTotal();     //To account the checking sum

void ShowSendData();   //To display the sending data

void ShowReceData();   //To display the receiving data

To definite the constant:

const BufferMax          = 26;  //The max data buffer

const POWER_ADDRESS  = 0x00; //Power address

const ORDER_WRITE     = 0x80; //To set the order

const ORDER_READ      = 0x81; //To read the parameter

const ORDER_CONTROL  = 0x82; //The control order

const PC_CONTROL      = 0x02; //PC controlling

const SELF_CONTROL    = 0x00; //Powe self-controlling

const POWER_ON        = 0x03; //To open the output

const POWER_OFF       = 0x02; //To close the output

3. Function Part:

3．1//To account the checking sum

void CCommDlg::CalDataTotal()

{

BYTE i;

BYTE Value1;

Value1=0;

for (i=0;i<=BufferMax-2;i++)

{

Value1=Value1+SendBuf.GetAt(i);

}

SendBuf.SetAt(BufferMax-1,Value1);

}

3．2 //To initialize the dada buffer

void CCommDlg::InitData()

{

Rece_Count=0;

SendBuf.SetSize(26);

ReceBuf.SetSize(26);

SendBuf.RemoveAll();

ReceBuf.RemoveAll();

for (BYTE i=0;i<=BufferMax-1;i++)

{

SendBuf.Add(0);

ReceBuf.Add(0);

}

SendBuf.SetAt(0,0xAA);

SendBuf.SetAt(1,POWER_ADDRESS);

SendBuf.SetAt(2,ORDER_READ);

Cur_Order=ORDER_CONTROL;

Add_Order=POWER_OFF;

CalDataTotal();

ShowSendData();

}

3．3//To display the sending data

void CCommDlg::ShowSendData()

{

BYTE i;

BYTE Value;

CString Temp;

m_SendData="";

for (i=0;i<=BufferMax-1;i++)

{

Value=SendBuf.GetAt(i);

Temp.Format("%2x",Value);

if (Value < 16)

Temp.SetAt(0,'0');

m_SendData+=Temp;

m_SendData+=" ";

}

m_SendData.MakeUpper();

UpdateData(FALSE);

}

3．4//To display the receiving data

void CCommDlg::ShowReceData()

{

BYTE i;

BYTE Value;

CString Temp;

for (i=0;i<=BufferMax-1;i++)

{

Value=ReceBuf.GetAt(i);

Temp.Format("%2x",Value);

if (Value < 16)

Temp.SetAt(0,'0');

m_ReceiveData+=Temp;

m_ReceiveData+=" ";

}

m_ReceiveData+="\r\n";

m_ReceiveData.MakeUpper();

UpdateData(FALSE);

}

3．5//To convert the characters into hexadecimal number

int Str2Hex(CString str,CByteArray &data)

{//To convert a character string as a hexadecimal string into a byte group. The bytes can be divided by spaces. The length of the converted byte group will be returned. Simultaniously the length of the byte group will be set automatically.

int t,t1;

int rlen=0,len=str.GetLength();

data.SetSize(len/2);

for(int i=0;i<len;)

{

char l,h=str[i];

if(h==' ')

{

i++;

continue;

}

i++;

if(i>=len)break;

l=str[i];

t=HexChar(h);

t1=HexChar(l);

if((t==16)||(t1==16))

break;

else

t=t*16+t1;

i++;

data[rlen]=(char)t;

rlen++;

}

data.SetSize(rlen);

return rlen;

}

3．6//To test a character be a hexademical character or not. If it is, it will return the relative value. And if it is not, it will return 0x10;

char HexChar(char c)

{	if((c>='0')&&(c<='9'))

return c-0x30;

else if((c>='A')&&(c<='F'))

return c-'A'+10;

else if((c>='a')&&(c<='f'))

return c-'a'+10;

else return 0x10;

}

Event Processing Part

4．1 Event of receiving data

void CCommDlg::OnComm()

{

if(stop)return;

VARIANT m_input1;

COleSafeArray m_input2;

long length,i;

BYTE data[1024];

CString str;

if(m_Comm.GetCommEvent()==2)//Receiving the characters in buffer zone

{

m_input1=m_Comm.GetInput();//Readubg the data in the buffer zone

m_input2=m_input1;//Convert the VARIANT variable into the ColeSafeArray variable

length=m_input2.GetOneDimSize();//Defining the length of the data

for(i=0;i<length;i++)

m_input2.GetElement(&i,data+i);//Convert the data into BYTE array

for(i=0;i<length;i++)//Convert the array into Cstring variable

{

BYTE a=* (char *)(data+i);

if(m_hex.GetCheck())

{

str.Format("%02X ",a);

if ((a==0xAA) && (Rece_Count>=26))

Rece_Count=0;

//Save the data to ReceBuf

ReceBuf.SetAt(Rece_Count+i,a);

}

else

str.Format("%c",a);

}

Rece_Count=Rece_Count+length;

UpdateData(FALSE);//Renew the contents of the editing frame

//To process the receiving data

if (Rece_Count == 26)

{

//1. To check the correct of the lock head

if (ReceBuf.GetAt(0) != 0xAA)

exit(0);

//2. To check the correct of the address

if (ReceBuf.GetAt(1) != POWER_ADDRESS)

exit(0);

//3. To check the command word

if (ReceBuf.GetAt(2) < 0x80)

exit(0);

if (ReceBuf.GetAt(2) > 0x82)

exit(0);

//4. To check the checking sum

BYTE Total,i;

Total=0;

for (i=0;i<=BufferMax-2;i++)

Total=Total+ReceBuf.GetAt(i);

if (Total != ReceBuf.GetAt(BufferMax-1))

exit(0);

//Correct part of data processing

ShowReceData();

...

}

}

}

4．2 To initializing the dialogue frame

BOOL CCommDlg::OnInitDialog()

{

…

// TODO: Add extra initialization here

//To initialize the control file and buffer storage the data

m_com.SetCurSel(0);

m_speed.SetCurSel(4);

m_Order.SetCurSel(0);

m_hexsend.SetCheck(1);

m_hex.SetCheck(1);

UpdateData(TRUE);

InitData();

return TRUE;  // return TRUE unless you set the focus to a control

}

4．3 To open the COM port

void CCommDlg::OnButton1()

{

if( !m_Comm.GetPortOpen())

m_Comm.SetPortOpen(TRUE);//To open the Com

else

{

m_Comm.SetPortOpen(FALSE);

m_Comm.SetPortOpen(TRUE);//To open the Com

}

UpdateData(TRUE);

}

4．4 To delete the receiving data

void CCommDlg::OnButton2()

{

m_ReceiveData.Empty();//To delete the data in the receiving dialogue frame

//m_SendData.Empty();//To delete the data in the sending dialogue frame

UpdateData(FALSE);

}

4．5 To select the COM port

void CCommDlg::OnComselect()

{

if(m_Comm.GetPortOpen())

m_Comm.SetPortOpen(FALSE);

m_Comm.SetCommPort(m_com.GetCurSel()+1);

}

4．6 To set the Baud Rate

void CCommDlg::OnComspeed()

{

CString temp;

int i=m_speed.GetCurSel();

switch(i)

{

case 0:

i=2400;

break;

case 1:

i=4800;

break;

case 2:

i=9600;

break;

case 3:

i=19200;

break;

case 4:

i=38400;

break;

}

temp.Format("%d,n,8,1",i);

m_Comm.SetSettings(temp);

}

4．7 The receiving data event

void CCommDlg::OnSend()

{

// TODO: Add your control notification handler code here

//To set the sending data

SendBuf.SetAt(2,Cur_Order);

SendBuf.SetAt(3,Add_Order);

if (m_Order.GetCurSel()==1)

{

// Set the output current as 3A and the proportion coefficient as 1000

SendBuf.SetAt(3,3000 % 256);

SendBuf.SetAt(4,3000 / 256);

// Set the output voltage as 36V and the proportion coefficient as 1000

SendBuf.SetAt(5,36000 % 256);

SendBuf.SetAt(6,36000 / 256);

// Set the power as 108W and the proportion coefficient as 1000

SendBuf.SetAt(7,10800 % 256);

SendBuf.SetAt(8,10800 / 256);

//Set the output voltage as 3V and the proportion coefficient as 1000

SendBuf.SetAt(9,3000 % 256);

SendBuf.SetAt(10,3000 / 256);

//Set the address as: POWER_ADDRESS

SendBuf.SetAt(11,POWER_ADDRESS);

}

if( m_Comm.GetPortOpen())

{

CalDataTotal();

ShowSendData();

if(m_hexsend.GetCheck())

{

int len=Str2Hex(m_SendData,SendBuf);

m_Comm.SetOutput(COleVariant(SendBuf));//Sending data

}

else

m_Comm.SetOutput(COleVariant(m_SendData));//Sending data

}

else

MessageBox("Please open the COM connector first!", NULL, MB_OK);

}

4．8 Command Selection

void CCommDlg::OnSelendokOrder()

{  int i=m_Order.GetCurSel();

switch(i)

{

case 0:

Cur_Order=ORDER_READ;

break;

case 1:

Cur_Order=ORDER_WRITE;

break;

case 2:

Cur_Order=ORDER_CONTROL;

Add_Order=PC_CONTROL;

break;

case 3:

Cur_Order=ORDER_CONTROL;

Add_Order=SELF_CONTROL;

break;

case 4:

Cur_Order=ORDER_CONTROL;

Add_Order=POWER_ON;

break;

case 5:

Cur_Order=ORDER_CONTROL;

Add_Order=POWER_OFF;

break;

}

}