# Q-MAC Fixed Helical Dipole Antenna (QM7500 Series)

## Description:

The QM7500 Series Fixed Helical Dipole Antenna offers improved efficiency in a fixed base station application where there is limited space for installation up to 5 fixed frequencies may be offered with this antenna.

Assembly of the antenna is made easier by threaded mounting bolts on a metal center hub.   Each frequency is pre-tuned, making installation in the field more simplified and less time consuming.

The QM7500 series antennas have an overall span of 7m and are shrink plastic coated.

Note that the QM7500 series antennas are supplied complete with 30m of RG-58 coaxial cable.  No mounting structures are included.

Specifications:

| ### ELECTRICAL Frequency Range: 2 – 30MHz Feed Impedance: 50Ω Power Input: 100W Continuous duty 300W PEP Balun: Integral 1:1 Termination: SO239 Radiation: Similar to ½ wave dipole antenna | ### MECHANICAL Average weight: 8kg 5 frequency 11kg Minimum Install Height 6m to center hub Frontal area: 0.7m2 max (5 frequency) Overall Width: 7m Radiating elements: Fibreglass |
| --- | --- |

Helical Dipole Antenna.