HOW TO USE KERMIT FOR TRANSFERRING FILES

BETWEEN MICROCOMPUTERS

Norman Weatherby, Ph.D.

Center for Population and Family Health

Columbia University

July 24, 1984

KERMIT (like the frog, a registered trademark of Henson

Associates, Inc., used by permission) is ideal for transferring

ASCII and binary files between computers of all sizes.  It is a

protocol for transferring sequential files between computers of

all sizes over ordinary asynchronous telecommunication lines

using packets, checksums, and retransmission to promote data

integrity.  KERMIT is non-proprietary, thoroughly documented,

well tested, and in wide use.  The protocol and the original

implementations were developed at Columbia University and have

been shared with many other institutions, some of which have made

significant contributions of their own.  KERMIT programs have

been written for a wide variety of microcomputers, minicomputers,

and mainframes.

Which ones?  At the end of this document is an extensive

listing that was downloaded (using KERMIT) from a DEC mainframe

at Columbia University.  In addition, you will find the address

to write for futher documentation, and some information for those

who would like to participate in the growth and acceptance of

KERMIT as one of the major file-transfer protocols.

I wrote this guide so that people who have very little

computer experience could use KERMIT to transfer files between

CP/M micros or between CP/M and IBM PC-type systems.  It simply

tells you how to work with KERMIT, without going into its

features.

This guide was submitted to the KERMIT distributors, and

they like it, but they have NOT stamped their approval on it.  It

is not a substitute for the real documentation.  And, as the real

documentation states:

No warranty of the software nor of the accuracy of the

documentation surrounding it is expressed or implied,

and neither the authors nor Columbia University

acknowledge any liability resulting from program or

documentation errors.

KERMIT is copyrighted, and it is not "public domain".  Simply, it

is a free program that is not to be sold as a product.  As the

distributors state, you are free to redistribute KERMIT on your

own terms, and are encouraged to do so, with the following

stipulations: KERMIT should not be sold for profit; credit should

be given where it is due; and new material should be sent back to

Columbia University at the address given in the summary so that

they can maintain a definitive and comprehensive set of KERMIT

implementations for further distribution.

Where do you get KERMIT?  As stated below, Columbia

University distributes versions of the program only on nine track

magnetic tape.  You can get the program from friends, bulletin

board systems, and various areas on services such as CompuServe.

Look around for the latest version for your micro, and be aware

that the program is constantly improving.

NOW, KERMIT

Without going into the details of KERMIT, this guide shows

you how to use the program to transfer files between micro-

computers that have KERMIT. The sections include

1. Connecting the computers, either directly or through

modems

2. Setting the baud rates

3. Loading KERMIT into memory

4. Setting KERMIT's options

5. Communicating between computers

6. Sending and receiving files

7. Checking what was sent

8. Figuring out why (If) KERMIT does not work

9. Summary

CONNECTING THE COMPUTERS

If you are going to transfer the file over the telephone

system through modems, hook up a

full duplex modem

to each computer's serial communications port (com1: on IBM PC

type machines, or on CP/M machines, rdr:/pun: or auxin:/auxout:).

You may need to get a cable that switches pins 2 and 3 to a modem

if the port is a printer port (lst: or printer).  If you are

working with acoustically coupled modems, set one modem to answer

mode and the other modem to originate mode.

If you do not have two modems, and the computers are in the

same room, you can

a. Directly connect the serial modem ports of the two

machines.  The cable that is needed is one in which pins

2 and 3 are switched when connecting two modem ports.

OR

b: Directly connect the serial modem port of one computer to

the serial (not parallel) printer port of the other

computer.  This may cause a problem, since you have to

make sure that KERMIT talks to the printer port of the

other machine.  However, it is the only way to get, for

example, the Osborne Executive to KERMIT a file to an IBM

PC-type machine.

One benefit of using a direct connection between the computers is

that you can use high baud rates, such as 4800 baud, to transfer

the files.  Since you are not using modems or telephones, ignore

the parts of this guide that concern use of the modems and

dialing of the telephone.

SETTING THE BAUD RATE

The baud rate controlls the speed of the file transfer.

The baud rate can be set from within KERMIT some CP/M machines,

such as the Osborne 1 and the Intertec Superbrain, and on MSDOS

(PCDOS) computers.  One really nice feature of KERMIT is that you

do not have to cope with that poorly-documented mode command on

the IBM PC-type micros.

If you do not know whether KERMIT handles the baud rate,

skip forward in these instructions and load KERMIT into memory.

At the KERMIT prompt, give the command

set baud<cr>

where  <cr>  means to "hit" the return key.  KERMIT will tell you

if the baud rate setting command has not been implemented.

If you cannot set the baud rate from within KERMIT, such as

on H89 systems, then you can get back to your operating system by

typing, at the prompt,

exit<cr>

Then use the computer's software to set its baud rate, usually to

300 or 1200 baud when modems are used.  The baud rate of a CP/M

machine is set through the SETUP or CONFIGUR utilities supplied

with the system.

LOADING KERMIT INTO MEMORY

Loading KERMIT into the memory of an Apple II or Atari

computer is more difficult than loading it into CP/M or MSDOS

(PCDOS) systems.  Thus, readers should refer to other

documentation and help files for assistance with these systems.

IF YOU HAVE A HARD DISK: Micros with hard disks, such as the

IBM PC-XT, often refer to all or part of the hard disk as disk

C  .  If you have a hard disk, copy your KERMIT program and

associated utility, help, and documentation files from the KERMIT

floppy to the hard disk (if someone else has not copied them over

already).  Then work off of the hard disk.  That is, if you do

not see the prompt for the hard disk on your screen, type the

command

c:<cr>

Then, to load KERMIT into memory, type the name of your kermit

program, which is usually KERMIT.  For example, at your operating

system's prompt, type

kermit<cr>

Then skip to the section below about setting KERMIT's options.

IF YOU HAVE TWO FLOPPY DRIVES: Put the KERMIT diskette in

drive A of each computer.  Note that on a CP/M machine such as a

Kaypro, H89, or Osborne 1, it is wise to have the operating

system on the KERMIT disk (through the use of the SYSGEN utility)

and to type the command

cntl C (without a carriage return)

where  cntl  is the control key and  C  can be upper or lower

case, to make the computer realize that a new disk has been put

in drive A.  This "warm boot" command is not necessary on micros

that use CP/M 3.0, MSDOS, or PCDOS operating systems.

Put a formatted disk in drive B of the system which is to

receive the file.  On the sending system, put the disk with the

file that you want to transfer in drive B.  Go to drive B, and

then get into KERMIT on each machine by typing  a: and the name

of the program, which is usually KERMIT.  For example, type

a:kermit<cr>

Newer versions of KERMIT allow you to change the "logged disk

drive," but it is wise to follow the above advice so that you are

sure of the disk that you are sending from or receiving to.

SETTING KERMIT'S OPTIONS

To see the options that are available in your version of

KERMIT, type at the prompt the command

status<cr>

Note that the toggle command   set ibm  , which can be set  on

or  off , is NOT FOR IBM PC MICROS--it is a command for Columbia

University's IBM mainframes.

SETTING THE PARITY: A computer "byte" is composed of  eight

bits--where each bit is a zero or one.  All (english) printable

letters, numbers, punctuation marks, and spaces between words can

be represented by seven of the eight bits.  The eighth bit is

reserved for checking to make sure that the other seven bits are

correct.  However, some microcomputer software packages (such as

WordStar) use the eighth bit for special characters that allow

features such as right justification.  In any event, if both

computers allow the following command, issue it to allow the

transmission of all eight bits:

set parity none<cr>

SETTING THE FILE-MODE: Microcomputers store files in two

ways:

ASCII files--codes such as letters, numbers, punctuation

marks, and spaces, or as

binary files--codes that are recognizable to a computer but

not to you, such as the way programs are stored.

If you are transferring an ASCII file, issue the command

set file ascii<cr>

If your are transferring a binary file, use

set file binary<cr>

SETTING THE BAUD RATE: If you can set the baud rate from

within KERMIT, make the baud rates of both computers equal by

typing at the prompt the command

set baud xxxx<cr>

where  xxxx  is the baud rate you want.  For example, you may

type

set baud 1200<cr>

If these commands do not work, try

set baud ? <cr>

and select the correct baud rate from a menu that is given on the

screen.  If you get messages that  set baud  command is not

implemented,  you will have to get out of KERMIT and set it from

your operating system, as discussed above.

COMMUNICATING BETWEEN COMPUTERS

Then type at the prompt the command

connect<cr>

on each computer.  This will put you into the connect mode, which

allows one computer operator to send messages to the other

operator through the modems or the direct connection.  Note that

KERMIT replies with an "escape" message that tells you how to get

back to the command state of the program.  WRITE DOWN THIS

MESSAGE, AS YOU WILL NEED IT LATER.

In fact, it is a good idea to test the command now.  The way

you get back to the KERMIT prompt varies by the type of system.

In general, you will have to issue the command but add a  C

(upper or lower case) to it to get back.   For example, on many

CP/M machines that have the backslash character \ , the "escape"

command is control \  , but you type

ctrl\c  (without typing a return)

where  ctrl  is the control key.  After some experimentation, you

will see the KERMIT prompt, and then you can type the command

connect<cr> again.

For those who are using modems: From the originate modem,

dial the answering modem.  If you have an auto-dial modem, you

can issue the dial command to it.  For example, with a touch tone

telephones and a Hayes SmartModem, the only command that is

necessary is for the originate modem:

ATDTxxxxxxx

where xxxxxxx is the telephone number to dial.

When the connection is made, each  you each will be able to

send and receive messages that serve to test the connection.

If you want to see what you are typing, get back to the

KERMIT prompt by typing the "escape" command that you

wrote down and tested above.  Then issue the command

set loc on<cr>

to have a local echo, but you must remember to

set loc off<cr>

before attempting to transfer a file.

If the connection is not made, so that it is not possible to

send AND receive messages, then you should check

the baud rates of the computers,

the modems, and the wiring to the modems,

or the wiring between the directly-connected

computers,

and make sure that the computer is sending or receiving data

to the correct port on the computer.  It is also helpful, as a

last resort, to check whether or not the telephone died because

you forgot to pay the bill (don't laugh...it once happened to

us!)

SENDING AND RECEIVING FILES

When each computer operator is satisfied that a connection

has been made, then both operators return to the KERMIT prompt,

as explained above.

The operator that wants to receive a file types

receive<cr>

The file name will be sent from the other machine.

The operator that wants to send a file types the command

send [drive:]filename.ext<cr>

For example, to send the file on disk B that is named

myfile.txt , the operator would type

send b:myfile.txt<cr>

It is usually not necessary to type in the drive specification,

since you should be using disk B as the "logged disk drive", but

it is always safe to do so.  If the receiving operator hits the

return before the sending operator hits the return, it may be

necessary for the receiving operator to hit another return before

the file will be sent.

KERMIT will wait for a few seconds, and then the operators

will see on their screens that packets of information are being

sent and received.  KERMIT first sends the file name, and it is a

good sign to the receiving operator when the file name appears on

his or her screen.  The number of packets will increment on both

machines until the transfer is complete, and then each computer

will return to the KERMIT prompt.  At that time, the receiving

operator can check the file or another file can be sent.

CHECKING WHAT WAS SENT

Once a connection has been established with KERMIT, it is

not broken if one or both of the operators return to the

operating system of their computers to check something such as

the length or name of a file.  Thus, when a transfer is complete,

the receiving operator can get out of KERMIT by typing

exit<cr>

at the KERMIT prompt.  The file that was received can be

examined, usually by issuing the command

type [drive:]filename.ext<cr>

The file will scroll by on the screen, and you will see that it

has been transferred without error!

The receiving operator can then return to KERMIT by

reloading KERMIT into memory.  As stated above, go to your B disk

and type A:KERMIT or whatever the file name of your program is.

If you need to, reset the baud rate, and you will then be able to

transfer another file or get into connect mode and send messages.

FIGURING OUT WHY (IF) KERMIT DOES NOT WORK

KERMIT has never, in my two years of experience with the

program, made a mistake in sending and receiving files between

CP/M computers or between CP/M computers and mainframes.  It

accurately transfers files even when the telephone line is

unusable for voice transmissions because of static and noise.

Operators do, however, tend to make mistakes.  One such

problem is impatience.  Please let KERMIT wait for a few seconds

before you touch a carriage return on the receiving computer to

"make it work."  If KERMIT seems to be dead after about thirty

seconds of waiting, then something is wrong.  If you were able to

send messages back and forth in connect mode, or if KERMIT fails

after the first four or five packets, then probably the problem

is that one of the operators set the local echo switch on but

forgot to set it off.  It is also possible that the file that you

wanted to send does not exist, because the sending operator

missppelled its name in the send command.

Occasionally, KERMIT will fail for reasons beyond its

control.  The transfer will fail if one of the computers involved

goes down.  For example, mainframe computers or their communi-

cation ports often go down (that's why we are now using micros

for most of our work, right?).  A feature such as call waiting or

intercom messages on a telephone line will stop the transmission

if they happen during a transfer.  I view these failures as

advantages in that it shows that KERMIT is smart enough to quit

when there is a major problem.

We have also encountered a problem with the way BASICA and

dBASE II on IBM PC-type computers mark the end of a file and save

it.  The solution is to load the file into a text editor or word

processor and then resave it before sending it.  For example,

load a file into WordStar and then save it.  An unreliable

alternative is to copy the file using the /A parameter.  The

problem is not fully understood; it centers around the PCDOS and

MSDOS software's use of buffered output when writing disk files.

Apple II versions of KERMIT work well with DOS 3.3, but

Apple IIs do not work so well.  They are slow, there are

differences between the II, the II+ and the IIe, and there is no

standard way that people set up their Apple's for communications.

If you plan to use Apple KERMIT, please check with other Apple

KERMIT users before ordering your communications card and modem.

Currently, the best cards may be the Apple Communications card,

the Hayes Micromodem II, or the Super Serial Card.  Actually, the

best way may be to install a Z80 card in the Apple II and run the

Apple CP/M version of KERMIT.

SUMMARY

KERMIT is a very powerful program, with extensive error

checking, and thus it is ideal for transferring files between

computers.  The full documentation will show you many commands

and options for the program.  You may want to order the

documentation, but the simple procedures in this guide may be

adequate for your needs.

Extensive documentation is available for this program from

KERMIT Distribution

Columbia University Center for Computing Activities

7th Floor, Watson Laboratory

612 West 115th Street

New York, NY  10025

There are three publications: the User's Guide, the Protocol

Manual, and the Source Listing for your system.  Each costs

$5.00, and you should make out your check to Columbia University

Center for Computing Activities.  Make sure and tell them what

microcomputer and operating system you are using.  Those who are

interested in mainframe implementations (which currently cost

$100 for the complete distribution) should write for ordering

information.  Note that the KERMIT distributors can provide the

programs only on 9-track magnetic tape in a varity of formats.

They do not provide the program on floppies.

When necessary, there are specific help files that are

distributed with KERMIT to help with its use.  For example, there

is a help file with the Kaypro II version because of the non-

standard way a Kaypro handles tab stops on its screen.

There are also articles and announcements about KERMIT.  For

example, see BUSS #88 and June-July, 1984 issues of BYTE.

Please do not call the Kermit Distribution Center for

assistance with KERMIT for your micro.  After all, it is a free

program, and national support, sales, service, debugging,

revisions, everything else is being handled by only two or three

part-time people.  You can write, however, for the publications.

Finally, KERMIT is changing and improving constantly --

versions are added for new systems, old versions are improved,

and documentation is rewritten.  You are encouraged to make your

own contributions to "KERMIT culture" and to make them available

not only to your friends, but also to send them back to Columbia

at the address given above, on 9-track magnetic tape or IBM PC

eight sector floppy disk.  For example, we need KERMIT for

TurboDos version 1.2, and we need to know how to implement the

"break" key on many CP/M machines (like the H89) through

software.

The most current version of KERMIT for CP/M machines is

3.9a, which fixes a bug in version 3.9 that may garble the

transmission of apmersands.  A brand-new MSDOS version is soon to

be released.  Watch for new releases and updates.  Meanwhile,

below is a recent but even now out-of-date listing of available

versions.

CURRENT KERMIT VERSIONS as of 4:42pm  Wednesday, 6 June 1984

Listed in reverse chronological order of installation at Columbia.  "Code" is

the prefix under which the files are stored in the KERMIT distribution area.

CURRENT KERMIT VERSIONS as of 9:46pm  Wednesday, 18 July 1984

Listed in reverse chronological order of installation at Columbia.  "Code" is

the prefix under which the files are stored in the KERMIT distribution area.

New entries are added at the top.

Code  Machine/OS        Language     Ver #   dd/mm/yy  Author or Contact

TSO   IBM 370/MVS/TSO   IBM Asm       1.0    18/07/84  SYSRONR@UCHIVM1.BITNET

APP   Apple II/DOS      CROSS         2.1    18/07/84  G.BUSH@COLUMBIA-20

20    DEC-20/TOPS-20    MACRO-20    4.1(236) 18/07/84  CC.FDC@COLUMBIA-20

K11   PDP11/RSX11M,M+   MACRO-11      2.18   17/07/84  ATSBDN@UOFT01.BITNET

K11   PDP11/RSTS        MACRO-11      2.18   17/07/84  ATSBDN@UOFT01.BITNET

K11   PDP11/RT-11       MACRO-11      2.18   17/07/84  ATSBDN@UOFT01.BITNET

APO   Apollo/Aegis      FORTRAN       1.0    13/07/84  John Lee, RCA Labs

HPM   HP1000/RTE        FORTRAN       1.0    13/07/84  John Lee, RCA Labs

SIR   Sirius-1          MASM          1.20   12/06/84  Barry Devlin, Dublin

ST    HP3000/SoftwareTools/Ratfor     1n     18/02/84  kpd%HP-LABS@CSNET-Relay

ST    Univac/SoftwareTools/Ratfor     1n     11/06/84  kpd%HP-LABS@CSNET-Relay

CPM   Apple II/CPM-80   ASM           3.9A    6/06/84  CC.FDC@COLUMBIA-20

CPM   DECmate II/CPM80  ASM           3.9A    6/06/84  CC.FDC@COLUMBIA-20

CPM   Generic CPM80 2.2 ASM           3.9A    6/06/84  CC.FDC@COLUMBIA-20

CPM   Generic CPM80 3.0 ASM           3.9A    6/06/84  CERRITOS@USC-ECL

CPM   H/Z-19/CPM-80     ASM           3.9A    6/06/84  CC.FDC@COLUMBIA-20

CPM   H/Z-100/CPM-80    ASM           3.9A    6/06/84  CC.FDC@COLUMBIA-20

CPM   Kaypro II/CPM-80  ASM           3.9A    6/06/84  CC.FDC@COLUMBIA-20

CPM   Osborne 1/CPM-80  ASM           3.9A    6/06/84  CC.FDC@COLUMBIA-20

CPM   Superbrain/CPM80  ASM           3.9A    6/06/84  CC.FDC@COLUMBIA-20

CPM   TRS80-II/CPM80    ASM           3.9A    6/06/84  CERRITOS@USC-ECL

CPM   Vector Graphx/CPM ASM           3.9A    6/06/84  CC.FDC@COLUMBIA-20

CPM   VT180/CPM-80      ASM           3.9A    6/06/84  CC.FDC@COLUMBIA-20

CPM   TelconZorba/CPM80 ASM           3.9A    6/06/84  G.BUSH@COLUMBIA-20

PRO   DEC Pro-350       Bliss         1.0     1/06/84  G.BUSH@COLUMBIA-20

VMS   VAX/VMS           Bliss         3.0.051 1/06/84  G.BUSH@COLUMBIA-20

K10   DEC-10/TOPS-10    MACRO/Bliss   3(124)  1/06/84  G.BUSH@COLUMBIA-20

TRS   TRS-80 I and III  M80           3.5    30/05/84  stan@RICE

170   Cyber 170/NOS     Fortran77,asm 2.0    25/05/84  knutson@UT-NGP

UCI   IBM PC/p-System   UCSD Pascal   0.1    23/05/84  KMM@CORNELLA.BITNET

RBLC  Rainbow/MS-DOS    CI/86         2.14   23/05/84  LCAMPBELL@DEC-MARLBORO

800   ABC-800/ABCDOS    BASIC-II      2.2     8/05/84  Per_Lindberg_QZ@QZCOM

86/APC NEC APC/CPM-86   ASM86         2.7     7/05/84  CONTEXT@WASHINGTON

86/RB Rainbow/CPM-86    ASM86         2.7     4/05/84  CC.FDC@COLUMBIA-20

MAC80 8080 Cross Asmblr MACRO-10  10E(120)   30/04/84  CERRITOS@USC-ECL

CPM   Morrow Decision I ASM           3.9    26/04/84  CC.FDC@COLUMBIA-20

CPM   Nokia MikroMikko  ASM           3.9    26/04/84  CC.FDC@COLUMBIA-20

CPM   Ohio Sci/CPM-80   ASM           3.9    26/04/84  CC.FDC@COLUMBIA-20

MP    PDP-11/MUMPS-11   MUMPS(1982)          11/04/84  KMM@CORNELLA.BITNET

UCT   Terak/p-System    UCSD Pascal          11/04/84  KMM@CORNELLA.BITNET

KPROTO"KERMIT Protocol Manual"        5th Ed  2/04/84  CC.FDC@COLUMBIA-20

KUSER "KERMIT User Guide" English     5th Ed  5/03/84  CC.FDC@COLUMBIA-20

MU    Honeywell/MULTICS PL/I                 28/02/84  P.Amaranth, Oakland U

TA2   Tandy 2000/MS DOS MASM          1.20   16/02/84  smp@UTEXAS-11

PRI   Prime/PRIMOS      PL/P                 10/02/84  L.Spira, The SOURCE

AOS   Data General/AOS  Ratfor                2/02/84  John Lee, G.E.

HP1   HP-150/MS-DOS     MASM          1.3     2/02/84  Frank Heartney, H-P

CMS   IBM 370/VM/CMS    IBM Asm               1/02/84  CC.DAPHNE@COLUMBIA-20

MDS   Intel DevSys/ISIS PL/M                 27/01/84  Chris@COLUMBIA-20

RT    PDP11/RT11        OMSI Pascal          24/01/84  Bruce Pinn, U Toronto

HP9   HP-98xx/p-System  HP Pascal            20/01/84  Gallaher@RUTGERS

VF    VAX/VMS           Pascal/Fortran       18/01/84  Bruce Pinn, U Toronto

ATA   Atari/DOS         Action!               9/01/84  G.TANG@SU-SCORE

MTS   IBM 370/MTS       Asm, Pascal           6/01/84  "Gavin Eadie"b@UMich-MTS

UN    Univac 1100/Exec  Exec Assembler       22/12/83  STEVENS@MACCWISC.MAILNET

PC    IBM PC etc/PC-DOS MASM          1.20   30/11/83  CC.Daphne@COLUMBIA-20

PC    H/Z-100           MASM          1.20   30/11/83  CC.Daphne@COLUMBIA-20

VIC   Victor 9000/CPM86 ASM86         1.1    25/11/83  Barry Devlin, Dublin

SEE   Seequa/MS DOS     MASM          1.18   17/11/83  Glenn Everhart, RCA

VIC   Victor 9000/MSDOS MASM          1.18   10/11/83  Bryan Peterson, ORNL

UX    (various)         Unix                 17/10/83  CC.FDC@COLUMBIA-20

VIC   Victor 9000/MSDOS MASM          1.1     3/06/83  Barry Devlin, Dublin

VX    VAX/VMS           VAX-11 C             24/04/83  Todd Little, DEC