| To : | Doc. No. |  |
| --- | --- | --- |
| C.C. : | Country | STA, UK, AUS, Italy, etc |
| From : PB Lim | Tel: 82-31-279-4394 E-mail: pblim@samsung.com |  |
| Subject: Release ITM3 S/W Version V3.03(030701) |  |  |

Overview :

Release ITM3 S/W Version V3.03(030701)

Contents :

a) H323 Stack V3 → V4

- Performance improved

- Progress Message support (compibility with Cisco GW)

b) TOS is applied

- Feature is added for RTP Media Packet

- TOS menu is in the MMC835.

c) T38 fax stability and compatibility is improved

- RTP redundancy can be configured (default:0)

- T38 Fax redundancy menu is in the MMC835.

d) Tunneling menu is moved

- Tunneling menu is moved to MMC834 from SNMP IP address.

e) List of Added MMC

List            default            Range

T38 Redundancy           0              0 ~ 4

H.245 Tunneling       Disable        Disable, Enable

Type Of service       00000000       00000000 ~ 11111111 (0x00~0xff)

TOS(Type Of Service) Field   |

+---------------------------------+

|  Priority | D | T | R | M | 0 |

+---------------------------------+

D: if  00010000(0x10), Minimum Delay

T: if  00001000(0x08), Maximum Throughput

R: if  00000100(0x04), Maximum Reliability

M: if  00000010(0x02), Minimum Monetary Cost

Range:0000000 ~  1111111 (0x00 ~ 0xFF), Default:0x00

S/W Compatibility Table :

1. STA

- ITM3 V3.01 are compatible with old MP (from iDCS 500 V1.25)

b. ITM3 V3.03 are compatible with New MP (from iDCS 500 V1.30)

c. ITM3 V3.03 are compatible with New MP (from iDCS 100 V1.30)

|  | IDCS500(M) | IDCS500(L) | IDCS500(L) | DCS100(MEM3) | DCS100(MEM4) |
| --- | --- | --- | --- | --- | --- |
| Version | B1.30 | E1.30 | S1.30 | B1.30 | E1.30 |
| Date | 03.07.01 | 03.07.01 | 03.07.01 | 03.07.01 | 03.07.01 |
| ITM3 | V3.03 | V3.03 | V3.03 | V3.03 | V3.03 |

2. EURO, LSP, Russia, and etc

a. ITM3 V3.01 are compatible with old MP (from iDCS 500 V1.25)

b. ITM3 V3.03 are compatible with New MP (from iDCS 500 V1.30)

c. ITM3 V3.03 are compatible with New MP (from iDCS 100 V1.30)

|  | IDCS500(M) | IDCS500(L) | DCS100(MEM3) | DCS100(MEM4) |
| --- | --- | --- | --- | --- |
| Version | M1.30 | L1.30 | B1.30 | F1.30 |
| Date | 03.07.01 | 03.07.01 | 03.07.01 | 03.07.01 |
| ITM3 | V3.03 | V3.03 | V3.03 | V3.03 |

3. KOREA

- ITM3 V3.01 are compatible with old MP (from DCS828 V1.08)

b. ITM3 V3.03 are compatible with New MP (from iDCS828 V1.30)

|  | IDCS500(M) | IDCS500(L) | DCS828 |
| --- | --- | --- | --- |
| Version | M1.30 | L1.30 | 1.09 |
| Date | 03.07.01 | 03.07.01 | 03.07.01 |
| ITM3 | V3.03 | V3.03 | V3.03 |