SMG-400 Update Notes

Oct 15, 2002

- Version   : 400V3.0:021015

- previous : 400V2.3:020515(or 400V2.4T020914  or  400V2.4T)

- Package Files

- smg.000 ~ smg.027  (28 files)

- dsps.000 ~ dsps.001  (2 files)

- snd.004 ~ snd.010   (6 or 7files,  snd.009: for only Korea)

- sig_ver.c           (1 file)

- Updated Function List

| Item | Updated Type | # Description | CLI |
| --- | --- | --- | --- |
| IVR Control | Added | - Added IVR Control each Release Reason - The “clicmd” includes the many commands which controls the special operation of SMG. - The Command List(Added) ivrhelp: Help IVR Command -> Clearivrtable : Clear IVR Control Table -> Showivrtable : Show IVR Control Table -> Setivrtable : Set IVR Control Table - upgrade Method Sound Files for IVR (different sound file each country. Ex: USA, UK, Russia, Korea, etc) -> File Transfer sig_ver.c and snd.xxx with binary mode -> siglo "snd.004" -> siglo "snd.005" -> siglo "snd.006" -> siglo "snd.007" -> siglo "snd.008" -> siglo "snd.009" (for only Korea) -> siglo "snd.010" | clicmd |
| Call Block | Added | - Added Call Block each port - The “clicmd” includes the many commands which controls the special operation of SMG. - The Command List(Added) callblock: Help Call Block Command ->Clearcallblock : Set Default Vaule at Call Block Table -> Showcallblock : Show Call Block Table -> Setcallblcok : Set Call Block Table | clicmd |
| Call Block | Added | - Added Call Blocking according as VoIP outbound Access Code | voipmenu /3/0 |
| Loop In Coming | Changed Added | - Changed Loop Incoming Direct Call By Preset Number - Added Loop In Coming IVR - The “clicmd” includes the many commands which controls the special operation of SMG -> loopincall : Help Loop In Coming Call -> Setloopincall: Set Tone/Off/IVR, Auto Dial Number -> Clearloopincall: Clear LOOP Incoming Control Table ->Showloopincall: Show LOOP Incoming Control Table | clicmd |
| Date and Time | Changed Added | - Added Synchronize Date & Time with Time Server - Added ntp command - The “clicmd” includes the many commands which controls the special operation of SMG -> ntphelp: help ntp command -> Onntp: Turn On NTP Time Service -> Offntp: Turn Off NTP Time Service(default) -> Setntpip "IP": Set NTP Time Server IP -> Setntptimezone: Set NTP Time Zone -> Setntptime period: Set NTP Period ( 1 ~ 1440(60*24) min ) -> Showntp : Display NTP Configuration | clicmd |
| SNMP Trap | Changed Added | - Added SNMP Reporting Feature with SNMP Trap - Added snmptrap command - The “clicmd” includes the many commands which controls the special operation of SMG -> snmphelp: Help SNMP Trap Command -> Showsnmp: Show SNMP Manager List -> Onsnmpagent: Turn On the SNMP Agent -> Offsnmpagent: Turn Off the SNMP Agent -> Setsnmp: Set SNMP Manager IP & Port & Community -> Ontrapcritical: Turn On the Critical Alarm(System Restart) -> Offtrapcritical: Turn Off the Critical Alarm -> Ontrapmajor: Turn On the Major Alarm(DTRK Down,GK Down) -> Offtrapmajor: Turn Off the Major Alarm -> Ontrapminor: Turn On the Minor Alarm(Call Fail..) -> Offtrapminor: Turn Off the Minor Alarm(Default) -> Ontrapusermode: Turn On the User Mode(DB Change, Login..) -> Offtrapusermode: Turn Off the User Mode(Default) | clicmd |
| H.323 Stack V4 | Added | - Upgrade H.323 Stack Version V4 (H.323 V3  V4) |  |
| Performance | Changed | - Changed Task Priority, DRAM delay Mode for Performance |  |
| RRQ | Added | - Added Feature Right Weight RRQ |  |
| RRQ Time | Added | - Added Registration Request Time When GK Link Failed -> Setrrqtimewhenfail ttl: Set RRQ Time When GK Link Failed | optionhelp |
| URQ | Added | - Add Checking URQ Reason -> Offurqreasonmode: Turn Off Checking URQ Reason(Default) -> Onurqreasonmode: Turn On Checking URQ Reason | optionhelp |
| RAS Method(GRQ) | Added | - Add RAS Method(Auto with GRQ) | voipmenu /2/1 |
| PSTN Routing | Added | - Added PSTN Routing to Specific Port at VoIP Inbound Call | voipmenu /3/1 |
| PSTN Reroute | Added | - Added PSTN Reroute On/Off Feature ->Onpstnreroute: Turn On Pstn Reroute(Default) ->Offpstnreroute: Turn On Pstn Reroute(Default) - Added PSTN Rerouting to Specific Port -> Setrerouteport slot,port: Set PSTN Rerouting to Specific Port | optionhelp |
| Normal User | Added | - Added Normal user(Read Only DB when Login by Telnet) | sysmneu /2 |
| Logging | Added | - Added On/Off System Logging(ex: Changed DB, Login System, occur some error) -> Offsystemlog: Turn Off and Create System Log File(Default) -> Onsystemlog: Turn On and Create System Log File | optionhelp |
| G.723(5.3k) | Added | - Added G.723(5.3k) Codec | voipmenu /1/0 |
| Remote IP Table | Changed | - Changed Remote IP Table from 32 * 32 to 64 * 16 | voipmenu /5 |
| Dial Digit | Changed | - Changed Maximum Dial Digit(16 digit  19digit) |  |
| Progress MSG | Added | - Added Supporting VoIP Progress Message |  |
| Check GW List | Added | - Added Checking Permission GW List -> Oncheckgwlist: Turn On Checking Permission GW List -> Offcheckgwlist: Turn Off Checking Permission GW List(Default) | optionhelp |
| Web Session | Added | - Added Web Session On/Off Feature -> Offwebsession: Turn Off WebSession Service -> Onwebsession: Turn On WebSession Service(Default) | optionhelp |
| Connect | Changed | - Changed Supporting Connect without Alert (or Progress ) |  |
| Cellular Network | Changed | - Changed Supporting progress of cellular network for IVR |  |
| FXO Disconnect | Changed | - Changed Supporting 440 Frequency for FXO Disconnect -> Onloopcheck 440: Turn On Detecting LOOP-Disconnect by Tone | optionhelp |
| Auto Nego | Bug Fixed | - Changed Fast Start Setup Message for Auto Nego -> Offdynamicnego: Turn Off Codec Dynamic Nego(Default) -> Ondynamicnego: Turn On Codec Dynamic Nego | optionhelp |
| FXO Caller ID | Add | - Added Caller ID Table for FXO Outgoing | voipmenu /4 |
| PRI Interface | Bug Fixed | - Changed PRI Initial Flow for clearing VoIP Port Block and System fuzzy call state |  |
| Array Full | Bug Fixed | - Changed VoIP Call Flow for clearing Array full problem |  |
| Voice After FAX | Bug Fixed | - Changed VoIP Call Flow for clearing voice after fax problem -> Offvoiceafterfax: Turn Off Transition From Fax to Voice(Default) -> Onvoiceafterfax: Turn On Transition From Fax to Voice | optionhelp |
| T.38 FAX | Changed | - Changed VoIP Call Flow for Voice – FAX - Voice- |  |
| CDR | Bug Fixed | - Changed CDR for clearing source IP not display problem |  |
| Busy Tone | Bug Fixed | - Changed Pstn Call Flow for clearing No busy tone problem (PSTN Port is All Busy at VoIP Incoming Call) (VoIP Port is All Busy at VoIP Incoming Call) |  |
| EMS | Changed | - Changed EMS Agent for EMS |  |