TV’s  SONY  Chassis  FE-1.   Repair  Typs.

Sony TV (FE-1 Chassis)
No power (no standby power supply op)
IC609 (TOP209) was faulty, also checked C640 (47uF, 25v), D631 (MTZJ-12B), D628 (P6KE200), D629 (UF4005PKG23) and R627 (4.7ohms, 5%, 0.25w) as a precaution
(Josh)

Sony TV (FE-1 Chassis)
Changes channel or volume adjusts by itself
Check S900/1/2 on pcb H1, alternatively this could be due to failure of IC005 (CD4052BCM-FL63) on pcb A
(Josh)

Sony TV (FE-1 Chassis)
Set shuts down shortly after start up with standby led flashing 2, 4 or 5 times
Check / replace chip resistors R543 (47Kohms) and R541, R573 and R575 (all 100Kohms)
(Josh)

Failing Sony Tubes, Usual cause is a heater cathode short causing colour flashes or shut down
Isolate the heater pins on the tube, and feed with a very well insulated winding around the limb of the flyback transformer. to gain the voltage required needs just over 2 turns on the limb so do 3 and reduce with a resistor
(Rob)

FE1                INT COLOR FLASHES   ------ CRT FAULT (INT CATHODE/HEATER S/C) 
FE1                SNOWY PIC & TONE ON SOUND -----    IC001 MICRO FAULTY 
KV25K5U     SNOWY PIC        ----         IC001,MICROCONTROLLER CHIP 
KV25K5U    NO REMOTE     ------  D011. 
KV25KSU    FRAME IC HOT LINES ON SCREEN ------   REPLACE C506 0.01MFD 
KV29X5U    VOL OSD PERM DISPLAYED & FRONT CONTROLS GIVE WRONG  FUNCTS      -----LEAKY FRONT CONTROL SWITCHES - CAN BE CLEANED WITH    LUCK/CARE 
KV29X5U    INT PIC            -------   JOINTS EVERYWHERE

| FE1 | Buzzing only on sound from tuner - ok on AV | Incorrect sound system selected, the cause being memory corruption. To correct the problem, enter test mode to obtain "TT - -" on screen, & type in "24". This will reset sound system back to PAL I. Turn set off & re-tune, & sound should then return to normal. |
| --- | --- | --- |
| FE1 | Dark pic | The ABL circuit is operating incorrectly. Replace R516 (56K) & R517 (10K or 27K depending on CRT) |
| FE1 | Dead except for st/by LED flashing twice | oscillator STR & phototransistor 123 & FOP chip STV |
| FE1 | Reverts to st/by at sw on with 2 or 4 flashes from LED | 2 flashes indicates an over-current error in line stage, & 4 indicates a frame fault. However, quite often, when 4 flashes occur it can still in fact be an over-current error as well as a frame fault. To prove which is the correct fault, measure pin 52 of the micro (IC001) at switch-on, and if this voltage goes high (anything above 0.6v) then it is in fact over-current. If it stays at 0v then the fault is more than likely within frame stage. |
| FE1 | Snowy pic & tone on sound | IC001 micro faulty |
| FE-1 KV21X5U | burst of EHT then shuts down with LED flashing error 2 | regulator IC603 SE135N (874992061) |
| FE-1 KV21X5U | pic closing in from sides and going off (poor HT regulation) | error amp IC603 SE135N (874992061) |
| FE1 KV25K5U | Dead with LED flashing 4 times | Check for 9v at pins 33 & 34 IC301. If present, check R546 47R 1/2w for o/c (135v Line dr supply) |
| FE1 KV25K5U | Trips off with a crack - LED flashes 5x | CRT faulty |
| FE-1 KV25K5U | lines across screen, frame IC running hot | C506 0.01 uF |
| FE-1 KV25K5U | not starting up, LED flashes 4 times | T511 breaking down at switch on, replace T511 (145326411 for 25", 145326511 for 29") |
| FE-1 KV25X5U | comes on then goes off, red LED is on | stby IC609 TOP209P u/c |
| FE-1 KV25X5U | yellow line down screen about 2 inches from RHS | IC301 CXA2060BS (875208838) |
| FE1 KV25X5U etc | Enters search mode as soon as its switched on | replace SS902 preset search switch and clean PCB that it is mounted in |
| FE-1 KV29K5U | dead, in stby | C515 220uF 25V s/c |
| FE-1 KV29K5U | no remote operation | D907 BZX79C9V1 leaky on SIRCS line |
| FE-1 KV29K5U | pink band on LHS, only obvious on dark bachground | video lead from CN504 on A PCB to CN704 on CRT wrapped around L537 (LINE LIN), tie bottom edge of loom to the hole provided in top right corner of S PCB |
| FE1 KV29X5U | Int pic - sound ok or nothing with LED flashing 4 times | Dry joint line dr Trfmr T531 (under plastic cradle) |
| FE-1 KV29X5U | no remote, front buttons are Ok | D908 9.1V zener leaky on H board |
| KV21X5U FE-1 | Dead with 2 flashes from led | When D512 was unsoldered, LOP stage operated - Q703 (BF420), Q704 (BF421) and Q713 (BF420) all in the red output. Plus D701 |
|  |  | (MTZ J 5.6, a 5.6V zener) leaky and D705, 1SS133T unsoldered, LOP stage operated - Q703 (BF420), Q704 (BF421) and Q713 (BF420) all in the red output. Plus D701 |
|  |  | (MTZ J 5.6, a 5.6V zener) leaky and D705, 1SS133T (1N4148) O/C. all on crt base |
| KV25K5U | LED flashes twice - LOPTr s/c but still goes s/c at sw on even with LOPTx replaced | Q635 line driver & check R546 47R feed res |
| KV25KSU | FRAME IC HOT LINES ON SCREEN | REPLACE C506 0.01MFD |
| KV25X5U | Low width, poor e/w correction, high contrast picture going blue after a while. | IC606 9v regulator 7809 s/c. |
| KV29X5L | Stuck in standby with two blinks from front LED every few seconds. | Replace L.O.P transistor 2SD2539. S/C due to poor soldering of scan caps on collector |
| KV29X5U FE1 | Vol OSD perm displayed & front controls give wrong functs | Leaky front control switches - can be cleaned with luck/care |