SR575 v2.1 release notes

--------------------------

Version 2.1 of the SR575 program supports operation with the

National Instruments MC-GPIB card on IBM PS/2 computers. However,

as the SR5CON program does not include the MC-GPIB option, it

will be necessary to manually edit the SR575.CFG file. First run

SR5CON as if installing a National Instruments PCIII card. Then

use a text editor to edit the resulting SR575.CFG file, which

will look like this:

2    0   87   55C

49152  696  640    2

0   15   10   11   14

We want  to change the 3rd and 4th numbers in the second line so

the file looks like this:

2    0   87   55C

49152  696 3584    3

0   15   10   11   14

The number 3584 is decimal for E00 hex, the default  I/O address

of the MC-GPIB card. If you have installed your card at any other

address, use the decimal equivalent of that address.

The program is now ready to run with the MC-GPIB card.