Thurlby Thandar TG1010 Func. Generator

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for Thurlby Thandar TG1010

Func. Generator.

It contains functions for opening, configuring, taking measurement from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

-  the instrument is connected to the GPIB.

-  the GPIB address supplied to the initialize function must match the

GPIB address of the instrument.

-  there must be at least one unused device name in the handler.

________________________________________________________________________________

LabWindows Libraries:

In order to use this instrument driver outside of the LabWindows

environment, it must be linked with the following LabWindows libraries:

-  GPIB Library

-  Format and I/O Library

-  LabWindows System Library

National Instruments strongly recommends that you use the LWMAKE utility to

build any standalone executable program based on this instrument driver.

________________________________________________________________________________

Error Codes:

Error codes are returned as the return value of each instrument driver

function. A program should examine this value after each call to an instrument

driver function to determine if any error occurred. Possible error codes and

their meanings are listed with the corresponding instrument driver function.

________________________________________________________________________________

How To Use This Document:

This document is intended to be used as a programming reference manual.

It describes each function in the

Thurlby Thandar TG1010 Func. Generator

instrument driver. The functions are listed in alphabetical order with

a description of the function, C and BASIC syntax of the function,

a description of each parameter, and possible error codes.

________________________________________________________________________________

Function Tree Layout:

Thurlby Thandar TG1010 Func. Generator     Function Name:

Initialize                              tg1010_init

Configure

Main Generator                       tg1010_config_main

(main_offset)                         tg1010_main_offset

(main_symmetry)                       tg1010_main_symmetry

(main_phase)                          tg1010_main_phase

(main_zout)                           tg1010_main_zout

Modulation

AM and VCA                        tg1010_config_am

Sweep                             tg1010_config_sweep

FSK                               tg1010_fsk

Hop                               tg1010_hop

Trigger (Burst)                      tg1010_trig

Gate                                 tg1010_gate

Set Status                           tg1010_set_status

Send Trigger                            tg1010_send_trg

Local Storage                           tg1010_loc_stores

Get Status                              tg1010_get_status

Miscellaneous

Send command string                  tg1010_snd_cmd

Get Response string                  tg1010_get_resp

File operations                      tg1010_file_ops

Arbitary

New ARB/STAIRCASE                 tg1010_new_arb

ARB Local storage                 tg1010_arb_stores

Close                                   tg1010_close

Note:  The items in parentheses are secondary functions in the panels

under which they are listed.

________________________________________________________________________________

Instrument         : Thurlby Thandar TG1010 Func. Generator

Instrument Name: Thurlby Thandar TG1010 10MHz

DDS Function Generator.

Description:     This instrument module provides

programming support for the

TG1010.  The module is divided

into the following functions:

Functions/Classes:

(1) Initialize

Initialize the instrument and optionally

interrogate the ID and set to the specified

reset conditions.

(2) Configure...

This class of functions configures the main

generator and allows access

to the Modulation capabilities of the TG1010.

(1) Main Generator

Set up such TG1010 Main Generator

parameters as  output function,

frequency and amplitude.

(2) Modulation...

This class of functions configures and

controls the Modulation capabilities

of the TG1010.

(1) AM and VCA

Amplitude Modulation.

(2) Sweep

Software controlled Frequency.

(3) FSK

Frequency shift keying

(4) Hop

Set up and run Hop mode

(3) Trigger (Burst)

Set up and control the Trigger

capabilities of the TG1010.

(4) Gate

Set up and control the Trigger

capabilities of the TG1010.

(5) Set Status

Configure the IEEE 488.2 status

reporting capabilities of the TG1010.

(3) Local Storage

Executes the Store and Recall commands.

(4) Get Status

Returns the current value of the specified

IEEE 488.2 status byte.

(5) Miscellaneous

This class of functions allows access to

additional TG1010 remote control commands

which is not available elsewhere in this

driver. Also included are file operations.

(1) Send Command String

(2) Get Response String

(3) File Operations

(4) Arbitary

This class of functions gives access

to the arbitary waveform capabilities

of the TG1010

(1) New ARB

(2) ARb Local storage

(3) Define Staircase waveform

(6) Close

Take the instrument offline.

________________________________________________________________________________

Following functions are in alphabetical order.

________________________________________________________________________________

Function Name      : tg1010_arb_stores

C Syntax           : int tg1010_arb_stores(Instrument_ID, Operation,

Store_Number, Name);

int Instrument_ID;

int Operation;

int Store_Number;

char *Name;

BASIC Syntax       : Error% = tg1010.arb.stores(Instrument.ID%, Operation%, _

Store.Number%, Name$)

Purpose            :  Panel Name:    ARB Local storage

Description:   Allows ARB waveforms to be stored

and recalled from local

non-volatile memory. The current

ARB is always the last recalled or

downloaded.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Operation

0 - Recall (Default)

1 - Store

Control Name:  Operation

Description:   Selects the recall or store

operation.

Variable Type: Integer

Valid Range:   0 = Recall

1 = Store

Default Value: 0 - Recall

Store Number

Control name:   Store Number

Description:    Select the number of the store to

Recall from or Store to.

Variable type:  integer

Valid range:    1 to max number of stores

Default value:  1

Note: the max number of stores will depend upon

the software level of the instrument V1.1 has 21.

However only the first 5 are writeable and an

error will result if writing is attempted above

this number.

Name

Control name:   Name

Description:    The name to use for a store

operatrion.

Variable type:  string - max length 16 characters

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Operation value out of range

-3   Store number illegal

228   Error writing to instrument from file

229   Error reading from instrument to file

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1010_close

C Syntax           : int tg1010_close(Instrument_ID);

int Instrument_ID;

BASIC Syntax       : Error% = tg1010.close(Instrument.ID%)

Purpose            :  Panel Name:    Close

Description:   Closes the instrument and takes it

off line.

Note: The  instrument must  be reinitialized

to use it again.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

221   Unable to close instrument

232   Instrument not initialized

________________________________________________________________________________

Function Name      : tg1010_config_am

C Syntax           : int tg1010_config_am(Instrument_ID, Enable, Waveform,

Mod_Depth , Mod_Frequency_Hz);

int Instrument_ID;

int Enable;

int Waveform;

int Mod_Depth ;

double Mod_Frequency_Hz;

BASIC Syntax       : Error% = tg1010.config.am(Instrument.ID%, Enable%, _

Waveform%, Mod.Depth %, Mod.Frequency.Hz#)

Purpose            :  Panel Name:    AM and VCA

Description:   Enables AM and sets up Waveform,

Modulation Depth and Frequency.

If the modulation source is

external VCA mode is selected.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Enable

0 - Off (Default)

1 - On

2 - On + config

Control Name:  Enable

Description:   Sets the status of AM.

Variable Type: Integer

Valid Range:

0 = Off

1 = On

2 = On and configure

Default Value: 0 - Off

Note: If set to ON or OFF all other controls

are ignored, to use these you must use

On + config.

Waveform

1 - Square (Default)

0 - Sine (1kHz)

2 - External VCA

Control Name:  Waveform

Description:   Selects the modulation waveform.

Variable Type: Integer

Valid Range:   0 = Sine  (1kHz)

1 = Square

2 = External (VCA mode)

Default Value: 1

Note: If sinewave modulation is selected The

Modulation frequency (TGEN) will be fixed to

1kHz. Otherwise the TGEN will be left free for

other modes. Selecting sinewave causes the Mod

frequency control to be ignored.

Mod Depth (%)

Control Name:  Mod Depth

Description:   Sets the Depth of modulation

expressed as a percentage of the

carrier. This parameter has no

effect if the modulation Waveform

is External.

Variable Type: integer

Valid Range:   0% to 100%

Default Value: 30

Mod Frequency (Hz)

Control Name:  Mod Frequency

Description:   Sets the internal modulation

frequency in Hz.This parameter has

no effect if the modulation

Waveform is External.

Note. This parameter is actually

the internal Trigger generator

frequency.

Variable Type: double

Valid Range:   5e-3 to 50e+3 (5mHz to 50kHz)

Default Value: 1000

Note: The internal trigger generator period has

a setting resolution of 20us max. This

driver converts the requested frequency to a

period value and sends the resulting number to

the TG1010. This may cause a warning that the

20us resolution has been exceeded. This can

usually be ignored.

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Enable value out of range

-3   Waveform value out of range

-4   Mod depth value out of range

-5   Frequency value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1010_config_main

C Syntax           : int tg1010_config_main(Instrument_ID, Function, Output,

Frequency_Hz, PkPk_Output_V, Sense);

int Instrument_ID;

int Function;

int Output;

double Frequency_Hz;

double PkPk_Output_V;

int Sense;

BASIC Syntax       : Error% = tg1010.config.main(Instrument.ID%, Function%, _

Output%, Frequency.Hz#, PkPk.Output.V#, Sense%)

Purpose            :  Panel Name:    Main Generator

Description:   Sets up Function, Frequency,

Peak-Peak Output level and Output

status for the Main Generator.

Secondary functions are provided

to set DC Offset, Symmetry, Phase

and output impedance.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Function

0 - Sine (Default)

1 - Triangle

2 - Square

3 - Pos Pulse

4 - Neg Pulse

5 - Pos Ramp

6 - Neg Ramp

7 - Staircase

8 - ARB

9 - Noise

Control Name:  Function

Description:   Specifies the output waveform

function.

Variable Type: Integer

Valid Range:   0 = Sine

1 = Triangle

2 = Square

3 = Positive Pulse

4 = Negative Pulse

5 = Positive Ramp

6 = Negative Ramp

7 = Staircase

8 = ARB

9 = Noise

Default Value: 0 - Sine

Output

1 - On

0 - Off (Default)

Control Name:  Output

Description:   Turns the Main output on or off.

Variable Type: Integer

Valid Range:   1 = On

0 = Off

Default Value: 1 - On

Frequency (Hz)

Control Name:  Frequency

Description:   The output Frequency expressed in

Hz.

Variable Type: double

Valid Range:   1e-4 to 10e+6 (0.1mHz to 10MHz)

Default Value: 10kHz

Pk-Pk Output (V)

Control Name:  Pk-Pk Output

Description:   Specifies the output level as a

peak to peak EMF value expressed

in Volts.

Variable Type: double

Valid Range:   Waveform function dependent. See

TG1010 instruction manual for more

information.

Default Value: 20V

Sense

1 - Invert

0 - Normal (Default)

Control Name:  Sense

Description:   Sets the output sense to NORMAL or

INVERTed.

Variable Type: Integer

Valid Range:   1 = Invert

0 = Normal

Default Value: 0

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Function value out of range

-3   Output value out of range

-4   Frequency value out of range

-5   DC offset value out of range

-6   Symmetry value out of range

-7   Phase value out of range

-8   Zout value out of range

-9   Sense value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1010_config_sweep

C Syntax           : int tg1010_config_sweep(Instrument_ID, Enable, Mode,

Law, Time_sec, Start_frequency_Hz,

Stop_frequency_Hz, Trigger_Source, Marker_B,

Marker_A);

int Instrument_ID;

int Enable;

int Mode;

int Law;

double Time_sec;

double Start_frequency_Hz;

double Stop_frequency_Hz;

int Trigger_Source;

double Marker_B;

double Marker_A;

BASIC Syntax       : Error% = tg1010.config.sweep(Instrument.ID%, Enable%, _

Mode%, Law%, Time.sec#, Start.frequency.Hz#, _

Stop.frequency.Hz#, Trigger.Source%, Marker.B#, _

Marker.A#)

Purpose            :  Panel Name:    Sweep

Description:   Enables Sweep (Software Controlled

Frequency) and sets up the Mode,

Start and Stop Frequencies and the

sweep time.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Enable

0 - Off (Default)

1 - On

2 - On + config

Control Name:  Enable

Description:   Sets the status of Sweep.

Variable Type: Integer

Valid Range:

0 = Off

1 = On

2 = On and configure

Default Value: 0 - Off

Note: If set to ON or OFF all other controls

are ignored, to use these you must use

On + config.

Mode

1 - Start - Stop (Default)

0 - Stop - Start

Control Name:  Mode

Description:   Sets the Sweep mode.

Variable Type: Integer

Valid Range:   1 = Start - Stop

0 = Stop - Start

Default Value: 1 - Start - Stop

Note: Setting this control to 1 will cause the

sweep to go from the Start frequency to the Stop

frequency. 0 will reverse the direction.

Law

1 - Log

0 - Lin (Default)

Control Name:  Law

Description:   Sets the Sweep Law.

Variable Type: Integer

Valid Range:   0 = Linear

1 = Logarithmic

Default Value: 0 - linear

Time (sec)

Control Name:  Time

Description:   Specifies the internal Sweep ramp

time in seconds.

Variable Type: double

Valid Range:   10e-3 to 999

Default Value: none

Start frequency (Hz)

Control Name:  Start Frequency

Description:   The Sweep Start Frequency

expressed in Hz.

Variable Type: double

Valid Range:   1e-4 to 10e+6 (0.1mHz to 10MHz)

Default Value: none

Stop frequency (Hz)

Control Name:  Stop Frequency

Description:   The Sweep Stop Frequency

expressed in Hz.

Variable Type: double

Valid Range:   1e-4 to 10e+6 (0.1mHz to 10MHz)

Default Value: none

Trigger Source

0 - Continuous (Default)

1 - Man/Remote

2 - External

Control Name:  Trigger Source

Description:   Sets the Sweep Trigger source.

Variable Type: Integer

Valid Range:   0 = Continuous

1 = Man/Remote

2 = External

Default Value: 0 - Continuous

Marker B

Control Name:  Marker B

Description:   The Frequency for Marker B

expressed in Hz.

Variable Type: double

Valid Range:   1e-4 to 10e+6 (0.1mHz to 10MHz)

Default Value: none

Note: This is the Main generator frequency.

Marker A

Control Name:  Marker A

Description:   The Frequency for Marker A

expressed in Hz.

Variable Type: double

Valid Range:   1e-4 to 10e+6 (0.1mHz to 10MHz)

Default Value: none

Note: This is the Marker frequency set on the

sweep menu.

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Enable value out of range

-3   Mode value out of range

-4   Law value out of range

-5   Time value out of range

-6   Start frequency value out of range

-7   Stop frequency value out of range

-8   Trigger source value out of range

-9   Marker A frequency value out of range

-10   Marker B frequency value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1010_file_ops

C Syntax           : int tg1010_file_ops(Instrument_ID, Operation,

File_Name);

int Instrument_ID;

int Operation;

char *File_Name;

BASIC Syntax       : Error% = tg1010.file.ops(Instrument.ID%, Operation%, _

File.Name$)

Purpose            : Panel name:     File Operations

Description:    Send or Receive a binary block to

or from the TG1010. The data will

be read from or written to a file.

Note            This operation is usefull for

LRN blocks.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Operation

0 - Send File (Default)

1 - Get File

Control Name:  Operation

Description:   Selects to send a file to the

TG1010 or Get a file from TG1010.

Variable Type: Integer

Valid Range:   0 = Send file to TG1010

1 = Get file from TG1010

Default Value: 0 - Send file

File Name

Control name:   File name

Description:    This control specifies file name

from which the binary block will

be sent to the TG1010.

Variable type:  String - max length 50 characters

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Operation value out of range

228   Error writing to instrument from file

229   Error reading from instrument to file

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1010_fsk

C Syntax           : int tg1010_fsk(Instrument_ID, Enable, Trigger_Source,

Frequency_A, Frequency_B, Internal_Generator);

int Instrument_ID;

int Enable;

int Trigger_Source;

double Frequency_A;

double Frequency_B;

double Internal_Generator;

BASIC Syntax       : Error% = tg1010.fsk(Instrument.ID%, Enable%, _

Trigger.Source%, Frequency.A#, Frequency.B#, _

Internal.Generator#)

Purpose            :  Panel Name:    FSK

Description:   Enables FSK and sets up parameters

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Enable

0 - Off (Default)

1 - On

2 - On + config

Control Name:  Enable

Description:   Sets the status of FSK.

Variable Type: Integer

Valid Range:

0 = Off

1 = On

2 = On and configure

Default Value: 0 - Off

Note: If set to ON or OFF all other controls

are ignored, to use these you must use

On + config.

Trigger Source

0 - Internal (Default)

1 - Man/Remote

2 - External

Control Name:  Trigger Source

Description:   Sets the FSK Trigger source.

Variable Type: Integer

Valid Range:   0 = Internal

1 = Man/Remote

2 = External

Default Value: 0 - Internal

Frequency A

Control Name:  Frequency A

Description:   FSK frequency A, actually

the main generator frequency,

expressed in Hz.

Variable Type: double

Valid Range:   1e-4 to 10e+6 (0.1mHz to 10MHz)

Default Value: none

Frequency B

Control Name:  Stop Frequency

Description:   FSK frequency B expressed in Hz.

Variable Type: double

Valid Range:   1e-4 to 10e+6 (0.1mHz to 10MHz)

Default Value: none

Internal Generator

Control Name:  Internal Generator

Description:   Sets the internal trigger

frequency in Hz.

Note. This parameter is actually

the internal Trigger generator

frequency (TGEN).

Variable Type: double

Valid Range:   5e-3 to 50e+3 (5mHz to 50kHz)

Default Value: 1000

Note: The internal trigger generator period has

a setting resolution of 20us max. This

driver converts the requested frequency to a

period value and sends the resulting number to

the TG1010. This may cause a warning that the

20us resolution has been exceeded. This can

usually be ignored.

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Enable value out of range

-3   Trigger Source value out of range

-4   FSK frequency A value out of range

-5   FSK frequency B value out of range

-6   Internal generator value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1010_gate

C Syntax           : int tg1010_gate(Instrument_ID, Enable, Trigger_Source,

Internal_Generator);

int Instrument_ID;

int Enable;

int Trigger_Source;

double Internal_Generator;

BASIC Syntax       : Error% = tg1010.gate(Instrument.ID%, Enable%, _

Trigger.Source%, Internal.Generator#)

Purpose            :  Panel Name:    Gate

Description:   This panel allows access to the

Gating function of the TG1010

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Enable

0 - Off (Default)

1 - On

2 - On + config

Control Name:  Enable

Description:   Sets the status of Gate.

Variable Type: Integer

Valid Range:

0 = Off

1 = On

2 = On and configure

Default Value: 0 - Off

Note: If set to ON or OFF all other controls

are ignored, to use these you must use

On + config.

Trigger Source

0 - Internal (Default)

1 - Man/Remote

2 - External

Control Name:  Trigger Source

Description:   Sets the Gating trigger source.

Variable Type: Integer

Valid Range:   0 = Internal

1 = Man/Remote

2 = External

Default Value: 0 - Internal

Internal Generator

Control Name:  Internal Generator

Description:   Sets the internal trigger

frequency in Hz.

Note. This parameter is actually

the internal Trigger generator

frequency (TGEN).

Variable Type: double

Valid Range:   5e-3 to 50e+3 (5mHz to 50kHz)

Default Value: 1000

Note: The internal trigger generator period has

a setting resolution of 20us max. This

driver converts the requested frequency to a

period value and sends the resulting number to

the TG1010. This may cause a warning that the

20us resolution has been exceeded. This can

usually be ignored.

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Enable value out of range

-3   Trigger source value out of range

-4   Internal generator value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1010_get_resp

C Syntax           : int tg1010_get_resp(Instrument_ID, Response,

Number_Of_Bytes);

int Instrument_ID;

char *Response;

int *Number_Of_Bytes;

BASIC Syntax       : Error% = tg1010.get.resp(Instrument.ID%, Response$, _

Number.Of.Bytes%)

Purpose            : Panel name:     Get response string

Description:    Reads a response from the TG1010

into a string variable.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Response

Control name:   Response Stirng

Description:    This control specifies a string

variable to receive the response

from the TG1010. The string must

be long enough to receive the

response to avoid overflow. To aid

in checking this the number of

bytes put into the string is

returned from the function. The

absolute maximum length allowed is

2000 bytes.

Variable type:  String

Number Of Bytes

Control name:   Number of Bytes

Description:    This value is the number of bytes

returned by the TG1010 and

placed in the string.

Variable type:  Integer

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1010_get_status

C Syntax           : int tg1010_get_status(Instrument_ID, Read_Status,

Value);

int Instrument_ID;

int Read_Status;

int *Value;

BASIC Syntax       : Error% = tg1010.get.status(Instrument.ID%, Read.Status%, _

Value%)

Purpose            : Panel name:     Read Status

Description:    Returns the byte value from the

selected status register.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Read Status

0 - Status byte (Default)

1 - Std Event

2 - SRQ enable

3 - Event Enable

4 - Parallel Pol

5 - Exeq. error

Control name:   Read Status

Description:    Used to select which status

register to read

Variable type:  integer

Valid range:    0 = Status Byte

1 = Standard Event Status

2 = SRQ enable

3 = Standard Event Enable

4 = Parallel Poll Enable

5 = Execution Error

Default value   0 - Status Byte

Value

Control name:   Value

Description:    Returns the value read from the

selected register.

Variable type:  integer

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Status Enable value out of range

-3   Register Value out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1010_hop

C Syntax           : int tg1010_hop(Instrument_ID, Enable, Step_Number,

Function, Frequency_Hz, PkPk_Output_V,

DC_Offset_V, Last_Step, Time_sec);

int Instrument_ID;

int Enable;

int Step_Number;

int Function;

double Frequency_Hz;

double PkPk_Output_V;

double DC_Offset_V;

int Last_Step;

double Time_sec;

BASIC Syntax       : Error% = tg1010.hop(Instrument.ID%, Enable%, _

Step.Number%, Function%, Frequency.Hz#, _

PkPk.Output.V#, DC.Offset.V#, Last.Step%, _

Time.sec#)

Purpose            :  Panel Name:    Hop

Description:   Enables HOP or sets up

Parameters for each step

Note: To avoid possible bus lockouts you

should not execute any other panels or functions

from this driver (except the Send Trigger panel)

while Hop is running. Always set enable to OFF

and execute this pannel first.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Enable

0 - Off

1 - Run

2 - Config Step (Default)

Control Name:  Enable

Description:   Sets the status/configuration of

Hop.

Variable Type: Integer

Valid Range:

0 = Off

1 = On

2 = Configure step

Default Value: 2 - Configure step

Note: If set to OFF all other controls

are ignored, RUN uses only the Last step control,

Config step uses all other controls.

Note: To avoid possible bus lockouts you

should not execute any other panels or functions

from this driver (except the Send Trigger panel)

while Hop is running. Always set enable to OFF

and execute this pannel first.

Step Number

Control Name:  Step number

Description:   The step to configure

Variable Type: Integer

Valid Range:   0 to 15

Default Value: 0

Function

0 - Sine (Default)

1 - Triangle

2 - Square

3 - Pos Pulse

4 - Neg Pulse

5 - Pos Ramp

6 - Neg Ramp

7 - Staircase

8 - ARB

Control Name:  Function

Description:   Specifies the output waveform

function.

Variable Type: Integer

Valid Range:   0 = Sine

1 = Triangle

2 = Square

3 = Positive Pulse

4 = Negative Pulse

5 = Positive Ramp

6 = Negative Ramp

7 = Staircase

8 = ARB

Default Value: 0 - Sine

Frequency (Hz)

Control Name:  Frequency

Description:   The output Frequency expressed in

Hz.

Variable Type: double

Valid Range:   1e-4 to 10e+6 (0.1mHz to 10MHz)

Default Value: 1kHz

Pk-Pk Output (V)

Control Name:  Pk-Pk Output

Description:   Specifies the output level as a

peak to peak EMF value expressed

in Volts.

Variable Type: double

Valid Range:   Waveform function dependent. See

TG1010 instruction manual for more

information.

Default Value: 20V

DC Offset (V)

Control Name:  DC Offset

Description:   Sets the output Offset voltage as

an EMF value expressed in Volts

Variable Type: double

Valid Range:   -10 to +10

Default Value: 0.0

Last Step

Control Name:  Last Step

Description:   The last step in the sequence to

execute.

Variable Type: Integer

Valid Range:   0 to 15

Default Value: 1

Time (sec)

Control Name:  Time

Description:   The time for this step in seconds.

0 and 1ms have special meanings,

see below

Variable Type: Double

Valid Range:   0ms = Man/Remote

1ms = External

2ms to 65s = time duration

Default Value: 1 sec

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Enable value out of range

-3   Step value out of range

-4   frequency value out of range

-5   DC offset value out of range

-6   Last step value out of range

-7   time value out of range

-8   Function vaqlue out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

Note: To avoid possible bus lockouts you

should not execute any other panels or functions

from this driver (except the Send Trigger panel)

while Hop is running. Always set enable to OFF

and execute this pannel first.

________________________________________________________________________________

Function Name      : tg1010_init

C Syntax           : int tg1010_init(Address, ID_Query, Reset_Device,

Instrument_ID);

int Address;

int ID_Query;

int Reset_Device;

int *Instrument_ID;

BASIC Syntax       : Error% = tg1010.init(Address%, ID.Query%, Reset.Device%, _

Instrument.ID%)

Purpose            :  Panel Name:    Initialize

Description:   Initializes the instrument in the

following ways:

- opens the instrument and sets

its GPIB address to the speci-

fied value

- optionally performs an

identification query on the

instrument

- optionally sets the instrument

to a default configuration

- returns an instrument ID which

is used to identify the

instrument in subsequent calls.

Parameter List     : Address

Control Name:  Address

Description:   Specifies the address on the

GPIB to be used to communicate

with the instrument.  This value

must correspond to the current

internal setting of the

instrument.

Variable Type: Integer

Valid Range:   0 to 30

Default Value: 5

ID Query

1 - Do Query (Default)

0 - Skip Query

Control Name:  ID Query

Description:   Specifies if an ID Query is sent

to the instrument during the

initialization procedure.

Variable Type: Integer

Valid Range:   1 = Yes

0 = No

Default Value: 1 - Yes

NOTE:  Under normal circumstances the ID Query

insures that the instrument initialized

over the GPIB is the type supported by

this driver. However circumstances may

arise where it is undesirable to send an

ID Query to the instrument. In those

cases; set this control to Skip Query

and this function will initialize the

GPIB and the Command arrays in the driver,

without doing an ID Query.

Reset Device

1 - Yes (Default)

0 - No

Control Name:  Reset

Description:   Specifies if the instrument is to

be reset during the initialization

procedure.

Variable Type: Integer

Valid Range:   0 = No Reset

1 = Reset device

Default Value: 1 = Reset

NOTE: If you do not want the instrument reset.

Set this control to No Reset while

initializing the instrument.

Instrument ID

Control Name:  Instrument ID

Description:   Returns an Instrument ID that is

used in all subsequent function

calls to select the instrument at

the Address specified.

If more than one instrument of the

same type is used, this value will

be used to differentiate between

them. The first instrument

initialized will be assigned an

Instrument ID of 1.

Variable Type: Integer

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   GPIB Address out of range

-2   ID Query out of range

-3   Reset Device out of range

220   Unable to open instrument

221   Unable to close instrument

223   Instrument identification query failed

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1010_loc_stores

C Syntax           : int tg1010_loc_stores(Instrument_ID, Operation,

Store_Number);

int Instrument_ID;

int Operation;

int Store_Number;

BASIC Syntax       : Error% = tg1010.loc.stores(Instrument.ID%, Operation%, _

Store.Number%)

Purpose            : Panel name:     Local Storage

Description:    Stores or Recalls the complete

state of the TG1010 to or from

local non-volatile memory.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Operation

0 - Recall (Default)

1 - Store

Control Name:  Operation

Description:   Selects the recall or store

operation.

Variable Type: Integer

Valid Range:   0 = Recall

1 = Store

Default Value: 0 - Recall

Store Number

Control name:   Store Number

Description:    Select the number of the store to

Recall from or Store to.

Variable type:  integer

Valid range:    0 to 9

Default value:  1

NOTE:  Store 0 is read only and contains the

default parameters as used in the *RST command.

Attempting to write to store 0 will generate an

error.

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Operation value out of range

-3   Store Number out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1010_main_offset

C Syntax           : void tg1010_main_offset(Instrument_ID, DC_Offset_V);

int Instrument_ID;

double DC_Offset_V;

BASIC Syntax       : CALL tg1010.main.offset(Instrument.ID%, DC.Offset.V#)

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

DC Offset (V)

Control Name:  DC Offset

Description:   Sets the output Offset voltage as

an EMF value expressed in Volts

Variable Type: double

Valid Range:   -10 to +10

Default Value: 0.0

________________________________________________________________________________

Function Name      : tg1010_main_phase

C Syntax           : void tg1010_main_phase(Instrument_ID, Phase_deg);

int Instrument_ID;

int Phase_deg;

BASIC Syntax       : CALL tg1010.main.phase(Instrument.ID%, Phase.deg%)

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Phase (deg)

Control Name:  Phase

Description:   Sets the Start/Stop Phase

parameter expressed in degrees.

Variable Type: integer

Valid Range:   -360 to +360

Default Value: 0

________________________________________________________________________________

Function Name      : tg1010_main_symmetry

C Syntax           : void tg1010_main_symmetry(Instrument_ID, Symmetry );

int Instrument_ID;

double Symmetry ;

BASIC Syntax       : CALL tg1010.main.symmetry(Instrument.ID%, Symmetry #)

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Symmetry (%)

Control Name:  Symmetry

Description:   Sets the output Symmetry as a

percentage.

Variable Type: double

Valid Range:   0.1% to 99.9%

Default Value: 50.0%

Note: The symmetry can always be specified over

the range specified above but some waveform

functions impose temporary limit of their own,

see the instruction manual for more information.

________________________________________________________________________________

Function Name      : tg1010_main_zout

C Syntax           : void tg1010_main_zout(Instrument_ID, Zout);

int Instrument_ID;

int Zout;

BASIC Syntax       : CALL tg1010.main.zout(Instrument.ID%, Zout%)

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Zout

1 - 600 Ohm

0 - 50 Ohm (Default)

Control Name:  Zout

Description:   Set the output impedance.

Variable Type: Integer

Valid Range:   1 = 600 Ohms

0 =  50 Ohms

Default Value: 0

________________________________________________________________________________

Function Name      : tg1010_new_arb

C Syntax           : int tg1010_new_arb(Instrument_ID, File_Name);

int Instrument_ID;

char *File_Name;

BASIC Syntax       : Error% = tg1010.new.arb(Instrument.ID%, File.Name$)

Purpose            :  Panel Name:    New ARB/STAIRCASE

Description:   Allows a new arbitary

or Staircase waveform to

be downloaded to the TG1010. After

downloading a new ARB should be

saved to ARB Local storage to

avoid it being overwritten.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

File Name

Control name:   File name

Description:    This control specifies file name

from which the waveform data will

be sent to the TG1010.

Variable type:  String - max length 50 characters

Note; The file format will be as follows

ARB

SETARB followed by a space and then

1024 integers comma delimited

valid range -512 to +511

Stair

SETSTAIR followed by a space and then

1 to 16 pairs of integers length,level

comma delimited

valid range length 0 to 1024

level  -512 to +511

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Operation value out of range

228   Error writing to instrument from file

229   Error reading from instrument to file

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1010_send_trg

C Syntax           : int tg1010_send_trg(Instrument_ID);

int Instrument_ID;

BASIC Syntax       : Error% = tg1010.send.trg(Instrument.ID%)

Purpose            : Panel name:     Send Trigger

Description:    Send a trigger command to the

TG1010. If MAN/REMOTE is selected

as the trigger source for any

active mode then that mode will be

triggered by this command. If no

modes are active then the command

will be ignored.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Status Enable value out of range

-3   Register Value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1010_set_status

C Syntax           : int tg1010_set_status(Instrument_ID, Set_Status_Enable,

Value);

int Instrument_ID;

int Set_Status_Enable;

int Value;

BASIC Syntax       : Error% = tg1010.set.status(Instrument.ID%, _

Set.Status.Enable%, Value%)

Purpose            : Panel name:     Status Reporting

Description:    Specify the way in which status is

reported and Service Request is

generated as defined in the IEEE

488.2 standard.

The function provides access to

the three status enable registers

in the TG1010, these are:

Service Request Enable Register

- Specifies which bits, when set,

in the Status Byte register will

generate a Service Request.

Standard Event Enable Register

- Specifies which bits, when set,

in the Standard Event Status

register will set the ESB bit in

the Status Byte Register.

Parallel Poll Enable Register

- Specifies which bits, when set,

in the Status Byte register will

be used to form the ist local

message

See the IEEE 488.2 Standard and

the instrument  instruction book

for a full description of status

reporting capabilities.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Set Status Enable

0 - SRQ enable (Default)

1 - Event Enable

2 - Parallel Pol

Control name:   Set Status Enable

Description:    Used to select which enable

register will receive the byte

value.

Variable type:  integer

Valid range:    0 = SRQ enable

1 = Standard Event Enable

2 = Parallel Poll

Default value   0 - SRQ Enable

Value

Control name:   Value

Description:    A byte value which will be written

to the selected enable register.

Variable type:  integer

Valid range:    0 to 255

Default value:  0

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Status Enable value out of range

-3   Register Value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1010_snd_cmd

C Syntax           : int tg1010_snd_cmd(Instrument_ID, Command);

int Instrument_ID;

char *Command;

BASIC Syntax       : Error% = tg1010.snd.cmd(Instrument.ID%, Command$)

Purpose            : Panel name:     Send command string

Description:    Sends a string of bytes to the

TG1010 from a string variable.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Command

Control name:   Command String

Description:    This control specifies data to be

sent to the TG1010. The absolute

maximum length allowed is 200

bytes.

Variable type:  String

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1010_trig

C Syntax           : int tg1010_trig(Instrument_ID, Enable, Trigger_Source,

Burst_Count, Internal_Generator, Phase_deg);

int Instrument_ID;

int Enable;

int Trigger_Source;

double Burst_Count;

double Internal_Generator;

int Phase_deg;

BASIC Syntax       : Error% = tg1010.trig(Instrument.ID%, Enable%, _

Trigger.Source%, Burst.Count#, _

Internal.Generator#, Phase.deg%)

Purpose            :  Panel Name:    Trigger (Burst)

Description:   This panel allows access to the

Trigger function of the TG1010

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Enable

0 - Off (Default)

1 - On

2 - On + config

Control Name:  Enable

Description:   Sets the status of Trigger.

Variable Type: Integer

Valid Range:

0 = Off

1 = On

2 = On and configure

Default Value: 0 - Off

Note: If set to ON or OFF all other controls

are ignored, to use these you must use

On + config.

Trigger Source

0 - Internal (Default)

1 - Man/Remote

2 - External

Control Name:  Trigger Source

Description:   Sets the Trigger source.

Variable Type: Integer

Valid Range:   0 = Internal

1 = Man/Remote

2 = External

Default Value: 0 - Internal

Burst Count

Control Name:  Burst Count

Description:   Sets the number of cycles of the

main generator waveform in each

burst.

Variable Type: double

Valid Range:   0.5 to 1023

Default Value: 1.0

Internal Generator

Control Name:  Internal Generator

Description:   Sets the internal trigger

frequency in Hz.

Note. This parameter is actually

the internal Trigger generator

frequency (TGEN).

Variable Type: double

Valid Range:   5e-3 to 50e+3 (5mHz to 50kHz)

Default Value: 1000

Note: The internal trigger generator period has

a setting resolution of 20us max. This

driver converts the requested frequency to a

period value and sends the resulting number to

the TG1010. This may cause a warning that the

20us resolution has been exceeded. This can

usually be ignored.

Phase (deg)

Control Name:  Phase

Description:   Sets the Start/Stop Phase

parameter expressed in degrees.

Variable Type: integer

Valid Range:   -360 to +360

Default Value: 0

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Enable value out of range

-3   Trigger source value out of range

-4   Burst count value out of range

-5   Internal generator value out of range

-6   Phase value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out