Thurlby Thandar TG1304 Func. Generator

By John Tothill

First issue V1.0 November 1994

Revised V1.1 November 1994

Errors in some help panels corrected. Errors in the DOC file corrected.

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for Thurlby Thandar TG1304

Func. Generator.

It contains functions for opening, configuring, taking measurement from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

-  the instrument is connected to the GPIB.

-  the GPIB address supplied to the initialize function must match the

GPIB address of the instrument.

-  there must be at least one unused device name in the handler.

________________________________________________________________________________

LabWindows Libraries:

In order to use this instrument driver outside of the LabWindows

environment, it must be linked with the following LabWindows libraries:

-  GPIB Library

-  Format and I/O Library

-  LabWindows System Library

National Instruments strongly recommends that you use the LWMAKE utility to

build any standalone executable program based on this instrument driver.

________________________________________________________________________________

Error Codes:

Error codes are returned as the return value of each instrument driver

function. A program should examine this value after each call to an instrument

driver function to determine if any error occurred. Possible error codes and

their meanings are listed with the corresponding instrument driver function.

________________________________________________________________________________

How To Use This Document:

This document is intended to be used as a programming reference manual.

It describes each function in the Thurlby Thandar TG1304 Func. Generator

instrument driver. The functions are listed in alphabetical order with

a description of the function, C and BASIC syntax of the function,

a description of each parameter, and possible error codes.

________________________________________________________________________________

Function Tree Layout:

Thurlby Thandar TG1304 Func. Generator     Function Name:

Initialize                              tg1304_init

Configure

Main Generator                       tg1304_config_main

(main_offset)                         tg1304_main_offset

(main_symmetry)                       tg1304_main_symmetry

(main_phase)                          tg1304_main_phase

Auxiliary Generator                  tg1304_config_aux

(aux_symmetry)                        tg1304_aux_symmetry

Modulation

AM                                tg1304_config_am

VCA                               tg1304_config_vca

Sum                               tg1304_config_sum

FM                                tg1304_config_fm

Sweep (VCF)                       tg1304_config_sweep

Trigger (Burst/Gate)                 tg1304_trig

Set Status                           tg1304_set_status

Local Storage                           tg1304_loc_stores

Get Status                              tg1304_get_status

Miscellaneous

Send command string                  tg1304_snd_cmd

Get Response string                  tg1304_get_resp

File operations                      tg1304_file_ops

Reset                                tg1304_reset

Close                                   tg1304_close

Note:  The items in parentheses are secondary functions in the panels

under which they are listed.

________________________________________________________________________________

Instrument         : Thurlby Thandar TG1304 Func. Generator

Instrument Name: Thurlby Thandar TG1304 13MHz

Function Generator.

Description:     This instrument module provides

programming support for the

TG1304.  The module is divided

into the following functions:

Functions/Classes:

(1) Initialize

Initialize the instrument and optionally

interrogate the ID and set to the specified

reset conditions.

(2) Configure...

This class of functions configures the main

and Auxilliary generators and allows access

to the Modulation capabilities of the TG1304.

(1) Main Generator

Set up such TG1304 Main Generator

parameters as  output function,

frequency and amplitude.

(2) Auxiliary Generator

Set up such TG1304 Auxiliary Generator

parameters as  output function,

frequency and amplitude.

(3) Modulation...

This class of functions configures and

controls the Modulation capabilities

of the TG1304.

(1) AM

Amplitude Modulation.

(2) VCA

Voltage Controlled Attenuation.

(3) Sum

Sum the Main and Auxiliary

generator outputs.

(4) FM

Frequency Modulation.

(5) VCF (Sweep)

Voltage controlled Frequency.

(4) Trigger (Burst/Gate)

Set up and control the Trigger and

Gate capabilities of the TG1304.

(5) Set Status

Configure the IEEE 488.2 status

reporting capabilities of the TG1304.

(3) Local Storage

Executes the Store and Recall commands.

(4) Get Status

Returns the current value of the specified

IEEE 488.2 status byte.

(5) Miscellaneous

This class of functions allows access to

additional TG1304 remote control commands

which is not available elsewhere in this

driver. Also included are file operations and

the user defined reset function which is

similar to that provided in the Initialise

function.

(1) Send Command String

(2) Get Response String

(3) File Operations

(4) Reset

(6) Close

Take the instrument offline.

________________________________________________________________________________

Following functions are in alphabetical order.

________________________________________________________________________________

Function Name      : tg1304_aux_symmetry

C Syntax           : void tg1304_aux_symmetry(Instrument_ID, Symmetry );

int Instrument_ID;

int Symmetry ;

BASIC Syntax       : CALL tg1304.aux.symmetry(Instrument.ID%, Symmetry %)

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Symmetry (%)

Description:   Sets the output Symmetry as a

percentage.

Variable Type: integer

Valid Range:   5% to 95%

Default Value: 50

________________________________________________________________________________

Function Name      : tg1304_close

C Syntax           : int tg1304_close(Instrument_ID);

int Instrument_ID;

BASIC Syntax       : Error% = tg1304.close(Instrument.ID%)

Purpose            :  Panel Name:    Close

Description:   Closes the instrument and takes it

off line.

Note: The  instrument must  be reinitialized

to use it again.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

221   Unable to close instrument

232   Instrument not initialized

________________________________________________________________________________

Function Name      : tg1304_config_am

C Syntax           : int tg1304_config_am(Instrument_ID, Enable, Waveform,

Mod_Depth , Mod_Frequency_Hz);

int Instrument_ID;

int Enable;

int Waveform;

int Mod_Depth ;

double Mod_Frequency_Hz;

BASIC Syntax       : Error% = tg1304.config.am(Instrument.ID%, Enable%, _

Waveform%, Mod.Depth %, Mod.Frequency.Hz#)

Purpose            :  Panel Name:    AM

Description:   Enables AM and sets up Waveform,

Modulation Depth and Frequency.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Enable

0 - On (Default)

1 - Off

2 - Clear

Description:   Sets the status of AM.

Variable Type: Integer

Valid Range:   0 = On  - configured and active

1 = Off - configured and inactive

2 = Clear - Unconfigured

Default Value: 0 - On

Waveform

0 - Sine (Default)

1 - Triangle

2 - Square

3 - External

Description:   Selects the modulation waveform.

Variable Type: Integer

Valid Range:   0 = Sine

1 = Triangle

2 = Square

3 = External

Default Value: 0 - Sine

Mod Depth (%)

Description:   Sets the Depth of modulation

expressed as a percentage of the

carrier. This parameter has no

effect if the modulation Waveform

is External.

Variable Type: integer

Valid Range:   1% to 99%

Default Value: none

Mod Frequency (Hz)

Description:   Sets the internal modulation

frequency in Hz.This parameter has

no effect if the modulation

Waveform is External.

Note. This parameter is actually

the Auxiliary generator frequency.

Variable Type: double

Valid Range:   5e-3 to 50e+3 (5mHz to 50kHz)

Default Value: none

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Enable value out of range

-3   Waveform value out of range

-4   Mod depth value out of range

-5   Frequency value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1304_config_aux

C Syntax           : int tg1304_config_aux(Instrument_ID, Function, Output,

Frequency_Hz, PkPk_Output_V);

int Instrument_ID;

int Function;

int Output;

double Frequency_Hz;

double PkPk_Output_V;

BASIC Syntax       : Error% = tg1304.config.aux(Instrument.ID%, Function%, _

Output%, Frequency.Hz#, PkPk.Output.V#)

Purpose            :  Panel Name:    Auxiliary Generator

Description:   Sets up Function, Frequency,

Peak-Peak Output level and Output

status for the Auxiliary

Generator. A secondary functiom

is provided to set Symmetry.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Function

0 - Sine (Default)

1 - Triangle

2 - Square

3 - Ramp

4 - FCV

5 - ACV

Description:   Specifies the output waveform

function.

Variable Type: Integer

Valid Range:   0 = Sine

1 = Triangle

2 = Square

3 = Ramp

4 = FCV

5 = ACV

Default Value: 0 - Sine

Output

1 - On (Default)

0 - Off

Description:   Turns the Aux. output on or off.

Variable Type: Integer

Valid Range:   1 = On

0 = Off

Default Value: 1 - On

Frequency (Hz)

Description:   The output Frequency expressed in

Hz.

Variable Type: double

Valid Range:   5e-3 to 50e+3 (5mHz to 50KHz)

Default Value: none

Pk-Pk Output (V)

Description:   Specifies the output level as a

peak to peak EMF value expressed

in Volts.

Variable Type: double

Valid Range:   Waveform function dependent. See

the TG1304 specification for

details.

Default Value: none

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Function value out of range

-3   Output value out of range

-4   Frequency value out of range

-5   Symmetry value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1304_config_fm

C Syntax           : int tg1304_config_fm(Instrument_ID, Enable, Waveform,

Deviation_Hz, Mod_Frequency_Hz);

int Instrument_ID;

int Enable;

int Waveform;

double Deviation_Hz;

double Mod_Frequency_Hz;

BASIC Syntax       : Error% = tg1304.config.fm(Instrument.ID%, Enable%, _

Waveform%, Deviation.Hz#, Mod.Frequency.Hz#)

Purpose            :  Panel Name:    FM

Description:   Enables FM and sets up Waveform,

Deviation and Frequency.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Enable

0 - On (Default)

1 - Off

2 - Clear

Description:   Sets the status of FM.

Variable Type: Integer

Valid Range:   0 = On  - configured and active

1 = Off - configured and inactive

2 = Clear - Unconfigured

Default Value: 0 - On

Waveform

0 - Sine (Default)

1 - Triangle

2 - Square

3 - External

Description:   Selects the modulation waveform.

Variable Type: Integer

Valid Range:   0 = Sine

1 = Triangle

2 = Square

3 = External

Default Value: 0 - Sine

Deviation (Hz)

Description:   Sets the frequency deviation

of the carrier in Hz. This

parameter has no effect if the

modulation Waveform is External.

Variable Type: double

Valid Range:   Carrier wave frequency dependant.

Default Value: none

Mod Frequency (Hz)

Description:   Sets the internal modulation

frequency in Hz.This parameter has

no effect if the modulation

Waveform is External.

Note. This parameter is actually

the Auxiliary generator frequency.

Variable Type: double

Valid Range:   5e-3 to 50e+3 (5mHz to 50kHz)

Default Value: none

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Enable value out of range

-3   Waveform value out of range

-4   Frequency value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1304_config_main

C Syntax           : int tg1304_config_main(Instrument_ID, Function, Output,

Frequency_Hz, PkPk_Output_V);

int Instrument_ID;

int Function;

int Output;

double Frequency_Hz;

double PkPk_Output_V;

BASIC Syntax       : Error% = tg1304.config.main(Instrument.ID%, Function%, _

Output%, Frequency.Hz#, PkPk.Output.V#)

Purpose            :  Panel Name:    Main Generator

Description:   Sets up Function, Frequency,

Peak-Peak Output level and Output

status for the Main Generator.

Secondary functions are provided

to set DC Offset, Symmetry and

Phase.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Function

0 - Sine (Default)

1 - Triangle

2 - Square

3 - Pos Pulse

4 - Neg Pulse

5 - Pos Peak

6 - Neg Peak

7 - DC

Description:   Specifies the output waveform

function.

Variable Type: Integer

Valid Range:   0 = Sine

1 = Triangle

2 = Square

3 = Positive Pulse

4 = Negative Pulse

5 = Positive Peak

6 = Negative Peak

7 = DC

Default Value: 0 - Sine

Output

1 - On (Default)

0 - Off

Description:   Turns the Main output on or off.

Variable Type: Integer

Valid Range:   1 = On

0 = Off

Default Value: 1 - On

Frequency (Hz)

Description:   The output Frequency expressed in

Hz.

Variable Type: double

Valid Range:   1e-2 to 13e+6 (10mHz to 13MHz)

Default Value: none

Pk-Pk Output (V)

Description:   Specifies the output level as a

peak to peak EMF value expressed

in Volts.

Variable Type: double

Valid Range:   Waveform function dependent. See

the TG1304 specification for

details.

Default Value: none

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Function value out of range

-3   Output value out of range

-4   Frequency value out of range

-5   DC offset value out of range

-6   Symmetry value out of range

-7   Phase value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1304_config_sum

C Syntax           : int tg1304_config_sum(Instrument_ID, Enable, Waveform,

Sum_Amplitude_V, Sum_Frequency_Hz);

int Instrument_ID;

int Enable;

int Waveform;

double Sum_Amplitude_V;

double Sum_Frequency_Hz;

BASIC Syntax       : Error% = tg1304.config.sum(Instrument.ID%, Enable%, _

Waveform%, Sum.Amplitude.V#, Sum.Frequency.Hz#)

Purpose            :  Panel Name:    Sum

Description:   Enables Sum mode and sets up Sum

Waveform, Amplitude and Frequency.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Enable

0 - On (Default)

1 - Off

2 - Clear

Description:   Sets the status of Summing of the

Main and Aux generators.

Variable Type: Integer

Valid Range:   0 = On  - configured and active

1 = Off - configured and inactive

2 = Clear - Unconfigured

Default Value: 0 - On

Waveform

0 - Sine (Default)

1 - Triangle

2 - Square

3 - External

Description:   Selects the waveform to be summed.

Variable Type: Integer

Valid Range:   0 = Sine

1 = Triangle

2 = Square

3 = External

Default Value: 0 - Sine

Sum Amplitude (V)

Description:   Sets the amplitude of the summing

waveform.

Variable Type: double

Valid Range:   Waveform function dependent. See

the TG1304 specification for

details.

Default Value: none

Sum Frequency (Hz)

Description:   Sets the Frequency of the summing

waveform in Hz.This parameter has

no effect if the modulation

Waveform is External.

Note. This parameter is actually

the Auxiliary generator frequency.

Variable Type: double

Valid Range:   5e-3 to 50e+3 (5mHz to 50kHz)

Default Value: none

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Enable value out of range

-3   Waveform value out of range

-4   Frequency value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1304_config_sweep

C Syntax           : int tg1304_config_sweep(Instrument_ID, Enable, Mode,

Time_sec, Start_frequency_Hz,

Stop_frequency_Hz);

int Instrument_ID;

int Enable;

int Mode;

double Time_sec;

double Start_frequency_Hz;

double Stop_frequency_Hz;

BASIC Syntax       : Error% = tg1304.config.sweep(Instrument.ID%, Enable%, _

Mode%, Time.sec#, Start.frequency.Hz#, _

Stop.frequency.Hz#)

Purpose            :  Panel Name:    Sweep (VCF)

Description:   Enables Sweep (Voltage Controlled

Frequency) and sets up the Mode,

Start and Stop Frequencies and the

sweep time.

Note: It is not possible to control the VCF shape

or the Trigger source with this dirver.

When Internal VCF is selected They will

default to Triangle and Free respectively.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Enable

0 - On (Default)

1 - Off

2 - Clear

Description:   Sets the status of Sweep.

Variable Type: Integer

Valid Range:   0 = On  - configured and active

1 = Off - configured and inactive

2 = Clear - Unconfigured

Default Value: 0 - On

Mode

0 - Int. Lin. (Default)

1 - Int. Log.

2 - Ext. Lin.

3 - Ext. Log.

Description:   Sets the Sweep mode.

Variable Type: Integer

Valid Range:   0 = Internal linear

1 = Internal log

2 = External linear

3 = External log

Default Value: 0 - Internal linear

Time (sec)

Description:   Specifies the internal Sweep

triangle waveform period expressed

in seconds.

Note. This parameter is actually

the Auxiliary generator period.

Variable Type: double

Valid Range:   20e-6 to 1e+3 (20us to 1000s)

Default Value: none

Start frequency (Hz)

Description:   The Sweep Start Frequency

expressed in Hz.

Variable Type: double

Valid Range:   1e-2 to 13e+6 (10mHz to 13MHz)

Max sweep range 1000:1.

Default Value: none

Stop frequency (Hz)

Description:   The Sweep Stop Frequency

expressed in Hz.

Variable Type: double

Valid Range:   1e-2 to 13e+6 (10mHz to 13MHz)

Max sweep range 1000:1.

Default Value: none

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Enable value out of range

-3   Mode value out of range

-4   Time value out of range

-5   Start frequency value out of range

-6   Stop frequency value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1304_config_vca

C Syntax           : int tg1304_config_vca(Instrument_ID, Enable, Mode,

Time_sec, Min_Amplitude_V, Max_Amplitude_V);

int Instrument_ID;

int Enable;

int Mode;

double Time_sec;

double Min_Amplitude_V;

double Max_Amplitude_V;

BASIC Syntax       : Error% = tg1304.config.vca(Instrument.ID%, Enable%, _

Mode%, Time.sec#, Min.Amplitude.V#, _

Max.Amplitude.V#)

Purpose            :  Panel Name:    VCA

Description:   Enables Voltage Controlled

Attenuator and sets up the Mode,

Max and Min Amplitudes and the

sweep time.

Note: It is not possible to control the VCA shape

or the Trigger source with this dirver.

When Internal VCA is selected They will

default to Triangle and Free respectively.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Enable

0 - On (Default)

1 - Off

2 - Clear

Description:   Sets the status of VCA.

Variable Type: Integer

Valid Range:   0 = On  - configured and active

1 = Off - configured and inactive

2 = Clear - Unconfigured

Default Value: 0 - On

Mode

0 - Int. Lin. (Default)

1 - Int. Log.

2 - Ext. Lin.

3 - Ext. Log.

Description:   Sets the VCA mode.

Variable Type: Integer

Valid Range:   0 = Internal linear

1 = Internal log

2 = External linear

3 = External log

Default Value: 0 - Internal linear

Time (sec)

Description:   Specifies the internal VCA

triangle waveform period expressed

in seconds.

Note. This parameter is actually

the Auxiliary generator period.

Variable Type: double

Valid Range:   20e-6 to 1e+3 (20us to 1000s)

Default Value: none

Min Amplitude (V)

Description:   Sets the minimum amplitude of the

carrier at the lower peak of the

VCA control waveform.

Variable Type: double

Valid Range:   Waveform function dependent. See

the TG1304 specification for

details.

Default Value: none

Max Amplitude (V)

Description:   Sets the maximum amplitude of the

carrier at the upper peak of the

VCA control waveform.

Variable Type: double

Valid Range:   Waveform function dependent. See

the TG1304 specification for

details.

Default Value: none

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Enable value out of range

-3   Mode value out of range

-4   Time value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1304_file_ops

C Syntax           : int tg1304_file_ops(Instrument_ID, Operation,

File_Name);

int Instrument_ID;

int Operation;

char *File_Name;

BASIC Syntax       : Error% = tg1304.file.ops(Instrument.ID%, Operation%, _

File.Name$)

Purpose            : Panel name:     File Operations

Description:    Send or Receive a binary block to

or from the TG1304. The data will

be read from or written to a file.

Note            This operation is usefull for

Store blocks.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Operation

0 - Send File (Default)

1 - Get File

Description:   Selects to send a file to the

TG1304 or Get a file from TG1304.

Variable Type: Integer

Valid Range:   0 = Send file to TG1304

1 = Get file from TG1304

Default Value: 0 - Send file

File Name

Description:    This control specifies file name

from which the binary block will

be sent to the TG1304.

Variable type:  String - max length 50 characters

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Operation value out of range

228   Error writing to instrument from file

229   Error reading from instrument to file

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1304_get_resp

C Syntax           : int tg1304_get_resp(Instrument_ID, Response,

Number_Of_Bytes);

int Instrument_ID;

char *Response;

int *Number_Of_Bytes;

BASIC Syntax       : Error% = tg1304.get.resp(Instrument.ID%, Response$, _

Number.Of.Bytes%)

Purpose            : Panel name:     Get response string

Description:    Reads a response from the TG1304

into a string variable.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Response

Description:    This control specifies a string

variable to receive the response

from the TG1304. The string must

be long enough to receive the

response to avoid overflow. To aid

in checking this the number of

bytes put into the string is

returned from the function. The

absolute maximum length allowed is

2000 bytes.

Variable type:  String

Number Of Bytes

Description:    This value is the number of bytes

returned by the TG1304 and

placed in the string.

Variable type:  Integer

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1304_get_status

C Syntax           : int tg1304_get_status(Instrument_ID, Read_Status,

Value);

int Instrument_ID;

int Read_Status;

int *Value;

BASIC Syntax       : Error% = tg1304.get.status(Instrument.ID%, Read.Status%, _

Value%)

Purpose            : Panel name:     Read Status

Description:    Returns the byte value from the

selected status register.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Read Status

0 - Status byte (Default)

1 - Std Event

2 - SRQ enable

3 - Event Enable

4 - Parallel Pol

Description:    Used to select which status

register to read

Variable type:  integer

Valid range:    0 = Status Byte

1 = Standard Event Status

2 = SRQ enable

3 = Standard Event Enable

4 = Parallel Poll Enable

Default value   0 - Status Byte

Value

Description:    Returns the value read from the

selected register.

Variable type:  integer

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Status Enable value out of range

-3   Register Value out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1304_init

C Syntax           : int tg1304_init(Address, ID_Query, Reset_Device,

Instrument_ID);

int Address;

int ID_Query;

int Reset_Device;

int *Instrument_ID;

BASIC Syntax       : Error% = tg1304.init(Address%, ID.Query%, Reset.Device%, _

Instrument.ID%)

Purpose            :  Panel Name:    Initialize

Description:   Initializes the instrument in the

following ways:

- opens the instrument and sets

its GPIB address to the speci-

fied value

- optionally performs an

identification query on the

instrument

- optionally sets the instrument

to a default configuration

- returns an instrument ID which

is used to identify the

instrument in subsequent calls.

Parameter List     : Address

Description:   Specifies the address on the

GPIB to be used to communicate

with the instrument.  This value

must correspond to the current

internal setting of the

instrument.

Variable Type: Integer

Valid Range:   0 to 30

Default Value: 5

ID Query

1 - Do Query (Default)

0 - Skip Query

Description:   Specifies if an ID Query is sent

to the instrument during the

initialization procedure.

Variable Type: Integer

Valid Range:   1 = Yes

0 = No

Default Value: 1 - Yes

NOTE:  Under normal circumstances the ID Query

insures that the instrument initialized

over the GPIB is the type supported by

this driver. However circumstances may

arise where it is undesirable to send an

ID Query to the instrument. In those

cases; set this control to Skip Query

and this function will initialize the

GPIB and the Command arrays in the driver,

without doing an ID Query.

Reset Device

0 - No Reset

1 - Trig Clear (Default)

2 - Burst Int

3 - Burst Ext

4 - Gate Int

5 - Gate Ext

Description:   Specifies if the instrument is to

be reset and if so how the Trigger

(Burst/Gate) should be set during

the initialization procedure.

Variable Type: Integer

Valid Range:   0 = No Reset

1 = Trigger Clear

2 = Burst Int

3 = Burst Ext

4 = Gate Int

5 = Gate Ext

Default Value: 1 - Trigger Clear

NOTE: If you do not want the instrument reset.

Set this control to No Reset while

initializing the instrument.

Instrument ID

Description:   Returns an Instrument ID that is

used in all subsequent function

calls to select the instrument at

the Address specified.

If more than one instrument of the

same type is used, this value will

be used to differentiate between

them. The first instrument

initialized will be assigned an

Instrument ID of 1.

Variable Type: Integer

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   GPIB Address out of range

-2   ID Query out of range

-3   Reset Device out of range

220   Unable to open instrument

221   Unable to close instrument

223   Instrument identification query failed

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1304_loc_stores

C Syntax           : int tg1304_loc_stores(Instrument_ID, Operation,

Store_Number);

int Instrument_ID;

int Operation;

int Store_Number;

BASIC Syntax       : Error% = tg1304.loc.stores(Instrument.ID%, Operation%, _

Store.Number%)

Purpose            : Panel name:     Local Storage

Description:    Stores or Recalls the complete

state of the TG1304 to or from

local non-volatile memory.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Operation

0 - Recall (Default)

1 - Store

Description:   Selects the recall or store

operation.

Variable Type: Integer

Valid Range:   0 = Recall

1 = Store

Default Value: 0 - Recall

Store Number

Description:    Select the number of the store to

Recall from or Store to.

Variable type:  integer

Valid range:    1 to 64

Default value:  1

NOTE:  Store 64 is used by the Reset function. it

is therefore advisable not to use it here

except in exceptional circumstances.

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Operation value out of range

-3   Store Number out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1304_main_offset

C Syntax           : void tg1304_main_offset(Instrument_ID, DC_Offset_V);

int Instrument_ID;

double DC_Offset_V;

BASIC Syntax       : CALL tg1304.main.offset(Instrument.ID%, DC.Offset.V#)

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

DC Offset (V)

Description:   Sets the output Offset voltage as

an EMF value is expressed in Volts

Variable Type: double

Valid Range:   -10 to +10

Default Value: 0.0

________________________________________________________________________________

Function Name      : tg1304_main_phase

C Syntax           : void tg1304_main_phase(Instrument_ID, Phase_deg);

int Instrument_ID;

int Phase_deg;

BASIC Syntax       : CALL tg1304.main.phase(Instrument.ID%, Phase.deg%)

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Phase (deg)

Description:   Sets the Start/Stop Phase

parameter expressed in degrees.

Variable Type: integer

Valid Range:   -360 to +360

Default Value: 0

________________________________________________________________________________

Function Name      : tg1304_main_symmetry

C Syntax           : void tg1304_main_symmetry(Instrument_ID, Symmetry );

int Instrument_ID;

double Symmetry ;

BASIC Syntax       : CALL tg1304.main.symmetry(Instrument.ID%, Symmetry #)

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Symmetry (%)

Description:   Sets the output Symmetry as a

percentage.

Variable Type: double

Valid Range:   5% to 95%

Default Value: 50.0%

________________________________________________________________________________

Function Name      : tg1304_reset

C Syntax           : int tg1304_reset(Instrument_ID, Reset_Device);

int Instrument_ID;

int Reset_Device;

BASIC Syntax       : Error% = tg1304.reset(Instrument.ID%, Reset.Device%)

Purpose            :  Panel Name:    Reset

Description:   This panel is provided to avoid

the need to Close and then

re-Initialise the instrument when

a reset is required

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Reset Device

1 - Trig Clear (Default)

2 - Burst Int

3 - Burst Ext

4 - Gate Int

5 - Gate Ext

Description:   Specifies how the instrument is to

be reset and how the Trigger

(Burst/Gate) should be set.

Variable Type: Integer

Valid Range:   1 = Trigger Clear

2 = Burst Int

3 = Burst Ext

4 = Gate Int

5 = Gate Ext

Default Value: 1 - Trigger Clear

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   ID Query out of range

-2   Reset Device out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1304_set_status

C Syntax           : int tg1304_set_status(Instrument_ID, Set_Status_Enable,

Value);

int Instrument_ID;

int Set_Status_Enable;

int Value;

BASIC Syntax       : Error% = tg1304.set.status(Instrument.ID%, _

Set.Status.Enable%, Value%)

Purpose            : Panel name:     Status Reporting

Description:    Specify the way in which status is

reported and Service Request is

generated as defined in the IEEE

488.2 standard.

The function provides access to

the three status enable registers

in the TG1304, these are:

Service Request Enable Register

- Specifies which bits, when set,

in the Status Byte register will

generate a Service Request.

Standard Event Enable Register

- Specifies which bits, when set,

in the Standard Event Status

register will set the ESB bit in

the Status Byte Register.

Parallel Poll Enable Register

- Specifies which bits, when set,

in the Status Byte register will

be used to form the ist local

message

See the IEEE 488.2 Standard and

the instrument  instruction book

for a full description of status

reporting capabilities.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Set Status Enable

0 - SRQ enable (Default)

1 - Event Enable

2 - Parallel Pol

Description:    Used to select which enable

register will receive the byte

value.

Variable type:  integer

Valid range:    0 = SRQ enable

1 = Standard Event Enable

2 = Parallel Poll

Default value   0 - SRQ Enable

Value

Description:    A byte value which will be written

to the selected enable register.

Variable type:  integer

Valid range:    0 to 255

Default value:  0

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Status Enable value out of range

-3   Register Value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1304_snd_cmd

C Syntax           : int tg1304_snd_cmd(Instrument_ID, Command);

int Instrument_ID;

char *Command;

BASIC Syntax       : Error% = tg1304.snd.cmd(Instrument.ID%, Command$)

Purpose            : Panel name:     Send command string

Description:    Sends a string of bytes to the

TG1304 from a string variable.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Command

Description:    This control specifies data to be

sent to the TG1304. The absolute

maximum length allowed is 200

bytes.

Variable type:  String

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tg1304_trig

C Syntax           : int tg1304_trig(Instrument_ID, Enable, Gate_On_Time_sec,

Gate_Off_Time_sec, Ext_Trig_Thrshld_V,

Int_Trig_Period_s, Burst_Count);

int Instrument_ID;

int Enable;

double Gate_On_Time_sec;

double Gate_Off_Time_sec;

double Ext_Trig_Thrshld_V;

double Int_Trig_Period_s;

double Burst_Count;

BASIC Syntax       : Error% = tg1304.trig(Instrument.ID%, Enable%, _

Gate.On.Time.sec#, Gate.Off.Time.sec#, _

Ext.Trig.Thrshld.V#, Int.Trig.Period.s#, _

Burst.Count#)

Purpose            :  Panel Name:    Trigger (Burst/Gate)

Description:   This panel allows the selected

mode (Burst/Gate), see note below,

to be Enabled and the Burst Count,

Internal Trigger Frequency,

External Trigger Threshold and

Gate On and Off Times to be set.

Note: It is not possible to directly select the

Trigger (Burst and Gate) functions with

this dirver. All the numeric parameters may

be set on this panel but the choice of

Burst/Gate and Int/Ext must be made using

the Reset function. See the Initialise and

Reset panels for more information.

The common parameters must be specified for

any mode. The Trigger and Gate parameters

need only be specified when the relevent

mode has been activated by reset.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Enable

0 - On (Default)

1 - Off

Description:   Sets the status of the Trigger.

Variable Type: Integer

Valid Range:   0 = On  - configured and active

1 = Off - configured and inactive

Default Value: 0 - On

Note:  This control will enable and disable the

trigger mode that was selected the last

time the Reset function was called. If no

explicit call has been made then the mode

set in the Initialise function is used.

You must specify trigger modes in this

way as there are no direct commands to

allow the trigger mode to be set

otherwise.

Gate On Time (sec)

Description:   The On Time of the internal

Gate signal expressed in sec.

Variable Type: double

Valid Range:   800e-9 to 850

Default Value: 0.05

Gate Off Time (sec)

Description:   The Off Time of the internal

Gate signal expressed in sec.

Variable Type: double

Valid Range:   800e-9 to 850

Default Value: 0.05

Ext Trig Thrshld (V)

- Common Parameter

Description:   The external trigger threshold

expressed in Volts.

Variable Type: double

Valid Range:   -10.0 to +10.0

Default Value: 0.5

Int Trig Period (s)

- Trigger Parameter

Description:   The Period of the internal

Trigger generator expressed in sec

Variable Type: double

Valid Range:   150e-9 to 850

Default Value: 0.1

Burst Count

Description:   Sets the number of cycles of the

main generator waveform in each

burst. Has no effect if Gate mode

is selected,

Variable Type: double

Valid Range:   0.5 to 524288.5

Default Value: 1.0

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Enable value out of range

-3   Gate on value out of range

-4   Gate off value out of range

-5   Trigger threshold value out of range

-6   Trigger period value out of range

-7   Burst count value out of range

-10   Instrument not reset by this driver

-11   instrument reset to Trigger Off

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

Note: It is not possible to directly select the

Trigger (Burst and Gate) functions with

this dirver. All the numeric parameters may

be set on this panel but the choice of

Burst/Gate and Int/Ext must be made using

the Reset function. See the Initialise and

Reset panels for more information. Errors

-10 and -11 indicate that no Trigger or

Gate mode has been selected.

-------------------------------------------------------------------------------

########