TTi TGA1240 Arbitrary Generator

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for

TTi TGA1240 ArbitraryGenerator.

It contains functions for opening, configuring, taking measurements from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

For GPIB instrument drivers:

-  the instrument is connected to the GPIB.

-  the GPIB address supplied to the initialize function must match the

GPIB address of the instrument.

For VXI instrument drivers:

-  the instrument is installed in the VXI mainframe and you are

using one of the following controller options:

Embedded controller

MXI

MXI2

GPIB-VXI

-  the logical address supplied to the initialize function must

match the logical address of the instrument.

For RS-232 instrument drivers:

-  the instrument is connected to the RS-232 interface.

-  the COM port, baud rate, parity, and timeout supplied to the

initialize function must match the settings of the instrument.

________________________________________________________________________________

Error and Status Information:

Each function in this instrument driver returns a status code that

either indicates success or describes an error or warning condition.

Your program should examine the status code from each call to an

instrument driver function to determine if an error occurred.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Non-zero Values        Errors

The description of each instrument driver function lists possible error

codes and their meanings

________________________________________________________________________________

How To Use This Document:

Use this document as a programming reference manual.

It describes each function in the

TTi TGA1240 Arbitrary Generator

instrument.  The functions appear in alphabetical order, with a description

of the function and its C syntax, a description of each parameter, and a list

of possible error codes.

________________________________________________________________________________

Function Tree Layout:

TTi TGA1240 Arbitrary Generator        Function Name:

Initialize                                  tga1240_init

Configure

Waveform                                 tga1240_config_wfm

Std Waveform Setup

Pulse Setup                           tga1240_config_pulse

Pulse Train Setup                     tga1240_config_pulse_trn

Arb Waveform Setup

Arb Info                              tga1240_arb_info

Create New Arb                        tga1240_arb_create

Modify Arb                            tga1240_arb_modify

Edit Arb                              tga1240_arb_edit

Mode setup                               tga1240_setup_mode

Connector setup                          tga1240_setup_conector

Sweep setup                              tga1240_setup_sweep

Sequence Setup                           tga1240_setup_seq

Tone setup                               tga1240_setup_tone

Modulation setup                         tga1240_setup_mod

Sum setup                                tga1240_setup_sum

Inter Channel setup                      tga1240_set_inter

Set Status                               tga1240_set_status

Data functions

Read Arb Data                            tga1240_arb_read

Write Arb Data                           tga1240_arb_write

Local Storage                               tga1240_loc_stores

Get Status                                  tga1240_get_status

Miscellaneous

Send command string                      tga1240_snd_cmd

Get Response string                      tga1240_get_resp

File operations                          tga1240_file_ops

Close                                       tga1240_close

________________________________________________________________________________

TTi TGA1240 Arbitrary Generator

Instrument Name: Thurlby Thandar TGA1240 series

Arbitrary Generators.

Description:     This instrument module provides

programming support for the

TGA1240 series. The instruments

supported are:

TGA1241    one channel

TGA1242    two channels

TGA1244    four channels

The module is divided into the

following functions:

Functions/Classes:

(1) Initialize

Initialize the instrument and optionally

interrogate the ID and set to the specified

reset conditions.

(2) Configure...

This class of functions configures the main

generator and allows access

to the Modulation capabilities of the instrument.

(1) Waveform

Set up such parameters as output waveform,

frequency and amplitude.

(2) Std Waveform setup...

This class of functions allows setting the

parameters for std waveforms

(1) Pulse setup

(2) Pulse train setup

(3) Arb Waveform setup...

This class of functions allows setting of arb

waveforms

(1) Arb info

(2) Create new arb

(3) Modify arb

(4) Edit arb

(4) Mode setup

Allows setup an selection of the various modes

(5) Connector setup

Configure and control trigrer input and sync outputs

etc.

(6) Sweep setup

Configures and controls the sweep operation

(7) Sequence setup

configure and control waveform sequencing.

(8) Modulation

Colfigure and control AM and SCM

(9) Sum setup

Configure and control signal summing

(10) Sum setup

Configure and control inter channel phase locking

(11) Set Status

Configure the IEEE 488.2 status

reporting capabilities of the Instrument.

(3) Data functions

This class of functions allow arb waveforms to be exchanged

between the controller and the instrument

(1) Read arb data

Returns data from an arb to the controller

(2) Write arb data

Accepts data from the controller and stores it in

the specified arb waveform

(4) Local Storage

Executes the Store and Recall commands.

(5) Get Status

Returns the current value of the specified

IEEE 488.2 status byte.

(6) Miscellaneous

This class of functions allows access to

additional remote control commands

which are not available elsewhere in this

driver. Also included are file operations.

(1) Send Command String

(2) Get Response String

(3) File Operations

(7) Close

Take the instrument offline.

________________________________________________________________________________

The following functions are in alphabetical order.

________________________________________________________________________________

tga1240_arb_create

int tga1240_arb_create (int instrumentID, char newarbname[], int create,

int length, int waveformtocopy, char arbitraryname[]);

Purpose

Description:    Create a new arbitrary waveform of the

specified length either as a blank or a copy

of an existing waveform.

Parameter List

instrumentID

Variable Type       int

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

newarbname

Variable Type       char []

Description:   The name for the new arb.

Variable Type: String

create

Variable Type       int

Description:   Create a blank arb or a copy.

Variable Type: Integer

Valid Range:   0 = Blank

1 = Copy

Default Value: 0 - Blank

length

Variable Type       int

Description:   The length, in points, for the new arb.

Variable Type: Integer

Valid Range:   4 to 65536

Default Value: 1024

waveformtocopy

Variable Type       int

Description:   Specifies the waveform to use for the copy

function.

Variable Type: Integer

Valid Range:   0 = Sine

1 = Square

2 = Triangle

3 = DC

4 = Positive Ramp

5 = Negative Ramp

6 = Cosine

7 = Haversine

8 = Havercos

9 = Sync

10 = ARB

Default Value: 0 - Sine

arbitraryname

Variable Type       char []

Description:   The arb to use if the waveform to copy is set

to 10 - arb.

Variable Type: String

Return Value

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Create value out of range

-3   Length value out of range

-4   Waveforn to copy value out of range

100 to 199 are instrument execution errors

see the instrument instruction manual

for details.

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

tga1240_arb_edit

int tga1240_arb_edit (int instrumentID, char arbname[], int operation,

int firstpointidx, int firstpointvalue,

int secondpointidx, int secondpointvalue,

int destinationidx, float amplitudemultiplier,

int offsetvalue, int waveformtoinsert, char insertarb[]);

Purpose

Description:    Allows the specified arb to be edited by the

selected edit operation.

Parameter List

instrumentID

Variable Type       int

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

arbname

Variable Type       char []

Description:   The name of the arb to edit.

Variable Type: String

operation

Variable Type       int

Description:   Selects the edit operation to perform.

Variable Type: Integer

Valid Range:   0 = Point edit

1 = Line draw

2 = Wave insert

3 = Block copy

4 = Wave amplitude

5 = Wave offset

6 = Wave invert

7 = Set position marker

8 = Del all position markers

Default Value: 0 - Point edit

firstpointidx

Variable Type       int

Description:   Set the address index for the first edit point.

Variable Type: Integer

Valid Range:   0 to Second point index - 4

Default Value: 0

Note: The first and second point index specify the point

addresses between which the edit operation will act.

For point edit and set position marker only the first

point index is used.

firstpointvalue

Variable Type       int

Description:   Set the value for the first edit point.

Variable Type: Integer

Valid Range:   -2048 to 2047

Default Value: 0

Note: The first and second point index specify the point

addresses between which the edit operation will act.

For point edit and set position marker only the first

point index is used.

secondpointidx

Variable Type       int

Description:   Set the address index for the second edit point.

Variable Type: Integer

Valid Range:   First point index +4 to last waveform point

Default Value: 0

Note: The first and second point index specify the point

addresses between which the edit operation will act.

For point edit and set position marker only the first

point index is used.

secondpointvalue

Variable Type       int

Description:   Set the value for the second edit point.

Variable Type: Integer

Valid Range:   -2048 to 2047

Default Value: 0

Note: The first and second point index specify the point

addresses between which the edit operation will act.

For point edit and set position marker only the first

point index is used.

destinationidx

Variable Type       int

Description:   Set the value for the destinationstart point

of a block copy operation.

Variable Type: Integer

Valid Range:   0 to last waveform point

Default Value: 0

amplitudemultiplier

Variable Type       float

Description:   Set the multiplier fo the wave amplitude

operation.

Variable Type: Float

Valid Range:   0.1 to 100.0

Default Value: 1.0

offsetvalue

Variable Type       int

Description:   Set the value for the wave offset operation.

Variable Type: Integer

Valid Range:   -4096 to 4095

Default Value: 0

waveformtoinsert

Variable Type       int

Description:   Specifies the waveform to use for the insert

operation.

Variable Type: Integer

Valid Range:   0 = Sine

1 = Square

2 = Triangle

3 = DC

4 = Positive Ramp

5 = Negative Ramp

6 = Cosine

7 = Haversine

8 = Havercos

9 = Sync

10 = ARB

Default Value: 0 - Sine

insertarb

Variable Type       char []

Description:   The arb name for the insert operation when the

Waveform to insert is set to arb.

Variable Type: String

Return Value

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Operation value out of range

-3   Newlength value out of range

100 to 199 are instrument execution errors

see the instrument instruction manual

for details.

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

tga1240_arb_info

int tga1240_arb_info (int instrumentID, char arbName[], int *length);

Purpose

Description:    This function is used to verify the existance

of a particular arbitrary waveform and to find

its length (number of points).

Parameter List

instrumentID

Variable Type       int

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

arbName

Variable Type       char []

Description:   The arb name to search for

Variable Type: String

length

Variable Type       int (passed by reference)

Description:   Returns the result of the search

If the value is zero the arb does not exist.

A non-zero value indicates that the arb exists

and the actual value is the length in points.

Variable Type: Integer

Return Value

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

100 to 199 are instrument execution errors

see the instrument instruction manual

for details.

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

tga1240_arb_modify

int tga1240_arb_modify (int instrumentID, char arbname[], int operation,

int newlength, char newarbname[]);

Purpose

Description:    Allows an existing arb to be renamed, resized

or deleted.

Parameter List

instrumentID

Variable Type       int

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

arbname

Variable Type       char []

Description:   The name of the arb to modify

Variable Type: String

operation

Variable Type       int

Description:   Selects the modify operation to perform.

Variable Type: Integer

Valid Range:   0 = Delete

1 = Rename

2 = Resize

3 = Channel del

Default Value: 0 - Delete

Note: Channel del removes the arb from the current channel

memory. Delete can only be used after the arb has been

removed from all channels.

Channel del cannot be used to remove the arb currently

being played back or any arb if the channel is playing

a waveform sequence.

newlength

Variable Type       int

Description:   The new length for the resize operation.

Variable Type: Integer

Valid Range:   4 to 65536

Default Value: 1024

newarbname

Variable Type       char []

Description:   The new name for the rename operation.

Variable Type: String

Return Value

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Operation value out of range

-3   Newlength value out of range

100 to 199 are instrument execution errors

see the instrument instruction manual

for details.

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

tga1240_arb_read

int tga1240_arb_read (int instrumentID, char arbwfmname[], int start, int end,

short wfmdataarray[]);

Parameter List

instrumentID

Variable Type       int

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

arbwfmname

Variable Type       char []

Description:   Specifies the arb waveform name to

use when the function is set to Arb.

Variable Type: String

Valid Range:   Must be an existing arb waveform.

Default Value: wv01

start

Variable Type       int

end

Variable Type       int

wfmdataarray

Variable Type       short []

Description:   Receives the point data read from the arb

waveform. There must be enough elements in

the array to hold all the point data.

The data in the array will range between 0 and

4095. This array may be used as the input for

the Write arb data function.

Variable Type: Short integer array

Return Value

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Start point is greater then End point

-3   Arb does not exist.

-4   End point is past end of waveform

-5   No of points exceeds 32768

-10   Channel value out of range

100 to 199 are instrument execution errors

see the instrument instruction manual

for details.

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

240   Instrument timed out

________________________________________________________________________________

tga1240_arb_write

int tga1240_arb_write (int instrumentID, char arbwfmname[], int start, int end,

short wfmdataarray[]);

Parameter List

instrumentID

Variable Type       int

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

arbwfmname

Variable Type       char []

Description:   Specifies the arb waveform name to

use when the function is set to Arb.

Variable Type: String

Valid Range:   Must be an existing arb waveform.

Default Value: wv01

start

Variable Type       int

end

Variable Type       int

wfmdataarray

Variable Type       short []

Description:   Contains the point data to send to the arb

waveform. There must be enough elements in

the array to hold at least the length

required between the start and stop points.

The data in the array will range between 0 and

4095. e.g. an array already filled by the Read

arb data function.

Variable Type: Short integer array

Return Value

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Start point is greater then End point

-3   Arb does not exist.

-4   End point is past end of waveform

-5   No of points exceeds 32768

-10  Channel value out of range

100 to 199 are instrument execution errors

see the instrument instruction manual

for details.

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

240   Instrument timed out

________________________________________________________________________________

tga1240_close

int tga1240_close (int instrumentID);

Purpose

Panel Name:    Close

Description:   Closes the instrument and takes it

off line.

Note: The  instrument must  be reinitialized

to use it again.

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

221   Unable to close instrument

232   Instrument not initialized

________________________________________________________________________________

tga1240_config_pulse

int tga1240_config_pulse (int instrumentID, int channel, double pulsePeriod,

double pulseWidth, double pulseDelay);

Purpose

Description:    Setup the pulse parameters for the specified

output channel.

Parameter List

instrumentID

Variable Type       int

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

channel

Variable Type       int

Description:   Selects the channel to configure.

Variable Type: Integer

Valid Range:   0 =  CH1

1 =  CH2

2 =  CH3

3 =  CH4

Default Value: 0 - CH1

Note. The instrument will ganerate an error if the selected

channel is not present.

pulsePeriod

Variable Type       double

Description:   The pulse Period expressed in seconds.

Variable Type: double

Valid Range:   100e-9 to 100

Default Value: 100e-6

pulseWidth

Variable Type       double

Description:   The Pulse width expressed in seconds.

Variable Type: double

Valid Range:   25e-9 to 99.99

Default Value: 50e-6

pulseDelay

Variable Type       double

Description:   The Pulse delay expressed  seconds.

Variable Type: double

Valid Range:   -99.99 to +99.99

Default Value: 0

Return Value

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Period value out of range

-3   Width value out of range

-4   Delay value out of range

-10  Channel value out of range

100 to 199 are instrument execution errors

see the instrument instruction manual

for details.

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

tga1240_config_pulse_trn

int tga1240_config_pulse_trn (int instrumentID, int channel, int pulseNumber,

double pulseTrainPeriod, double baseline,

double pulseLevel, double pulseWidth,

double pulseDelay, int numberofPulses, int make);

Purpose

Description:    Setup the pulse train parameters for the

specified output channel.

Parameter List

instrumentID

Variable Type       int

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

channel

Variable Type       int

Description:   Selects the channel to configure.

Variable Type: Integer

Valid Range:   0 =  CH1

1 =  CH2

2 =  CH3

3 =  CH4

Default Value: 0 - CH1

Note. The instrument will ganerate an error if the selected

channel is not present.

pulseNumber

Variable Type       int

Description:   The pulse number to receive the values.

Variable Type: Integer

Valid Range:   1 to 10

Default Value: 1

pulseTrainPeriod

Variable Type       double

Description:   The pulse train period expressed in seconds.

Variable Type: double

Valid Range:   100e-9 to 100

Default Value: 100e-6

baseline

Variable Type       double

Description:   The pulse train baseline position expressed

in Volts.

Variable Type: double

Valid Range:   -5 to +5

Default Value: 0

pulseLevel

Variable Type       double

Description:   The pulse Period expressed in seconds.

Variable Type: double

Valid Range:   10e6 to 100

Default Value: 100e-6

pulseWidth

Variable Type       double

Description:   The Pulse width expressed in seconds.

Variable Type: double

Valid Range:   25e-9 to 99.99

Default Value: 50e-6

pulseDelay

Variable Type       double

Description:   The Pulse delay expressed  seconds.

Variable Type: double

Valid Range:   -99.99 to +99.99

Default Value: 0

numberofPulses

Variable Type       int

Description:   The number of pulses in the pulse train.

Variable Type: integer

Valid Range:   1 to 10

Default Value: 1

make

Variable Type       int

Description:   Specifies if the pulse train should be built

immediately after the new values are installed.

Variable Type: Integer

Valid Range:   0 = No

1 = Yes

Default Value: 0 - No

Note: This parameter is provided to allow several pulses to be

defined before the new pulse train is made thus saving

execution time.

Return Value

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Period value out of range

-3   Width value out of range

-4   Delay value out of range

-5   Pulse number value out of range

-6   Baseline value out of range

-7   Pulse level value out of range

-8   Number of pulses out of range

-10   Channel value out of range

100 to 199 are instrument execution errors

see the instrument instruction manual

for details.

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

tga1240_config_wfm

int tga1240_config_wfm (int instrumentID, int function, double frequencyPeriod,

double amplitudeVpp, double DCOffsetV, int sense,

int load, int output, char arbwfmname[], int freqPer,

int channel);

Purpose

Description:   Sets up Function, Frequency,

Peak-Peak Output level,

output status, DC Offset,

and load impedance for the

specified output channel.

Parameter List

instrumentID

Variable Type       int

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

function

Variable Type       int

Description:   Specifies the output waveform

function.

Variable Type: Integer

Valid Range:   0 = Sine

1 = Square

2 = Triangle

3 = DC

4 = Positive Ramp

5 = Negative Ramp

6 = Cosine

7 = Haversine

8 = Havercos

9 = Sync

10 = Pulse

11 = Pulse train

12 = ARB

13 = Sequence

Default Value: 0 - Sine

frequencyPeriod

Variable Type       double

Description:   The output Frequency or Period expressed in

Hz or seconds.

Variable Type: double

Valid Range:   1e-4 to maximum value for the selected

Function (0.1mHz to 16MHz max)

Default Value: 10kHz

amplitudeVpp

Variable Type       double

Description:   Sets the output level in Volts peak-peak.

Variable Type: double

Valid Range:   10mV to 20V

Default Value: 2.0

DCOffsetV

Variable Type       double

Description:   Sets the output Offset voltage in Volts

Variable Type: double

Valid Range:   -10 to +10

Default Value: 0.0

sense

Variable Type       int

Description:   Sets the output sense to NORMAL or

INVERTed.

Variable Type: Integer

Valid Range:   1 = Invert

0 = Normal

Default Value: 0 (normal)

load

Variable Type       int

Description:   Set the output load.

Variable Type: Integer

Valid Range:   0 =  HiZ

1 =  50 Ohms

2 = 600 Ohms

Default Value: 0 (HiZ)

output

Variable Type       int

Description:   Turns the Main output on or off.

Variable Type: Integer

Valid Range:   1 = On

0 = Off

Default Value: 0 - Off

arbwfmname

Variable Type       char []

Description:   Specifies the arb waveform name to

use when the function is set to Arb.

Variable Type: String

Valid Range:   Must be an existing arb waveform.

Default Value: "wv01"

freqPer

Variable Type       int

Description:   Specifies the data type for the

Frequency/Period control..

Variable Type: Integer

Valid Range:   0 = Waveform frequency

1 = Waveform period

2 = Sample frequency

3 = Sample period

Default Value: 0 - Waveform period

channel

Variable Type       int

Description:   Selects the channel to configure.

Variable Type: Integer

Valid Range:   0 =  CH1

1 =  CH2

2 =  CH3

3 =  CH4

Default Value: 0 - CH1

Note. The instrument will ganerate an error if the selected

channel is not present.

Return Value

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Function value out of range

-3   Output value out of range

-4   Frequency value out of range

-5   DC offset value out of range

-6   Freq/per value out of range

-8   Zout value out of range

-9   Sense value out of range

-10  Channel value out of range

100 to 199 are instrument execution errors

see the instrument instruction manual

for details.

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

tga1240_file_ops

int tga1240_file_ops (int instrumentID, int operation, char fileName[]);

Purpose

Description:    Send or Receive a binary block to

or from the instrument. The data will

be read from or written to a file.

Note            This operation is usefull for

LRN blocks.

Parameter List

instrumentID

Variable Type       int

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

operation

Variable Type       int

Description:   Selects to send a file to the

instrument or Get a file from instrument.

Variable Type: Integer

Valid Range:   0 = Send file to instrument

1 = Get file from instrument

Default Value: 0 - Send file

fileName

Variable Type       char []

Description:    This control specifies file name

from which the binary block will

be sent to the instrument.

Variable type:  String - max length 50 characters

Return Value

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Operation value out of range

228   Error writing to instrument from file

229   Error reading from instrument to file

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

tga1240_get_resp

int tga1240_get_resp (int instrumentID, char response[], int *numberOfBytes);

Purpose

Description:    Reads a response from the instrument

into a string variable.

Parameter List

instrumentID

Variable Type       int

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

response

Variable Type       char []

Description:    This control specifies a string

variable to receive the response

from the instrument. The string must

be long enough to receive the

response to avoid overflow. To aid

in checking this the number of

bytes put into the string is

returned from the function. The

absolute maximum length allowed is

2000 bytes.

Variable type:  String

numberOfBytes

Variable Type       int (passed by reference)

Description:    This value is the number of bytes

returned by the instrument and

placed in the string.

Variable type:  Integer

Return Value

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

tga1240_get_status

int tga1240_get_status (int instrumentID, int readStatus, int *value);

Purpose

Description:    Returns the byte value from the

selected status register.

Parameter List

instrumentID

Variable Type       int

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

readStatus

Variable Type       int

Description:    Used to select which status

register to read

Variable type:  integer

Valid range:    0 = Status Byte

1 = Standard Event Status

2 = SRQ enable

3 = Standard Event Enable

4 = Parallel Poll Enable

5 = Execution Error

Default value   0 - Status Byte

value

Variable Type       int (passed by reference)

Description:    Returns the value read from the

selected register.

Variable type:  integer

Return Value

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Status Enable value out of range

-3   Register Value out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

240   Instrument timed out

________________________________________________________________________________

tga1240_init

int tga1240_init (int address, int IDQuery, int resetDevice, int *instrumentID);

Purpose

Description:   Initializes the instrument in the

following ways:

- opens the instrument and sets

its GPIB address to the speci-

fied value

- optionally performs an

identification query on the

instrument

- optionally sets the instrument

to a default configuration

- returns an instrument ID which

is used to identify the

instrument in subsequent calls.

Parameter List

address

Variable Type       int

Description:   Specifies the address on the

GPIB to be used to communicate

with the instrument.  This value

must correspond to the current

internal setting of the

instrument.

Variable Type: Integer

Valid Range:   0 to 30

Default Value: 5

IDQuery

Variable Type       int

Description:   Specifies if an ID Query is sent

to the instrument during the

initialization procedure.

Variable Type: Integer

Valid Range:   1 = Yes

0 = No

Default Value: 1 - Yes

NOTE:  Under normal circumstances the ID Query

insures that the instrument initialized

over the GPIB is the type supported by

this driver. However circumstances may

arise where it is undesirable to send an

ID Query to the instrument. In those

cases; set this control to Skip Query

and this function will initialize the

GPIB and the Command arrays in the driver,

without doing an ID Query.

resetDevice

Variable Type       int

Description:   Specifies if the instrument is to

be reset during the initialization

procedure.

Variable Type: Integer

Valid Range:   0 = No Reset

1 = Reset device

Default Value: 1 = Reset

NOTE: If you do not want the instrument reset.

Set this control to No Reset while

initializing the instrument.

instrumentID

Variable Type       int (passed by reference)

Description:   Returns an Instrument ID that is

used in all subsequent function

calls to select the instrument at

the Address specified.

If more than one instrument of the

same type is used, this value will

be used to differentiate between

them. The first instrument

initialized will be assigned an

Instrument ID of 1.

Variable Type: Integer

Return Value

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   GPIB Address out of range

-2   ID Query out of range

-3   Reset Device out of range

220   Unable to open instrument

221   Unable to close instrument

223   Instrument identification query failed

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

240   Instrument timed out

________________________________________________________________________________

tga1240_loc_stores

int tga1240_loc_stores (int instrumentID, int operation, int storeNumber);

Purpose

Description:    Stores or Recalls the complete

state of the instrument to or from

local non-volatile memory.

Parameter List

instrumentID

Variable Type       int

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

operation

Variable Type       int

Description:   Selects the recall or store

operation.

Variable Type: Integer

Valid Range:   0 = Recall

1 = Store

Default Value: 0 - Recall

storeNumber

Variable Type       int

Description:    Select the number of the store to

Recall from or Store to.

Variable type:  integer

Valid range:    1 to 9

Default value:  1

Return Value

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Operation value out of range

-3   Store Number out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

tga1240_setup_conector

int tga1240_setup_conector (int instrumentID, int channel, int output,

int syncout, int syncoutsignal, int outputfilter,

int triggerout, int hold);

Purpose

Description:   Sets output, sync out, trigger out, waveform

filter and hold mode for the selected channel.

Parameter List

instrumentID

Variable Type       int

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

channel

Variable Type       int

Description:   Selects the channel to configure.

Variable Type: Integer

Valid Range:   0 =  CH1

1 =  CH2

2 =  CH3

3 =  CH4

Default Value: 0 - CH1

Note. The instrument will ganerate an error if the selected

channel is not present.

output

Variable Type       int

Description:   Turns the Main output on or off.

Variable Type: Integer

Valid Range:   1 = On

0 = Off

Default Value: 1 - On

syncout

Variable Type       int

Description:   Turns the Sync output on or off.

Variable Type: Integer

Valid Range:   1 = On

0 = Off

Default Value: 1 - On

syncoutsignal

Variable Type       int

Description:   Sets the sync output signal source.

Variable Type: Integer

Valid Range:   0 = Auto

1 = Waveform sync

2 = Position markers

3 = Burst done

4 = Sequence sync

5 = Trigger

6 = Sweep sync

7 = Phase lock

Default Value: 0 - Auto

outputfilter

Variable Type       int

Description:   Sets the waveform filter.

Variable Type: Integer

Valid Range:   0 = Auto

1 = 10MHz eliptic

2 = 16MHz eliptic

3 = 10MHz bessel

4 = None

Default Value: 0 - Auto

triggerout

Variable Type       int

Description:   Sets the trigger out signal source.

Variable Type: Integer

Valid Range:   0 = Auto

1 = Waveform end

2 = Position markers

3 = Sequence sync

4 = Burst done

Default Value: 0 - Auto

hold

Variable Type       int

Description:   Controls the hold function.

Variable Type: Integer

Valid Range:   0 = Enable

1 = Disable

2 = On

3 = Off

Default Value: 1 - Disable

Return Value

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Output value out of range

-3   Sync out value out of range

-4   Sync out signal value out of range

-5   Output filter value out of range

-6   Trigger out signal value out of range

-7   Hold value out of range

-10  Channel value out of range

100 to 199 are instrument execution errors

see the instrument instruction manual

for details.

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

tga1240_setup_mod

int tga1240_setup_mod (int instrumentID, int channel, int source, int type,

float SCMlevel, float AMdepth);

Purpose

Description:    Set the modulation for the selected channel..

Parameter List

instrumentID

Variable Type       int

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

channel

Variable Type       int

Description:   Selects the channel to configure.

Variable Type: Integer

Valid Range:   0 =  CH1

1 =  CH2

2 =  CH3

3 =  CH4

Default Value: 0 - CH1

Note. The instrument will ganerate an error if the selected

channel is not present.

source

Variable Type       int

Description:   Selects the modulation source.

Variable Type: Integer

Valid Range:   0 = Off

2 = External

3 = Previous channel

Default Value: 0 - Off

type

Variable Type       int

Description:   Selects the type of modulation.

Variable Type: Integer

Valid Range:   0 = AM/VCA

1 = SCM

Default Value: 0 - AM/VCA

SCMlevel

Variable Type       float

Description:   Selects the level of modulation in Vp-p if

the type is set to SCM and the source is set

to previous channel.

Variable Type: Float

Valid Range:   0 to 10

Default Value: 5

AMdepth

Variable Type       float

Description:   Selects the percentage depth of modulation if

the type is  set to AM and the source is set to

previous channel.

Variable Type: Float

Valid Range:   0 to 105

Default Value: 30

Return Value

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Source value out of range

-3   Type value out of range

-10  Channel value out of range

100 to 199 are instrument execution errors

see the instrument instruction manual

for details.

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

tga1240_setup_mode

int tga1240_setup_mode (int instrumentID, int channel, int mode,

int triggersource, double internalperiod,

int burstcount, int triggerslope, float phaseangle);

Purpose

Description:   Sets the Waveform playback mode for the selected

channel.

Parameter List

instrumentID

Variable Type       int

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

channel

Variable Type       int

Description:   Selects the channel to configure.

Variable Type: Integer

Valid Range:   0 =  CH1

1 =  CH2

2 =  CH3

3 =  CH4

Default Value: 0 - CH1

Note. The instrument will ganerate an error if the selected

channel is not present.

mode

Variable Type       int

Description:   Specifies the mode.

Variable Type: Integer

Valid Range:   0 = Continuous

1 = Gated

2 = Triggered

3 = Sweep

4 = Tone

Default Value: 0 - Continuous

triggersource

Variable Type       int

Description:   Specifies Trigger source for Gates, Triggered

and Tone modes.

Variable Type: Integer

Valid Range:   0 = Internal

1 = External

2 = Manual

3 = Previous chan

4 = Next chan

Default Value: 0 - Internal

internalperiod

Variable Type       double

Description:   Sets the period of the internal trigger

generator in seconds

Variable Type: double

Valid Range:   10us to 200s

Default Value: 1e-3 (1ms)

burstcount

Variable Type       int

Description:   The Burst count expressed in complete cycles of

the waveform. Used in triggered and gated modes.

Variable Type: Integer

Valid Range:   1 to 1048575

Default Value: 1

triggerslope

Variable Type       int

Description:   Set the Trigger slope.

Variable Type: Integer

Valid Range:   0 =  Positive

1 =  Negative

Default Value: 0 - positive

phaseangle

Variable Type       float

Description:   Specifies the start (and stop) phase for

triggered and gated modes expressed in degrees.

Variable Type: float

Valid Range:   -360 to +360 but dependent on selected waveform

Default Value: 0

Return Value

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Mode value out of range

-3   Trigger source value out of range

-4   Frequency value out of range

-5   Burst count value out of range

-10  Channel value out of range

100 to 199 are instrument execution errors

see the instrument instruction manual

for details.

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

tga1240_setup_seq

int tga1240_setup_seq (int instrumentID, int channel, int segmentno,

char arbwfmname[], int segstatus, int count, int stepon);

Purpose

Description:    Set the parameters for each sequence step on

the selected channel.

To run the sequence select it as the function

in the configure waveform window.

Parameter List

instrumentID

Variable Type       int

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

channel

Variable Type       int

Description:   Selects the channel to configure.

Variable Type: Integer

Valid Range:   0 =  CH1

1 =  CH2

2 =  CH3

3 =  CH4

Default Value: 0 - CH1

Note. The instrument will ganerate an error if the selected

channel is not present.

segmentno

Variable Type       int

Description:   Selects the segment number to edit.

Variable Type: Integer

Valid Range:   1 to 16

Default Value: 1

arbwfmname

Variable Type       char []

Description:   Specifies the arb waveform name for the segment.

Variable Type: String

Valid Range:   Must be an existing arb waveform.

Default Value: wv01

segstatus

Variable Type       int

Description:   Set the status of the current segment.

Variable Type: Integer

Valid Range:   0 = Off

1 = On

Default Value: 0 - Off

Note: Setting status off will also set all segments above to

off.

Setting status on will also set all segments below to on.

count

Variable Type       int

Description:   The count value.

Variable Type: Integer

Valid Range:   1 to 32767

Default Value: 1

stepon

Variable Type       int

Description:   Selects the step specification for the segment.

Variable Type: Integer

Valid Range:   0 = Count

1 = Trigger edge

2 = Trigger level

Default Value: 0 - Count

Return Value

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Segment no. out of range

-3   Seg status value out of range

-4   Step on value out of range

-10  Channel value out of range

100 to 199 are instrument execution errors

see the instrument instruction manual

for details.

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

tga1240_setup_sum

int tga1240_setup_sum (int instrumentID, int channel, int source, float ratio,

int attenuator);

Purpose

Description:    Set the summing for the selected channel..

Parameter List

instrumentID

Variable Type       int

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

channel

Variable Type       int

Description:   Selects the channel to configure.

Variable Type: Integer

Valid Range:   0 =  CH1

1 =  CH2

2 =  CH3

3 =  CH4

Default Value: 0 - CH1

Note. The instrument will ganerate an error if the selected

channel is not present.

source

Variable Type       int

Description:   Selects the summing source.

Variable Type: Integer

Valid Range:   0 = Off

2 = External

3 = Previous channel

Default Value: 0 - Off

ratio

Variable Type       float

Description:   Selects the summing ratio if the source is set

to previous channel.

Variable Type: Float

Valid Range:   Amplitude dependent

Default Value: 1.00

attenuator

Variable Type       int

Description:   Selects the attenuation applied to external

summing signals.

Variable Type: Float

Valid Range:   0 = 0dB

1 = 10dB

2 = 20dB

3 = 30dB

4 = 40dB

5 = 50dB

Default Value: 0 - 0dB

Return Value

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Source value out of range

-3   Attenuation value out of range

-10  Channel value out of range

100 to 199 are instrument execution errors

see the instrument instruction manual

for details.

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

tga1240_setup_sweep

int tga1240_setup_sweep (int instrumentID, int channel, int sweeptype,

int direction, int spacing, float startfrequency,

float stopfrequency, int sync, float duration,

float markerfrequency, int control);

Purpose

Description:   Sets the global sweep parameters.

The sweep parameters are common to all channels but each

channel may be set to sweep independantly.

Parameter List

instrumentID

Variable Type       int

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

channel

Variable Type       int

Description:   Selects the channel to set sweep on or off.

Variable Type: Integer

Valid Range:   0 =  CH1

1 =  CH2

2 =  CH3

3 =  CH4

Default Value: 0 - CH1

Note. The instrument will ganerate an error if the selected

channel is not present.

The sweep parameters are common to all channels but each

channel may be set to sweep independantly.

sweeptype

Variable Type       int

Description:   Sets the type of sweep.

Variable Type: Integer

Valid Range:   0 = Continuous

1 = Triggered

2 = Triggered hold and reset

Default Value: 0 - Continuous

direction

Variable Type       int

Description:   Set the sweep direction.

Variable Type: Integer

Valid Range:   0 = Up

1 = Down

2 = Up-down

3 = Down-up

Default Value: 0 - Up

spacing

Variable Type       int

Description:   Controls the sweep spacing.

Variable Type: Integer

Valid Range:   0 = Linear

1 = Logarithmic

3 = Off

Default Value: 0 - Linear

startfrequency

Variable Type       float

Description:   Set the sweep start frequency in Hz.

Variable Type: Float

Valid Range:   0.1 to 16e6

Default Value: 1000

stopfrequency

Variable Type       float

Description:   Set the sweep stop frequency in Hz.

Variable Type: Float

Valid Range:   0.1 to 16e6

Default Value: 10000

sync

Variable Type       int

Description:   Set sync sweep start o on or off.

Variable Type: Integer

Valid Range:   0 = Off

1 = On

Default Value: 0 - Off

duration

Variable Type       float

markerfrequency

Variable Type       float

Description:   Set the sweep marker frequency in Hz.

Variable Type: Float

Valid Range:   0.1 to 16e6

Default Value: 5000

control

Variable Type       int

Description:   Turns the sweep on or off.

Variable Type: Integer

Valid Range:   1 = On

0 = Off

Default Value: 0 - Off

Return Value

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Control value out of range

-3   Sweep type value out of range

-4   Direction value out of range

-5   Spacing value out of range

-6   Sync value out of range

-10  Channel value out of range

100 to 199 are instrument execution errors

see the instrument instruction manual

for details.

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

tga1240_setup_tone

int tga1240_setup_tone (int instrumentID, int channel, int tonenumber,

float tonefrequency, int tonetype);

Purpose

Description:    Set the frequencies for the tone list on the

selected channel.

Parameter List

instrumentID

Variable Type       int

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

channel

Variable Type       int

Description:   Selects the channel to configure.

Variable Type: Integer

Valid Range:   0 =  CH1

1 =  CH2

2 =  CH3

3 =  CH4

Default Value: 0 - CH1

Note. The instrument will ganerate an error if the selected

channel is not present.

tonenumber

Variable Type       int

Description:   Selects the tone number to edit.

Variable Type: Integer

Valid Range:   1 to 16

Default Value: 1

tonefrequency

Variable Type       float

Description:   The tone frequency expressed in Hz.

Variable Type: float

Valid Range:   1e-4 to 10e6

Default Value: 1000

tonetype

Variable Type       int

Description:   Selects type of switching between tones.

Variable Type: Integer

Valid Range:   0 = Gate

1 = Trig

2 = FSK

Default Value: 0 - Gate

Return Value

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Tone type value out of range

-3   Tone number value out of range

-4   Frequency value out of range

-5   DC offset value out of range

-6   Freq/per value out of range

-8   Zout value out of range

-9   Sense value out of range

-10  Channel value out of range

100 to 199 are instrument execution errors

see the instrument instruction manual

for details.

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

tga1240_set_inter

int tga1240_set_inter (int instrumentID, int channel, int mode,

float phaseangle, int status);

Purpose

Description:    Sets the inter channel phase locking mode and

phase angle for the selected channel and

allows the setting of the instrument locking

status.

Parameter List

instrumentID

Variable Type       int

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

channel

Variable Type       int

Description:   Selects the channel to configure.

Variable Type: Integer

Valid Range:   0 =  CH1

1 =  CH2

2 =  CH3

3 =  CH4

Default Value: 0 - CH1

Note. The instrument will ganerate an error if the selected

channel is not present.

mode

Variable Type       int

Description:   Sets the locking mode.

Variable Type: Integer

Valid Range:   0 = Independent

2 = Master

3 = Master with frequency tracking

4 = Slave

Default Value: 0 - Independent

phaseangle

Variable Type       float

Description:   Specifies the lock phase angle expressed in

degrees.

Variable Type: float

Valid Range:   -360 to +360 but dependent on selected waveform

Default Value: 0

status

Variable Type       int

Description:   Sets the instrument locking status

Variable Type: Integer

Valid Range:   0 = Off

1 = On

Default Value: 0 - Off

Return Value

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Mode value out of range

-3   Status value out of range

-10  Channel value out of range

100 to 199 are instrument execution errors

see the instrument instruction manual

for details.

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

tga1240_set_status

int tga1240_set_status (int instrumentID, int setStatusEnable, int value);

Purpose

Description:    Specify the way in which status is

reported and Service Request is

generated as defined in the IEEE

488.2 standard.

The function provides access to

the three status enable registers

in the TG1010, these are:

Service Request Enable Register

- Specifies which bits, when set,

in the Status Byte register will

generate a Service Request.

Standard Event Enable Register

- Specifies which bits, when set,

in the Standard Event Status

register will set the ESB bit in

the Status Byte Register.

Parallel Poll Enable Register

- Specifies which bits, when set,

in the Status Byte register will

be used to form the ist local

message

See the IEEE 488.2 Standard and

the instrument  instruction book

for a full description of status

reporting capabilities.

Parameter List

instrumentID

Variable Type       int

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

setStatusEnable

Variable Type       int

Description:    Used to select which enable

register will receive the byte

value.

Variable type:  integer

Valid range:    0 = SRQ enable

1 = Standard Event Enable

2 = Parallel Poll

Default value   0 - SRQ Enable

value

Variable Type       int

Description:    A byte value which will be written

to the selected enable register.

Variable type:  integer

Valid range:    0 to 255

Default value:  0

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Status Enable value out of range

-3   Register Value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

tga1240_snd_cmd

int tga1240_snd_cmd (int instrumentID, char command[]);

Purpose

Description:    Sends a string of bytes to the

instrument from a string variable.

Parameter List

instrumentID

Variable Type       int

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

command

Variable Type       char []

Description:    This control specifies data to be

sent to the instrument. The absolute

maximum length allowed is 200

bytes.

Variable type:  String

Return Value

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out