Thurlby Thandar 1906 Multimeter

By John Tothill

First Issue V1.0 November 1994

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for Thurlby Thandar 1906

Multimeter.

It contains functions for opening, configuring, taking measurement from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

For GPIB instruments drivers:

-  the instrument is connected to the GPIB.

-  the GPIB address supplied to the initialize function must match the

GPIB address of the instrument.

-  there must be at least one unused device name in the handler.

________________________________________________________________________________

LabWindows Libraries:

In order to use this instrument driver outside of the LabWindows

environment, it must be linked with the following LabWindows libraries:

For GPIB instruments drivers:

- 	GPIB Library

- 	Format and I/O Library

-	LabWindows System Library

National Instruments strongly recommends that you use the LWMAKE utility to

build any standalone executable program based on this instrument driver.

________________________________________________________________________________

Error Codes:

Error codes are returned as the return value of each instrument driver

function. A program should examine this value after each call to an instrument

driver function to determine if any error occurred. Possible error codes and

their meanings are listed with the corresponding instrument driver function.

________________________________________________________________________________

How To Use This Document:

This document is intended to be used as a programming reference manual.

It describes each function in the Thurlby Thandar 1906 Multimeter

instrument driver. The functions are listed in alphabetical order with

a description of the function, C and BASIC syntax of the function,

a description of each parameter, and possible error codes.

________________________________________________________________________________

Function Tree Layout:

Thurlby Thandar 1906 Multimeter     Function Name:

Initialize                       tt1906_init

Configure Function/Range         tt1906_config

Measure                          tt1906_measure

Trigger                          tt1906_trig

Programs

Delta Percent                 tt1906_delta

DB                            tt1906_db

Ax+B                          tt1906_axb

Limits                        tt1906_limits

Min/Max                       tt1906_minmax

Null                          tt1906_null

Local Storage                    tt1906_store

Status Reporting

Set Status                    tt1906_set_stat

Get Status                    tt1906_get_stat

Miscellaneous

Send Command String           tt1906_send_cmd

Get Respinse String           tt1906_get_resp

File Operations               tt1906_file

Close                            tt1906_close

________________________________________________________________________________

Instrument         : Thurlby Thandar 1906 Multimeter

Instrument Name: Thurlby Thandar 1906 Multimeter

Description:     This instrument module provides

programming support for the

TTi 1906 multimeter.  The module

is divided into the following

functions:

Functions/Classes:

(1) Initialize

Initialize the instrument and set to default

configuration.

(2) Configure

Sets the measurement function and range and

other minor parameters.

(3) Measure

Returns readings, max/min values or limit

values.

(4) Trigger

Allows the instrument to be triggered to make

a reading.

(5) Programs

This Class of functions gives access to the

computational capabilities of the instrument.

-Delta percent. Computes the offset about a

reference value.

-DB. Computes the readings in deci-Bels from

a reference value

-Ax+B. Performs scaling and offset

calculations on the readings.

-Limits. Compares Readings to reference

limits.

-Min/Max. Computes and stores the maximum and

minimum readings over time.

-Null. Uses the current reading to offset

future readings.

(6) Local Storage. Gives access to the local

non-volatile stores in the instrument.

(7) Status Reporting

This class of functions gives access to the

status reporting capabilities of the

instrument.

-Set Status. Determines how statue will be

reported.

-Get Status. Returns the requested status

register value.

(8) Miscellaneous

This class of functions allows access to

additional 1906 remote control commands

which is not available elsewhere in this

driver. Also included are file operations.

(1) Send Command String

(2) Get Response String

(3) File Operations

(9)  Close

Take the instrument offline.

________________________________________________________________________________

Following functions are in alphabetical order.

________________________________________________________________________________

Function Name      : tt1906_axb

C Syntax           : int tt1906_axb(Instrument_ID, Enable, A, B);

int Instrument_ID;

int Enable;

double A;

double B;

BASIC Syntax       : Error% = tt1906.axb(Instrument.ID%, Enable%, A#, B#)

Purpose            :  Panel Name:    Ax+B

Description:   Performs scaling and offset

calculations on the readings.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Enable

0 - Off (Default)

1 - On

2 - On + config.

Control Name:  Enable

Description:   Sets the Ax+B computational

program of or on.

Variable Type: Integer

Valid Range:   0 = Off

1 = On

2 = On and configure

Default Valeu: 0 - Off

A

Control Name:  A

Description:   Sets the A value for the Ax+B

program.

Variable Type: Double

Valid Range:   -999999 to +999999

Default Value: 1.0

Note: This value is ignored if the Enable is set

to Off or On.

B

Control Name:  B

Description:   Sets the B value for the Ax+B

program.

Variable Type: Double

Valid Range:   Range/Function dependent

Default Value: 0.0

Note: This value is ignored if the Enable is set

to Off or On.

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   ID Query out of range

-2   Enable value out of range

-3   The A value is out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

240   Instrument timed out

319   The B value is out of range

________________________________________________________________________________

Function Name      : tt1906_close

C Syntax           : int tt1906_close(Instrument_ID);

int Instrument_ID;

BASIC Syntax       : Error% = tt1906.close(Instrument.ID%)

Purpose            :  Panel Name:    Close

Description:   Closes the instrument and takes it

off line.

Note: The  instrument must  be reinitialized

to use it again.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

221   Unable to close instrument

232   Instrument not initialized

________________________________________________________________________________

Function Name      : tt1906_config

C Syntax           : int tt1906_config(Instrument_ID, Function, Range,

Resolution, Hi_Z, Filter, Trigger_Set,

Cancel_All_Programs);

int Instrument_ID;

int Function;

int Range;

int Resolution;

int Hi_Z;

int Filter;

int Trigger_Set;

int Cancel_All_Programs;

BASIC Syntax       : Error% = tt1906.config(Instrument.ID%, Function%, _

Range%, Resolution%, Hi.Z%, Filter%, _

Trigger.Set%, Cancel.All.Programs%)

Purpose            :  Panel Name:    Configure  Function/Range

Description:   Configures the instrument to take

readings.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Function

0 - Volts DC (Default)

1 - Volts AC

2 - mA DC

3 - mA AC

4 - 10A DC

5 - 10A AC

6 - Ohms

Control Name:  Function

Description:   Sets the measurement function of

the instrument

Variable Type: Integer

Valid Range:   0 = Volts DC

1 = Volts AC

2 = mA DC

3 = mA AC

4 = 10A DC

5 = 10A AC

6 = Ohms

Default value: 0 - Volts DC

Range

0 - 210mV,uA,

2 - 21V,mA,k

4 - 2.1kV,-,M

6 - Autoranging (Default)

7 - Manual

Control Name:  Range

Description:   Sets the measurement range. If the

range is illegal for the selected

function an error will be

returned. Unsuported combinations

such as 21kV are detected by this

driver and no command will be sent

to the instrument in these cases.

See the Error control for a full

list of errors.

Variable Type: Integer

Valid Range:   0 = 210,mV,uA,

2 =  21, V,mA,k

4 = 2.1,kV.--.M

6 = Autoranging

7 = Manual - locks instrument to

the present range

Default Value: 6 - Autoranging

Resolution

1 - 4.5dig

0 - 5.5dig (Default)

Control Name:  Resolution

Description:   Selects 4.5 or 5.5 digit mode

Variable Type: Integer

Valid Range:   0 = 5.5

1 = 4.5

Default Value: 0 - 5.5

Hi Z

1 - On

0 - Off (Default)

Control Name:  Hi Z

Description:   Sets very high input impedance

(>1 GOhm) for the 210mV and 2.1V

DC ranges. Has no effect on other

ranges.

Variable Type: Integer

Valid Range:   0 = Off

1 = On

Default Value: 0 - Off

Filter

Control Name:  Filter

Description:   Set the digital reading filter to

one of the ten possibilities

described in the instrument

manual.

Variable Type: Integer

Valid Range:   0 to 9

default Value: 0 - uses the defaut values

optimised for each range and

function.

Trigger Set

1 - Nxt Reading (Default)

0 - Filtered

Control Name:  Trigger Set

Description:   Set the type of reading trigger to

use when the instrument is

triggered.

Note: The Read control on the

Measure panel must be set to

Triggered for this control to have

any effect. The filter setting on

this panel also determines how

the readings will be made if this

control is set to filtered.

Variable Type: Integer

Valid Range:   0 = Filtered

1 = Next Reading

Default Value: 1 - Next Reading

Cancel All Programs

1 - Yes

0 - No (Default)

Control Name:  Cancel All Programs

Description:   If set to Yes all computational

operations will be set to their

inactive state.

Variable Type: Integer

Valid Range:   0 = No

1 = Yes

Default Value: 0 - No

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   ID Query out of range

-2   Function value out of range

-3   Range value out of range

-4   Resolution value out of range

-5   Hi Z value out of range

-6   Filter value out of range

-7   Trigger Set value out of range

-8   Cancel value out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

240   Instrument timed out

319   Range value illegal for the selected

function

________________________________________________________________________________

Function Name      : tt1906_db

C Syntax           : int tt1906_db(Instrument_ID, Enable, Reference_Value);

int Instrument_ID;

int Enable;

double Reference_Value;

BASIC Syntax       : Error% = tt1906.db(Instrument.ID%, Enable%, _

Reference.Value#)

Purpose            :  Panel Name:    DB

Description:   Computes the readings in deci-Bels

from a reference value

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Enable

0 - Off (Default)

1 - On

2 - On + config.

Control Name:  Enable

Description:   Sets the DB computational

program of or on.

Variable Type: Integer

Valid Range:   0 = Off

1 = On

2 = On and configure

Default Valeu: 0 - Off

Reference Value

Control Name:  Reference Value

Description:   Sets the reference value for the

DB program.

Variable Type: Double

Valid Range:   0.001 to 9.999

Default Value: 1.0

Note: This value is ignored if the Enable is set

to Off or On.

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   ID Query out of range

-2   Enable value out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

240   Instrument timed out

319   Reference value out of range

________________________________________________________________________________

Function Name      : tt1906_delta

C Syntax           : int tt1906_delta(Instrument_ID, Enable,

Reference_Value);

int Instrument_ID;

int Enable;

double Reference_Value;

BASIC Syntax       : Error% = tt1906.delta(Instrument.ID%, Enable%, _

Reference.Value#)

Purpose            :  Panel Name:    Delta percent

Description:   Computes the offset about a

reference value.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Enable

0 - Off (Default)

1 - On

2 - On + config.

Control Name:  Enable

Description:   Sets the Delta Percent

computational program of or on.

Variable Type: Integer

Valid Range:   0 = Off

1 = On

2 = On and configure

Default Valeu: 0 - Off

Reference Value

Control Name:  Reference Value

Description:   Sets the reference value for the

Delta Percent program.

Variable Type: Double

Valid Range:   Range/Function dependent

Default Value: 0.0

Note: This value is ignored if the Enable is set

to Off or On.

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   ID Query out of range

-2   Enable value out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

240   Instrument timed out

319   Reference value out of range

________________________________________________________________________________

Function Name      : tt1906_file

C Syntax           : int tt1906_file(Instrument_ID, Operation, File_Name);

int Instrument_ID;

int Operation;

char *File_Name;

BASIC Syntax       : Error% = tt1906.file(Instrument.ID%, Operation%, _

File.Name$)

Purpose            :  Panel Name:    File Operations

Description:   Send or Receive a binary block to

or from the 1906. The data will

be read from or written to a file.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Operation

0 - Send File (Default)

1 - Get File

Control Name:  Operation

Description:   Selects to send a file to the

1906 or Get a file from 1906.

Variable Type: Integer

Valid Range:   0 = Send file to 1906

1 = Get file from 1906

Default Value: 0 - Send file

File Name

Control name:   File name

Description:    This control specifies file name

from which the binary block will

be sent to the 1906.

Variable type:  String - max length 50 characters

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Operation value out of range

228   Error writing to instrument from file

229   Error reading from instrument to file

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tt1906_get_resp

C Syntax           : int tt1906_get_resp(Instrument_ID, Response,

Number_Of_Bytes);

int Instrument_ID;

char *Response;

int *Number_Of_Bytes;

BASIC Syntax       : Error% = tt1906.get.resp(Instrument.ID%, Response$, _

Number.Of.Bytes%)

Purpose            :  Panel Name:    Get Response String

Description:   Reads a response from the 1906

into a string variable.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Response

Control name:   Response Stirng

Description:    This control specifies a string

variable to receive the response

from the 1906. The string must

be long enough to receive the

response to avoid overflow. To aid

in checking this the number of

bytes put into the string is

returned from the function. The

absolute maximum length allowed is

2000 bytes.

Variable type:  String

Number Of Bytes

Control name:   Number of Bytes

Description:    This value is the number of bytes

returned by the 1906 and

placed in the string.

Variable type:  Integer

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tt1906_get_stat

C Syntax           : int tt1906_get_stat(Instrument_ID, Read_Status, Value);

int Instrument_ID;

int Read_Status;

int *Value;

BASIC Syntax       : Error% = tt1906.get.stat(Instrument.ID%, Read.Status%, _

Value%)

Purpose            :  Panel Name:    Get Status

Description:   Returns the requested status

register value.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Read Status

0 - Status byte (Default)

1 - Std Event

2 - SRQ enable

3 - Event Enable

4 - Parallel Pol

5 - Exec. Error

Control name:   Read Status

Description:    Used to select which status

register to read

Variable type:  integer

Valid range:    0 = Status Byte

1 = Standard Event Status

2 = SRQ enable

3 = Standard Event Enable

4 = Parallel Poll Enable

5 = Execution Error Register

Default value   0 - Status Byte

Value

Control name:   Value

Description:    Returns the value read from the

selected register.

Variable type:  Integer

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Status Enable value out of range

-3   Register Value out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

240   Instrument timed out

________________________________________________________________________________

Function Name      : tt1906_init

C Syntax           : int tt1906_init(Address, ID_Query, Reset_Device,

Instrument_ID);

int Address;

int ID_Query;

int Reset_Device;

int *Instrument_ID;

BASIC Syntax       : Error% = tt1906.init(Address%, ID.Query%, Reset.Device%, _

Instrument.ID%)

Purpose            :  Panel Name:    Initialize

Description:   Initializes the instrument in the

following ways:

- opens the instrument and sets

its GPIB address to the speci-

fied value

- performs an identification query

on the instrument

- sets the instrument to a default

configuration

- returns an instrument ID which

is used to identify the

instrument in subsequent calls.

Parameter List     : Address

Control Name:  Address

Description:   Specifies the address on the

GPIB to be used to communicate

with the instrument.  This value

must correspond to the current

internal setting of the

instrument.

Variable Type: Integer

Valid Range:   0 to 30

Default Value: 1

NOTE:  The Default Value of this control may be

set to the factory default of the

instrument, however if your actual

instrument setting is changed this

control's default should also also be

changed.

ID Query

1 - Do Query (Default)

0 - Skip Query

Control Name:  ID Query

Description:   Specifies if an ID Query is sent

to the instrument during the

initialization procedure.

Variable Type: Integer

Valid Range:   1 = Yes

0 = No

Default Value: 1 - Yes

NOTE:  Under normal circumstances the ID Query

insures that the instrument initialized

over the GPIB is the type supported by

this driver. However circumstances may

arise where it is undesirable to send an

ID Query to the instrument. In those

cases; set this control to Skip Query

and this function will initialize the

GPIB and the Command arrays in the driver,

without doing an ID Query.

Reset Device

1 - Reset Device (Default)

0 - Don't Reset

Control Name:  Reset

Description:   Specifies if the instrument is to

be reset to its power-on settings

during the initialization

procedure.

Variable Type: Integer

Valid Range:   1 = Yes

0 = No

Default Value: 1 - Yes

NOTE: If you do not want the instrument reset.

Set this control to No while initializing

the instrument.

Instrument ID

Control Name:  Instrument ID

Description:   Returns an Instrument ID that is

used in all subsequent function

calls to select the instrument at

the Address specified.

If more than one instrument of the

same type is used, this value will

be used to differentiate between

them. The first instrument

initialized will be assigned an

Instrument ID of 1.

Variable Type: Integer

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   GPIB Address out of range

-2   ID Query out of range

-3   Reset Device out of range

220   Unable to open instrument

221   Unable to close instrument

223   Instrument identification query failed

224   Error clearing instrument

225   Error triggering instrument

226   Error polling instrument

228   Error writing to instrument from file

229   Error reading from instrument to file

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

234   Error placing instrument in local mode

236   Error interpreting instrument response

239   Error configuring time out

240   Instrument timed out

3xx   Instrument Specific errors

NOTES: (1) This control should be all the way

down and one space out of the lower

right corner of all function panels.

(2) When finished with the instrument

driver, each function panel should

have all unused error codes deleted.

________________________________________________________________________________

Function Name      : tt1906_limits

C Syntax           : int tt1906_limits(Instrument_ID, Enable, Low_Limit,

High_Limit);

int Instrument_ID;

int Enable;

double Low_Limit;

double High_Limit;

BASIC Syntax       : Error% = tt1906.limits(Instrument.ID%, Enable%, _

Low.Limit#, High.Limit#)

Purpose            :  Panel Name:    Limits

Description:   Compares Readings to reference

limits.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Enable

0 - Off (Default)

1 - On

2 - On + config.

Control Name:  Enable

Description:   Sets the Limits computational

program of or on.

Variable Type: Integer

Valid Range:   0 = Off

1 = On

2 = On and configure

Default Valeu: 0 - Off

Low Limit

Control Name:  Low Limit

Description:   Sets the Low Limit value for the

Limits program.

Variable Type: Double

Valid Range:   Range/Function dependent

Default Value: 0.0

Note: This value is ignored if the Enable is set

to Off or On.

High Limit

Control Name:  Hign Limit

Description:   Sets the High Limitvalue for the

Limits program.

Variable Type: Double

Valid Range:   Range/Function dependent

Default Value: 0,0

Note: This value is ignored if the Enable is set

to Off or On.

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   ID Query out of range

-2   Enable value out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

240   Instrument timed out

319   Low or High value is out of range

________________________________________________________________________________

Function Name      : tt1906_measure

C Syntax           : int tt1906_measure(Instrument_ID, Read, Result_Numeric,

Result_String);

int Instrument_ID;

int Read;

double *Result_Numeric;

char *Result_String;

BASIC Syntax       : Error% = tt1906.measure(Instrument.ID%, Read%, _

Result.Numeric#, Result.String$)

Purpose            :  Panel Name:    Measure

Description:   Returns readings, max/min values

or limit values.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Read

0 - Immediate (Default)

1 - Triggered

2 - Limits

3 - Min/Max

Control Name:  Read

Description:   Selects the type of reading to

return.

Variable Type: Integer

Valid Range:   0 = Immediate (Note 1)

1 = Triggered (Note 2)

2 = Limits    (Note 3)

3 = Min/Max   (Note 4)

Default Value: 0 - immediate

Notes:

(1) Immediate returns the next reading

immediately after the command is received.

The reading is placed in both the Result

Numeric and Result String variables

(2) Triggered returns the next reading

after the instrument is triggered, See the

Trigger Panel for more information.

(3) Returns the results of the Limits

program in the Result String variable.

(4) Returns the recorded Min and Max

values in the Result String variable.

Result Numeric

Control Name:  Result Numeric

Description:   Contains the result of an

Immediate read command. This

variable will contain zero for any

other Read operation.

Variable Type: Double

Note:  To receive the result of a Triggered Read

you must execute the Trigger panel and use

the values returned from there.

Result String

Control Name:  Result String

Description:   Contaims the response string data

from Immediate, Limits and Min/Max

Read operations. An empty string

will result from a Triggered Read

operation.

Variable Type: String

Note:  To receive the result of a Triggered Read

you must execute the Trigger panel and use

the values returned from there.

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   ID Query out of range

-2   Read control value out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

240   Instrument timed out

________________________________________________________________________________

Function Name      : tt1906_minmax

C Syntax           : int tt1906_minmax(Instrument_ID, Enable);

int Instrument_ID;

int Enable;

BASIC Syntax       : Error% = tt1906.minmax(Instrument.ID%, Enable%)

Purpose            :  Panel Name:    Min/Max

Description:  Computes and stores the maximum and

minimum readings over time.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Enable

0 - Off (Default)

1 - On

Control Name:  Enable

Description:   Sets the Min Max computational

program of or on.

Variable Type: Integer

Valid Range:   0 = Off

1 = On

Default Valeu: 0 - Off

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   ID Query out of range

-2   Enable value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tt1906_null

C Syntax           : int tt1906_null(Instrument_ID, Enable);

int Instrument_ID;

int Enable;

BASIC Syntax       : Error% = tt1906.null(Instrument.ID%, Enable%)

Purpose            :  Panel Name:    Null

Description:   Uses the current reading to offset

future readings.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Enable

0 - Off (Default)

1 - On

Control Name:  Enable

Description:   Sets the Null computational

program of or on.

Variable Type: Integer

Valid Range:   0 = Off

1 = On

Default Valeu: 0 - Off

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   ID Query out of range

-2   Enable value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

321   In porgram - see instrument manual for

more information

________________________________________________________________________________

Function Name      : tt1906_send_cmd

C Syntax           : int tt1906_send_cmd(Instrument_ID, Command);

int Instrument_ID;

char *Command;

BASIC Syntax       : Error% = tt1906.send.cmd(Instrument.ID%, Command$)

Purpose            :  Panel Name:    Send Command String

Description:   Sends a string of bytes to the

instrument from a string variable.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Command

Control name:   Command String

Description:    This control specifies data to be

sent to the 1906. The absolute

maximum length allowed is 200

bytes.

Variable type:  String

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tt1906_set_stat

C Syntax           : int tt1906_set_stat(Instrument_ID, Set_Status_Enable,

Value);

int Instrument_ID;

int Set_Status_Enable;

int Value;

BASIC Syntax       : Error% = tt1906.set.stat(Instrument.ID%, _

Set.Status.Enable%, Value%)

Purpose            :  Panel Name:    Set Status

Description:   Determines how status will be

reported.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Set Status Enable

0 - SRQ enable (Default)

1 - Event Enable

2 - Parallel Pol

Control name:   Set Status Enable

Description:    Used to select which enable

register will receive the byte

value.

Variable type:  integer

Valid range:    0 = SRQ enable

1 = Standard Event Enable

2 = Parallel Poll

Default value   0 - SRQ Enable

Value

Control name:   Value

Description:    A byte value which will be written

to the selected enable register.

Variable type:  integer

Valid range:    0 to 255

Default value:  0

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Status Enable value out of range

-3   Register Value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tt1906_store

C Syntax           : int tt1906_store(Instrument_ID, Operation,

Store_Number);

int Instrument_ID;

int Operation;

int Store_Number;

BASIC Syntax       : Error% = tt1906.store(Instrument.ID%, Operation%, _

Store.Number%)

Purpose            :  Panel Name:    Local Storage

Description:   Gives access to the local

non-volatile stores in the

instrument.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Operation

0 - Recall (Default)

1 - Store

Control Name:  Operation

Description:   Selects the recall or store

operation.

Variable Type: Integer

Valid Range:   0 = Recall

1 = Store

Default Value: 0 - Recall

Store Number

Control name:   Store Number

Description:    Select the number of the store to

Recall from or Store to.

Variable type:  integer

Valid range:    0 to 5

Default value:  1

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   instrument ID out of range

-2   Operation value out of range

-3   Store Number out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tt1906_trig

C Syntax           : int tt1906_trig(Instrument_ID, Result_Numeric,

Result_String);

int Instrument_ID;

double *Result_Numeric;

char *Result_String;

BASIC Syntax       : Error% = tt1906.trig(Instrument.ID%, Result.Numeric#, _

Result.String$)

Purpose            :  Panel Name:    Trigger

Description:   Triggers the instrument to make a

reading then reads the result.

Note:  Before executing this function a Triggered

Read must have been executed from the

Measure function

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Result Numeric

Control Name:  Result Numeric

Description:   Contains the result of an

Triggered read command.

Variable Type: Double

Note:  Before executing this function a Triggered

Read must have been executed from the

Measure function if not Error 231 will be

returned

Result String

Control Name:  Result String

Description:   Contaims the response string data

from a Triggered Read command

Variable Type: String

Note:  Before executing this function a Triggered

Read must have been executed from the

Measure function if not Error 231 will be

returned

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   ID Query out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

240   Instrument timed out

===============================================================================