Thurlby-Thandar TPLP series PSU.

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for Thurlby-Thandar TPLP

series PSU..

It contains functions for opening, configuring, taking measurement from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

-  the instrument is connected to the GPIB.

-  the GPIB address supplied to the initialize function must match the

GPIB address of the instrument.

-  there must be at least one unused device name in the handler.

________________________________________________________________________________

LabWindows Libraries:

In order to use this instrument driver outside of the LabWindows

environment, it must be linked with the following LabWindows libraries:

-  GPIB Library

-  Format and I/O Library

-  LabWindows System Library

National Instruments strongly recommends that you use the LWMAKE utility to

build any standalone executable program based on this instrument driver.

________________________________________________________________________________

Error Codes:

Error codes are returned as the return value of each instrument driver

function. A program should examine this value after each call to an instrument

driver function to determine if any error occurred. Possible error codes and

their meanings are listed with the corresponding instrument driver function.

________________________________________________________________________________

How To Use This Document:

This document is intended to be used as a programming reference manual.

It describes each function in the

Thurlby-Thandar TPLP series PSU.

instrument driver. The functions are listed in alphabetical order with

a description of the function, C and BASIC syntax of the function,

a description of each parameter, and possible error codes.

________________________________________________________________________________

Function Tree Layout:

Thurlby-Thandar TPLP series PSU.     Function Name:

Initialise                        tplp_init

Configure

Set Volts                      tplp_set_volts

Set Current                    tplp_set_current

Set Output Status              tplp_set_op_stat

Set Meter Damping              tplp_set_damping

Output Switch Control          tplp_op_switch

Status Reporting               tplp_set_status

Measurement and Status

Read Output Volts              tplp_read_op_volts

Read Output Current            tplp_read_op_current

Read Status                    tplp_read_status

Miscellaneous

Send Command String            tplp_send_cmd

Get Response String            tplp_get_response

Close                             tplp_close

________________________________________________________________________________

Instrument         : Thurlby-Thandar TPLP series PSU.

Thurlby-Thandar TPL----P series power supplies

This instrument module provides GPIB support for

the TPL----P series power supplies. Instruments in

this range meet the requirements of IEEE 488.2

standard. The module will be updated from time to

time as new instruments become available in the

range.

Instruments supported are:

PL330P        32 volt, 3 amp

PL330DP       32 volt, 3 amp Dual

PL330TP       32 volt, 3 amp Dual + 5 volt, 7 amp

The module contains the following functions:

1. Initialise.

2. Configuration

1. Set Volts

2. Set Current

3. Set Output Status

4. Set Meter Damping

5. Output switch controls

6. Status Reporting

3. Measurement and Status.

1. Read Voltage

2. Read Current

3. Read Status

4. Miscellaneoue

1. Send Command String

2. Get Response String

5. Close

________________________________________________________________________________

Following functions are in alphabetical order.

________________________________________________________________________________

Function Name      : tplp_close

C Syntax           : int tplp_close(Instrument_ID);

int Instrument_ID;

BASIC Syntax       : Error% = tplp.close(Instrument.ID%)

Purpose            : Panel name:     Close

Description:    Takes the instrument off-line.

The  instrument must  be

reinitialized to use it again.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

221   Unable to close instrument

232   Instrument not initialized

________________________________________________________________________________

Function Name      : tplp_get_response

C Syntax           : int tplp_get_response(Instrument_ID, Response_String,

Number_of_Bytes);

int Instrument_ID;

char *Response_String;

int *Number_of_Bytes;

BASIC Syntax       : Error% = tplp.get.response(Instrument.ID%, _

Response.String$, Number.of.Bytes%)

Purpose            : Panel name:     Get response string

Description:    Reads a response from the TPL----P

into a string variable.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Response String

Control name:   Response Stirng

Description:    This control specifies a string

variable to receive the response

from the TPL----p. The string must

be long enough to receive the

response to avoid overflow. To aid

in checking this the number of

bytes put into the string is

returned from the function. The

absolute maximum length allowed is

2000 bytes.

Variable type:  String

Number of Bytes

Control name:   Number of Bytes

Description:    This value is the number of bytes

returned by the TPL----P and

placed in the string.

Variable type:  Integer

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tplp_init

C Syntax           : int tplp_init(GPIB_Address, Set_Defaults,

Instrument_ID);

int GPIB_Address;

int Set_Defaults;

int *Instrument_ID;

BASIC Syntax       : Error% = tplp.init(GPIB.Address%, Set.Defaults%, _

Instrument.ID%)

Purpose            : Panel name:     Initialise

Description:

Initialises the TPL----P in the following way

- opens the instrument and sets the GPIB

address to the specified value.

- sends device clear and clear status

commands.

- queries the instrument to determine the

type/model number.

- optionally sets the instrument to its

default state.

Parameter List     : GPIB Address

Control name:   GPIB Address

Description:    This control specifies the

address used to communicate with

the TPL----P power supply. It must

correspond with the value set up

at the instrument rear panel.

Variable type:  Integer

Valid range:    0 to 30

Default value:  11

Set Defaults

1 - Yes

0 - No (Default)

Control name:   Set Defaults

Description:    When set to Yes the parameters

in the instrument will be set to

their default values. If set to No

the instrument settings will not

be changed.

Variable type:  Integer

Valid range:    No = 0 , Yes = Any non-zero value

Default value:  No

Instrument ID

Control Name:  Instrument ID

Description:   Returns an Instrument ID that is

used in all subsequent function

calls to select the instrument at

the Address specified.

If more than one instrument of the

same type is used, this value will

be used to differentiate between

them. The first instrument

initialized will be assigned an

Instrument ID of 1.

Variable Type: Integer

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   GPIB Address out of range

-2   Reset Device out of range

220   Unable to open instrument

221   Unable to close instrument

223   Instrument identification query failed

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

240   Instrument timed out

________________________________________________________________________________

Function Name      : tplp_op_switch

C Syntax           : int tplp_op_switch(Instrument_ID, Master_Output);

int Instrument_ID;

int Master_Output;

BASIC Syntax       : Error% = tplp.op.switch(Instrument.ID%, Master.Output%)

Purpose            : Panel name:     Output Switch Control

Description:

Set all output switches on or off.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Master Output

1 - On

0 - Off (Default)

Control name:   Master Output

Description:    Used to set the all outputs of the

instrument on or off together.

Variable type:  integer

Valid range:    0 = OFF

1 = ON

default value:  Off

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Output switch value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tplp_read_op_current

C Syntax           : int tplp_read_op_current(Instrument_ID, Output_Number,

Output_Current);

int Instrument_ID;

int Output_Number;

double *Output_Current;

BASIC Syntax       : Error% = tplp.read.op.current(Instrument.ID%, _

Output.Number%, Output.Current#)

Purpose            : Panel name:     Read Current

Description:

Returns the Current from the selected output.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Output Number

1 - 1 (Default)

2 - 2

3 - 3

Control name:   Output Number

Description:    Indicates the output number from

which the values will be read.

Variable type:  integer

Valid range:    min = 1 , max = number of outputs

available for the particular

instrument.

Default value   1

Output Current

Control Name:   Output current

Description:    Current being output by the power

supply.

Control Type:   Real

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Output number out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tplp_read_op_volts

C Syntax           : int tplp_read_op_volts(Instrument_ID, Output_Number,

Output_Voltage);

int Instrument_ID;

int Output_Number;

double *Output_Voltage;

BASIC Syntax       : Error% = tplp.read.op.volts(Instrument.ID%, _

Output.Number%, Output.Voltage#)

Purpose            : Panel name:     Read Volts

Description:

Returns the Voltage from the selected output.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Output Number

1 - 1 (Default)

2 - 2

3 - 3

Control name:   Output Number

Description:    Indicates the output number from

which the values will be read.

Variable type:  integer

Valid range:    min = 1 , max = number of outputs

available for the particular

instrument.

Default value   1

Output Voltage

Control Name:   Output voltage

Description:    Voltage being output from the

power supply.

Control Type:   Real.

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Output number out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tplp_read_status

C Syntax           : int tplp_read_status(Instrument_ID, Read_Status, Value);

int Instrument_ID;

int Read_Status;

int *Value;

BASIC Syntax       : Error% = tplp.read.status(Instrument.ID%, Read.Status%, _

Value%)

Purpose            : Panel name:     Read Status

Description:    Returns the byte value from the

selected status register.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Read Status

0 - Status byte (Default)

1 - Std Event

2 - Limit Event

3 - SRQ enable

4 - Event Enable

5 - Limit Enable

6 - Parallel Pol

Control name:   Read Status

Description:    Used to select which status

register to read

Variable type:  integer

Valid range:    0 = Statue byte

1 = Standard event

2 = Limit event

3 = ERQ enable

4 = Event enable

5 = Limit enable

6 = Parallel poll enable

Default value   0 (Status Byte)

Value

Control Name:   Value

Description:    byte value of the

selected register.

Control Type:   integer

Valid range:    0 to 255

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Register out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tplp_send_cmd

C Syntax           : int tplp_send_cmd(Instrument_ID, Command_String);

int Instrument_ID;

char *Command_String;

BASIC Syntax       : Error% = tplp.send.cmd(Instrument.ID%, Command.String$)

Purpose            : Panel name:     Send command string

Description:    Sends a string of bytes to the

TPL----P from a string variable.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Command String

Control name:   Command String

Description:    This control specifies data to be

sent to the TPL----P. The absolute

maximum length allowed is 2000

bytes.

Variable type:  String

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tplp_set_current

C Syntax           : int tplp_set_current(Instrument_ID, Output_Number,

Current);

int Instrument_ID;

int Output_Number;

double Current;

BASIC Syntax       : Error% = tplp.set.current(Instrument.ID%, _

Output.Number%, Current#)

Purpose            : Panel name:     Set Current

Description:

Set current for each output.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Output Number

1 - 1 (Default)

2 - 2

3 - 3

Control name:   Output Number

Description:    Indicates the output number to

which the commands will be sent.

Variable type:  integer

Valid range:    min = 1 , max = number of outputs

available for the particular

instrument.

Default value   1

Current

Control name:   Current

Description:    The current limit value to which

the specified output will be set.

The value is expressed in Amps.

Variable type:  real

Valid range:    instrument dependent

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Output number out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tplp_set_damping

C Syntax           : int tplp_set_damping(Instrument_ID, Output_Number,

Meter_Damping);

int Instrument_ID;

int Output_Number;

int Meter_Damping;

BASIC Syntax       : Error% = tplp.set.damping(Instrument.ID%, _

Output.Number%, Meter.Damping%)

Purpose            : Panel name:     Set Meter damping

Description:

Set readback meter damping for each output to ON

or OFF.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Output Number

1 - 1 (Default)

2 - 2

3 - 3

Control name:   Output Number

Description:    Indicates the output number to

which the commands will be sent.

Variable type:  integer

Valid range:    min = 1 , max = number of outputs

available for the particular

instrument.

Default value   1

Meter Damping

1 - on

0 - Off (Default)

Control name:   Meter Damping

Description:    Specifies the dampimg applied to

the current metering of the

selected output.

Variable type:  integer

Valid range:    On = 1 , Off = 0

default value:  Off

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Output number out of range

-3   Meter damping value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tplp_set_op_stat

C Syntax           : int tplp_set_op_stat(Instrument_ID, Output_Number,

Output_Status);

int Instrument_ID;

int Output_Number;

int Output_Status;

BASIC Syntax       : Error% = tplp.set.op.stat(Instrument.ID%, _

Output.Number%, Output.Status%)

Purpose            : Panel name:     Set Output status

Description:

Set output status for each output to ON or OFF.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Output Number

1 - 1 (Default)

2 - 2

3 - 3

Control name:   Output Number

Description:    Indicates the output number to

which the commands will be sent.

Variable type:  integer

Valid range:    min = 1 , max = number of outputs

available for the particular

instrument.

Default value   1

Output Status

1 - On (Default)

0 - Off

Control name:   Output Status

Description:    Specifies the status of the

specified output.

Variable type:  integer

Valid range:    Off = 0 , On = 1

default value:  Off

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Output number out of range

-3   Output status value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tplp_set_status

C Syntax           : int tplp_set_status(Instrument_ID, Set_Status_Enable,

Value);

int Instrument_ID;

int Set_Status_Enable;

int Value;

BASIC Syntax       : Error% = tplp.set.status(Instrument.ID%, _

Set.Status.Enable%, Value%)

Purpose            : Panel name:     Status Reporting

Description:

Specify the way in which status is reported and

Service Request is generated as defined in the

IEEE 488.2 standard.

The function provides access to the four status

enable registers in the TPL----P power supply,

these are:

Service Request Enable Register

- Specifies which bits, when set, in the

Status Byte register will generate a

Service Request.

Standard Event Enable Register

- Specifies which bits, when set, in the

Standard Event Status register will set

the ESB bit in the Status Byte Register.

Limit Event Register

- Specifies which bits, when set, in the

Limit Event Status register will set the

LIM bit in the Status Byte Register.

Parallel Poll Enable Register

- Specifies which bits, when set, in the

Status Byte register will be used to form

the ist local message

See the IEEE 488.2 Standard and the instrument

instruction book for a full description of status

reporting capabilities.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Set Status Enable

0 - SRQ enable (Default)

1 - Event Enable

2 - Limit Enable

3 - Parallel Pol

Control name:   Set Status Enable

Description:    Used to select which enable

register will receive the byte

value.

Variable type:  integer

Valid range:    0 = SRQ Enable

1 = Standard Event Enable

2 = Limit Event Enable

3 = Parallel Poll Enable

Default value   0 (SRQ Enable)

Value

Control name:   Value

Description:    A byte value which will be written

to the selected enable register.

Variable type:  integer

Valid range:    0 to 255

Default value:  0

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Status enable value out of range

-3   Byte value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tplp_set_volts

C Syntax           : int tplp_set_volts(Instrument_ID, Output_Number,

Voltage);

int Instrument_ID;

int Output_Number;

double Voltage;

BASIC Syntax       : Error% = tplp.set.volts(Instrument.ID%, Output.Number%, _

Voltage#)

Purpose            : Panel name:     Set Volts

Description:

Set voltage for each output.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Output Number

1 - 1 (Default)

2 - 2

3 - 3

Control name:   Output Number

Description:    Indicates the output number to

which the commands will be sent.

Variable type:  integer

Valid range:    min = 1 , max = number of outputs

available for the particular

instrument.

Default value   1

Voltage

Control name:   Voltage

Description:    The voltage value to which the

specified output will be set. The

value is expressed in Volts.

Variable type:  real

Valid range:    instrument dependent

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Output number out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out