Thurlby-Thandar TSP3220 series PSU.

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for Thurlby-Thandar

TSP3220 series PSU..

It contains functions for opening, configuring, taking measurement from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

-  the instrument is connected to the GPIB.

-  the GPIB address supplied to the initialize function must match the

GPIB address of the instrument.

-  there must be at least one unused device name in the handler.

________________________________________________________________________________

LabWindows Libraries:

In order to use this instrument driver outside of the LabWindows

environment, it must be linked with the following LabWindows libraries:

-  GPIB Library

-  Format and I/O Library

-  LabWindows System Library

National Instruments strongly recommends that you use the LWMAKE utility to

build any standalone executable program based on this instrument driver.

________________________________________________________________________________

Error Codes:

Error codes are returned as the return value of each instrument driver

function. A program should examine this value after each call to an instrument

driver function to determine if any error occurred. Possible error codes and

their meanings are listed with the corresponding instrument driver function.

________________________________________________________________________________

How To Use This Document:

This document is intended to be used as a programming reference manual.

It describes each function in the

Thurlby-Thandar TSP3220 series PSU.

instrument driver. The functions are listed in alphabetical order with

a description of the function, C and BASIC syntax of the function,

a description of each parameter, and possible error codes.

________________________________________________________________________________

Function Tree Layout:

Thurlby-Thandar TSP3220 series PSU.     Function Name:

Initialise                           tsp3220_init

Configure

Set Mode                          tsp3220_set_mode

Set Voltage                       tsp3220_set_volts

Set Current                       tsp3220_set_current

Set OVP                           tsp3220_set_ovp

Set Output Status                 tsp3220_set_op_stat

Set Damping                       tsp3220_set_damping

Master Output Control             tsp3220_master_op

Status Reporting                  tsp3220_set_status

Measurement and Status

Read Output Voltage               tsp3220_read_op_volts

Read Output Current               tsp3220_read_op_current

Read Status                       tsp3220_read_status

Local Storage

Store                             tsp3220_store

Recall                            tsp3220_recall

Miscellaneous

Send Command String               tsp3220_send_cmd

Get Response String               tsp3220_get_response

Send Block From File              tsp3220_send_from_file

Get Block to File                 tsp3220_get_to_file

Close                                tsp3220_close

________________________________________________________________________________

Instrument         : Thurlby-Thandar TSP3220 series PSU.

Thurlby-Thandar TSP3220 series power supplies

This instrument module provides GPIB support for

the TSP3220 series power supplies. Instruments in

this range meet the requirements of IEEE 488.2

standard. The module will be updated from time to

time as new instruments become available in the

range.

Instruments supported are:

TSP3222         Dual 32 volt, 2 amp, Quad mode

The module contains the following functions:

1. Initialise.

2. Configuration

1. Set Mode

2. Set Output

3. Master Output control

4. Status Reporting

3. Measurement and Status.

1. Read Output

2. Read Status

4.Local Storage.

1. Store

2. Recall

5. Close

________________________________________________________________________________

Following functions are in alphabetical order.

________________________________________________________________________________

Function Name      : tsp3220_close

C Syntax           : int tsp3220_close(Instrument_ID);

int Instrument_ID;

BASIC Syntax       : Error% = tsp3220.close(Instrument.ID%)

Purpose            : Panel name:     Close

Description:    Takes the instrument off-line.

The  instrument must  be

reinitialized to use it again.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   GPIB Address out of range

-2   ID Query out of range

-3   Reset Device out of range

221   Unable to close instrument

223   Instrument identification query failed

232   Instrument not initialized

________________________________________________________________________________

Function Name      : tsp3220_get_response

C Syntax           : int tsp3220_get_response(Instrument_ID, Response_String,

Number_of_Bytes);

int Instrument_ID;

char *Response_String;

int *Number_of_Bytes;

BASIC Syntax       : Error% = tsp3220.get.response(Instrument.ID%, _

Response.String$, Number.of.Bytes%)

Purpose            : Panel name:     Get response string

Description:    Reads a response from the tsp322x

into a string variable.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Response String

Control name:   Response Stirng

Description:    This control specifies a string

variable to receive the response

from the TSP322x. The string must

be long enough to receive the

response to avoid overflow. To aid

in checking this the number of

bytes put into the string is

returned from the function. The

absolute maximum length allowed is

2000 bytes.

Variable type:  String

Number of Bytes

Control name:   Number of Bytes

Description:    This value is the number of bytes

returned by the TSP322x and

placed in the string.

Variable type:  Integer

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Status register number out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tsp3220_get_to_file

C Syntax           : int tsp3220_get_to_file(Instrument_ID, File_name);

int Instrument_ID;

char *File_name;

BASIC Syntax       : Error% = tsp3220.get.to.file(Instrument.ID%, File.name$)

Purpose            : Panel name:     Get block to file

Description:    Gets a binary block from the

tsp322x and writes it to a disc

file.

Note            This operation is usefull for

Learn and Store blocks.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

File name

Control name:   File name

Description:    This control specifies file name

to which the binary block will be

written when read from the

TSP322x.

Variable type:  String - max length 50 characters

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

229   Error reading from instrument to file

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tsp3220_init

C Syntax           : int tsp3220_init(GPIB_Address, Set_Defaults,

Perform_Self_Test, Instrument_ID);

int GPIB_Address;

int Set_Defaults;

int Perform_Self_Test;

int *Instrument_ID;

BASIC Syntax       : Error% = tsp3220.init(GPIB.Address%, Set.Defaults%, _

Perform.Self.Test%, Instrument.ID%)

Purpose            : Panel name:     Initialise

Description:

Initialises the TSP322x in the following way

- opens the instrument and sets the GPIB

address to the specified value.

- sends device clear and clear status

commands.

- queries the instrument to determine the

type/model number.

- optionally sets the instrument to its

default state.

- optionally performs a self test on the

instrument.

Parameter List     : GPIB Address

Control name:   GPIB Address

Description:    This control specifies the

address used to communicate with

the TSP322x power supply. It must

correspond with the value set up

at the instrument front panel.

Variable type:  Integer

Valid range:    0 to 30

Default value:  11

Set Defaults

1 - Yes (Default)

0 - No

Control name:   Set Defaults

Description:    When set to Yes the parameters

in the instrument will be set to

their default values. If set to No

the instrument settings will not

be changed.

Variable type:  Integer

Valid range:    No = 0 , Yes = Any non-zero value

Default value:  Yes

Perform Self Test

1 - Yes

0 - No (Default)

Control name:   Perform Self Test

Description:    Specifies if a self test should

be performed or not. If the test

is performed the returned value

will be in the global variable

TSP3220_err. A value of zero

indicates that the test passed.

See the Error control for a list

of possible returned values.

Variable type:  Integer

Valid range:    No = 0 , Yes = Any non-zero value

Default value:  No

Instrument ID

Control Name:  Instrument ID

Description:   Returns an Instrument ID that is

used in all subsequent function

calls to select the instrument at

the Address specified.

If more than one instrument of the

same type is used, this value will

be used to differentiate between

them. The first instrument

initialized will be assigned an

Instrument ID of 1.

Variable Type: Integer

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   GPIB Address out of range

-2   ID Query out of range

-3   Reset Device out of range

220   Unable to open instrument

223   Instrument identification query failed

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

233   Error configuring GPIB address

236   Error interpreting instrument response

239   Error configuring time out

240   Instrument timed out

________________________________________________________________________________

Function Name      : tsp3220_master_op

C Syntax           : int tsp3220_master_op(Instrument_ID, Master_Output);

int Instrument_ID;

int Master_Output;

BASIC Syntax       : Error% = tsp3220.master.op(Instrument.ID%, _

Master.Output%)

Purpose            : Panel name:     Master Output Control

Description:

Set master output switch on or off.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Master Output

1 - On

0 - Off (Default)

Control name:   Master Output

Description:    Used to set the master output

switch On or Off. When On The

voltage on all outputs, which

have their individual Output

Status switches set to On, will

appear at the output terminals of

the instrument.

Variable type:  integer

Valid range:    0 = Off

1 = On

default value:  Off

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Output status out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tsp3220_read_op_current

C Syntax           : int tsp3220_read_op_current(Instrument_ID,

Output_Number, Output_Current);

int Instrument_ID;

int Output_Number;

double *Output_Current;

BASIC Syntax       : Error% = tsp3220.read.op.current(Instrument.ID%, _

Output.Number%, Output.Current#)

Purpose            : Panel name:     Read Output Currant

Description:

Returns the current from the selected output.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Output Number

1 - 1 (Default)

2 - 2

Control name:   Output Number

Description:    Indicates the output number from

which the values will be read.

Variable type:  integer

Valid range:    min = 1 , max = number of outputs

available for the particular

instrument.

Default value   1

Output Current

Control Name:   Output current

Description:    Current being output by the power

supply.

Control Type:   Real

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Output number out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tsp3220_read_op_volts

C Syntax           : int tsp3220_read_op_volts(Instrument_ID, Output_Number,

Output_Voltage);

int Instrument_ID;

int Output_Number;

double *Output_Voltage;

BASIC Syntax       : Error% = tsp3220.read.op.volts(Instrument.ID%, _

Output.Number%, Output.Voltage#)

Purpose            : Panel name:     Read Output Voltage

Description:

Returns the voltage from the selected output.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Output Number

1 - 1 (Default)

2 - 2

Control name:   Output Number

Description:    Indicates the output number from

which the values will be read.

Variable type:  integer

Valid range:    min = 1 , max = number of outputs

available for the particular

instrument.

Default value   1

Output Voltage

Control Name:   Output voltage

Description:    Voltage being output from the

power supply.

Control Type:   Real.

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Output number out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tsp3220_read_status

C Syntax           : int tsp3220_read_status(Instrument_ID, Read_Status,

Value);

int Instrument_ID;

int Read_Status;

int *Value;

BASIC Syntax       : Error% = tsp3220.read.status(Instrument.ID%, _

Read.Status%, Value%)

Purpose            : Panel name:     Read Status

Description:    Returns the byte value from the

selected status register.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Read Status

0 - Status byte (Default)

1 - Std Event

2 - Limit Event

3 - SRQ enable

4 - Event Enable

5 - Limit Enable

6 - Parallel Pol

Control name:   Read Status

Description:    Used to select which status

register to read

Variable type:  integer

Valid range:    min = 0 , max = 6

Default value   0 (Status Byte)

Value

Control Name:   Value

Description:    Retuened byte value of the

selected register.

Control Type:   integer

Valid range:    0 to 255 - register value

-1 - an error was encountered.

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Status register number out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tsp3220_recall

C Syntax           : int tsp3220_recall(Instrument_ID, Store_number);

int Instrument_ID;

int Store_number;

BASIC Syntax       : Error% = tsp3220.recall(Instrument.ID%, Store.number%)

Purpose            : Panel name:     Recall

Description:    Recalls an instrument set up from

a previous store operation.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Store number

Control name:   Store number

Description:    Select the number of the store

from which to recall the

instrument set up.

Variable type:  integer

Valid range:    1 to 25

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Set status enable out of range

-3   Byte value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tsp3220_send_cmd

C Syntax           : int tsp3220_send_cmd(Instrument_ID, Command_String);

int Instrument_ID;

char *Command_String;

BASIC Syntax       : Error% = tsp3220.send.cmd(Instrument.ID%, _

Command.String$)

Purpose            : Panel name:     Send command string

Description:    Sends a string of bytes to the

tsp322x from a string variable.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Command String

Control name:   Command String

Description:    This control specifies data to be

sent to the TSP322x. The absolute

maximum length allowed is 2000

bytes.

Variable type:  String

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Set status enable out of range

-3   Byte value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tsp3220_send_from_file

C Syntax           : int tsp3220_send_from_file(Instrument_ID, File_name);

int Instrument_ID;

char *File_name;

BASIC Syntax       : Error% = tsp3220.send.from.file(Instrument.ID%, _

File.name$)

Purpose            : Panel name:     Send block from file

Description:    Sends a binary block to the

tsp322x from a disc file.

Note            This operation is usefull for

Learn and Store blocks.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

File name

Control name:   File name

Description:    This control specifies file name

from which the binary block will

be sent to the TSP322x.

Variable type:  String - max length 50 characters

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

228   Error writing to instrument from file

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tsp3220_set_current

C Syntax           : int tsp3220_set_current(Instrument_ID, Output_Number,

Current);

int Instrument_ID;

int Output_Number;

double Current;

BASIC Syntax       : Error% = tsp3220.set.current(Instrument.ID%, _

Output.Number%, Current#)

Purpose            : Panel name:     Set Current

Description:    sets the current limit on the

selected output.

The values which are legal depend on the

instrument type and the Mode and Tracking

specification.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Output Number

1 - 1 (Default)

2 - 2

Control name:   Output Number

Description:    Indicates the output number to

which the commands will be sent.

Variable type:  integer

Valid range:    min = 1 , max = number of outputs

available for the particular

instrument.

Default value   1

Current

Control name:   Current

Description:    The current limit value to which

the specified output will be set.

The value is expressed in Amps.

Variable type:  real

Valid range:    instrument dependent

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Output number out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tsp3220_set_damping

C Syntax           : int tsp3220_set_damping(Instrument_ID, Output_Number,

Meter_Damping);

int Instrument_ID;

int Output_Number;

int Meter_Damping;

BASIC Syntax       : Error% = tsp3220.set.damping(Instrument.ID%, _

Output.Number%, Meter.Damping%)

Purpose            : Panel name:     Set Damping

Description:    Specifies the reading meter

damping for the voltage and

current of the specified output.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Output Number

1 - 1 (Default)

2 - 2

Control name:   Output Number

Description:    Indicates the output number to

which the commands will be sent.

Variable type:  integer

Valid range:    min = 1 , max = number of outputs

available for the particular

instrument.

Default value   1

Meter Damping

1 - On

0 - Off (Default)

Control name:   Meter Damping

Description:    Specifies the dampimg applied to

the readback metering of the

selected output.

Variable type:  integer

Valid range:    0 = off

1 = On

default value:  Off

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Output number out of range

-3   Danmping value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tsp3220_set_mode

C Syntax           : int tsp3220_set_mode(Instrument_ID, Mode, Tracking_A,

Tracking_B, Trip_Mode);

int Instrument_ID;

int Mode;

int Tracking_A;

int Tracking_B;

int Trip_Mode;

BASIC Syntax       : Error% = tsp3220.set.mode(Instrument.ID%, Mode%, _

Tracking.A%, Tracking.B%, Trip.Mode%)

Purpose            : Panel name:     Set Mode

Description:

Set the instrument to the required mode. Modes

available are instrument dependant. Tracking is

possible only between outputs which are

internally isolated. The number of legal modes is

instrument dependent.

TSP3222 Modes:

Mode No.            Description

------------   ------------------------------

0          Outputs 1 and 2 isolated.

Outputs 1 and 2 may be set to

tracking (Tracking A = 3).

1         Outputs 1 and 2 series connected.

Tracking not possible.

2         Outputs 1 and 2 parallel

connected.

Tracking not possible.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Mode

0 - Isolated (Default)

1 - Series

2 - Parallel

Control name:   Mode

Description:    Selects The required mode.

Variable type:  Integer

Valid range:    Instrument dependent

(maximum is 0 to 13)

Default value:  0 (all outputs isolated)

Tracking A

0 - off (Default)

3 - 1 track 2

Control name:   Tracking A

Description:    Selects one of two possible

tracking specifications.

Variable type:  Integer

Valid range:    Instrument dependent - see manual

Default value:  0 - no tracking

Tracking B

0 - off (Default)

Control name:   Tracking B

Description:    Selects one of two possible

tracking specifications. This

value should be set to zero for

all except quadruple output

instruments.

Variable type:  Integer

Valid range:    Instrument dependent - see manual

Default value:  0 - no tracking

Trip Mode

1 - Single

0 - All (Default)

Control name:   Trip Mode

Description:    Specifies the action to be taken

when any output trips for any

reason - ovp or temperature. When

set to All then the complete

instrument is shut down - the

Master On/Off switch is set to

Off. When set to Single then only

the output which tripped is shut

down - Output Status is set Off.

Variable type:  Integer

Valid range:   0 = All

1 = Single

Default value:  All

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Mode value out of range

-3   Tracking A value out of range

-4   Tracking B valur out of range

-5   Trip mode value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tsp3220_set_op_stat

C Syntax           : int tsp3220_set_op_stat(Instrument_ID, Output_Number,

Output_Status);

int Instrument_ID;

int Output_Number;

int Output_Status;

BASIC Syntax       : Error% = tsp3220.set.op.stat(Instrument.ID%, _

Output.Number%, Output.Status%)

Purpose            : Panel name:     Set output status

Description:    Set output status, specifies

whether the output status of the

specified output is on or off.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Output Number

1 - 1 (Default)

2 - 2

Control name:   Output Number

Description:    Indicates the output number to

which the commands will be sent.

Variable type:  integer

Valid range:    min = 1 , max = number of outputs

available for the particular

instrument.

Default value   1

Output Status

1 - On

0 - Off (Default)

Control name:   Output Status

Description:    Specifies the status of the

specified output.

Variable type:  integer

Valid range:    0 = Off

1 = On

default value:  Off

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Output number out of range

-3   Status value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tsp3220_set_ovp

C Syntax           : int tsp3220_set_ovp(Instrument_ID, Output_Number, OVP);

int Instrument_ID;

int Output_Number;

double OVP;

BASIC Syntax       : Error% = tsp3220.set.ovp(Instrument.ID%, Output.Number%, _

OVP#)

Purpose            : Panel name:     Set OVP

Description:    sets the OVP on the selected

output.

The values which are legal depend on the

instrument type and the Mode and Tracking

specification.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Output Number

1 - 1 (Default)

2 - 2

Control name:   Output Number

Description:    Indicates the output number to

which the commands will be sent.

Variable type:  integer

Valid range:    min = 1 , max = number of outputs

available for the particular

instrument.

Default value   1

OVP

Control name:   OVP

Description:    The over voltage protection value

to which the specified output will

be set. The value is expressed in

Volts.

Variable type:  real

Valid range:    instrument dependent

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Output number out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tsp3220_set_status

C Syntax           : int tsp3220_set_status(Instrument_ID, Set_Status_Enable,

Value);

int Instrument_ID;

int Set_Status_Enable;

int Value;

BASIC Syntax       : Error% = tsp3220.set.status(Instrument.ID%, _

Set.Status.Enable%, Value%)

Purpose            : Panel name:     Status Reporting

Description:

Specify the way in which status is reported and

Service Request is generated as defined in the

IEEE 488.2 standard.

The function provides access to the four status

enable registers in the TSP322x power supply,

these are:

Service Request Enable Register

- Specifies which bits, when set, in the

Status Byte register will generate a

Service Request.

Standard Event Enable Register

- Specifies which bits, when set, in the

Standard Event Status register will set

the ESB bit in the Status Byte Register.

Limit Event Register

- Specifies which bits, when set, in the

Limit Event Status register will set the

LIM bit in the Status Byte Register.

Parallel Poll Enable Register

- Specifies which bits, when set, in the

Status Byte register will be used to form

the ist local message

See the IEEE 488.2 Standard and the instrument

instruction book for a full description of status

reporting capabilities.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Set Status Enable

0 - SRQ enable (Default)

1 - Event Enable

2 - Limit Enable

3 - Parallel Pol

Control name:   Set Status Enable

Description:    Used to select which enable

register will receive the byte

value.

Variable type:  integer

Valid range:    min = 0 , max = 3

Default value   0 (SRQ Enable)

Value

Control name:   Value

Description:    A byte value which will be written

to the selected enable register.

Variable type:  integer

Valid range:    0 to 255

Default value:  0

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Set status enable out of range

-3   Byte value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tsp3220_set_volts

C Syntax           : int tsp3220_set_volts(Instrument_ID, Output_Number,

Voltage);

int Instrument_ID;

int Output_Number;

double Voltage;

BASIC Syntax       : Error% = tsp3220.set.volts(Instrument.ID%, _

Output.Number%, Voltage#)

Purpose            : Panel name:     Set Voltage

Description:    sets the voltage on the selected

output.

The values which are legal depend on the

instrument type and the Mode and Tracking

specification.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Output Number

1 - 1 (Default)

2 - 2

Control name:   Output Number

Description:    Indicates the output number to

which the commands will be sent.

Variable type:  integer

Valid range:    min = 1 , max = number of outputs

available for the particular

instrument.

Default value   1

Voltage

Control name:   Voltage

Description:    The voltage value to which the

specified output will be set. The

value is expressed in Volts.

Variable type:  real

Valid range:    instrument dependent

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Output number out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tsp3220_store

C Syntax           : int tsp3220_store(Instrument_ID, Store_number);

int Instrument_ID;

int Store_number;

BASIC Syntax       : Error% = tsp3220.store(Instrument.ID%, Store.number%)

Purpose            : Panel name:     Store

Description:    Saves the complete state of the

TSP322x to local non-volatile

memory.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Store number

Control name:   Store number

Description:    Select the number of the store to

receive the instrument set up.

Variable type:  integer

Valid range:    1 to 25

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Instrument ID out of range

-2   Set status enable out of range

-3   Byte value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out