Thurlby-Thandar TSXP series PSU.

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for Thurlby-Thandar TSXP

series PSU..

It contains functions for opening, configuring, taking measurement from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

For GPIB instruments drivers:

-  the instrument is connected to the GPIB.

-  the GPIB address supplied to the initialize function must match the

GPIB address of the instrument.

-  there must be at least one unused device name in the handler.

For VXI instruments drivers:

-  the instrument is installed in the VXI mainframe and you are

using one of the following controller options:

Embedded controller

AT-MXI

MC-MXI

GPIB-VXI

-  the logical address supplied to the initialize function must

match the logical address of the instrument.

-  if you are using GPIB-VXI, there must be at least one unused

device name in the handler.

For RS-232 instruments drivers:

-  the instrument is connected to the RS-232 interface.

-  the COM port, baud rate, parity, and timeout supplied to the

initialize function must match the settings of the instrument.

________________________________________________________________________________

LabWindows Libraries:

In order to use this instrument driver outside of the LabWindows

environment, it must be linked with the following LabWindows libraries:

For GPIB instruments drivers:

- 	GPIB Library

- 	Format and I/O Library

-	LabWindows System Library

For VXI instruments drivers:

- 	The appropriate VXI Library:

VXI386.LIB for embedded controllers

VXIAT.LIB for the AT-MXI

VXIMC.LIB for the MC-MXI

GPIB_C.LIB for GPIB-VXI (C users)

GPIB_QB.LIB for GPIB-VXI (BASIC users)

-	Format and I/O Library

-	LabWindows System Library

For RS-232 instruments drivers:

- 	RS-232 Library

- 	Format and I/O Library

-	LabWindows System Library

National Instruments strongly recommends that you use the LWMAKE utility to

build any standalone executable program based on this instrument driver.

________________________________________________________________________________

Error Codes:

Error codes are returned as the return value of each instrument driver

function. A program should examine this value after each call to an instrument

driver function to determine if any error occurred. Possible error codes and

their meanings are listed with the corresponding instrument driver function.

________________________________________________________________________________

How To Use This Document:

This document is intended to be used as a programming reference manual.

It describes each function in the

Thurlby-Thandar TSXP series PSU.

instrument driver. The functions are listed in alphabetical order with

a description of the function, C and BASIC syntax of the function,

a description of each parameter, and possible error codes.

________________________________________________________________________________

Function Tree Layout:

Thurlby-Thandar TSXP series PSU.     Function Name:

Initialise                        tsxp_init

Configure

Set Output                     tsxp_set_op

(set_ovp)                       tsxp_set_ovp

(set_damping)                   tsxp_set_damping

Output Switch                  tsxp_op_switch

Status Reporting               tsxp_set_status

Measurement and Status

Read Output                    tsxp_

(read_current)                  tsxp_read_current

(set_ovp)                       tsxp_set_ovp

Read Status                    tsxp_read_status

Local Storage

Store                          tsxp_store

Recall                         tsxp_recall

Miscellaneous

Send Command String            tsxp_send_cmd

Get Response String            tsxp_get_response

Send Block From File           tsxp_send_from_file

Get Block to File              tsxp_get_to_file

Close                             tsxp_close

Note:  The items in parentheses are secondary functions in the panels

under which they are listed.

________________________________________________________________________________

Instrument         : Thurlby-Thandar TSXP series PSU.

Thurlby-Thandar TSX----P series power supplies

This instrument module provides GPIB support for

the TSX----P series power supplies. Instruments in

this range meet the requirements of IEEE 488.2

standard. The module will be updated from time to

time as new instruments become available in the

range.

Instruments supported are:

TSX3510P         35 volt, 10 amp

The module contains the following functions:

1. Initialise.

2. Configuration

1. Set Output

2. Master Output control

3. Status Reporting

3. Measurement and Status.

1. Read Output

2. Read Status

4. Local Storage

1. Store

2. recall

5. Miscellaneous

1. Send Command String

2. Get Response String

3. Send Block From File

4. Get Block To File

6. Close

________________________________________________________________________________

Following functions are in alphabetical order.

________________________________________________________________________________

Function Name      : tsxp_

C Syntax           : void tsxp_(Instrument_ID);

int Instrument_ID;

BASIC Syntax       : CALL tsxp.(Instrument.ID%)

Purpose            : Panel name:     Read Output

Description:

Returns the output voltage and/or current.

Both functions are secondary so that either one

or both mat be used.

The secondary function read_volts returns the

output voltage.

The secondary function read_current returns the

current being drawn from the output.

NOTE: Errors are returned in the tsxp_err variable

Variable type:  Integer.

This value is placed in the global

variable tsxp_err. The value zero indicates

complete success.

Error              Description

------------------------------------------------

0     Success

-1   instrument ID out of range

230     GPIB write error

231     No response from instrument

232     Device not initialised

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

________________________________________________________________________________

Function Name      : tsxp_close

C Syntax           : int tsxp_close(Instrument_ID);

int Instrument_ID;

BASIC Syntax       : Error% = tsxp.close(Instrument.ID%)

Purpose            : Panel name:     Close

Description:    Takes the instrument off-line.

The  instrument must  be

reinitialized to use it again.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Return Value       :                    Error

This returned value is placed in the global

variable tsxp_err. The value zero indicates

complete success.

Error              Description

------------------------------------------------

0     Success

-1     instrument ID out of range

221     Unable to close device

230     GPIB write error

232     Device not initialised

Variable type:  Integer.

________________________________________________________________________________

Function Name      : tsxp_get_response

C Syntax           : int tsxp_get_response(Instrument_ID, Response_String,

Number_of_Bytes);

int Instrument_ID;

char *Response_String;

int *Number_of_Bytes;

BASIC Syntax       : Error% = tsxp.get.response(Instrument.ID%, _

Response.String$, Number.of.Bytes%)

Purpose            : Panel name:     Get response string

Description:    Reads a response from the TSX----P

into a string variable.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Response String

Control name:   Response Stirng

Description:    This control specifies a string

variable to receive the response

from the TSX----P. The string must

be long enough to receive the

response to avoid overflow. To aid

in checking this the number of

bytes put into the string is

returned from the function. The

absolute maximum length allowed is

2000 bytes.

Variable type:  String

Number of Bytes

Control name:   Number of Bytes

Description:    This value is the number of bytes

returned by the TSX----P and

placed in the string.

Variable type:  Integer

Return Value       :                    Error

Variable type:  Integer.

This returned value is placed in the global

variable tsxp_err. The value zero indicates

complete success.

Error              Description

------------------------------------------------

0     Success

-1     instrument ID out of range

231     No response from instrument

232     Device not initialised

________________________________________________________________________________

Function Name      : tsxp_get_to_file

C Syntax           : int tsxp_get_to_file(Instrument_ID, File_name);

int Instrument_ID;

char *File_name;

BASIC Syntax       : Error% = tsxp.get.to.file(Instrument.ID%, File.name$)

Purpose            : Panel name:     Get block to file

Description:    Gets a binary block from the

TSX----P and writes it to a disc

file.

Note            This operation is usefull for

Learn and Store blocks.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

File name

Control name:   File name

Description:    This control specifies file name

to which the binary block will be

written when read from the

TSX----P.

Variable type:  String - max length 50 characters

Return Value       :                    Error

Variable type:  Integer.

This returned value is placed in the global

variable tsxp_err. The value zero indicates

complete success.

Error              Description

------------------------------------------------

0     Success

-1     instrument ID out of range

300     Refer to ibsta and iberr as specified

for the GPIB library function ibrdf

________________________________________________________________________________

Function Name      : tsxp_init

C Syntax           : int tsxp_init(GPIB_Address, Instrument_ID);

int GPIB_Address;

int *Instrument_ID;

BASIC Syntax       : Error% = tsxp.init(GPIB.Address%, Instrument.ID%)

Purpose            : Panel name:     Initialise

Description:

Initialises the TSX----P in the following way

- opens the instrument and sets the GPIB

address to the specified value.

- sends device clear and clear status

commands.

- queries the instrument to determine the

type/model number.

Parameter List     : GPIB Address

Control name:   GPIB Address

Description:    This control specifies the

address used to communicate with

the TSX----P power supply. It must

correspond with the value set up

at the instrument front panel.

Variable type:  Integer

Valid range:    0 to 30

Default value:  11

Instrument ID

Control Name:  Instrument ID

Description:   Returns an Instrument ID that is

used in all subsequent function

calls to select the instrument at

the Address specified.

If more than one instrument of the

same type is used, this value will

be used to differentiate between

them. The first instrument

initialized will be assigned an

Instrument ID of 1.

Variable Type: Integer

Return Value       : Control nams:   Error

Variable Type: Integer

Description:

This returned value is placed in the global

variable tsxp_err. The value zero indicates

complete success.

Error              Description

------------------------------------------------

0     Success

-1     GPIB Address out of range

220     Unable to open instrument

223     Device at specified address is not

a TSX----P

230     GPIB write error

231     No response from instrument

233     Unable to configure GPIB address

300     Unable to configure board/device

when attempting to set timeout to

30sec or end message to on.

________________________________________________________________________________

Function Name      : tsxp_op_switch

C Syntax           : int tsxp_op_switch(Instrument_ID, Output_Switch);

int Instrument_ID;

int Output_Switch;

BASIC Syntax       : Error% = tsxp.op.switch(Instrument.ID%, Output.Switch%)

Purpose            : Panel name:     Output Switch

Description:

Set the output switch on or off.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Output Switch

1 - On

0 - Off (Default)

Control name:   Output Switch

Description:    Used to set the output

switch On or Off.

Variable type:  integer

Valid range:    Off = 0 , On = any non-zero value

default value:  Off

Return Value       : Control name:   Error

Variable type:  Integer.

Description:

This returned value is placed in the global

variable tsxp_err. The value zero indicates

complete success.

Error              Description

------------------------------------------------

0     Success

-1   instrument ID out of range

-2   Output Switch value out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tsxp_read_current

C Syntax           : void tsxp_read_current(Instrument_ID, Output_Current);

int Instrument_ID;

double *Output_Current;

BASIC Syntax       : CALL tsxp.read.current(Instrument.ID%, Output.Current#)

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Output Current

Control Name:   Output current

Description:    Current being output by the power

supply.

Control Type:   Real

________________________________________________________________________________

Function Name      : tsxp_read_status

C Syntax           : int tsxp_read_status(Instrument_ID, Read_Status, Value);

int Instrument_ID;

int Read_Status;

int *Value;

BASIC Syntax       : Error% = tsxp.read.status(Instrument.ID%, Read.Status%, _

Value%)

Purpose            : Panel name:     Read Status

Description:    Returns the byte value from the

selected status register.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Read Status

0 - Status byte (Default)

1 - Std Event

2 - Limit Event

3 - SRQ enable

4 - Event Enable

5 - Limit Enable

6 - Parallel Pol

Control name:   Read Status

Description:    Used to select which status

register to read

Variable type:  integer

Valid range:    min = 0 , max = 6

Default value   0 (Status Byte)

Value

Control Name:   Value

Description:    Retuened byte value of the

selected register.

Control Type:   integer

Valid range:    0 to 255 - register value

-1 - an error was encountered.

Return Value       :                    Error

Variable type:  Integer.

This returned value is placed in the global

variable tsxp_err. The value zero indicates

complete success.

Error              Description

------------------------------------------------

0     Success

-1     instrument ID out of range

-2     Status register identifier out of

range

230     GPIB write error

231     No response from instrument

232     Device not initialised

________________________________________________________________________________

Function Name      : tsxp_recall

C Syntax           : int tsxp_recall(Instrument_ID, Store_number);

int Instrument_ID;

int Store_number;

BASIC Syntax       : Error% = tsxp.recall(Instrument.ID%, Store.number%)

Purpose            : Panel name:     Recall

Description:    Recalls an instrument set up from

a previous store operation.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Store number

Control name:   Store number

Description:    Select the number of the store

from which to recall the

instrument set up.

Variable type:  integer

Valid range:    1 to 25

Return Value       :                    Error

Variable type:  Integer.

This returned value is placed in the global

variable tsxp_err. The value zero indicates

complete success.

Error              Description

------------------------------------------------

0     Success

-1     instrument ID out of range

-2     Store number out of range

230     GPIB write error

________________________________________________________________________________

Function Name      : tsxp_send_cmd

C Syntax           : int tsxp_send_cmd(Instrument_ID, Command_String);

int Instrument_ID;

char *Command_String;

BASIC Syntax       : Error% = tsxp.send.cmd(Instrument.ID%, Command.String$)

Purpose            : Panel name:     Send command string

Description:    Sends a string of bytes to the

TSX----P from a string variable.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Command String

Control name:   Command String

Description:    This control specifies data to be

sent to the TSX----P. The absolute

maximum length allowed is 2000

bytes.

Variable type:  String

Return Value       :                    Error

Variable type:  Integer.

This returned value is placed in the global

variable tsxp_err. The value zero indicates

complete success.

Error              Description

------------------------------------------------

0     Success

-1     instrument ID out of range

230     GPIB write error

232     Device not initialised

________________________________________________________________________________

Function Name      : tsxp_send_from_file

C Syntax           : int tsxp_send_from_file(Instrument_ID, File_name);

int Instrument_ID;

char *File_name;

BASIC Syntax       : Error% = tsxp.send.from.file(Instrument.ID%, File.name$)

Purpose            : Panel name:     Send block from file

Description:    Sends a binary block to the

TSX----P from a disc file.

Note            This operation is usefull for

Learn and Store blocks.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

File name

Control name:   File name

Description:    This control specifies file name

from which the binary block will

be sent to the TSX----P.

Variable type:  String - max length 50 characters

Return Value       :                    Error

Variable type:  Integer.

This returned value is placed in the global

variable tsxp_err. The value zero indicates

complete success.

Error              Description

------------------------------------------------

0     Success

-1     instrument ID out of range

300     Refer to ibsta and iberr as specified

for the GPIB library function ibwrtf

________________________________________________________________________________

Function Name      : tsxp_set_damping

C Syntax           : void tsxp_set_damping(Instrument_ID, Meter_Damping);

int Instrument_ID;

int Meter_Damping;

BASIC Syntax       : CALL tsxp.set.damping(Instrument.ID%, Meter.Damping%)

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Meter Damping

1 - On

0 - Off (Default)

Control name:   Meter Damping

Description:    Specifies the dampimg applied to

the readback current metering of

the output.

Variable type:  integer

Valid range:    On = 0 , Off = any non-zero

value

default value:  Off

________________________________________________________________________________

Function Name      : tsxp_set_op

C Syntax           : int tsxp_set_op(Instrument_ID, Voltage, Current);

int Instrument_ID;

double Voltage;

double Current;

BASIC Syntax       : Error% = tsxp.set.op(Instrument.ID%, Voltage#, Current#)

Purpose            : Panel name:     Set Output

Description:

Set voltage, current, OVP, output status and

readback meter damping. The values which are

legal depend on the instrument type.

The primary function sets the voltage and

current.

The secondary function set_ovp sets the over

voltage limit on the output.

The secondary function, set damping, specifies

the current meter damping to be on or off.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Voltage

Control name:   Voltage

Description:    The voltage value to which the

output will be set. The

value is expressed in Volts.

Variable type:  real

Valid range:    instrument dependent

Current

Control name:   Current

Description:    The current limit value to which

the output will be set.

The value is expressed in Amps.

Variable type:  real

Valid range:    instrument dependent

Return Value       : Control name:   Error

Variable Type: Integer

Description:

This returned value is placed in the global

variable tsxp_err. The value zero indicates

complete success.

Error              Description

------------------------------------------------

0   Success

-1   instrument ID out of range

230   Error writing to instrument

232   Instrument not initialized

240   Instrument timed out

________________________________________________________________________________

Function Name      : tsxp_set_ovp

C Syntax           : void tsxp_set_ovp(Instrument_ID, OVP);

int Instrument_ID;

double OVP;

BASIC Syntax       : CALL tsxp.set.ovp(Instrument.ID%, OVP#)

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

OVP

Control name:   OVP

Description:    The over voltage protection value.

The value is expressed in

Volts.

Variable type:  real

Valid range:    instrument dependent

________________________________________________________________________________

Function Name      : tsxp_set_ovp

C Syntax           : void tsxp_set_ovp(Instrument_ID, OVP);

int Instrument_ID;

double OVP;

BASIC Syntax       : CALL tsxp.set.ovp(Instrument.ID%, OVP#)

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

OVP

Control name:   OVP

Description:    The over voltage protection value.

The value is expressed in

Volts.

Variable type:  real

Valid range:    instrument dependent

________________________________________________________________________________

Function Name      : tsxp_set_status

C Syntax           : int tsxp_set_status(Instrument_ID, Set_Status_Enable,

Value);

int Instrument_ID;

int Set_Status_Enable;

int Value;

BASIC Syntax       : Error% = tsxp.set.status(Instrument.ID%, _

Set.Status.Enable%, Value%)

Purpose            : Panel name:     Status Reporting

Description:

Specify the way in which status is reported and

Service Request is generated as defined in the

IEEE 488.2 standard.

The function provides access to the four status

enable registers in the TSX----P power supply,

these are:

Service Request Enable Register

- Specifies which bits, when set, in the

Status Byte register will generate a

Service Request.

Standard Event Enable Register

- Specifies which bits, when set, in the

Standard Event Status register will set

the ESB bit in the Status Byte Register.

Limit Event Register

- Specifies which bits, when set, in the

Limit Event Status register will set the

LIM bit in the Status Byte Register.

Parallel Poll Enable Register

- Specifies which bits, when set, in the

Status Byte register will be used to form

the ist local message

See the IEEE 488.2 Standard and the instrument

instruction book for a full description of status

reporting capabilities.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Set Status Enable

0 - SRQ enable (Default)

1 - Event Enable

2 - Limit Enable

3 - Parallel Pol

Control name:   Set Status Enable

Description:    Used to select which enable

register will receive the byte

value.

Variable type:  integer

Valid range:    min = 0 , max = 3

Default value   0 (SRQ Enable)

Value

Control name:   Value

Description:    A byte value which will be written

to the selected enable register.

Variable type:  integer

Valid range:    0 to 255

Default value:  0

Return Value       : Control name:   Error

Variable type:  Integer.

Description:

This returned value is placed in the global

variable tsxp_err. The value zero indicates

complete success.

Error              Description

------------------------------------------------

0     Success

-1   instrument ID out of range

-2     Control value out of range

-3     Byte value out of range

230     GPIB write error

231     No response from instrument

232     Device not initialised

________________________________________________________________________________

Function Name      : tsxp_store

C Syntax           : int tsxp_store(Instrument_ID, Store_number);

int Instrument_ID;

int Store_number;

BASIC Syntax       : Error% = tsxp.store(Instrument.ID%, Store.number%)

Purpose            : Panel name:     Store

Description:    Saves the complete state of the

TSX----P to local non-volatile

memory.

Parameter List     : Instrument ID

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Store number

Control name:   Store number

Description:    Select the number of the store to

receive the instrument set up.

Variable type:  integer

Valid range:    1 to 25

Return Value       :                    Error

Variable type:  Integer.

This returned value is placed in the global

variable tsxp_err. The value zero indicates

complete success.

Error              Description

------------------------------------------------

0     Success

-1     instrument ID out of range

-2     Store number out of range

230     GPIB write error