INTRODUCTION

This interface board has sixteen digital input/output channels which can be used as either input or output as desired. In addition, there are eight analogue outputs with 6 bit resolution, one analogue output with 8 bit resolution, and four analogue inputs of 8 bit resolution. If more digital outputs are required, then the analogue outputs can be used by setting to minimum or maximum output voltage.  If there are too few inputs, then the analogue inputs may be used. In this way it is very simple to track the state of a rotating switch by connecting in a different voltage for every state. Please note that these extra input and output channels are not optically isolated.

The number of inputs/outputs can be further expanded by connecting more cards together, up to a maximum of four. Each card is given its own identification by means of  a two pole DIP-switch SW1 (see table below for channel numbering).

To connect the card to the computer the printer port is used (note that there is no need to install an extra printer port as the printer can be connected to the interface card). Three lines from this port are used, namely, "Select" (pin 13), "Autofeed" (pin 14) and "Select in" (pin 17). Communication between the computer and interface card is by means of a serial link. One line (Select in) serves as the clock signal, the second (Autofeed) as the data output, and the third (Select) as the data input.

All communication routines are contained in a Dynamic Link Library (DLL) K8D.DLL. This document describes all the DLL functions and procedures that are available for  your application programme. Using the DLL allows you to write custom Windows 9x,  NT or XP based applications in Visual C++, Delphi, Visual Basic, or any other 32-bit Windows application development tool that supports calls to a DLL. Thus, there is no need to be concerned about the communication protocol.

A complete overview of all procedures, functions and variables that are used by the "K8D.DLL" follows. The example program can also be carefully studied in order to gain an insight as to how to construct your customised application programmes.

Note that all examples are written for Delphi.

At the end of this document there are full declarations for the DLL function and procedures for Delphi and Visual Basic.

TABLE 'SW1' SETTINGS

CARD NUMBER	CHIP NUMBER	CHANNEL NUMBERS

0 (OFF-OFF)	IO-chip no: 0	IO-channels: 1...8

IO-chip no: 1	IO-channels: 9...16

DAC-chip no: 0	DAC-channels: 1...8

AD-chip no: 0	AD-channels: 1...4

DA-channel: 1

1 (OFF-ON)	IO-chip no: 2	IO-channels: 17...24

IO-chip no: 3	IO-channels: 25...32

DAC-chip no: 1	DAC-channels: 9...16

AD-chip no: 1	AD-channels: 5...8

DA-channel: 2

2 (0N-OFF)	IO-chip no: 4	IO-channels: 33...40

IO-chip no: 5	IO-channels: 41...48

DAC-chip no: 2	DAC-channels: 17...24

AD-chip no: 2	AD-channels: 9...12

DA-channel: 3

3 (ON-ON)	IO-chip no: 6	IO-channels: 49...56

IO-chip no: 7	IO-channels: 57...64

DAC-chip no: 3	DAC-channels: 25...32

AD-chip no: 3	AD-channels: 13...16

DA-channel: 4

OVERVIEW OF THE 'K8D.DLL'  VARIABLES

The application program has access to the the following variables via the k8d.dll:

INITIAL

VARIABLE	TYPE		VALUE         DESCRIPTION

DA		Array[1..4] of Integer		0	Contains the data (value between 0 and 255) of the four 8-bit Digital to Analogue converters

DAC		Array[1..32] of Integer		0	Contains the data (value between 0 and 63) of the thirty two 6-bit Digital to Analogue Converter channels

IOconfig		Array[0..7] of Integer		$0FF	Each bit contains the configuration of the corresponding channels of the eight IO-ports. Bit high (1) = input; bit low (0) = output

IOdata		Array[0..7] of Integer			Each bit contains the status of the respective channel of the eight IO-ports. Bit  high (1) = channel on; bit low (0) = channel off

The global constants used in the following descriptions:

CONSTANT	VALUE	DESCRIPTION

MaxIOcard	3	Highest possible address of the interface card

MaxIOchip	7	Highest possible Input/Output chip number

MaxIOchannel	64	Highest possible Input/Output channel number

MaxDACchannel	32	Highest possible 6-bit Digital  to  Analogue Converter channel number

MaxADchannel	16	Highest possible Analogue to Digital channel number

MaxDAchannel	4	Highest possible 8-bit Digital  to  Analogue channel number

OVERVIEW OF 'K8D.DLL' PROCEDURES AND FUNCTIONS

8-bit Analogue to Digital converter procedures

ReadADchannel(Channelno)		Reads the status of the analogue input-channel

8-bit Digital to Analogue conversion procedures

OutputDAchannel(Channelno,Data)	Sets the analogue output channel according to the data

ClearDAchannel(Channelno)		Sets the analogue output channel to minimum

ClearAllDA			Sets all analogue output channels to minimum

SetDAchannel(Channelno)		Sets the analogue output channel to maximum

SetAllDA			Sets all analogue output channels to maximum

6-bit Digital to Analogue  Conversion procedures

OutputDACchannel(Channelno,Data)	Sets the analogue output channel according to the data

ClearDACchannel(Channelno)		Sets the analogue output channel to minimum

ClearDACchip(Chipno)		Sets the 8 analogue output channels of the DAC-chip to minimum

ClearAllDAC			Sets all analogue output channels to minimum

SetDACchannel(Channelno)		Sets the analogue output channel  to maximum

SetDACchip(Chipno)		Sets the 8 analogue output channels of the DAC-chip to maximum

SetAllDAC			Sets all analogue output channels to maximum

IO configuration procedures

ConfigAllIOasInput		Configures all IO-channels as inputs

ConfigIOchipAsInput(Chipno)		Configures all IO-channels of the IO-chip as inputs

ConfigIOchannelAsInput(Channelno)	Configures the IO-channel as input

ConfigAllIOasOutput		Configures all IO-channels as outputs

ConfigIOchipAsOutput(Chipno)		Configures all IO-channels of the IO-chip as outputs

ConfigIOchannelAsOutput(Channel)	Configures the IO-channel as output

Setting of IOdata & IO variables (the physical status of the IO-channels does not change)

UpdateIOdataArray(Chipno,Data)	Sets the output status according to the data (inputs do not change)

ClearIOchArray(Channelno)		Clears the output status of the selected channel  (set low)

ClearIOdataArray(Chipno)		Clears the output status of the channels of the IO-chip  (set low)

SetIOchArray(Channelno)		Sets the output status of the selected channel  (set high)

SetIOdataArray(Chipno)		Sets the output status of the channels of the IO-chip  (set high)

Output procedures

IOoutput(Chipno,Data)		Sets the outputs of the IO-chip according to the data (inputs do not change)

UpdateIOchip(Chipno)		Sets the outputs of the IO-chip according to the status of the 'IOdata' variable

UpdateAllIO			Sets all outputs according to the status of the 'IOdata' variables

ClearIOchannel(Channelno)		Clears the output channel

ClearIOchip(Chipno)		Clears the output channels of the IO-chip

ClearAllIO			Clears all output channels

SetIOchannel(Channelno)		Sets the output channel

SetIOchip(Chipno)		Sets the output channels of the IO-chip

SetAllIO			Sets all output channels

Input procedures and functions

ReadIOchannel(Channelno)		Reads the status of the input channel

ReadIOchip(Chipno)		Reads the status of the input channels of the IO-chip

ReadIOconficArray(Buffer)		Reads the IO configuration data from the DLL  to the application program

ReadIOdataArray(Buffer)		Reads the IO status data from the DLL  to the application program

ReadDACarray(Buffer)		Reads the DAC data from the DLL  to the application program

ReadDAarray(Buffer)		Reads the DA data from the DLL  to the application program

General procedures

SelectI2CprinterPort(Printer_no)	Chooses the communication port

Start_K8000			Opens a link to the K8000 device

Stop_K8000			Closes the link to the K8000 device

'K8D.DLL' PROCEDURES AND FUNCTIONS

Start_K8000

Syntax

PROCEDURE Start_K8000;

Description

Initializes the communication routines for the K8000 card. Loads the drivers needed to communicate via the LPT port. This procedure must be performed in the beginning of the application program.

Example

BEGIN

Start_K8000;

END;

Sop_K8000

Syntax

PROCEDURE Stop_K8000;

Description

Unloads the communication routines for K8000 card and unloads the drivers needed to communicate via the PLT port. This is the last action of the application program before termination.

Example

BEGIN

Stop_K8000;

END;

SelectI2CprinterPort

Syntax

PROCEDURE SelectI2CprinterPort(Printer_no: Longint);

Parameter

Printer_no:	Value between 0 and 2 given by the printer port to which the interface card is connected.

0: printer port  address is 0BC (hex)

1: printer port  address is 378 (hex), usually the address of LPT1

2: printer port  address is 278 (hex), usually the address of LPT2

Result

Communication between the PC and the K8000 card will occur via the selected LPT port address.

Description

The printer port  to be used must be specified at the start of your programme in order to run the interface card so that the communication routines are carried out correctly. The default designation is LPT1,  but this can be changed using this procedure.

Example

BEGIN

SelectI2CprinterPort(1);

// LPT1 address on mainboard is set to 378

END;

ReadADchannel	AD

Syntax

FUNCTION ReadADchannel(Channel_no: Longint):Longint;

Parameter

Channel_no:	Value between 1 and 16 which corresponds to the AD channel whose status is to be read.

Result

AD:		The corresponding 'AD' data is read according to the status of the AD input.

Description

The input voltage of the selected 8-bit Analogue to Digital converter channel is converted to a value which lies between 0 and 255 and registered in the respective 'AD' data variable.

Example

var data: longint;

BEGIN

data := ReadADchannel(1);

// AD channel 1 is read to variable 'data'

END;

OutputDAchannel                                                               	DA

Syntax

PROCEDURE OutputDAchannel(Channel_no: Longint; Data: Longint);

Parameter

Channel_no:	Value between 1 and 4 which corresponds to the 8-bit DA channel number whose data is to be changed.

Data:	Value between 0 and 255 which is to be sent to the 8-bit Digital Analogue Converter .

Result

DA:		The 'DA' data variable of the selected channel is set according to the data which is to be sent.

The selected  DA-channel is changed.

Description

The indicated 8-bit Digital to Analogue Converter channel is altered according to the new data. This means that the data corresponds to a specific voltage. The value 0 corresponds to a minimum output voltage (0 Volt) and the value 255 corresponds to a maximum output voltage (Vmax)  which is set according to the preset on the interface board. A value of 'Data' lying in between these extremes can be translated by the following formula :  Data x Vmax/255.

Example

BEGIN

OutputDAchannel(1,127);

// DA channel 1 is at 1/2 Vmax

END;

ClearDAchannel                                                                          	DA

Syntax

PROCEDURE ClearDAchannel(Channel_no: Longint);

Parameter

Channel_no:	Value between 1 and 4 which corresponds to the 8-bit DA channel number in which the data is to be erased.

Result

DA:		The  'DA' data variable of the selected DA-channel is set to  minimum (0) .

The selected DA-channel is set to minimum output voltage (0 Volt).

Description

The selected 8-bit Digital to Analogue Converter channel is set to minimum output voltage (0 Volt).

Example

BEGIN

ClearDAchannel(1);

// DA channel 1 is at Vmin

END;

ClearAllDA                                                                           	DA

Syntax

PROCEDURE ClearAllDA;

Result

All DA-channels are set to minimum output voltage (0 Volt) .

Description

All DA-channels of the 8-bit Digital to Analogue Converters are set to minimum output voltage (0 Volt) .

Example

BEGIN

ClearAllDA;

// All DA channels 1...4 are at Vmin

END;

SetDAchannel                                                                       	DA

Syntax

PROCEDURE SetDAchannel(Channel_no: TDAchannel);

Parameter

Channel_no:	Value between 1 and 4 which corresponds to the 8-bit DA channel number in which the data is to be set to maximum.

Result

The selected DA-channel is set to maximum output voltage.

Description

The selected 8-bit Digital to Analogue Converter channel is set to maximum output voltage.

Example 15

PROGRAM Set_DA_channel;

USES I2C, WinCrt;

BEGIN

SetDAchannel(1);

Writeln('Set DA channel 1 at Vmax');

END;

SetAllDA                                                                                  	DA

Syntax

PROCEDURE SetAllDA;

Result

All DA-channels are set to maximum output voltage.

Description

All DA-channels of the 8-bit Digital to Analogue Converters are set to maximum output voltage.

Example

BEGIN

SetAllDA;

// All DA channels 1...4 are at Vmax

END;

OutputDACchannel                                                               	DAC

Syntax

PROCEDURE OutputDACchannel(Channel_no: Longint; Data: Longint);

Parameters

Channel_no:	Value between 1 and 32 which corresponds to the 6-bit DAC channel number in which the data is to be changed.

Data:	Value between 0 and 63 which is to be sent to the 6-bit Digital Analogue Converter .

Result

The selected  DAC-channel is updated.

Description

The indicated 6-bit Digital to Analogue Converter channel is modified with the new data. This means that the data corresponds to a specific voltage. The value 0 corresponds to a minimum output voltage (Vmin) and the value 63 corresponds to a maximum output voltage (Vmax) which can be set using the preset on the interface board. A value of 'Data' lying in between these two limits corresponds to a voltage according to the formula:

Vmin + Data x (Vmax - Vmin)/63.

Example

BEGIN

OutputDACchannel(1,21);

// DAC channel 1 is at Vmin + 1/3(Vmax-Vmin)

END;

ClearDACchannel                                                              	DAC

Syntax

PROCEDURE ClearDACchannel(Channel_no: Longint);

Parameter

Channel_no:	Value between 1 and 32 which corresponds to the 6-bit DAC channel number in which the data is to be erased.

Result

The selected DAC-channel is set to minimum output voltage (Vmin).

Description

The selected 6-bit Digital to Analogue Converter channel is set to minimum output voltage (Vmin).

Example

BEGIN

ClearDACchannel(2);

// DAC channel 2 is at Vmin

END;

ClearDACchip                                                                  	DAC

Syntax

PROCEDURE ClearDACchip(Chip_no: Longint);

Parameter

Chip_no:	Value between 0 and 3 which corresponds to the address of the 6-bit DAC chip in which the 8 channels are to be set to minimum output voltage.

Result

The 8 DAC-channels of the indicated DAC-chip are set to minimum output voltage.

Description

The eight DAC-channels of the selected 6-bit Digital to Analogue Converter chip are set to minimum (Vmin).

Example

BEGIN

ClearDACchip(0);

// DAC channels 1...8 are at Vmin

END;

ClearAllDAC                                                                      	DAC

Syntax

PROCEDURE ClearAllDAC;

Result

All DAC-channels are set to minimum output voltage.

Description

All DAC-channels of the 6-bit Digital to Analogue Converters are set to minimum output voltage (Vmin) .

Example

BEGIN

ClearAllDAC;

// All DAC channels 1...32 are at Vmin

END;

SetDACchannel                           	DAC

Syntax

PROCEDURE SetDACchannel(Channel_no: Longint);

Parameter

Channel_no:	Value between 1 and 32 which corresponds to the 6-bit DAC channel number in which the data is to be set to maximum.

Result

The selected DAC-channel is set to maximum output voltage.

Description

The selected 6-bit Digital to Analogue Converter channel is set to maximum output voltage.

Example

BEGIN

SetDACchannel(3);

// Set DAC channel 3 at Vmax

END;

SetDACchip                                                             	DAC

Syntax

PROCEDURE SetDACchip(Chip_no: Longint);

Parameter

Chip_no:	Value between 0 and 3 which corresponds to the address of the 6-bit DAC chip in which the 8 channels are to be set to maximum output voltage.

Result

The 8 DAC-channels of the indicated DAC-chip are set to maximum output voltage.

Description

The eight DAC-channels of the selected 6-bit Digital to Analogue Converter chip are set to the maximum output voltage (Vmax). The 'DAC' data variables of the respective DAC-channels are likewise adjusted.

Example

BEGIN

SetDACchip(0);

// DAC channels 1...8 are at Vmax

END;

SetAllDAC                                                             	DAC

Syntax

PROCEDURE SetAllDAC;

Result

All DAC-channels are set to maximum output voltage.

Description

All DAC-channels of the 6-bit Digital to Analogue Converters are set to maximum output voltage (Vmax).

Example

BEGIN

SetAllDAC;

// All DAC channels 1...32 are at Vmax

END;

ConfigAllIOasInput                                                           	IO

Syntax

PROCEDURE ConfigAllIOasInput;

Result

IOconfig:	The 'IOconfig' variables for all Input/Output ports are given the value 255.

Description

All digital Input/Output channels (1...64) are configured as inputs. Each IO chip (0...7) contains a variable in which the configuration of each IO pin is returned in the form of a data bit which is not necessarily high. If this bit is high (1) it means that this is an input. Writing to this IO-channel will have no effect. The status of the IO-channels can only be determined by an external signal.

Example

BEGIN

ConfigAllIOasInput;

// All IO channels are now configured as inputs

END;

ConfigIOchipAsInput                                                   	IO

Syntax

PROCEDURE ConfigIOchipAsInput(Chip_no: Longint);

Parameter

Chip_no:	Value between 0 and 7 which corresponds to the address setting of the Input/Output chip in which all channels are configured as inputs.

Result

IOconfig:	The 'IOconfig variable of the respective Input/Output chip is given the value 255.

Description

The  8 digital Input/Output channels of the selected IO chip are configured as inputs. Each IO chip (0...7)  contains a variable in which the configuration of each IO pin is returned in the form of a data bit which is not necessarily high. If this bit is high (1) it means that it is an input. The status of the IO-channels can only be determined by an external signal.

Example

BEGIN

ConfigIOchipAsInput(0);

// The 8 channels from  IO Chip 0 are now configured as inputs

END;

ConfigIOchannelAsInput                                              	IO

Syntax

PROCEDURE ConfigIOchannelAsInput(Channel_no: Longint);

Parameter

Channel_no:	Value between 1 and 64 which corresponds to the Input/Output channel that is to be configured as an input.

Result

IOconfig:	Via the channel number, the IO-chip number and bit number are determined in order to set this bit high  (1) in the 'IOconfig' variable.

Description

The selected Input/Output channel is configured as an input while the configuration of the other channels remain unchanged. This occurs by making the correct bit high (1) in the configuration variable of the respective IO chip. The status of the IO-channel can only be determined by an external signal that is fed to it.

Example

BEGIN

ConfigIOchannelAsInput(1);

// IO channel 1 is now configured as input');

END;

ConfigAllIOasOutput                                  	IO

Syntax

PROCEDURE ConfigAllIOasOutput;

Result

IOconfig:	The 'IOconfig' variables for all Input/Output ports are set low (0).

Description

All digital Input/Output channels (1...64) are configured as outputs. Each IO chip (0...7) contains a variable in which the configuration of each IO pin is returned in the form of a data bit which is not necessarily high. If this bit is low (0) it means an output. The status of the IO-channels are determined by the value written to them.

Example

BEGIN

ConfigAllIOasOutput;

// All IO channels are now configured as outputs

END;

ConfigIOchipAsOutput                                                               	IO

Syntax

PROCEDURE ConfigIOchipAsOutput(Chip_no: Longint);

Parameter

Chip_no:	Value between 0 and 7 which corresponds to the address setting of the Input/Output chip in which all channels are to be configured as outputs.

Result

IOconfig:	The 'IOconfig variable of the respective Input/Output chip is set low (0).

Description

The  8 digital Input/Output channels of the selected IO chip are configured as outputs. Each IO chip (0...7) contains a variable in which the configuration of each IO pin is returned in the form of a data bit which is not necessarily high. If this bit is low (0) then it means an output. The status of these IO-channels are determined by the value written to them.

Example

BEGIN

ConfigIOchipAsOutput(1);

// The 8 channels from IO Chip 1 are now configured as outputs

END;

ConfigIOchannelAsOutput                                                        	IO

Syntax

PROCEDURE ConfigIOchannelAsOutput(Channel_no: Longint);

Parameter

Channel_no:	Value between 1 and 64 which corresponds to the Input/Output channel that is to be configured as an output .

Result

IOconfig:	Via the channel number, the IO-chip number and bit number are determined in order to set the 'IOconfig' variable low (0).

Description

The selected Input/Output channel is configured as an output while the configuration of the other channels remain unchanged. This occurs by making the correct bit  low (0) in the  configuration variable of the respective IO chip. The status of this IO-channel is determined by the value written to it.

Example

BEGIN

ConfigIOchannelAsOutput(2);

// IO channel 2 is now configured as output

END;

UpdateIOdataArray                                                                    	IO

Syntax

PROCEDURE UpdateIOdataArray(Chip_no: Longint; Data: Longint);

Parameters

Chip_no:	Value between 0 and 7 which corresponds to the address setting of the Input/Output chip in which the data is to be changed.

Data:	Value between 0 and 255 which is to be sent to the IO port (8 channels).

Result

IOdata:	The 'IOdata' variable of the selected chip is updated with the new data such that the status of the input channels remains unchanged.

Description

The status of the Input/Output ports are held in the 'IO data' variable. Each bit of this variable corresponds to the status of an IO-channel. If the status of the 8 channels of an IO-chip is to be changed then it is not advisable to immediately set the data in the 'IOdata' variable, because the status of the input channels would no longer correspond to the physical status. In order to prevent such conflict this procedure must be used.

Note that the 'IOdata' variable is just a buffer memory. When its value is changed the outputs do not  immediately change status. Status only changes when its value is sent to the IO-chip.

Example

BEGIN

ConfigIOchipAsOutput(0);

UpdateIOdataArray(0,204);

// 204 is in binary format 11001100

// this sets channels 3,4,7,8  and clears channels 1,2,5,6

END;

ClearIOchArray                                                             	IO

Syntax

PROCEDURE ClearIOchArray(Channel_no: Longint);

Parameter

Channel_no:	Value between 1 and 64 which corresponds to the Input/Output channel that is to be cleared.

Result

IOdata:	If selected channel is an output then the correct bit in the 'IOdata' variable of the corresponding chip is set low (0).

Description

The status of the Input/Output ports is held in the 'IO data' variable. Each bit of this variable corresponds to the status of an IO-channel. If a particular output of the IO-chip is to be cleared then it is not advisable to immediately set the corresponding bit in the 'IOdata' variable to low. If the selected channel has been configured as an input then its status would no longer correspond to the physical status. In order to avoid such conflict this procedure must be used.

Note that the 'IOdata' variable is just a buffer memory. When its value is changed the outputs do not immediately change status. This only occurs when this value is sent to the IO-chip.

Example

BEGIN

ConfigIOchipAsOutput(0);

ConfigIOchannelAsInput(1);     {channel 1 configured as input}

ConfigIOchannelAsInput(2);     {channel 2 configured as input}

ConfigIOchannelAsInput(3);     {channel 3 configured as input}

ConfigIOchannelAsInput(4);     {channel 4 configured as input}

ClearIOchArray(2); // Clear channel 2 from IOdata array

// The status of the inputs stays unchanged

ClearIOchArray(8); // Clear channel 8 from IOdata array

END;

ClearIOdataArray                                                                        IO

Syntax

PROCEDURE ClearIOdataArray(Chip_no: Longint);

Parameter

Chip_no:	Value between 0 and 7 which corresponds to the address setting of the Input/Output chip in which the outputs are to be cleared.

Result

IOdata:	The bits of the 'IOdata' variable of the selected chip are set low (0) but the bits that correspond to inputs remain unchanged.

Description

The status of the Input/Output ports are held in the 'IOdata' variable. Each bit of this variable corresponds to the status of an IO-channel. If the outputs of the IO-chip are to be cleared then it is not recommended to immediately set the 'IOdata' variable to 0, because the status of the input channels would no longer correspond to the physical status. In order to avoid such conflict  this procedure must be used.

Note that the 'IOdata' variable is just a buffer memory. When this changes its value  the outputs do not immediately change status. This occurs only when this value is sent to the IO-chip.

Example

BEGIN

ConfigIOchipAsOutput(0);

ConfigIOchannelAsInput(1);     {channel 1 configured as input}

ConfigIOchannelAsInput(2);     {channel 2 configured as input}

ConfigIOchannelAsInput(3);     {channel 3 configured as input}

ConfigIOchannelAsInput(4);     {channel 4 configured as input}

ClearIOdataArray(0); // Clear channels 1 to 8 from IOdata array

// The status of the inputs stays unchanged

END;

SetIOchArray                                                         	IO

Syntax

PROCEDURE SetIOchArray(Channel_no: Longint);

Parameter

Channel_no:	Value between 1 and 64 which corresponds to the Input/Output channel that is to be set.

Result

IOdata:	If the selected channel is an output then the correct bit in the 'IOdata' variable of the corresponding chip is set high (1).

Description

The status of the Input/Output ports is held in the 'IOdata' variable. Each bit of this variable corresponds to the status of an IO-channel. If a particular output of the IO-chip is to be set then it is not advisable to immediately set the corresponding bit in the 'IOdata' variable to high. If the selected channel has been configured as an input then the status would no longer correspond to the physical status. In order to prevent such conflict this procedure must be used.

Note that the 'IOdata' variable is just a buffer memory. When it changes its value the outputs do not immediately change status. This only occurs when this value is sent to the IO-chip.

Example

BEGIN

ConfigIOchipAsOutput(0);

ConfigIOchannelAsInput(1);     {channel 1 configured as input}

ConfigIOchannelAsInput(2);     {channel 2 configured as input}

ConfigIOchannelAsInput(3);     {channel 3 configured as input}

ConfigIOchannelAsInput(4);     {channel 4 configured as input}

SetIOchArray(1); // Set channels 1 from IOdata array

SetIOchArray(5); // Set channels 5 from IOdata array

// The status of the input channel 1 stays unchanged

END;

SetIOdataArray                                                     	IO

Syntax

PROCEDURE SetIOdataArray(Chip_no: Longint);

Parameter

Chip_no:	Value between 0 and 7 that corresponds to the address setting of the Input/Output chip in which the outputs are to be set.

Result

IOdata:	The bits of the 'IOdata' variable of the selected chip are set high (1) but the bits that correspond to inputs remain unchanged.

Description

The status of the Input/Output ports are held in the 'IOdata' variable. Each bit of this variable corresponds to the status of an IO-channel. If the outputs of the IO-chip are to be set then it is not advisable to immediately to load 255 in the 'IOdata' variable, because the status of the input channels would no longer correspond to the physical status. In order to avoid such conflict this procedure must be used.

Note that the 'IOdata' variable is just a buffer memory. When it changes its value the outputs do not immediately change status. This only happens when this value is sent to the IO-chip.

The 'IO' variables of the respective IO-chip are likewise adjusted.

Example

BEGIN

ConfigIOchipAsOutput(0);

ConfigIOchannelAsInput(1);     {channel 1 configured as input}

ConfigIOchannelAsInput(2);     {channel 2 configured as input}

ConfigIOchannelAsInput(3);     {channel 3 configured as input}

ConfigIOchannelAsInput(4);     {channel 4 configured as input}

SetIOdataArray(0); // Set channels 1 to 8 from IOdata array

// The status of the inputs stays unchanged

END;

IOoutput                                                       	IO

Syntax

PROCEDURE IOoutput(Chip_no: Longint; Data: Longint);

Parameters

Chip_no:	Value between 0 and 7 that corresponds to the address setting of the Input/Output chip in which the data is to be changed.

Data:	Value between 0 and 255 that is sent to the IO port (8 channels).

Result

IOdata:	The 'IOdata' variable of the selected chip is updated with the new data such that the status of the input channels remains unchanged.

The outputs of the selected IO-chip are updated.

Description

The channels of the selected IO-chip which have been configured as outputs are updated with the status of the corresponding bits in the data parameter. A high (1) level means that the  output is set, and a low (0) level means that the output is cleared. The status of the inputs remains unchanged.

The 'IOdata' and 'IO' variables of the respective IO-chip are likewise updated.

Example

BEGIN

ConfigIOchipAsOutput(1);

IOoutput(1,128);

// Output channels 9..15 are off, Output channel 16 is on

END;

UpdateIOchip                                                        	IO

Syntax

PROCEDURE UpdateIOchip(Chip_no: Longint);

Parameter

Chip_no:	Value between 0 and 7 that corresponds to the address setting of the Input/Output chip in which the outputs are to be changed.

Result

All outputs of the selected Input/Output chip are changed according to the status of the corresponding bits in the 'IOdata'  variables.

Description

All channels of the selected IO-chip that have been configured as outputs are changed according to the status of the corresponding bits in the 'IOdata' array. A high (1) level means that the output is set, a low (0) level means that the output is cleared. The status of the inputs remain unchanged.

Example

BEGIN

ConfigIOchipAsOutput(1);

UpdateIOdataArray(1,64); // channels 9...14 & 16 off, 15 on

UpdateIOchip(1);

// Output channels 9..14 & 16 are off, Output channel 15 is on

END;

UpdateAllIO                                                      	IO

Syntax

PROCEDURE UpdateAllIO;

Result

All outputs are changed according to the status of the corresponding bits in the 'IOdata'  variables.

Description

All channels of the IO-chips which have been configured as outputs are changed according to the status of the corresponding bits in the 'IOdata' array. A high  (1) level means that the output is set, a low (0) level means that the output is cleared. The status of the inputs remain unchanged.

Example

BEGIN

ConfigAllIOasOutput;

UpdateIOdataArray(0,1);      // channel  1 on

UpdateIOdataArray(1,128);    // channel 16 on

UpdateAllIO;

// Output channel 1 & 16 are on, Output channels 2..15 are off

END;

ClearIOchannel                                            	IO

Syntax

PROCEDURE ClearIOchannel(Channel_no: Longint);

Parameter

Channel_no:	Value between 1 and 64 which corresponds to the Input/Output channel that is to be cleared.

Result

IOdata:	If the selected channel is an output then the correct bit in the 'IOdata' variable of the corresponding chip is set low (0).

Description

If the selected channel has been configured as an output then it is cleared. The status of the inputs remain unchanged. The 'IOdata' and 'IO' variables are adjusted according to the new status.

Example

BEGIN

ConfigIOchannelAsOutput(9);

ClearIOchannel(9); // Output channel 9 is off

END;

ClearIOchip                                                                             	IO

Syntax

PROCEDURE ClearIOchip(Chip_no: Longint);

Parameter

Chip_no:	Value between 0 and 7 that corresponds to the address setting of the Input/Output chip in which the outputs are to be cleared.

Result

IOdata:	The bits in the 'IOdata' variable of the selected IO-chip that correspond to outputs are set low (0) , those of the inputs remain unchanged.

Description

All channels of the selected Input/Output chip which have been configured as outputs are cleared. The status of the inputs remain unchanged. The 'IOdata' and 'IO' variables are changed according to the new status.

Example

BEGIN

ConfigIOchipAsOutput(1);

ClearIOChip(1); // Output channels 9...16 are off

END;

ClearAllIO                                                                 	IO

Syntax

PROCEDURE ClearAllIO;

Result

IOdata:	The bits in the 'IOdata' variables that correspond to outputs are set low (0), those of the inputs remain unchanged.

All outputs are cleared.

Description

All channels of the IO-chips that have been configured  as outputs are cleared. The status of the inputs remain unchanged. The 'IOdata' is changed according to the new status.

Example

BEGIN

ConfigAllIOasOutput;

ConfigIOchannelAsInput(2);   // channel 2 configured as input

ClearAllIO;

// All Output channels are off

// Input channel 2 unchanged

END;

SetIOchannel                                                    	IO

Syntax

PROCEDURE SetIOchannel(Channel_no: Longint);

Parameter

Channel_no:	Value between 1 and 64 which corresponds to the Input/Output channel that is to be set.

Result

IOdata:	If the selected channel is an output then the the correct bit in the 'IOdata' variable of the corresponding chip is set high (1).

The selected output channel is set.

Description

If the selected channel has been configured as an output then it is set. The status of the inputs remain unchanged. The 'IOdata'  is changed according to the new status.

Example

BEGIN

ConfigIOchannelAsOutput(9);

SetIOchannel(9); // Output channel 9 is on

END;

SetIOchip                                                      	IO

Syntax

PROCEDURE SetIOchip(Chip_no: Longint);

Parameter

Chip_no:	Value between 0 and 7 which corresponds to the address setting of the Input/Output chip in which the outputs are to be set.

Result

IOdata:	The bits in the 'IOdata' variable of the selected IO-chip that correspond to outputs are set high (1), those of the inputs remain unchanged.

Description

All channels of the selected Input/Output chip that have been configured as outputs are set. The status of the inputs remain unchanged.

Example

BEGIN

ConfigIOchipAsOutput(1);

SetIOchip(1); // Output channels 9...16 are on

END;

SetAllIO                                           	IO

Syntax

PROCEDURE SetAllIO;

Result

IOdata:	The bits in the 'IOdata' variables that correspond to outputs are set high (1), those of the inputs remain unchanged.

IO:		The 'IO' variables that correspond to outputs are set high (True). Those of the input channels remain unchanged.

All outputs are set.

Description

All channels of the IO-chips that have been configured as outputs are set. The status of the inputs remain unchanged. The 'IOdata' variables are changed according to the new status.

Example

BEGIN

ConfigAllIOasOutput;

ConfigIOchannelAsInput(2);  // channel 2 configured as input

SetAllIO;

// All Output channels are on

// Input channel 2 unchanged

END;

ReadIOchannel                                                	IO

Syntax

FUNCTION ReadIOchannel(Channel_no: Longint): Boolean;

Parameter

Channel_no:	Value between 1 and 64 which corresponds to the Input/Output channel whose status is to be read.

Result

IOdata:	The bit in the 'IOdata' variable of the selected channel is changed according to the status of that channel. A high (1) means that the channel has been set, a low (0) means that it has been cleared.

Description

The status of the selected Input/Output channel is read and registered in the 'IOdata' variable. The function returns the status of the channel.

Example

var status: boolean;

BEGIN

ConfigIOChannelAsInput(2);

status := ReadIOchannel(2); // Read Input channel 2

END;

ReadIOchip                                                 	IO

Syntax

FUNCTION ReadIOchip(Chip_no: Longint): Longint;

Parameter

Chip_no:	Value between 0 and 7 which corresponds to the address setting of the Input/Output chip in which the status of the inputs are to be read.

Result

IOdata:	The bits in the 'IOdata' variable of the selected IO-chip are changed according to the status of the IO-channels. A high (1) means that the channel has been cleared, a low (0) means that it has been cleared.

Description

The status of all channels of the selected Input/Output chip are read and registered in the 'IOdata'  variables. The function returns the status of the chip inputs.

Example

var status: longint;

BEGIN

ConfigIOchipAsInput(0);

status := ReadIOchip(0); // Read Input channels from chip 0

END;

ReadIOconfigArray                   	IO

Syntax

PROCEDURE ReadIOconfigArray(Buffer:Pointer);

Parameter

Buffer:	 A pointer to the array of long integers where the IO configuration data will be read.

Description

The IO configuration data array is read from the K8D.DLL to the application program.

Example

var  // global variables

IOconfig: ARRAY[0..MaxIOchip] OF Integer;

procedure TForm1.Button1Click(Sender: TObject);

var p:pointer;

i:integer;

s:string;

begin

p:=@IOconfig;    // Address of the data buffer for config array

ReadIOconficArray(p); // Read the data from K8D.DLL

memo1.clear;

s:='';

for i:=0 to MaxIOchip do s:=s +inttostr(IOconfig[i])+chr(9);

memo1.lines.add(s); // Display the IO cofig data

end;

ReadIOconfigArray                            	IO

Syntax

PROCEDURE ReadIOdataArray(Buffer:Pointer);

Parameter

Buffer:	 A pointer to the array of long integers where the IO data will be read.

Description

The IO status data array is read from the K8D.DLL to the application program.

Example

var  // global variables

IOdata: ARRAY[0..MaxIOchip] OF Integer;

procedure TForm1.Button1Click(Sender: TObject);

var p:pointer;

i:integer;

s:string;

begin

p:=@IOdata;      // Address of the data buffer for the data array

ReadIOdataArray(p); // Read the data from K8D.DLL

memo1.clear;

s:='';

for i:=0 to MaxIOchip do s:=s +inttostr(IOdata[i] and $ff)+chr(9);

memo1.lines.add(s); // Display the IO data

end;

ReadDACarray                                                      	DAC

Syntax

PROCEDURE ReadDACarray(Buffer:Pointer);

Parameter

Buffer:	 A pointer to the data array of long integers where the DAC data will be read.

Description

The DAC data array is read from the K8D.DLL to the application program.

Example

var  // global variables

DAC: ARRAY[1..MaxDACchannel] OF Integer;

procedure TForm1.Button1Click(Sender: TObject);

var p:pointer;

i:integer;

s:string;

begin

p:=@DAC;         // Address of the data buffer for the DAC array

ReadDACarray(p); // Read the data from K8D.DLL

memo1.clear;

s:='';

for i:=1 to MaxDACchannel do s:=s +inttostr(DAC[i] and $ff)+chr(9);

memo1.lines.add(s); // Display the DAC dataend;

ReadIOconfigArray                                                      	DA

Syntax

PROCEDURE ReadDAarray(Buffer:Pointer);

Parameter

Buffer:	 A pointer to the data array of long integers where the data will be read.

Description

The DA data array is read from the K8D.DLL to the application program.

Example

var  // global variables

DA: ARRAY[1..MaxDAchannel] OF Integer;

procedure TForm1.Button1Click(Sender: TObject);

var p:pointer;

i:integer;

s:string;

begin

p:=@DA;          // Address of the data buffer for the DA array

ReadDAarray(p);  // Read the data from K8D.DLL

memo1.clear;

s:='';

for i:=1 to MaxDAchannel do s:=s +inttostr(DA[i] and $ff)+chr(9);

memo1.lines.add(s); // Display the DA data

end;

Using the DLL in Delphi

Here is the declaration of some array variables, K8D.DLL procedures and functions and the two most important procedures of the application program (FormCreate and FormClose).

var

IOconfig: ARRAY[0..MaxIOchip] OF Integer;

IOdata: ARRAY[0..MaxIOchip] OF Integer;

DAC: ARRAY[1..MaxDACchannel] OF Integer;

DA: ARRAY[1..MaxDAchannel] OF Integer;

implementation

{$R *.DFM}

{IO CONFIGURATION PROCEDURES}

PROCEDURE ConfigAllIOasInput; stdcall; external 'K8D.dll';

PROCEDURE ConfigAllIOasOutput; stdcall; external 'K8D.dll';

PROCEDURE ConfigIOchipAsInput(Chip_no: TIOchip); stdcall; external 'K8D.dll';

PROCEDURE ConfigIOchipAsOutput(Chip_no: TIOchip); stdcall; external 'K8D.dll';

PROCEDURE ConfigIOchannelAsInput(Channel_no: TIOchannel); stdcall; external 'K8D.dll';

PROCEDURE ConfigIOchannelAsOutput(Channel_no: TIOchannel); stdcall; external 'K8D.dll';

{UPDATE IOdata & IO ARRAY PROCEDURES}

PROCEDURE UpdateIOdataArray(Chip_no: TIOchip; Data:Longint); stdcall; external 'K8D.dll';

PROCEDURE ClearIOdataArray(Chip_no: TIOchip); stdcall; external 'K8D.dll';

PROCEDURE SetIOdataArray(Chip_no: TIOchip); stdcall; external 'K8D.dll';

PROCEDURE SetIOchArray(Channel_no: TIOchannel); stdcall; external 'K8D.dll';

PROCEDURE ClearIOchArray(Channel_no: TIOchannel); stdcall; external 'K8D.dll';

{OUTPUT PROCEDURES}

PROCEDURE IOoutput(Chip_no: TIOchip ; Data: Longint); stdcall; external 'K8D.dll';

PROCEDURE UpdateAllIO; stdcall; external 'K8D.dll';

PROCEDURE ClearAllIO; stdcall; external 'K8D.dll';

PROCEDURE SetAllIO; stdcall; external 'K8D.dll';

PROCEDURE UpdateIOchip(Chip_no: TIOchip); stdcall; external 'K8D.dll';

PROCEDURE ClearIOchip(Chip_no: TIOchip); stdcall; external 'K8D.dll';

PROCEDURE SetIOchip(Chip_no: TIOchip); stdcall; external 'K8D.dll';

PROCEDURE SetIOchannel(Channel_no: TIOchannel); stdcall; external 'K8D.dll';

PROCEDURE ClearIOchannel(Channel_no: TIOchannel); stdcall; external 'K8D.dll';

{6 BIT DAC CONVERTER PROCEDURES}

PROCEDURE OutputDACchannel(Channel_no: TDACchannel ; Data: Longint); stdcall; external 'K8D.dll';

PROCEDURE ClearDACchannel(Channel_no: TDACchannel); stdcall; external 'K8D.dll';

PROCEDURE SetDACchannel(Channel_no: TDACchannel); stdcall; external 'K8D.dll';

PROCEDURE ClearDACchip(Chip_no: TIOcard); stdcall; external 'K8D.dll';

PROCEDURE SetDACchip(Chip_no: TIOcard); stdcall; external 'K8D.dll';

PROCEDURE ClearAllDAC; stdcall; external 'K8D.dll';

PROCEDURE SetAllDAC; stdcall; external 'K8D.dll';

{8 BIT DA CONVERTER PROCEDURES}

PROCEDURE OutputDAchannel(Channel_no: TDAchannel ; Data: Longint); stdcall; external 'K8D.dll';

PROCEDURE ClearDAchannel(Channel_no: TDAchannel); stdcall; external 'K8D.dll';

PROCEDURE SetDAchannel(Channel_no: TDAchannel); stdcall; external 'K8D.dll';

PROCEDURE ClearAllDA; stdcall; external 'K8D.dll';

PROCEDURE SetAllDA; stdcall; external 'K8D.dll';

{GENERAL PROCEDURES}

PROCEDURE SelectI2CprinterPort(Printer_no: Longint); stdcall; external 'K8D.dll';

PROCEDURE Start_K8000; stdcall; external 'K8D.dll';

PROCEDURE Stop_K8000; stdcall; external 'K8D.dll';

{INPUT FUNCTIONS}

function ReadIOchip(Chip_no: TIOchip):longint; stdcall; external 'K8D.dll';

function ReadIOchannel(Channel_no: TIOchannel):boolean; stdcall; external 'K8D.dll';

function ReadADchannel(Channel_no:TADchannel):longint; stdcall; external 'K8D.dll'

PROCEDURE ReadIOconficArray(Buffer:Pointer); stdcall; external 'K8D.dll';

PROCEDURE ReadIOdataArray(Buffer:Pointer); stdcall; external 'K8D.dll';

PROCEDURE ReadDACarray(Buffer:Pointer); stdcall; external 'K8D.dll';

PROCEDURE ReadDAarray(Buffer:Pointer); stdcall; external 'K8D.dll';

procedure TForm1.FormCreate(Sender: TObject);

begin

Start_K8000;

end;

procedure TForm1.FormClose(Sender: TObject; var Action: TCloseAction);

begin

Stop_K8000;

end;

Using the DLL in Visual basic

Here is the declaration of the K8D.DLL procedures and functions, some array variables, and the two most important procedures of the application program (Form_Load and Form_Terminate).

Option Explicit

'Declare use of the DLL

'K8D.DLL interfaces

'IO CONFIGURATION PROCEDURES

Private Declare Sub ConfigAllIOasInput Lib "k8d.dll" ()

Private Declare Sub ConfigAllIOasOutput Lib "k8d.dll" ()

Private Declare Sub ConfigIOchipAsInput Lib "k8d.dll" (ByVal Chip_no As Long)

Private Declare Sub ConfigIOchipAsOutput Lib "k8d.dll" (ByVal Chip_no As Long)

Private Declare Sub ConfigIOchannelAsInput Lib "k8d.dll" (ByVal Channel_no As Long)

Private Declare Sub ConfigIOchannelAsOutput Lib "k8d.dll" (ByVal Channel_no As Long)

'OUTPUT PROCEDURES

Private Declare Sub IOoutput Lib "k8d.dll" (ByVal Chip_no As Long, ByVal Data As Long)

Private Declare Sub UpdateAllIO Lib "k8d.dll" ()

Private Declare Sub ClearAllIO Lib "k8d.dll" ()

Private Declare Sub SetAllIO Lib "k8d.dll" ()

Private Declare Sub UpdateIOchip Lib "k8d.dll" (ByVal Chip_no As Long)

Private Declare Sub ClearIOchip Lib "k8d.dll" (ByVal Chip_no As Long)

Private Declare Sub SetIOchip Lib "k8d.dll" (ByVal Chip_no As Long)

Private Declare Sub SetIOchannel Lib "k8d.dll" (ByVal Channel_no As Long)

Private Declare Sub ClearIOchannel Lib "k8d.dll" (ByVal Channel_no As Long)

'INPUT FUNCTIONS AND PROCEDURES

Private Declare Function ReadIOchip Lib "k8d.dll" (ByVal Chip_no As Long) As Long

Private Declare Function ReadIOchannel Lib "k8d.dll" (ByVal Channel_no As Long) As Boolean

Private Declare Sub ReadIOconficArray Lib "k8d.dll" (Array_Pointer As Long)

Private Declare Sub ReadIOdataArray Lib "k8d.dll" (Array_Pointer As Long)

Private Declare Sub ReadDACarray Lib "k8d.dll" (Array_Pointer As Long)

Private Declare Sub ReadDAarray Lib "k8d.dll" (Array_Pointer As Long)

'How to use these calls:

' ReadIOconficArray IOconfig(0)

' ReadIOdataArray IOdata(0)

' ReadDACarray DAC(1)

' ReadDAarray DA(1)

'6 BIT DAC CONVERTER PROCEDURES

Private Declare Sub OutputDACchannel Lib "k8d.dll" (ByVal Channel_no As Long, ByVal Data As Long)

Private Declare Sub ClearDACchannel Lib "k8d.dll" (ByVal Channel_no As Long)

Private Declare Sub SetDACchannel Lib "k8d.dll" (ByVal Channel_no As Long)

Private Declare Sub ClearDACchip Lib "k8d.dll" (ByVal Chip_no As Long)

Private Declare Sub SetDACchip Lib "k8d.dll" (ByVal Chip_no As Long)

Private Declare Sub ClearAllDAC Lib "k8d.dll" ()

Private Declare Sub SetAllDAC Lib "k8d.dll" ()

'8 BIT DA CONVERTER PROCEDURES

Private Declare Sub OutputDAchannel Lib "k8d.dll" (ByVal Channel_no As Long, ByVal Data As Long)

Private Declare Sub ClearDAchannel Lib "k8d.dll" (ByVal Channel_no As Long)

Private Declare Sub SetDAchannel Lib "k8d.dll" (ByVal Channel_no As Long)

Private Declare Sub ClearAllDA Lib "k8d.dll" ()

Private Declare Sub SetAllDA Lib "k8d.dll" ()

'8 BIT AD CONVERTER FUNCTION

Private Declare Function ReadADchannel Lib "k8d.dll" (ByVal Channel_no As Long) As Boolean

'GENERAL PROCEDURES

Private Declare Sub SelectI2CprinterPort Lib "k8d.dll" (ByVal port As Long)

Private Declare Sub Start_K8000 Lib "k8d.dll" ()

Private Declare Sub Stop_K8000 Lib "k8d.dll" ()

'COMMON USED GLOBALS

Const MaxIOcard As Long = 3

Const MaxIOchip As Long = 7

Const MaxDACchannel As Long = 32

Const MaxDAchannel As Long = 4

'Declare variables

Dim IOconfig(0 To MaxIOchip) As Long

Dim IOdata(0 To MaxIOchip) As Long

Dim DAC(1 To MaxDACchannel) As Long

Dim DA(1 To MaxDAchannel) As Long

Private Sub Form_Load()

Start_K8000

End Sub

Private Sub Form_Terminate()

Stop_K8000

End Sub