| ELEKTRONİK A.Ş. |  |  | TECHNICAL CHANGE BULLETIN TEKNİK DEĞİŞİKLİK BÜLTENİ |  |  | CHANGE NO: 104 REF NO : DATE :19.12.2001 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AFFECTED MODELS: ALL 11AK19-Y11 CHASSIS BASED TV MODELS. |  |  |  |  |  |  |  |
| START DATE : 20.12.2001 |  |  |  |  |  |  |  |
| REASONS OF CHANGE: DEFECT OF THE RGB AMPLIFIER TDA 6108 ( IC 901 ) TO CATCH A BETTER RELIABILITY ACCORDING SPARKS OF THE PICTURE TUBE THE LAYOUT OF THE CRT-BOARD WAS CHANGED . NOTE 1: THE SPARE-PART NUMBER OF THE COMPLETE INSERTED CRT-BOARD DOES NOT CHANGED OUT |  |  |  |  |  |  |  |
| NO | PCB CODE | VESTEL CODE |  | COMPONENT NAME | POSITION NUMBER |  | QUANTITY |
| 1 | 11TP19-Y2 | 30016007 |  | PCB 11TP19Y2 | - |  | 1 |
| 2 | 11TP19-Y2 | 30001242 |  | RES FUSE 1/4W 47R J | R914 |  | 1 |
| 3 |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |

IN

| NO | PCB CODE | VESTEL CODE | COMPONENT NAME | POSITION NUMBER | QUANTITY |
| --- | --- | --- | --- | --- | --- |
| 1 | 11TP19-Y3 | 30016572 | PCB 11TP19Y3 | - | 1 |
| 2 | ##### 11TP19-Y3 | 30002583 | JUMPER WIRE | R914 | 1 |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |

| APPLICATION ENGINEER | APPLICATION ENGINEER CHIEF | APPLICATION ENGINEER MANAGER |
| --- | --- | --- |
| F. UZGAN |  | BEYHAN DAĞITAN |

Form Rev.No:00										Form No: 04.UMB.01