| VESTEL ELEKTRONİK A.Ş. |  |  | CHANGE BULLETIN (TCN) |  |  | CHANGE NO: 40 REF NO :ÜDM-46/1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AFFECTED MODELS: ALL MODELS WITH 11AK19 CHASSIS. NOTE 1: Connect two resistors (RES CF 1/4W 2K7) as shown in the schematic diagram. Solder one of the pins of 2K7 resistors to the cathodes of diodes D503 & D504 and connect the other ends together. Then make a connection (use a 13 cm. length isolated cable) between this common point and the jumper wire J808 (+5V supply) on the component side of the PCB. Fix all the additional components to the PCB by means of some silicon adhesive. NOTE 2: Add a CAP CER 100 NF/50V between the pin 44 of IC 501 and the ground under the PCB. |  |  |  |  |  |  |  |
| START DATE : 30/01/1999 |  |  |  |  |  |  |  |
| 44 ## IC501 ## SDA52XX 1 2 3 44 4 5 6 R595 R596 REASONS OF CHANGE: TO PREVENT SPONTANEOUS SWITCHING-ON IN ST-BY MODE. APPLY THE BELOW MENTIONED MODIFICATION. S+5V +5V (J808) D503 D504 2X 2K7 ¼ W 100 nF 470R 470R P- P+ OUT |  |  |  |  |  |  |  |
| NO | PCB CODE | VESTEL CODE |  | COMPONENT NAME | POSITION NUMBER |  | QUANTITY |
| 1 | 11AK19 | 30002583 |  | JUMPER WIRE 0.6 MM. | J126, J127 |  | 2 |

IN

| NO | PCB CODE | VESTEL CODE | COMPONENT NAME | POSITION NUMBER | QUANTITY |
| --- | --- | --- | --- | --- | --- |
| 1 | 11AK19 | 30000712 | RES CF 1/4W 470R | J126, J127 | 2 |
| 2 | 11AK19 | 30000628 | RES CF 1/4W 2K7 | NOTE 1 | 2 |
| 3 | 11AK19 | 30000295 | CAP CER 100 NF 50V KB | NOTE 2 | 1 |
| 4 | 11AK19 | 30002388 | CABLE BLUE 0.6MM (13 CM) | NOTE 1 | 1 |

| PRODUCTION SUPPORT ENGINEER | PRODUCTION SUPPORT CHIEF | PRODUCTION SUPPORT MANAGER |
| --- | --- | --- |
| YILDIRAY BAŞKURT |  | CELAL ÖZKUL |

KALT-F.04-00.04