| VESTEL ELEKTRONİK A.Ş. |  |  | CHANGE BULLETIN (TCN) |  |  | CHANGE NO: 41 REF NO :ÜDM-549, 550, 562, 561, 584 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AFFECTED MODELS: ALL STEREO MODELS WITH 11AK19 CHASSIS. |  |  |  |  |  |  |  |
| START DATE : 14.07.1998 |  |  |  |  |  |  |  |
| REASONS OF CHANGE: USAGE OF 2nd VERSION SOUND PROCESSOR IC TDA987X-A. OUT |  |  |  |  |  |  |  |
| NO | PCB CODE | VESTEL CODE |  | COMPONENT NAME | POSITION NUMBER |  | QUANTITY |
| 1 | 11ST19 | 3621598700 |  | TDA9870 (1st VERSION) | IC301 |  | 1 |
| 2 | 11ST19 | 3621598750 |  | TDA9875 (1st VERSION) | IC301 |  | 1 |
| 3 | 11ST19 | 3311001030 |  | RES SMD 10R 1/10W J | R309, R302, R324 |  | 3 |
| 4 | 11ST19 | 3312711030 |  | RES SMD 270R 1/10W | R307 |  | 1 |
| 5 | 11ST19 | 3052210031 |  | CAP SMD 220PF 50V J | C313, C314 |  | 2 |
| 6 | 11ST19 | 3053300031 |  | CAP SMD 33PF 50V K | C333, C334 |  | 2 |
| 7 | 11ST19 | 3063330241 |  | CAP SMD 33NF 50V K | C331 |  | 1 |
| 8 | 11ST19 | 3311031030 |  | RES SMD 10K 1/10W J | R318 |  | 1 |
| 9 | 11ST19 | 3312031030 |  | RES SMD 20K 1/10W J | R317 |  | 1 |
| 10 | 11ST19 | 3013341036 |  | CAP MKT 330NF 63V J | C332 |  | 1 |
| 11 | 11ST19 | 4011336522 |  | FIXED COIL | L301 |  | 1 |
| 12 | 11ST19 | 3520501350 |  | DIODE VAR BB135 SMD | D301 |  | 1 |
| 13 | 11ST19 | 3313931030 |  | RES SMD 39K 1/10W J | R319 |  | 1 |
| 14 | - | 5725428007 |  | 11ST19-6 (With TDA987X) | - |  | - |

IN

| NO | PCB CODE | VESTEL CODE | COMPONENT NAME | POSITION NUMBER | QUANTITY |
| --- | --- | --- | --- | --- | --- |
| 1 | 11ST19 | 3621598701 | TDA9870A | IC301 | 1 |
| 2 | 11ST19 | 3621598751 | TDA9875A | IC301 | 1 |
| 3 | 11ST19 | 3821080500 | JUMPER SMD | R307 | 1 |
| 4 | 11ST19 | 3312291030 | RES SMD 2R2 1/10W J | R302 | 1 |
| 5 | 11ST19 | 3821080500 | JUMPER SMD | C334 | 1 |
| 6 | 11ST19 | 3821080500 | JUMPER SMD | R318 | 1 |
| 7 | 11ST19-7A | 3821080500 | JUMPER SMD | S310, S311 | 2 |
| 8 | 11ST19-5B | 382108500 | JUMPER SMD | S308, S309 | 2 |
| 9 | - | 5725428012 | 11ST19-7A (With TDA987XA) | - | - |

| PRODUCTION SUPPORT ENGINEER | PRODUCTION SUPPORT CHIEF | PRODUCTION SUPPORT MANAGER |
| --- | --- | --- |
|  |  | CELAL ÖZKUL |

KALT-F.04-00.04