| VESTEL ELEKTRONİK A.Ş. |  |  | CHANGE BULLETIN (TCN) |  |  | CHANGE NO: 43 REF NO :ÜDM- 010, 011 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AFFECTED MODELS: ALL TV MODELS WITH 11AK19 CHASSIS INCLUDING MSP34XX SOUND PROCESSOR BASED ST. SOUND BOARD. |  |  |  |  |  |  |  |
| # START DATE : 15.01.1999 |  |  |  |  |  |  |  |
| REASONS OF CHANGE: TO IMPROVE SOUND PERFORMANCE. NOTE 1: MUST BE CONNECTED BETWEEN THE PIN OF S402 WHICH IS CONNECTED TO THE PIN 7 OF PL300 AND THE GROUND PIN OF C333. NOTE 2: MUST BE CONNECTED BETWEEN THE PIN14 OF PL301 AND THE GROUND POINT OF J110. NOTE 3 :MUST BE CONNECTED BETWEEN THE GROUND POINT OF C401 AND THE GROUND POINT OF C293. CONNECTION BETWEEN THE PIN 27 OF IC301 AND HP-GROUND MUST BE REMOVED. OUT |  |  |  |  |  |  |  |
| NO | PCB CODE | VESTEL CODE |  | COMPONENT NAME | POSITION NUMBER |  | QUANTITY |
| 1 | 11ST19-A3 | 3821080500 |  | SMD JUMPER | S401, S402 |  | 2 |
| 2 | 11ST19-A3 | 5913225000 |  | JUMPER WIRE | J111 |  | 1 |
| 3 | 11ST19-A3 | 3061020241 |  | CAP SMD 1 NF 50V | C101, C107 |  | 2 |
| 4 | 11ST19-A3 (With HP) | 3081010356 |  | CAP EL 100uF /16V | C293 |  | 1 |

IN

| NO | PCB CODE | VESTEL CODE | COMPONENT NAME | POSITION NUMBER | QUANTITY |
| --- | --- | --- | --- | --- | --- |
| 1 | 11ST19-A3 | 4942010001 | CABLE O.6 MM BLUE | NOTE1 (6 CM) | 1 |
| 2 | 11ST19-A3 | 4942010001 | CABLE 0.6 MM BLUE | NOTE 2 (3 CM) | 1 |
| 3 | 11ST19-A3 | 3065620241 | CAP SMD 5.6NF/50V | C101, C107 | 2 |
| 4 | 11ST19-A3 (With HP) | 3083310356 | CAP EL 330uF /16V | C293 | 1 |
| 5 | 11ST19-A3 (With HP) | 4942010001 | CABLE 0.6 MM BLUE | NOTE 3 (6 CM) | 1 |

| PRODUCTION SUPPORT ENGINEER | PRODUCTION SUPPORT CHIEF | PRODUCTION SUPPORT MANAGER |
| --- | --- | --- |
| YILDIRAY BAŞKURT |  | CELAL ÖZKUL |

KALT-F.04-00.04