| VESTEL ELEKTRONİK A.Ş. |  |  | CHANGE BULLETIN (TCN) |  |  | CHANGE NO: 49 REF NO : |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AFFECTED MODELS: ALL MODELS WITH 11AK19 CHASSIS. |  |  |  |  |  |  |  |
| START DATE : 28.04.1999 |  |  |  |  |  |  |  |
| REASONS OF CHANGE: TO REMOVE FLASH-OVER PROBLEM. NOTE-1 : CONNECT THE CABLE BETWEEN THE DEFLECTION YOKE FIXING SCREW ON THE PICTURE TUBE NECK AQUA-GND ON THE CRT BOARD. NOTE-2 : USE THIS NUT TO FIX THE CABLE TO THE SCREW ON THE TUBE NECK. NOTE-3: PLEASE APPLY THIS MODIFICATION TO ALL THE TVs THAT RETURNED TO YOUR SERVICE SHOPS FOR ANY KIND OF REASON. OUT |  |  |  |  |  |  |  |
| NO | PCB CODE | VESTEL CODE |  | COMPONENT NAME | POSITION NUMBER |  | QUANTITY |
| - | - | - |  | - | - |  | - |

IN

| NO | PCB CODE | VESTEL CODE | COMPONENT NAME | POSITION NUMBER | QUANTITY |
| --- | --- | --- | --- | --- | --- |
| 1 | 11AK19 | 30002876 | 15 CM CABLE WITH CONNECTOR | NOTE – 1 | 1 |
| 2 | 11AK19 | 35000512 | 4 MM METRIC NUT | NOTE – 2 | 1 |

| PRODUCTION SUPPORT ENGINEER | PRODUCTION SUPPORT CHIEF | PRODUCTION SUPPORT MANAGER |
| --- | --- | --- |
| YILDIRAY BAŞKURT |  | CELAL ÖZKUL |

KALT-F.04-00.04