| VESTEL ELEKTRONİK A.Ş. |  |  | CHANGE BULLETIN (TCN) |  |  | CHANGE NO: 52 REF NO :UMB - 701 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AFFECTED MODELS: ALL MODELS WITH 11AK19 CHASSIS. |  |  |  |  |  |  |  |
| START DATE : 01.07.1999 |  |  |  |  |  |  |  |
| REASONS OF CHANGE: TO IMPROVE TELETEXT AND SYNCRONIZATION PERFORMANCE. NOTE 1: This SMD type resistor must be soldered between the Base and Collector pins of Q406 under the PCB. NOTE 2: This resistor must be soldered between the collectors of Q408 and Q409 on the component side of the PCB. OUT |  |  |  |  |  |  |  |
| NO | PCB CODE | VESTEL CODE |  | COMPONENT NAME | POSITION NUMBER |  | QUANTITY |
| 1 | 11AK19 | 30000558 |  | RES SMD 1/10W 180R J | R465 |  | 1 |
| 2 | 11AK19 | 30000524 |  | RES SMD 1/10W 150R J | R429 |  | 1 |
| 3 | 11AK19 | 30002583 |  | JUMPER WIRE | J264 |  | 1 |
| 4 | 11AK19 | 30000668 |  | RES SMD 1/10W 33K J | R556 |  | 1 |

IN

| NO | PCB CODE | VESTEL CODE | COMPONENT NAME | POSITION NUMBER | QUANTITY |
| --- | --- | --- | --- | --- | --- |
| 1 | 11AK19 | 30000688 | RES SMD 1/10W 390R J | R465 | 1 |
| 2 | 11AK19 | 30000626 | RES SMD 1/10W 270R J | R429 | 1 |
| 3 | 11AK19 | 30000469 | RES SMD 1/10W 1K J | J264 | 1 |
| 4 | 11AK19 | 30000475 | RES SMD 1/10W 10K J | # NOTE 1 | 1 |
| 5 | 11AK19 | 30000594 | RES CF 1/4W 22K J | # NOTE 2 | 1 |

| PRODUCTION SUPPORT ENGINEER | PRODUCTION SUPPORT CHIEF | PRODUCTION SUPPORT MANAGER |
| --- | --- | --- |
| YILDIRAY BAŞKURT |  | CELAL ÖZKUL |

KALT-F.04-00.04