| VESTEL ELEKTRONİK A.Ş. |  |  | CHANGE BULLETIN (TCN) |  |  | CHANGE NO: 54 REF NO :- |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AFFECTED MODELS: 90o AND 14” MODELS WITH 11AK19 CHASSIS, 170-270V POWER SUPPLY. |  |  |  |  |  |  |  |
| # START DATE : 16.08.1999 |  |  |  |  |  |  |  |
| REASONS OF CHANGE: DELIVERY PROBLEMS OF MC44604 SMPS CONTROLLER IC. NOTE 1:A SERIAL 3V6 ZENER HAS TO BE ADDED TO THE 11V ZENER (THE CATHODE OF THE 11V ZENER HAS TO BE CONNECTED TO THE ANODE OF 3V6 ZENER). LATERON THESE TWO DIODES WILL BE REPLACED WITH ONE 15V ZENER. NOTE 2: PIN15 OF MC44603 IC HAS TO BE CUT BEFORE ASSEMBLY. NOTE 3: NORMAL 3V6 ZENER DIODE HAS TO BE ADDED FROM SOLDER SIDE (THE CATHODE SHOULD BE CONNECTED TO R537) UNTIL SMD 3V6 DIODE IS AVAILABLE. NOTE 4: SERIAL CONNECTED TO D606 NOTE 5: TWO BYW95A DIODES HAVE TO BE CONNECTED IN SERIES AND HAVE TO BE ASSEMLED AT THE SAME POSITION. ## OUT |  |  |  |  |  |  |  |
| NO | PCB CODE | VESTEL CODE |  | COMPONENT NAME | POSITION NUMBER |  | QUANTITY |
| 1 | ##### 11AK19 | 30000599 |  | RES CF 1/4W 220K | R821 |  |  |
| 2 | “ | 30000466 |  | RES CF 1/4W 1K | R809 |  |  |
| 3 | “ | 30000074 |  | CAP MKT 100nF 63V | C807 |  |  |
| 4 | “ | 30001128 |  | RES MO 2W 22K J | R806 |  |  |
| 5 | “ | 30000815 |  | RES CF 1/4W 8.2K J | R808 |  |  |
| 6 | “ | 30000283 |  | CAP CER 1nF 50V K | C855 |  |  |
| 7 | “ | 30001349 |  | DIODE ZENER 11V | D819 |  | (NOTE-1) |
| 8 | “ | 30000594 |  | RES CF 1/4W 22K J | R810 |  |  |
| 9 | “ | 30002583 |  | JUMPER SMD | S801 |  |  |
| 10 | “ | 30000593 |  | RES SMD 1/10W 2.2K J | R537 |  |  |
| 11 | “ | 30000525 |  | RES CF 1/2W 1K5 J | R815 |  |  |
| 12 |  | 30001499 |  | IC MC44604 | IC802 |  |  |
| 13 |  | 30002583 |  | JUMPER WIRE | R802,R803 |  | 2 |

## IN

| NO | PCB CODE | VESTEL CODE | COMPONENT NAME | POSITION NUMBER | QUANTITY |
| --- | --- | --- | --- | --- | --- |
| 1 | 11AK19B-2 | 30000495 | RES CF 1/4W 1.2K J | R821 |  |
| 2 | “ | 30000718 | RES CF 1/4W 4.7K J | R809 |  |
| 3 | “ | 30000384 | CAP EL 2.2uF 50V | C807 |  |
| 4 | “ | 30001107 | RES MO 3W 15K J | R806 |  |
| 5 | “ | 30002583 | JUMPER WIRE | R808 |  |
| 6 | “ | 30003722 | DIODE ZENER 15V | D819 | (NOTE-1) |
| 7 | “ | 30000696 | RES SMD 1/10W 39K J | R887 |  |
| 8 | “ | 30000688 | RES SMD 1/10W 390R J | R537 |  |
| 9 | “ | 30000664 | RES SMD 1/10W 3K3 J | R883 |  |
| 10 | “ | 30007761 | DIODE ZENER 3V6 | D820 | (NOTE- 3) |
| 11 | “ | 30000475 | RES SMD 1/10W 10K J | R885 |  |
| 12 | “ | 30000475 | RES SMD 1/10W 10K J | R886 |  |
| 13 | “ | 30001235 | RES FUSE 1/2W 0.33R J | RXXX | (NOTE-4) |
| 15 | “ | 30001478 | IC MC44603AP | IC802 | (NOTE-2) |
| 16 | “ | 30001307 | DIODE BYW95A | D814,D815 | (NOTE-5) |
| 17 | “ | 30001235 | RES FUSE 1/2W 0R33 J | R802,R803 | 2 |

| CUSTOMER SUPPORT ENGINEER | PRODUCTION SUPPORT CHIEF | PRODUCTION SUPPORT MANAGER |
| --- | --- | --- |
| ARZU ÇIDARER | YILDIRAY BAŞKURT | CELAL ÖZKUL |

KALT-F.04-00.04