| VESTEL ELEKTRONİK A.Ş. | CHANGE BULLETIN (TCN) | CHANGE NO: 57 REF NO : ÜDM-828 |
| --- | --- | --- |
| AFFECTED MODELS: MODELS WITH 11AK19 CHASSIS HAVING DOLBY+PIP OR ONLY DOLBY FEATURE |  |  |
| REASONS OF CHANGE: INTRODUCTION OF THE SIEMENS OTP IC SDA 545X INTO THE 11AK19-8A CHASSIS INSTEAD OF THE PIGGY PACK CARD HAVING SDA 5257. |  |  |

OUT

| NO | PCB CODE | VESTEL CODE | COMPONENT NAME | POSITION NUMBER | QUANTITY |
| --- | --- | --- | --- | --- | --- |
| 1 | 11AK19-8A | 30000247 | CAP SMD 39PF 50V | C524, C525 | 1 |
| 2 | “ | 30000818 | RES SMD 1/10W 8.2K | R540 | 1 |
| 3 | “ | 30000294 | CAP SMD 100NF 50V | C506 | 1 |
| 4 | “ | 30000820 | RES CF 1/4W 82K | R547 | 1 |
| 5 | “ | 30000099 | CAP MKT 33NF 63V | C518,C522 | 1 |
| 6 | “ | 30000252 | CAP SMD 47PF 50V | C511,C512 | 1 |
| 7 | “ | 30000778 | RES SMD 1/10W 6.8K | R535,R536 | 1 |
| 8 | “ | 30000345 | CAP EL 10UF 50V | C507 | 1 |
| 9 | “ | 30000672 | RES SMD 1/10W 330K | R584 | 1 |
| 10 | “ | 30000471 | RES CF 1/4W 10K | R871 | 1 |
| 11 | “ | 30000815 | RES CF 1/4W 8.2K J | R808 | 1 |
| 12 | “ | 30000717 | RES SMD 1/10W 470R | R465 | 1 |
| 13 | “ | 30000469 | RES SMD 1/10W 1K J | R447 | 1 |
| 14 | “ | 30001988 | FIXED COIL 6.8UH K RAD | L501 | 1 |
| 15 | “ | 30001129 | RES MO 3W 22KJ | R806 | 1 |
| 16 | “ | 30001775 | REZ 18MHZ | X501 | 1 |

IN

| NO | PCB CODE | VESTEL CODE | COMPONENT NAME | POSITION NUMBER | QUANTITY |
| --- | --- | --- | --- | --- | --- |
| 1 | 11AK19-8A | 30000237 | CAP SMD 33PF 50V | C524,C525 | 1 |
| 2 | “ | 30000631 | RES SMD 1/10W 2.7K | R540 | 1 |
| 3 | “ | 30000289 | CAP SMD 10NF 50V | C506 | 1 |
| 4 | “ | 30000633 | RES CF 1/4W 27K | R547 | 1 |
| 5 | “ | 30000484 | RES SMD 1/10W 1M | R584 | 1 |
| 6 | “ | 30000660 | RES CF 1/4W 3.3K | R871 | 1 |
| 7 | “ | 30000801 | RES CF 1/4W 7.5K J | R808 | 1 |
| 8 | “ | 30000558 | RES SMD 1/10W 180R | R465 | 1 |
| 9 | “ | 30000626 | RES SMD 1/10W 270R | R447 | 1 |
| 10 | “ | 30007431 | RES MO 5W 10K J | R806 | 1 |
| 11 | “ | 30006662 | XTAL 6MHZ | X501 | 1 |
| 12 | “ | 30006718 | IC SDA545X OTP | IC501 | 1 |

| PRODUCTION SUPPORT ENGINEER | PRODUCTION SUPPORT CHIEF | PRODUCTION SUPPORT MANAGER |
| --- | --- | --- |
| YILDIRAY BAŞKURT |  | CELAL ÖZKUL |

KALT-F.04-00.04